3GPP TSG RAN WG1 #110bis-e		R1-2209098

e-Meeting, October 10th – 19th, 2022

Agenda Item: 	9.3.1

Source: 	Sony

Title: 	Preliminary System Level Simulation Results for SBFD

Document for: 	Discussion / decision

# Introduction

This contribution presents some preliminary system level simulation results comparing the performance in using SBFD over legacy TDD in an indoor office environment.  The simulation assumptions follow those agreed in RAN1#109e [1] and RAN1#110 [2].

# Simulation Assumptions

The general simulation assumptions used for the system level simulations to compare SBFD against legacy TDD are summarized in

Table 1: General simulation assumptions

| Parameters | Legacy TDD (Baseline) | SBFD |
| --- | --- | --- |
| Deployment | Single operator with single carrier. All cells use a fixed slot format & frame structure. |  |
| Scenario | Indoor office (as defined in 38.901) |  |
| Frequency | 4GHz (FR1) |  |
| Frame structure | {DDDSU} S=[12D:2G:0U] | Frame Structure #2 = {XXXXU} X = {DUD} SBFD Slot |
| Bandwidth | 100 MHz 273 RBs @ 30 kHz SCS | 100 MHz 273 RBs @ 30 kHz SCS {DUD} = {104: 53: 104} Guard subbands = 6 RB |
| Traffic | FTP3 Each UE is either assigned UL traffic or DL traffic (Option 1) Symmetric UL/DL packet size (Option 1) = 0.5 Mbytes Packet arrival rates: - lD= 2.5, lU= 1.25 - lD= 2.5, lU= 0.8 |  |
| Modulation | Up to 64 QAM |  |
| Max HARQ ReTx | 4 |  |
| Receiver | MMSE-IRC |  |
| BS Tx power | 24 dBm per 20 MHz for 4GHz Note: For system BW larger than above, Tx power scales up accordingly factor. |  |
| BS antenna configurations (FR1 Indoor Office) | (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1; 4,4) (dH, dV) = (0.5, 0.5)λ, +45°/-45° polarization | SBFD antenna configuration option-1 (Method 1) - Two panel groups - For each panel group: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 1) - Number of TxRUs: same as legacy TDD - (dH, dV) = (0.5, 0.5)λ, +45°/-45° polarization, (da,H, da,V) = (0, 4)λ |
| BS antenna panels | Legacy TDD | SBFD Option 2: Method 2-1 |
| BS Rx noise figure | 5 dB |  |
| UE Tx Power | 23 dBm |  |
| UE Antenna Configuration | 2Tx/2Rx, (M, N, P, Mg, Ng; Mp, Np) = (1,1,2,1,1;1,2) (dH, dV) = (0.5, N/A)λ |  |
| UE Rx noise figure | 9 dB |  |

The Indoor Office parameters used for legacy TDD and SBFD simulations are summarized in Table 2.

Table 2: Indoor office parameters

| Parameters | Values |
| --- | --- |
| Channel model | InH (38.901) |
| UE Distribution | 100% indoor |
| Layout | 120×50m |
| Number of BS | 12 |
| Inter-BS (2D) distance | 20 m |
| Minimum BS-UE 2D distance | 0 m (TR38.802) |
| Minimum UE-UE 2D distance | 1 m (TR38.828) NOTE: This was FFS in RAN1#110 agreement |
| BS antenna height | 3 m |
| UE numbers per BS | 4 |

## Inter Subband Interference

RAN1 has sent an LS to RAN4 asking for recommendations on inter subband interference models [3], notably on the interference experience at RB n in a subband due to transmission in RB m in another subband.  Since the response from RAN4 is still pending, in this simulation, we used a simplistic model where the transmission power in RB m to RB m+k, is attenuated by X dB in RB m+k+1 to RB m+2k and RB m-k to RB m-1.  Any RBs further from m-k and m+2k experiences no interference.  That is, the attenuation to the adjacent set of RBs is X dB as shown in Figure 1.  NOTE: The attenuation X excludes any further isolation such as spatial isolation between Tx and Rx panels in the gNB or radio propagation loss.  The attenuation values used for X follows the ACIR values in 38.828 and are summarized in Table 3.

Figure 1: Power attenuation into adjacnet set of RBs

Table 3: Attenuation X to adjacnet set of RBs

| Transmitter | Receiver | Attenuation (X) |
| --- | --- | --- |
| BS | BS | 43 dB |
| BS | UE | 33 dB |
| UE | BS | 30 dB |
| UE | UE | 28 dB |

For gNB, i.e. BS, self interference, i.e. inter subband interference from DL transmission to its UL transmission, we assumed a spatial isolation of 80 dB between Tx and Rx panels [4].  An example is shown in Figure 2, where a gNB is configured with {DU} SBFD operation.  The DL subband starts from RB 0 to RB N whilst the UL subband starts from RB N+1 to RB N+J.  The gNB makes a DL transmission in RB m to RB m+K at Y dBm/Hz and using the simplified model, RBs N to RB m+2K experiences interference of Y-X-80 = Y-123 dBm/Hz, where X=43 dB from Table 3 and 80 dB is the spatial isolation between Tx and Rx panels.

Figure 2: gNB self inter subband interference

# System Level Simulation Results

The Mean, 5%, 50% and 95% User Perceived Throughput (UPT) for baseline TDD and SBFD with packet arrival rate lD= 2.5, lU= 1.25, in the DL and UL are plotted in Figure 3 and Figure 4 respectively.

Figure 3: DL UPT for lD= 2.5, lU= 1.25

Figure 4: UL UPT for lD= 2.5, lU= 1.25

The Mean, 5%, 50% and 95% User Perceived Throughput (UPT) for baseline TDD and SBFD with packet arrival rate lD= 2.5, lU= 0.8, in the DL and UL are plotted in Figure 5 and Figure 6 respectively.

Figure 5: DL UPT for lD= 2.5, lU= 0.8

Figure 6: UL UPT for lD= 2.5, lU= 0.8

The gain in UPT using SBFD against the baseline TDD for both sets of packet arrival rates are summarized in Table 4 and Table 5 for DL and UL respectively.  Since the DL frequency resources are reduced by 23.8% in SBFD compared to the baseline TDD, we observed major losses in DL UPT of about 32%.  However, in the UL, the UPT using SBFD provides significant gain especially for the case with higher UL traffic load (lU= 1.25).  The gain is most significant at the 5% UPT, where in the higher UL traffic load case (lU= 1.25), the baseline TDD struggles to serve the cell edge UEs giving only 3 Mbps.

Table 4: DL UPT Gain/Loss

| Packet Arrival Rate | DL UPT Gain/Loss (%) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Mean | 5% | 50% | 95% |
| lD= 2.5, lU= 1.25 | -32% | -20% | -39% | -36% |
| lD= 2.5, lU= 0.8 | -32% | -20% | -39% | -36% |

Table 5: UL UPT Gain/Loss

| Packet Arrival Rate | UL UPT Gain/Loss (%) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Mean | 5% | 50% | 95% |
| lD= 2.5, lU= 1.25 | 350% | 2031% | 323% | 108% |
| lD= 2.5, lU= 0.8 | 84% | 231% | 79% | 30% |

The Type 1 Resource Utilization (RU) is defined as the number of RBs per cell used by traffic for the given link direction during observation time / Total number of all the RBs per cell including DL, UL and guard bands over observation time [2].  The Type 1 RUs observed are summarized in Table 6.  For the DL, due to reduced DL resources, the RU increases in SBFD as expected.  In the UL, the RU reduces in SBFD due to increase in UL resources.

Table 6: Type 1 Resource Utilisation

| Scenario | lD= 2.5, lU= 1.25 |  | lD= 2.5, lU= 0.8 |  |
| --- | --- | --- | --- | --- |
|  | DL RU (%) | UL RU (%) | DL RU (%) | UL RU (%) |
| Baseline TDD | 4.86% | 50.55% | 4.86% | 10.41% |
| SBFD | 7.24% | 11.46% | 7.24% | 5.68% |

The major loss in DL UPT can be reduced if the resources in the DL subbands are increased.  That is, the SBFD configuration of the frequency locations would need to be adjusted to the expected traffic load.  Since the traffic load may change, a semi-statically configured SBFD locations may not be able to adapt to these changes and may led to loss in DL or UL UPT.  Hence, to better adapt to the traffic load, the SI should consider dynamic SBFD configurations.

Observation 1: In the system level simulation, SBFD provides significant UL User Perceived Throughput gain compared to baseline TDD with mean gain of up to 350% gain at the expense of noticeable DL UPT loss with a mean loss of 32%.

Observation 2: The DL subband resource may be under configured for the traffic load, thereby leading to major losses in UPT.

Proposal 1: Consider dynamic configurations of SBFD frequency and time locations to adapt to changing traffic loads.

# Conclusion

In this contribution, we provided some preliminary system level simulation results for SBFD.  We observed the following:

Observation 1: In the system level simulation, SBFD provides significant UL User Perceived Throughput gain compared to baseline TDD with mean gain of up to 350% gain at the expense of noticeable DL UPT loss with a mean loss of 32%.

Observation 2: The DL subband resource may be under configured for the traffic load, thereby leading to major losses in UPT.

We therefore propose the following:

Proposal 1: Consider dynamic configurations of SBFD frequency and time locations to adapt to changing traffic loads.

# References

[1] R1-2205540, “Summary# of email discussion on evaluation of NR duplex evolution,” Moderator (CMCC), RAN1#109e

[2] R1-2208270, “Summary#6 on evaluation on NR duplex evolution,” Moderator (CMCC), RAN1#110

[3] R1-2205543, “LS on interference modelling for duplex evolution,” RAN1, RAN1#109e

[4] R1-2207231, “Feasibility and techniques for Subband non-overlapping full duplex,” Qualcomm, RAN1#110