3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2209181

# e-Meeting, October 10th – 19th, 2022

Agenda Item:	5

Source:	Ericsson

Title:	Discussion on reply LS from RAN4 on the minimum guard period between two SRS resources for antenna switching

Document for:	Discussion, Decision

# 1	Introduction

This contribution contains a discussion related to the reply LS from RAN4 on the minimum guard period between two SRS resources for antenna switching [1].

# 2	Background

In RAN1#107bis-e, the following agreement was made in RAN1 regarding minimum guard period between two SRS resources used for antenna switching for 480 and 960 kHz SCS.

Agreement:

Like in Rel-15, a minimum guard period Y between two SRS resources of an SRS resource set for antenna switching is supported for 480 kHz and 960 kHz

- FFS: Whether to define different values of Y for 480 kHz and 960 kHz or not

- FFS: Values of Y dependent on RAN4 feedback on the switching time requirement

Based on this, an LS was sent to RAN4 [2] asking what the value of Y should be for 480 and 960 kHz SCS. RAN4 has now provided a reply [1] with the following answer:

Answer from RAN4: RAN4 has considered the absolute switching time evaluated for FR2-1 in R15, i.e. the antenna switching time is 15 usec with a view to reusing this value for FR2-2. The detailed evaluation results for the switching scenarios considered are included in the R15 reply LS R4-1710048 [1]. RAN4 will further consider this result and its feasibility for SRS transmission used for antenna switching and whether the switching time assumed could be improved for 480 an 960 kHz operation.

# Discussion

Assuming the absolute switching time is 15 us for both 480 and 960 kHz SCS as agreed in RAN4, this translates to Y = 7 and 14 OFDM symbols, respectively, when rounding up to the nearest integer value.

For 15, 30, 60, and 120 kHz SCS, Table 6.2.1.2-1 in 38.214 specifies values for Y. We propose to extend this table with two more rows to capture the value of Y for 480 and 960 kHz. Clearly, a CR is needed for this, and we provide a draft CR in [3]. Discussion of this draft CR can occur under Agenda Item 8.2 "Maintenance on Supporting NR from 52.6GHz to 71 GHz."

Discuss RAN4 reply LS under AI 8.2.

Adopt draft CR in [3].

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Discuss RAN4 reply LS under AI 8.2.

Proposal 2	Adopt draft CR in [3].

# References

- R1-2208352, "Reply LS for the minimum guard period between two SRS resources for antenna," RAN4, RAN1#110bis-e, October 2022.

R1-2200796, "LS on a minimum guard period between two SRS resources for antenna switching," RAN1, RAN1#107bis-e, January 2022.

R1-2209179 “Draft CR on minimum guard period between SRS resources”, Ericsson, RAN1#110bis-e, October 2022.