3GPP TSG RAN WG1 #110-bis-e                                      R1-2209197

e-Meeting, Oct. 10th – 19th, 2022

Source: 	ZTE, Sanechips

Title:	Discussion on XR specific power saving techniques

Agenda item:	9.10.1

Document for:	Discussion

Introduction

The Rel-18 XR SID was approved in RAN#94 [1]. For each objective involved in RAN1 in the SID, the high/medium/low priority issues were identified in RAN1#109-e meeting [2]. Based on the high priority XR-specific power saving issues, a recommendation from RAN1 was made based on evaluations provided in RAN1#110 meeting. Then, in this contribution, at least solutions to high priority issues or way forward to handle these issues are discussed based on corresponding evaluations and progress.

Discussion

Periodicity alignment between enhanced CDRX and XR traffic

In RAN1#110, RAN1 made an agreement as shown in Appendix B that RAN1 recommended a solution for this issue. Moreover, detailed options for periodicity alignment have been discussed, including RRC-based mechanisms and dynamic indication solutions.

In general, it seems controversial to down-select those options in RAN1. And in fact, RAN2 is also involved to this issue, where RRC-based solutions were proposed by companies in RAN2#119-e [2]. As we elaborated in [3], RRC-based mechanisms are basic RAN2’s expertise, RAN1 can evaluate the detailed power saving performance, so it will be easier for RAN2 to finalize this problem without considering dynamic indication solutions which may happen in a more complicate scenario. And it can be a start point for the XR normative work. More specifically, the options include following solutions.

- Non-uniform CDRX cycle pattern

For a legacy CDRX configuration, drx-LongCycle is configured as a fixed value, which may not be adapted to XR traffic periodicity. Thus, non-uniform CDRX cycle pattern can be considered. In this way, multiple small CDRX cycles are used to compose a longer cycle, e.g.,50 ms. For example, a CDRX cycle set with three small CDRX cycles, e.g., (17ms, 17ms, 16ms) or (16ms,16ms,18ms), can form a long cycle. And the alignment between XR traffic arrival and CDRX cycle is achieved at the start of each long cycle.

- Uniform non-integer CDRX cycle

For this case, the formula defined in TS 38.321 can be basically reused with small modification as follows, i.e., start the drx-onDurationTimer if floor([(SFN×10) + subframe number] modulo (drx-LongCycle)) = drx-StartOffset. wherein the value of drx-LongCycle can be a non-integer CDRX cycle (e.g., 1000/FPS ms). Although a non-integer CDRX cycle value is introduced, this option actually achieves CDRX cycle pattern with e.g., (17ms, 17ms, 16ms).

- Multiple CDRX configurations

In our understanding, enhanced multiple CDRX configurations are proper to handle multiple XR flows. And it can be also used to align the CDRX cycle with XR traffic. For example, the common CDRX parameters include drx-LongCycle, drx-onDurationTimer, drx-InactivityTimer, and separate CDRX parameter such as staggered start offsets can be used in order to achieve alignment per long CDRX cycle.

The performance evaluations of these solutions are provided in Table 1~Table 3. The scenarios such as indoor hotspot scenario, and dense urban are considered. And results of both joint UL&DL and DL only are provided.

Table 1 FR1 power consumption results in Indoor Hotspot scenario (VR30M, fps=60, DL + pose/control)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all UEs (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline | - | - | - | 11 | 11 | 93.18% | - |
| R15 CDRX | 10 | 8 | 4 | 11 | 11 | 90.15% | 6.65% |
|  | 16 | 10 | 5 | 11 | 11 | 81.82% | 13.9% |
| Non-uniform CDRX cycle | (17,17,16) | 6 | 4 | 11 | 11 | 90.15% | 23.86% |
| Uniform non-integer CDRX cycle | (1000/60) | 6 | 4 | 11 | 11 | 90.15% | 23.85% |
| Enhanced multiple CDRX (3 CDRX configurations) | 50ms DRX cycle | 6 | 4 | 11 | 11 | 90.11% | 24% |

Table 2 FR1 power consumption results in Indoor Hotspot scenario (VR30M, fps=60, DL only)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all UEs (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline | - | - | - | 11 | 11 | 93.18% | - |
| R15 CDRX | 10 | 8 | 4 | 11 | 11 | 90.15% | 7% |
|  | 16 | 10 | 5 | 11 | 11 | 81.82% | 18.47% |
| Non-uniform CDRX cycle | (17,17,16) | 6 | 4 | 11 | 11 | 90.15% | 33.9% |
| Uniform non-integer CDRX cycle | (1000/60) | 6 | 4 | 11 | 11 | 90.18% | 33.8% |
| Enhanced multiple CDRX (3 CDRX configurations) | 50ms DRX cycle | 6 | 4 | 11 | 11 | 90.1% | 34% |

Table 3 FR1 power consumption results in Dense Urban scenario (VR45M, fps=60, DL only, jitter=[-4,4]ms)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Always On | - | - | - | 7 | 7 | 91.16% |  |
| Aligned CDRX with XR traffic | Aligned every 50ms | 8 | 5 | 7 | 7 | 89.8% | 23% |
| Aligned CDRX with XR traffic | Aligned every 50ms | 10 | 5 | 7 | 7 | 90.48% | 16% |

- Observation 1:   The CDRX alignment schemes including non-uniform CDRX cycle, uniform non-integer CDRX cycle, or enhanced multiple CDRX configurations can achieve periodicity alignment between enhanced CDRX and XR traffic.

- For FR1, Indoor Hotspot, [-4,4]ms jitter range, VR30M

  - For DL+UL, the CDRX alignment schemes have 23.85%~24% power saving gain.

  - For DL only, the CDRX alignment schemes have 33.8%~34% power saving gain.

- For FR1, Dense Urban, [-4,4]ms jitter range, VR45M DL only, the CDRX alignment schemes have 16%~23% power saving gain.

Proposal 1:	Capture the above evaluation results of solutions of matching periodicities between XR traffic and CDRX into TR 38.835.

With these options which have been evaluated by RAN1, in our view, RAN2 can get the low-hanging fruit when consider more specific implementations. In addition, other implementations are possible to be accounted.

Proposal 2:	RAN1 can recommend at least the studied RRC-based solutions for enhancement of CDRX to align with XR traffic periodicity for normative phase.

More discussion on the differences among these solutions are highlighted as Table 4. For example, detailed differences from specification/ implementation aspects, are given for better understanding of these options and evaluations. Meanwhile, in our view, RAN2 will continue to study the details of RRC-based mechanisms. Furthermore, whether or not adopt dynamic indication-based methods for XR services/applications, should be handed in RAN1, this issue is further discussed in section 2.3.

Table 4 Comparison among semi-static RRC configuration approaches.

|  | 1)Non-uniform CDRX cycle pattern | 2)Uniform non-integer CDRX cycle | 3)Multiple CDRX configurations |
| --- | --- | --- | --- |
| Performance | Similar power saving gain with acceptable capacity loss. |  |  |
| New CDRX parameters/ RRC signaling | Low/medium RRC signaling overhead, with new parameters, Form flexible DRX cycle pattern, e.g., (17,17,16), (16,16,18) etc. | a set of CDRX parameters are required with new parameters Form uniform DRX cycle pattern, e.g., (17,17,16) | Multiple sets of CDRX parameters are required with new parameters |
| Specification impact | Complex switching among multiple sub-cycles | Modified the formula. The cycle in the formula can be non-integer, e.g., 50/3 ms | Complex management of multiple sets of timers Support multiple CDRX in one serving cell |

Jitter handling

As discussed for jitter handling, if the CDRX onDuration was extended e.g., up to 12ms under a CDRX configuration e.g.,(16,12, 4), UE could monitor PDCCH for arbitrary packet arrival. However, the power consumption will increase because of such a long time of PDCCH monitoring. Moreover, extremely long onDurationTimer would not be proper for the DRX configuration for service other than XR in the cell. On the other hand, if a shorter onDurationTimer is configured, e.g., (16, 6, 4), the performance could degrade dramatically. More importantly, for some realistic DL video traces, the instantaneous jitter value can be larger than 4ms, in this case, it become more essential to identify solutions to handle the worse cases. And jitter range will be provided by SA2 such as statistic assistance information to RAN, if a larger jitter range is given from SA2, the network definitely should take it into account.

Then given the previous discussion of solutions, we consider to support additional active time based on current CDRX design, which has minor specification impact. And there is related ongoing discussion in RAN2 as well.

In this case, an additional PDCCH monitoring time on demand can be applied to monitor the scheduling which is out of CDRX active time. That is, if a data packet arrives later than the end of DRX On, an additional active time can be activated for receiving this data packet. For instance, an additional active time can be triggered based on no data reception. That means, the UE can be implicitly indicated to activate another active time in response to no data reception before the end of original DRX On duration.

From RAN1’s evaluations perspective, the proposed solutions can be supported for both (-4ms,4ms) and larger jitter value range, and they show different level of PSG with different assumption of jitter ranges. As shown in Table 5 ~Table 12, flexible additional active time triggered via non-data reception can achieve better performance in comparison with the performance of aligned CDRX solutions. That basically justify the enhancement of CDRX mechanism to accommodate to transmission with jitter impact.

Table 5 FR1 power consumption results in Indoor Hotspot scenario (VR30M, fps=60, DL + pose/control)

{Jitter= [-8,8]ms; additional active time = original DRX On duration}

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all UEs (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aligned CDRX with XR traffic | Aligned every 50ms | 12 | 4 | 11 | 11 | 88.36% | 7.5% |
| Flexible additional active time + aligned CDRX | Aligned every 50ms | 5 | 4 | 11 | 11 | 90% | 14.18% |

Table 6 FR1 power consumption results in Indoor Hotspot scenario (VR30M, fps=60, DL only)

{Jitter=[-8,8]ms; additional active time = original DRX On duration}

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all UEs (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aligned CDRX with XR traffic | Aligned every 50ms | 12 | 4 | 11 | 11 | 88.36% | 10.4% |
| Flexible additional active time + aligned CDRX | Aligned every 50ms | 5 | 4 | 11 | 11 | 90% | 30% |

Table 7 FR1 power consumption results in Indoor Hotspot scenario (VR45M, fps=60, DL + pose/control)

{Jitter=[-8,8]ms; additional active time = original DRX On duration}

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aligned CDRX with XR traffic | Aligned every 50ms | 12 | 5 | 7 | 7 | 90% | 7.5% |
| Flexible additional active time + aligned CDRX | Aligned every 50ms | 6 | 5 | 7 | 7 | 88.1% | 15.16% |

Table 8 FR1 power consumption results in Indoor Hotspot scenario (VR45M, fps=60, DL only)

{Jitter=[-8,8]ms; additional active time = original DRX On duration}

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aligned CDRX with XR traffic | Aligned every 50ms | 12 | 5 | 7 | 7 | 90% | 8.47% |
| Flexible additional active time + aligned CDRX | Aligned every 50ms | 6 | 5 | 7 | 7 | 88.1% | 20% |

Table 9 FR1 power consumption results in Indoor Hotspot scenario (CG30M + pose/control)

{Jitter=[-4,4]ms; additional active time = original DRX On duration}

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CDRX alignment | 16 | 6 | 4 | 12 | 12 | 84% | 21.2% |
| Flexible additional active time + aligned CDRX | 16 | 6 | 4 | 12 | 12 | 88.19% | 21.3% |

Table 10 FR1 power consumption results in Indoor Hotspot scenario (CG30M, DL only)

{Jitter=[-4,4]ms; additional active time = original DRX On duration}

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CDRX alignment | 16 | 6 | 4 | 12 | 12 | 84% | 30.9% |
|  | 16 | 8 | 6 | 12 | 12 | 88.89% | 20.6% |
| Flexible additional active time + aligned CDRX | 16 | 6 | 4 | 12 | 12 | 88.19% | 32.4% |

Table 11 FR1 power consumption results in Dense Urban scenario (VR45M, fps=60, DL only, jitter=[-4,4]ms)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aligned CDRX with XR traffic | Aligned every 50ms | 10 | 5 | 7 | 7 | 90.48% | 16% |
| Flexible additional active time + aligned CDRX | Aligned every 50ms | 2 | 5 | 7 | 7 | 91.84% | 26.24% |

Table 12 FR1 power consumption results in Dense Urban scenario (VR45M, fps=60, DL only, jitter=[-8,8]ms)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aligned CDRX with XR traffic | Aligned every 50ms | 14 | 5 | 7 | 7 | 87% | 4.86% |
| Flexible additional active time + aligned CDRX | Aligned every 50ms | 2 | 5 | 7 | 7 | 90.48% | 20.53% |

- Observation 2:

- For FR1, Indoor Hotspot, [-4,4]ms jitter range, CG30M,

  - For DL+UL, the flexible additional active time based on aligned CDRX has 21.3% power saving gain.

  - For DL only, the flexible additional active time based on aligned CDRX has 32.4% power saving gain.

- For FR1, Dense Urban, [-4,4]ms jitter range, VR45M,

  - For DL only, the flexible additional active time based on aligned CDRX has 26.24% power saving gain.

- Observation 3:

- For FR1, Indoor Hotspot, [-8,8]ms jitter range, VR30M

  - For DL+UL, the flexible additional active time based on aligned CDRX has 14.18% power saving gain.

  - For DL only, the flexible additional active time based on aligned CDRX has 30% power saving gain.

- For FR1, Indoor Hotspot, [-8,8]ms jitter range, VR45M,

  - For DL+UL, the flexible additional active time based on aligned CDRX has 15.16% power saving gain.

  - For DL only, the flexible additional active time based on aligned CDRX has 20% power saving gain.

- For FR1, Dense Urban, [-8,8]ms jitter range, VR45M,

  - For DL only, the flexible additional active time based on aligned CDRX has 20.53% power saving gain.

Moreover, since both RAN1 and RAN2 consider jitter handling as high priority issue, we have the following proposals:

Proposal 3:	Capture solutions of jitter handling and evaluation results into TR38.835.

- Proposal 4:  Support flexible additional active time based on current CDRX design, with minor specification change.

In RAN1#110 meeting, companies also studied the PDCCH skipping/ SSSG switching functions. In this way, the extended onDuration or InactivityTimer may be reduced to improve the power saving gain, if the scheduling DCI before the end of the packet transmission is feasible for the indication.

In our contribution, the evaluation results are given in Table 13~14, aligned CDRX with XR traffic periodicity are assumed to be applied for all other power saving schemes. The legacy R17 PDCCH skipping is assumed without further enhancement, such as using non-scheduling DCI. And we assume the PDCCH skipping duration indicated in a scheduling DCI can’t be too large, because the PDCCH skipping may introduce potential retransmission delay scheduling.

Table 13 FR1 power consumption results in Dense Urban scenario (VR45M, fps=60, DL only, jitter=[-4,4]ms)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aligned CDRX with XR traffic | Aligned every 50ms | 10 | 5 | 7 | 7 | 90.48% | 16% |
| Flexible additional active time + aligned CDRX | Aligned every 50ms | 2 | 5 | 7 | 7 | 91.84% | 26.24% |
| Flexible additional active time + aligned CDRX + PDCCH skipping(duration = 2ms) | Aligned every 50ms | 2 | 5 | 7 | 7 | 91.16% | 33.5% |
| PDCCH Skipping (duration = 2ms,4ms) | Aligned every 50ms | 10 | 5 | 7 | 7 | 84% | 24% |

Table 14 FR1 power consumption results in Dense Urban scenario (VR45M, fps=60, DL only, jitter=[-8,8]ms)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aligned CDRX with XR traffic | Aligned every 50ms | 14 | 5 | 7 | 7 | 87% | 4.86% |
| Flexible additional active time + aligned CDRX | Aligned every 50ms | 2 | 5 | 7 | 7 | 90.48% | 20.53% |
| PDCCH skipping (duration = 2ms,4ms) | Aligned every 50ms | 14 | 5 | 7 | 7 | 82% | 9.2% |

We can observe the PSG differences of these power saving schemes w/wo consideration of PDCCH skipping in Table 15. For instance, flexible additional active time has 10.24% PSG over the aligned CDRX. And outperforms PDCCH skipping about 7.84% capacity gain as well as 2.24% PSG. For larger jitter range, e.g. [-8,8] ms, larger gaps can be observed.

In addition, if the PDCCH is miss-detected, the UE would waste power consumption for monitoring unnecessary PDCCHs. What’s worse, only one timer is configured for FR1 or FR2, if a long onDurationTimer in CDRX configuration is configured, it will be problematic for other services in the cell group.

Table 15 Power consumption results w/wo consideration of PDCCH skipping

- Observation 4:   For FR1, Dense Urban, [-4,4]ms jitter range, VR45M DL only,

- flexible additional active time has 26.24% PSG with 91.84% of satisfied UE

- flexible additional active time with PDCCH skipping has 33.5% PSG with 91.16% of satisfied UE

- PDCCH skipping has 24% PSG with 84% of satisfied UE

- Observation 5: 	For FR1, Dense Urban, [-8,8]ms jitter range, VR45M DL only,

- flexible additional active time has 20.53% PSG with 90.48% of satisfied UE

- PDCCH skipping has 9.2% PSG with 82% of satisfied UE

Observation 6:   The limitations of legacy PDCCH skipping should be accounted.

- PDCCH skipping is indicated in a scheduling DCI

- The PDCCH skipping duration may delay potential retransmission scheduling

Dynamic CDRX enhancement

Apart from RRC based mechanisms for handling CDRX enhancements, we think RAN1 can spent more time on remaining work, e.g., dynamic approaches, which basically can address concerns to semi-static methods, such as not robust to varied frame generation rates. Besides, when RAN awareness is enabled at CN side, the generation of video frame can be adjusted according to network situation, thus dynamic CDRX enhancement can be beneficial for, e.g., various video data rate. At last it is also motivated that dynamic mechanism is feasible to address both the periodicity alignment and jitter at the same time.

- Proposal 5:   Dynamic CDRX enhancement deserves further study to at least address concerns about semi-static methods, in case of specific XR applications.

- Dynamic indication to adjust StarOffset of CDRX configuration to align the CDRX cycle with XR traffic

Dynamic signaling can be used for adjustment of CDRX start offset/duration for every set of cycles. In this regard, additional dynamic signaling would be needed for CDRX configuration which is actually semi-static configured in current specification. From UE perspective, the UE needs to monitor the PDCCH to obtain the information update.

- WUS/DCI indication of adjust of the startOffset of CDRX configuration

If a data packet arrives earlier than the DRX onDuration because of jitter impact, the DRX onDuration can be wakened up if WUS is configured. In this way, UE needs to monitor multiple occasions for WUS reception during a time window outside active time. Meanwhile either the start position of a drx-onDurationTimer needs to be changed by WUS, or an addition DRX onDuration is triggered by WUS signal. However, according to the studies, the latency and power saving gain are highly dependent on the design of WUS (e.g., the periodicity of WUS, the signal of the WUS and so on). And in this case, the DRX onDuration can be normal, while WUS may handle the bottleneck of saving energy during the period before the packet arrival.

Table 16 FR1 power consumption results in Dense Urban scenario (VR45M, fps=60, DL only, jitter=[-4,4]ms)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aligned CDRX with XR traffic | Aligned every 50ms | 10 | 5 | 7 | 7 | 90.48% | 16% |
| WUS(start at the beginning of jitter range, WUS is monitored every 1 slot) | Aligned every 50ms | 5 | 5 | 7 | 7 | 90.48% | 24.87% |
| PDCCH Skipping (duration = 2ms,4ms) | Aligned every 50ms | 10 | 5 | 7 | 7 | 84% | 24% |

Table 17 FR1 power consumption results in Dense Urban scenario (VR45M, fps=60, DL only, jitter=[-8,8]ms)

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aligned CDRX with XR traffic | Aligned every 50ms | 14 | 5 | 7 | 7 | 87% | 4.86% |
| WUS(start at the beginning of jitter range, WUS is monitored every 1 slot) | Aligned every 50ms | 5 | 5 | 7 | 7 | 86% | 20.84% |
| PDCCH skipping (duration = 2ms,4ms) | Aligned every 50ms | 14 | 5 | 7 | 7 | 82% | 9.2% |

- Observation 7:   For FR1, Dense Urban, VR45M DL only,

- For [-4,4]ms jitter range, WUS enhancement has 24.87% PSG with 90.48% of satisfied UE

- For [-8,8]ms jitter range, WUS enhancement has 20.84% PSG with 86% of satisfied UE

- Proposal 6:   Dynamic CDRX enhancement can study following aspects:

- For Dynamic indication to adjust StartOffset of CDRX configuration for addressing both issues.

- Signalling indicates to start the additional DRX onDuration.

Rel-17 PDCCH monitoring adaption enhancement applied to XR scenarios

It can be foreseen CDRX technique is basic and technique to be applied in XR services. As discussed above, the enhancements of CDRX can solve both the alignment and jitter handling issues in XR traffic. Compared with Always-On, the enhancement of CDRX technique also provides about 20%-30% PSG which is similar with or even better than the PDCCH skipping technique.

- Observation 8:   CDRX enhancement without PDCCH skipping can provide about 20%-30% PSG.

In Rel-17 PDCCH skipping, RAN1 has no consensus in whether UE resumes monitor of PDCCH during skipping duration when DRX retransmission timer DL/UL is started. One reason is that the latency impact on retransmission can be solved by NW implementation, for example, gNB can indicate with a lower MCS to UE based on the information of traffic QoS. Since low latency is an important requirement for XR traffic, selecting appropriate scheduling parameters can minimize the impact of latency. Therefore, the necessity of PDCCH skipping enhancement needs further discussion.

- Observation 9:   The latency impact of PDCCH skipping on retransmission can be solved by NW implementation.

Coordination between RAN1 and RAN2

The RRC based solutions for matching periodicities of XR traffic and CDRX are discussed as candidates in RAN2 [2], such as non-integer CDRX cycle, multiple CDRX configurations etc. Given the previous study and evaluations in RAN1, comprehensive results have been provided. Thus, in our view, RAN2 can get the low-hanging fruit when consider more specific implementations.

And jitter issue has also been considered in RAN2 [2]. With aforementioned approaches and results, the jitter issue can be resolved completely under CDRX framework, and we think RAN2 can finalize the solution with effort of introducing reasonable specification change.

In addition, regarding how to convey the assistance information, for example, enhanced SR/BSR can be considered to convey UE’s assistance information without introducing additional reporting procedure.

Proposal 7:	RAN1 can send LS to RAN2 to inform the studied CDRX enhancements and evaluation results for matching periodicities of XR traffic and CDRX, handling jitter issue, and the BSR related evaluations.

Conclusion

In this contribution, we discuss the XR-specific power saving techniques and we have the following observations and proposals.

- Observation 1:   The CDRX alignment schemes including non-uniform CDRX cycle, uniform non-integer CDRX cycle, or enhanced multiple CDRX configurations can achieve periodicity alignment between enhanced CDRX and XR traffic.

- For FR1, Indoor Hotspot, [-4,4]ms jitter range, VR30M

  - For DL+UL, the CDRX alignment schemes have 23.85%~24% power saving gain.

  - For DL only, the CDRX alignment schemes have 33.8%~34% power saving gain.

- For FR1, Dense Urban, [-4,4]ms jitter range, VR45M DL only, the CDRX alignment schemes have 16%~23% power saving gain.

- Observation 2:

- For FR1, Indoor Hotspot, [-4,4]ms jitter range, CG30M,

  - For DL+UL, the flexible additional active time based on aligned CDRX has 21.3% power saving gain.

  - For DL only, the flexible additional active time based on aligned CDRX has 32.4% power saving gain.

- For FR1, Dense Urban, [-4,4]ms jitter range, VR45M,

  - For DL only, the flexible additional active time based on aligned CDRX has 26.24% power saving gain.

- Observation 3:

- For FR1, Indoor Hotspot, [-8,8]ms jitter range, VR30M

  - For DL+UL, the flexible additional active time based on aligned CDRX has 14.18% power saving gain.

  - For DL only, the flexible additional active time based on aligned CDRX has 30% power saving gain.

- For FR1, Indoor Hotspot, [-8,8]ms jitter range, VR45M,

  - For DL+UL, the flexible additional active time based on aligned CDRX has 15.16% power saving gain.

  - For DL only, the flexible additional active time based on aligned CDRX has 20% power saving gain.

- For FR1, Dense Urban, [-8,8]ms jitter range, VR45M,

  - For DL only, the flexible additional active time based on aligned CDRX has 20.53% power saving gain.

- Observation 4:   For FR1, Dense Urban, [-4,4]ms jitter range, VR45M DL only,

- flexible additional active time has 26.24% PSG with 91.84% of satisfied UE

- flexible additional active time with PDCCH skipping has 33.5% PSG with 91.16% of satisfied UE

- PDCCH skipping has 24% PSG with 84% of satisfied UE

- Observation 5: 	For FR1, Dense Urban, [-8,8]ms jitter range, VR45M DL only,

- flexible additional active time has 20.53% PSG with 90.48% of satisfied UE

- PDCCH skipping has 9.2% PSG with 82% of satisfied UE

Observation 6:   The limitations of legacy PDCCH skipping should be accounted.

- PDCCH skipping is indicated in a scheduling DCI

- The PDCCH skipping duration may delay potential retransmission scheduling

- Observation 7:   For FR1, Dense Urban, VR45M DL only,

- For [-4,4]ms jitter range, WUS enhancement has 24.87% PSG with 90.48% of satisfied UE

- For [-8,8]ms jitter range, WUS enhancement has 20.84% PSG with 86% of satisfied UE

Observation 8:   CDRX enhancement without PDCCH skipping can provide about 20%-30% PSG.

- Observation 9:   The latency impact of PDCCH skipping on retransmission can be solved by NW implementation.

Proposal 1:	Capture the above evaluation results of solutions of matching periodicities between XR traffic and CDRX into TR 38.835.

Proposal 2:	RAN1 can recommend at least the studied RRC-based solutions for enhancement of CDRX to align with XR traffic periodicity for normative phase.

Proposal 3:	Capture solutions of jitter handling and evaluation results into TR.38.835.

- Proposal 4:  Support flexible additional active time based on current CDRX design, with minor specification change.

- Proposal 5:   Dynamic CDRX enhancement deserves further study to at least address concerns about semi-static methods, in case of specific XR applications.

- Proposal 6:   Dynamic CDRX enhancement can study following aspects:

- For Dynamic indication to adjust StartOffset of CDRX configuration for addressing both issues.

- Signalling indicates to start the additional DRX onDuration.

Proposal 7:	RAN1 can send LS to RAN2 to inform the studied CDRX enhancements and evaluation results for matching periodicities of XR traffic and CDRX, handling jitter issue, and the BSR related evaluations.

References

RP-220285, Study on XR Enhancements for NR.

RAN2#119-e Chair Notes.

R1-2207061, Evaluation on XR specific power saving techniques, ZTE, Sanechips, RAN1#110.

R1-2203606, Discussion on XR specific power saving techniques, ZTE, Sanechips, RAN1#109-e.

- Appendix A

Simulation parameters and power model adopted in our simulation refer to [4], and the assumptions are in line with TR 38.838.

In our contribution, evaluations of power consumption in terms of both joint UL&DL and DL only are provided.

- Appendix B

Agreement and conclusions in RAN1#110

Agreement

RAN1 recommends identifying a solution for enhancement of CDRX to align with XR traffic periodicity

Conclusion

Conclude that “SFN wraparound mismatch” is a RAN2 issue. It can be left to RAN2 to address. RAN1 does not further study it.

Conclusion

RAN1 does not assume instantaneous jitter value for a frame is predictable for Rel-18 XR SI power saving study before further input is provided by SA.

Conclusion

All the proposed PDCCH monitoring adaptation/reduction schemes including those for jitter handling need to be compared against the Rel-17 PDCCH monitoring adaptation which is to be used as performance reference.

Conclusion

UE transmission and reception alignment for Issue 3-1 is deprioritized for power saving in Rel-18 XR SI.

Conclusion

RAN1 does not assume dynamic switch of different XR video data rates or frame rates for Rel-18 XR power saving study before further input is provided by SA.

For future meetings

Companies are encouraged to account the enhancement of CDRX to align with XR traffic periodicity in their further evaluations for XR power saving enhancements.

Conclusion:

Companies are requested to use the Excel sheet attached with TR 38.838 in RP-213652 for recording the simulation results that are provided in their contributions.