3GPP TSG RAN WG1 Meeting #110bis-e			    	R1-2209236

e-Meeting, October 10th – 19th, 2022

Source 	: CAICT

Title 	: Considerations on channel access mechanism of SL-U

Agenda Item	: 9.4.1.1

Document for	: Discussion and decision

Backgrounds

In last meeting, the following agreements have been reached:

Agreement - CW adjustment - NR-U DL CW adjustment mechanism is used as the baseline for SL-U when SL-HARQ feedback is enabled in SCI for unicast - FFS any necessary update for SL-U operation - FFS: how to determine CW size when SL-HARQ feedback is disabled in SCI FFS the case of groupcast option 1 (NACK-only) and groupcast option 2 Agreement - Type 2A/2B/2C SL channel access procedures - Type 2A channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE for a gap ≥ 25μs in a shared channel occupancy - FFS any other transmission by a UE (e.g., other than COT sharing) - FFS whether Type 2A is used also for the case of short control signalling transmission - Type 2B channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE at least when the gap is 16μs in a shared channel occupancy - FFS the case when the gap is between 16 and 25us - FFS any other transmission by a UE (e.g., other than COT sharing) - Type 2C channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE for a gap ≤ 16μs in a shared channel occupancy and the duration of the corresponding transmission is at most 584us. - FFS any other transmission by a UE (e.g., other than COT sharing) - FFS whether Type 2C is used also for the case of short control signalling transmission - FFS under which conditions (other than the gap) UEs can apply the Type 2A/2B/2C SL channel access procedures - FFS under which conditions Type 2B or Type 2C is applied in case of a gap of 16 μs Agreement Multi-consecutive slots transmission (MCSt) is supported for Mode 1 and Mode 2 resource allocation in SL-U. - FFS details Agreement - For UE-to-UE COT sharing, continue considering the following alternatives: - Alt. 1: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the at least COT initiating UE’s PSSCH data transmission in the COT. - When the responding UE uses the shared COT for its transmission has an equal or smaller CAPC value than the CAPC value indicated in a shared COT information - FFS any additional conditions - Alt. 2: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the COT initiating UE’s transmission in the COT. - When the responding UE uses the shared COT for its transmission has an equal or smaller CAPC value than the CAPC value indicated in a shared COT information - FFS how to determine a SL UE is a target receiverFFS: details of the channel type of the COT initiating UE’s transmission - FFS any additional conditions - For Alt1 and Alt2: When a responding UE uses a shared COT for its transmission(s), the COT initiating UE is a target receiver of the responding UE’s transmission(s). - FFS: details of the channel type of the responding UE’s transmission(s) - gNB relaying/forwarding a UE initiated COT to another UE is not supported in Rel-18 - FFS whether a Mode 1 UE can report a COT or related information to gNB for aiding Mode 1 RA

In this document, we will present our considerations on FFS issues including CW adjustment and COT sharing. Observations and proposals are provided accordingly.

Discussions

CW Adjustment

About how to determine CW size when SL-HARQ feedback is disabled in SCI, in our view, it is applicable and beneficial to adapt the CW size based on the channel busy ratio (CBR) measurement results. When the measured channel is busy, UE can enlarge the CW size to reduce the channel collision possibility. And UE can reduce CW size to access the channel more quickly if the measured CBR level is low. The CBR measurement is available for current SL UE andusually used for channel congestion control to determine the transmission parameters including transmit power, maximum retransmission time, maximum and minimum MCS, subchannel number limit and CR limit. We can configure CW size as one of transmission parameters in channel congestion control which are also associated to the transmission priority or access category level to determine the CW size in case of disabled SL-HARQ feedback.

Proposal 1: Propose to configure CW size as one of transmission parameters in channel congestion control based on CBR measurement, associated to the transmission priority or access category level to determine the CW size in case of disabled SL-HARQ feedback.

As for the groupcast scenario, it is obviously that the reliability of the HARQ feedback would be strongly affected by the unexpected channel access states for PSFCH transmissions of a number of groupcast receivers due to LBT. Therefore, the HARQ feedback mechanism might be enhanced for groupcast, which is under discussion in the parallel item 9.4.1.2. We suggest to discuss the CW size determination method for groupcast with HARQ feedback after the HARQ feedback mechanism enhancement is well discussed. In addition, if the reliability of HARQ feedback for groupcast is unaccepted, then we can adopt CW size determination method for transmissions without HARQ feedback to groupcast with HARQ feedback.

Proposal2: It is suggested to discuss the CW size determination method for groupcast with HARQ feedback after the HARQ feedback mechanism enhancement is well discussed.

COT Sharing

In order to enable SL-U UE to access channel more efficiently, UE-to-UE COT sharing is supported, for which a UE can access the channel with COT sharing initiated by another UE. More details to apply UE-to-UE COT sharing need to be discussed and specified.

COT Sharing in Mode 1 RA

For Mode 1 RA for which the SL resources are allocated by gNB, we notice that if a SL UE is not indicated of the information about LBT type, inter-UE blocking would happen as shown in Fig.1, which should be avoided. This issue could be solved through UE-UE COT sharing and gNB-UE COT sharing, assuming that gNB can be aware of the channel state of SL through sensing or assistant information from UE. Therefore, we suggest to support gNB-UE COT sharing or UE-UE COT sharing for Mode1 RA. If UE-UE COT sharing is supported here, the mechanism should be designed same as the case of Mode 2 RA.

FIG. 1 Inter-UE blocking issue in Mode 1 RA

Proposal3: In order to avoid inter-UE blocking issue, it is suggested to introduce gNB-UE COT sharing or UE-UE COT sharing for Mode 1 RA, where the designed UE-UE COT sharing can also be applied to Mode 2 RA.

Responding UE for COT Sharing

For a UE initiating COT sharing, its purpose is to receive data from other UE with quick channel access and to ensure its reserved resource for the next transmission period, as shown in Fig.2. Actually, a UE initiating COT sharing cannot guarantee that the shared COT can be occupied by the UE which is the target receiver of COT initiating UE’s PSSCH transmission (Alt1) or any possible transmissions (Alt2). Moreover, if UE-UE COT sharing is introduced in Mode1 RA, we think the COT sharing among UEs depends on the resource position determined by gNB, which means that a responding UE could not be the target receiver of COT initiating UE’s transmission. Therefore, if one UE-UE COT sharing mechanism is introduced for both Mode 1 RA and Mode 2 RA, we suggest to support the responding UE to share the COT could be not the target receiver of the COT initiating UE’s transmission. In that case, if the destination ID is not indicated for COT sharing, any UEs who receive the COT sharing information could attempt to access the channel with the shared COT.

FIG. 2 UE-UE COT sharing could to ensure quick channel access for the reserved resource in the next transmission period.

Obervation1: A UE initiating COT sharing cannot guarantee that the shared COT can be occupied by the UE which is the target receiver of COT initiating UE’s PSSCH transmission (Alt1) or any possible transmissions (Alt2).

Proposal 4: if one UE-UE COT sharing mechanism is introduced for both Mode 1 RA and Mode 2 RA, we suggest to support the responding UE to share the COT could be not the target receiver of the COT initiating UE’s transmission.

Cast Type for COT Sharing

In our observation, the cast type for COT sharing should be considered. For a UE initiating the COT sharing, it can share its COT to one target UE by unicast or to a number of UEs by groupcast. And the information content for COT sharing in unicast and groupcast might be different. For example, for unicast COT sharing, it might need to indicate the time and frequency resources for the responding UE’s data transmission. For groupcast COT sharing, it might aim to indicate channel access type for the PSFCH transmissions of the receivers of COT initiating UE’s PSSCH transmission. And the time and frequency resources in this case might not be required. Therefore, we suggest to study the cast type for UE-UE COT sharing before determining the information content for COT sharing.

Obervation2: For a UE initiating the COT sharing, it can share its COT to one target UE by unicast or to a number of UEs by groupcast.

Proposal 5: It is suggested to study the cast type for UE-UE COT sharing before determining the information content for COT sharing.

Conclusions

In this contribution, we have presented our considerations on the FFS issues including CW adjustment and COT sharing, and put forward the following observations and proposals:

Proposal 1: Propose to configure CW size as one of transmission parameters in channel congestion control based on CBR measurement, associated to the transmission priority or access category level to determine the CW size in case of disabled SL-HARQ feedback.

Proposal2: It is suggested to discuss the CW size determination method for groupcast with HARQ feedback after the HARQ feedback mechanism enhancement is well discussed.

Proposal3: In order to avoid inter-UE blocking issue, it is suggested to introduce gNB-UE COT sharing or UE-UE COT sharing for Mode 1 RA, where the designed UE-UE COT sharing can also be applied to Mode 2 RA.

Obervation1: A UE initiating COT sharing cannot guarantee that the shared COT can be occupied by the UE which is the target receiver of COT initiating UE’s PSSCH transmission (Alt1) or any possible transmissions (Alt2).

Proposal 4: if one UE-UE COT sharing mechanism is introduced for both Mode 1 RA and Mode 2 RA, we suggest to support the responding UE to share the COT could be not the target receiver of the COT initiating UE’s transmission.

Obervation2: For a UE initiating the COT sharing, it can share its COT to one target UE by unicast or to a number of UEs by groupcast.

Proposal 5: It is suggested to study the cast type for UE-UE COT sharing before determining the information content for COT sharing.

References

- RP-220300, WID revision: NR sidelink evolution, OPPO, March 2022.

- 3GPP RAN1#110, RAN1 Chair’s Notes, Aug. 2022.

- R1-2207793 3GPP RAN1#110, FL summary for AI 9.4.1.1: SL-U channel access mechanism, Moderator (OPPO), Aug. 2022.