3GPP TSG RAN WG1 #110bis-e                                       	       R1-2209256

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.1.1.1

Source:	Xiaomi

Title:                     Unified TCI framework extension for multi-TRP

Document for:	Discussion and Decision

Introduction

The WI for R18 MIMO was approved in RAN-94 e-meeting, and the detailed objectives [1] are as follows:

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows:

  - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis

  - UE reporting of time-domain channel properties measured via CSI-RS for tracking

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework.

- Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM,

  - Striving for a common design between DL and UL DMRS

  - Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable)

  - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission

    - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

  - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation

    - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

- Study, and if justified, specify the following

  - Two TAs for UL multi-DCI for multi-TRP operation

  - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

While in this contribution, we will focus on the unified TCI framework extension for M-TRP, which including the object 2, the second bullet of objective 6 and the first bullet of objective 7.

Indication of multiple DL/UL TCI states

Switching between joint or separate DL/UL TCI state configuration

During the discussion of R17 unified TCI state framework, there is a conclusion in RAN1-105 e-meeting [2] seen below. It concludes that the configuration of joint TCI or separate DL/UL TCI state is based on RRC signaling. While for M-TRP scenario, since different TRP may experience different environment, thus separate configuration of joint TCI or separate DL/UL TCI for different TRP is preferred. It means that it is possible that one TRP is configured with joint TCI and the other one is configured with separate DL/UL TCI.

Conclusion

On Rel-17 unified TCI framework, for a UE configured with both joint TCI and separate DL/UL TCI, configuration of joint TCI or separate DL/UL TCI is based on RRC signaling

- There is no consensus in RAN1 on how to support dynamic switching (either MAC-CE or codepoint based)

Proposal 2-1: For M-TRP unified TCI framework, support one TRP is configured with joint TCI and the other one is configured with separate DL/UL TCI.

In addition to UL to micro cell and DL/UL only TRP, separate DL/UL TCI state will be used in the case of MPE issue and the PHR related to MPE issue is indicated by MAC CE [3], thus the configuration of joint TCI or separate DL/UL TCI state is based on RRC signaling can’t handle the MPE issue with short latency, which will introduce some performance loss by using non-suitable UL TCI state. While in order to reduce UE complexity, DCI based switching is not recommended.

Proposal 2-2: For M-TRP unified TCI framework, support to switch between joint TCI and separate DL/UL TCI based on MAC CE.

Unified TCI state indication in S-DCI case

TCI state mapping/association for PDCCH in S-DCI case

For PDCCH reception, the agreement in RAN1-109 e-meeting [4] and RAN1-110 meeting [5] can be seen as below.

Agreement

On unified TCI framework extension for S-DCI based MTRP, consider at least the following alternatives to map/associate a joint/DL TCI state to PDCCH reception(s)

- Atl1: Use RRC configuration to inform the mapping/association between a configured or indicated joint/DL TCI state and a CORESET or a CORESET group

- Alt2: Use RRC configuration to inform the mapping/association between a configured or indicated joint/DL TCI state and a search space set

- Alt3: Use MAC-CE to inform the mapping/association between an activated or indicated joint/DL TCI state and a CORESET or a CORESET group

- Alt4: Use DCI to inform the mapping/association between an indicated joint/DL TCI state and a CORESET or a CORESET group

- Alt5: Based on a fixed mapping/association rule, e.g., the first indicated joint/DL TCI state always applies to PDCCH receptions

Consider above alternatives for PDCCH repetition, PDCCH-SFN, PDCCH w/o repetition/SFN, and potential support of dynamic switching between S-TRP and M-TRP for PDCCH. It is not precluded to adopt one single alternative or multiple alternatives to support these cases.

Agreement

On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, down-selection at least one alternative from the followings:

- Alt1-1: Use RRC parameter(s) in a CORESET configuration to inform the UE whether and/or which indicated joint/DL TCI state(s) shall be applied to the corresponding PDCCH receptions on the CORESET

  - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE

- Alt1-2: Use an RRC parameter in a CORESET configuration to inform that the CORESET belongs to which CORESET group(s), and the indicated joint/DL TCI state(s) is associated with each CORESET group

  - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the CORESET group(s)

  - FFS: How to associate the indicated joint/DL TCI state(s) with each CORESET group

  - FFS: The UE applies the indicated joint/DL TCI state(s) to a CORESET according to the CORESET group(s) the CORESET belongs to, or the UE applies the indicated joint/DL TCI state(s) associated with the CORESET group(s) in which the beam indication DCI is received to all PDCCH receptions

- Alt2: The association between a CORESET and the indicated joint/DL TCI state(s) is determined based on a fixed rule, and the UE shall apply the indicated joint/DL TCI state(s) to the corresponding PDCCH receptions on the CORESET

  - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE

- Alt3: Use MAC-CE to inform the UE whether and/or which indicated joint/DL TCI state(s) shall be applied to the corresponding PDCCH receptions on a CORESET

  - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE

Switching between multi-TRP and single TRP operation is not precluded

When at least one channel will be transmitted by M-TRP, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) should indicate TCI states of M-TRP. But for each type of channel, independent additional signaling will be used for indication of a part or all of indicated TCI states.

And the additional signaling can be different for different type of channel. For PDCCH SFN, MAC CE is used to activate one or two TCI states of the CORESET in Rel-17. Also for PDCCH repetition, the TCI state of corresponding CORESETs for two associated SS sets are activated by MAC CE. If different TCI states are activated for two CORESETs, it means PDCCH repetition by M-TRP. If same TCI state is activated for two CORESETs, it means PDCCH repetition by S-TRP.  In one word, dynamic switching between S-TRP and M-TRP for PDCCH can be realized by MAC CE in Rel-17. Thus it is straightforward to use MAC CE in Rel-18 to support dynamic switching between S-TRP and M-TRP for PDCCH repetition/SFN. For example, for PDCCH repetition, if MAC CE associate same one of two indicated TCI states for both two SS sets or both two CORESETs, S-TRP transmission will be assumed. If MAC CE associate different indicated TCI states for two SS sets or two CORESETs, M-TRP transmission will be assumed. While for PDCCH SFN, if MAC CE associate one of the two indicated TCI states to the CORESET, S-TRP transmission will be assumed. If MAC CE associate two indicated TCI states to the CORESET, M-TRP transmission will be assumed.

Proposal 2-3: For PDCCH repetition/SFN, support Alt 3, use MAC-CE to inform the mapping/association between an activated or indicated joint/DL TCI state and a CORESET.

While for PDCCH without repetition/SFN, there are two ways to inform the mapping/association between a configured or indicated Joint/DL TCI state and a CORESET. The first one is to reuse the MAC CE for PDCCH repetition/SFN, in this case, unified framework can be archived. And MAC CE can indicate the TCI state per CORESET. The second one is to use different mechanism, e.g.,  use RRC signaling for PDCCH without repetition/SFN to inform the mapping/association between a configured or indicated Joint/DL TCI state and a CORESET. To be detail, RRC signaling can be used to inform the mapping/association per CORESET or per CORESET group.

Proposal 2-4: For PDCCH without repetition/SFN, down select from Alt 1-1, Alt 1-2 and Alt 3:

- Alt 1-1: use RRC configuration to inform the mapping/association between a configured or indicated joint/DL TCI state and a CORESET.

- Alt 1-2: use RRC configuration to inform the mapping/association between a configured or indicated joint/DL TCI state and a CORESET group.

- Alt 3: use MAC CE to inform the mapping/association between an activated or indicated joint/DL TCI state and a CORESET.

If the mapping/association between TCI states and a CORESET is informed per CORESET group, it is necessary to configure a CORESET group ID for each CORESET since there is no CORESETPoolIndex for S-DCI M-TRP scenario. In order to solve this problem, we suggest to configure a CORESET group ID for each CORESET in S-DCI M-TRP scenario and indicate the association between CORESET group ID and the indicated TCI state.

Proposal 2-5: For S-DCI M-TRP scenario, support to configure CORESET group ID for each CORESET.

In addition, for a CORESET configured with SFN, two TCI states can be activated/indicated for such CORESET. If only one CORESET group ID is configure for such CORESET, at most 3 CORESET group ID is needed. Which means for CORESET group ID#0 and #1, one TCI state will be activated/indicated. While for CORESET group ID#2, up to two TCI states can be activated/indicated. In order to align with the maximum number of CORESETPoolindex, two CORESET group ID is another alternative. In this case, two CORESET group ID will be configured for CORESET configured with SFN, and one TCI state will be activated/indicated for each CORESET group ID.

Proposal 2-6: For CORESET group ID configuration for CORESET with PDCCH SFN in S-DCI M-TRP scenario, consider following two alternatives.

- Alt 1: configure CORESET group ID#2 to CORESET with SFN, and up to two TCI states can be activated/indicated for CORESET group ID#2.

- Alt 2: configure CORESET group ID#0 and CORESET group ID#1 to CORESET with SFN, and one TCI state can be activated/indicated for each CORESET group.

TCI state mapping/association for PDSCH in S-DCI case

For PDSCH reception, following are some possible solutions discussed in RAN1-110 meeting. As for Alt 1, DCI format 1_1/1_2 without DL assignment is not supported. For Alt 3, it can’t support M-TRP PDCCH/PUCCH/PUSCH and S-TRP PDSCH. For Alt 4, dynamic switching between S-TRP and M-TRP PDSCH can’t support. For Alt 5, we are confusing how to support M-TRP with this mechanism since it is S-DCI based MTRP, which means DCI will be transmitted from only one CORESET group. Based on above analysis, we support the following proposal and prefer Alt 2. Since with Alt 2, the mapping/association can be informed by DCI format 1_1/1_2 without DL assignment. If it can only be informed by DCI format 1_1/1_2 with DL assignment, it is also necessary to define a default behavior when the gap between DCI and PDSCH is less than timeDurationForQCL as well as the PDSCH scheduled by DCI format 1_0. But if it can be informed by both DCI format 1_1/1_2 with and without DL assignment, the mapping/association can be valid until new mapping/association is informed, which is similar to indicated DLorJointTCIState and UL-TCIState. In addition, if DCI format 1_1/1_2 with DL assignment can support to inform the mapping/association, it is straightforward to support this function in DCI format 1_1/1_2 without DL assignment.

Proposal 3.B: On unified TCI framework extension for S-DCI based MTRP, to inform the association with joint/DL TCI state(s) indicated by DCI/MAC-CE and enable dynamic switching between STRP and MTRP operations for PDSCH reception, down-selection at least one alternative from the followings:

- Alt1: Use an indicator field other than the existing TCI field (could be reusing an existing DCI field or introducing a new DCI field) in a DCI format 1_1/1_2 with DL assignment to inform which indicated joint/DL TCI state(s) the UE shall apply to PDSCH reception scheduled/activated by the DCI format 1_1/1_2

  - FFS: Joint/DL TCI state(s) applying to PDSCH reception scheduled/activated by DCI format 1_0

- Alt2: Use an indicator field other than the existing TCI field (could be reusing an existing DCI field or introducing a new DCI field) in a DCI format 1_1/1_2 with and without DL assignment to inform which indicated joint/DL TCI state(s) the UE shall apply to all PDSCH receptions after an application time

  - FFS: Detail of the application time

- Alt3: Reuse the existing TCI field in a DCI format 1_1/1_2, i.e., the UE shall apply the joint/DL TCI state(s) mapped to the TCI codepoint indicated by the DCI format 1_1/1_2 to PDSCH reception scheduled/activated by the DCI format 1_1/1_2 if the PDSCH reception is scheduled/activated after the beam application time as defined in Rel-17

- Alt4: Use RRC parameter(s) in a PDSCH configuration in a DL BWP to inform which indicated joint/DL TCI state(s) the UE shall apply to PDSCH reception in the DL BWP

- Alt5: Use an RRC parameter in a CORESET configuration to inform that the CORESET belongs to which CORESET group(s), and the indicated joint/DL TCI state(s) is associated with each CORESET group. When a scheduling/activation DCI with DL assignment is received in a CORESET group, the indicated joint/DL TCI state(s) associated with the CORESET group is applied to PDSCH reception scheduled/activated by the scheduling/activation DCI.

Proposal 2-7: Support to use an indicator field other than the existing TCI field (could be reusing an existing DCI field or introducing a new DCI field) in a DCI format 1_1/1_2 with and without DL assignment to inform which indicated joint/DL TCI state(s) the UE shall apply to all PDSCH receptions after an application time.

TCI state mapping/association for PUCCH in S-DCI case

For Rel-17 S-DCI based M-TRP PUCCH repetition scheme, dynamic switching between S-TRP and M-TRP is supported based on MAC CE to activate one or two spatial-relation-info. Thus it is straightforward to use MAC CE to inform the UE which indicated joint/UL TCI state(s) should apply to PUCCH resource configured with TDM scheme. While for S-TRP PUCCH, it can reuse the MAC CE for PUCCH TDM scheme based on MAC CE to inform which indicated joint/UL TCI state(s) should apply to each PUCCH resource, or use RRC signaling to inform the mapping/association between a configured or indicated joint/UL TCI state and PUCCH resource/group and a PUCCH resource group ID should be configured to each PUCCH resource for TCI state mapping/association.

According to the agreement in RAN1-110 meeting, four alternatives are listed. Based on our analysis above, Alt 3 is preferred for PUCCH repetition and Alt 1 or Alt 3 is preferred for S-TRP PUCCH.

Agreement

On unified TCI framework extension for S-DCI based MTRP, to inform the association with joint/UL TCI state(s) indicated by DCI/MAC-CE for PUCCH transmission, down-selection at least one alternative from the followings:

- Alt1: Use RRC configuration to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/ group

- Alt2: Use RRC configuration to inform the association between a CORESET group and a PUCCH resource/group, and the indicated joint/UL TCI state(s) associated with the CORESET group applies to the PUCCH resource/group

- Alt3: Use MAC-CE to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group

- Alt4: Use DCI to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group

Proposal 2-8: On unified TCI framework extension for S-DCI based MTRP, to inform the association with joint/UL TCI state(s) indicated by DCI/MAC-CE for PUCCH transmission:

- For M-TRP PUCCH repetition, support Alt 3, i.e., use MAC CE to map/associate one or two indicated joint/UL TCI state to a PUCCH resource/PUCCH resource group.

- For S-TRP PUCCH, support Alt 1 or Alt 3, i.e., use MAC CE or RRC configuration to map/associate one or two indicated joint/UL TCI state to a PUCCH resource/PUCCH resource group.

TCI state mapping/association for PUSCH in S-DCI case

For Rel-17 DG M-TRP PUSCH repetition scheme, dynamic switching between S-TRP and M-TRP is supported based on SRS resource set indicator in DCI format 0_1/0_2. While for M-TRP unified TCI framework, SRS resource set indicator in DCI can be reused to inform the UE which indicated joint/UL TCI state(s) should apply to PUSCH by the scheduling DCI.

According to the agreement in RAN1-110 meeting, three alternatives are listed for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2. With Alt 2, the he spatial domain transmission filter(s) used for the SRS resource(s) can be different from indicated TCI states, which is not aligned with the motivation of unified TCI state. With Alt 3, dynamic switching for S-TRP and M-TRP PUSCH can’t be supported for PUSCH only. Based on our analysis above, Alt 1 is preferred.

Agreement

On unified TCI framework extension for S-DCI based MTRP, for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2, down-selection one alternative from the followings:

- Alt1: Use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in a DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2

- Alt2: PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2

- Alt3: Use an RRC parameter in a CORESET configuration to inform that the CORESET belongs to which CORESET group(s), and the indicated joint/UL TCI state(s) is associated with each CORESET group. When a scheduling/activation DCI format 0_1/0_2 is received in a CORESET group, the indicated joint/UL TCI state(s) associated with the CORESET group is applied to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2

  - FFS: Details of CORESET group(s)

FFS: PUSCH transmission scheduled/activated by a DCI format 0_0 and Type-1 CG-PUSCH

Proposal 2-9: On unified TCI framework extension for S-DCI based MTRP, for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2:

- Support Alt 1, i.e., Use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in a DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2

For Type 1 CG M-TRP PUSCH repetition scheme, for retransmission, similar mechanism for DG PUSCH repetition scheme can be used. Otherwise, RRC configuration is preferred to inform the mapping/association between a configured or indicated joint/UL TCI state and SRS resource set. In addition, the SRS resource set is determined by the number of field for ‘pathlossReferenceIndex’, 'srs-ResourceIndicator', 'precodingAndNumberOfLayers', 'p0-PUSCH-Alpha' and 'powerControlLoopToUse'. If only one field for above indicator, the first indicated joint/UL TCI state will be applied. Else, both two indicated joint/UL TCI state will be applied.

Proposal 2-10: On unified TCI framework extension for S-DCI based MTRP, for Type 1 CG PUSCH repetition, support to use RRC configuration or default rule to inform the mapping/association.

For PUSCH transmission scheduled/activated by a DCI format 0_0, there are some alternatives to inform the mapping/association between indicated joint/UL TCI state(s) and PUSCH transmission scheduled/activated by the DCI format 0_0.

- Alt 1: The TCI state of PUSCH transmission scheduled/activated by the DCI format 0_0 is same as that of the CORESET with the scheduling PDCCH.

- Alt 2: Based on default rule that use the first TCI state of the indicated joint/UL TCI state(s).

- Alt 3: Use MAC CE to inform the mapping/association between indicated joint/UL TCI state(s) and PUSCH transmission scheduled/activated by the DCI format 0_0.

- Alt 4: Based on the mapping/association informed by the latest DCI format 0_1/0_2.

Proposal 2-11: On unified TCI framework extension for S-DCI based MTRP, for PUSCH transmission scheduled/activated by a DCI format 0_0, down-selection one alternative from the followings

- Alt 1: The TCI state of PUSCH transmission scheduled/activated by the DCI format 0_0 is same as that of the CORESET with the scheduling PDCCH.

- Alt 2: Based on default rule, e.g., use the first TCI state of the indicated joint/UL TCI state(s).

- Alt 3: Use MAC CE to inform the mapping/association between indicated joint/UL TCI state(s) and PUSCH transmission scheduled/activated by the DCI format 0_0.

- Alt 4: Based on the mapping/association informed by the latest DCI format 0_1/0_2.

Application time for TCI state mapping/association information

For activated TCI state by MAC CE, the application time is the first slot that is after slot n+3Nslotsubframe,µ , where n is the slot for a PUCCH with HARQ-ACK information corresponding to the activation command. For TCI state of PDSCH in Rel-15/16, the application time is timeDurationForQCL after the scheduling DCI. For indicated joint/DL/UL TCI state in Rel-17, the application time is BeamAppTime_r17 after the last symbol of a PUCCH or PUSCH with HARQ-ACK information to the DCI carrying the TCI State indication and without DL assignment, or corresponding to the PDSCH scheduled by the DCI carrying the TCI State indication.

Based on the above analysis, it is possible to use MAC CE or DCI to inform the mapping/association between an indicated joint/DL/UL TCI state and each channel or to inform which indicated joint/DL/UL TCI state should apply to each channel. Thus it is also necessary to define the application time for the TCI state mapping/association information. If it is in the same MAC CE or DCI with the indicated joint/DL/UL TCI state, the application time can be same as the indicated joint/DL/UL TCI state. Else, independent application time should be defined. For PDCCH/PUCCH, we prefer to use MAC CE to inform the TCI state mapping/association information, the application time can be 3ms after the HARQ-ACK corresponding to the MAC CE. For PDSCH/PUSCH, we prefer to use DCI to inform the TCI state mapping/association information. The application time can be timeDurationForQCL after the scheduling DCI.

Proposal 2-12: The application time for TCI state mapping/association information based on MAC CE or DCI should be defined.

Unified TCI state indication in M-DCI case

In RAN1-109 e-meeting [4], the agreement for M-DCI based MTRP can be seen as below.

Agreement

On unified TCI framework extension for M-DCI based MTRP, consider the following alternatives for TCI state update:

- Alt1: Reuse the same TCI state update scheme for S-DCI based MTRP

- Atl2: Use the existing TCI field in the DCI format 1_1/1_2 (with or without DL assignment) associated with one of CORESETPoolIndex values to indicate the joint/DL/UL TCI state(s) corresponding to the same CORESETPoolIndex value

- Alt3: Use the existing TCI field in any DCI format 1_1/1_2 (with or without DL assignment) to indicate all joint/DL/UL TCI states corresponding to both CORESETPoolIndex values

  - Study the association between the indicated joint/DL/UL TCI state(s) and a CORESETPoolIndex value

- Alt4: Use the existing TCI field in the DCI format 1_1/1_2 (with or without DL assignment) associated with one of CORESETPoolIndex values to indicate joint/DL/UL TCI state(s) corresponding to the same or different CORESETPoolIndex value.

  - Study whether the indicated joint/DL/UL TCI state(s) applies to the channels/signals associated with the same CORESETPoolIndex value or different CORESETPoolIndex value is indicated by DCI

In R17, both MAC CE and MAC CE+DCI can be used for unified TCI state indication. Thus it is straightforward to support both mechanism for M-TRP.

For different TRP, different CORESETPoolIndex is configured for CORESET. And unified TCI state for CORESETPoolIndex will be applied for the CORESET with same CORESETPoolIndex. While for the transmission scheduled by DCI, such as PDSCH/PDSCH/dynamic grant PUSCH type 2/ aperiodic PUCCH, the indicated unified TCI state of the corresponding scheduling PDCCH will be applied.

Proposal 2-13: For M-DCI M-TRP scenario, for channel /signal transmission scheduled by DCI, the unified TCI state of the corresponding scheduling CORESET will be applied.

While for transmissions not scheduled by DCI, such as periodic/ semi-persistent PUCCH /configured grant PUSCH type 1, it needs to define that the TCI state of which one TRP will be applied. The straightforward method is to configure a group ID for periodic/ semi-persistent PUCCH /configured grant PUSCH type 1. And associate TCI state of each TRP with each group ID.

Proposal 2-14: For M-DCI M-TRP scenario, configure CORESETPoolIndex to non-DCI indicated PUCCH and Type 1 CG PUSCH.

In addition, for M-DCI M-TRP scenario, if unified TCI state is indicated by MAC CE+DCI, it is straightforward to indicate the unified TCI state for the CORESETPoolIndex by the CORESET with same CORESETPoolIndex. But if for CORESET#1, the performance of previous TCI state becomes bad, it may be necessary to support cross CORESETPoolIndex indication for unified TCI state. It means that indicate the unified TCI state for CORESETPoolIndex #0 by CORESET #1 and CORESET#1 is configured with CORESETPoolIndex#1.

Proposal 2-15: For M-DCI M-TRP scenario, support Alt 4.

- Alt4: Use the existing TCI field in the DCI format 1_1/1_2 (with or without DL assignment) associated with one of CORESETPoolIndex values to indicate joint/DL/UL TCI state(s) corresponding to the same or different CORESETPoolIndex value.

Unified TCI state indication for CJT

As for CJT, up to 4 TRP can be supported. But supportive of up to 4 TCI states will increase UE complexity and introduce additional spec impact other than unified TCI state extension for M-TRP. Thus we prefer to support up to 2 TCI states for CJT. As for the QCL type, we think the QCL parameters for SFN-PDSCH can be reused. That means {average delay, delay spread, Doppler shift, Doppler spread} can be considered. If there are more than 2 sets of {Doppler shift, Doppler spread} for N coordinated TRP, gNB side pre-compensation of SFN-PDSCH can be reused.

Proposal 2-16: For CJT, support up to 2 TCI states.

Simultaneous multi-panel UL transmission

The Rel-18 WID for MIMO Evolution for Downlink and Uplink is approved [1], which includes the following objective mainly targeting UL enhancements:

Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable)

  - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission

- The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

  - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation

    - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

In Rel-16 mTRP enhancements, the focus was mostly on PDSCH reliability enhancements, while in Rel-17 the enhancements for PDCCH, PUCCH, and PUSCH was taken to ensure that both data and control for both downlink and uplink are reliable as  the overall reliability of the system. For the uplink mTRP enhancements alone, in Rel-17 the transmission schemes for PUSCH and PUCCH are TDM based solutions considering the UE capabilities in FR2. PUSCH or PUCCH is transmitted on multiple repetitions in time-domain hence making these schemes suitable for UEs that can transmit on a single panel at a time. In Rel-18, the uplink mTRP enhancements mainly consider the simultaneous transmissions with multiple UE panels in FR2. In the multi-panel transmission case, a UE has the capability to simultaneously transmit over multiple panels. Compared to single panel transmission case, the main benefit is the reduced latency and higher throughput. And simultaneous UL transmission specified in R18 would include PUSCH/PUCCH for single DCI based operation and PUSCH+PUSCH and PUCCH+PUCCH for multi-DCI based operation.

The following agreement was made to consider involve STxMP into the extension of unified TCI framework in RAN1#109 [4].

Agreement

On unified TCI framework extension, consider all the intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17

    - Consider, if STxMP is supported, Rel-18 MTRP scheme(s) with STxMP

In NR Rel-15/16, the spatial transmission properties for PUSCH are associated with the SRS(s) configured in a SRS resource set with usage of ‘Codebook’ or ‘non-Codebook’.  In NR Rel-17 feMIMO WI, a joint TCI state for both UL and DL or a TCI state for UL was introduced. So in Rel-18, the PUSCH multi-TRP enhancements can be extended to cover the unified TCI state framework as the Rel-17 design of unified TCI state framework for multi-TRP is stable.

Similar to downlink, uplink communication can also benefit from multi-TRP deployment for improving reliability and robustness. Up to Rel-16 NR, the following schemes are supported for multi-TRP operation in downlink: SDM, FDM, and TDM. The SDM, FDM, and TDM schemes are all feasible for uplink. A TDM-based solution requires low UE capabilities by paying extra latency and power, repetition in time domain is well performed since for the uplink power is limited which has already been specified in Rel-17. SDM and FDM schemes are considered to be able to achieve a lower latency and higher spectral efficiency. While comparing with SDM, FDM has the benefits that there’s no inter-layer interference and power boosting may be utilized, while SDM normally achieves a larger spectral efficiency. On the other hand, SDM/FDM schemes require power split and may be harmful for coverage, so power control issues need to be further studied. Based on the above discussion, we propose to focus on SDM/FDM-based schemes for uplink in R18.

In last RAN1 meeting, the following WA was approved [5].

Working assumption

Support the following schemes for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18:

- SDM scheme

- In RAN1#110bis-e, RAN1 will only consider SFN based transmission scheme to support in addition to the above. Decision to support or not to be made in RAN1#110bis-e.

- FFS: FDM-A scheme

- FFS: FDM-B scheme

- FFS: SFN-based transmission scheme

For schemes other than SDM, final decision to support or not will be made in RAN1#110bis-e.

If PUSCH/PUCCH is transmitted in SDM/SFN scheme, multiple beams are needed to transmit to different TRPs simultaneously and each beam is associated with one TRP of the two TRPs. However, the PL RS associated with each beam is also different for each TRP. Therefore, the power of both transmissions from different panels would have to be different in the same OFDM symbols within a slot.

Proposal 3-1: For STxMP, support the extension of unified TCI to the transmission schemes for both single DCI based and multi-DCI based PUSCH/PUCCH.

Several issues according to the beam indication for the simultaneous transmission of multi-panel/multi-TRP based on the unified TCI framework can be discussed.

For STxMP based PUSCH/PUCCH transmissions, 2 beams need to be indicated (via DCI or MAC-CE) for the transmissions towards different TRPs from different UE panels. TCIs or SRIs can be used for the detailed indication for the 2 beams. For the purpose of scheduling, the first problem need to be solved is how to identify two UL beams to support the simultaneous UL transmission. The association between the indicated TCI(s)/SRI(s) and specific TRP(s)/panel(s) should take the UE capability value set index into consideration. To support this simultaneous multi-panel transmission, UE has to let gNB aware whether the selected two beams from UE beam reporting can be simultaneously transmitted or not, and whether they can be transmitted from different panels.

In R17, the beam reporting process is enhanced with the index of UE capability value set to indicate the maximum supported number of SRS ports related to each pair of CRI/SSBRI and L1-RSRP/L1-SINR.But the gNB cannot select two UE panels with same UE capability value set for simultaneous UL transmission, and also the gNB cannot separate the case of the beam reporting for the DL-only panel case.

To determine the simultaneous uplink transmission beams, the methods based on current CSI measurement and reporting mechanism can be further enhanced.

The first approach is based on group-based beam reporting with the following options,

- Option 1: UE can report the beam pair with an additional ID (2bits) to provide information about the supported UL/DL transmission types. Three different corresponding UL/DL transmission types are:

- Type 1: DL-only simultaneous transmission

- Type 2: DL simultaneous transmission and UL non-simultaneous transmission

- Type 3: DL&UL simultaneous transmission

And an example of codepoints of the beam group index mapping to the supported UL/DL transmission types. 2bits are needed for one reported beam pair. If DL-only case is precluded, then 1bit indication is enough for each beam pair.

Table 1. Codepoint mapping of option.1

| Codepoints indicating the transmition type of the Beam pair | UL/DL transmission types supported |
| --- | --- |
| 00 | DL-only simultaneous transmission |
| 01 | DL simultaneous transmission and UL non-simultaneous transmission |
| 10 | DL simultaneous transmission and UL simultaneous transmission including STxMP |
| 11 | Reserved |

- Option 2: UE can report the Rx beam group index for each reported beam, each Rx beam related to the measured simultaneously received DL beam. Two grouping methods can be considered:

- Alt.1: UE can receive the DL beams simultaneously only within the same Rx beam group. The same Rx/Tx beam group includes beams only across different panels. The report of same group indices within a reported beam pair means support of Type 3 above. The report of different group indices of each beam of the beam pair means support of the Type 2 above.  If DL-only panel is within the group, no group index would be reported for the corresponding beam.

Table 2. The illustration of beam grouping Alt.1

| Reported Beam indices | beam#1 | beam#2 | beam#3 | beam#4 |
| --- | --- | --- | --- | --- |
| Beam pair # | Beam pair#1 |  | Beam pair#2 |  |
| Corresponding physical panel ID | #1 | #2 | #1 | #4 |
| UL/DL | UL/DL | UL/DL | UL/DL | DL |
| Beam group ID | #1 | #1 | #2 | #2 |
| Reported beam group ID | #1 | #1 | #2 | none |
| Supported transmission type | DL/UL transmission including STxMP |  | DL-only simultaneous transmission |  |

- Alt.2: UE can receive the DL beams simultaneously only within different Rx beam groups. The different Rx/Tx beam group include beams only across different panels. The report of different group indices within a reported beam pair means support of Type 3 above. The report of same group indices of each beam of the beam pair means support of the Type 2 above.  If DL-only panel is within the group, no group index would be reported for the corresponding beam.

Table 3. The illustration of beam grouping Alt.2

| Reported Beam indices | beam#1 | beam#2 | beam#3 | beam#4 |
| --- | --- | --- | --- | --- |
| Beam pair # | Beam pair#1 |  | Beam pair#2 |  |
| Corresponding physical panel ID | #1 | #2 | #1 | #4 |
| UL/DL | UL/DL | UL/DL | UL/DL | DL |
| Beam group ID | #1 | #2 | #1 | #3 |
| Reported beam group ID | #1 | #2 | #1 | none |
| Supported transmission type | DL/UL transmission including STxMP |  | DL-only simultaneous transmission |  |

For both Alt.1 and Alt.2, per beam reporting can provide more information about the beams. For example, the beam pair that cannot support STxMP transmission but only DL simultaneous transmission, may also provide information for the gNB to schedule a STRP uplink transmission as shown in Table.3.

We prefer Alt.1 which exposes less of UE panel implementation.

- Option 3: UE can further extend the UE capability value set index with additional ID when reporting the beams with same UE capability value set index.  UE report the additional beam ID based on the reordering of the actual physical panel ID along with the report of each UE capability value set index reporting.  With the reordering of all panels needed for the indication for all reported beams, less exposure of the physical panel implementation would be needed.

Table 4. The illustration of beam grouping Option 3

| Reported Beam indices | beam#1 | beam#2 | beam#3 | beam#4 |
| --- | --- | --- | --- | --- |
| Beam pair # | Beam pair#1 |  | Beam pair#2 |  |
| Corresponding physical panel ID | #2 | #3 | #3 | #3 |
| UL/DL | UL/DL | UL/DL | UL/DL | UL/DL |
| UE capability value set ID | #2 | #2 | #2 | #2 |
| Additional Reported beam ID | #1 | #2 | #2 | #2 |
| Supported transmission type | DL/UL transmission including STxMP |  | DL simultaneous transmission and UL non-simultaneous transmission |  |

Another approach is to enhance the beam reporting mechanism. Currently, either group based beam reporting or non-group based beam reporting is supported for the CSI measurement and reporting of beam management.  For the simultaneous uplink transmission, a new configuration of the beam reporting like groupBasedUplinkBeamReporting targeting STxMP can be considered for the report of beams/beam pair(s) for the STxMP transmissions.  UE would guarantee all the supported beams/beam pair(s) can support the STxMP scheduling, the beams/beam pair(s) reported can be differentiated at least for whether to support the STxMP or the TDM transmission mode for the MTRP operation.

Proposal 3-2：To determine the simultaneous uplink transmission beams, the enhancement is based on group-based beam reporting with the following options,

- Option 1: UE can report the beam pair with an additional ID (2bits) to provide information about the supported UL/DL transmission types;

- Option.2: UE can report the Rx beam group index for each reported beam, each Rx beam related to the measured simultaneously received DL beam.

- Option 3: UE can further extend the UE capability value set index with additional ID when reporting the beams with same UE capability value set index.

Proposal 3-3: To determine the simultaneous uplink transmission beams, introducing a new beam reporting configuration “groupBasedUplinkBeamReporting” targeting STxMP for the report of beams/beam pair(s) for the STxMP transmissions also can be considered.

For the current group-based beam reporting, only 2 beams (N=2) or one beam pair can be configured to be reported. From the analysis above, we can see that some beam pair selected cannot even be used for STxMP. Even though one pair of beams is supportive of STxMP, it may not be appropriate for the scheduling of uplink transmission because of the interference situation estimated at the gNB. So more beam pairs should be allowed to be configured for the selection to provide more flexibility for the gNB scheduler and make the schedule of STxMP much more feasible.

Proposal 3-4: Considering the flexibility for the gNB scheduling and the feasibility of the support of STxMP,  support to report in a single reporting instance of more than two (N>=2) different beams/beam pairs that UE can receive/transmit simultaneously.

For the enhanced beam reporting, it can be applied to all types of CSI reporting types, i.e. P-CSI/SP-CSI/AP-CSI.

Proposal 3-5: For the enhanced CSI measurement and reporting targeting STxMP, the enhancement can be applied to all CSI types, i.e. P-CSI/SP-CSI/AP-CSI.

For the simultaneous uplink transmission, there can be different combinations of simultaneous transmission such as PUSCH/PUSCH or PUCCH/PUCCH that can be considered for reliability enhancements. A tight coordination with potentially CSI exchange between the TRPs for the co-scheduling is needed to reduce interference between the simultaneous transmissions for SDM solutions. In these operations, uplink beams needs to be carefully selected to mitigate interference towards the other TRP.

Proposal 3-6: Beam selection enhancement needs to be further considered for the coordination between panels/TRPs to mitigate the interference for the SDM schemes.

For the simultaneous uplink transmission, different TCIs/SRIs are associated with different transmission parameters for each transmission schemes defined in AI 9.1.4.1. The detailed discussion should be based on the agreed simultaneous transmission schemes. We support to specify both SDM and SFN schemes.

Proposal 3-7: For SDM scheme, one TB is partitioned to be transmitted on different sets of transmission layer(s)/DMRS port(s) corresponding to different TRP/panel/TCI/SRI.

Proposal 3-8: For SFN scheme, same TB associated with different TCIs/SRIs is assumed to be transmitted on same set of transmission layer(s)/DMRS port(s), each corresponding to a specific TRP or a specific panel.

Power control for UL single DCI

Power control for single DCI mTRP UL transmission

The PC parameters for UL transmission includes P0, alpha, closed loop index and PL-RS. In Rel17 unified TCI framework, the PC parameter PL-RS is always included in joint/UL TCI state while the PC parameters {P0, alpha, closed loop index} can be associated with joint/UL TCI state or configured independently, which means without association with joint/UL TCI state.

Observed from TS 38.331 [1], there is a parameter, ul-powerControl-r17, in UL BWP configuration and it is configured with Uplink-powerControl-r17 which includes power control parameters { P0, alpha, closed loop index } as shown in table 1. ul-powerControl-r17 in UL BWP configuration is optional, and it only will be configured when no TCI state is associated with Uplink-powerControl-r17. Usually, when UE is indicated with R17 TCI state, the power control parameters in Uplink-powerControl-r17 associated with the TCI state will be used to decide the UL transmission power. While, when no TCI state is associated with Uplink-powerControl-r17, the power control parameter corresponding to ul-powerControl-r17 will be used.

Tab.1 UL power control parameter Uplink-powerControl-r17 in UL BWP configuration

BWP-UplinkDedicated ::= SEQUENCE { … ul-powerControl-r17 Uplink-powerControlId-r17 OPTIONAL, -- Cond NoTCI-PC … ]] } Uplink-powerControl-r17 ::= SEQUENCE { ul-powercontrolId-r17 Uplink-powerControlId-r17, p0AlphaSetforPUSCH-r17 P0AlphaSet-r17 OPTIONAL, -- Need R p0AlphaSetforPUCCH-r17 P0AlphaSet-r17 OPTIONAL, -- Need R p0AlphaSetforSRS-r17 P0AlphaSet-r17 OPTIONAL -- Need R } P0AlphaSet-r17 ::= SEQUENCE { p0-r17 INTEGER (-16..15) OPTIONAL, -- Need R alpha-r17 Alpha OPTIONAL, -- Need R closedLoopIndex-r17 ENUMERATED { i0, i1 } } Uplink-powerControlId-r17 ::= INTEGER(1.. maxUL-TCI-r17)

However, based on Table 1, there is only one set of {P0, Alpha, closed Loop Index} for each UL Channel/signal. Then, to support single DCI based multi-TRP UL transmission, two sets of PC parameters { P0, alpha, closed loop index} need to be configured in UL BWP configuration or other RRC IE when these parameters are not associated with joint/UL TCI state.

Proposal 4-1: Two sets of PC parameters {P0, alpha, closed loop index} can be configured via RRC to support single DCI based multi-TRP UL transmission when these parameters are not associated with joint/UL TCI state.

Power control for simultaneous multi-panel UL transmission

As described in objective 6, simultaneous multi-panel UL transmission (STxMP) will be specified in Rel18 where the total number of layers is up to four across all panels. According to TS38.213, UE will split the power equally across the antenna ports. Then, TRP-specific power control cannot be support in current power control framework. For example, considering the scenario shown in Figure1, layer 1 and layer 2 mapping to panel 1 is transmitted to TRP1 and layer 3 layer 4 mapping to panel 2 is sent to TRP2. The transmit power for each panel will be the same based on current specification.

Fig.1 Simultaneous multi-panel UL transmission

There are two possible methods to deal with this issue. Firstly, the power control for PUSCH in STxMP is still channel specific and the UE splits the transmit power across panels considering the number of antenna ports, channel conditions and other aspects for each panel. But the exact way to split the power and indicate power control parameters needs further discussion. Secondly, the power control is panel specific, in which UE calculates the transmit power for each panel according to the parameters indicated for each panel. Then the PUSCH transmission power PPUSCH is PPUSCH=Ppanel1+Ppanel2. More details can be discussed after that we make a decision on the power control framework for simultaneous multi-panel UL transmission.

Proposal 4-2: There are two possible power control methods for STxMP

- Option1: UE determines a total PUSCH transmission power and splits the power across the multiple panels.

- Option2: UE determines the transmit power for each panel independently

Conclusion

In this contribution, we discuss about the unified TCI framework extension for M-TRP. Based on above discusses, we provide the following proposals.

Indication of multiple DL/UL TCI states

Proposal 2-1: For M-TRP unified TCI framework, support one TRP is configured with joint TCI and the other one is configured with separate DL/UL TCI.

Proposal 2-2: For M-TRP unified TCI framework, support to switch between joint TCI and separate DL/UL TCI based on MAC CE.

Proposal 2-3: For PDCCH repetition/SFN, support Alt 3, use MAC-CE to inform the mapping/association between an activated or indicated joint/DL TCI state and a CORESET.

Proposal 2-4: For PDCCH without repetition/SFN, down select from Alt 1-1, Alt 1-2 and Alt 3:

- Alt 1-1: use RRC configuration to inform the mapping/association between a configured or indicated joint/DL TCI state and a CORESET.

- Alt 1-2: use RRC configuration to inform the mapping/association between a configured or indicated joint/DL TCI state and a CORESET group.

- Alt 3: use MAC CE to inform the mapping/association between an activated or indicated joint/DL TCI state and a CORESET.

Proposal 2-5: For S-DCI M-TRP scenario, support to configure CORESET group ID for each CORESET.

Proposal 2-6: For CORESET group ID configuration for CORESET with PDCCH SFN in S-DCI M-TRP scenario, consider following two alternatives.

- Alt 1: configure CORESET group ID#2 to CORESET with SFN, and up to two TCI states can be activated/indicated for CORESET group ID#2.

- Alt 2: configure CORESET group ID#0 and CORESET group ID#1 to CORESET with SFN, and one TCI state can be activated/indicated for each CORESET group.

Proposal 2-7: Support to use an indicator field other than the existing TCI field (could be reusing an existing DCI field or introducing a new DCI field) in a DCI format 1_1/1_2 with and without DL assignment to inform which indicated joint/DL TCI state(s) the UE shall apply to all PDSCH receptions after an application time

Proposal 2-8: On unified TCI framework extension for S-DCI based MTRP, to inform the association with joint/UL TCI state(s) indicated by DCI/MAC-CE for PUCCH transmission:

- For M-TRP PUCCH repetition, support Alt 3, i.e., use MAC CE to map/associate one or two indicated joint/UL TCI state to a PUCCH resource/PUCCH resource group.

- For S-TRP PUCCH, support Alt 1 or Alt 3, i.e., use MAC CE or RRC configuration to map/associate one or two indicated joint/UL TCI state to a PUCCH resource/PUCCH resource group.

Proposal 2-9: On unified TCI framework extension for S-DCI based MTRP, for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2:

- Support Alt 1, i.e., Use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in a DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2

Proposal 2-10: On unified TCI framework extension for S-DCI based MTRP, for Type 1 CG PUSCH repetition, support to use RRC configuration or default rule to inform the mapping/association.

Proposal 2-11: On unified TCI framework extension for S-DCI based MTRP, for PUSCH transmission scheduled/activated by a DCI format 0_0, down-selection one alternative from the followings

- Alt 1: The TCI state of PUSCH transmission scheduled/activated by the DCI format 0_0 is same as that of the CORESET with the scheduling PDCCH.

- Alt 2: Based on default rule, e.g., use the first TCI state of the indicated joint/UL TCI state(s).

- Alt 3: Use MAC CE to inform the mapping/association between indicated joint/UL TCI state(s) and PUSCH transmission scheduled/activated by the DCI format 0_0.

- Alt 4: Based on the mapping/association informed by the latest DCI format 0_1/0_2.

Proposal 2-12: The application time for TCI state mapping/association information based on MAC CE or DCI should be defined.

Proposal 2-13: For M-DCI M-TRP scenario, for channel /signal transmission scheduled by DCI, the unified TCI state of the corresponding scheduling CORESET will be applied.

Proposal 2-14: For M-DCI M-TRP scenario, configure CORESETPoolIndex to non-DCI indicated PUCCH and Type 1 CG PUSCH.

Proposal 2-15: For M-DCI M-TRP scenario, support Alt 4.

- Alt4: Use the existing TCI field in the DCI format 1_1/1_2 (with or without DL assignment) associated with one of CORESETPoolIndex values to indicate joint/DL/UL TCI state(s) corresponding to the same or different CORESETPoolIndex value.

Proposal 2-16: For CJT, support up to 2 TCI states.

Simultaneous multi-panel UL transmission

Proposal 3-1: For STxMP, support the extension of unified TCI to the transmission schemes for both single DCI based and multi-DCI based PUSCH/PUCCH.

Proposal 3-2：To determine the simultaneous uplink transmission beams, the enhancement is based on group-based beam reporting with the following options,

- Option 1: UE can report the beam pair with an additional ID (2bits) to provide information about the supported UL/DL transmission types;

- Option.2: UE can report the Rx beam group index for each reported beam, each Rx beam related to the measured simultaneously received DL beam.

- Option 3: UE can further extend the UE capability value set index with additional ID when reporting the beams with same UE capability value set index.

Proposal 3-3: To determine the simultaneous uplink transmission beams, introducing a new beam reporting configuration “groupBasedUplinkBeamReporting” targeting STxMP for the report of beams/beam pair(s) for the STxMP transmissions also can be considered.

Proposal 3-4: Considering the flexibility for the gNB scheduling and the feasibility of the support of STxMP,  support to report in a single reporting instance of more than two (N>=2) different beams/beam pairs that UE can receive/transmit simultaneously.

Proposal 3-5: For the enhanced CSI measurement and reporting targeting STxMP, the enhancement can be applied to all CSI types, i.e. P-CSI/SP-CSI/AP-CSI.

Proposal 3-6: Beam selection enhancement needs to be further considered for the coordination between panels/TRPs to mitigate the interference for the SDM schemes.

Proposal 3-7: For SDM scheme, one TB is partitioned to be transmitted on different sets of transmission layer(s)/DMRS port(s) corresponding to different TRP/panel/TCI/SRI.

Proposal 3-8: For SFN scheme, same TB associated with different TCIs/SRIs is assumed to be transmitted on same set of transmission layer(s)/DMRS port(s), each corresponding to a specific TRP or a specific panel.

Power control for UL single DCI

Proposal 4-1: Two sets of PC parameters {P0, alpha, closed loop index} can be configured via RRC to support single DCI based multi-TRP UL transmission when these parameters are not associated with joint/UL TCI state.

Proposal 4-2: There are two possible power control methods for STxMP

- Option1: UE determines a total PUSCH transmission power and splits the power across the multiple panels.

- Option2: UE determines the transmit power for each panel independently

References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink

- RAN1-105 e-meeting Chairman notes.

- TS 38.321 h00, V17.0.0 (2022-03)

- RAN1-109 e-meeting Chairman notes.

- RAN1-110 meeting Chairman notes.