3GPP TSG RAN WG1 #110bis-e			R1-2209283

e-Meeting, October 10th – 19th, 2022

Source: 	     Xiaomi

Title:		     Discussion on evaluation on NR duplex

Agenda item:    9.3.1

Document for:  Discussion

Introduction

In RAN1#110 meeting, we further discussed issues related to evaluation on NR duplex evolution. [1] The detailed agreements can be found in the appendix.

In this contribution, we share views on deployment scenarios and methodology. Furthermore, we provide preliminary SLS evaluation results based on the current assumptions.

Discussion

Evaluation for subband based full duplex

Scenarios for SBFD evaluation

Two sub-cases have been identified in RAN1#109 meeting, i.e. 1-layer case and 2-layer case. In the followed RAN1#110 meeting, it was agreed that deployment case 3-1 can be discussed with low priority. Even so, we think indoor scenario is not as important as macro scenario given indoor deployment has more flexibility and low cost. For 2-layer case, there are two different understandings, i.e. Macro+Micro and Macro+Indoor/Hotspot. From our understanding, Macro+Micro defined in TR38.802 should be the intended 2-layer scenario.  Furthermore, the deployment of legacy gNB and SBFD gNB needs clarification. There are two potential deployment strategies for SBFD gNB which are explained in Figure 1.

- Interpretation#1: Layer-based SBFD gNB deployment. In this case, one layer is constructed with SBFD gNB while the other layer is constructred with legacy TDD gNB.

- Interpretation#2: Mixed SBFD gNB deployment. In this case, each layer consists of different type of gNB, i.e. SBFD gNB and legacy TDD gNB.

Proposal 1: The following aspects corresponding to deployment case 3 need to be further clarified:

- For 1-layer case, dense urban Macro or Urban macro scenario is adopted

- For 2-layer case, dense urban Macro with two layers is adopted

  - The SFBD gNB deployment needs further clarification, i.e. per layer deployment or mixed deployment across layers.

Figure1:Examples of 2-layer scenarios for co-existence between legacy TDD gNB and SBFD gNB

In RAN1#110, we discussed the relevant parameters of several deployment scenarios and the a working assumption was achieved.[1] Basically, the principle is to reuse the scenarios defined in TR38.802 as much as possible. Considering the scenarios in TR38.802 have been extensively used for NR simulation, which are mature enough, we think the working assumption can be confirmed. Additional scenarios proved to be more realistic for SBFD/DTDD can also be supported if necessary.

Regarding the minimum UE-UE(2D) distance, the controversy on 3m is that it cannot reflect the realistic connection between UEs as it is typical that people are more close to each other than 3m in a room or in the street. It is a reasonable assumption to set a smaller minimum distance between two UEs. However, it may not make much difference between two values, i.e. 1m and 3m, as it can only determine the minimum distance and UEs are still dropped uniformly within cell area. More importantly, we don’t see the necessity to define different UE-to-UE minimum distance for different scenarios.

Proposal 2: Confirm the following working assumption with a unified minimum UE-UE(2D) distance across different scnarios:

Working Assumption

| Parameters | Indoor office | Urban macro / Dense Urban Macro layer | Dense Urban with 2-layer |
| --- | --- | --- | --- |
| Layout | Single layer Indoor floor: (12BSs per 120m x 50m) | Single layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around. | Two layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around. Micro layer: According to previous agreement - Baseline: 3 Micro BSs per Macro BS - Optional: 6, or 9 Micro BSs per Macro BS |
| Inter-BS (2D) distance | 20m [TR 38.802 Table A.2.1-11] | 500m for Urban Macro [TR 38.802 Table A.2.1-11] 200m for Dense Urban Macro layer [TR 38.802 Table A.2.1-1] | Macro-to-macro: 200m Minimum Macro-to-micro-center distance: 105m Minimum Micro-center-to-micro-center distance: 57.9m |
| Minimum BS-UE (2D) distance | 0m [TR 38.802 Table A.2.1-11] | 35m [TR 38.802 Table A.2.1-11] | Macro-to-UE: 35m Micro-to-UE: 10m [TR 38.802 Table A.2.1-11] |
| Minimum UE-UE (2D) distance | FFS 3m [TR 38.802 Table A.2.1-11] | FFS :3m [TR 38.802 Table A.2.1-11] | FFS: 3m [TR 38.802 Table A.2.1-11] |
| BS antenna height | 3 m [TR 38.802 Table A.2.1-1] | 25 m [TR 38.802 Table A.2.1-1] | 25m for macro cells and 10m for micro cells [TR 38.802 Table A.2.1-1] |

Parameters for UE distribution

In RAN1#110 meeting, cluster based UE distribution was agreed as a baseline. The key parameters for determining cluster-based UE distribution is still open. The detail procedures to distribute UE in the network is described as the following agreement achieved in RAN1#110:

Agreement Update the previous agreement as below: For UE distribution of Urban Macro and Dense Urban Macro layer, - Baseline: (UE clustering at least for FR1) - M users per macro TRP - Step 1: Randomly drop X UE cluster centers within one macro cell geographical area considering the minimum distance between macro TRP to UE cluster center as Dmacro-to-cluster and the minimum distance between two UE cluster centers as Dinter-cluster - Step 2: Y% UEs are randomly and uniformly dropped within the UE clusters with the radius of R, (1-Y%) users randomly and uniformly dropped in the macro geographical area outside the clusters - Note: UEs dropped within the UE cluster(s) are indoor with 3km/h; UEs dropped outside the UE cluster(s) are outdoor in car with 30km/h - UE outdoor/indoor proportion: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h - Outdoor UEs: 1.5 m; - FFS: Indoor UEs height - Y%=80% - FFS the values of M, X, Dmacro-to-cluster, Dinter-cluster, R - Optional: - 10 users per macro TRP (per direction), and all users are randomly and uniformly dropped within the macro cell - At least for FR1: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h - Outdoor UEs: 1.5 m; - Indoor UEs: 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) [refer to TR 36.873 Table 6-1] - FFS: FR2 details

The principle of FTP model 3 is  to generating data packet for each UE according to the packet size and arrival rate. Therefore, the total number of  dropped UEs within gNB area would impact resource utilization. It should be considered together with relevant parameters of FTP model 3. For example, if a large number of UE is distributed in the cell, the arrival data rate has to be reduced. On the other hand, the motivation of cluster based UE distribution is to simulate UE-to-UE CLI as realistic as possible. In TR38.802, 60 UEs per Macro cell is assumed for dense urban scenario, wherein 3 Micro per Macro TRP are dropped.[2] Hence, the number of UE per Macro TRP can be set as 20.  UE-to-UE CLI comes from different UE clusters may be not as severe as that within a same UE cluster. One UE cluster per Macro TRP can be a baseline.  Generally speaking, the procedure of cluster based UE distribution is same as that of dense urban with two layers. The parameters defined in TR38.802 for dense urban can be reused here. A Micro droped in a Macro area can be reused as a UE cluster. To be specific, the following parameters can be used as the starting point for UE distribution:

- Dmacro-to-cluster=35+28.9m

- Dinter-cluster = 57.9 m

- R = 28.9m

Regarding to Dmacro-to-cluster, 35m is the minum distance between UE and Macro gNB as defined in TR38.802. 28.9m is the radius of Micro cell.

Regarding indoor UE height, we think it should be same as outdoor UE, i.e. 1.5m should be reused.

Proposal 3: For UE distribution of Urban Macro and Dense Urban Macro layer, the following parameter values can be applied to cluster based UE distribution at least for FR1:

- Dmacro-to-cluster=35+28.9m

- Dinter-cluster = 57.9 m

- R = 28.9m

- Indoor UE height = 1.5m

Traffic model for SBFD evaluation

In RAN1#109 e-meeting, it was agreed that at least FTP3 is considered for SBFD and dynamic/flexible TDD evaluation. Whether the other traffic models, e.g. XR, VoIP, should be evaluated or not is still open. For VoIP, it has a nature of periodicity, small packet size and less sensitivity of latency. Considering the objective of SBFD, it is not the suitable technique for VoIP which can be supported pretty well by nowadays mechanism.  In summary, XR traffic pursues low latency, high reliability and high data rate. For example, AR UL stream requires as high as 20 Mbps data rate and as low as 10ms PDB.[2] SBFD can significantly improve uplink throughput and reduce latency thanks to more available uplink resources.

Proposal 4: XR traffic models can be considered for SBFD and dynamic/flexible TDD evaluation.

Definition of output metrics

Druing RAN1#109 e-meeting, it was agreed that at least the following metrics are considered for SBFD and dynamic/flexible TDD evaluation:

- DL/UL UPT or user throughput (CDF or {mean, 5%, 50%, 95%}) using SLS

- Latency (CDF or {mean, 5%, 50%, 95%}) using SLS

- Resource utilization using SLS

- DL/UL received SINR using SLS

- Coverage metric

In RAN1#110 meeting, we agreed to at least use RU, Packet latency and DL/UL UPT as the SLS metric and further clarified the definition.[1] However, the definition of DL/UL received SINR using SLS and coverage metric are still open. For covenience, we summarize the possible definition for each metric in Table 1.

Table 1: Summary of definition for output metrics

| Output metric | Definition | Source |
| --- | --- | --- |
| DL/UL received SINR | Received SINR = Effective signal power / (Interference+Noise) |  |
| Coverage | The budget template defined for coverage enhancement can be used as a starting point. Self-interference and CLI should be reflected. | TR38.830 |

Proposal 5:  The definition provided in Table 1 is adopted for SBFD and dynamic/flexible TDD evaluation.

UL subband configuration

UL subband configuration is a key aspect for SBFD evaluation. A larger UL subband brings better UL performance due to additional available resources compared with legacy TDD system. The UL subband configuration includes two aspects, i.e. time domain and frquency domain. On the other hand, an aggressive UL subband configuration will certainly degrade downlink performance. In order to comprehensively evaluate SBFD, a set of UL subband configurations should be used in SLS evaluation. In RAN1#109 e-meeting, several alternatives for performance evaluation were agreed for further consideration. [1] In order to strive for the same UL/DL resource ratio between legacy TDD and SBFD, alt.3 and alt.4 were proposed, which were quoted as below:

- Alt 3 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD):

  - Legacy TDD: Static TDD UL/DL configuration with {DDSUU}, where S=[12D:2G:0U]

  - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

- Alt 4 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD):

  - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

  - SBFD: Frame structure#3 (XXXXX), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

Basically, the mechanism to strive for the same UL/DL resoruce ratio between Legacy TDD and SBFD is to treat a UL slot as a SBFD slot, wherein SBFD UL subband spans all the symbols in the slot. However, it is quite wired to put a restriction for uplink transmission in UL slot. Furthermore, the reasoning of alt.3 and alt.4 is unclear to us. The potential benefits of SBFD, including higher thoughput, improved coverage and low latency, comes from additional uplink resources. We don’t understand why we need to evaluate SBFD performance on the assumption without introducing any additional UL resources. Accordingly, we have the following observation:

Observation 1: For alt 3 and alt 4 under umbrella of SBFD Deployment Case 1,

- It restricts the uplink transmission on the UL symbols with confining available UL resources within UL subband.

- The same UL/DL resource ratio between Legacy TDD and SBFD degrades or even eliminate the potential benefits of SBFD.

- System performance is further degraded due to the guard band between UL subband and DL subband on UL slot.

Another issue is whether to use dynamic TDD for legacy TDD for comparison. The methodogy and feasibility of dynamic TDD is under discussion. It may lead to duplicated work if dynamic TDD is also considered in the SBFD evaluation. Furthermore, we are still not clear on how to handle the CLI and which scenario(s) is/are recommended for dynamic TDD. It is premature to use dynamic TDD for legacy TDD for comparison.

Proposal 6: Dynamic TDD is not used for legacy TDD for comparison.

Channel model

In RAN1#110 meeting, we premilinarily discussed gNB-to-gNB channel model and UE-to-UE channel model. Two working assumptions for gNB-to-gNB channel model and UE-to-UE channel model were made respectively. Generally speaking,  the following principles are applied to both channel model:

- For gNB-to-gNB channel model across different scenarios, channel models defined in TR38.901 are reused

- For UE-to-UE channel model across different scenarios, depends on the frequency range:

  - For FR1, channel models defined in either TR38.901 or TR36.843 can be reported by companies.

  - For FR2-1, channel models defined in TR38.901 are reused.

Channel model is fundamental factor for SLS simulation which needs comprehensive analyses and discussions. We don’t think it is feasible to define or introduce new channel models for SBFD evaluation during study item. On the other hand, we don’t see the motivation to introduce new channel model or modify the channel models defined in TR38.901 for SBFD/DTDD evaluation purpose. Based on aforementioned anaylses, we propose to confirm the working assumption on gNB-to-gNB channel model and UE-to-UE channel model achieved in the last meeting.

Proposal 7: Confirm the two working assumptions related to gNB-to-gNB channel model and UE-to-UE channel model achieved in RAN1#110 meeting.

Evaluation for dynamic TDD

In Rel-16 CLI, we study the CLI introduced by dynamic TDD, wherein the cross link interference comes from adjacent channel. In Rel-18 duplex operation, we need to further exploit the possibility and the potential enhancement of dynamic TDD in co-channel case. After comprehensive evaluation, it was observed that even if the aggressor gNB and victim gNB are located on adjacent channel, it is still not recommended to apply dynamic TDD because of the performance degradation due to severe gNB-to-gNB interference. [2] The recommendation from previous work should be taken into account. For co-channel dynamic TDD, the gNB-to-gNB interference would be much more serious as it comes from the same channel.  Based on the knowledge we learned from Rel-16 CLI, scenarios with single layout should not be considered for co-channel dynamic TDD evaluation, unless there is sufficient isolation among gNBs. Accordingly, HetNet scenario with both macro layout and indoor/hotspot layout can be considered. For macro layout, semi-static TDD UL-DL configuration can be assumed as the statistical UL DL traffic ratio is perceived. For indoor/hotspot layout, dynamic TDD can be considered to improve the performance, e.g. DL or UL throughput, as the UL/DL traffic load is more fluctuating. Considering there are walls between macro layout and indoor/hotspot layout, the cross link interference between these two layouts may be acceptable. Furthermore, the low transmission power of indoor/hotspot TRP is friendly for the application of dynamic TDD on indoor/hotspot layout.

Figure 2: Illustration of HetNet consist of urban Macro layout and indoor/hotspot layout

Proposal 8: For flexible/dynamic TDD, evaluate and study the performance in HetNet scenario.

Generic aspects related to evaluation

Evaluation cases

In RAN1#110 meeting, very good progress has been achieved on evaluation methodology and deployment scenarios. [1] On the other hand, there is a lot of freedom for companies to select the detail parameters of SLS simulation. We list the key parameters for system level evaluations with different options as below:

- UL/DL traffic generation: there are two options for UL/DL traffic generation, i.e. 1) Each UE is either assigned UL traffic or DL traffic. 2) Each UE is assigned both UL traffic and DL traffic.

- FTP packet size: symmetric packet size or asymmetric packet size.

- Channel estimation: ideal channel estimation or realistic channel estimation.

- BS transmit power: two options for Macro scenario.

- UE-UE channel model: two options for FR1.

- gNB antenna architecture: three options were identified in RAN1#110 meeting.

We want to emphasize the options for each listed key parameter are equal and which option is adopted in the simulation is fully up to company. Without futher guidence, the optional parameters would lead to divergent parameter  combinations. Considering the above parameters are fundamental for SLS evaluation, different parameter would leads to quite different results. It makes comparasion among companies very difficult if not impossible. Accordingly, it is important to identify a baseline combination of the above parameters.

Observation 2: A baseline combination is needed for the following parameters for easy comparison among companies:

- UL/DL traffic generation

- FTP packet size

- Channel estimation

- BS transmit power

- UE-UE channel model

- gNB antenna architecture

Interference definition

In RAN1#109 e-meeting, a bunch of interference type were defined for facilitating further discussion. Regarding to gNB-UE co-channel interference and UE-gNB co-channel interference, the following two types were identified:

- gNB-UE co-channel intra-subband interference: This is the same as the legacy DL interference type in legacy TDD network with static TDD UL/DL configuration.

- UE-gNB co-channel intra-subband interference: This is the same as the legacy UL interference type in legacy TDD network with static TDD UL/DL configuration.

However, the above interference cannot cover all cases in deployment case 2, wherein inter-subband interference is introduced by unaligned UL subband configuration across gNB. One example is shown in Figue 3, wherein the aggressor gNB#1 and victim gNB#2 have different UL subband configuration in a DL slot. Accordingly, the following two interference should be taken into consideration in the simualtion if needed.

- gNB-UE co-channel inter-subband interference: Interfernece caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to DL reception of the victim UE on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

- UE-gNB co-channel inter-subband interference: Interfernece caused by UL transmission of the aggressor UE on a first set of RBs in a carrier to UL reception of the victim gNB on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

Figure 3: Illustration of gNB-UE/UE-gNB co-channel inter-subband interference

Observation 3: For deployment case 2, the following two interference type should be take into account:

- gNB-UE co-channel inter-subband interference: Interfernece caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to DL reception of the victim UE on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

- UE-gNB co-channel inter-subband interference: Interfernece caused by UL transmission of the aggressor UE on a first set of RBs in a carrier to UL reception of the victim gNB on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

SLS evaluation results for SBFD

In this section, we provide preliminary SLS evaluation results for SBFD deployment case 1 to investigate the performance gain offered by SBFD.[1] At the same time, we provide results for DL transmission when SBFD is introduced in order to see the impacts on DL direction. Furthermore, we also provide the results with different assumptions on self-interference suppression capability, which comes from the reply LS from RAN4.  Dense urban with single layer scenario is assumed wherein 20 UEs are dropped per sector. In each sector, one UE cluster is dropped and 80% UEs are uniformly distributed within the cluster. More detail simulation assumptions can be found in the appendix. Regarding the evaluation metrics, UL/DL user perceived thoughput, UL/DL latency and UL/DL RU are used in the simulation so as to comprehensively evaluate SBFD technique.  The aforementioned metrics are defined in previous meeting.[1]

Basically, the following cases are simulated:

- Baseline-DDDSU: Legacy TDD with TDD UL-DL configuration DDDSU.

- SBFD-XXXXU-UL20%-145dB: On top of DDDSU frame structure, the first four slots are SBFD slot, on which UL subband occupies 20% portion of system bandwidth. Self-interference capability is assumed to be 145dB, i.e. the isolation between Tx and Rx of a SBFD gNB is 145dB.

- SBFD-XXXXU-UL20%-185dB: On top of DDDSU frame structure, the first four slots are SBFD slot, on which UL subband occupies 20% portion of system bandwidth. Self-interference capability is assumed to be 145dB, i.e. the isolation between Tx and Rx of a SBFD gNB is 185dB.

- SBFD-XXXXU-UL50%-145dB: On top of DDDSU frame structure, the first four slots are SBFD slot, on which UL subband occupies 50% portion of system bandwidth. Self-interference capability is assumed to be 145dB, i.e. the isolation between Tx and Rx of a SBFD gNB is 145dB.

- SBFD-XXXXU-UL50%-185dB: On top of DDDSU frame structure, the first four slots are SBFD slot, on which UL subband occupies 50% portion of system bandwidth. Self-interference capability is assumed to be 145dB, i.e. the isolation between Tx and Rx of a SBFD gNB is 185dB.

- SBFD- DXXXU -UL20%-145dB: On top of DDDSU frame structure, the first four slots are SBFD slot, on which UL subband occupies 20% portion of system bandwidth. Self-interference capability is assumed to be 145dB, i.e. the isolation between Tx and Rx of a SBFD gNB is 145dB.

- SBFD- DXXXU -UL20%-185dB: On top of DDDSU frame structure, the first four slots are SBFD slot, on which UL subband occupies 20% portion of system bandwidth. Self-interference capability is assumed to be 145dB, i.e. the isolation between Tx and Rx of a SBFD gNB is 185dB.

- SBFD- DXXXU -UL50%-145dB: On top of DDDSU frame structure, the first four slots are SBFD slot, on which UL subband occupies 50% portion of system bandwidth. Self-interference capability is assumed to be 145dB, i.e. the isolation between Tx and Rx of a SBFD gNB is 145dB.

- SBFD- DXXXU -UL50%-185dB: On top of DDDSU frame structure, the first four slots are SBFD slot, on which UL subband occupies 50% portion of system bandwidth. Self-interference capability is assumed to be 145dB, i.e. the isolation between Tx and Rx of a SBFD gNB is 185dB.

Dense urban Macro and indoor/hotspot are both evaluated. Furthermore, different

- 2.4.1 System simulation results for dense urban scenario

In this section, the evaluation results for dense urban scenarios are provided which take different assumptions into account.

- 2.4.1.1 UL and DL User Perceived Throughput

In this section, we provide results derived from following user perceived througput definition for DL and UL respectively.

  - Average-UPT of a user: defined as the average from all UPTs for all FTP packets intended for this user [Refer to TR36.814].

  - Tail-UPT of a user: defined as the worst 5% UPT among all FTP packets intended for this user [Refer to TR36.814].

  - Median-UPT of a user: defined as the 50% UPT among all FTP packets intended for this user.

In order to reflect the traffic load in real world, we also provide results corresponding to different RU. The RU is determined by the baseline legacy TDD.

Figure 4: UL Average-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 5: DL Average-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 6: UL Medium-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 7: DL Medium-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 8: UL Tail-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 9: DL Tail-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Based on the simualtion results, we can have the following observations:

- DL user perceived thoughput is degraded significantly once UL subband is configured in DL slots, no matter which kind of UPT is used.

  - The degradation of performance is caused by less available DL resources in each SBFD slot

  - The larger UL subband in a SBFD slot, the more serious impacts on DL user thoughput

- UL user perceived thoughput is improved with introduction of UL subband in SBFD slots.

  - Significant performance gain can be observed when UL subband is introduced. The gain increases with the UL subband size, i.e. a larger UL subband brings better UL user throughput

  - The gain obtained from SBFD is impacted by the capability of self-interference suppression. Self-interference is one of the major obstacles in-between to harvest benefits from SBFD.

- In ratio, improvement of UL performance is much more than the degradation of DL performance.

- The maximum UL user throughput depends on how much additional UL resources can be provided by UL subband. For example, the UL user perceived throughput of a UE is better when XXXXU is configured comparing with DXXXU.

- Self-interference isolation impacts the benefits we can harvest from SBFD. For 185 dB SI suppression capability and 145 dB SI suppression capability, the former assumption slightly outperforms the later one. In the other word, 145 dB SI suppression capability already provides enough isolation for self-interference

Observation 4: When UL subband is introduced in DL slot, UL user throughput is improved significantly:

- Degradation of DL user thoughput is also observed, which depends on the UL subband configuration.

- In ratio, improvement of UL performance is much more than the degradation of DL performance.

- Better self-interference isolation achieves higher UL user throughput. 145 dB SI suppression can provide sufficient isolation between Tx and Rx at gNB side.

- 2.4.1.2 UL and DL latency

In this section, we provide results derived from following latency definition for DL and UL respectively.

- Option 1: Calculate the latency for each packet for each UE, and then generate CDF of latency for all these packets from all the UEs.

  - Packet-Latency CDF: The CDF of the packet latencies of all the packets from all the UEs.

  - Mean/5%/50%/95% Packet-Latency: The mean/5%/50%/95% value of Packet-Latency of all the packets from all the UEs.

- Option 2: Calculate the latency for each packet for each UE, and then calculate the average latency for each UE, then generate the CDF for these average latency for each UE

  - UE-Average-Latency: defined as the average packet latency for a UE

  - UE-Average-Latency CDF: The CDF of the UE-Average-Latency for all users.

  - Mean/5%/50%/95% UE-Average-Latency: The mean/5%/50%/95% value of UE-Average-Latency for all users.

Figure 10: UL Packet-latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 11: DL Packet-latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 12: UL UE-latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 13: DL UE-latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 14: UL UE-average(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 15: DL UE-average(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Based on the simualtion results, we can have the following observations:

- DL latecy is increased once UL subband is configured in DL slots no matter which kind of packet latency is adopted.

  - The degradation of performance comes from less available DL resources in each SBFD slot

  - The larger UL subband in a SBFD slot, the more serious impacts on DL latency

- UL latency is reduced with introduction of UL subband in SBFD slots no matter which kind of packet latency is adopted.

  - Significant performance gain can be observed when UL subband is introduced. The gain increases with the UL subband size, i.e. a larger UL subband brings better performance on latency.

  - The gain obtained from SBFD is impacted by the capability of self-interference suppression. Self-interference is one of the major obstacles in-between to harvest benefits from SBFD.

- In ratio, the magnitudes of improvement for UL performance is larger than degradation for DL performance.

- Self-interference isolation impacts the benefits we can harvest from SBFD. Lower UL latency can be achieved when larger self-interference isolation is assumed.

Observation 5: When UL subband is introduced in DL slot, UL lateny can be reduced significantly:

- Increasement of DL latency is also observed, which depends on the UL subband configuration.

- Better self-interference isolation brings lower UL latency.

- 2.4.1.3 UL and DL resource utilization

Baseed on the simulation results, the DL and UL resource utilization at 5%, 50% and percentile are summarized in Figure 16 and Figure 17 respectively.

Figure 16: DL resource utilization for Dense urban scenario

Figure 17: UL resource utilization for Dense urban scenario

Based on the simualtion results, we can have the following observations:

- DL resource utilization is increased once UL subband is configured in DL slots across light traffic load, medium traffic load and high traffic load.

  - The increasement of DL resource ultilization comes from less available DL resources in each SBFD slot

- UL resource utilization is reduced with introduction of UL subband in SBFD slots across light traffic load, medium traffic load and high traffic load.

- Better self-interference isolation can further reduce UL resource ultilization.

Observation 6: When UL subband is introduced in DL slot, UL resource utilization can be reduced.

- Increasement of DL RU is also observed, which depends on the UL subband configuration.

- In ratio, reduction of UL performance is close to the increasement of DL performance.

- Better self-interference isolation achieves lower UL resource utilization.

- 2.4.2 System simulation results for Indoor/Hotspot scenario

In this section, the evaluation results for Indoor/Hotspot scenarios are provided which take different assumptions into account.

- 2.4.2.1 UL and DL User Perceived Throughput

In this section, we provide results derived from following user perceived througput definition for DL and UL respectively.

  - Average-UPT of a user: defined as the average from all UPTs for all FTP packets intended for this user [Refer to TR36.814].

  - Tail-UPT of a user: defined as the worst 5% UPT among all FTP packets intended for this user [Refer to TR36.814].

  - Median-UPT of a user: defined as the 50% UPT among all FTP packets intended for this user.

In order to reflect the traffic load in real world, we also provide results corresponding to different RU. The RU is determined by the baseline legacy TDD.

Figure 18: UL Average-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 19: DL Average-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 20: UL Average-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 21: DL Average-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 22: UL Tail-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 23: DL Tail-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Based on the simualtion results, we can have the following observations:

- DL user perceived thoughput is degraded significantly once UL subband is configured in DL slots, no matter which kind of UPT is used.

  - The degradation of performance is caused by less available DL resources in each SBFD slot

  - The larger UL subband in a SBFD slot, the more serious impacts on DL user thoughput

- UL user perceived thoughput is improved with introduction of UL subband in SBFD slots.

  - Significant performance gain can be observed when UL subband is introduced. The gain increases with the UL subband size, i.e. a larger UL subband brings better UL user throughput

  - The gain obtained from SBFD is impacted by the capability of self-interference suppression. Self-interference is one of the major obstacles in-between to harvest benefits from SBFD.

- In ratio, improvement of UL performance is much more than the degradation of DL performance.

- The maximum UL user throughput depends on how much additional UL resources can be provided by UL subband. For example, the UL user perceived throughput of a UE is better when XXXXU is configured comparing with DXXXU.

- Self-interference isolation impacts the benefits we can harvest from SBFD. For 185 dB SI suppression capability and 145 dB SI suppression capability, the former assumption slightly outperforms the later one. In the other word, 145 dB SI suppression capability already provides enough isolation for self-interference

Hence similar observation for indoor/hotspot can be observed as dense urban scenario.

- 2.4.1.2 UL and DL latency

In this section, we provide results derived from following latency definition for DL and UL respectively.

- Option 1: Calculate the latency for each packet for each UE, and then generate CDF of latency for all these packets from all the UEs.

  - Packet-Latency CDF: The CDF of the packet latencies of all the packets from all the UEs.

  - Mean/5%/50%/95% Packet-Latency: The mean/5%/50%/95% value of Packet-Latency of all the packets from all the UEs.

- Option 2: Calculate the latency for each packet for each UE, and then calculate the average latency for each UE, then generate the CDF for these average latency for each UE

  - UE-Average-Latency: defined as the average packet latency for a UE

  - UE-Average-Latency CDF: The CDF of the UE-Average-Latency for all users.

  - Mean/5%/50%/95% UE-Average-Latency: The mean/5%/50%/95% value of UE-Average-Latency for all users.

Figure 24: UL Packet-latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 25: DL Packet-latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 26: UL UE-Average-Latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 27: DL UE-Average-Latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Based on the simualtion results, we can have the following observations:

- DL latecy is increased once UL subband is configured in DL slots no matter which kind of packet latency is adopted.

  - The degradation of performance comes from less available DL resources in each SBFD slot

  - The larger UL subband in a SBFD slot, the more serious impacts on DL latency

- UL latency is reduced with introduction of UL subband in SBFD slots no matter which kind of packet latency is adopted.

  - Significant performance gain can be observed when UL subband is introduced. The gain increases with the UL subband size, i.e. a larger UL subband brings better performance on latency.

  - The gain obtained from SBFD is impacted by the capability of self-interference suppression. Self-interference is one of the major obstacles in-between to harvest benefits from SBFD.

- In ratio, the magnitudes of improvement for UL performance is larger than degradation for DL performance.

- Self-interference isolation impacts the benefits we can harvest from SBFD. Lower UL latency can be achieved when larger self-interference isolation is assumed.

Hence similar observation for indoor/hotspot can be observed as dense urban scenario.

- 2.4.1.3 UL and DL resource utilization

Baseed on the simulation results, the DL and UL resource utilization at 5%, 50% and percentile are summarized in Figure 16 and Figure 17 respectively.

Figure 28: DL resource utilization for Indoor/Hotspot

Figure 29: UL resource utilization for Indoor/Hotspot

Based on the simualtion results, we can have the following observations:

- DL resource utilization is increased once UL subband is configured in DL slots across light traffic load, medium traffic load and high traffic load.

  - The increasement of DL resource ultilization comes from less available DL resources in each SBFD slot

- UL resource utilization is reduced with introduction of UL subband in SBFD slots across light traffic load, medium traffic load and high traffic load.

- Better self-interference isolation can further reduce UL resource ultilization.

Hence similar observation for indoor/hotspot can be observed as dense urban scenario.

Observation 7: SBFD technology can harvest more benefits in Indoor/Hotspot scenario compared with Dense Urban scenaro, thanks to low gNB power and good isolation.

Conclusion

In this contribution, we provide our views on evaluation on NR duplex evolution. We have the following observation:

Observation 1: For alt 3 and alt 4 under umbrella of SBFD Deployment Case 1,

- It restricts the uplink transmission on the UL symbols with confining available UL resources within UL subband.

- The same UL/DL resource ratio between Legacy TDD and SBFD degrades or even eliminate the potential benefits of SBFD.

- System performance is further degraded due to the guard band between UL subband and DL subband on UL slot.

Observation 2: A baseline combination is needed for the following parameters for easy comparison among companies:

- UL/DL traffic generation

- FTP packet size

- Channel estimation

- BS transmit power

- UE-UE channel model

- gNB antenna architecture

Observation 3: For deployment case 2, the following two interference type should be take into account:

- gNB-UE co-channel inter-subband interference: Interfernece caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to DL reception of the victim UE on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

- UE-gNB co-channel inter-subband interference: Interfernece caused by UL transmission of the aggressor UE on a first set of RBs in a carrier to UL reception of the victim gNB on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

Observation 4: When UL subband is introduced in DL slot, UL user throughput is improved significantly:

- Degradation of DL user thoughput is also observed, which depends on the UL subband configuration.

- In ratio, improvement of UL performance is much more than the degradation of DL performance.

- Better self-interference isolation achieves higher UL user throughput. 145 dB SI suppression can provide sufficient isolation between Tx and Rx at gNB side.

Observation 5: When UL subband is introduced in DL slot, UL lateny can be reduced significantly:

- Increasement of DL latency is also observed, which depends on the UL subband configuration.

- Better self-interference isolation brings lower UL latency.

Observation 6: When UL subband is introduced in DL slot, UL resource utilization can be reduced.

- Increasement of DL RU is also observed, which depends on the UL subband configuration.

- In ratio, reduction of UL performance is close to the increasement of DL performance.

- Better self-interference isolation achieves lower UL resource utilization.

Observation 7: SBFD technology can harvest more benefits in Indoor/Hotspot scenario compared with Dense Urban scenaro, thanks to low gNB power and good isolation.

Furthermore, we have the following proposals:

Proposal 1: The following aspects corresponding to deployment case 3 need to be further clarified:

- For 1-layer case, dense urban Macro or Urban macro scenario is adopted

- For 2-layer case, dense urban Macro with two layers is adopted

  - The SFBD gNB deployment needs further clarification, i.e. per layer deployment or mixed deployment across layers.

Proposal 2: Confirm the following working assumption with a unified minimum UE-UE(2D) distance across different scnarios:

Working Assumption

| Parameters | Indoor office | Urban macro / Dense Urban Macro layer | Dense Urban with 2-layer |
| --- | --- | --- | --- |
| Layout | Single layer Indoor floor: (12BSs per 120m x 50m) | Single layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around. | Two layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around. Micro layer: According to previous agreement - Baseline: 3 Micro BSs per Macro BS - Optional: 6, or 9 Micro BSs per Macro BS |
| Inter-BS (2D) distance | 20m [TR 38.802 Table A.2.1-11] | 500m for Urban Macro [TR 38.802 Table A.2.1-11] 200m for Dense Urban Macro layer [TR 38.802 Table A.2.1-1] | Macro-to-macro: 200m Minimum Macro-to-micro-center distance: 105m Minimum Micro-center-to-micro-center distance: 57.9m |
| Minimum BS-UE (2D) distance | 0m [TR 38.802 Table A.2.1-11] | 35m [TR 38.802 Table A.2.1-11] | Macro-to-UE: 35m Micro-to-UE: 10m [TR 38.802 Table A.2.1-11] |
| Minimum UE-UE (2D) distance | FFS 3m [TR 38.802 Table A.2.1-11] | FFS :3m [TR 38.802 Table A.2.1-11] | FFS: 3m [TR 38.802 Table A.2.1-11] |
| BS antenna height | 3 m [TR 38.802 Table A.2.1-1] | 25 m [TR 38.802 Table A.2.1-1] | 25m for macro cells and 10m for micro cells [TR 38.802 Table A.2.1-1] |

Proposal 3: For UE distribution of Urban Macro and Dense Urban Macro layer, the following parameter values can be applied to cluster based UE distribution at least for FR1:

- Dmacro-to-cluster=35+28.9m

- Dinter-cluster = 57.9 m

- R = 28.9m

- Indoor UE height = 1.5m

Proposal 4: XR traffic models can be considered for SBFD and dynamic/flexible TDD evaluation.

Proposal 5:  The definition provided in Table 1 is adopted for SBFD and dynamic/flexible TDD evaluation.

Proposal 6: Dynamic TDD is not used for legacy TDD for comparison.

Proposal 7: Confirm the two working assumptions related to gNB-to-gNB channel model and UE-to-UE channel model achieved in RAN1#110 meeting.

Proposal 8: For flexible/dynamic TDD, evaluate and study the performance in HetNet scenario.

Reference

  - RAN1 Chairman notes, RAN1#110 meeting

  - 3GPP TR 38.802 V14.0.0

  - R4-2214376, Reply LS on interference modelling for duplex evolution, RAN4

  - RP-213591, New SI: Study on evolution of NR duplex operation, CMCC

  - TR38.828, Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR, v16.1.0

Appendix

Simulation assumptions for SBFD SLS

Table 5-1: Simulation parameters for dense urban

| Parameters | Values |
| --- | --- |
| Layout | Single layer with 7 hexagonal cell with wrap around |
| Minimum BS-UE (2D) distance | 35 m: TR38.828 |
| Minimum UE-UE (2D) distance | 3 m: TR38.828 |
| Inter-BS distance | 200 m |
| Carrier frequency | 4 GHz |
| BS Tx power | 44 dBm for 100MHz [refer to TR 38.802 Table A.2.1-1] |
| UE Tx power | 23 dBm for 100MHz TR38.802 |
| Path-loss model | -Macro(Aggressor) → Macro(Victim) - BS-to-BS: TR38.901 - BS-to-UE: TR38.901 - UE-to-UE: Option 1: A.2.1.2 in TR36.843(*), penetration loss between UEs follows Table A.2.1-13 in TR38.802 |
| BS antenna configurations | For legacy TDD gNB: M,N,P,Mg,Ng;Mp,Np= (8,8,2,1,1;2,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization For SBFD gNB: - SBFD antenna configuration option-2 (Method 2-1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (8,8,2,1,1). - Number of TxRUs: same as legacy TDD dH,dV = (0.5, 0.8)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ |
| BS antenna height | 25 m: TR38.802 |
| BS antenna element gain + connector loss | 5 dBi : TR38.828 |
| BS receiver noise figure | 5 dB: TR38.802 |
| UE antenna configuration | Omni: TR38.802 |
| UE antenna height | hUT=3(nfl-1)+1.5 nfl for outdoor UEs: 1 nfl for indoor UEs: nfl~uniform(1,Nfl) where Nfl ~uniform(4,8) TR36.873 |
| UE antenna gain | 0 dBi: TR38.828 |
| UE receiver noise figure | 9 dB: TR38.802 |
| UE power control | Power control as defined in TR 37.910, (p0=-86dBm, alpha=0.9) |
| Cell selection criteria | Cell selection is based on coupling loss |
| Traffic model | FTP model 3 Packet size: 0.5Mbytes Arrivial rate(DL/UL): 0.2/0.1, 1.5/0.25, 3/0.5 |
| Output | UL/DL UPT (5%,50%, 95%), UL/DL Latency, Resource utilization |
| TDD UL-DL configuration | DDDSU |
| Subband configuration | XXXXU, DXXXU; Structure of flexible slot: 12D2G0U UL subband portion in system bandwidth: 50% |
| SI suppression capability | Advanced capability#1: 145 dB Advanced capability#2: 185 dB Note: legacy gNB-gNB ACIR is incorporated in the above capability |
| Other assumption | Reuse SI suppression capability for inter-sector CLI |
| Antenna architecture | The number of antenna element for SBFD gNB is two times of that for legacy TDD gNB. The number of antenna element for each direction is same as that of legacy TDD |
| Simulaltion bandwidth | 100MHz Agreement Note: Tx power needs to be scaled accordingly |
| UE distribution | Clustering based, refer to chairman notes. Number of UE cluster=1, Dmacro-to-cluster=35+Micro Radius, Dinter-cluster,=2* Micro Radius Note: Micro Radius =28.9 |
| gNB-to-gNB LOS probability | If the 2D distance between two Macro gNBs are less than or equal to the ISD (200m for Dense Urban, and 500m for Urban Macro), set the LOS probability to X; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. - X = 0.75 |
| Number of UEs per Macro TRP | 20 Note: 20% UEs are distributed in Macro area 80% UEs are distributed in UE clusters |

Table 5-2: BS antenna element pattern for dense urban

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=65°,SLAV=25dB |
| Antenna element horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=65°,Am=25dB |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi (assuming 1.8dB loss) |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note 3: A 65 degree horizontal element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation results. |  |

Table 5-1: Simulation parameters for indoor

| Parameters | Indoor |
| --- | --- |
| Layout | Single layer with 12 BSs per 120m x 50m |
| Minimum BS-UE (2D) distance | 0 m: TR38.802 |
| Minimum UE-UE (2D) distance | 1 m: TR38.828 |
| Inter-BS distance | 20 m TR38.802 |
| Carrier frequency | 4 GHz |
| BS Tx power | 24 dBm for 100MHz |
| UE Tx power | 23 dBm: TR38.828 |
| Path-loss model | - BS-to-BS: TR38.901 - BS-to-UE: TR38.901 - UE-to-UE: Option 1: A.2.1.2 in TR36.843(*), penetration loss between UEs follows Table A.2.1-13 in TR38.802 |
| BS antenna configurations | For legacy TDD gNB: M,N,P,Mg,Ng;Mp,Np= (4,4,2,1,1;4,4) dH,dV = (0.5, 0.5)λ, +45°/-45° polarization For SBFD gNB: - SBFD antenna configuration option-2 (Method 2-1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (4,4,2,1,1). - Number of TxRUs: same as legacy TDD dH,dV = (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ |
| BS antenna height | 3 m: TR38.802 |
| BS antenna element gain + connector loss | 3.5 dBi : TR38.828 |
| BS receiver noise figure | 5 dB: TR38.802 |
| UE antenna configuration | Omni: TR38.802 |
| UE antenna height | hUT=1.5 m TR38.802 |
| UE antenna gain | 0 dBi: TR38.828 |
| UE receiver noise figure | 9 dB: TR38.802 |
| UE power control | Power control as defined in TR37.910, (p0=-60dBm, alpha=0.6) |
| Cell selection criteria | Cell selection is based on coupling loss |
| Traffic model | FTP model 3 Packet size: 0.5Mbytes Arrivial rate(DL/UL): 0.8/0.2, 2.5/0.6, 4.5/ 1.1 |
| Output | UL/DL UPT (5%,50%, 95%), UL/DL Latency, Resource utilization |
| TDD UL-DL configuration | DDDSU |
| Subband configuration | XXXXU, DXXXU; Structure of flexible slot: 12D2G0U UL subband portion in system bandwidth: 50% |
| SI suppression capability | Advanced capability#1: 145 dB Advanced capability#2: 185 dB Note: legacy gNB-gNB ACIR is incorporated in the above capability |
| Other assumption | Reuse SI supporession capability for inter-sector CLI |
| Antenna architecture | The number of antenna element for SBFD gNB is two times of that for legacy TDD gNB. The number of antenna element for each direction is same as that of legacy TDD |
| Simulaltion bandwidth | 100MHz Note: Tx power needs to be scaled accordingly |
| UE distribution | 100% Indoor |
| gNB-to-gNB LOS probability | TR38.901 |
| gNB-to-UE LOS probability | TR38.901 |
| Number of UEs per Macro TRP | 10 UEs per BS TR38.802 |

Simulation results for SBFD SLS

Simulation results for Dense urban Macro scenrio

DL/UL resource utilization

Table 1: UL resource utilization for Dense urban scenario

| Simulation cases | Low RU | Medium RU | High RU |
| --- | --- | --- | --- |
| Baseline-DDDSU | 3.3% | 24.7% | 53% |
| SBFD-XXXXU-UL50%-145dB | 6.9% | 55% | 95.8% |
| SBFD-DXXXU-UL50%-145dB | 5.6% | 43.8% | 86.5% |
| SBFD-XXXXU-UL50%-185dB | 7% | 56% | 95% |
| SBFD-DXXXU-UL50%-185dB | 5.8% | 55.9% | 86.5% |

Table 2: DL resource utilization for Dense urban scenario

| Simulation cases | Low RU | Medium RU | High RU |
| --- | --- | --- | --- |
| Baseline-DDDSU | 10% | 23.7% | 50.3% |
| SBFD-XXXXU-UL50%-145dB | 3% | 9.7% | 22.4% |
| SBFD-DXXXU-UL50%-145dB | 3.9% | 10.3% | 24.8% |
| SBFD-XXXXU-UL50%-185dB | 3% | 9.3% | 21% |
| SBFD-DXXXU-UL50%-185dB | 3.8% | 9.3% | 24.1% |

UPT

Table 3: UL Mean/5%/50%/95% Average-UPT assuming Low RU for legacy TDD

| Simulation cases | Mean(Mbps) | 5%( Mbps) | 50%( Mbps) | 95%( Mbps) |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 101.55 | 56.2 | 114.27 | 120.35 |
| SBFD-XXXXU-UL50%-145dB | 278.66 | 200.79 | 289.8 | 295.15 |
| SBFD-DXXXU-UL50%-145dB | 224.23 | 146.14 | 233.43 | 240.54 |
| SBFD-XXXXU-UL50%-185dB | 274.11 | 179.34 | 289.79 | 295.06 |
| SBFD-DXXXU-UL50%-185dB | 220.48 | 144.33 | 233.23 | 241.02 |

Table 4: UL Mean/5%/50%/95% Average-UPT assuming medium RU for legacy TDD

| Simulation cases | mean(Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 92.35 | 44.14 | 97.99 | 118.8 |
| SBFD-XXXXU-UL50%-145dB | 260.71 | 133.91 | 288.59 | 294.7 |
| SBFD-DXXXU-UL50%-145dB | 218.51 | 144.77 | 233.18 | 240.51 |
| SBFD-XXXXU-UL50%-185dB | 264.63 | 170.42 | 288.95 | 294.15 |
| SBFD-DXXXU-UL50%-185dB | 264.63 | 170.42 | 288.95 | 294.15 |

Table 5: UL Mean/5%/50%/95% Average-UPT assuming high RU for legacy TDD

| Simulation cases | mean(Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 79.21 | 22.69 | 83.66 | 117.32 |
| SBFD-XXXXU-UL50%-145dB | 242.73 | 124.33 | 274.08 | 292.82 |
| SBFD-DXXXU-UL50%-145dB | 203.62 | 113.15 | 229.71 | 238.29 |
| SBFD-XXXXU-UL50%-185dB | 247.06 | 129.24 | 280.96 | 294.32 |
| SBFD-DXXXU-UL50%-185dB | 207.19 | 113.54 | 228.9 | 238.04 |

Table 6: DL Mean/5%/50%/95% Average-UPT assuming low RU for legacy TDD

| Simulation cases | mean(Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 438.29 | 377.01 | 446.07 | 463.82 |
| SBFD-XXXXU-UL50%-145dB | 223.82 | 145.07 | 233.8 | 240.38 |
| SBFD-DXXXU-UL50%-145dB | 281.27 | 219.98 | 289.81 | 301.25 |
| SBFD-XXXXU-UL50%-185dB | 223.03 | 170.46 | 233.4 | 240.38 |
| SBFD-DXXXU-UL50%-185dB | 277.5 | 219.41 | 289.08 | 298.84 |

Table 7: DL Mean/5%/50%/95% Average-UPT assuming medium RU for legacy TDD

| Simulation cases | mean(Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 365.47 | 287.67 | 371.53 | 429.8 |
| SBFD-XXXXU-UL50%-145dB | 156.8 | 87.03 | 156.75 | 224.43 |
| SBFD-DXXXU-UL50%-145dB | 212.7 | 116.68 | 218.48 | 284.08 |
| SBFD-XXXXU-UL50%-185dB | 153.63 | 72.97 | 159.2 | 224.28 |
| SBFD-DXXXU-UL50%-185dB | 153.63 | 72.97 | 159.2 | 224.28 |

Table 8: DL Mean/5%/50%/95% Average-UPT assuming high RU for legacy TDD

| Simulation cases | mean(Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 272.3 | 151.21 | 278.7 | 382.82 |
| SBFD-XXXXU-UL50%-145dB | 50.76 | 1.96 | 36.95 | 136.57 |
| SBFD-DXXXU-UL50%-145dB | 95.94 | 3.53 | 90.97 | 208.9 |
| SBFD-XXXXU-UL50%-185dB | 52.88 | 2.26 | 41.86 | 132.68 |
| SBFD-DXXXU-UL50%-185dB | 100.35 | 4.73 | 90.69 | 215.72 |

Table 9: UL Mean/5%/50%/95% Medium-UPT assuming low RU for legacy TDD

| Simulation cases | mean(Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 102.98 | 56.2 | 114.3 | 120.67 |
| SBFD-XXXXU-UL50%-145dB | 279.11 | 200.79 | 290.45 | 295.5 |
| SBFD-DXXXU-UL50%-145dB | 225.04 | 146.17 | 243.31 | 241.41 |
| SBFD-XXXXU-UL50%-185dB | 275.04 | 178.8 | 290.09 | 295.44 |
| SBFD-DXXXU-UL50%-185dB | 219.7 | 144.33 | 231.72 | 241.02 |

Table 10: UL Mean/5%/50%/95% Medium-UPT assuming medium RU for legacy TDD

| Simulation cases | mean(Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 96.73 | 43.39 | 110.93 | 119.81 |
| SBFD-XXXXU-UL50%-145dB | 261.4 | 133.95 | 289.14 | 294.99 |
| SBFD-DXXXU-UL50%-145dB | 219.83 | 144.47 | 233.51 | 241.34 |
| SBFD-XXXXU-UL50%-185dB | 265.46 | 170.42 | 289.08 | 294.97 |
| SBFD-DXXXU-UL50%-185dB | 262.17 | 167.69 | 286.91 | 293.7 |

Table 11: UL Mean/5%/50%/95% Medium-UPT assuming high RU for legacy TDD

| Simulation cases | mean(Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 84.9 | 20.65 | 95.32 | 119.58 |
| SBFD-XXXXU-UL50%-145dB | 247.06 | 124.33 | 281.69 | 294.47 |
| SBFD-DXXXU-UL50%-145dB | 205.76 | 113.15 | 230.38 | 240.51 |
| SBFD-XXXXU-UL50%-185dB | 250.09 | 130.7 | 287.11 | 294.97 |
| SBFD-DXXXU-UL50%-185dB | 201.92 | 107 | 222.75 | 236.92 |

Table 12: DL Mean/5%/50%/95% Medium-UPT assuming low RU for legacy TDD

| Simulation cases | mean(Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 445.89 | 425.03 | 449.89 | 467.79 |
| SBFD-XXXXU-UL50%-145dB | 228.93 | 148.51 | 234.83 | 241.43 |
| SBFD-DXXXU-UL50%-145dB | 288.16 | 278.9 | 292 | 205.17 |
| SBFD-XXXXU-UL50%-185dB | 230.01 | 198.02 | 234.56 | 241.66 |
| SBFD-DXXXU-UL50%-185dB | 286.75 | 277.03 | 291.67 | 303.74 |

Table 13: DL Mean/5%/50%/95% Medium-UPT assuming medium RU for legacy TDD

| Simulation cases | mean(Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 409.56 | 241.67 | 433.75 | 452.67 |
| SBFD-XXXXU-UL50%-145dB | 173.18 | 69.01 | 186.51 | 235.17 |
| SBFD-DXXXU-UL50%-145dB | 237.2 | 107.89 | 276.33 | 294.04 |
| SBFD-XXXXU-UL50%-185dB | 172.09 | 62.87 | 193.45 | 243.75 |
| SBFD-DXXXU-UL50%-185dB | 172.09 | 62.87 | 193.45 | 234.75 |

Table 14: DL Mean/5%/50%/95% Medium-UPT assuming high RU for legacy TDD

| Simulation cases | mean(Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 281.38 | 106.73 | 258.21 | 438.43 |
| SBFD-XXXXU-UL50%-145dB | 43.11 | 1.31 | 21.75 | 156.08 |
| SBFD-DXXXU-UL50%-145dB | 93.4 | 2.61 | 71.69 | 276.31 |
| SBFD-XXXXU-UL50%-185dB | 44.68 | 1.69 | 23.66 | 159.98 |
| SBFD-DXXXU-UL50%-185dB | 95.87 | 3.6 | 82.61 | 264.57 |

Table 15: UL Tail-UPT assuming Low RU for legacy TDD

| Simulation cases | mean(Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 99.92 | 53.55 | 113.81 | 120.35 |
| SBFD-XXXXU-UL50%-145dB | 278.04 | 200.79 | 289.17 | 295.15 |
| SBFD-DXXXU-UL50%-145dB | 223.13 | 144.72 | 231.48 | 240.48 |
| SBFD-XXXXU-UL50%-185dB | 272.43 | 178.11 | 288.63 | 295.06 |
| SBFD-DXXXU-UL50%-185dB | 219.7 | 144.33 | 231.72 | 241.02 |

Table 16: UL Tail-UPT assuming medium RU for legacy TDD

| Simulation cases | mean(Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 84.42 | 30.06 | 89.89 | 118.68 |
| SBFD-XXXXU-UL50%-145dB | 259.08 | 133.91 | 287.15 | 293.9 |
| SBFD-DXXXU-UL50%-145dB | 216.16 | 139.75 | 230.77 | 240.1 |
| SBFD-XXXXU-UL50%-185dB | 262.17 | 167.69 | 286.91 | 293.7 |
| SBFD-DXXXU-UL50%-185dB | 262.17 | 167.69 | 286.91 | 293.7 |

Table 17: UL Tail-UPT assuming high RU for legacy TDD

| Simulation cases | mean(Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 65.01 | 13.47 | 60.46 | 116.41 |
| SBFD-XXXXU-UL50%-145dB | 234.18 | 112.8 | 262.78 | 292.27 |
| SBFD-DXXXU-UL50%-145dB | 197.22 | 93.29 | 223.41 | 238.19 |
| SBFD-XXXXU-UL50%-185dB | 238.87 | 96.54 | 270.83 | 294.32 |
| SBFD-DXXXU-UL50%-185dB | 201.92 | 107 | 222.75 | 236.92 |

Table 18: DL Tail-UPT assuming low RU for legacy TDD

| Simulation cases | mean(Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 420.54 | 237.41 | 437.98 | 462.96 |
| SBFD-XXXXU-UL50%-145dB | 212.97 | 119.04 | 230.97 | 240.33 |
| SBFD-DXXXU-UL50%-145dB | 266.53 | 148.05 | 286.2 | 300.95 |
| SBFD-XXXXU-UL50%-185dB | 209.2 | 118.75 | 230.71 | 239.68 |
| SBFD-DXXXU-UL50%-185dB | 259.52 | 147.96 | 285.89 | 296.83 |

Table 19: DL Tail-UPT assuming medium RU for legacy TDD

| Simulation cases | mean(Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 176.76 | 70.51 | 158.54 | 361.99 |
| SBFD-XXXXU-UL50%-145dB | 72.5 | 18.19 | 58.44 | 221.74 |
| SBFD-DXXXU-UL50%-145dB | 102.33 | 24.09 | 80.96 | 275.03 |
| SBFD-XXXXU-UL50%-185dB | 74.48 | 15.84 | 60.94 | 219.69 |
| SBFD-DXXXU-UL50%-185dB | 102.68 | 27.84 | 76.8 | 276.04 |

Table 20: DL Tail-UPT assuming high RU for legacy TDD

| Simulation cases | mean(Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 90.57 | 22.61 | 75.39 | 229.79 |
| SBFD-XXXXU-UL50%-145dB | 14.89 | 0.86 | 4.63 | 65.25 |
| SBFD-DXXXU-UL50%-145dB | 33.24 | 1.29 | 18.5 | 129.62 |
| SBFD-XXXXU-UL50%-185dB | 16.01 | 0.84 | 5.67 | 61.69 |
| SBFD-DXXXU-UL50%-185dB | 34.14 | 1.27 | 15.13 | 144.76 |

Latency

Table 21: UL Mean/5%/50%/95% Packet-Latency assuming low RU for legacy TDD

| Simulation cases | Mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.042179 | 0.033135 | 0.034994 | 0.074692 |
| SBFD-XXXXU-UL50%-145dB | 0.014403 | 0.013539 | 0.013803 | 0.019603 |
| SBFD-DXXXU-UL50%-145dB | 0.01795 | 0.016596 | 0.017142 | 0.024887 |
| SBFD-XXXXU-UL50%-185dB | 0.015266 | 0.013552 | 0.013812 | 0.022454 |
| SBFD-DXXXU-UL50%-185dB | 0.018651 | 0.016553 | 0.017165 | 0.027574 |

Table 22: UL Mean/5%/50%/95% Packet-Latency assuming medium RU for legacy TDD

| Simulation cases- Low RU for legacy TDD | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.049259 | 0.033371 | 0.03606 | 0.0958 |
| SBFD-XXXXU-UL50%-145dB | 0.014403 | 0.013539 | 0.013803 | 0.019603 |
| SBFD-DXXXU-UL50%-145dB | 0.018879 | 0.016607 | 0.017177 | 0.027687 |
| SBFD-XXXXU-UL50%-185dB | 0.019023 | 0.013558 | 0.013836 | 0.022485 |
| SBFD-DXXXU-UL50%-185dB | 0.019023 | 0.013558 | 0.013836 | 0.022485 |

Table 23: UL Mean/5%/50%/95% Packet-Latency assuming high RU for legacy TDD

| Simulation cases- Low RU for legacy TDD | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.103945 | 0.03339 | 0.041886 | 0.217036 |
| SBFD-XXXXU-UL50%-145dB | 0.039913 | 0.013537 | 0.013907 | 0.029647 |
| SBFD-DXXXU-UL50%-145dB | 0.063596 | 0.016614 | 0.017273 | 0.031152 |
| SBFD-XXXXU-UL50%-185dB | 0.035126 | 0.013537 | 0.013904 | 0.029597 |
| SBFD-DXXXU-UL50%-185dB | 0.024032 | 0.016565 | 0.017294 | 0.032234 |

Table 24: DL Mean/5%/50%/95% Packet-Latency assuming low RU for legacy TDD

| Simulation cases- Low RU for legacy TDD | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.009319 | 0.008541 | 0.008947 | 0.011555 |
| SBFD-XXXXU-UL50%-145dB | 0.018648 | 0.016564 | 0.017123 | 0.033148 |
| SBFD-DXXXU-UL50%-145dB | 0.014701 | 0.013098 | 0.013784 | 0.026168 |
| SBFD-XXXXU-UL50%-185dB | 0.018784 | 0.016571 | 0.017141 | 0.033308 |
| SBFD-DXXXU-UL50%-185dB | 0.015039 | 0.013132 | 0.013802 | 0.026708 |

Table 25: DL Mean/5%/50%/95% Packet-Latency assuming medium RU for legacy TDD

| Simulation cases- Low RU for legacy TDD | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.0013218 | 0.008569 | 0.009265 | 0.027882 |
| SBFD-XXXXU-UL50%-145dB | 0.039924 | 0.016683 | 0.024986 | 0.115941 |
| SBFD-DXXXU-UL50%-145dB | 0.027767 | 0.013288 | 0.014864 | 0.076403 |
| SBFD-XXXXU-UL50%-185dB | 0.042142 | 0.016672 | 0.025877 | 0.123794 |
| SBFD-DXXXU-UL50%-185dB | 0.042142 | 0.016672 | 0.025877 | 0.123794 |

Table 26: DL Mean/5%/50%/95% Packet-Latency assuming high RU for legacy TDD

| Simulation cases- Low RU for legacy TDD | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.0025454 | 0.008633 | 0.016623 | 0.076927 |
| SBFD-XXXXU-UL50%-145dB | 0.747691 | 0.017438 | 0.168415 | 3.621043 |
| SBFD-DXXXU-UL50%-145dB | 0.273034 | 0.013665 | 0.055241 | 1.528552 |
| SBFD-XXXXU-UL50%-185dB | 0.680729 | 0.017683 | 0.153597 | 3.194878 |
| SBFD-DXXXU-UL50%-185dB | 0.0269789 | 0.013672 | 0.048858 | 1.35869 |

Table 27: DL Mean/5%/50%/95% UE-Average-Latency assuming low RU for legacy TDD

| Simulation cases- Low RU for legacy TDD | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.009283 | 0.008633 | 0.00898 | 0.011591 |
| SBFD-XXXXU-UL50%-145dB | 0.018752 | 0.01664 | 0.017112 | 0.027979 |
| SBFD-DXXXU-UL50%-145dB | 0.014671 | 0.01328 | 0.013806 | 0.020211 |
| SBFD-XXXXU-UL50%-185dB | 0.018677 | 0.01664 | 0.01714 | 0.02557 |
| SBFD-DXXXU-UL50%-185dB | 0.014979 | 0.01339 | 0.013845 | 0.020395 |

Table 28: DL Mean/5%/50%/95% UE-Average-Latency assuming medium RU for legacy TDD

| Simulation cases- Low RU for legacy TDD | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.013159 | 0.009489 | 0.012371 | 0.019211 |
| SBFD-XXXXU-UL50%-145dB | 0.040292 | 0.017844 | 0.033364 | 0.081089 |
| SBFD-DXXXU-UL50%-145dB | 0.027547 | 0.014093 | 0.023504 | 0.058584 |
| SBFD-XXXXU-UL50%-185dB | 0.043989 | 0.017961 | 0.033868 | 0.099848 |
| SBFD-DXXXU-UL50%-185dB | 0.043989 | 0.017961 | 0.033868 | 0.099848 |

Table 29: DL Mean/5%/50%/95% UE-Average-Latency assuming high RU for legacy TDD

| Simulation cases- Low RU for legacy TDD | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.025867 | 0.011666 | 0.021283 | 0.057492 |
| SBFD-XXXXU-UL50%-145dB | 0.802625 | 0.043538 | 0.355067 | 2.981509 |
| SBFD-DXXXU-UL50%-145dB | 0.348698 | 0.0222 | 0.096277 | 1.930077 |
| SBFD-XXXXU-UL50%-185dB | 0.716461 | 0.040516 | 0.294553 | 2.541553 |
| SBFD-DXXXU-UL50%-185dB | 0.331382 | 0.020828 | 0.107889 | 1.543485 |

Table 30: UL Mean/5%/50%/95% UE-Average-Latency assuming low RU for legacy TDD

| Simulation cases- Low RU for legacy TDD | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.042978 | 0.033237 | 0.035057 | 0.071179 |
| SBFD-XXXXU-UL50%-145dB | 0.014616 | 0.013553 | 0.013803 | 0.019921 |
| SBFD-DXXXU-UL50%-145dB | 0.018274 | 0.016629 | 0.017142 | 0.027373 |
| SBFD-XXXXU-UL50%-185dB | 0.015199 | 0.013556 | 0.013804 | 0.022305 |
| SBFD-DXXXU-UL50%-185dB | 0.018984 | 0.016596 | 0.01715 | 0.027715 |

Table 31: UL Mean/5%/50%/95% UE-Average-Latency assuming medium RU for legacy TDD

| Simulation cases- Low RU for legacy TDD | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.051137 | 0.03367 | 0.042706 | 0.093024 |
| SBFD-XXXXU-UL50%-145dB | 0.016732 | 0.013573 | 0.01386 | 0.029871 |
| SBFD-DXXXU-UL50%-145dB | 0.01948 | 0.016632 | 0.017161 | 0.027839 |
| SBFD-XXXXU-UL50%-185dB | 0.017786 | 0.013599 | 0.013845 | 0.023471 |
| SBFD-DXXXU-UL50%-185dB | 0.017786 | 0.013599 | 0.013845 | 0.023471 |

Table 32: UL Mean/5%/50%/95% UE-Average-Latency assuming high RU for legacy TDD

| Simulation cases- Low RU for legacy TDD | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.109028 | 0.034114 | 0.053445 | 0.221303 |
| SBFD-XXXXU-UL50%-145dB | 0.059917 | 0.013662 | 0.014601 | 0.034598 |
| SBFD-DXXXU-UL50%-145dB | 0.053262 | 0.016787 | 0.017423 | 0.042875 |
| SBFD-XXXXU-UL50%-185dB | 0.033355 | 0.013591 | 0.014262 | 0.030951 |
| SBFD-DXXXU-UL50%-185dB | 0.026365 | 0.016808 | 0.01753 | 0.036968 |

- 5.2.2 Simulation results for Indoor/Hotspot scenrio

DL/UL resource utilization

Table 33: DL resource utilization for Indoor/Hotspot scenario

| Simulation cases | Low RU | Medium RU | High RU |
| --- | --- | --- | --- |
| Baseline-DDDSU | 6% | 22.9% | 49.1% |
| SBFD-XXXXU-UL50%-145dB | 13.8% | 58.2% | 94% |
| SBFD-DXXXU-UL50%-145dB | 11.3% | 44.4% | 88.1% |
| SBFD-XXXXU-UL50%-185dB | 13.8% | 58.1% | 94% |
| SBFD-DXXXU-UL50%-185dB | 11.4% | 44.4% | 88.1% |

Table 34: UL resource utilization for Indoor/Hotspot scenario

| Simulation cases | Low RU | Medium RU | High RU |
| --- | --- | --- | --- |
| Baseline-DDDSU | 8.8% | 23.1% | 54.5% |
| SBFD-XXXXU-UL50%-145dB | 2.1% | 6.5% | 12.3% |
| SBFD-DXXXU-UL50%-145dB | 2.6% | 7.7% | 16% |
| SBFD-XXXXU-UL50%-185dB | 2% | 6.5% | 12.2% |
| SBFD-DXXXU-UL50%-185dB | 2.6% | 7.7% | 16% |

UPT

Table 35: DL Average-UPT assuming low RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 425.78 | 363.7 | 437.56 | 456.51 |
| SBFD-XXXXU-UL50%-145dB | 210.35 | 158.59 | 216.18 | 236.37 |
| SBFD-DXXXU-UL50%-145dB | 267.53 | 210.05 | 278.02 | 295.48 |
| SBFD-XXXXU-UL50%-185dB | 210.35 | 158.59 | 216.18 | 236.37 |
| SBFD-DXXXU-UL50%-185dB | 267.53 | 210.05 | 278.02 | 295.48 |

Table 36: DL Average-UPT assuming medium RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 371.98 | 293.74 | 378.98 | 430.45 |
| SBFD-XXXXU-UL50%-145dB | 123.95 | 52.13 | 123.8 | 193.83 |
| SBFD-DXXXU-UL50%-145dB | 197.77 | 97.26 | 211.63 | 265.45 |
| SBFD-XXXXU-UL50%-185dB | 123.95 | 52.13 | 123.8 | 193.83 |
| SBFD-DXXXU-UL50%-185dB | 197.77 | 97.26 | 211.63 | 265.45 |

Table 37: DL Average-UPT assuming high RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 256.74 | 123.41 | 259.01 | 358.71 |
| SBFD-XXXXU-UL50%-145dB | 36.85 | 3.2 | 26.8 | 106.43 |
| SBFD-DXXXU-UL50%-145dB | 68.17 | 4.64 | 51.49 | 174.03 |
| SBFD-XXXXU-UL50%-185dB | 36.85 | 3.2 | 26.8 | 106.43 |
| SBFD-DXXXU-UL50%-185dB | 68.17 | 4.64 | 51.49 | 174.03 |

Table 38: UL Average-UPT assuming low RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 109.24 | 67.58 | 115.38 | 119.6 |
| SBFD-XXXXU-UL50%-145dB | 286.98 | 264.26 | 290.33 | 294.45 |
| SBFD-DXXXU-UL50%-145dB | 233.33 | 228.63 | 233.49 | 239.82 |
| SBFD-XXXXU-UL50%-185dB | 286.98 | 264.26 | 290.33 | 294.45 |
| SBFD-DXXXU-UL50%-185dB | 233.33 | 228.63 | 233.49 | 239.82 |

Table 39: UL Average-UPT assuming medium RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 99.14 | 56.77 | 109.23 | 120.21 |
| SBFD-XXXXU-UL50%-145dB | 273.78 | 187.54 | 289.53 | 294.05 |
| SBFD-DXXXU-UL50%-145dB | 229.19 | 207.42 | 232.63 | 237.87 |
| SBFD-XXXXU-UL50%-185dB | 273.78 | 187.54 | 289.53 | 294.05 |
| SBFD-DXXXU-UL50%-185dB | 229.19 | 207.42 | 232.63 | 237.87 |

Table 40: UL Average-UPT assuming high RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 90.51 | 26.27 | 95.57 | 119.14 |
| SBFD-XXXXU-UL50%-145dB | 273.29 | 213.21 | 286.78 | 293.09 |
| SBFD-DXXXU-UL50%-145dB | 215.76 | 116.52 | 230.84 | 238.55 |
| SBFD-XXXXU-UL50%-185dB | 273.29 | 213.21 | 286.78 | 293.09 |
| SBFD-DXXXU-UL50%-185dB | 215.76 | 116.52 | 230.84 | 238.55 |

Table 41: DL Median-UPT assuming low RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 444.36 | 429.19 | 445.86 | 465.62 |
| SBFD-XXXXU-UL50%-145dB | 225.35 | 160.09 | 233.06 | 238.38 |
| SBFD-DXXXU-UL50%-145dB | 284.36 | 241.93 | 289.08 | 300.43 |
| SBFD-XXXXU-UL50%-185dB | 225.35 | 160.09 | 233.06 | 238.38 |
| SBFD-DXXXU-UL50%-185dB | 284.36 | 241.93 | 289.08 | 300.43 |

Table 42: DL Median-UPT assuming medium RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 416.86 | 329.47 | 431.84 | 451.48 |
| SBFD-XXXXU-UL50%-145dB | 137.88 | 53.13 | 135.59 | 231.76 |
| SBFD-DXXXU-UL50%-145dB | 219.1 | 92.77 | 242.17 | 291.59 |
| SBFD-XXXXU-UL50%-185dB | 137.88 | 53.13 | 135.59 | 231.76 |
| SBFD-DXXXU-UL50%-185dB | 219.1 | 92.77 | 242.17 | 291.59 |

Table 43: DL Median-UPT assuming high RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 275.65 | 108.94 | 277.62 | 426.22 |
| SBFD-XXXXU-UL50%-145dB | 29.58 | 1.61 | 14.17 | 110.74 |
| SBFD-DXXXU-UL50%-145dB | 62.24 | 2.07 | 31.52 | 211.69 |
| SBFD-XXXXU-UL50%-185dB | 29.58 | 1.61 | 14.17 | 110.74 |
| SBFD-DXXXU-UL50%-185dB | 62.24 | 2.07 | 31.52 | 211.69 |

Table 44: UL Median-UPT assuming low RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 111.26 | 60.5 | 115.85 | 120.28 |
| SBFD-XXXXU-UL50%-145dB | 290.19 | 286.46 | 291.44 | 294.9 |
| SBFD-DXXXU-UL50%-145dB | 234.75 | 229.05 | 234.13 | 241.68 |
| SBFD-XXXXU-UL50%-185dB | 290.19 | 286.46 | 291.44 | 294.9 |
| SBFD-DXXXU-UL50%-185dB | 234.75 | 229.05 | 234.13 | 241.68 |

Table 45: UL Median-UPT assuming medium RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 105.79 | 58.09 | 115.54 | 121.05 |
| SBFD-XXXXU-UL50%-145dB | 279.93 | 230.36 | 291.09 | 294.86 |
| SBFD-DXXXU-UL50%-145dB | 233.82 | 228.92 | 233.98 | 240.11 |
| SBFD-XXXXU-UL50%-185dB | 279.93 | 230.36 | 291.09 | 294.86 |
| SBFD-DXXXU-UL50%-185dB | 233.82 | 228.92 | 233.98 | 240.11 |

Table 46: UL Median-UPT assuming high RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 94.99 | 26.27 | 107.03 | 120.64 |
| SBFD-XXXXU-UL50%-145dB | 280.98 | 253.72 | 290.25 | 294.28 |
| SBFD-DXXXU-UL50%-145dB | 225.12 | 180.24 | 232.99 | 239.9 |
| SBFD-XXXXU-UL50%-185dB | 280.98 | 253.72 | 290.25 | 294.28 |
| SBFD-DXXXU-UL50%-185dB | 225.12 | 180.24 | 232.99 | 239.9 |

Table 47: DL Tail-UPT assuming low RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 346.83 | 189.59 | 421.34 | 443.86 |
| SBFD-XXXXU-UL50%-145dB | 165.35 | 70.11 | 162.01 | 233.54 |
| SBFD-DXXXU-UL50%-145dB | 217.96 | 124.68 | 253.13 | 291.32 |
| SBFD-XXXXU-UL50%-185dB | 165.35 | 70.11 | 162.01 | 233.54 |
| SBFD-DXXXU-UL50%-185dB | 217.96 | 124.68 | 253.13 | 291.32 |

Table 48: DL Tail-UPT assuming medium RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 201.85 | 82.65 | 174.88 | 390.34 |
| SBFD-XXXXU-UL50%-145dB | 54.34 | 13.31 | 45.49 | 144.01 |
| SBFD-DXXXU-UL50%-145dB | 108.74 | 34.22 | 88.44 | 254.9 |
| SBFD-XXXXU-UL50%-185dB | 54.34 | 13.31 | 45.49 | 144 |
| SBFD-DXXXU-UL50%-185dB | 108.74 | 34.22 | 88.44 | 254.9 |

Table 49: DL Tail-UPT assuming high RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 95.78 | 24.31 | 83.42 | 204.48 |
| SBFD-XXXXU-UL50%-145dB | 7.1 | 0.88 | 4.3 | 25.87 |
| SBFD-DXXXU-UL50%-145dB | 16.5 | 0.99 | 7.19 | 76.34 |
| SBFD-XXXXU-UL50%-185dB | 7.1 | 0.88 | 4.3 | 25.87 |
| SBFD-DXXXU-UL50%-185dB | 16.5 | 0.99 | 7.19 | 76.34 |

Table 50: UL Tail-UPT assuming low RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 104.91 | 59.44 | 114.35 | 119.6 |
| SBFD-XXXXU-UL50%-145dB | 281.48 | 232.42 | 288.42 | 294.45 |
| SBFD-DXXXU-UL50%-145dB | 230.02 | 228.25 | 231.3 | 239.81 |
| SBFD-XXXXU-UL50%-185dB | 281.48 | 232.42 | 288.42 | 294.45 |
| SBFD-DXXXU-UL50%-185dB | 230.02 | 228.25 | 231.3 | 239.81 |

Table 51: UL Tail-UPT assuming medium RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 88.34 | 35.9 | 105.19 | 120.21 |
| SBFD-XXXXU-UL50%-145dB | 255.15 | 65.27 | 286.43 | 292.65 |
| SBFD-DXXXU-UL50%-145dB | 217.37 | 138.52 | 229.41 | 235.78 |
| SBFD-XXXXU-UL50%-185dB | 255.15 | 65.27 | 286.43 | 292.65 |
| SBFD-DXXXU-UL50%-185dB | 217.37 | 138.52 | 229.41 | 235.78 |

Table 52: UL Tail-UPT assuming high RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 77.77 | 22.4 | 89.78 | 119.14 |
| SBFD-XXXXU-UL50%-145dB | 247.29 | 59.77 | 279.59 | 292.16 |
| SBFD-DXXXU-UL50%-145dB | 196.32 | 29.9 | 223.83 | 236.97 |
| SBFD-XXXXU-UL50%-185dB | 247.29 | 59.77 | 279.59 | 292.16 |
| SBFD-DXXXU-UL50%-185dB | 196.32 | 29.9 | 223.83 | 236.97 |

Latency

Table 53: DL Packet-Latency assuming low RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.009818 | 0.008543 | 0.008987 | 0.017043 |
| SBFD-XXXXU-UL50%-145dB | 0.020956 | 0.016604 | 0.017219 | 0.03721 |
| SBFD-DXXXU-UL50%-145dB | 0.015561 | 0.013176 | 0.013858 | 0.026993 |
| SBFD-XXXXU-UL50%-185dB | 0.020956 | 0.016604 | 0.017219 | 0.03721 |
| SBFD-DXXXU-UL50%-185dB | 0.015561 | 0.013176 | 0.013858 | 0.026993 |

Table 54: DL Packet-Latency assuming medium RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.012498 | 0.008575 | 0.009304 | 0.026125 |
| SBFD-XXXXU-UL50%-145dB | 0.049681 | 0.016959 | 0.031439 | 0.143762 |
| SBFD-DXXXU-UL50%-145dB | 0.025438 | 0.013357 | 0.016285 | 0.069085 |
| SBFD-XXXXU-UL50%-185dB | 0.049681 | 0.016959 | 0.031439 | 0.143762 |
| SBFD-DXXXU-UL50%-185dB | 0.025438 | 0.013357 | 0.016285 | 0.069085 |

Table 55: DL Packet-Latency assuming high RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.023363 | 0.008666 | 0.014283 | 0.071406 |
| SBFD-XXXXU-UL50%-145dB | 0.825653 | 0.026635 | 0.320204 | 3.227324 |
| SBFD-DXXXU-UL50%-145dB | 0.429219 | 0.014544 | 0.09899 | 2.047278 |
| SBFD-XXXXU-UL50%-185dB | 0.825653 | 0.026635 | 0.320204 | 3.227324 |
| SBFD-DXXXU-UL50%-185dB | 0.429219 | 0.014544 | 0.09899 | 2.047278 |

Table 56: UL Packet-Latency assuming low RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.03823 | 0.033446 | 0.03467 | 0.060737 |
| SBFD-XXXXU-UL50%-145dB | 0.020414 | 0.013551 | 0.013787 | 0.015892 |
| SBFD-DXXXU-UL50%-145dB | 0.017214 | 0.016552 | 0.017149 | 0.017484 |
| SBFD-XXXXU-UL50%-185dB | 0.020414 | 0.013551 | 0.013787 | 0.015892 |
| SBFD-DXXXU-UL50%-185dB | 0.017214 | 0.016552 | 0.017149 | 0.017484 |

Table 57: UL Packet-Latency assuming medium RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.045941 | 0.033112 | 0.03511 | 0.090156 |
| SBFD-XXXXU-UL50%-145dB | 0.057582 | 0.013531 | 0.013776 | 0.018479 |
| SBFD-DXXXU-UL50%-145dB | 0.017708 | 0.016593 | 0.017175 | 0.019432 |
| SBFD-XXXXU-UL50%-185dB | 0.057582 | 0.013531 | 0.013776 | 0.018479 |
| SBFD-DXXXU-UL50%-185dB | 0.017708 | 0.016593 | 0.017175 | 0.019432 |

Table 58: UL Packet-Latency assuming high RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.058903 | 0.033494 | 0.040213 | 0.152283 |
| SBFD-XXXXU-UL50%-145dB | 0.045707 | 0.013546 | 0.013817 | 0.020874 |
| SBFD-DXXXU-UL50%-145dB | 0.026882 | 0.016594 | 0.017214 | 0.028044 |
| SBFD-XXXXU-UL50%-185dB | 0.045707 | 0.013546 | 0.013817 | 0.020874 |
| SBFD-DXXXU-UL50%-185dB | 0.026882 | 0.016594 | 0.017214 | 0.028044 |

Table 59: DL UE-Average-Latency assuming low RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.009779 | 0.008776 | 0.009176 | 0.012419 |
| SBFD-XXXXU-UL50%-145dB | 0.020807 | 0.016924 | 0.018982 | 0.030894 |
| SBFD-DXXXU-UL50%-145dB | 0.015926 | 0.013548 | 0.01464 | 0.022562 |
| SBFD-XXXXU-UL50%-185dB | 0.020807 | 0.016924 | 0.018982 | 0.030894 |
| SBFD-DXXXU-UL50%-185dB | 0.015926 | 0.013548 | 0.01464 | 0.022562 |

Table 60: DL UE-Average-Latency assuming medium RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.012662 | 0.009392 | 0.011753 | 0.019897 |
| SBFD-XXXXU-UL50%-145dB | 0.053492 | 0.02263 | 0.042549 | 0.11865 |
| SBFD-DXXXU-UL50%-145dB | 0.028296 | 0.0152 | 0.022966 | 0.055362 |
| SBFD-XXXXU-UL50%-185dB | 0.053492 | 0.02263 | 0.042549 | 0.11865 |
| SBFD-DXXXU-UL50%-185dB | 0.028296 | 0.0152 | 0.022966 | 0.055362 |

Table 61: DL UE-Average-Latency assuming high RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.02492 | 0.012696 | 0.020676 | 0.050757 |
| SBFD-XXXXU-UL50%-145dB | 0.746885 | 0.065761 | 0.40866 | 2.396608 |
| SBFD-DXXXU-UL50%-145dB | 0.569057 | 0.030925 | 0.209084 | 2.215153 |
| SBFD-XXXXU-UL50%-185dB | 0.746885 | 0.065761 | 0.40866 | 2.396608 |
| SBFD-DXXXU-UL50%-185dB | 0.569057 | 0.030925 | 0.209084 | 2.215153 |

Table 62: UL UE-Average-Latency assuming low RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.038174 | 0.033446 | 0.03467 | 0.060737 |
| SBFD-XXXXU-UL50%-145dB | 0.015803 | 0.013585 | 0.013787 | 0.015137 |
| SBFD-DXXXU-UL50%-145dB | 0.017174 | 0.016679 | 0.017131 | 0.017496 |
| SBFD-XXXXU-UL50%-185dB | 0.015803 | 0.013585 | 0.013787 | 0.015137 |
| SBFD-DXXXU-UL50%-185dB | 0.017174 | 0.016679 | 0.017131 | 0.017496 |

Table 63: UL UE-Average-Latency assuming medium RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.045397 | 0.033276 | 0.037089 | 0.081749 |
| SBFD-XXXXU-UL50%-145dB | 0.133546 | 0.013603 | 0.013816 | 0.059281 |
| SBFD-DXXXU-UL50%-145dB | 0.017759 | 0.016816 | 0.017209 | 0.019615 |
| SBFD-XXXXU-UL50%-185dB | 0.133546 | 0.013603 | 0.013816 | 0.059281 |
| SBFD-DXXXU-UL50%-185dB | 0.017759 | 0.016816 | 0.017209 | 0.019615 |

Table 64: UL UE-Average-Latency assuming hig RU for legacy TDD

| Simulation cases | mean | 5% | 50% | 95% |
| --- | --- | --- | --- | --- |
| Baseline-DDDSU | 0.05648 | 0.033574 | 0.044551 | 0.152283 |
| SBFD-XXXXU-UL50%-145dB | 0.044741 | 0.013648 | 0.013954 | 0.031508 |
| SBFD-DXXXU-UL50%-145dB | 0.034833 | 0.01677 | 0.017356 | 0.102019 |
| SBFD-XXXXU-UL50%-185dB | 0.044741 | 0.013648 | 0.013954 | 0.031508 |
| SBFD-DXXXU-UL50%-185dB | 0.034833 | 0.01677 | 0.017356 | 0.102019 |

Agreements achieved in RAN1#110 e-meeting

In RAN1#110 e-meeting, the following agreements were achieved:

Agreement

Two types of RU (Resource utilization) are defined for SBFD evaluation.

- Type-1 RU: DL/UL Type-1 RU = Number of RBs per cell used by traffic for the given link direction during observation time / Total number of all the RBs per cell including DL, UL and guard bands over observation time.

- Type-2 RU (Follow TR 36.814): DL/UL Type-2 RU = Number of RBs per cell used by traffic for the given link direction during observation time / Total number of RBs per cell available for traffic for the given link direction over observation time

- Note: In case of MU-MIMO, one RB allocated to N users within a cell is only counted as used once.

- Companies are to submit results for both RU definitions

- FFS: RU definition for dynamic TDD evaluations

Agreement

For UE distribution of Urban Macro and Dense Urban Macro layer,

- Baseline: (UE clustering)

  - 10 users per macro TRP

    - Step 1: Randomly drop X UE cluster centers within one macro cell geographical area considering the minimum distance between macro TRP to UE cluster center as Dmacro-to-cluster and the minimum distance between two UE cluster centers as Dinter-cluster

    - Step 2: Y% UEs are randomly and uniformly dropped within the UE clusters with the radius of R, (1-Y%) users randomly and uniformly dropped throughout the macro geographical area

    - UE outdoor/indoor proportion: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h

    - FFS the values of X, Dmacro-to-cluster, Dinter-cluster, R, Y%

- Optional:

  - 10 users per macro TRP, and all users are randomly and uniformly dropped within the macro cell

  - At least for FR1: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h

    - FFS: FR2 details

Agreement

For Dense Urban with 2-layer for FR1, consider micro cell TRPs are deployed as following

- Step 1: Randomly drop [3] micro TRP centers within one macro cell geographical area considering the minimum distance between micro TRP centers (Dinter-micro-center) and the minimum distance between macro TRP and micro TRP center (Dmacro-to-micro-center).

- Step 2: Randomly deploy one micro TRP on the area circle around each micro TRP center with the radius of half of Dinter-micro-center

- Step 3: Determine the horizontal angle of the micro TRPs with the planer facing to the micro TRP center.

- Dinter-micro-center =[57.9 m], Dmacro-to-micro-center = [105 m]

Agreement

For latency related performance metric for FTP model 3 in SLS, option 1 is baseline, it is up to companies to report the latency with option 2.

- Packet latency: defined as the time which starts when the packet is received in the transmit buffer and ends when the last bit of the packet is correctly delivered to the receiver.

  - (baseline) Option 1: Calculate the latency for each packet for each UE, and then generate CDF of latency for all these packets from all the UEs.

    - Packet-Latency CDF: The CDF of the packet latencies of all the packets from all the UEs.

    - Mean/5%/50%/95% Packet-Latency: The mean/5%/50%/95% value of Packet-Latency of all the packets from all the UEs.

  - (optional) Option 2: Calculate the latency for each packet for each UE, and then calculate the average latency for each UE, then generate the CDF for these average latency for each UE

    - UE-Average-Latency: defined as the average packet latency for a UE

    - UE-Average-Latency CDF: The CDF of the UE-Average-Latency for all users.

    - Mean/5%/50%/95% UE-Average-Latency: The mean/5%/50%/95% value of UE-Average-Latency for all users.

- Note: HARQ re-transmission should be considered for latency evaluation.

- Unfinished/dropped FTP packets are not incorporated in the packet latency calculation.

  - Unfinished/dropped Packet Rate is defined as the number of the unfinished packets for all users divided by the total number of generated packets for all users

    - To be reported as part of the system level simulation results

Agreement

For UPT (user perceived throughput) related performance metrics for FTP model 3 in SLS, adopt the following option.

- Option 1: UPT is defined as the size of an FTP packet divided by the time which starts when the packet is received in the transmit buffer and ends when the last bit of the packet is correctly delivered to the receiver [Refer to TR36.814].

  - Unfinished FTP packets should be incorporated in the UPT calculation. The number of served bits (possibly zero) of an unfinished FTP packet by the end of the simulation is divided by the served time (simulation end time – file arrival time) [Refer to TR36.889].

  - Consider zero bit for dropped FTP packets.

  - Average-UPT of a user: defined as the average from all UPTs for all FTP packets intended for this user [Refer to TR36.814].

  - Tail-UPT of a user: defined as the worst 5% UPT among all FTP packets intended for this user [Refer to TR36.814].

  - Median-UPT of a user: defined as the 50% UPT among all FTP packets intended for this user.

  - Average-UPT CDF: The CDF of the Average-UPTs for all users.

  - Tail-UPT CDF: The CDF of the Tail-UPTs for all users.

  - Median-UPT CDF: The CDF of the Median-UPTs for all users.

  - Mean/5%/50%/95% Average-UPT: The mean/5%/50%/95% value of Average-UPTs for all users.

  - Mean/5%/50%/95% Tail-UPT: The mean/5%/50%/95% value of Tail-UPTs for all users.

  - Mean/5%/50%/95% Median-UPT: The mean/5%/50%/95% value of Median-UPTs for all users.

Agreement

- Adopt the following table for traffic model of FTP model 3 for scenarios in deployment case 1 for SBFD.

|  | Indoor office (FR1&FR2) | Urban Macro (FR1) | Dense Urban Macro layer (FR1&FR2) | Dense Urban Micro layer (FR2) | Dense Urban with 2-layer (FR1) |
| --- | --- | --- | --- | --- | --- |
| General | UL and DL are simulated simultaneously. Companies to report which option is used. - Option 1: Each UE is either assigned UL traffic or DL traffic. - assume the same number of UEs for UL and DL, FFS the total number of UEs - FFS how to handle the UE clustering case - Option 2: Each UE is assigned both UL traffic and DL traffic. |  |  |  |  |
| FTP packet size | Both symmetric and asymmetric packet size for UL and DL can be considered. Companies to report which option is used. - Option 1: Symmetric packet size: - 1Kbyte for DL/UL, 0.1Mbytes for DL/UL, 0.5Mbytes for DL/UL, 2Mbytes for DL/UL - Option 2: Asymmetric packet size: - 4Kbytes for DL and 1Kbyte for UL, 0.5Mbyte for DL and 0.125 Mbytes for UL |  |  |  |  |
| UL arrival rate for legacy TDD | - The UL arrival rate is selected to reach a target UL traffic load (RU). - UL Traffic load: low UL RU ([<10%]), medium UL RU ([20%-30%]), and high UL RU ([~50%]). - Note: Type-2 RU definition (calculated per link direction) is used |  |  |  | - The UL arrival rate#1 of Macro cell and UL arrival rate#2 of Micro cell are selected to reach target UL traffic load (RU)#1 of Macro cell and target UL traffic load (RU)#2 of Micro cell, respectively - UL Traffic load: low UL RU ([<10%]), medium UL RU ([20%-30%]), and high UL RU ([~50%]). - Note: Type-2 RU definition (calculated per link direction) is used |
| DL arrival rate for legacy TDD | - The DL arrival rate is selected to reach a target DL traffic load (RU). - DL Traffic load: low DL RU ([<10%]), medium DL RU ([20%-30%]), and high DL RU ([~50%]). - Note: Type-2 RU definition (calculated per link direction) is used |  |  |  | - The DL arrival rate#1 of Macro cell and DL arrival rate#2 of Micro cell are selected to reach target DL traffic load (RU)#1 of Macro cell and target DL traffic load (RU)#2 of Micro cell, respectively - DL Traffic load: low DL RU ([<10%]), medium DL RU ([20%-30%]), and high DL RU ([~50%]). - Note: Type-2 RU definition (calculated per link direction) is used |
| Arrival rate for SBFD | The UL and DL FTP packet arrival rate for SBFD are the same as legacy TDD. |  |  |  |  |

Working assumption:

- Adopt the following table for gNB-gNB channel model and gNB-UE channel model.

|  | Dense urban, Urban macro | Indoor office |
| --- | --- | --- |
| Large-scale channel parameters | FR1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa in TR 38.901 (hUE =25m), - Macro-to-Micro: UMa in TR 38.901 (hUE =10m) - Micro-to-Micro: UMi-Street canyon in TR 38.901 (hUE =10m) FR2-1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa in TR 38.901 (hUE =25m) - Macro-to-Micro: UMa in TR 38.901 (hUE =10m) - Micro-to-Micro: UMi-Street canyon in TR 38.901 (hUE =10m) | FR1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) FR2-1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) |
| Fast fading parameters | FR1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa O2O in TR 38.901 (hUE =25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro-to-Micro: UMa O2O in TR 38.901 - Micro-to-Micro: UMi-Street canyon O2O in TR 38.901 (hUE=10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 FR2-1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa O2O in TR 38.901 (hUE=25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro-to-Micro: UMa O2O in TR 38.901 - Micro-to-Micro: UMi-Street canyon O2O in TR 38.901 (hUE=10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 | FR1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE=3m), ASA and ZSA statistics updated to be the same as ASD and ZSD FR2-1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m), ASA and ZSA statistics updated to be the same as ASD and ZSD |

Agreement

- For evaluation of SBFD and dynamic/flexible TDD, adopt the following evaluation assumptions.

|  | FR1 | FR2-1 |
| --- | --- | --- |
| System bandwidth | 100MHz | 100MHz |
| Numerology | 14 OFDM symbol slot SCS = 30kHz | 14 OFDM symbol slot SCS = 120kHz |
| UE Tx power | 23dBm | 23 dBm. EIRP should not exceed 43 dBm [refer to TR 38.802 Table A.2.1-1] |
| Open loop power control parameters | Companies to report power control parameters. For calibration: - P0= -60 dBm, alpha = 0.6 for InH [refer to TR 37.910, evaluation assumption in B.4.1_eMBB_SE.zip] - P0= -86 dBm, alpha = 0.9 for Dense Urban [refer to TR 37.910, evaluation assumption in B.4.1_eMBB_SE.zip] - P0= -80 dBm, alpha = 0.8 for Urban Macro |  |
| BS receiver noise figure | 5dB [refer to TR 38.802 Table A.2.1-1] | 7dB [refer to TR 38.802 Table A.2.1-1] |
| UE receiver noise figure | 9 dB [refer to TR 38.802 Table A.2.1-1] | 13 dB (baseline), 10 dB (optional) [refer to TR 38.802 Table A.2.1-1] |
| UE receiver | MMSE-IRC as the baseline receiver. Note: Advanced receiver is not precluded. [refer to TR 38.802 Table A.2.1-1] |  |
| Feedback assumption | Realistic [refer to TR 38.802 Table A.2.1-1] |  |
| Channel estimation | Companies to report the option used. Option 1: Ideal Option 2: Realistic [refer to TR 38.802 Table A.2.1-1] |  |
| UE processing capability | UE processing capability 1 as baseline | UE processing capability 1 as baseline |
| Handover margin | 3 dB [refer to TR 38.828 Table 5.2.1.4-1] |  |
| UE attachment | Based on RSRP from port 0 [refer to TR 37.910, evaluation assumption in B.4.1_eMBB_SE.zip] | Based on RSRP from port 0. The UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. [refer to TR 37.910, evaluation assumption in B.4.1_eMBB_SE.zip] |
| Polarized antenna model | Model-1 in clause 7.3.2 in TR 38.901 |  |
| DL/UL Modulation | Up to 256QAM |  |
| Transmission scheme | Companies to report transmission schemes (e.g., SU-MIMO, MU-MIMO, maximum layers for SU-MIMO/MU-MIMO, etc) For calibration, consider SU-MIMO with single layer for both DL and UL |  |
| Scheduling | PF |  |
| Overhead | Companies to report the overhead assumption |  |

Agreement

Update the previous agreement as below:

For UE distribution of Urban Macro and Dense Urban Macro layer,

- Baseline: (UE clustering at least for FR1)

  - M users per macro TRP

    - Step 1: Randomly drop X UE cluster centers within one macro cell geographical area considering the minimum distance between macro TRP to UE cluster center as Dmacro-to-cluster and the minimum distance between two UE cluster centers as Dinter-cluster

    - Step 2: Y% UEs are randomly and uniformly dropped within the UE clusters with the radius of R, (1-Y%) users randomly and uniformly dropped in the macro geographical area outside the clusters

    - Note: UEs dropped within the UE cluster(s) are indoor with 3km/h; UEs dropped outside the UE cluster(s) are outdoor in car with 30km/h

    - UE outdoor/indoor proportion: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h

      - Outdoor UEs: 1.5 m;

      - FFS: Indoor UEs height

    - Y%=80%

    - FFS the values of M, X, Dmacro-to-cluster, Dinter-cluster, R

- Optional:

  - 10 users per macro TRP (per direction), and all users are randomly and uniformly dropped within the macro cell

  - At least for FR1: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h

    - Outdoor UEs: 1.5 m;

    - Indoor UEs: 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) [refer to TR 36.873 Table 6-1]

    - FFS: FR2 details

Agreement

For LOS probability of gNB-gNB channel,

- For Macro-gNB-to-Macro-gNB case

  - Option 3: If the 2D distance between two Macro gNBs are less than or equal to the ISD (200m for Dense Urban, and 500m for Urban Macro), set the LOS probability to X; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901.

    - X = 0.75

- For other cases, reuse gNB-to-UE LOS probability equation in TR 38.901.

Agreement

For Dense Urban Micro layer for FR2-1,

- Regarding the layout, only consider the Micro TRPs of Dense Urban 2-layer network. All users communicate with micro TRPs, i.e. macro cell is only used for determining position of micro TRP.

- Regarding UE distribution, all users are randomly and uniformly dropped around Micro TRP center with the radius of R (R = [28.9m]).

Agreement

For UE distribution of Dense Urban with 2-layer, reuse the modeling in TR38.802 as much as possible.

- For FTP traffic model 3: 2/3 users randomly and uniformly dropped around micro TRP centers with radius of R (R = [28.9m]), 1/3 users randomly and uniformly dropped throughout the macro geographical area, and 60 users per macro geographical area.

- UE outdoor/indoor proportion: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h

  - Outdoor UEs: 1.5 m;

  - Indoor UEs: 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8)

Agreement

- For evaluation of SBFD and dynamic/flexible TDD, the following BS transmit power for legacy TDD are considered. These values are for the single operator case.

|  | FR1 | FR2-1 |
| --- | --- | --- |
| Urban macro | - Option 1: [53] dBm for 100MHz - Option 2: [49] dBm for 100MHz [refer to TR 38.828 Table 5.2.1.4-1] | N.A. |
| Dense Urban Macro layer | - Option 1: [53] dBm for 100MHz - Option 3: [44] dBm for 100MHz [refer to TR 38.802 Table A.2.1-1] | - Option 1: [43] dBm for 200MHz [refer to TR 38.828 Table 5.2.2.4-1] |
| Dense Urban Micro layer | - Option 3: [40] dBm for 100MHz [refer to TR 38.802 Table A.2.1-1] | - Option 2: [33] dBm for 200MHz. EIRP should not exceed 68 dBm. [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.2.4-1] |
| Indoor hotspot | - Option 2: [24] dBm for 100MHz [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.1.1.2-1] | - Option 1: [23] dBm for 200MHz. EIRP should not exceed 58 dBm. [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.2.4-1] |

Agreement

For evaluation of SBFD and dynamic/flexible TDD, use BS antenna radiation pattern as following:

- InH: reuse Table 10 in Report ITU-R M.2412 for both FR1&FR2-1 (Table A.2.1-7 in TR 38.802)

- Urban Macro/ Dense Urban Macro layer / Dense Urban Micro layer: reuse Table 9 in Report ITU-R M.2412 for both FR1&FR2-1 (same as 3-sector BS antenna radiation model in Table A.2.1-6 in TR 38.802)

  - Companies can also consider evaluation with other realistic BS antenna radiation pattern

Agreement

For evaluation of SBFD and dynamic/flexible TDD, use UE antenna radiation pattern as following:

- FR1: Omni-directional with 0 dBi element gain

- FR2: reuse Table 11 in Report ITU-R M.2412 (same as UE antenna radiation pattern model 1 in Table A.2.1-8 in TR 38.802)

Working Assumption

| Parameters | Indoor office | Urban macro / Dense Urban Macro layer | Dense Urban with 2-layer |
| --- | --- | --- | --- |
| Layout | Single layer Indoor floor: (12BSs per 120m x 50m) | Single layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around. | Two layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around. Micro layer: According to previous agreement - Baseline: 3 Micro BSs per Macro BS - Optional: 6, or 9 Micro BSs per Macro BS |
| Inter-BS (2D) distance | 20m [TR 38.802 Table A.2.1-11] | 500m for Urban Macro [TR 38.802 Table A.2.1-11] 200m for Dense Urban Macro layer [TR 38.802 Table A.2.1-1] | Macro-to-macro: 200m Minimum Macro-to-micro-center distance: 105m Minimum Micro-center-to-micro-center distance: 57.9m |
| Minimum BS-UE (2D) distance | 0m [TR 38.802 Table A.2.1-11] | 35m [TR 38.802 Table A.2.1-11] | Macro-to-UE: 35m Micro-to-UE: 10m [TR 38.802 Table A.2.1-11] |
| Minimum UE-UE (2D) distance | FFS | FFS :3m [TR 38.802 Table A.2.1-11] | FFS: 3m [TR 38.802 Table A.2.1-11] |
| BS antenna height | 3 m [TR 38.802 Table A.2.1-1] | 25 m [TR 38.802 Table A.2.1-1] | 25m for macro cells and 10m for micro cells [TR 38.802 Table A.2.1-1] |

Conclusion

- For SLS of NR duplex evolution, Rural scenario is not considered in Rel-18.

- For NR duplex evolution evaluation, FR2-2 is not considered in Rel-18.

Agreement

For SBFD evaluation from RAN1 perspective, the evaluation assumptions that are specific for Deployment Case 2 and Case 3-1 can be discussed with low priority.

Agreement

RAN1 strives to agree on system level simulation parameters for SBFD deployment case 4 by RAN1#110bis-e with specific focus on different power levels and load levels between two operators in adjacent carriers.

Agreement

For evaluation of SBFD operation, separate-Tx/Rx antenna array can be modelled by two panel groups.

- Legacy parameters M,N,P,Mg,Ng, dH,dV and dg,H,dg,V are used for description of each panel group:

  - M: Number of vertical antenna elements within a panel, on one polarization

  - N: Number of horizontal antenna elements within a panel, on one polarization

  - P: Number of polarizations

  - Mg: Number of panels in a column within a panel group.

  - Ng: Number of panels in a row within a panel group.

  - dg,H: Antenna panel spacing in horizontal direction within a panel group.

  - dg,V: Antenna panel spacing in vertical direction within a panel group.

- Companies are to report the separation of the two panel groups. Introduce new parameters da,H,da,V as illustrated in the following figure.

  - da,H: Panel group spacing in the horizontal direction. Typically, da,H = 0.

  - da,V: Panel group spacing in the vertical direction.

Agreement

For evaluation and comparison between SBFD and legacy TDD, the two options for the SBFD antenna configuration agreed in RAN1#109 are further clarified as below:

- SBFD antenna configuration option-1 (same as Opt 1 in RAN1#109 agreement): The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD.

- SBFD antenna configuration option-2 (same as Opt 2 in RAN1#109 agreement): The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD.

- SBFD antenna configuration option-3 (new): The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is half of the total number of TxRUs of the antenna array for legacy TDD.

These options are further clarified with examples in the following:

- For legacy TDD with shared-Tx/Rx antenna array, assume the antenna configuration is M,N,P,Mg,Ng;Mp,Np. The total number of TxRUs is  K=PMpNpMgNg, and the total number of antenna elements is L=PMNMgNg.

- For SBFD antenna configuration option-1, the separate-Tx/Rx antenna array has two panel groups, and the antenna configuration for each panel group is M,N,P,Mg/2,Ng. The total number of TXRUs is K=PMpNpMgNg (same as legacy TDD), and the total number of antenna elements is L=PMNMgNg(same as legacy TDD). One method on the usage of TXRUs and antenna elements in DL/UL/SBFD slots/symbols is illustrated as below. Other methods are not precluded and can be reported by companies.

  - Method 1:

    - In DL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Tx chains in TxRU group#2.

    - In UL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Rx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains in TxRU group#2.

    - In SBFD slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains in TxRU group#2.

- For SBFD antenna configuration option-2, the separate-Tx/Rx antenna array has two panel groups, and the antenna configuration for each panel group is M,N,P,Mg,Ng. The total number of TXRUs is K=PMpNpMgNg (same as legacy TDD), and the total number of antenna elements is 2L=2PMNMgNg(two times of that for legacy TDD). Two methods on the usage of TXRUs and antenna elements in DL/UL/SBFD slots/symbols are illustrated as below. Other methods are not precluded and can be reported by companies.

  - Method 2-1:

    - In DL slots, L antenna elements on panel group#1 are connected to K Tx chains.

    - In UL slots, L antenna elements on panel group#2 are connected to K Rx chains.

    - In SBFD slots, L antenna elements on panel group#1 are connected to K Tx chains, and L antenna elements on panel group#2 are connected to K Rx chains.

  - Method 2-2:

    - In DL slots, L antenna elements on panel group#1 are connected to K Tx chains.

    - In UL slots, L antenna elements on panel group#1 are connected to K Rx chains.

    - In SBFD slots, L antenna elements on panel group#1 are connected to K Tx chains, and L antenna elements on panel group#2 are connected to K Rx chains.

- For SBFD antenna configuration option-3, the separate-Tx/Rx antenna array has two panel groups, and the antenna configuration for each panel group is M,N,P,Mg/2,Ng. The total number of TXRUs is K/2=PMpNpMgNg/2 (half of that for legacy TDD), and the total number of antenna elements is L=PMNMgNg(same as legacy TDD). The method on the usage of TXRUs and antenna elements in DL/UL/SBFD slots/symbols are illustrated as below. Other methods are not precluded and can be reported by companies.

  - Method 3-1:

    - In DL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains.

    - In UL slots, L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains.

    - In SBFD slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains.

  - Method 3-2:

    - In DL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1.

    - In UL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Rx chains in TxRU group#1.

    - In SBFD slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains in TxRU group#1.

Working Assumption

For UE-UE channel model, reuse the UE-UE channel model for flexible duplex evaluation in TR 38.802 for both FR1 and FR2 as baseline, and other models are not precluded.

UE-UE channel model

|  | Dense urban, Urban macro | Indoor hotspot |
| --- | --- | --- |
| Large-scale channel parameters | FR1: - Option 1: UE-to-UE: A.2.1.2 in TR36.843(*), penetration loss between UEs follows Table A.2.1-13 in TR38.802 - Option 2: UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-13 in TR38.802 FR2-1: - UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 in TR38.802 | FR1: - Option1 : UE-to-UE: A.2.1.2 in TR36.843 (*) - Option 2: UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) FR2-1: - UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) |
| Fast fading parameters | FR1: - Option 1: UE-to-UE: A.2.1.2 in TR36.843 (ITU InH) for indoor to indoor, and 3D UMi for other cases. ASD and ZSD statistics updated to be the same as ASA and ZSA. - Optioin 2: UE-to-UE: UMi-Street canyon in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA. FR2-1: - UE-to-UE: UMi-Street canyon in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA. | FR1: - Option 1: UE-to-UE: A.2.1.2 in TR36.843 (ITU InH), ASD statistics updated to be the same as ASA. - Option2: UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA FR2-1: - UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA |
| (*): For outdoor to indoor case, and indoor to indoor case, use “Remaining Layout Options” in A.2.1.2 of TR36.843 for pathloss calculation, and “ITU-R IMT UMi” for LOS Probability derivation. For outdoor to indoor case, the penetration loss term “20.0+0.5* din” is excluded in pathloss formula given in A.2.1.2 of TR36.843, and the penetration loss is derived according to Table A.2.1-13 in TR38.802. |  |  |

Agreement

For evaluation of adjacent-channel coexistence between two networks for Urban Macro and Dense Urban Macro layer scenarios in RAN1, consider grid shifts between two networks of 0% and 100%.

- the topologies shown below can be used for the 0% and 100% grid shift for RAN1 evaluation.

Agreement

For evaluation of SBFD operation, it is up to companies to report the BS antenna configurations used in their simulations. The BS antenna configurations in the following table can be considered for calibration purpose.

| Scenarios | FR | Legacy TDD | SBFD |
| --- | --- | --- | --- |
| BS antenna configuration for Indoor office | FR1 | M,N,P,Mg,Ng;Mp,Np = (4,4,2,1,1; 4,4) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization | - SBFD antenna configuration option-1 (Method 1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (2,4,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV= (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ |
|  | FR2-1 | M,N,P,Mg,Ng;Mp,Np=(16,8,2,1,1; 1,1) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization | - SBFD antenna configuration option-1 (Method 1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (8,8,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV= (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ |
| BS antenna configuration for Urban Macro/ Dense Urban Macro layer/ Dense Urban Micro layer | FR1 | M,N,P,Mg,Ng;Mp,Np= (8,8,2,1,1;2,8) polarization | - SBFD antenna configuration option-1 (Method 1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (4,8,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV = (0.5, 0.8)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ |
|  | FR2-1 | M,N,P,Mg,Ng;Mp,Np= (4,16,2,2,2; 1,1) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization | - SBFD antenna configuration option-1 (Method 1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (4,8,2,2,2). - Number of TxRUs: same as legacy TDD - dH,dV = (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ |