3GPP TSG RAN WG1 #110-bis-e			                     R1-2209319

e-Meeting, October 10th – October 19th, 2022

Agenda item:	8.16.1

Source: 	CMCC

Title: 	Maintenance on Rel.17 UE features for NR coverage enhancement

Document for:	Discussion and Decision

Introduction

This document is to discuss the question in FL summary for discussion in RAN1 #110[2].

Overview on FGs for Coverage Enhancements

Proposal 1:

FG 30-4a to 30-4h should be per band reported and differentiated between FR1 and FR2.

Proposal 2：

No need to differentiate TDD and FDD for FG 30-4a to FG 30-4h.

Proposal 3:

The FG 30-4g should be suspended until the discussion about restarting DMRS bundling ended in AI 8.8.

30-4 to 30-4g: DM-RS bundling

In [1], FGs 30-4 to 30-4g are captured as below.

| 30. NR_cov_enh | 30-4 | The maximum duration for DM-RS bundling | The maximum duration during which UE is able to maintain power consistency and phase continuity to support DM-RS bundling for PUSCH/PUCCH |  | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | Per band | N/A | N/A | N/A | Candidate values for the maximum duration for FDD are {4, 8, 16, 32} Candidate values for the maximum duration for TDD are {2, 4, 8, 16} NOTE: DM-RS bundling is only applicable for UL transmissions with pi/2 BPSK, BPSK, and QPSK modulation orders for the corresponding physical channels. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-4a | DM-RS bundling for PUSCH repetition type A | Support DM-RS bundling for PUSCH repetition type A over consecutive symbols | 30-4 and one of {5-14, 5-16, 5-17} | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4b | DM-RS bundling for PUSCH repetition type B | Support DM-RS bundling for PUSCH repetition type B over consecutive symbols | 30-4, 11-5 | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4c | DM-RS bundling for TB processing over multi-slot PUSCH | Support DM-RS bundling for TB processing over multi-slot PUSCH over consecutive symbols | 30-4, 30-3 | Yes | N/A | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] | FFS | [No] | [N/A] | Note: If a UE reports support of FG 30-3a and 30-4c, the UE supports DMRS bundling for the repetitions of TBoMS | Optional with capability signalling |
| 30. NR_cov_enh | 30-4d | DMRS bunding for PUCCH repetitions | Support DM-RS bundling for PUCCH repetitions for PUCCH formats 1/3/4 over consecutive symbols | 30-4, 4-23 | Yes | N/A | UE does not support DMRS bunding for PUCCH repetitions | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4e | Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | 30-4a or 30-4b or 30-4c | Yes | N/A | UE does not support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4f | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | 30-4d | Yes | N/A | UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4g | [Restart DM-RS bundling after the events that violate power consistency and phase continuity] | [Support restarting DM-RS bundling after the events that violate power consistency and phase continuity] | 30-4 | Yes | N/A | [UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity] | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4h | DM-RS bundling for non-back-to-back transmission | Support DM-RS bundling for non-back-to-back transmission for consecutive slots for PUSCH and PUCCH only for corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d) | 30-4a, 30-4b, 30-4c, or 30-4d | Yes | N/A | UE does not Support DM-RS bundling for non-back-to-back transmission | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |

In the RAN4 LS R1-2205715(R4-2211225), RAN4 informed that in FR1+FR2 UL CA, FR1+FR2 DC, and EN-DC with NR on FR2, DMRS bundling configuration is limited to one uplink NR carrier in total on all FRs at a time. Based on the above information, per band report should be enough, because the UL/CA and DMRS bundling is independent.

And it was concluded that the FG 30-4 is per band reported. Then the FGs based on FG 30-4 should be also per band reported, such as FG 30-4a 30-4b. And consequently, for those per band reported FGs, FR1 and FR2 differentiation is needed.

Proposal 1:

FG 30-4a to 30-4h should be per band reported and differentiated between FR1 and FR2.

Considering the behavior of the DMRS bundling within the nominal and actual TDWs are the same for both TDD and FDD. There is no need to differentiate TDD and FDD for those features.

Proposal 2：

No need to differentiate TDD and FDD for FG 30-4a to FG 30-4h.

RAN 1 still have different understanding about the UE behavior when a dynamic event happened before or overlap with a semi-static event that violate power consistency and phase continuity. The FG 30-4g should be suspended until the discussion about restarting DMRS bundling ended in AI 8.8.

Proposal 3:

The FG 30-4g should be suspended until the discussion about restarting DMRS bundling ended in AI 8.8.

References

[1] R1-2207923, Updated RAN1 UE features list for Rel-17 NR after RAN1 #110 Thursday, Moderators (AT&T, NTT DOCOMO, INC.), 3GPP TSG RAN WG1 #110 Toulouse, France, August 22nd – 26th, 2022.

[2] R1-2208115, Summary#3 on UE features for NR coverage enhancement, Moderator (NTT DOCOMO, INC.), 3GPP TSG RAN WG1 #110 Toulouse, France, August 22nd – 26th, 2022.