3GPP TSG RAN WG1 #110bis	R1- 2209369

e-meeting, 10 – 19 October, 2022

Agenda item:		9.2.3.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Evaluation of ML for beam management

Document for:		Discussion and Decision

# Introduction

In RAN1#110 AI/ML for beam management, it has the following agreements and conclusions:

Agreement

The Following updated based on the agreements in RAN 1 #109-e is adopted

| Parameters | Values |
| --- | --- |
| UE distribution | - FFS 10 UEs per sector/cell for system performance related KPI (if supported) [e.g,, throughput] for full buffer traffic (if supported) evaluation (model inference). - X UEs per sector/cell for system performance related KPI for FTP traffic (if supported) evaluation (model inference). - Other values are not precluded - Number of UEs per/sector per cell during data collection (training/testing) is reported by companies if relevant - More UEs per sector/cell for data generation is not precluded. |
| UE Antenna Configuration | - Antenna setup and port layouts at UE: [1,2,1,4,2,1,1], 2 panels (left, right) - [Panel structure: (M,N,P) = (1,4,2)] - panels (left, right) with (Mg, Ng) = (1, 2) as baseline - Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Companies to explain number of UE beams |

Agreement

The Following updated based on the agreements in RAN 1 #109-e is adopted

| Parameters | Values |
| --- | --- |
| UE Speed | - For spatial domain beam prediction, 3km/h - For time domain beam prediction: 3km/h(optional), 30km/h (baseline), 60km/h (optional), 90km/h (optional), 120km/h (optional) - Other values are not precluded |
| UE distribution | - For spatial domain beam prediction: - Option 1: 80% indoor ,20% outdoor as in TR 38.901 - Option 2: 100% outdoor - For time domain prediction: 100% outdoor |

Agreement

- If UE orientation is modeled, it can be independently modeled from UE moving trajectory model.

  - This is not precluded that UE orientation coupled with UE moving trajectory model.

Agreement

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

    - FFS on the beams of Set B

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each report/measurement during training and/or inference)

    - FFS on fixed or variable number of beams (pairs)

    - FFS on the details

  - Other options are not precluded.

  - FFS on the number of beams (pairs) in Set B

  - Note: This does not preclude the alternative that Set B is different from Set A.

Agreement

- To evaluate the performance of AI/ML in beam management at least for NW side beam prediction, UCI report overhead can be further studied as one of KPI options.

  - FFS: number of UCI reports and UCI payload size

In addition to above agreements/conclusions, there are many other RAN1 agreements that are related to AI/ML for beam management, which can be found in Annex I and Annex II. Based on the discussion in RAN1 #109-e meeting, the focus of the study should be on BM-Case1 and BM-Case2. In this contribution, we discuss the characterization and baseline performance evaluations for sub use case BM-Case1 and for sub use case BM-Case2. We discuss different alternatives for each sub-use case including the details of simulation assumptions, baseline assumptions, and KPIs. We also provide simulation results for various alternatives.

# Discussion

## Common Assumptions for Beam Management Use Case

    - KPIs and Corresponding Requirements

To evaluate the performance of AI/ML in beam management, in the RAN1 #109-e meeting the agreement [1] was made to further study the following KPI options: Beam prediction accuracy related KPIs, which may include the options specified in Table 2.1-1, and System performance related KPIs, which may include the options listed in Table 2.1-2.

Table 2.1-1: List of Beam prediction accuracy related KPIs to evaluate the performance of AI/ML in beam management.

| KPIs | KPI description | Notes |
| --- | --- | --- |
| Beam prediction accuracy (%) for Top-1 beam | The beam prediction accuracy (%) is the percentage of “the Top-1 genie-aided beam is the Top-1 predicted beam. | ATop-1=1Nt n=1Ntl(bn=bn) Where bn is the index of the Top-1 predicted beam and bn is the index of the Top-1 genie-aided beam. Nt is the number of data points for obtaining the ML model performance, l() is the indicator function. If bn=bn, then l()=1. Otherwise, l()=0. Top-1 genie-aided beam index is selected based on bn=argmax(RSRPl) for l={1,…,M}, where M are all the beams in SetA. |
| Beam prediction accuracy (%) for Top-K beams | The beam prediction accuracy (%) is the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams. | ATop-K=1Nt n=1Ntk=1Kl(bk,n=bn) Where bk,n is the index of the k-th beam in the Top-K predicted beams and bn is the index of the Top-1 genie-aided beam. |
| Beam prediction accuracy (%) with 1dB margin for Top-1 beam | The beam prediction accuracy (%) with 1dB margin is the percentage of the Top-1 predicted beam “whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam” | ATop-1, 1 dB margin=1Nt n=1Ntl(RSRPbn-RSRPbn<1 dB) Where RSRPbn is the RSRP of the Top-1 genie-aided beam and RSRPbn is the RSRP of the Top-1 predicted beam. |
| Average L1-RSRP difference of Top-1 predicted beam | The difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam | eRSRP=1Nt n=1NtRSRPbn-RSRPbn |

Table 2.1-2: List of System performance related KPIs to evaluate the performance of AI/ML in beam management.

| KPIs | KPI description | Notes |
| --- | --- | --- |
| UE throughput | CDF of UE throughput, Average throughput and 5%-tile UE throughput | A similar mechanism as in Rel-16/17 MIMO BM simulations |
| RS overhead reduction | RS overhead reduction at least for spatial-domain beam prediction at least for Top-1 beam | RS overhead reduction=1-NM where N is the number of beams (with reference signal (SSB and/or CSI-RS)) required for measurement, M is the total number of beams. Non-AI/ML approach based on the measurement of these M beams may be used as a baseline. When N is variable, the overhead reduction is computed using an average measurement set size, such that RS overhead reduction=1-1M1Ntn=1NtNn Where Nn is the number of beams required for measurement during time slot n |

    - Assumptions for Data Generation and Simulations

The methodology for dataset construction and performance evaluation is based on the statistical channel models from TR 38.901 and the system level simulation approach is adopted as a baseline as agreed in RAN1 #109-e meeting [1]. Based on what is agreed upon, we adopt the system level assumptions and the parameters for the Dense Urban scenario detailed in Table 2.1-3.

Table 2.1-3: General SLS assumptions for dataset generation and performance evaluation for AI/ML in beam management.

| Parameters | Values |
| --- | --- |
| Frequency Range​ | FR2 @ 30 GHz ​ |
| Deployment​ | 200m ISD,​ BS Antenna height=25 m 2-tier model with wrap-around (7 sites, 3 sectors/cells per site)​ |
| Channel mode​ | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901.​ |
| System BW​ | 80MHz​, ​SCS: 120 kHz |
| UE Speed​ | - For spatial domain beam prediction, 3km/h​ - For time domain beam prediction: 30km/h |
| UE distribution​ | 10 UEs per sector/cell for data generation/evaluation​​, UE Antenna height=1.5 m - For spatial domain beam prediction: 100% outdoor​ - For time domain prediction: 100% outdoor​ |
| BS Tx Power​ | 40 dBm |
| BS Antenna Configuration​ | One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ ​ - TXRU weights mapping:​ one TXRU mapped to multiple antenna elements - Beam selection: based on maximum L1-RSRP - Number of BS beams​: 64 |
| BS Antenna radiation pattern​ | TR 38.802 Table A.2.1-6, Table A.2.1-7​ |
| Maximum UE Tx Power​ | 23 dBm​ |
| UE Antenna Configuration​ | Panel structure: (M,N,P) = (1,4,2)​, 2 panels (left, right) with (Mg, Ng) = (1, 2) - TXRU weights mapping: one TXRU mapped to multiple antenna elements - panel selection: ideal - number of UE beams: 8 - beam selection: - Fixed Rx beam direction to panel boresight - Optimal Rx beam selection based on maximum L1-RSRP |
| UE Antenna radiation pattern​ | TR 38.802 Table A.2.1-8, Table A.2.1-10​ |
| Link adaptation​ | Based on CSI-RS​ |
| Traffic Model​ | - Full buffer as default option - FTP traffic when specified |
| Inter-panel calibration for UE​ | Ideal​ |
| Control and RS overhead​ | common overhead is 30% based on TR 37.910 for DenseUrban-eMBB scenario [2] |
| Control channel decoding​ | Ideal |
| UE receiver type​ | MMSE-IRC​ |
| BF scheme​ | Analog Beamforming |
| TRP selection | Based on RSRP |
| BS receiver Noise Figure​ | 7 dB​ |
| UE receiver Noise Figure​ | 10 dB​ |

## BM Case-1: Spatial Domain Beam Prediction

### Set A/B are DL Tx Beams

Here, we provide the characterization and baseline performance evaluations for Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams.

#### Evaluation Methodology and Simulation Results

##### Simulation Assumptions

In this section, the synthetic dataset generated for the SLS assumptions is detailed. No history of RSRP measurements is required at the input of the ML model for beam prediction. The generated dataset consists of a large number of training samples from different UEs’ spatial positions.

For each UE, we collected the RSRP values measured at the UE assuming UE applies the optimal Rx beam, and with the gNB sweeping all the Tx beams over successive synchronization signal blocks (SSBs) and repeating the same operation for different Rx beams. In addition, we collected other relevant statistics such as the UE position, serving gNB position, gNB panel array parameters (bearing angle, mechanical downtilt, slant angle), and the beam pointing direction of the measured DL Tx beams, the beam ID of the measured DL Tx beams. We repeated the same data collection process, generating 200 different drops of the network to form a dataset with 42000 UEs and related RSRP measurements and extra statistics.

In Table 2.2-1, the training configurations and the list of hyper-parameters such as the number of layers, number of neurons, activation function, and other parameters are provided. The ML model is evaluated against the baseline assumptions, obtaining at first the KPIs related to the beam prediction accuracy, and then the system throughput performance per UE.

Table 2.2-1: Spatial Domain Beam Prediction ML Model Training Parameters

| Parameter | Value |
| --- | --- |
| ML Model Type | Deep Neural Network (DNN), Convolutional Neural Network (CNN) |
| Activation function | ReLu/Tanh/Leaky ReLu |
| Dropout | Enabled |
| Normalisation | Max-Min, layer normalization |
| Optimizer | Adam/SGD |
| Learning rate | Fixed or LR scheduler |
| Residual connections | Enabled |

Note:	Some of the ML model parameters are fine-tuned during the training.

We also provide throughput analysis simulations for the scenario in which Set B is different from Set A. The simulation procedure assumes UE attaching to the gNB based on the SSB measurements, and further performs a subset of CSI-RS measurements. These measurements are used as input of the ML model to predict the best CSI-RS beam for data transmission. We do not scale the throughput for the SLS results by accounting for the saved overhead as we understand the saved CSI-RS measurement overhead is non-critical [2].

##### Simulated Alternatives and Combinations

The following BM-Case1DL TX alternatives have been considered for evaluation:

BM-Case1DL TX Alt. 1 - Set B is a subset of Set A

- BM-Case1DL TX Alt. 1-1

  - ML model input: Set B beam L1-RSRP

  - ML model output: Set A best beam ID

  - Model training and testing with the same Set B

- BM-Case1DL TX Alt. 1-2

  - ML model input: Set B beam L1-RSRP + assistant info

    - Assistant info:

      - the beam ID for the measured beams.

      - the beam angle and/or the beam boresight direction for the measured DL Tx beams from NW to UE.

      - the UE position information.

      - the UE’s angle relative to a panel array of the gNB.

  - ML model output: Set A best beam ID

  - Model training and testing with the same Set B

- BM-Case1DL TX Alt. 1-3

  - ML model input: Set B beam L1-RSRP

  - ML model output: Set A best beam ID

  - Model training and testing with the different Set B

BM-Case1DL TX  Alt.2 - Set B is different to Set A

  - ML model input: Set B beam L1-RSRP

  - ML model output: Set A best beam ID

  - Model training and testing with the same Set B

The BM-Case1DL TX alternatives are further summarized in Table 2.2-2.

Table 2.2-2: Alternatives for BM-Case1DL TX.

| Method | Description | Set B Assumption |  | ML Input | ML Output |
| --- | --- | --- | --- | --- | --- |
|  |  | Model Train | Model Test |  |  |
| BM-Case1DL TX Alt. 1-1 | Set B is a subset of Set A | Same |  | Set B RSRP | Best beam ID in Set A |
| BM-Case1DL TX Alt. 1-2 |  | Same |  | Set B RSRP + Assistant Info |  |
| BM-Case1DL TX Alt. 1-3 |  | Random | Fixed | Set B RSRP |  |
| BM-Case1DL TX Alt. 2 | Set A and Set B are different | Same |  | Set B RSRP |  |

Notes: Set A is for DL beam prediction and Set B is for DL beam measurement.

##### Baseline Assumptions

Based on the agreed baselines from RAN1#109e, the following options are considered:

- BM-Case1 Baseline-option 1: Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping)

- BM-Case1 Baseline-option 2-1 (Set B is a subset of Set A/ Set B is different from Set A): Select the best beam within Set B

- BM-Case1 Baseline-option 2-2 (Set B is different from Set A): Hierarchical search for the best narrow beam from the best wide beam.

##### Simulation Results for Set B is a Subset of Set A

###### Fixed Set B for Training and Testing

Table 2.2-3: BM Case-1DL TX Alt. 1-1. Model Performance KPI

| Method | Prediction Accuracy (1 dB RSRP Margin) [%] | Average RSRP Error [dB] | RS overhead reduction [%] |
| --- | --- | --- | --- |
| BM-Case1 Baseline-option 2-1, Set B 16 beams | 37 | 2.95 | 75 |
| BM-Case1 Baseline-option 2-1, Set B 8 beams | 20 | 6.11 | 87.5 |
| BM-Case1 Baseline-option 2-1, Set B 4 beams | 11 | 9.597 | 93.75 |
| ML-based beam selection (Set B 32 beams) | 99 | 0.03 | 50 |
| ML-based beam selection (Set B 16 beams) | 96 | 0.2 | 75 |
| ML-based beam selection (Set B 8 beams) | 86 | 0.8 | 87.5 |
| ML-based beam selection (Set B 4 beams) | 69​ | 2.605​ | 93.75 |

Table 2.2-4: BM Case-1 (DL TX) Alt. 1-1. System Throughput Simulation.

| Method | Average cell throughput ratio – Method/Ideal [%] | 5th percentile UE throughput ratio – Method/Ideal [%] |
| --- | --- | --- |
| BM-Case1 Baseline-option 2-1, Set B 64 beams Ideal case | 100 | 100 |
| BM-Case1 Baseline-option 2-1, Set B 32 beams | 98 | 97 |
| BM-Case1 Baseline-option 2-1, Set B 16 beams | 85 | 86 |
| BM-Case1 Baseline-option 2-1, Set B 8 beams | 73 | 75 |
| ML-based beam selection (Set B 32 beams) | 100 | 100 |
| ML-based beam selection (Set B 16 beams) | 100 | 99 |
| ML-based beam selection (Set B 8 beams) | 98 | 84 |

From Table 2.2-3 and Table 2.2-4, we have the following observations:

- From Table 2.2 3, when using 32 beams Set B RSRP to predict the best beam in 64 beams Set A, the prediction accuracy is 99% and the prediction RSRP error mean is 0.2 dB, which is negligible. However, when using 16/8/4 beam Set B RSRP to predict 64 beams Set A, the prediction accuracy drops and the prediction RSRP error mean increases.

- From Table 2.2 4, the 32 beams baseline has a similar throughput to the 64 beams baseline, and the 16/8 beams baseline has worse throughput than the 64/32 beams baseline. Combining with bullet 1, it indicates that given the current agreed antenna configuration setup, using a large number of beams in Set A (i.e. 64 beams) is not useful as it may not provide any additional system throughput gain but introduce latency. Also, using a large number of beams in Set A may cause misinterpretation of the beam prediction performance as the beams in Set A are too correlated (too close to each other). For example, if Set A has 128 beams and Set B has 32 beams, now Set BSet A=14 but the prediction accuracy can still be close to 100%. For BM-Case1DL TX - Set B is a subset of Set A, companies should agree on the total number of beams in Set A based on the current agreed antenna configuration.

For BM-Case1, given the current agreed NW antenna configuration, the number of DL Tx beams in Set A should be 32 or 64.

- From Table 2.2-3 and Table 2.2-4, ML-based 16 beams have high prediction accuracy (>95%) and the prediction RSRP error mean is lower than 1 dB, and its good prediction performance is also reflected in the throughput that it has similar system throughput compared to the ideal baseline. On the other hand, from ML-based 8/4 beams, the prediction accuracy drops by a significant margin, and the ML-based 8 beams start losing nonnegligible cell-edge UE throughput compared to the ideal case. Therefore, if the beam prediction model input ONLY uses a “sparse” Set B or a poor Set B pattern design for the UE, may cause throughput loss, especially for the cell-edge UE. By this point, we understand that in certain cases, Set B RSRP may not be sufficient for beam prediction input, and additional assistant info may be needed to improve the prediction performance.

- For BM-Case1, a large number of beams in Set B (e.g., 32) may not improve the prediction accuracy and the system throughput. Therefore, ML-based beam selection should consider a Set B with a maximum of 16 beams when Set A has 64 beams, hence Set B should have a max of ¼ of Set A beams.

The design of Set A/B together with the ML model design should provide comparable or better sector throughput and cell-edge UE throughput compared to the non-ML baseline.

- For BM-Case1, a “sparse” Set B, or a random Set B pattern design, may cause throughput loss, especially for the cell-edge UE.

- For BM-Case1, Set B RSRP may not be sufficient for beam prediction input in certain cases.

For BM-Case1, RAN1 further study the case of Set A/B are DL Tx and Set B is a subset of Set A.

  - When Set B is a subset of Set A, RAN1 should consider a Set B with a maximum number of DL Tx beams that is ¼ of Set A beams.

We show our simulation results with model input as Set B beam RSRP + assistant info.

Table 2.2-5: BM Case-1DL TX Alt. 1-2. Model Performance KPI.

| ML Model & Assistance Info​ | Set B/A Dimensions​ | Accuracy with 1 dB margin [%]​ | RSRP Error mean [dB]​ | RS overhead reduction [%] |
| --- | --- | --- | --- | --- |
| RSRP+UE Angle​ | 8/64 | 89 | 0.54 | 87.5 |
| RSRP+UE Pos​ |  | 87​ | 0.69 |  |
| RSRP only​ |  | 87 | 0.74 |  |
| RSRP+Beam ID​ |  | 87​ | 0.76 |  |
| RSRP+Beam Angle​ |  | 86​ | 0.78 |  |
| RSRP+UE Angle​ | 4/64 | 82 | 1.02 | 93.75 |
| RSRP+UE Pos​ |  | 81​ | 1.25 |  |
| RSRP+Beam ID​ |  | 69​ | 2.54 |  |
| RSRP only​ |  | 69​ | 2.61​ |  |
| RSRP+Beam Angle​ |  | 69​ | 2.61 |  |

Notes: Rx beam selection fixed to panel boresight direction.

From Table 2.2-5, we make the following observations:

- Reducing the number of RSRP measurements used for the ML model input makes more relevant the use of assistance info as input of the ML model. Reducing the Set B dimension, the ML model has difficulties predicting the best beam, i.e., the angular directions with the highest RSRP values, especially when the few beams used as input, all show low RSRP values. In this case, the use of assistance info like the UE position becomes crucial to help the ML model to learn that the best beam has angular directions toward the UE position instead of any other angular directions (including the ones with low RSRP values). The use of the UE’s angle relative to a panel array of the gNB as assistance information, confirm this behavior, as it is easy for the ML model to interpret angles instead of coordinates, predicting those challenging cases even better.

- For BM-Case1, the ML model using as input only RSRP measurements has performances that reduce significantly changing the number of RSRP measurements from 8 to 4, i.e. further down sampling Set A, from a ratio of ¼ to a ratio of 1/8.

- For BM-Case1, when the ML model use the UE angle as the assistance information, it has a better performance than all the other variants.

- For BM-Case1, the ML model using as input RSRP measurements and UE Position has performances that outweigh the performance of the ML model using only RSRP.

- Using assistance information like Beam Angle and Beam ID related to the measured beams may not significantly improve the performance of the ML model that has input a fixed pattern of RSRP measurements. We believe that this behavior is due to the use of a fixed pattern of measurements for all the training samples. In this way, the ML model learns implicitly from the RSRP vector, that there are N different beams. Therefore, the information about the beam ID and beam angle of the measured beams does not have an impact on the prediction performance. The same conclusion may not hold when the measured beams used for ML model input are chosen at random by the UE.

- For BM-Case1, using assistance information like Beam Angle and Beam ID related to the measured beams may not significantly improve the performance of the ML model using as input only RSRP with a fixed pattern.

For BM-Case1, RAN1 further study the use of assistance information at the ML model input. The following assistance information can be prioritized:

- the beam angle and/or the beam boresight direction for the measured DL Tx beams from NW to UE.

- the UE position information.

- the UE’s angle relative to a panel array of the gNB

For BM-Case1, RAN1 further study Set B to be a fixed pattern.

###### Random Set B for Training

In the following, we consider the scenario BM-Case1DL TX Alt. 1-3. In RAN1#109e, the random Set B idea is proposed [1]. We understand that the benefit of training an ML model with random Set B is avoiding switching/indication/ transferring overhead/procedure for the multiple fixed Set B based trained models. Under the discussion of DL Tx beam prediction, we understand that:

- If the model inference is on the NW side, the random Set B based model is not necessary since NW has full control over Set A/B and the corresponding trained models. The model switching overhead/procedure is not critical.

- If the model inference is on the UE side, the random Set B based model can help UE to save model switching/indication/ transferring overhead. Nevertheless, ML performance may decrease because the Set B configuration may change between training and inference operation.

Even if the DL Tx beam prediction is on the UE side, it is still up to NW to configure the Set B beams. We understand that for beam prediction application, NW should configure to UE with a Set B which is beneficial for beam prediction. Therefore, the testing Set B should not be random but a predefined fixed pattern.

Overall, the benefit of applying DL Tx beam prediction on the UE side is still not clear and needs to be discussed.

For BM-Case1 model inference applies at the NW side, with DL Tx beams considered for Set A and Set B, the training a model with random Set B is not needed.

- For BM-Case1DL Tx model inference in UE side, training model with random Set B may reduce model switching/indication/ transferring overhead for UE. But the benefit of BM-Case1DL Tx model inference on the UE side is not yet clear.

For BM-Case1 model inference applies at the UE side, with DL Tx beams considered for Set A and Set B, the training a model with random Set B can be further studied.

##### Simulation Results for Set B is Different to Set A

###### Fixed Set B for Training and Testing

As mentioned in our other BM paper [5], for BM-Case1DL TX- Set B is different from Set A, we mainly consider Set B is a wide beam codebook and Set A is a refined (narrow) beam codebook. In the following, we will try to compare the Set A prediction performance based on different choices of Set B wide beam codebook, and mainly we consider the following wide beam construction methods:

- Wide beam codebook#1 - baseline

  - The wide beam codebook construction is based on [4]

- Wide beam codebook#2

  - The wide beam codebook construction is adding circular shift operation on top of [4]

- Wide beam codebook#3

  - Each wide beam codebook is the summation of the randomly selected refined beam in Set A.

- Wide beam codebook#4

  - The wide beam codebook is constructed in a hybrid fashion - some wide beams are constructed based wide beam codebook#1 method while other wide beams are constructed based on the wide beam codebook#3 method. The hybrid wide beam codebook has the same coverage as Set A.

Figure 2-1: Examples design of wide beam codebook #1 #2 #3.

Examples of wide beam codebook#1/#2/#3 are shown in Figure 2-1

All wide beam codebooks combine 4 refined beams into 1 wide beam. Based on our Proposal 1:, in the following, we consider Set A with 64 and 32 refined beams and Set B will have 16 and 8 wide beams correspondingly. Table 2.2-6 shows the model performance KPI for different wide beam codebooks

Table 2.2-6: BM Case-1 (DL TX) Alt.2 Model Performance KPI.

| Method | Prediction Accuracy (1 dB RSRP Margin) [%] |  | Average RSRP Error [dB] |  | RS overhead reduction [%] |
| --- | --- | --- | --- | --- | --- |
|  | ¦Set B¦ / ¦Set A¦ |  | ¦Set B¦ / ¦Set A¦ |  |  |
|  | 16/64 | 8/32 | 16/64 | 8/32 |  |
| ML-based beam selection Wide beam codebook#1 - baseline | 94 | 73 | 0.26 | 3.16 | 75 |
| ML-based beam selection Wide beam codebook#2 | 96 | 90 | 0.13 | 0.69 |  |
| ML-based beam selection Wide beam codebook#3 | 99 | 94 | 0.07 | 0.45 |  |
| ML-based beam selection Wide beam codebook#4 | 97 | 91 | 0.11 | 0.65 |  |

Table 2.2-7: BM Case-1(DL TX) Alt. 2 System Throughput Simulation with Set A 32 beams

| Method | Average cell throughput ratio – Method/ baseline [%] | 5th percentile UE throughput ratio – Method/ baseline [%] |
| --- | --- | --- |
| BM-Case1 Baseline-option 2-2 WB#1 with hierachical search1 -baseline | 100 | 100 |
| ML-based beam selection WB#1 | 92 | 71 |
| ML-based beam selection WB#2 | 98 | 93 |
| ML-based beam selection WB#3 | 92 | 71 |
| ML-based beam selection WB#4 | 99 | 94 |

From Table 2.2-6 and Table 2.2-7, we have the following observations:

- For Set A with 64 beams, the designed wide beam codebook #2/#3/#4 have marginal performance gain compared to the baseline wide beam codebook#1. This is mainly because of the large/oversampled beam number 64 in Set A for the current antenna configurations - with a 1 dB error margin, the wrong refined beam prediction may highly possible has less than 1 dB RSRP difference compared to the actual best beam since the refined beams now are too close to each other.

- For Set A with 32 beams, the designed wide beam codebook #2/#3/#4 perform significantly better than the baseline wide beam. The idea behind the wide beam codebook #2/#3/#4 can be found in our other BM paper [5]. And as expected, randomly combining the refined beam to form the wide beam can provide the richest correlation between the refined beam and the wide beam codebook.

- From Table 2.2-7, one can see that the refined beam prediction with WB#1 has significantly worse throughputs than the baseline. On the other hand, the suggested WB#2, #3, #4 have comparable throughput to the baseline. One should notice that the assumption for WB#2, #3, #4 is using the wide beam codebook for SSB and use the SSB measurements to predict the refined beams, while the hierarchical search will additionally require UE specific CSI-RS for P2 beam management. Also, as we point out in [5], WB#3 may not be good for SSB as it will degrade the cell coverage, and the throughput results also tells that the wide beam codebook with best beam prediction results may not be beneficial for throughput. For Set B is different to Set A with Set B is wide beam, the KPI for the wide beam codebook design should be both prediction accuracy and throughput performance.

- For Set B is different to Set A with Set B is wide beam, the KPI for the wide beam codebook design should be both prediction accuracy and throughput performance.

For BM-Case1, RAN1 may further study the case of Set A/B are DL Tx and Set B/Set A are different.

- Set B is a wide beam codebook and Set A is a refined beam codebook

- Advance Set B designs are needed to provide sufficient refined beam prediction performance.

### Set A/B are DL Tx-Rx Beam Pairs

The data generation and the simulation assumption will be similar to section 2.2.1, the only difference for DL Tx-Rx beam pairs is that now the training data include the UE best panel Rx beams information.

##### Simulated Alternatives

The following BM-Case1DL TX-RX alternatives have been considered for evaluation:

BM-Case1DL TX-RX Alt. 1

- ML model input: Set B beam pair L1-RSRP

- ML Model output: Set A best beam pair ID

- Model training and testing with different Set B – random Set B model training

##### Simulation Results for Set B is a Subset of Set A

###### Random Set B for Training

For BM-Case1DL TX-RX, we further consider training a model for generalization of Set A/B. Here, since Set A/B are beam pairs set, by the generalization of Set A/B we infer to generalization of the DL Tx beam part but not the Rx beam. This is because we think it will make more sense to have beam pair prediction on the UE side and because UE will have full knowledge about the Rx beams, therefore the generalization need is only for the Tx beam part.

To train the model for generalization, similar to the method in section 2.2.1.1.4.2, we consider the following model training and testing procedure:

- The model training output space is the beam pair set Set C = SetCDL TX (1000 beams)  × SetDL RX (8 beams), and the training input space is a random subset from Set C.

  - During training, the training beam pair data comes from randomly selected DL Tx beams from SetCDL TX (a generalization of DL Tx beam), but the corresponding Rx beam will be selected, instead of random selection, with a certain pattern (see section 2.3.1.2 in [6]). This is because UE should select the best Rx beam pattern for improving beam pair prediction.

- The model testing output space is the beam pair set Set A = SetADL TX (64 beams)  × SetDL RX (8 beams), and the testing input space is the beam pair set Set B has element number Set B=SetADL TX  or 12 |SetADL TX|.

  - During the model inference, NW transmits the RS through the DL Tx beams with or without configuring “repetition” for the RS resources. Here the assumption is NW does not configure any “repetition” for any RS. Therefore, UE can maximally collect SetADL TX  measurement from SetADL TX  number of DL Tx beams. And it is up to UE to choose which Rx beam to receive the SetADL TX DL Tx beams. Based on the SetADL TX measurement, UE predicts the best beam pair. Additionally, we also simulate the case that NW only uses 12 |SetADL TX| DL Tx beams.

  - Similar to section 2.2.1.1.4.2, we continue to assume NW will use the DL Tx beam pattern that is beneficial for beam prediction instead of a random pattern. Therefore, in model testing, we assume the DL Tx beams in Set B has a fixed pattern. And we also assume UE will apply a certain Rx beam pattern for receiving.

The simulation results are shown in Table 2.2-8.

Table 2.2-8: BM-Case1 (DL TX-RX) Alt. 1 Model Performance KPI.

| Method | Average RSRP Error [dB] |  |
| --- | --- | --- |
|  | Fixed DL Tx pattern |  |
|  | ¦Set B¦ = 64 = ¦SetADL TX¦ | ¦Set B¦ = 32 = ½ ¦SetADL TX¦ |
| BM-Case1DL TX-RX Alt. 1 Trained with random subsets of Set C | 0.9 | 3.7 |

From Table 2.2-8Table 2.2-9, we have the following observations:

- For DL Tx-Rx beam pair prediction, the prediction target space SetADL TX × SetDL RX. Based on the simulation results it shows that it requires a relatively larger number of measurements to have good beam pair prediction, i.e. |Set B| = |SetADL TX|.

- For DL Tx-Rx beam pair prediction, the prediction target space SetADL TX × SetDL RX. And it requires a large number of measurements to have good beam pair prediction.

- With the same number of measurements, the beam pair prediction performance is much worse than the one for Tx beam prediction shown in Table 2.2-6. This indicates that the two-step approach for Tx and Rx beam prediction may provide better accuracy results than the joint Tx/Rx beams prediction. Also, assuming given a performance target, doing independent Tx beam, Rx beam prediction may use a smaller number of measurements compared to joint Tx-Rx beam pair prediction. In conclusion, we suggest RAN1 further investigates the comparison between independent Tx beam, Rx beam prediction, and the joint Tx-Rx beam pair prediction.

RAN1 further investigates the comparison between independent Tx beam, Rx beam prediction, and joint Tx-Rx beam pair prediction.

### Model Generalization for Different Configurations

Assessing the model's generalization performance can be carried out in different ways. In a first option (Option 1), the model generalization capability is assessed towards new data generated with the same configuration/scenario. In this option, the configuration used for training the ML model remains unchanged during the ML model inference. Nevertheless, for practical reasons, a single ML model may be used for inference in different configurations or scenarios, implying that the ML model should accurately predict the beam even for unseen configurations/scenarios during the training operation. Therefore, in a second option (Option 2), the ML model is tested with a configuration or in a scenario that is not included in the training data. In this second option, the ML model performance is generally poor, since changing the configuration may lead to different data distributions and characteristics. Therefore, a third option (Option 3) assumes training the ML model with a mix of data collected with multiple configurations/scenarios. In this last option, a single ML model should adapt to the changes of the configurations/scenarios, without significant impact on performance.

#### Model Generalization for Different gNB Antenna Array Configurations

In this subsection, we study the model generalization capabilities for different gNB antenna array configurations. The following gNB antenna array configurations have been considered for evaluation:

- gNB antenna array Configuration-A:

  - One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ

  - Set A = 64 Tx Beams

  - Set B = 16 Tx Beams

- gNB antenna array Configuration-B:

  - One panel: (M, N, P, Mg, Ng) = (8, 16, 2,1, 1), (dV, dH) = (0.5, 0.5) λ

  - Set A = 64 Tx Beams

  - Set B = 16 Tx Beams

We summarize in Table 2.2-9 the list of results obtained combining the configurations in different ways during training and inference operations.

Table 2.2-9: BM Case-1(DL TX) training/testing configuration for evaluating ML model generalization. In all configurations Set A = 64 Tx Beams and Set B = 16 Tx Beams.

|  | ML Model Training Configuration | ML Model Testing Configuration | Results |
| --- | --- | --- | --- |
| Option 1 | Configuration-A | Configuration-A | Figure 2-2 |
|  | Configuration-B | Configuration-B | Figure 2-3 |
| Option 2 | Configuration-A | Configuration-B | Figure 2-2 |
|  | Configuration-B | Configuration-A | Figure 2-3 |
| Option 3 | Configuration-A + Configuration-B (50% split) | Configuration-A | Figure 2-4 |
|  | Configuration-A + Configuration-B (50% split) | Configuration-B | Figure 2-4 |

Figure 2-2 shows the CDF of the RSRP error for the case when the ML model is trained and tested on the same configuration of gNB antenna array (Option 1) and for the case when the ML model is trained on the configuration with 4x8 antenna elements and tested with a larger antenna configuration of 8x16 antenna elements (Option 2). Note that the ML model does not generalize well for the larger antenna array size and the beam prediction accuracy falls below 50%. This is due to the different gaps between beams when the antenna configuration changes from 4x8 to 8x16, which modifies the distribution of the RSRP measurements for the ML model input, making the beam difficult to predict.

A similar trend can be observed in Figure 2-3 where the ML model is trained on the configuration with 8x16 antenna elements and tested with a smaller antenna configuration of 4x8 antenna elements. Also, in this case, the beam prediction accuracy for the unseen antenna configuration of 4x8 has particularly low performance, whereas the prediction accuracy for the trained antenna configuration of 8x16 is about 85% with 1 dB margin.

Therefore, with Option 2 for supporting the two gNB antenna array configurations, two ML models may be required, and a switching mechanism should be in place to change between different ML models.

Figure 2-2: ML model is trained with gNB antenna array 4x8 and tested with gNB antenna arrays of 4x8 and 8x16.

On the other hand, Figure 2-4 shows the case when a dataset generated with mixed configurations is used for training a single ML model (Option 3). Performances are lower than the case when the ML model is trained and tested on the same configuration of the gNB antenna array.

With Option 3 for supporting the two gNB antenna array configurations a single ML model can be used. Nevertheless, some performance degradations are expected compared to the case when the ML model is trained with data from only one configuration of the gNB antenna array. Supporting multiple number of configurations may further reduce the ML performance.

Figure 2-3: ML model is trained with gNB antenna array of 8x16 and tested with gNB antenna arrays of 4x8 and 8x16.

Figure 2-4: ML model is trained with mixed data with gNB antenna array of 8x16 and 8x16, then tested with gNB antenna arrays of 4x8 and 8x16.

- model's generalization capabilities should be assessed considering different combination of configurations as ML model performances can be affected significantly.

- Several configurations/scenarios can be considered for assessing the ML model generalization capabilities. In this study, we considered the gNB antenna array dimensions, but other configurations/scenarios are not precluded. Supporting multiple configurations may further affect ML performance.

#### Model Generalization for Different Set A/B

By creating a large and oversampled beam Set C as the prediction target set is essentially equal to predicting the beam Angle of Departure (AoD) angles. From RAN1#109e, some companies propose to study the model generalization for different Tx beams, which we think by training a model based on a large/oversampled Set C satisfying Set B∈Set A∈Set C for any given Set A/B can handle the Set A/B generalization issue.

- For BM-Case1, the Set A/B model generalization issue can be addressed with a training model based on an oversampled Set C that satisfies Set B∈Set A∈Set C for any given Set A/B.

As a starting point of the Set A/B model generalization study, we focus on the case Set B is a subset of Set A and create a Set C with 1000 oversampled DL Tx beams and Set A has 64 beams. The model training and testing method is shown as follows:

Figure 2-5: Model training with Set C.

Figure 2-6: Model inference with Set A and Set B.

Figure 2-5 shows an example of the model training, where all the grey box represents the beams in Set C and the grey boxes are position sensitive – the position of a grey box uniquely maps to one beam in Set C. The green boxes represent the randomly selected beams for training and the model will try to output the best beam probability for all grey boxes. The model input is only with beam RSRP.

Figure 2-6 shows that after model training, use the Set B RSRP as model input, and only select the prediction result for the beams in Set A.

In Table 2.2-10, we show our simulation results. Based on the discussion above, model is tested based on certain fixed Set B patterns.

Table 2.2-10: BM Case-1(DL TX) Alt.1-3 Model Performance KPI.

|  | ML Model Training Configuration | ML Model Testing Configuration | Average RSRP Error [dB] |
| --- | --- | --- | --- |
| Option 1 | BM Case-1DL TX Alt. 1-1 ML-based beam selection (Set B 16 beams) | Fixed Set B 16 beams | 0.16 |
|  | BM Case-1DL TX Alt. 1-1 ML-based beam selection (Set B 32 beams) | Fixed Set B 32 beams | 0.03 |
| Option 2 | BM Case-1DL TX Alt. 1-1 ML-based beam selection (Set B 16 beams) | Fixed Set B 32 beams | 2 |
|  | BM Case-1DL TX Alt. 1-1 ML-based beam selection (Set B 32 beams) | Fixed Set B 16 beams | 3.6 |
| Option 3 | BM Case-1DL TX Alt. 1-3 ML-based beam selection (random Set B) | Fixed Set B 32 beams | 0.06 |
|  | BM Case-1DL TX Alt. 1-3 ML-based beam selection (random Set B) | Fixed Set B 16 beams | 0.55 |

From Table 2.2-10, we have the following observations:

- Training models based on fixed Set B pattern (i.e., 16, 32 beams) have poor generalization capability on the mismatch testing Set B pattern. The beam prediction capability will drop significantly if the training Set B does not match the testing Set B.

- For BM Case1, the training model with a fixed Set B pattern will have poor beam prediction performance if the testing Set B does not match with the training Set B.

- On the other hand, training the model with random Set B is possible to provide beam prediction performance close to the optimal case – training and testing on the same Set B.

- For BM-Case1, training the model with random Set B is possible to provide beam prediction performance close to the optimal case – training and testing on the same Set B.

Overall, we think RAN1 shall define cases to investigate the generalization performance of ML models that are considered in BM-Case1 (and similar set-up may be valid for BM-Case2).

RAN1 further investigates the ML model generalization capabilities of BM Case-1 with configurations that can be either included or not included in the training dataset. The following list of configurations can be prioritized:

- gNB antenna array dimensions, e.g., 4x8 and 8x16.

- UE antenna array dimensions/Number of Panels.

- Set A dimension, e.g., 64 beams and 128 beams.

- Set B dimension, e.g., 16 beams and 32 beams.

RAN1 further investigates the ML model generalization capabilities of BM Case-1 with scenarios that can be either included or not included in the training dataset. The following list of scenarios can be prioritized:

- Channel propagation models, e.g. UMa/UMi.

- Outdoor/Indoor UE distribution (e.g. 100% Oudoor, 80% Outdoor/20% Indoor).

## Further Enhancements on BM Case-1: Beam Prediction in Spatial Domain for Improving the Throughput and Latency

As highlighted by the above sections, when the BM-Case1 is applied at the gNB or UE, the node can predict Top-N1 beams based on limited measurements. However, these predictions may not fully suit as the beams to be used by the gNB if the traffic situation towards different UEs are different. Therefore, BM-Case1 shall be further enhanced by applying reinforcement learning techniques, and we refer it as RL-based beam prediction provided in section 2.1.2/2.2.2 of R1-2204574 [2].

### Evaluation Methodology and Results

#### Simulation Assumptions

In this section, based on the assumptions given in Table 2.1-3 and Table 2.3-1, we use the system level simulations to evaluate the performance of spatial domain beam prediction for improving throughput and latency. Furthermore, to simulate the errors that would occur when a ML model is used to predict the RSRP values, we have added noise to the RSRP values of the beams in set A which are not measured. The mean and std. deviation of the normal distribution used here to generate noise samples are the mean and std. deviation of the errors that we observed when testing a RSRP prediction model which is trained for the same scenario. Here, the RSRP prediction is performed before L1/L2 filtering. Hence noise is added before any filtering occurs. For the baseline case where the best beam is selected based on L1-RSRP, noise is not added to the beams which are not measured.

Here, LinUCB, a contextual multi-armed bandit approach proposed in [4], is trained online to estimate a QoS based metric for each beam by exploring different beams and observing the QoS based metric as reward. Given the measured RSRP values of the best N1 beams along with beam usage values as a context vector to the model, the model will estimate QoS based metric for each action, i.e., the beam ID. Here, the QoS based metric is the amount of data, in bytes, transmitted during a single measurement interval. To mitigate the risk of beam failures in the exploration, in the simulations, it is constrained to the best N1 beams.

System level simulations were carried out for different loads (data rates) assuming an FTP traffic model. The UEs consist of 100% eMBB (best-effort) UEs.

Table 2.3-1: Additional assumptions for beam prediction for improving throughput and latency simulations.

| Parameter | Value |
| --- | --- |
| Beam measurement reporting interval | Periodical with 40 ms periodicity |
| Traffic Model​ | FTP traffic model |
| FD scheduling method | By RB |
| FD scheduling algorithm | Even resources |
| TD beam selection priority metric | Min. HOL delay |
| TD no. of beams activated in a TTI | 1 |
| Set B | 16 beams and set B is a subset of set A |
| Noise (ML error) mean | 0.45 |
| Noise (ML error) std. deviation | 5.49 |

For LinUCB model, the parameters can be found in Table 2.3-2.

Table 2.3-2: ML Model Training Parameters

| Parameter | Value |
| --- | --- |
| ML Model Type | LinUCB contextual multi-armed bandit algorithm |
| Exploration weight (α) | 0.1 |
| Reward | Data in bytes transmitted during a single measurement interval |
| No. of beam measurements in the input/No. of beams that can be explored (N1) | 4 |

#### Baseline Assumption

The baseline specific for spatial domain beam prediction is detailed in Table 2.3-3.

Table 2.3-3: Baseline for Spatial Domain Beam Prediction.

| Method | Description |
| --- | --- |
| Option 1 | Select the best beam based on the L1-RSRP reported by the UE |

#### System-Level Simulation Results

From , Figure 2-7 it can be observed that the QoS based beam prediction improves the overall per UE throughputs by finding the beams, which are activated more frequently, for the UEs discriminated by the scheduler due to the limitations in the no. of beams that can be activated simultaneously. It can also be observed that the gains at different percentiles depend on the data rate. Here, in our understanding, the improvements in data rates are a result of multiple UEs being clustered to a single beam with acceptable SINR, rather than serving each UE with its best RSRP based beam. As a results, when a certain beam which serves multiple UEs, is activated in a given TTI, scheduler has more traffic to fill the entire bandwidth. Furthermore, Figure 2-8 presents the throughput gains when the input RSRPs of the beams which are not measured are corrupted with noise (i.e., beams not in set B). There, it can be observed that the QoS based model provide gains even when the RSRP values of the beams which are not measured are corrupted with noise. This robustness enables the QoS based beam prediction model to take the predicted RSRP values from a ML model and use them to predict the QoS based best beam.

- Selecting the beam based on the QoS based model output can improve the throughput performance of each UE by clustering the UEs to a single beam.

- Even with errors in the input RSRP values of beams which are not in set B, QoS based beam selection can improve the throughput performance of each UE.

For BM-Case1, RAN1 further investigate QoS-based beam prediction with predicted RSRPs of the beams in set A.

Figure 2-7: Downlink throughput gain distribution obtained with QoS-based beam selection without any noise added to the input RSRP values.

Figure 2-8 Downlink throughput gain distribution obtained with QoS-based beam selection with noise added to the input RSRP values which are not measured (i.e., not in set B).

Table 2.3-4: BM Case-1 Alternative-1 System performance.

| Data rate | Average cell throughput Gain [%] |  | 5th percentile UE throughput Gain [%] |  |
| --- | --- | --- | --- | --- |
|  | Without noise added to the Set A measurements | With noise added to the Set A measurements | Without noise added to the Set A measurements | With noise added to the Set A measurements |
| 25 Mbps | 6.0% | 3.4% | -11.4% | -1.3% |
| 20 Mbps | 5.9% | 3.8% | -8.4% | 1.4% |
| 17 Mbps | 4.6% | 3.0% | -0.2% | 1.3% |
| 15 Mbps | 3.9% | 2.6% | 1.3% | 4.7% |
| 10 Mbps | 1.5% | 0.5% | 8.5% | 3.2% |

## BM Case-2: Temporal Domain Beam Prediction

Here we provide the characterization and baseline performance evaluations for Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams.

### Evaluation Methodology and Results

#### Simulation Assumptions

For simulations, the parameters can be found in Table 2.1-3. Additional parameters specific to the sub-use case can be found in Table 2.4-1.

In this section, the synthetic dataset is generated with the SLS assumptions detailed in Table 2.1-3. For the temporal domain beam prediction method, we adopt the trajectory model described in Section 2.4.1.2, where the UE follows a straight trajectory with random orientation. The ML model leverages the historical sequence of beam information (RSRP measurements and assistance information) to predict the information about the best beam (beam ID or RSRP) in a future time instant.

For data generation, we set up the simulation assuming 210 UEs uniformly distributed over the network. Also, the duration of the simulation is set to 4 s, which corresponds to the UE speed of 30 Km/h to a travelled distance of 33.33 m.

For ML model training, the parameters can be found in Table 2.4-2.

Table 2.4-1: Additional assumptions for spatial-temporal domain beam prediction.

| Parameter | Value |
| --- | --- |
| UE speed | 30km/h |
| UE trajectory model | Straight trajectory with random direction |
| CSI measurement/report periodicity | 40ms or 80ms |
| Channel Spatial consistency | Type A and B |

Table 2.4-2: ML Model Training Parameters

| Parameter | Value |
| --- | --- |
| ML Model Type | LSTM-based, Conv2D |
| Input layer Normalization | Amplitude scaling in [0,1] |
| Observation window length/prediction window length | 5/5 |
| Solver name | Adam, SGD |
| Learning rate | StepLR |

Table 2.4-3: ML (Bayesian Optimization) Model Parameters

| Parameter | Value |
| --- | --- |
| Lower bound on the number of beams measured per time slot | 4 |
| Upper bound on the number of beams measured per time slot | 32 |
| Spatial dimension kernel(s) | Matérn 3/2 |
| Temporal dimension kernel | Radial basis function |
| Acquisition function | Augmented Expected Improvement |
| Expected improvement threshold (arbitrary units) | 0.01 |

#### Mobility Model

Option #4: Random direction straight-line trajectories.

- Initial UE location, moving direction and speed: UE is randomly dropped in a cell, and an initial moving direction is randomly selected, with a fixed speed.

- The initial UE location should be randomly dropped within the following blue area

where d1 is the minimum distance that UE should be away from the BS.

- Each sector is a cell and the cell association is geometry based.

- During the simulation, inter-cell handover or switching should be disabled.

For training data generation

- For each UE moving trajectory: the total length of the UE trajectory can be set as T second if it is in time, of set as D meter if it is in distance.

- The value of T (or D) can be further discussed

- The trajectory sampling interval granularity depends on UE speed and it may change.

- UE can move straightly along the entire trajectory

- If the UE trajectory hit the cell boundary (the red line), the trajectory is terminated.

- If the trajectory length (in time) is less than the length of the observation window + prediction window, the time samples are discarded from the training set.

- At the current stage, the length of the observation window + prediction window may adopt different values.

RAN1 further investigates the trajectory model for BM-Case#2, adopting Option #4 as a starting point for further studies.

#### Simulated Alternatives and Combinations

The list of alternatives for temporal domain beam prediction is detailed in Table 2.4-4.

Table 2.4-4: List of Alternative for Time Domain Beam Prediction.

| Method | Description |
| --- | --- |
| Method 1 | Set B is a fixed subset of Set A |
| Method 2 | Set B is a variable subset of Set A |
| Method 3 | Set B is the same as Set A |
| Methods 1 or 3 + Assistance Info | ML model input consists of L1-RSRP measurement based on Set B and assistance information ​ |

Note: Predicted beam(s) are selected from Set A and measured beams used as input are selected from Set B.

For BM-Case2, support Method 3 as the main scenario while other methods can be further studied :

- Method 1:	Set B is a fixed subset of Set A

- Method 2:	Set B is a variable subset of Set A

- Method 3:	Set B is the same as Set A

- Methods 1 or 3 + Assistance Info:	ML model input consists of L1-RSRP measurement based on Set B and assistance information

#### Simulations Results

Table 2.4-5: BM Case-2 Methods 1, 2 and 3.

| Method | Set B /Set A Dimensions​ | Observation Window​ [ms]​ | Prediction Window (F=1) [ms]​ | Accuracy with 1 dB margin [%]​ | RSRP Error mean [dB]​ | RS overhead reduction [%] |
| --- | --- | --- | --- | --- | --- | --- |
| Method 1 | 32/64​ | 200​ | 40​ | 74.83​ | 0.7699​ | 50% |
| Method 1 | 32/64​ | 200​ | 80​ | 70.77​ | 0.9451​ | 50% |
| Method 2 (Bayesian Optimization) | Variable/64​ | -​ | 80​ | 84.1 | 0.515 | 67% |
| Method 3 | 64/64​ | 200​ | 40​ | 89.25​ | 0.2901​ | 0% |
| Method 3 | 64/64​ | 200​ | 80​ | 79.81​ | 0.5867​ | 0% |

Notes: Method 1,3 use Spatial consistency Type B, Method 2 uses Spatial consistency Type A.

Table 2.4-6: BM Case-2 Methods 1 or 3 + Assistance Info.

| ML Model & Assistance Info​ | Set B /Set A Dimensions​ | Observation Window​ [ms]​ | Prediction Window [ms]​ | Accuracy with 1 dB margin [%]​ | RSRP Error mean [dB]​ | RS overhead reduction [%] |
| --- | --- | --- | --- | --- | --- | --- |
| RSRP+UE Pos​ | 64/64​ | 200​ | 40​ | 89.29​ | 0.2879​ | - |
| RSRP+UE Pos​ | 32/64​ | 200​ | 40​ | 75.17​ | 0.7574​ | 50% |
| RSRP+UE Pos​ | 64/64​ | 200​ | 80​ | 79.59​ | 0.5923​ | - |
| RSRP+UE Pos​ | 32/64​ | 200​ | 80​ | 70.77​ | 0.9451​ | 50% |

Notes: Spatial consistency Type B.

From Table 2.4-5 and Table 2.4-6, we make the following observations:

- From Table 2.4-5 for Methods 1 and 3, the ML model using as input only RSRPs has performance that decreases when reducing the number of measurements from 64 to 32 or when increasing the length of the prediction window. This is expected because a smaller number of measurements reduces the number of beams tracked and thus the possibility to predict which one would be the best in the future time instance. At the same time, a larger prediction window makes the prediction tasks more challenging as beam behaviour is more unpredictable with a larger prediction horizon.

- On the other hand, for the Method 2 in Table 2.4-5 using Bayesian Optimization to optimally choose beam indices for sampling allows for greater RS overhead reduction whilst maintaining good accuracy

- For BM-Case2, the ML model using as input only RSRPs has performance that decreases when Set B is a subset of Set A and if no advanced algorithm is applied for beam selection in Set B.

- For BM-Case2, the ML model using as input only RSRPs has performance that decreases when increasing the length of the prediction window.

- For BM-Case2, additional algorithm (i.e. Bayesian Optimization) should be applied for choosing the beam measurements in Set B for the scenario of Set B is a subset of Set A.

For BM-Case2, with Set B is a subset of Set A, measurement instances K and prediction instances F shall be carefully investigated prior supporting the sub-use case.

- For the results in Table 2.4-6 with assistance information, the ML model performance decreases when reducing the number of measurements from 64 to 32 or when increasing the length of the prediction window as much as in the ML model using as input only RSRP. We believe that this is due to the variation in the UE position within the observation window. For the UE speed of 30 Km/h, the distance gap between two successive time samples is ∆x=0.33 m, which is not a relevant variation for the ML model input data and can’t improve the information given by the history of RSPR measurements. Moreover, the UE trajectory is straight, which may facilitate the prediction with only RSPR measurements. Differently, a UE trajectory changing direction may become more difficult to predict, and in this case, we expect the use of assistance information such as UE position to become relevant.

- For the UE speed of 30 Km/h and prediction windows of 40 and 80 ms, the ML model using as input RSRP and assistance info (UE position) does not provide significant gains to the ML model using as input only RSRP.

For BM-Case2, RAN1 further verify whether there is any use of using assistance information at the input of the ML model. The following assistance information can be further considered:

- the UE position information.

Support RAN1 to further study scenarios/ trajectory model for the BM-Case2.

# Conclusion

In this contribution, we discuss remaining details of evaluation assumption of ML for Beam management. In particular, we have following observations and proposals.

Proposal 1: For BM-Case1, given the current agreed NW antenna configuration, the number of DL Tx beams in Set A should be 32 or 64.

Observation 1: For BM-Case1, a large number of beams in Set B (e.g., 32) may not improve the prediction accuracy and the system throughput. Therefore, ML-based beam selection should consider a Set B with a maximum of 16 beams when Set A has 64 beams, hence Set B should have a max of ¼ of Set A beams.

Observation 2: For BM-Case1, a “sparse” Set B, or a random Set B pattern design, may cause throughput loss, especially for the cell-edge UE.

Observation 3: For BM-Case1, Set B RSRP may not be sufficient for beam prediction input in certain cases.

Proposal 2:	For BM-Case1, RAN1 further study the case of Set A/B are DL Tx and Set B is a subset of Set A.

- When Set B is a subset of Set A, RAN1 should consider a Set B with a maximum number of DL Tx beams that is ¼ of Set A beams.

Observation 4: For BM-Case1, the ML model using as input only RSRP measurements has performances that reduce significantly changing the number of RSRP measurements from 8 to 4, i.e. further down sampling Set A, from a ratio of ¼ to a ratio of 1/8.

Observation 5: For BM-Case1, when the ML model use the UE angle as the assistance information, it has a better performance than all the other variants.

Observation 6: For BM-Case1, the ML model using as input RSRP measurements and UE Position has performances that outweigh the performance of the ML model using only RSRP.

Observation 7: For BM-Case1, using assistance information like Beam Angle and Beam ID related to the measured beams may not significantly improve the performance of the ML model using as input only RSRP with a fixed pattern.

Proposal 3:	For BM-Case1, RAN1 further study the use of assistance information at the ML model input. The following assistance information can be prioritized:

- the beam angle and/or the beam boresight direction for the measured DL Tx beams from NW to UE.

- the UE position information.

- the UE’s angle relative to a panel array of the gNB

Proposal 4:	For BM-Case1, RAN1 further study Set B to be a fixed pattern.

Proposal 5:	For BM-Case1 model inference applies at the NW side, with DL Tx beams considered for Set A and Set B, the training a model with random Set B is not needed.

Observation 8: For BM-Case1DL Tx model inference in UE side, training model with random Set B may reduce model switching/indication/ transferring overhead for UE. But the benefit of BM-Case1DL Tx model inference on the UE side is not yet clear.

Proposal 6: For BM-Case1 model inference applies at the UE side, with DL Tx beams considered for Set A and Set B, the training a model with random Set B can be further studied.

Observation 9: For Set B is different to Set A with Set B is wide beam, the KPI for the wide beam codebook design should be both prediction accuracy and throughput performance.

- Proposal 7: For BM-Case1, RAN1 may further study the case of Set A/B are DL Tx and Set B/Set A are different.

- Set B is a wide beam codebook and Set A is a refined beam codebook

- Advance Set B designs are needed to provide sufficient refined beam prediction performance.

Observation 10: For DL Tx-Rx beam pair prediction, the prediction target space SetADL TX × SetDL RX. And it requires a large number of measurements to have good beam pair prediction.

- Proposal 8: RAN1 further investigates the comparison between independent Tx beam, Rx beam prediction, and joint Tx-Rx beam pair prediction.

Observation 11: model's generalization capabilities should be assessed considering different combination of configurations as ML model performances can be affected significantly.

Observation 12: Several configurations/scenarios can be considered for assessing the ML model generalization capabilities. In this study, we considered the gNB antenna array dimensions, but other configurations/scenarios are not precluded. Supporting multiple configurations may further affect ML performance.

Observation 13: For BM-Case1, the Set A/B model generalization issue can be addressed with a training model based on an oversampled Set C that satisfies Set B∈Set A∈Set C for any given Set A/B.

Observation 14: For BM Case1, the training model with a fixed Set B pattern will have poor beam prediction performance if the testing Set B does not match with the training Set B.

Observation 15: For BM-Case1, training the model with random Set B is possible to provide beam prediction performance close to the optimal case – training and testing on the same Set B.

- Proposal 9: RAN1 further investigates the ML model generalization capabilities of BM Case-1 with configurations that can be either included or not included in the training dataset. The following list of configurations can be prioritized:

- gNB antenna array dimensions, e.g., 4x8 and 8x16.

- UE antenna array dimensions/Number of Panels.

- Set A dimension, e.g., 64 beams and 128 beams.

- Set B dimension, e.g., 16 beams and 32 beams.

- Proposal 10: RAN1 further investigates the ML model generalization capabilities of BM Case-1 with scenarios that can be either included or not included in the training dataset. The following list of scenarios can be prioritized:

- Channel propagation models, e.g. UMa/UMi.

- Outdoor/Indoor UE distribution (e.g. 100% Oudoor, 80% Outdoor/20% Indoor).

Observation 16:	Selecting the beam based on the QoS based model output can improve the throughput performance of each UE by clustering the UEs to a single beam.

Observation 17:	Even with errors in the input RSRP values of beams which are not in set B, QoS based beam selection can improve the throughput performance of each UE.

Proposal 11:	For BM-Case1, RAN1 further investigate QoS-based beam prediction with predicted RSRPs of the beams in set A.

Proposal 12:	RAN1 further investigates the trajectory model for BM-Case#2, adopting Option #4 as a starting point for further studies.

Proposal 13:	For BM-Case2, support Method 3 as the main scenario while other methods can be further studied :

- Method 1:	Set B is a fixed subset of Set A

- Method 2:	Set B is a variable subset of Set A

- Method 3:	Set B is the same as Set A

- Methods 1 or 3 + Assistance Info:	ML model input consists of L1-RSRP measurement based on Set B and assistance information

Observation 18:	For BM-Case2, the ML model using as input only RSRPs has performance that decreases when Set B is a subset of Set A and if no advanced algorithm is applied for beam selection in Set B.

Observation 19:	For BM-Case2, the ML model using as input only RSRPs has performance that decreases when increasing the length of the prediction window.

Observation 20:	For BM-Case2, additional algorithm (i.e. Bayesian Optimization) should be applied for choosing the beam measurements in Set B for the scenario of Set B is a subset of Set A.

Proposal 14:	For BM-Case2, with Set B is a subset of Set A, measurement instances K and prediction instances F shall be carefully investigated prior supporting the sub-use case.

Observation 21:	For the UE speed of 30 Km/h and prediction windows of 40 and 80 ms, the ML model using as input RSRP and assistance info (UE position) does not provide significant gains to the ML model using as input only RSRP.

Proposal 15:	For BM-Case2, RAN1 further verify whether there is any use of using assistance information at the input of the ML model. The following assistance information can be further considered:

•	the UE position information.

Proposal 16:	Support RAN1 to further study scenarios/ trajectory model for the BM-Case2.

# References

- RP-2205641, “Feature lead summary #4 evaluation of AI/ML for beam management” 3GPP TSG RAN WG1 #109-e.

- TR 37.910 – B.4.1_eMBB_SE – Spectral Efficiency - 02 Dense Urban-eMBB.

- Li, Lihong, et al. "A contextual-bandit approach to personalized news article recommendation." Proceedings of the 19th international conference on World wide web. 2010.

- Noh, Song, Michael D. Zoltowski, and David J. Love. "Multi-resolution codebook and adaptive beamforming sequence design for millimeter wave beam alignment." IEEE Transactions on Wireless Communications 16.9 (2017): 5689-5701.

- R1-2209370	Other aspects on ML for beam management, Nokia, Nokia Shanghai Bell

# Annex I

Agreement

- For dataset construction and performance evaluation (if applicable) for the AI/ML in beam management, system level simulation approach is adopted as baseline

  - Link level simulation is optionally adopted

Agreement

- At least for temporal beam prediction, companies report the one of spatial consistency procedures:

  - Procedure A in TR38.901

  - Procedure B in TR38.901

Agreement

- At least for temporal beam prediction, Dense Urban (macro-layer only, TR 38.913) is the basic scenario for dataset generation and performance evaluation.

  - Other scenarios are not precluded.

- For spatial-domain beam prediction, Dense Urban (macro-layer only, TR 38.913) is the basic scenario for dataset generation and performance evaluation.

  - Other scenarios are not precluded.

Agreement

- At least for spatial-domain beam prediction in initial phase of the evaluation, UE trajectory model is not necessarily to be defined.

Agreement

- At least for temporal beam prediction in initial phase of the evaluation, UE trajectory model is defined. FFS on the details.

Agreement

- UE rotation speed is reported by companies.

  - Note: UE rotation speed = 0, i.e., no UE rotation, is not precluded.

Agreement

- For AI/ML in beam management evaluation, RAN1 does not attempt to define any common AI/ML model as a baseline.

Conclusion

- Further study AI/ML model generalization in beam management evaluating the inference performance of beam prediction under multiple different scenarios/configurations.

  - FFS on different scenarios/configurations

  - Companies report the training approach, at least including the dataset assumption for training

Agreement

- For evaluation of AI/ML in BM, the KPI may include the model complexity and computational complexity.

  - FFS: the details of model complexity and computational complexity

Agreement

- For spatial-domain beam prediction, further study the following options as baseline performance

  - Option 1: Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping)

    - FFS CSI-RS/SSB as the RS resources

  - Option 2: Select the best beam within Set A of beams based on the measurement of RS resources from Set B of beams

    - FFS: Set B is a subset of Set A and/or Set A consists of narrow beams and Set B consists of wide beams

    - FFS: how conventional scheme to obtain performance KPIs

    - FFS: how to determine the subset of RS resources is reported by companies

  - Other options are not precluded.

Agreement

- For dataset generation and performance evaluation for AI/ML in beam management, take the parameters (if applicable) in Table 1.2-1b for Dense Urban scenario for SLS

Table 1.2-1b Assumptions for Dense Urban scenario for AI/ML in beam management

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz - SCS: 120 kHz |
| Deployment | 200m ISD, - 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) Other deployment assumption is not precluded |
| Channel mode | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | - For spatial domain beam prediction, 3km/h - For time domain beam prediction: 30km/h (baseline), 60km/h (optional) - Other values are not precluded |
| UE distribution | - FFS UEs per sector/cell for evaluation. More UEs per sector/cell for data generation is not precluded.For spatial domain beam prediction: FFS: - Option 1: 80% indoor ,20% outdoor as in TR 38.901 - Option 2: 100% outdoor - For time domain prediction: 100% outdoor |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration |  [One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline]  [Four panels: (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2), (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ as optional]  Other assumptions are not precluded. Companies to explain TXRU weights mapping. Companies to explain beam selection. Companies to explain number of BS beams |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | [Panel structure: (M,N,P) = (1,4,2)]  2 panels (left, right) with (Mg, Ng) = (1, 2) as baseline  Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Companies to explain number of UE beams |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| Beam correspondence | Companies to explain beam correspondence assumptions (in accordance to the two types agreed in RAN4) |
| Link adaptation | Based on CSI-RS |
| Traffic Model | FFS: - Option 1: Full buffer - Option 2: FTP model Other options are not precluded |
| Inter-panel calibration for UE | Ideal, non-ideal following 38.802 (optional) – Explain any errors |
| Control and RS overhead | Companies report details of the assumptions |
| Control channel decoding | Ideal or Non-ideal (Companies explain how it is modelled) |
| UE receiver type | MMSE-IRC as the baseline, other advanced receiver is not precluded |
| BF scheme | Companies explain what scheme is used |
| Transmission scheme | Multi-antenna port transmission schemes Note: Companies explain details of the using transmission scheme. |
| Other simulation assumptions | Companies to explain serving TRP selection Companies to explain scheduling algorithm |
| Other potential impairments | Not modelled (assumed ideal). If impairments are included, companies will report the details of the assumed impairments |
| BS Tx Power | [40 dBm] |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200m |
| BS Antenna height | 25m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |

Agreement

- For temporal beam prediction, the following options can be considered as a starting point for UE trajectory model for further study. Companies report further changes or modifications based on the following options for UE trajectory model. Other options are not precluded.

  - Option #2: Linear trajectory model with random direction change.

    - UE moving trajectory: UE will move straightly along the selected direction to the end of an time interval, where the length of the time interval is provided by using an exponential distribution with average interval length, e.g., 5s, with granularity of 100 ms.

      - UE moving direction change: At the end of the time interval, UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°].

      - UE move straightly within the time interval with the fixed speed.

    - FFS on UE orientation

  - Option #3: Linear trajectory model with random and smooth direction change.

    - UE moving trajectory: UE will change the moving direction by multiple steps within an time internal, where the length of the time interval is provided by using an exponential distribution with average interval length, e.g., 5s, with granularity of 100 ms.

      - UE moving direction change: At the end of the time interval, UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°].

      - The time interval is further broken into N sub-intervals, e.g. 100ms per sub-interval, and at the end of each sub-interval, UE change the direction by the angle of A_diff/N.

      - UE move straightly within the time sub-interval with the fixed speed.

    - FFS on UE orientation

  - Option #4: Random direction straight-line trajectories.

- Initial UE location, moving direction and speed: UE is randomly dropped in a cell, and an initial moving direction is randomly selected, with a fixed speed.

  - The initial UE location should be randomly drop within the following blue area

where d1 is the minimum distance that UE should be away from the BS.

    - Each sector is a cell and that the cell association is geometry based.

    - During the simulation, inter-cell handover or switching should be disabled.

For training data generation

- For each UE moving trajectory: the total length of the UE trajectory can be set as T second if it is in time, of set as D meter if it is in distance.

  - The value of T (or D) can be further discussed

  - The trajectory sampling interval granularity depends on UE speed and it can be further discussed.

- UE can move straightly along the entire trajectory, or

- UE can move straightly during the time interval, where the time interval is provided by using an exponential distribution with average interval length ΔT

  - UE may change the moving direction at the end of the time interval. UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°]

- If the UE trajectory hit the cell boundary (the red line), the trajectory should be terminated.

  - If the trajectory length (in time) is less than the length of observation window + prediction window, the trajectory should be discarded.

  - At the current stage, the length of observation window + prediction window is not fixed and the companies can report their values.

    - FFS on UE orientation

- Generalization issue is FFS

Agreement

- For temporal beam prediction, further study the following options as baseline performance

  - Option 1a: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources or all possible beams from Set A of beams at the time instants within T2

  - Option 2: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources from Set B of beams at the time instants within T1

    - Companies explain the detail on how to select the best beam for T2 from Set A based on the measurements in T1

  - Where T2 is the time duration for the best beam selection, and T1 is a time duration to obtain the measurements of all the RS resource from Set B of beams.

    - T1 and T2 are aligned with those for AI/ML based methods

  - Whether Set A and Set B are the same or different depend on the sub-use case

  - Other options are not precluded.

Agreement

- For dataset generation and performance evaluation for AI/ML in beam management, take the following assumption for LLS as optional methodology

| Parameter | Value |
| --- | --- |
| Frequency | 30GHz. |
| Subcarrier spacing | 120kHz |
| Data allocation | [8 RBs] as baseline, companies can report larger number of RBs First 2 OFDM symbols for PDCCH, and following 12 OFDM symbols for data channel |
| PDCCH decoding | Ideal or Non-ideal (Companies explain how is oppler) |
| Channel model | FFS: LOS channel: CDL-D extension, DS = 100ns NLOS channel: CDL-A/B/C extension, DS = 100ns Companies explains details of extension methodology considering spatial consistency Other channel models are not precluded. |
| BS antenna configurations | - One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline - Other assumptions are not precluded. Companies to explain TXRU weights mapping. Companies to explain beam selection. Companies to explain number of BS beams |
| BS antenna element radiation pattern | Same as SLS |
| BS antenna height and antenna array downtile angle | 25m, 110° |
| UE antenna configurations | Panel structure: (M, N, P) = (1, 4, 2), - 2 panels (left, right) with (Mg, Ng) = (1, 2) as baseline - 1 panel as optional - Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Companies to explain number of UE beams |
| UE antenna element radiation pattern | Same as SLS |
| UE moving speed | Same as SLS |
| Raw data collection format | Depends on sub-use case and companies’ choice. |

Agreement

- For UE trajectory model, UE orientation can be independent from UE moving trajectory model. FFS on the details.

  - Other UE orientation model is not precluded.

Agreement

- Companies are encouraged to report the following aspects of AI/ML model in RAN 1 #110. FFS on whether some of aspects need be defined or reported.

  - Description of AI/ML model, e.g, NN architecture type

  - Model inputs/outputs (per sub-use case)

  - Training methodology, e.g.

    - Loss function/optimization function

    - Training/ validity /testing dataset:

      - Dataset size, number of training/ validity /test samples

      - Model validity area: e.g., whether model is trained for single sector or multiple sectors

      - Details on Model monitoring and model update, if applicable

  - Others related aspects are not precluded

Agreement

- To evaluate the performance of AI/ML in beam management, further study the following KPI options:

  - Beam prediction accuracy related KPIs, may include the following options:

    - Average L1-RSRP difference of Top-1 predicted beam

    - Beam prediction accuracy (%) for Top-1 and/or Top-K beams, FFS the definition:

      - Option 1: The beam prediction accuracy (%) is the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams”

      - Option 2: The beam prediction accuracy (%) is the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”

    - CDF of L1-RSRP difference for Top-1 predicted beam

    - Beam prediction accuracy (%) with 1dB margin for Top-1 beam

      - The beam prediction accuracy (%) with 1dB margin is the percentage of the Top-1 predicted beam “whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam”

    - the definition of L1-RSRP difference of Top-1 predicted beam:

      - the difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam

    - Other beam prediction accuracy related KPIs are not precluded and can be reported by companies.

  - System performance related KPIs, may include the following options:

    - UE throughput: CDF of UE throughput, avg. and 5%ile UE throughput

    - RS overhead reduction at least for spatial-domain beam prediction at least for top-1 beam:

      - 1-N/M,

      - where N is the number of beams (with reference signal (SSB and/or CSI-RS)) required for measurement

      - where (FFS) M is the total number of beams

      - Note: Non-AI/ML approach based on the measurement of these M beams may be used as a baseline

      - FFS on whether to define a proper value for M for evaluation.

    - Other System performance related KPIs are not precluded and can be reported by companies.

o   Other KPIs are not precluded and can be reported by companies, for example:

  Reporting overhead reduction: (FFS) The number of UCI report and UCI payload size, for temporal /spatial prediction

  Latency reduction:

  (FFS) (1 – [Total transmission time of N beams] / [Total transmission time of M beams])

       where N is the number of beams (with reference signal (SSB and/or CSI-RS)) in the input beam set required for measurement

       where M is the total number of beams

  Power consumption reduction: FFS on details

# Annex II

For AI/ML-based beam management, In the RAN1 #109-e meeting the agreement [1] was made to support characterization and baseline performance evaluations of two sub use cases:

Agreement For AI/ML-based beam management, support BM-Case1 and BM-Case2 for characterization and baseline performance evaluations - BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams - BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams - FFS: details of BM-Case1 and BM-Case2 - FFS: other sub use cases Note: For BM-Case1 and BM-Case2, Beams in Set A and Set B can be in the same Frequency Range

For the sub use case BM-Case1, RAN1 concluded to study the following alternatives:​

Conclusion: For the sub use case BM-Case1, consider the following alternatives for further study: - Alt.1: Set B is a subset of Set A o FFS: the number of beams in Set A and B o FFS: how to determine Set B out of the beams in Set A (e.g., fixed pattern, random pattern, …) - Alt.2: Set A and Set B are different (e.g. Set A consists of narrow beams and Set B consists of wide beams) o FFS: the number of beams in Set A and B o FFS: QCL relation between beams in Set A and beams in Set B o FFS: construction of Set B (e.g., regular pre-defined codebook, codebook other than regular pre-defined one) - Note1: Set A is for DL beam prediction and Set B is for DL beam measurement. - Note2: The narrow and wide beam terminology is for SI discussion only and have no specification impact - Note3: The codebook constructions of Set A and Set B can be clarified by the companies.

Moreover, regarding the sub use case BM-Case1, RAN1 concluded to study few alternatives that can be considered as AI/ML input.

Conclusion Regarding the sub use case BM-Case1, further study the following alternatives for AI/ML input: - Alt.1: Only L1-RSRP measurement based on Set B - Alt.2: L1-RSRP measurement based on Set B and assistance information - FFS: Assistance information. The following were mentioned by companions in the discussion: Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.), expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction), UE position information, UE direction information, Tx beam usage information, UE orientation information, etc. - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side. - Alt.3: CIR based on Set B - Alt.4: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID - Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives - Note2: All the inputs are “nominal” and only for discussion purpose.

Similarly, for the sub use case BM-Case2, RAN1 concluded to study the following alternatives for Set A and Set B relationship:​

Conclusion For the sub use case BM-Case2, further study the following alternatives with potential down-selection: - Alt.1: Set A and Set B are different (e.g. Set A consists of narrow beams and Set B consists of wide beams) - FFS: QCL relation between beams in Set A and beams in Set B - Alt.2: Set B is a subset of Set A (Set A and Set B are not the same) - FFS: how to determine Set B out of the beams in Set A (e.g., fixed pattern, random pattern, …) - Alt.3: Set A and Set B are the same - Note1: Predicted beam(s) are selected from Set A and measured beams used as input are selected from Set B. - Note2: It is up to companies to provide other alternative(s) - Note3: The narrow and wide beam terminology is for SI discussion only and have no specification impact

Moreover, regarding the sub use case BM-Case2, RAN1 concluded to study the following alternatives as AI/ML input:

Conclusion Regarding the sub use case BM-Case2, further study the following alternatives of measurement results for AI/ML input (for each past measurement instance): - Alt.1: Only L1-RSRP measurement based on Set B - Alt 2: L1-RSRP measurement based on Set B and assistance information - FFS: Assistance information. The following were mentioned by companies in the discussion:, Tx and/or Rx beam angle, position information, UE direction information, positioning-related measurement (such as Multi-RTT), expected Tx and/or Rx beam/occasion for the prediction (e.g., expected Tx and/or Rx beam angle for the prediction, expected occasions of the prediction), Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam pointing angles beam boresight directions (azimuth and elevation), 3dB beamwidth, etc.) , increase ratio of L1-RSRP for best N beams, UE orientation information - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side. - Alt.3: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID - Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives - Note2: All the inputs are “nominal” and only for discussion purpose.