3GPP TSG RAN WG1 #110bis-e       	          			          R1-2209389

e-Meeting, October 10th – 19th, 2022

Source:               TCL Communication

Title:                   Discussions on Common Aspects of AI/ML Framework

Agenda item:     9.2.1

Document for:   Discussion and Decision

- Introduction

The study item on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface has been established in RAN plenary #94e meeting for Rel.18 [1]. The three use cases to be studied are respectively the enhanced CSI feedback, the beam management, and the positioning. They are further discussed in the RAN1 110 meeting, in which various solutions and sub use cases are provided. The merits from AI/ML have begun to show in the three typical use cases. In the meantime, more sub use cases are proposed. The common aspects covering the three typical use cases are discussed. The following agreements have been achieved [2].

Agreement Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management - Data collection - Note: This also includes associated assistance information, if applicable. - Model training - [Model registration] - Model deployment - Note: Terminology is to be defined. This includes process of compiling a trained AI/ML model and packaging it into an executable format and delivering to a target device. - [Model configuration] - Model inference operation - Model selection, activation, deactivation, switching, and fallback operation - Note: some of them to be refined - Model monitoring - Model update - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training. - Model transfer - UE capability Note: Some aspects in the list may not have specification impact. Note: Aspects with square brackets are tentative and pending terminology definition. Note: More aspects may be added as study progresses. Agreement The following is an initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML - Performance - Intermediate KPIs - Link and system level performance - Generalization performance - Over-the-air Overhead - Overhead of assistance information - Overhead of data collection - Overhead of model delivery/transfer - Overhead of other AI/ML-related signaling - Inference complexity - Computational complexity of model inference: FLOPs - Computational complexity for pre- and post-processing - Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte) - Training complexity - LCM related complexity and storage overhead - FFS: specific aspects - FFS: Latency, e.g., Inference latency Note: Other aspects may be added in the future, e.g. training related KPIs Note: Use-case specific KPIs may be additionally considered for the given use-case. Working Assumption Terminology; Description. Online training; An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.). Offline training; An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions.. Note: It is encouraged for the 3gpp discussion to proceed without waiting for online/offline training terminologies. Working Assumption Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion. Terminology; Description. AI/ML model delivery; A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc.. Note: Companies are encouraged to bring discussions on various options and their views on how to define Level y/z boundary in the next RAN1 meeting.

From the last RAN1 meeting, some terminologies are agreed or to be further discussed, such as model registration, and collaboration levels. The study on the common aspects of AI/ML framework begin to shape up. In this contribution, we will provide our views on the common aspects of AI/ML framework.  Especially, the data collection and quantization, the model complexity, and the model life cycle management will also be discussed.

- Discussions

  - Functional Framework

A functional framework for RAN intelligence is proposed in RAN3, which is a starting point for RAN1 study. In our view, as the discussion of RAN1 goes on, some requirements are becoming clear, in order to match the characteristics of AI/ML for NR air interface, the related modifications of RAN3 functional framework are needed. Besides, to be future-proof, the new framework should be general to contain various features of these three use cases. Hence, it is better to study a new common functional framework of AI/ML for NR air-interface.

Proposal 1: A new common functional framework of AI/ML for air-interface need to be studied.

Therefore, we provide a new common functional framework of AI/ML for air-interface in this contribution, it may include the functions such as, data collection, model training. model inference, model performance and actor, which are shown in Fig.1.

Fig.1. Common functional framework of AI/ML for NR air-interface

- Data collection

In data collection, the collected data can be divided into three categories for other AI/ML function to meet the different requirements. One is used for model training to produce trained AI/ML model, the other is used for model inference to produce a set of output, another is used for model monitoring to evaluate the output of inference. Besides, the collected data can be real data or synthetic data.

- Model training

For the model training block, it is a whole process for an AI/ML model generation. The block of model training may include the model training, validation, test, and finally produce a trained model.

- Model inference

The model inference block is responsible for producing the inference output of AI/ML models. Regarding the data from data collection as input, using the trained AI/ML model given by model training function to provide a set of inference output. According to output of model monitoring, it can trigger a set of AI/ML model actions and also can be an input for model monitoring function.

- Model monitoring

Model monitoring is used to evaluate the inference performance of model inference. Due to the uncertainty of the collected data, it had better monitor whether the trained AI/ML model works properly. This task is completed by model monitoring. The output of model monitoring will trigger a set of actions.

- Actor

The actor receives the output from model inference or model monitoring. It performs the corresponding actions, such as model updating, model activation, model deactivation fallback and so on. Meanwhile, it will send the feedback to data collection. For an example, according the output of model monitoring, the actor triggers the action of model updating.

Proposal 2: A common functional framework of AI/ML over air-interface may include the following functions: data collection, model training. model inference, model monitoring and actor.

  - Collaboration Levels

During last meeting, it takes a lot of efforts to discuss the collaboration levels between UE and gNB, while there are still some remaining issues as followings, which are needed to clarify:

- FFS: The sub-levels of collaboration can be further discussed after sufficient progress in LCM is made. - FFS: Clarification is needed for Level x-y boundary - FFS: Clarification is needed for Level y-z boundary (i.e., what constitutes model transfer).

According to the agreement in RAN1#109e, the current collaboration levels are categorized based on the aspect of model transfer. In our understanding, once the model training and model inference are located in the different nodes, the model transfer will occur. So, the model transfer is related to two functions, i.e., model training function and model inference function, which are the components of LCM.

The definition of one-sided model or two-sided models depends on the location of deployed model. In other words, the essential difference of one-sided model or two-sided models is based on the place of model deployment.

Observation 1: The essential difference of one-sided model or two-sided models is on the location of ML model deployment.

For level x-y boundary in FFS, Level x is regarded as an implementation-based AI/ML operation, which means it is transparent between UE and network. There is no specification impacts for level x. While for Level y, there are some assistant information that is transferred between UE and network. More specification impacts are brought by level y.

Proposal 3: The level x-y boundary is whether the AI/ML operation is implementation-based without specification impacts.

For level y-z boundary in FFS, it spends a lot time to discuss it in the last meeting. In our views, the current collaboration levels are categorized based on the aspect of model transfer. Both of them have specification impacts in the collaboration, Level y only needs some assistance information for AI/ML approach, which may occur before or after model transfer, such as the signaling of data collection, the signaling carried feedback. It is worth noting that for proprietary AI/ML models, there is no specification impacts for model transfer since the model is transferred on its privacy way. While for Level z, it involves the information related AI/ML model during AI/ML model transfer, such as the parameters and structure of AI/ML model.

Proposal 4: The level y-z boundary is whether specification impacts on model transfer exist.

  - Quantization

For an ML model, ground truth data is needed either for model training or model monitoring. The ground truth data has to be collected. The collection of ground truth data can be completed either by a UE or a gNB.

- The data collection is completed by a UE. The ground truth data is processed, if needed, and reported to a gNB or a third node.

- The data collection is completed by a gNB. The ground truth data is processed, and reported to a third node, if needed.

Nevertheless, the ground truth is reported, after quantization. The quantization is an indispensable step before the delivery of the ground truth. At one hand, the quantization of ground truth should be with high precision, in order to faithfully reflect the truth without losing critical features. On the other hand, the reporting or the delivery of ground truth is resource consuming. It is an overhead to the involved UE or the gNB. Thus, the quantization should be properly designed and the quantization bits should be kept less. This problem needs to be discussed. In the RAN1 110 meeting, the CSI quantization method is encouraged to be reported, e.g., vector quantization, scalar quantization. Whereas the quantization for ground truth is not mentioned. Accordingly, the vector quantization and scalar quantization can be a starting pointing for the ground truth quantization.

Observation 2: The delivery of ground truth is an overhead to the involved UE or gNB. The quantization method should be designed with low quantization bits.

A straightforward solution is to extend the group reporting mechanism in the L1-RSRP report. A reference L1-RSRP value is selected and quantized directly. The other L1-RSRP values are quantized differentially. These L1-RSRPs together make a group and are reported at one shot. The group size is 4 at maximum. In this way, the quantization bits of the other L1-RSRP values are saved. This mechanism can be extended to the ground truth report of the three use cases currently under study. Furthermore, the buffering and reporting of grouped ground truth may need UE processing and assistance. Whether supporting the group reporting of ground truth can be a UE capability.

Proposal 5: The group reporting of ground truth is a feasible way to reduce the reporting overhead.

  - Model Complexity

The ML model complexity can be described by various factors, such as the FLOPs, the number of parameters of the ML model, the ML model size, the memory size etc. They are related to either ML attributes, or the UE hardware, even both.

An ML model can be very complex or less complex, since the AI/ML models are with diverse structure and parameter size. To describe an ML model complexity, several factors can be considered, such as FLOPs and model size. A rule is need to roughly classify the model complexities, so that UE or the gNB does not have to deal the low level parameters, such as the bus bandwidth. Deciding how the model with different complexities is supported either by a UE or a gNB becomes simple. The signallings between the gNB and UE is thereby simplified.

Proposal 6: To reduce the signaling overhead between the UE and the gNB, a rule is need to roughly classify the model complexity.

Besides, the complexity of an ML model is not fixed and unchanged. The ML model can be post-processed after model training, or before model deployment. That would decide various aspects of the model, such as the model size, the number of parameters, model performance, and finally lead to complexity change. Thus the model complexity is related to post-processing. In a way, the post-processing can reshape the ML model. The complexity of the ML model can be controlled within the supporting range of the UE capability by post-processing.

Observation 3: The ML complexity can be decided by post-processing.

One problem about the post-processing of an ML model is that an oversimplified model may be obtained from the post-processing, such that the ML model performance is excessively degraded. The rise of this problem is due to that the post-processing is tackling the balance between the model complexity and ML model performance. If one end is emphasized too much without considering the other, problem occurs. Some constraints have to be added on the post-processing. As an example, some ML models is not allowed to be post-processed after ML training.

Proposal 7: Some constraints shall be added on the post-processing, in order to avoid obtaining an oversimplified low-performance model from post-processing.

  - Model Life Cycle Management

The model life cycle management covers model training, inference and model monitoring. There are several stages and operations involved.

- Model training: An ML model is generated from training, which can be taken in an online or offline manner.

- Preprocessing and post-processing: There are pre-processing and post-processing of an ML model. The role of pre-processing is feature engineering, which can ease the training of an ML model. The post-processing can decide the complexity of an ML model. It usually strikes and balance between complexity and ML model performance.

- Model deployment: The ML model can be deployed after post-processing or fine tuning.

- Model inference: After the deployment, the ML model is activated. The inference stage begins.

- Model monitoring: During the inference stage, the ML model should be monitored to check whether it works properly.

If the ML model malfunctions during model monitoring, the ML model should be deactivated and re-trained. The ML model can be replaced by a backup ML model. If there is more than one-back up model, the UE or gNB can select a model randomly. Or they can select a backup model according to the preference of the UE or the gNB. Furthermore, if there is no such backup ML model, the node has to fall back to the non-ML working way.

Proposal 8: At the inference stage, the ML model has to be monitored. If the ML model does not work properly, it can be replaced by a backup ML model or fall back to the non-ML working way.

Proposal 9: During model switching, a backup model can be randomly selected, or according to the preference of UE or gNB.

The following features can be selected for further study.

- Real-time AI, non real-time AI. Since RAN1 works with a much small time scale than RAN3. The real time or non real time would lead very different requirements on UE capability and constraints of each stage in LCM. Thus, they can have different LCM methods.

- Online/offline operations. The online training and offline training has been discussed a lot in the RAN1 110 meeting. Obviously, the offline training does not need frequent interactions between UE and gNB. That means less assistant information and a simple LCM diagram than the online training.

- Model type. In the last RAN1 meeting, the LCM according to the ML model type are discussed. However, due to no consensus on the ML model boundary between level y and level z, the LCM based on them has no progress either. The LCM methods can be different for different ML model types.

The relations between these factors and each stage of ML LCM need to be studied.

Proposal 10: The LCM need to consider at least one of the following factors in RAN1,

- Real-time/non real-time;

- Online/offline operations;

- Model type.

- Conclusions

In this contribution the following proposals been proposed:

Proposal 1: A new common functional framework of AI/ML for air-interface need to be studied.

Proposal 2: A common functional framework of AI/ML over air-interface may include the following functions: data collection, model training. model inference, model monitoring and actor.

Observation 1: The essential difference of one-sided model or two-sided models is on the location of ML model deployment.

Proposal 3: The level x-y boundary is whether the AI/ML operation is implementation-based without specification impacts.

Proposal 4: The level y-z boundary is whether specification impacts on model transfer exist.

Observation 2: The delivery of ground truth is an overhead to the involved UE or gNB. The quantization method should be designed with low quantization bits.

Proposal 5: The group reporting of ground truth is a feasible way to reduce the reporting overhead.

Proposal 6: To reduce the signaling overhead between the UE and the gNB, a rule is need to roughly classify the model complexity.

Observation 3: The ML complexity can be decided by post-processing.

Proposal 7: Some constraints shall be added on the post-processing, in order to avoid obtaining an oversimplified low-performance model from post-processing.

Proposal 8: At the inference stage, the ML model has to be monitored. If the ML model does not work properly, it can be replaced by a backup ML model or fall back to the non-ML working way.

Proposal 9: During model switching, a backup model can be randomly selected, or according to the preference of UE or gNB.

Proposal 10: The LCM need to consider at least one of the following factors in RAN1,

- Real-time/non real-time;

- Online/offline operations;

- Model type.

References

- RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #94e, Dec. 6 - 17, 2021.

- Chairman notes of 3GPP TSG RAN WG1 #110, 3GPP RAN1 Meeting #110, Toulouse, France, August 22 – 26, 2022