3GPP TSG RAN WG1 #110bis-e       	          	R1-2209390

e-Meeting, October 10th – 19th, 2022

Source:                TCL Communication

Title:                    Discussions on Sub-Use Cases in AI/ML for CSI Feedback Enhancement

Agenda item:      9.2.2.2

Document for:    Discussion and Decision

- Introduction

The study item on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface is established in RAN plenary meeting #94e for Rel.18 [1] to investigate the benefits of AI/ML in air interface performance enhancement. The AI/ML will be an icon technology in 5G advanced and afterwards.

According to current 5G Standard, the UE is continuously burdened by a large amount of data/bits for CSI reporting to the gNB; especially, after the introduction of the codebook type II and the growth of the antenna ports to up to 32. That all leads to an increased amount of feedback data. Furthermore, the CSI feedback should be continuously sent to the gNB, e.g., for a MU-MIMO case, the feedback is very critical to the (BER) performance. The more delicate and in-time the feedback is, the better the scheduling will be. In the meantime, in AI/ML, the auto-encoder model is a good fit for overcoming the CSI feedback problem. The auto-encoder model is capable of compressing an input vector by the encoder, and recovering the vector at decoder. The compressed information can be delivered via the air interface. The channel measured by the CSI is sparse. The channel is sparse in a certain dimension under certain description, e.g., the tapped delay line model is sparse in time. Therefore, the measured channel is compressible. The auto-encoder model is able to compress the channel from a higher dimension to a lower dimension, then decode the output. According to this, the gain of AI/ML based CSI feedback over the conventional codebook type I, codebook type II and even enhanced codebook type II can be seen. Such a gain is coming from the fact that the conventional codebook type I, codebook type II and enhanced codebook type II are general designs which are applicable to a number of scenarios; whereas, an auto-encoder model is an ML model is a scenario-specific. In an auto-encoder model, there can be some generalization opportunity. On the other hand, an auto-encoder model can be bounden to specific application scenarios. A scenario-specific model with a high dimension description can compress the measured channel better than a general design.

In RAN1 110 meeting, the sub use cases and potential standard impacts are discussed. the following agreements and conclusions are achieved [2]. The discussion covers various aspects, covering training, and

## Agreement In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied: - Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided. - Type 2: Joint training of the two-sided model at network side and UE side, respectively. - Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively. - Note: Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation. Joint training could be done both at single node or across multiple nodes (e.g., through gradient exchange between nodes). - Note: Separate training includes sequential training starting with UE side training, or sequential training starting with NW side training [, or parallel training] at UE and NW - Other collaboration types are not excluded. ## Conclusion CSI-RS configuration and overhead reduction is NOT selected as one representative sub-use case for CSI feedback enhancement use case. ## Conclusion Resource allocation and scheduling is NOT selected as one representative sub-use case for CSI feedback enhancement use case. Agreement In CSI compression using two-sided model use case, further study potential specification impact on CSI report, including at least - CSI generation model output and/or CSI reconstruction model input, including configuration(size/format) and/or potential post/pre-processing of CSI generation model output/CSI reconstruction model input. - CQI determination - RI determination ## Agreement In CSI compression using two-sided model use case, further study potential specification impact on output CSI, including at least - Model output type/dimension/configuration and potential post processing ## Agreement In CSI compression using two-sided model use case, further discuss at least the following aspects, including their necessity/feasibility/potential specification impact, for data collection for AI/ML model training/inference/update/monitoring: - Assistance signaling for UE’s data collection - Assistance signaling for gNB’s data collection - Delivery of the datasets.

The CSI feedback sub use cases and possible standard impacts will be further discussed in this contribution.  We will provide possible sub use cases in spatial domain, time domain, and frequency domain. The potential standard impacts will be discussed for each sub use case.

- CSI Feedback Compression in Spatial Domain

In this section, we will discuss the possible sub use cases of CSI feedback based on auto-encoder model. They are classified according to the input time range and the output time range as below.

The basic auto-encoder based CSI feedback model is shown in Fig 1. There are two parts. The encoder resides at the UE side. And the decoder resides at the gNB side.

Fig 1. An auto-encoder model.

The CSI feedback based on codebook is a process of compression and decompression. The auto-encoder model is able to exactly compress a vector into lower dimension and then recover it. The basic auto-encoder model is a replacement of the CSI feedback based on codebook at the UE side and at the gNB side. The compression and decompression is similar to the Rel.15 codebook (codebook type I) design. The CSI-RSs here are projected to a certain lower dimension similar to the calculation of PMI, which is for beamforming, and obviously spatial compression.

Observation 1: The CSI feedback based on codebook is a process of compression and decompression. The auto-encoder model is able to exactly compress a vector into lower dimension and then recover it.

The basic CSI feedback model based on auto-encoder is further explained according to Fig. 2.

At encoder side,

- The input is the CSI measurement of time instance t.

- The output of the encoder at UE side is the compressed CSI measurement of time instance t. This output is reported to gNB at time tp1.

At decoder side,

- The input is the compressed CSI measurement of time t, received via the air-interface.

- The output at the decoder side is decompressed. The output is obtained at time tp2.

Finally, the output of the decoder is applied to scheduling at time ts, for example, scheduling according to the calculated CQI from the decoder output.

Fig. 2 The compression and decompression of the CSI measurement of time t.

The merits of the basic CSI feedback model based on auto-encoder compared to the CSI feedback based on codebook is saving some feedback bits through the air-interface. Whereas, the scheduling delay and processing delay are still there and left unhandled. Due to them, the scheduling policy at time ts is determined according to an outdated measurement at time t.

Proposal 1: The basic CSI feedback model based on auto-encoder reduces feedback bits through the air-interface, compared to the CSI feedback based on codebook. It is a functional replacement of the CSI feedback based on codebook.

- CSI Feedback Compression in Time Domain

  - Predictive Single CSI Feedback

To make the CSI feedback in-time and truly reflect the channel conditions at the scheduling time, the CSI decoded at the gNB side can be set as the predicted values at an intended future time. Thereby, the predictive CSI feedback model based on auto-encoder can be obtained. This procedure is described as given in Fig. 3.

At encoder side,

- The input is the CSI measurements, e.g., ranging from time t-n to time t, where n is a positive integer.

- The output at the encoder side is the compressed output. The output is reported to gNB at tp1.

At decoder side,

- The input is the compressed CSI measurements received via the air-interface.

- The output at the encoder side is the predicted CSI at time ts.

Fig. 3 The compression and decompression of the CSI measurements with prediction.

This model utilizes the channel correlations to predict the channel conditions in a future time. The historical channel measurements are processed to generate the compressed feedback. The merits of this model is the reduced CSI feedback bits and predicted future CSI. Considering the UE should process multiple CSIs, the model complexity is higher than the basic CSI feedback model and the requirement on UE processing ability grows higher. Therefore, multiple historical CSI measurements are needed as input to the ML model to track the channel fluctuations and make the prediction.

Proposal 2: To fix the problem of outdated CSI feedback, the predictive CSI feedback model is necessary to predict CSI at the scheduling time.

  - Predictive Multi-CSI Feedback

Since the channel is correlated in time, the measured CSIs are correlated in time too. Thus, the feedback CSIs can be further compressed in time domain. Multiple CSIs across a time range can be feedback to gNB at one report occasion. A schematic figure is shown in Fig. 4.

At encoder side,

- The input is the CSI measurements, e.g., ranging from time t-n to time t, where n is a positive integer.

- The output at the encoder side is the compressed output. The output is reported to gNB at tp1 at one report occasion.

At decoder side,

- The input is the compressed CSI measurements, received via the air-interface.

- The output at the encoder side are the predicted CSIs from time ts to time ts+m, where m is a positive integer.

Fig. 4 The feedback multi-CSI measurements in time domain.

The CSIs are further compressed along the time domain. Since the CSIs are correlated in time, the redundancy among them can be compressed. At one CSI report occasion, the compressed multiple CSIs are feedback to gNB. The model complexity is increased and the requirement on UE processing ability on the CSI compression grows higher.

Proposal 3: Multiple CSI measurements can be compressed together and feedback at one shot to further reduces the feedback overhead.

- CSI Feedback Compression in Frequency Domain

  - Sub-band CSI Feedback

In the current CSI feedback method based on codebook, the wideband PMI is generated at first. Subsequently, the PMI for each the sub band is calculated according to the generated wideband PMI. The Codebook Type II is enhanced in Rel.16, by compressing the CSI report in the frequency domain to reduce the CSI feedback overhead. The compression is performed across sub-bands.

Whereas for enhanced CSI feedback, in some sub bands the CSI-RS may not be configured. The ML models is able to interpolate across the sub bands when the CSI is recovered. As shown in Fig. 5, the CSI-RS is not configured in some sub bands.

Fig. 5 Channel prediction across sub bands

The merit of this method is that the input vector size of the ML model for CSI feedback at the UE side is reduced. At the gNB side, the measurements of all the sub-bands with and without CSI-RS REs are recovered by an ML model. In the meantime, the resource utilization with sparser CSI-RS REs is improved by allocating more REs to data transmission.

Proposal 4: The CSI feedback compression along the frequency dimension can be designed with new CSI-RS configurations across sub bands. The resource utilization is improved by allocating more REs to data transmission.

- Possible Standard Impacts

The standard impacts mainly come from the following aspects, besides the model life cycle management, model training and mode deployments.

- The CSI-RS configuration across sub bands

- The reporting configuration of encoder output

- The pre-processing and post-processing of the ML model

- The reporting of the ground truth

- Model training

They need to be further discussed for enhanced CSI feedback.

Besides, the quantization of the training data need to be studied during ML model training. The quantization of ground truth is discussed in one of our parallel contribution [?]. The quantization methods of the forward propagation data and back propagation data need to be decided, especially for joint training. For a UE or a gNB, the forward propagation data and back propagation data are nevertheless an overhead. The number of quantization bits should be kept low when they are delivered. The ML model training may absorb the quantization error of forward propagation, which means the forward propagation data can be coarsely quantized. Whereas, the back propagation data is suggested to be quantized with high precision, otherwise, the ML model may not converge smoothly.

Proposal 5: The standard impacts, especially the quantization during model training, need to be studied.

- Conclusion

In this contribution the following proposals been made:

Observation 1: The CSI feedback based on codebook is a process of compression and decompression. The auto-encoder model is able to exactly compress a vector into lower dimension and then recover it.

Proposal 1: The basic CSI feedback model based on auto-encoder reduces feedback bits through the air-interface, compared to the CSI feedback based on codebook. It is a functional replacement of the CSI feedback based on codebook.

Proposal 2: To fix the problem of outdated CSI feedback, the predictive CSI feedback model is necessary to predict CSI at the scheduling time.

Proposal 3: Multiple CSI measurements can be compressed together and feedback at one shot to further reduces the feedback overhead.

Proposal 4: The CSI feedback compression along the frequency dimension can be designed with new CSI-RS configurations across sub bands. The resource utilization is improved by allocating more REs to data transmission.

Proposal 5: The standard impacts, especially the quantization during model training, need to be studied.

References

- RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #94e, Dec. 6 - 17, 2021.

- Chairman notes of 3GPP TSG RAN WG1 #110, 3GPP RAN1 Meeting #110, Toulouse, France, August 22 – 26, 2022