3GPP TSG RAN WG1 #110bis-e       	          	R1-2209391

e-Meeting, October 10th – 19th, 2022

Source:                TCL Communication

Title:                    Discussions on Sub-Use Cases in AI/ML for Beam Management

Agenda item:      9.2.3.2

Document for:    Discussion and Decision

- Introduction

Artificial Intelligence (AI)/Machine Learning (ML) has the potential to be a landmark technology in 5G advanced. The investigation of the gains of AI/ML in enhancing RAN performance have been started since last year during RAN plenary meetings #92e and #93e. Based on the investigation, a study item on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface is established in RAN plenary #94e meeting for Rel.18 [1].

Beam management was selected as one use case for study in Rel.18. The previous discussion related to beams was mainly try to utilize the prediction ability of AI/ML on choosing a proper beam predictively during the beam switching produces. The beam switching may cause delays. For example, if a beam is selected improperly, additional beam switching will be triggered which may bring more delays and processing overhead. Thus, choosing a beam with less beam switching/handover will ease the system maintenance and improve user experience. Besides, the prediction in ML is a kind of time series analysis in which the UE traces along the time are sampled into a time sequence by measurements. Due to the fact that, the moving traces of a certain kind of UEs, e.g., vehicles, may satisfy such a certain prediction rule (i.e. the future data is correlated with historical data). Therefore, the prediction is highly possible for the beam management use case.

In the objective of the SI [1], the use case of beam management is to focus on,

Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1].

In the RAN1 110e meeting, the following agreements were achieved [2]. The discussions mainly focus on defining the input and output for the beam prediction in time domain and spatial domain, respectively.

Agreement For the sub use case BM-Case1, support the following alternatives for further study: - Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A) - Alt.2: Set B is a subset of Set A - Note1: Set A is for DL beam prediction and Set B is for DL beam measurement. - Note2: The beam patterns of Set A and Set B can be clarified by the companies. Agreement For the data collection for AI/ML model training (if supported), study the following aspects as a starting point for potential necessary specification impact: - Signaling/configuration/measurement/report for data collection, e.g., signaling aspects related to assistance information (if supported), Reference signals - Content/type of the collected data - Other aspect(s) is not precluded Agreement At least for the sub use case BM-Case1 and BM-Case2, support both Alt.1 and Alt.2 for the study of AI/ML model training: - Alt.1: AI/ML model training at NW side; - Alt.2: AI/ML model training at UE side. Note: Whether it is online or offline training is a separate discussion. Agreement For the sub use case BM-Case1 and BM-Case2, further study the following alternatives for the predicted beams: - Alt.1: DL Tx beam prediction - Alt.2: DL Rx beam prediction - Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam) - Note1: DL Rx beam prediction may or may not have spec impact Agreement For the sub use case BM-Case2, further study the following alternatives: - Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A) - Alt.2: Set B is a subset of Set A (Set A and Set B are not the same) - Alt.3: Set A and Set B are the same - Note1: The beam pattern of Set A and Set B can be clarified by the companies. Agreement Regarding the model monitoring for BM-Case1 and BM-Case2, to investigate specification impacts from the following aspects - Performance metric(s) - Benchmark/reference for the performance comparison - Signaling/configuration/measurement/report for model monitoring, e.g., signaling aspects related to assistance information (if supported), Reference signals - Other aspect(s) is not precluded Agreement In order to facilitate the AI/ML model inference, study the following aspects as a starting point: - Enhanced or new configurations/UE reporting/UE measurement, e.g., Enhanced or new beam measurement and/or beam reporting - Enhanced or new signaling for measurement configuration/triggering - Signaling of assistance information (if applicable) - Other aspect(s) is not precluded Agreement Regarding the sub use case BM-Case1 and BM-Case2, study the following alternatives for AI/ML output: - Alt.1: Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams - E.g., N predicted beams can be the top-N predicted beams - Alt.2: Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and other information - FFS: other information (e.g., probability for the beam to be the best beam, the associated confidence, beam application time/dwelling time, Predicted Beam failure) - E.g., N predicted beams can be the top-N predicted beams - Alt.3: Tx and/or Rx Beam angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams - E.g., N predicted beams can be the top-N predicted beams - FFS: details of Beam angle(s) - FFS: how to select the N DL Tx and/or Rx beams (e.g., L1-RSRP higher than a threshold, a sum probability of being the best beams higher than a threshold, RSRP corresponding to the expected Tx and/or Rx beam direction(s)) - Note1: It is up to companies to provide other alternative(s) - Note2: Beam ID is only used for discussion purpose - Note3: All the outputs are “nominal” and only for discussion purpose - Note4: Values of N is up to each company. - Note5: All of the outputs in the above alternatives may vary based on whether the AI/ML model inference is at UE side or gNB side. - Note 6: The Top-N beam IDs might have been derived via post-processing of the ML-model output

Mainly two sub use cases are summarized in beam management based on AI/ML. More details about them will be discussed. Besides, other sub use case will be provided and discussed.

- Discussions

  - Beam Prediction in Time Domain

For the fast moving UEs, a fast beam switching is necessary. The beam should be robust to reduce the probability of further beam switching. As UE moves, its measurements of the reference signals from different beams vary. The practice right now is to select the beam with the strongest signal strength. This is not the best policy for a moving UE. ML comes to help.

The beam prediction in time domain tries to utilized the historical information. The basic one is the historically connected beams for a UE regarding the UE traces and positions. The ML model input can be data pair of the index historically connected beam, the corresponding position.

The ML model can be deployed at the UE side, or the gNB side.

- The input is the historical information, e.g., the historically connected beams.

- The output is the beam to be connected at next future time unit.

Figure 1. An ML model for predictive beam switching.

An intuition is that the beam switching is bounded to the UE positions. The UE’s next future movement can be predicted, so is the best connected beam in the next future time unit. To some degree, the position information can reveal the user privacy. To avoid the privacy problem, the position information can be hidden, or not involved. The beam becomes narrow, as the frequency grows higher. The beam itself contains the spatial and direction information. A better choice for beam switching based on ML, is to use the historically connected beams as input, and output the beam to be connected at next future time unit, without a need for predicting the UE position.

Proposal 1: The UE position information is not necessary for predictive beam switching.

This case can be further discussed in inter-cell beam switching and intra-cell beam switching, in order to benefit beam switching and reduce latency. The intra-cell beam switching is relative simple, it can be completed between the UE and the severing gNB. Whereas, for inter-cell beam switching, the interaction between the serving gNB and the target gNB are needed.

Proposal 2: The predictive beam switching shall be discussed in sub use cases of inter-cell beam switching and intra-cell beam switching for latency reduction.

  - Beam Prediction in Spatial Domain

For beam training, after the connection is established, the UE and the gNB complete the beam pairing by P1, P2 and P3 according to TR 38.802. The UE has to take MN times measurements at maximum, to find a beam properly paired with a gNB beam, where M indicates the number of beams of the gNB and N is the number of beams of the UE. To save the signaling overhead and improve the resource utilization, two subsets with reduced number of beams can be maintained both at the gNB and at the UE. For gNB, the cardinality of the subset of beams is denoted by M1, M1<M. For UE, the cardinality of the sub set of beams is N1, N1<N.

The problem behind beam training is to find a mapping, from the current established beam (pair) to a new beam pair. This process is completed by beam training or machine learning (ML). The beam training has to measure all the beam pairs. Whereas, by ML, only a subset of the beam pairs are selected to be measured. It is a process of spatial sampling and interpolation.

There are two steps here.

- Based on the current beam pair get connected, to infer sub set of beam pairs to be measured.

- From the measurements of subset of beam pairs, infer the best beam pair.

The first step is neglected by most of the companies. The connected beam can provide more information about the UEs wireless environment, according to the 3dB beam width, the historical measurements etc.. This information can be used for deciding the subset of beam pairs to be measured. That can be classified as assistant information. The assistant information will be helpful in improving the performance of an ML model. However, the cost is higher ML model complexity.

Besides, how to decide the pattern of sparse beam measurements need to be discussed. The beam pattern goes one step further than deciding the relationship of beam set B and beam set A. The set B comprises sparse samplings in the spatial domain by RS measurements. The beam measurements alone will constitute a pattern. The problem on deciding pattern of set B is how to grasp the spatial features to a maximum degree with sparse samplings. Generally speaking, the samples near the connected beam will have higher correlations for low speed UEs. The samples far from the connected beams will have lower correlations for low speed UEs. Thus, the patterns of set B can be designed following the distribution of correlation in spatial domain.

Fig. 2 The beam measurements in spatial domain

A fixed regular pattern can be decided, for example, the pattern in Fig.3.  The four blue blocks denote the measured beams in spatial domain. The measurement pattern is fixed across time.

As an alternative, random pattern of set B can be an option for further discussion.

Fig.3 The pattern of beam measurements in set B.

Proposal 3: Some patterns can be designed for the input set B of beam prediction in spatial domain.

- A fixed pattern;

- A random pattern.

With these measurements from RSs and assist information as input, the ML model predicts the best beam pair.

Figure 3. ML assisted Beam Training. A few measurements of the gNB-UE beam pairs are the input for the ML model to infer the output. The outputs are the predicted best beam pair across all the measured and unmeasured possible beam pairs.

In this way, the measurements of the RSs in beam training are reduced. The measurement of the beam pairs are interpolated in spatial domain.

The subsets of beams are constructed according to the correlation between the established beam pair and the beams in the subset. The overhead for beam training is reduced with the decreased number of beam measurement.

Proposal 4: The subsets of beams at the gNB side and UE side, can be constructed with the assistance of an ML model to reduce the beam training overhead.

  - Beam Failure Recovery

The beam management based on ML is able to select a proper and robust beam, to reduce unnecessary beam switching, even to reduce the beam failure occurrences. However, the beam failure may happen. The beam failure recovery mechanism is necessary. The current beam failure detection is relying on a counter. If the detected hypothetical BLER is continuously larger than a threshold within a time duration, the beam failure is considered to occur and the beam failure recovery request will be triggered.

The counter based beam failure detection is usually not accurate and is time consuming. Thus, it is helpful to improve the beam failure recovery with ML. An ML model for beam failure detection is shown in Fig. 4. The model can be deployed either at the UE side, or at the gNB side.

An ML model for beam failure detection

- The input is historical measurements of the beams under beam failure detection.

- The output is the “0/1” indicating whether beam failure occurs with the beam.

Figure 4. The beam failure detection based on an ML model.

If the model is deployed at the UE side, one the standard impact comes from the ML model downloading. On the other hand, if the model is deployed at the gNB side, the foreseeable standard impacts are the measurement reporting and the new beam indication.

Proposal 5: The beam failure detection performance can be enhanced by an AI/ML model based on historical beam measurements.

- Conclusions

In this contribution the following proposals been proposed:

Proposal 1: The UE position information is not necessary for predictive beam switching.

Proposal 2: The predictive beam switching shall be discussed in sub use cases of inter-cell beam switching and intra-cell beam switching for latency reduction.

Proposal 3: Some patterns can be designed for the input set B of beam prediction in spatial domain.

- A fixed pattern;

- A random pattern.

Proposal 4: The subsets of beams at the gNB side and UE side, can be constructed with the assistance of an ML model to reduce the beam training overhead.

Proposal 5: The beam failure detection performance can be enhanced by an AI/ML model based on historical beam measurements.

References

- RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #94e, Dec. 6 - 17, 2021.

- Chairman notes of 3GPP TSG RAN WG1 #109-e, 3GPP RAN1 Meeting #109-e, e-Meeting, May 9th – 20th, 2022.