3GPP TSG RAN WG1 #110-bis-e	R1-2209434

e-Meeting, October 10th – 19th, 2022

Source:		KDDI Corporation

Title:		Views on enhanced SRS configuration for 8TX and TDD CJT

Agenda Item:	9.1.3.2	SRS enhancement targeting TDD CJT and 8 TX operation

Document for:	Discussion and Decision

- Introduction

In the RAN1 #110 meeting, the following items were agreed on in the " 9.1.3.2 SRS enhancement targeting TDD CJT and 8 TX operation" regarding SRS enhancements for 8TX and TDD CJT [1].

Agreement For 8 Tx SRS, at least support - 8 ports in 1 SRS resource for ‘antennaSwitching’; - FFS 8 ports in one or multiple SRS resources for ‘codebook’ Above does not imply support for 8 ports in one or multiple OFDM symbols Agreement For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in at least one OFDM symbol. - FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols. Agreement For SRS resource set(s) with usage ‘nonCodebook’ support 8 1-port SRS resources in one or multiple OFDM symbols. - Note: The maximum number of simultaneous SRS resources is determined via UE-capability signaling.

In this contribution, we provide our views on SRS configuration targeting TDD CJT and 8 TX operation.

- Discussion

  - SRS configuration for 8TX

    - Number of SRS resources for 8TX SRS

As already mentioned, to support 8 ports in one or multiple SRS resources for ‘codebook’ has been discussed. One way to realize 8 Tx SRS is to expand the number of antenna ports per SRS resource up to 8, the other is to use multiple existing SRS resources (up to 4 ports per SRS resource.) Various companies have presented Pros/Cons for each scheme. To support 8 ports in one SRS resource, the scheme for multiplexing 8 ports should be decided. Cyclic shift (CS) has been considered for multiplexing 8 ports. However, as shown in Table 6.4.1.4.2-1 [2], in the case of comb 4 and comb 8, multiplexing only by CS is difficult and should be used in conjunction with the other multiplexing scheme (e.g., FDM/TDM.)

- TS 38.211 - 6.4.1.4.2 Sequence generation Table 6.4.1.4.2-1: Maximum number of cyclic shifts nSRScs,max as a function of KTC. KTC; nSRScs,max. 2; 8. 4; 12. 8; 6.

On the other hand, in the case of multiple SRS resources, the existing SRS can be reused. Then, by changing the time/frequency resource allocated for each SRS resource, SRS multiplexing combining TDM/FDM and CS can be realized. Even in the case of CS-only multiplexing, it is possible to multiplex SRS within the same time/frequency resource by changing the base sequence for each SRS resource. Also, multiple SRS resources allow for more flexible SRS configurations. For example, by assigning SRS resources to each coherent group of partial-coherent UEs. For the above reasons, we propose the following:

Proposal 1:

Support multiple SRS resources for ‘codebook’

    - Number of OFDM symbols

The number of OFDM symbols for 8 Tx SRS is also discussed. Since the current SRS has a maximum of 4 ports, transmitting an 8-port SRS with 1 OFDM symbol will result in a lower transmission power per port. Even when the repetition is taken into account, in the case where SRS is multiplexed by CS, the orthogonally between ports may be worse than conventional 4-port multiplexing. In addition, TDM-based SRS multiplexing can be realized by supporting transmission over multiple OFDM symbols. Based on the above advantages, supporting transmission with multiple OFDM symbols is expected to improve the reception accuracy of SRS. In environments where the above problems do not exist (e.g., UEs near the center of a cell), SRS can be transmitted with a single OFDM symbol to reduce time resource consumption.

Proposal 2:

Support both one and multiple OFDM symbol(s) for ‘Codebook’ SRS

2.2 Antenna group of UE

In the RAN1 #109-e meeting, for 8TX UE, it was agreed to consider UE antenna layout for codebook design [3]. In use cases such as CPE envisioned for the 8TX, the terminals may be larger and each group of antennas may be situated in a different location. In this case, the channel conditions may differ significantly among antenna groups, and affect codebook selection. Also, considering coherency, in the case of 4TX, there is a total of three patterns: full coherent, partial coherent, and non-coherent, while in the case of 8TX, there are multiple partial coherent patterns. For these reasons, the UE needs to inform gNB of the antenna group of the UE and its coherency. Therefore, we propose the following:

Proposal 3:

For 8TX, the UE should inform gNB of its antenna group and its coherency.

2.3 SRS configuration considering antenna group

In the current SRS, the SRS is configured for the UE and is not associated with individual antennas. Since the UE antenna groups are taken into account when designing codebooks, associating the SRS with the UE antenna groups allows for more appropriate codebook selection.

Proposal 4:

For 8TX, SRS resources should be associated with UE antenna groups.

  - SRS configuration for TDD CJT

- The following opinions were put forward by the companies regarding power control in the TDD CJT [4].

- D: Per-TRP power control and/or power control of one SRS towards to multiple TRPs (e.g., ZTE, Google, Lenovo, Intel, Samsung, Apple, Ericsson, Nokia, Nokia Shanghai Bell, Futurewei) - D1: Per-TRP power control (Spreadtrum, Ericsson, KDDI) - Pros: No gNB receive power imbalance issue due to near-far problem - Cons: Higher SRS overhead - D2: Power control based on more than 1 DL pathloss RS (e.g., ZTE, Nokia, Nokia Shanghai Bell, Apple, Lenovo) - Pros: Low SRS overhead; one source shows throughput/channel estimation performance benefit - Cons: gNB receive power imbalance issue due to near-far problem

In the case of TDD CJT, the SRS transmitted from the UE is received by multiple TRPs. If the pathloss between the UE and multiple TRPs is different, a near-far problem occurs, which degrades the reception quality of the SRS and affects the accuracy of channel estimation. When the power control is performed with only the serving cell, SRS reception accuracy deteriorates due to the near-far problem. One way to mitigate the perspective problem is to use multiple reference signals for transmit power control (D2). Multiple reference signals from multiple TRPs with different pathloss are used for the power control. However, there is still gNB receive power imbalance issue due to near-far problem since path loss differences between different TRPs cannot be eliminated. Another way is the power control per-TRP basis (D1). The power imbalance problem can be solved by compensating for path loss differences between TRPs with per-TRP power control. On the other hand, when SRS is transmitted for each TRP, higher SRS overhead becomes a problem, so the methods to reduce the overhead should be considered.

Proposal 5:

For TDD CJT, support Per-TRP power control.

- Conclusion

In this contribution, we provided views on the SRS configuration targeting TDD CJT and 8 TX operation, and proposed the following.

Proposal 1:

Support multiple SRS resources for ‘codebook’

Proposal 2:

Support both one and multiple OFDM symbol(s) for ‘Codebook’ SRS

Proposal 3:

For 8TX, the UE should inform gNB of its antenna group and its coherency.

Proposal 4:

For 8TX, SRS resources should be associated with UE antenna groups.

Proposal 5:

For TDD CJT, support Per-TRP power control.

Reference

[1] RAN1 #110, Chair’s Notes.

[2] 3GPP TSG RAN 38.211 V17.2.0, “Physical channels and modulation”, June 2022.

[3] RAN1 #109-e, Chair’s Notes.

[4] R1-220814, FL Summary #3 on SRS enhancements, Moderator (FUTUREWEI).