3GPP TSG RAN WG1 #110bis-e			R1-2209442

e-Meeting, October 10th – 19th, 2022

Agenda Item:	8.2

Source: 	LG Electronics

Title: 	Discussion on type-1 HARQ-ACK codebook when time domain bundling is configured

Document for:	Discussion and decision

- Discussion

In RAN1#110 meeting [1], RAN1 discussed the following two interpretations on the type-1 HARQ-ACK codebook determination when time domain bundling is configured, and majority companies supported Interpretation 2.

- Interpretation 1: “a PDSCH associated with occasion m” is a PDSCH scheduled in the corresponding DL slot of occasion m, and the corresponding DL slot of occasion m is the DL slot where the last SLIV locates for determining occasion m.

- Interpretation 2: “a PDSCH associated with occasion m” is a PDSCH of which the corresponding HARQ-ACK information is mapping to occasion m.

If Interpretation 2 is taken considering majority companies’ view, at least we need to handle the issue that only one PDSCH can be associated with occasion m in the current pseudo code. As suggested by a company in RAN1#110 meeting, the following correction can address that issue.

if enableTimeDomainHARQ-Bundling is provided for serving cell c and more than one PDSCH associated with occasion m is scheduled by a DCI format indicating a TDRA row that includes more than one SLIV entry if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c ojACK = binary AND operation of the HARQ-ACK information bits corresponding to first transport blocks in PDSCH receptions, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; j=j+1; ojACK = binary AND operation of the HARQ-ACK information bits corresponding to second transport blocks in PDSCH receptions, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; ojACK= j=j+1 ojACK= j=j+1; elseif harq-ACK-SpatialBundlingPUCCH is provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c ojACK = binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format of serving cell c if the UE receives one transport block, the UE assumes ACK for the second transport block; ojACK j=j+1; else ojACK =binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format of serving cell c ojACK j=j+1; end if

One more remaining issue is how to generate type-1 HARQ-ACK codebook when more than one PDSCH is scheduled by a single DCI and only one PDSCH reception is valid for a serving cell configured with time domain bundling. Similar to the case of type-2 HARQ-ACK codebook, we can perform binary AND operation by assuming invalid PDSCH as correctly received. On top of the above correction, we can revise binary AND operation part as well, as follows and also proposed in [2].

if enableTimeDomainHARQ-Bundling is provided for serving cell c and more than one PDSCH associated with occasion m is scheduled by a DCI format indicating a TDRA row that includes more than one SLIV entry if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c ojACK = binary AND operation of the HARQ-ACK information bits corresponding to first transport blocks in PDSCH receptions scheduled by the DCI format on serving cell c, by assuming ACK for first transport blocks in PDSCH receptions that overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated; j=j+1; ojACK = binary AND operation of the HARQ-ACK information bits corresponding to second transport blocks in PDSCH receptions scheduled by the DCI format on serving cell c, by assuming ACK for second transport blocks in PDSCH receptions that overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated; ojACK= j=j+1 ojACK= j=j+1; elseif harq-ACK-SpatialBundlingPUCCH is provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c ojACK = binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs scheduled by the DCI format of serving cell c, by assuming ACK for all transport blocks in PDSCHs that overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if the UE receives one transport block, the UE assumes ACK for the second transport block; ojACK j=j+1; else ojACK =binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs scheduled by the DCI format of serving cell c, by assuming ACK for all transport blocks in PDSCHs that overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated ojACK j=j+1; end if

- Conclusion

Proposal: Adopt the following text proposal in TS 38.213.

#### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel *** Unchanged text is omitted *** Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cells including, when applicable, cells in the set S0 and the set S1 Set j=0- HARQ-ACK information bit index Set NcellsDL to the number of serving cells configured by higher layers for the UE while c<NcellsDL Set m=0 – index of occasion for candidate PDSCH reception, or SPS PDSCH release, or TCI state update while m<Mc if enableTimeDomainHARQ-Bundling is provided for serving cell c and more than one PDSCH associated with occasion m is scheduled by a DCI format indicating a TDRA row that includes more than one SLIV entry if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c ojACK = binary AND operation of the HARQ-ACK information bits corresponding to first transport blocks in PDSCH receptions scheduled by the DCI format on serving cell c, by assuming ACK for first transport blocks in PDSCH receptions that overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated; j=j+1; ojACK = binary AND operation of the HARQ-ACK information bits corresponding to second transport blocks in PDSCH receptions scheduled by the DCI format on serving cell c, by assuming ACK for second transport blocks in PDSCH receptions that overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated; ojACK= j=j+1 ojACK= j=j+1; elseif harq-ACK-SpatialBundlingPUCCH is provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c ojACK = binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs scheduled by the DCI format of serving cell c, by assuming ACK for all transport blocks in PDSCHs that overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if the UE receives one transport block, the UE assumes ACK for the second transport block; ojACK j=j+1; else ojACK =binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs scheduled by the DCI format of serving cell c, by assuming ACK for all transport blocks in PDSCHs that overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated ojACK j=j+1; end if

- Reference

R1-2208026, Summary #2 of PDSCH/PUSCH enhancements (Scheduling/HARQ), Moderator (LG Electronics)

R1-2209441, Draft CR for type-1 HARQ-ACK codebook when time domain bundling is configured, LG Electronics