# 3GPP TSG RAN WG1 #110bis-e	R1-2209470

# e-Meeting, October 10th – 19th, 2022

Source: 	ZTE

Title: 	Maintenance of broadcast and multicast for MBS

Agenda item:	8.12

Document for:	Discussion/Decision

1	Introduction

During RAN#88-e meeting, the revised WI on NR MBS has been approved [1]. Most of the functionalities of MBS have been finalized. In this contribution, we provide our analysis and TPs for the remaining issues.

2. Multicast: Scheduling mechanism

During RAN1#109-e, how to support FDMed SPS PDSCH reception between MBS and unicast was discussed with the following agreement and conclusion. The remaining issue is to define collision handling schemes under different UE capabilities.

Agreement For the FDMed or TDMed unicast PDSCH and group-common PDSCH capability of RRC_CONNECTED UE, the group-common PDSCH can be multicast group-common PDSCH or broadcast group-common PDSCH. - FFS single or separate UE capability for multicast and broadcast for intra-slot TDM unicast PDSCH and group-common PDSCH - Note 1: For the TDMed case, the maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16 - Note 2: For the FDMed case, only one unicast PDSCH and one group common PDSCH in a slot per CC is supported Conclusion For FDM between one unicast PDSCH and one group-common PDSCH in a slot, only case 1 in the following cases is supported. - Case 1: the unicast PDSCH and the group-common PDSCH in a slot are partially or fully overlapping in time domain and non-overlapping in frequency domain - Case 2: the unicast PDSCH and the group-common PDSCH in a slot are non-overlapping in time domain and non-overlapping in frequency domain - Case 3: the unicast PDSCH and the group-common PDSCH in a slot are non-overlapping in time domain and overlapping in frequency domain

For unicast SPS PDSCH and multicast SPS PDSCH collision handling, the following proposal was proposed during RAN1#109-e meeting.

Initial Proposal 1-2-v4: For unicast SPS PDSCH and multicast SPS PDSCH collision handling, - If a UE only supports TDM unicast SPS PDSCH and multicast SPS PDSCH in a slot, and more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, the UE resolves collisions among unicast SPS PDSCHs and multicast SPS PDSCHs by reusing Rel-16 rules. - If a UE only supports FDM unicast SPS PDSCH and multicast SPS PDSCH in a slot, and more than one unicast PDSCH and multicast PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, - the UE resolves collisions among unicast SPS PDSCHs resulting in one unicast SPS PDSCH and collisions among multicast SPS PDSCHs resulting in one multicast SPS PDSCH as in Rel-16, respectively. If the resulting unicast SPS PDSCH and multicast SPS PDSCH overlap in time but not overlap in frequency, the UE receives both PDSCHs; else, the UE receives the one with lower configured sps-ConfigIndex. - If a UE supports both FDM and TDM unicast SPS PDSCH and multicast SPS PDSCH in a slot, and more than one unicast PDSCH and multicast PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, - FFS

- During RAN1#110, there was no progress as the views from companies were still very divergent. If there is no conclusion at the end, it means that FDM based unicast and multicast PDSCH reception cannot be supported for SPS scheduling mechanism.

- To simplify the discussion, a simple method can be considered with less spec impact. it is recommended that the UEs can be divided into two types, i.e., FDM capable UE and non-FDM capable UE.

- Then, for unicast SPS PDSCH and multicast SPS PDSCH collision handling,

- If the UE is not capable of receiving FDMed unicast and multicast PDSCH per slot per carrier, the UE resolves collisions among unicast SPS PDSCHs and multicast SPS PDSCHs by reusing Rel-16 rules without differentiating them. In other words, the multicast SPS PDSCH is considered as unicast SPS PDSCH when processed.

- If the UE is capable of receiving FDMed unicast and multicast PDSCH per slot per carrier, the UE resolves collisions among unicast SPS PDSCHs resulting in one unicast SPS PDSCH and collisions among multicast SPS PDSCHs resulting in one multicast SPS PDSCH as step 0 and step 1 according to legacy procedure, respectively. If the resulting unicast SPS PDSCH and multicast SPS PDSCH overlap in time domain but not overlap in frequency domain, the UE receives both PDSCHs; else, the UE receives the one with lower configured sps-ConfigIndex.

According to the above analysis, we have the following proposal and the draft CR can also be found in [6].

Proposal 1: For SPS PDSCH processing for multicast and unicast,

- when the UE is not capable of receiving FDMed PDSCH between multicast and unicast, the current mechanism should be reused.

- when the UE is capable of receiving FDMed PDSCH between multicast and unicast, the current mechanism should be reused for multicast and unicast separately resulting in one multicast SPS PDSCH and unicast SPS PDSCH.

  - The UE should receive both of them if they overlap only in the time domain,

  - Otherwise, the UE should receive the one with lower SPS index

3. Multicast: UL feedback

3.1 Clarification on NACK-only feedback PUCCH resources

1-bit NACK-only feedback is introduced in the following agreement. It was agreed the PUCCH resource of the 1-bit NACK-only feedback should come from a PUCCH resource set with up to 32 PUCCH resources.

Agreement For the separate PUCCH-Config/ PUCCH-ConfigurationList configured to UE for NACK-only based feedback, - 1 PUCCH resource set in each PUCCH-Config. - up to 32 PUCCH resources in PUCCH resource set - Note: the separate PUCCH-Config/PUCCH-ConfigurationList applies to all configured G-RNTIs configured with NACK-only based feedback.

For NACK-only mode 2, the UE select PUCCH resource from at most 15 PUCCH resources according to the PDSCH decoding results as described in Table 18-1 in TS38.213. Whether the PUCCH resources in Table 18-1 are from the above-mentioned PUCCH resource set with up to 32 PUCCH resources has not been discussed yet. One possibility is that the PUCCHs in Table 18-1 are configured independently. We also note the description in the above agreement: "the separate PUCCH-Config/PUCCH-ConfigurationList applies to all configured G-RNTIs configured with NACK-only based feedback".

So, it is not clear so far. It is suggested to clarify the relationship between the PUCCH resources in Table 18-1 and PUCCH resource sets with up to 32 PUCCH resources.

Proposal 2: RAN1 clarifies the configuration of PUCCH resources in Table 18-1 in TS38.213-h30 [2].

3.2 About NACK-only feedback for NACK-only mode2

In RAN1#108-e, #109-e and #110 meetings, the following agreements on NACK-only were reached in [3] [4] [5], but still some FFSs are left.

Agreement For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements): - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. - FFS: how to determine PUCCH resource - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission. - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast. - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID. - FFS mapping details. - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook. - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs - Alt4 is not supported for more than 4 TBs - FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR - FFS: UE capability Agreement For UE configured with NACK-only HARQ-ACK feedback, for Alt4 (from previous agreement), define the mapping between HARQ-ACK values and the PUCCH resources for the TBs configured with NACK-only as follows: - The mapping is applied to both a single G-RNTI - FFS: multiple configured G-RNTIs cases. - FFS: how to order the TBs, whether/how to account for missed or non-scheduled TBs - FFS whether/how PRI is used for indication. Agreement For multicast, for addressing how to count and order the HARQ-ACK bits for NACK-only for Alt4: - Case 1: when the HARQ-ACK-to-PUCCH mapping table for Alt4 (from previous agreement) is applied to a single G-RNTI (from network and UE perspective) - Opt1-1: based on C-DAI included in DL DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. - FFS: whether Opt1-1 can also work when different G-RNTIs are used for different UEs - Note: other cases are not precluded Agreement For multicast, for addressing how to count and order the HARQ-ACK bits for NACK-only for Alt4, further down-selection on the following cases/options in the next meeting: - Case 2: for the case of all UEs configured with the same set of G-RNTIs - Opt2-1: support Alt4 for this case - Opt2-1-1: based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI for counting the total number of HARQ-ACK bits. The ordering of HARQ-ACK bits is per C-DAI from each G-RNTI and in the ascending order of G-RNTI values. - Opt2-1-2: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs - Opt2-2: does not support Alt4 for this case. - Case 3: for the case of different UEs configured with different G-RNTIs - Opt3-1: support Alt4 for this case - Opt3-1-1: based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI for counting the total number of HARQ-ACK bits. The ordering of HARQ-ACK bits is per C-DAI from each G-RNTI and in the ascending order of G-RNTI values. - Opt3-1-2: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs. - Opt3-2: does not support Alt4 for this case. Agreement For PRI included in DCI format 4_1/4_2, - if the G-RNTI is configured with NACK-only mode1, PRI indicates the PUCCH for the HARQ-ACK feedback including the case of one TB in scheduled. - If the G-RNTI is configured with NACK-only mode2, - FFS on whether/how to interpret the PRI.

For NACK-only feedback, two modes are defined. For NACK-only mode1, NACK-only is converted into ACK/NACK for more than one TBs. For NACK-only mode2, PUCCH resource selection is based on PDSCH decoding result.

For case2, the UE receives more than one G-RNTIs and all the UE receive the same set of G-RNTIs. Opt2-1-2 can effectively reduce the number of last DCIs compared to the Opt2-1-1. For example, there is always only one last DCI regardless of how many G-RNTIs are there. Based on this, it can resolve the ambiguity on the number of bits of HARQ-ACKs for NACK-only when the UE misses the last DCI of the G-RNTI other than the last G-RNTI. Therefore, we prefer Opt2-1-2 to be used for counting and ordering HARQ-ACK bits for NACK-only mode2

For case3, we think it is a complex scenario. It can be switched to case2 to some extent. For example, the base station always divides UEs receiving the same G-RNTI set into one MBS group. The different MBS group can be configured with different NACK-only PUCCH resource set. However, if companies believe case 3 is still possible, we also prefer Opt3-1-2 for similar reasons as case2 and the common solution to be used.

In addition, there is no additional signaling overhead for Opt2-1-2 and Opt3-1-2. The C-DAI* should be used to count scheduled MBS TBs. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs. For case1, although only one G-RNTI is configured, the C-DAI* can still be used for NACK-only mode2.

Proposal 3: For multicast, the following options are adopted for addressing to count and order the HARQ-ACK bits for NACK-only for NACK-only mode2:

- Case 2: for the case of all UEs configured with the same set of G-RNTIs

- Opt2-1-2 is supported for NACK-only mode2

  - Opt2-1-2: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs

- Case 3: for the case of different UEs configured with different G-RNTIs

- Opt3-1-2 is supported for NACK-only mode2

  - Opt3-1-2: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs.

Whether/how to interpret the PRI for NACK-only mode2

It is agreed that if the G-RNTI is configured with NACK-only mode1, PRI included in DCI format 4_1/4_2 indicates the PUCCH for the HARQ-ACK feedback including the case of one TB in scheduled.

In NACK-only mode2, the PUCCH resource is determined based on the decoding result of MBS TBs when the number of MBS TBs is less than or equal to 4. In this case, the PRI field in DCI format 4_1/4_2 for scheduling MBS TBs is unused. Therefore, PRI included in DCI format 4_1/4_2 should be re-interpreted. From our understanding, PRI can be re-interpreted as:

- C-DAI*. The C-DAI* is accumulating the scheduled MBS TBs across different G-RNTIs.

- T-DAI. The T-DAI is the total number of scheduled MBS TBs across different G-RNTIs for NACK-only mode2. It can achieve the same purpose with RRC signaling to configure the number of HARQ-ACK bits for NACK-only feedback. For example, the base station needs to predetermine the number of MBS TBs corresponding to NACK-only feedback, and then set the PRI to indicate the number for NACK-only feedback. It is similar to RRC configuring the number of MBS TBs corresponding to NACK-only feedback, and only using PRI to indicate the number is more flexible.

About indicating the number of MBS TBs for NACK-only feedback in NACK-only mode2

For NACK-only mode2, once the last DCI is lost, it will cause serious problems.

For example, suppose that the base station sends 3 MBS TBs, the UE receives the first 2 MBS TBs correctly, but loses the last DCI corresponding to the third MBS TB. In this way, from the UE perspective, the base station sends 2 MBS TBs. if the first two MBS TBs are received correctly, the UE will not perform NACK-only feedback. However, from the base perspective, all of the 3 MBS TBs are considered to be correctly received by the UE. So, the base station will not retransmit the third MBS TB. In this case, the UE will completely lose the third TBs since it will not be retransmitted.

To overcome this problem, a simple method is provided: for NACK-only mode2, the number of HARQ-ACK bits for NACK-only feedback is fixed or configured by RRC.

It may be assumed that for NACK-only mode2, the number of HARQ-ACK bits is configured to 3 via RRC signaling, then the UE always determines the PUCCH resource according to 3 bits based on the decoding result and the Table 18-1. For example, in the above example, from the UE's point of view, the UE has correctly received 2 MBS TBs, but the UE still sends 3 bits of HARQ-ACKs by supplementing 0 for the third bit. That is, the HARQ-ACKs information (1, 1, 0) is used to determine the PUCCH resource for transmission. In this way, after the base station receives the PUCCH resource, it can know that the third MBS TB is not correctly received, and thus retransmit it.

Another example, for NACK-only mode2, the number of HARQ-ACK bits is configured 3 via RRC signaling. Suppose that the base station transmits 2 MBS TBs. The UE received 2 MBS TBs correctly, and it still feeds back the 3bits HARQ-ACKs by supplementing 0 for the third bit. That is, the HARQ-ACKs information (1, 1, 0) is used to determine the PUCCH resource for transmission. In this way, after the base station receives the PUCCH, it can know that the UE requests to retransmit the third MBS TB. The base station does not retransmit the third MBS TB since the third MBS TB is not actually transmitted,

Another example, for NACK-only mode2, the number of HARQ-ACK bits is configured 3 via RRC signaling. Assuming that the base station sends 3 MBS TBs, the UE receives 3 DCIs for the 3 MBS TBs, and it generates 3 bits HARQ-ACKs according to the decoding result. The UE does not need to add padding bits.

For NACK-only mode2, RRC signaling is used to configure the number of bits of HARQ-ACKs, which is also conducive to saving PUCCH resources. For example, in the above example, only 7 PUCCH resources need to be configured. This method is flexible and simple, and it is beneficial to reduce PUCCH resources without affecting the scheduling of the base station.

In addition, it can also consider the above reinterpreted PRI as T-DAI to indicate the number of HARQ-ACK bits for NACK-only feedback in NACK-only mode2. Compared with RRC signaling, it is more flexible to indicate the number of HARQ-ACK bits by reinterpreting PRI.

Proposal 4: For multicast, the following method is adopted in case 1/2/3 for NACK-only mode2:

In NACK-only mode2, the PRI field in the DCI format 4_1/4_2 is reinterpreted as the C-DAI* or T-DAI.

For NACK-only mode2, the number of HARQ-ACK bits for NACK-only feedback is fixed or indicated by RRC or by reinterpreting PRI as T-DAI.

- The T-DAI is the total number of scheduled MBS TBs across different G-RNTIs for NACK-only feedback in NACK-only mode2.

- The UE always adds NACK at the end of the determined HARQ-ACK bits if the number of determined HARQ-ACK bits is less than the number of fixed or indicated HARQ-ACK bits.

3.3 Remaining issue PUCCH resource for NACK-only mode1

In RAN1#109-e meeting, the following agreements on NACK-only were reached [3] [4], but there is still an FFS left for NACK-only mode1 on how to determine the PUCCH resource in the case NACK-only is for multicast SPS PDSCH only.

Agreement - For the case when the PUCCH transmission for the NACK-only based feedback for one G-RNTI collides with the PUCCH transmission for ACK/NACK feedback for another G-RNTI with the same priority and then UE multiplexes the NACK-only based feedback with the ACK/NACK feedback onto the same PUCCH by transforming NACK-only into the ACK/NACK based HARQ-ACK bit, the PUCCH resources for transmitting the multiplexed HARQ-ACK bits is from the PUCCH-Config/PUCCH-ConfigurationList configured for multicast with ACK/NACK based feedback. - For a UE configured with G-RNTI(s) with NACK-only HARQ-ACK feedback, when NACK-only HARQ-ACK bits are transformed into ACK/NACK HARQ-ACK bits, the PUCCH resource used for transmitting the multiplexed HARQ-ACK bits is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast with ACK/NACK based feedback based on the k1 and the PRI indication in the last DCI scheduling multicast. If PUCCH-Config/PUCCH-ConfigurationList configured for multicast with ACK/NACK based feedback is not configured, the PUCCH-Config/PUCCH-ConfigurationList configured for unicast is used. - FFS: the case NACK-only is for multicast SPS PDSCH only.

For dynamic scheduled PDSCH for MBS, it was agreed that the PUCCH resource configured for unicast can be used when the PUCCH resource configured for MBS is not configured. The same principle can be reused for SPS PDSCH feedback in our understanding. For SPS PDSCH for unicast, the PUCCH resource is configured by sps-PUCCH-AN-List for more than one PDSCHs. Therefore, PUCCH resource configured by PUCCH-Config/sps-PUCCH-AN-List can be used for HARQ-ack information for SPS PDSCH for MBS transferred from NACK-only when PUCCH-Config/PUCCH-ConfigurationList is not configured for MBS.

Proposal 5: PUCCH resource configured by PUCCH-Config/sps-PUCCH-AN-List can be used for HARQ-ack information for SPS PDSCH for MBS transferred from NACK-only when PUCCH-Config/PUCCH-ConfigurationList is not configured for MBS.

3.4 Processing timeline for NACK-only based feedback

In the RAN1#109-e meeting, the PDSCH/PUCCH processing timeline for NACK-only was discussed, the following agreement was reached, but some issues were left. This issue was discussed in the RAN1#110-e meeting but still no agreement was reached.

Agreement Regarding the PDSCH and/or PUCCH processing procedure timeline for NACK-only based feedback, further discuss on whether/how to extending the PDSCH and/or PUCCH processing procedure timeline. - Note1: The current PDSCH processing procedure timeline defined in clause 5.3 TS38.214 is for ACK/NACK based feedback. - Note2: For NACK-only based feedback for more than 1 TB, one possible issue is that the PUCCH resource for NACK-only can only be determined after obtaining the decoding result of all the related PDSCHs for the UE.

From our understanding, for the feedback of the HARQ-ACK information, the UE determines a PUCCH resource for the HARQ-ACK information corresponding to a PDSCH scheduled by a DCI after the DCI is decoded. However, since the decoding result of the PDSCH is required to perform encoding and modulation of the HARQ-ACK information, the encoding and modulation are performed after the PDSCH is decoded.

For the feedback of HARQ-ACKs information of more than one PDSCHs, the UE can perform the encoding and modulation of the HARQ-ACKs information after the last PDSCH is decoded in a same principle.

Based on the same principle, for HARQ-ACKs information of one or more MBS PDSCHs for NACK-only feedback, although the PUCCH resource cannot be determined after the DCI scheduling the PDSCHs is decoded, the UE can perform the encoding and modulation of the HARQ-ACKs information after the last MBS PDSCH is decoded.

Regarding the transmission power affected by the frequency resource location, MPR is defined by RAN4 for three RB allocations, i.e., edge RB allocations, outer RB allocations, and inner RB allocations. The same RB allocation has the same MPR. Therefore, the same transmission power can be obtained as long as the network configures the PUCCH resource for NACK-only with the same RB allocations.

Based on the above analysis, for NACK-only PUCCH with PF0 and PF1, it still needs the decoding result of MBS PDSCH to determine the transmitted information in NACK-only PUCCH. We found no need to redefine processing time for NACK-only feedback.

Proposal 6: For processing timeline for NACK-only based feedback, the current PDSCH processing procedure timeline defined in clause 5.3 TS38.214 for ACK/NACK based feedback is reused for NACK-only.

3.5 Disabling feedback for Type-1/2 codebook

In the RAN#109-e meeting, the following agreement was reached [4], but some issues were still left.

Agreement For Type-1 codebook generation, if at least one configured G-RNTI is with HARQ-ACK enabled by RRC, it is up to UE to report NACK or ACK/NACK for the G-RNTI with HARQ-ACK disabled - FFS whether UE needs to generate Type-1 CB, when UE does not detect any DCI for G-RNTI(s) with HARQ-ACK enabled when at least one configured G-RNTI is with HARQ-ACK enabled by RRC. - For the PUCCH resource for the codebook transmission, - Opt2: PUCCH resource/slot is based on last DCI for a G-RNTI with HARQ-ACK enabled

Similar issues to the above FFS were discussed in the RAN1#109 meeting, and the following agreement was reached, which can be reused here. In our opinion, the conditions in FFS are different from those in the agreement below, but the same result should be expected.

Agreement For the 2-bit UL-DAI for multicast indicating the total number of Type-2 HARQ-ACK bits multiplexed in PUSCH, UE generates the Type-2 HARQ-ACK sub-codebook for each G-RNTI per the UL-DAI and concatenates all the sub-codebooks in the ascending order of G-RNTI value. - If the 2-bit UL-DAI field for multicast has the value of and the UE has not received any PDCCH within the monitoring occasions with DCI format 4_1/4_2, the UE does not multiplex HARQ-ACK information for multicast feedback in the PUSCH transmission.

For the Type-1 CB, if the UE receives a UL DAI=1 in UL grant for scheduling a PUSCH, then the UE generates the Type-1 CB when UE does not detect any DCI for G-RNTI(s ) with HARQ-ACK enabled when at least one configured G-RNTI is with HARQ-ACK enabled by RRC. Otherwise, the UE does not generate the Type-1 CB.

A similar rule is for Type-2 CB. For the Type-2 CB, if the UE receives a UL DAI != 4 in UL grant for scheduling a PUSCH, then the UE generates the Type-2 CB when UE does not detect any DCI for G-RNTI(s ) with HARQ-ACK enabled when at least one configured G-RNTI is with HARQ-ACK enabled by RRC. Otherwise, the UE does not generate the Type-2 CB.

Proposal 7:

- For the Type-1 CB, if the UE receives a UL DAI = 1 in UL grant for scheduling a PUSCH, then the UE generates the Type-1 CB when UE does not detect any DCI for G-RNTI(s ) with HARQ-ACK enabled when at least one configured G-RNTI is with HARQ-ACK enabled by RRC. Otherwise, the UE does not generate the Type-1 CB.

- For the Type-2 CB, if the UE receives a UL DAI != 4 in UL grant for scheduling a PUSCH, then the UE generates the Type-2 CB when UE does not detect any DCI for G-RNTI(s ) with HARQ-ACK enabled when at least one configured G-RNTI is with HARQ-ACK enabled by RRC. Otherwise, the UE does not generate the Type-2 CB.

3.6 Time domain overlapping between NACK-only PUCCH and SR

In the RAN1#108-e meeting, NACK-only PUCCH and SR PUCCH overlapped in the time domain, for example, the following proposal was discussed, but no consensus was reached [3].

#### Proposal 3.2.1.4-1 (GTW) When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support the following: - For up to two HARQ-ACK bits for NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure of Rel-15/16. - Alt1-1: PUCCH resources configured for unicast will be used. - Note: either the PUCCH resource for SR or for ACK/NACK is used per the legacy procedure of Rel-15/16. - the PUCCH resource is determined by the PRI in the last DCI for multicast; - Alt1-2: the PUCCH resource is determined by RRC configuration; - Alt1-3: the PUCCH resource for NACK-only feedback transmission is SR resource. - For the case of more than two TBs of NACK-only, FFS which alternative from the following is supported: - Alt2-1: SR is dropped - Alt2-2: NACK-only feedback is transformed into ACK/NACK HARQ bits and PF 2/3/4 is used for the transmission per the legacy SR and HARQ-ACK multiplexing rule of Rel-15/16. - Alt2-3: NACK-only is dropped #### Proposal 3.2.1.4-2 (if proposal 3.2.1.4-1 is not agreeable) Multiplexing NACK-only based feedback for multicast with SR is not supported in Rel-17, - It is up to UE implementation when the PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR.

- From our point of view, if up to 2 TBs of NACK-only overlap with one SR in the time domain, then convert NACK-only to ACK/NACK bits, and then reuse the existing 1/2 bits HARQ-ACK and SR multiplexing mechanism. But the multiplexing result PUCCH must be the PUCCH resource of the SR, because NACK-only PUCCH is shared by multiple UEs and it cannot be used as the multiplexing result PUCCH.

- For more than 2 TBs of NACK-only and one SR overlapping in the time domain, multiple NACK-only are converted to ACK/NACK bits and SR is converted into ACK/NACK bits, and then reuse the existing more than 2 bits HARQ-ACKs and SR multiplexing mechanism. The multiplexing result PUCCH is determined from the unicast PUCCH set according to the PRI in the last DCI that schedules the more than 2 TBs of NACK-only.

Proposal 8: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority,

- For up to two HARQ-ACK bits for NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure of Rel-15/16.

  - Always use SR PUCCH as the multiplexing result PUCCH.

- For more than two HARQ-ACK bits for NACK-only feedback is transformed into ACK/NACK HARQ bits and SR is transformed into ACK/NACK bits, and then reuse the existing more than 2 bits HARQ-ACKs and SR multiplexing mechanism, and always use SR PUCCH as the multiplexing result PUCCH

  - The multiplexing result PUCCH is determined from the unicast PUCCH set according to the PRI in the last DCI from the DCIs corresponding to the more than 2 HARQ-ACK bits.

3.7 Multiplexing with different priorities

It was agreed that MBS can be configured with different priorities. The priority is indicated by the DCI. It was agreed that the current intra-UE mechanism is reused for MBS. In Rel-16, a PUCCH for MBS with higher priority can cancel a PUCCH or PUSCH with lower priority. For MBS, NACK-only feedback is supported. The UE transmits the PUCCH only when at least one of the PDSCHs is not decoded correctly. In this case, when PUCCH for NACK-only feedback with higher priority overlaps with a PUSCH or another PUCCH with lower priority in the time domain, the UE should not cancel the PUSCH or another PUCCH when the UE decode the PDSCH for MBS correctly. It can improve the spectrum efficiency significantly since the probability of correct decoding is quite high.

Proposal 9: When the NACK-only PUCCH with higher priority for MBS PDSCH overlaps with a PUSCH or another PUCCH with lower priority in the time domain, the UE should not drop the PUSCH or another PUCCH with lower priority if the UE decode the MBS PDSCH correctly.

For NACK-only mode 2, the UE selects the PUCCH resource according to the PDSCH decoding results. When a NACK-only PUCCH with higher priority overlaps with a NACK-only PUCCH with lower priority in the time domain, the PDSCH with higher priority and PDSCH with lower priority can both be considered together to determine the PUCCH resource since it may not degrade the PUCCH decoding performance. In some cases, the UE may select the same PUCCH resource. For example, if only the PDSCH with higher priority is considered, i.e., the NACK-only PUCCH with lower priority is dropped, the first PUCCH resource is selected when the PDSCH is not decoded correctly. If the PDSCH with higher priority and the PDSCH with lower priority are both considered together, the first PUCCH resource is selected when the two PDSCHs are not decoded correctly.

Proposal 10: When a NACK-only PUCCH with higher priority overlaps with a NACK-only PUCCH with lower priority in the time domain, the PDSCH with higher priority and PDSCH with lower priority can both be considered together to determine the PUCCH resource for NACK-only mode 2.

4	Conclusion

In this contribution, we discuss the potential candidate solutions to remaining issues of NR MBS with the following observations and proposals.

## Multicast: Scheduling mechanism

Proposal 1: For SPS PDSCH processing for multicast and unicast,

- when the UE is not capable of receiving FDMed PDSCH between multicast and unicast, the current mechanism should be reused.

- when the UE is capable of receiving FDMed PDSCH between multicast and unicast, the current mechanism should be reused for multicast and unicast separately resulting in one multicast SPS PDSCH and unicast SPS PDSCH.

  - The UE should receive both of them if they overlap only in the time domain,

  - Otherwise, the UE should receive the one with lower SPS index

## Multicast: UL feedback

Proposal 2: RAN1 clarifies the configuration of PUCCH resources in Table 18-1 in TS38.213-h30 [2].

Proposal 3: For multicast, the following options are adopted for addressing to count and order the HARQ-ACK bits for NACK-only for NACK-only mode2:

- Case 2: for the case of all UEs configured with the same set of G-RNTIs

- Opt2-1-2 is supported for NACK-only mode2

  - Opt2-1-2: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs

- Case 3: for the case of different UEs configured with different G-RNTIs

- Opt3-1-2 is supported for NACK-only mode2

  - Opt3-1-2: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs.

Proposal 4: For multicast, the following method is adopted in case 1/2/3 for NACK-only mode2:

In NACK-only mode2, the PRI field in the DCI format 4_1/4_2 is reinterpreted as the C-DAI* or T-DAI.

For NACK-only mode2, the number of HARQ-ACK bits for NACK-only feedback is fixed or indicated by RRC or by reinterpreting PRI as T-DAI.

- The T-DAI is the total number of scheduled MBS TBs across different G-RNTIs for NACK-only feedback in NACK-only mode2.

- The UE always adds NACK at the end of the determined HARQ-ACK bits if the number of determined HARQ-ACK bits is less than the number of fixed or indicated HARQ-ACK bits.

Proposal 5: PUCCH resource configured by PUCCH-Config/sps-PUCCH-AN-List can be used for HARQ-ack information for SPS PDSCH for MBS transferred from NACK-only when PUCCH-Config/PUCCH-ConfigurationList is not configured for MBS.

Proposal 6: For processing timeline for NACK-only based feedback, the current PDSCH processing procedure timeline defined in clause 5.3 TS38.214 for ACK/NACK based feedback is reused for NACK-only.

Proposal 7:

- For the Type-1 CB, if the UE receives a UL DAI = 1 in UL grant for scheduling a PUSCH, then the UE generates the Type-1 CB when UE does not detect any DCI for G-RNTI(s ) with HARQ-ACK enabled when at least one configured G-RNTI is with HARQ-ACK enabled by RRC. Otherwise, the UE does not generate the Type-1 CB.

- For the Type-2 CB, if the UE receives a UL DAI != 4 in UL grant for scheduling a PUSCH, then the UE generates the Type-2 CB when UE does not detect any DCI for G-RNTI(s ) with HARQ-ACK enabled when at least one configured G-RNTI is with HARQ-ACK enabled by RRC. Otherwise, the UE does not generate the Type-2 CB.

Proposal 8: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority,

- For up to two HARQ-ACK bits for NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure of Rel-15/16.

  - Always use SR PUCCH as the multiplexing result PUCCH.

- For more than two HARQ-ACK bits for NACK-only feedback is transformed into ACK/NACK HARQ bits and SR is transformed into ACK/NACK bits, and then reuse the existing more than 2 bits HARQ-ACKs and SR multiplexing mechanism, and always use SR PUCCH as the multiplexing result PUCCH

  - The multiplexing result PUCCH is determined from the unicast PUCCH set according to the PRI in the last DCI from the DCIs corresponding to the more than 2 HARQ-ACK bits.

Proposal 9: When the NACK-only PUCCH with higher priority for MBS PDSCH overlaps with a PUSCH or another PUCCH with lower priority in the time domain, the UE should not drop the PUSCH or another PUCCH with lower priority if the UE decode the MBS PDSCH correctly.

Proposal 10: When a NACK-only PUCCH with higher priority overlaps with a NACK-only PUCCH with lower priority in the time domain, the PDSCH with higher priority and PDSCH with lower priority can both be considered together to determine the PUCCH resource for NACK-only mode 2.

5	Reference

RP-201038, WID revision: NR Multicast and Broadcast Services, RAN#88e.

3GPP TS 38.213, Physical layer procedures for control, h30.

3GPP RAN1#108-e, Chairman’s notes.

3GPP RAN1#109-e, Chairman’s notes.

3GPP RAN1#110, Chairman’s notes.

R1-2209474, Draft CR on SPS collision handling