3GPP TSG RAN WG1 #110b-e		                                    		   R1- 2209486

e-Meeting, Oct. 10th – 19th, 2022

Agenda Item:	9.5.1.1

Source:	InterDigital, Inc.

Title:	Evaluation results for SL positioning

Document for:	Discussion and Decision

# Introduction

In RAN#110-e, the following agreements were made [1].

Agreement For SL positioning evaluation in IIOT use case, companies should report how to drop anchor UEs and how to select anchor UEs Agreement Adopt the tables in section 3 of R1-2207606 as templates to collect SL positioning simulation results from each company. Agreement In the evaluation, relative positioning or ranging is performed between two UEs within X m, where X value(s) are reported by companies, and companies should also report the minimum distance used in the evaluations for each use case. The assumption used for X will be included in the TR for each set of results. Agreement For SL positioning evaluation purpose, the following assumptions are further adopted - Companies should report whether SL-PRS and other SL signals are FDMed or not FDMed, and whether other SL signals are present - Adopting system level simulations (rather than the link level simulations) as the baseline tool - For SL positioning evaluation in highway scenario or urban grid scenario, the performance metrics can include absolute horizontal accuracy, relative horizontal accuracy, ranging with distance accuracy, and ranging with direction accuracy (optionally). - In highway and urban grid scenarios, companies can further consider other UE types, e.g. pedestrian UE or VRU devices.

In this contribution, SLS sidelink positioning evaluation results for IIoT scenarios are shown and observations are made.

# Evaluation results

Summary of evaluation assumptions

In this section, SLS evaluation results for sidelink positioning are shown. A summary of the evaluation assumptions is presented below. Detailed simulation parameter assumptions are listed in Table A1.

- Channel model : BS-2-UE channel model defined in TR 38.901 is revised (Option 1 agreed in RAN1#109e)

- The target UE selects anchor UEs based on

  - RSRP

  - RSRP and LOS indicator

- The following assumption is assumed with respect to the synchronization error between the anchor UEs

  - Perfect synchronization among anchor UEs

  - Synchronization error exists between anchor UEs (T1=50 ns)

- The locations of anchor UEs are known without any uncertainty

- Target UEs are dropped

  - uniformly across the entire floor

  - uniformly in a convex hull

We considered SL-TDOA and multi-RTT methods for evaluation of sidelink positioning. For evaluation of SL-TDOA method the following parameters are varied to study the effect of each parameter on the accuracy performance:

- Number of anchor UEs dropped and their distribution

- Anchor UE selection criteria (with and without LOS indicator)

- Distribution of the target UEs

- Synchronization error between anchor UEs

For evaluation of multi-RTT sidelink positioning target UE selects 4 anchor UEs corresponding to 4 highest RSRP values.

Summary of evaluation results

Horizontal and vertical accuracy results of different positioning methods in different scenarios are presented in Table 1 and Table 2, respectively.

Table 1 Horizontal accuracy of IIOT sidelink positioning (m)

| Simulation Case | 50% ile | 67% ile | 80% ile | 90 %ile |
| --- | --- | --- | --- | --- |
| Case 1: SL-TDOA, Nmax - 5, small hall, height of anchor UEs- 8m, 28 anchor UEs dropped uniformly | 0.7085 | 1.2506 | 1.8833 | 2.5966 |
| Case 2: SL-TDOA, Nmax - 5, small hall, height of anchor UE – 8 m, target UEs dropped in Convex hull, 18 Anchor UEs | 0.6275 | 1.1785 | 1.8569 | 2.5681 |
| Case 3: SL-TDOA, Nmax - 5, (flexible number of) LOS TRP selection , height of anchor UEs- 8m, 28 anchor UEs dropped uniformly | 0.7202 | 1.0812 | 1.6074 | 2.2322 |
| Case 4: SL-TDOA, Nmax - 5, small hall, height of anchor UEs- 8m, 28 anchor UEs dropped uniformly, enabled synchronization error between anchor UEs | 4.2254 | 5.4050 | 6.6720 | 8.9170 |
| Case 5: Multi-RTT, small hall, height of anchor UEs- 8m, 28 anchor UEs dropped uniformly | 0.1907 | 0.2375 | 0.2920 | 0.3650 |

Based on the results presented in Table 1 and Table 2, the following observations are made.

Observation 1: For sidelink based positioning, at 90%ile, multi-RTT method achieves approximately 2.23 m horizontal accuracy improvement and 2.14m vertical accuracy improvement over SL-TDOA.

Observation 2: Multi-RTT sidelink positioning method fulfills Set A horizontal accuracy requirements.

Observation 3: Synchronization error among anchor UEs resulting in horizontal accuracy loss of 6.32 m for 90%ile UEs. (Case 4 vs Case 1)

Observation 4: Use of LOS indicator to select target UE for sidelink positioning leads to accuracy gain of ~0.36 m for 90%ile UEs.

Table 2 Vertical accuracy of IIOT sidelink positioning (m)

| Simulation Case | 50% ile | 67% ile | 80% ile | 90 %ile |
| --- | --- | --- | --- | --- |
| Case 1: SL-TDOA, Nmax - 5, small hall, height of anchor UEs- 8m, 28 anchor UEs dropped uniformly | 0.9877 | 1.4061 | 2.0108 | 3.3708 |
| Case 2: SL-TDOA, Nmax - 5, small hall, height of anchor UE – 8 m, target UEs dropped in Convex hull, 18 Anchor UEs | 0.9324 | 1.4252 | 2.0053 | 3.2433 |
| Case 3: SL-TDOA, Nmax - 5, (flexible number of) LOS TRP selection, height of anchor UEs- 8m, 28 anchor UEs dropped uniformly | 1.1748 | 1.8494 | 2.7285 | 3.7424 |
| Case 4: SL-TDOA, Nmax - 5, small hall, height of anchor UEs- 8m, 28 anchor UEs dropped uniformly, enabled synchronization error between anchor UEs | 1.2893 | 1.6529 | 2.5486 | 3.5200 |
| Case 5: Multi-RTT, small hall, height of anchor UEs- 8m, 28 anchor UEs dropped uniformly | 0.6893 | 0.8201 | 0.9727 | 1.2306 |

Finally, percentiles of UEs which satisfied the requirements agreed in RAN1#109e are shown in Table 3.

Table 3 : UEs satisfying the target positioning accuracy

| Simulation case | Set A IIoT horizontal accuracy requirements of 1m | Set A IIoT vertical accuracy requirements of 1m | Set B IIoT horizontal accuracy requirements of 0.2m | Set B IIoT vertical accuracy requirements of 0.2m |
| --- | --- | --- | --- | --- |
| Case 3: SL-TDOA, Nmax - 5, (flexible number of) LOS TRP selection, height of anchor UEs- 8m, 28 anchor UEs dropped uniformly | 64%ile | 44%ile | 14.5%ile | 11%ile |
| Case 5: Multi-RTT, small hall, height of anchor UEs- 8m, 28 anchor UEs dropped uniformly | 97%ile | 82%ile | 54%ile | 7.9%ile |

Based on the observations, the following proposal is made.

Proposal 1: Study configuration/determination of a synchronization source to synchronize anchor UEs for SL-TDOA

# Conclusion.

In this contribution, the following observations and proposals are made.

Observation 1: For sidelink based positioning, at 90%ile, multi-RTT method achieves approximately 2.23 m horizontal accuracy improvement and 2.14m vertical accuracy improvement over SL-TDOA.

Observation 2: Multi-RTT sidelink positioning method fulfills Set A horizontal accuracy requirements.

Observation 3: Synchronization error among anchor UEs resulting in horizontal accuracy loss of 6.32 m for 90%ile UEs. (Case 4 vs Case 1)

Observation 4: Use of LOS indicator to select target UE for sidelink positioning leads to accuracy gain of ~0.36 m for 90%ile UEs.

Proposal 1: Study configuration/determination of a synchronization source to synchronize anchor UEs for SL-TDOA

# Reference

[1] RAN1 Chairman’s note, RAN1#110e, August 2022.

# Appendix.

Table A1: IIoT baseline scenario parameters

| Parameter | Values |
| --- | --- |
| Carrier frequency, GHz | 3.5GHz |
| Bandwidth, MHz | 100MHz |
| Subcarrier spacing, kHz | 30kHz |
| Channel model | InF-SH |
| Hall size | small hall (L=120m x W=60m): D=20m |
| Number of anchor UE and their locations | Shown in Table A2 |
| Room height | 10 m |
| Number of floors | 1 |
| Clutter parameters: {density , height ,size } | InF-SH - {20%, 2m, 10m} |
| Minimum distance between anchor UE and target UE | 5m |
| Synchronization error between anchor UEs (T1) | - 0 ns - 50ns |
| Target UE model parameters |  |
| UE noise figure, dB | 9dB |
| UE antenna configuration | Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) |
| UE antenna radiation pattern | Omni, 0dBi |
| UE horizontal drop procedure | Uniformly distributed - within convex hull of anchor UE deployment - covering entire factory floor |
| UE antenna height | 1.5 m |
| Anchor UE model parameters |  |
| Anchor UE TX power, dBm | 23dBm |
| Anchor UE antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH=dV=0.5λ |
| Anchor UE antenna radiation pattern | Omni, 0dBi |
| Anchor UE antenna height | 8 m |
| Timing measurement error | Error value ε per timing measurement is added . ε is modelled as a normal distribution with mean=0 and standard deviation=0.25 ns. |

Table A2: Placement of anchor UEs in IIoT scenarios

| Placement pattern # | Descriptions | Placement |
| --- | --- | --- |
| 1 | 18 anchor UEs on a square lattice with spacing D, located D/2 from the walls |  |
| 2 | 28 anchor UEs on equally spaced lattice with spacing D. |  |