3GPP TSG RAN WG1 #110b-e		                                    		   R1-2209489

e-Meeting, Oct. 10th – 19th, 2022

Agenda Item:	9.5.2.2

Source:	InterDigital, Inc.

Title:	Discussion on positioning based on NR carrier phase measurement

Document for:	Discussion and Decision

# Introduction

In RAN1-110e, the following agreements for positioning based on NR carrier phase measurement were made:

Agreement Endorse the templates in section 17 under (H)(Round 1) Proposal 17-1 in R1-2207690 to collect carrier-phase based positioning simulation results, with the following notes: - The TR editor can adjust the sections/sub-sections arrangement - Adjust the titles of the tables to refer to NR carrier-phase based positioning - The detailed rows of the tables can be further discussed Agreement In the evaluation of NR carrier phase positioning, the following frequency errors can be considered, which are modeled independently for each UE and each TRP: - Initial Residual CFO (is the same for one measurement instances [or multiple phase measurement instances]): - Ideal: 0 (UE/TRP) - Practical: uniform distribution within - [-30, +30] Hz (FR1, UE), [-100, +100] Hz (FR1, UE), - [-120, +120] Hz (FR2, UE), [-400, +400] Hz (FR2, UE), - [-10, +10] Hz (for each TRP, FR1), - [-40, +40] Hz (for each TRP, FR2). - Oscillator-drift (is the same for one or multiple phase measurement instances for positioning fix): - Ideal: 0 (UE/TRP) - Practical: uniform distribution within [-0.1, 0.1] ppm (UE), [-0.02, +0.02] ppm (each TRP) within measurement duration - Note: The Doppler frequency can be determined based on the UE speed in the evaluation assumption. Agreement In the evaluation of NR carrier phase positioning, the offset between the initial phase of the transmitter and the initial phase of the receiver can be modeled as a random variable uniformly distributed within [0, X]. - Possible values of X: 2pi - Other values FFS Agreement In the evaluation of NR carrier phase positioning, the antenna reference point (ARP) location error of a TRP can be modeled as follows: - Ideal: no ARP error - Practical: a zero-mean, truncated Gaussian distribution with zero mean and standard deviation of T=[1, 5] cm truncated to 2T in each of (x, y, z) direction Agreement In the evaluation of NR carrier phase positioning, the following the UE/TRP antenna phase center offset (PCO) model can be considered as the starting point: dPCO = a * dPhi + w where - a is the scale factor, a=[0, 1, 3] - FFS: other values - dPhi is the direction difference (in degrees): - Example 1, dPhi is the difference between the true and the calculated (or measured) directions between a transmitter (UE/TRP) and a receiver (TRP/UE). - Example 2: dPhi is the direction difference between one UE to two TRPs, or between one TRP to two UEs. - w is 0 or a random variable uniformly distributed within [-2, +2], or [-5, +5], or [-X, +X] degrees - FFS: value of X or left up to companies - Note: the above model is valid only when absolute value of dPhi < Y degrees - FFS: value of Y or left up to companies Agreement For the evaluation of NR carrier phase positioning, UE position can be calculated by the use of the carrier phase measurements obtained at the M sequential time instances, where - Baseline: - M=1 - Optional : - M=4 - Other values of M - Companies should report their assumptions on UE mobility (e.g. speed) Agreement Further evaluate the following multipath mitigation methods for the carrier phase positioning, which include, but are not limited to, the following: - The methods of estimating the carrier phase of the first path - Note: Both time-domain and frequency-domain methods can be considered - LOS/NLOS/ Multi-path indication for the carrier phase measurements for improving the accuracy of the position calculation - Rel-17 LOS/NLOS indicator can be used as the starting point - measurements of the first path and additional paths - E.g. carrier phase measurements, timing measurements - other channel information, such as RSRP/RSRPP, CIR/CFR, etc.

In this contribution, we discuss design aspects for carrier phase-based positioning.

# Phase measurements for SL positioning

In RAN1#109e, whether phase-based positioning for SL positioning should be included in the study was discussed. One of the agreements made in RAN1#109e identifies potential error sources for phase-based positioning, namely, phase noise, Doppler, phase error, antenna location errors. In SL positioning, both target and anchor UE(s) may be mobile and Doppler shift may be included in measurements. In addition, unknown phase offset may be present in Tx/Rx hardware in the UEs. If not treated, these error sources may degrade the quality of phase measurements significantly.

Thus, it may be more fruitful if the solutions for phase-based positioning for Uu link are solidified during Rel. 18 and developed framework is applied to SL positioning in a future release. Thus, the following proposal is made.

Proposal 1: Postpone the study of SL carrier phase positioning to a future release beyond Rel. 18.

# Design aspects for phase-based positioning

## Positioning using phase measurements

In GNSS, carrier phase measurements are used for positioning [2,3]. For phase-based positioning, phase difference-based positioning can be considered. For example, based on phase difference of one PRS, transmitted from a TRP, between two received antennas, distance between UE and TRP can be estimated using antenna geometry [2]. Alternatively, two TRPs can transmit two PRSs, which can be differentiated by scrambling codes or polarization if they are transmitted using the same time-frequency resource, and the UE can measure phase difference between the received PRSs. Thus, the following proposal is made.

Proposal 2: Study transmission and reception modes (e.g., 2 TX 1RX, 1Tx 2 RX) for phase-difference based positioning.

In addition, how phase measurement-based positioning can be configured for the UE should be discussed. In NR, both timing and angle-based positioning have been specified. Phase measurements can serve as assisting measurements which means that phase-based positioning is configured jointly with other positioning methods, e.g., DL-TDOA. Since phase information can be obtained through timing measurements, it is straightforward to associate RSTD and phase measurements. In addition, similar to GNSS, carrier phase only measurements-based positioning should also be supported for scenarios where both TX and RX are calibrated. Thus, the following proposal is made.

Proposal 3: For DL-based positioning, at least the following positioning methods should be supported: NR carrier phase positioning and joint carrier phase and DL-RSTD.

In phase-based positioning, differential or double-differential of phase measurements is used for positioning to eliminate unknown phase offset present in the measurements. For this purpose, for the UE to compute phase difference, transmitted PRS resources targeted for phase-difference based positioning, should share the common or similar unknown phase offset characteristics. Thus, a mechanism to include information indicating that PRS resources share similar unknown phase characteristics should be studied.

One possible approach is to specify a phase error group (PEG). PRS resources in the same PEG may share similar unknown phase offset (e.g., PRSs are transmitted from the same amplifier). Knowing that the PRS resources share similar unknown phase offset characteristics may allow the receiver to implement differential or double-differential phase calculation to mitigate residual phase in the measurements.

Proposal 4:  Study assistance information that allows the UE or network to mitigate unknown phase offset in phase measurements, e.g., phase error group.

## Reference signal design for phase measurements

In GNSS, carrier signals modulated by pseudo noise sequences are used for positioning [3, 6]. In NR, PRS are designed with N-comb patterns and N OFDM symbols are placed in a slot for positioning purposes. In the comb pattern, PRS is not placed continuously in a carrier across OFDM symbols. Thus, consistent carrier phase measurement is not possible.

PTRS is specified in NR for both DL and UL [4]. It is possible to use PTRS for phase measurements. However, since PTRS are multiplexed with data, continuous placement of RS in the time domain is not possible. In addition, configurations of PTRS are associated with DMRS [4] which limits flexibility in PTRS configuration. Thus, there is a need to place PRS in the time domain such that phase of the received signal can be measured accurately. In addition, if more than one PRSs are transmitted at the same time-frequency resource, there should be a mechanism to differentiate the transmitted PRSs by the receiver, i.e., multiplexing via pseudo random code.

Proposal 5: Study new PRS designs for carrier phase measurements for UL and DL.

Proposal 6: A method to differentiate multiple PRSs, if they are transmitted at the same time-frequency resource, should be identified.

## Measurements

Reference for phase measurement and report:

Regarding phase difference measurement and report, the UE can be configured with a reference. In this case the UE does not need to report the reference. However, it is possible that the configured reference PRS is not suitable (e.g., having low RSRP or small LOS/NLOS), which may result in inaccurate phase report of all resources. Since the UE is in charge of phase measurement in DL-PRS resources, the UE may have knowledge of the quality of the phase measurement in each PRS resource. Therefore, it is beneficial to allow the UE to select the reference for phase difference measurement. In addition, to guarantee the quality of the phase reference, the network can configure some conditions to be a phase reference (e.g., based on RSRP, LOS/NLOS).

Proposal 7: Support the UE to select and report the reference for phase difference measurements.

Measurement and report for integer ambiguity

In the last RAN1 meetings, RAN1 agreed to further study the impact and potential solutions to solve the integer ambiguity problem. Specifically, the propagation time between the transmitter and the receiver can be determined as the summation of the number of cycles and the fractional part of a cycle. However, for phase measurement, the UE can measure the fractional part of a cycle only and the number of cycles is not measurable. Precisely determine the number of cycles is crucial for the accuracy of the carrier phase positioning since if the number of cycles is not determined accurately, the carrier phase measurement at the UE, which measures the fractional part of the cycle, is not meaningful.

In addition, in the presence of integer ambiguity [6, 7] due to transmission sources located beyond one cycle of the wavelength of the carrier, the UE can estimate the number of cycles based on other positioning techniques, which can be within or outside 3GPP. Therefore, it is beneficial to allow the UE to report the estimated number of cycles for one PRS resource.

Proposal 8: Support the UE to report the estimated number of cycles of a received carrier.

To solve the integer ambiguity problem, the UE can be configured to measure multiple frequencies in a measurement window. For each frequency of DL-PRS, the UE may acquire one carrier phase measurement (e.g., one fractional part of a cycle). The UE then can determine the propagation time by combining multiple carrier phase measurement results in multiple frequencies of DL-PRS (e.g., by solving a linear programming problem).

Proposal 9: Support the UE to perform PRS measurement in multiple frequencies in one measurement window.

For UE-based carrier phase positioning, the UE can derive the propagation time and the distance to the TRP based on the set carrier phases measured in different frequencies in one measurement window. However, for UE-assisted, LMF needs to derive the location of the UE. Therefore, the phase measurement in multiple frequencies should be reported to the LMF. Specifically, the UE should report the phase measurement of DL-PRS transmitted in multiple frequencies in one measurement report.

Proposal 10: Support the UE to report the phase measurement in multiple frequencies in one measurement reporting.

Measurement and report for multipath or LOS/NLOS conditions:

Phase measurement is very sensitive to multipath and NLOS PRS transmission since a small change in a propagation path may result in a huge different in phase measurement. For a NLOS path, the propagation time can vary based on the reflection path. Therefore, the phase measurement can vary significantly as well. In this case, the phase measurement is not beneficial to determine the position of the UE. Therefore, it is necessary to study in which condition the UE should report phase measurement.

Proposal 11: Study the condition under which carrier phase should be measured and reported based on e.g.,

- LOS/NLOS value

- RSRP

## Use of PRU

During Rel. 17, a use of PRU (Positioning Reference Unit) [5] was discussed such that timing/antenna parameters can be calibrated to eliminate unknow phase/time offset in measurements. In TS 38.305, PRU is defined as a UE with a known location [5]. Using the measurements (e.g., RSTD, phase measurements) returned by PRU, double differential processing can be used to eliminate unknown timing/phase offset. Furthermore, if PRUs are located nearby the UE, it is possible to resolve or eliminate integer ambiguity in the measurement based on the measurements from the PRU. Thus, the following proposal is made.

Proposal 12: Support PRU for NR carrier phase-based positioning.

## UE-based vs. UE-assisted NR carrier phase positioning methods

In UE-assisted NR positioning, the UE makes measurements on the received PRS and reports the measurements to the LMF.  In UE-based NR positioning, the UE makes measurements on the received PRS and determines its position based on the measurements. Similarly, for NR carrier phase positioning, UE-based positioning is feasible if the UE obtains phase measurements from the received PRS. The measurements from PRU can be useful if there are unknown phase offset present/integer ambiguity in the received PRS. Thus, the following proposals are made:

Proposal 13: Support both UE-based and UE-assisted NR carrier phase positioning.

Proposal 14: Study a mechanism to transfer measurements from the PRU to the UE in UE-based positioning.

# Conclusion.

In this contribution, the following proposals are made for positioning based on NR carrier phase measurement:

Proposal 1: Postpone the study of SL carrier phase positioning to a future release beyond Rel. 18.

Proposal 2: Study transmission and reception modes (e.g., 2 TX 1RX, 1Tx 2 RX) for phase-difference based positioning.

Proposal 3: For DL-based positioning, at least the following positioning methods should be supported: NR carrier phase positioning and joint carrier phase and DL-RSTD.

Proposal 4:  Study assistance information that allows the UE or network to mitigate unknown phase offset in phase measurements, e.g., phase error group.

Proposal 5: Study new PRS designs for carrier phase measurements for UL and DL.

Proposal 6: A method to differentiate multiple PRSs, if they are transmitted at the same time-frequency resource, should be identified.

Proposal 7: Support the UE to select and report the reference for phase difference measurements.

Proposal 8: Support the UE to report the estimated number of cycles of a received carrier.

Proposal 9: Support the UE to perform PRS measurement in multiple frequencies in one measurement window.

Proposal 10: Support the UE to report the phase measurement in multiple frequencies in one measurement reporting.

Proposal 11: Study the condition under which carrier phase should be measured and reported based on e.g.,

- LOS/NLOS value

- RSRP

Proposal 12: Support PRU for NR carrier phase-based positioning.

Proposal 13: Support both UE-based and UE-assisted NR carrier phase positioning.

Proposal 14: Study a mechanism to transfer measurements from the PRU to the UE in UE-based positioning.

# Reference

[1] RAN1 Chairman’s note, RAN1#110, August 2022

[2] R. A. Brown, “Instantaneous GPS attitude determination,” IEEE Aerospace and Electronic Systems Magazine, vol. 7, no. 6, June 1992, pp. 3-8,

[3] P. Axelrad, C. J. Comp and P. F. Macdoran, “SNR-based multipath error correction for GPS differential phase,” IEEE Trans. On Aerospace and Electronic Systems, vol. 32, no. 2., April 1996, pp. 650-660.

[4] TS 38.211, “Physical channels and modulation,” ver. 16.5.0, March 2021.

[5] 3GPP TS 38.305, “NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Release 17), v17.0.0, March 2022

[6] F. V. Diggelen, “A-GPS: Assisted GPS, GNSS and SBAS,” Artech House, 2009.

[7] P. Henkel, “Reliable Carrier Phase Positioning”, Ph.D. Dissertation, Technical University of Munich, July 2010