3GPP TSG RAN WG1 Meeting #110bis-e 	 R1-2209518

e-Meeting, October 10th – 19th, 2022

Agenda Item: 9.10.2

Source: MediaTek Inc.

Title: On XR specific capacity improvement enhancements

Document for: Discussion and Decision

Introduction

During RAN1 #109 [1], companies have some discussions on the XR specific capacity improvement enhancements, and the following summary can be drawn from the RAN1 #109e agreements [1]:

- DG/CG/SPS enhancement would be further studied as they are listed in the R18 XR SID

  - Whether/how to support DG/CG/SPS enhancement in R18 XR WI would be further discussed (No RAN1 consensus to support SPS in RAN1 #110 [2])

- The following 3 category of techniques can be further studied

  - Link adaptation enhancement

  - Measure gap enhancement

  - Inter-UE/intra-UE multiplexing enhancement  (Deprioritized in RAN1 #110 [2])

- Common principles for assessment of the candidate capacity enhancement technique are:

  - Identify the XR-specific issue(s) that the enhancement technique is addressing

  - Identify the necessity of the enhancement technique to address the issues

  - Identify whether/how the enhancements provide benefit/performance capacity gain

In this contribution, we further discuss the techniques for XR specific capacity improvement enhancements.

Measurements Gap (MG) enhancement

In 5G NR system, measurement gaps are configured to allow UE to do inter-frequency neighbour cell measurement and the corresponding RF tuning for RRM purposes (e.g. mobility, load balancing, CA set-up). MG is a feature specific to mobile network, which is different from WiFi. In measurement gap, NW cannot schedule UE to transmit/receive data. Possible measurement gap length (MGL) and Measurement gap repetition period (MGRP) values in 5G NR are listed in Figure 1 below.

| Gap Patterns: - MGL: more values to accommodate - Different SMTC durations (1,2,3,4,5 ms) - Different RF switch time for FR1 (0.5ms) and FR2 (0.25ms) Note: GP stands for Gap pattern |  |
| --- | --- |

Figure 1. Possible measurement gap length (MGL) and Measurement gap repetition period (MGRP) values in 5G NR

Taking gap pattern (GP) 0 in Figure 2 below as an example (MGL=6ms, MGRP=40ms), it means that scheduling is excluded for 6ms duration every 40 ms.

Figure 2: An example for measurement gap configuration

This scheduling restriction is critical for efficiently serving XR traffic on NR. We run a system level simulation (SLS) to evaluate the impact of measurement gap as shown in Figure 3 below.

Figure 3: Capacity – max user number with (90% User @ “99% frame Tx done < 10ms”)

As it can be seen from Figure 3,

- the XR DL Capacity falls from 10 (no MG) to less than 2 (MGRP=80,MGL=6) and less than 1 (MGRP=40,MGL=6), if all UEs are configured with MG

- the XR DL Capacity falls from 10 (no MG) to 6 (MGRP=80,MGL=6) and less than 2 (MGRP=40,MGL=6), if only the 20% cell-edge UEs are configured with MG

  - The capacity loss is much more than 20%

Observation 1: In 5G NR system, measurement gaps (MG) are configured to allow UE to do inter-frequency neighbour cell measurement and the corresponding RF tuning for RRM purposes (e.g. mobility, load balancing, CA set-up). MG is a feature specific to mobile network, which is different from WiFi. In measurement gap, NW cannot schedule UE to transmit/receive data.

- System level simulation in Figure 3 shows that

  - XR DL Capacity falls from 10 (no MG) to less than 2 (MGRP=80, MGL=6) and less than 1 (MGRP=40, MGL=6), if all UEs are configured with MG

  - XR DL Capacity falls from 10 (no MG) to 6 (MGRP=80, MGL=6) and less than 2 (MGRP=40, MGL=6), if only the 20% cell-edge UEs are configured with MG

    - The capacity loss is much more than 20%

Therefore, it should be exploited to enhance measurement gap for XR with orchestrated gNB/UE coordination, say more dynamic MG activation/deactivation or period change, as current spec only allows RRC reconfiguration to change the MG settings or enable/disable MG.

Observation 2: It should be exploited to enhance measurement gap for XR with orchestrated gNB/UE coordination, say more dynamic MG activation/deactivation or MG setting (including duration and period) change

- current spec only allows RRC reconfiguration to change the MG settings or enable/disable MG.

Proposal 1: Support a more dynamic DCI/MAC-CE based MG activation/deactivation or MG setting (including duration and period) change for XR capacity enhancement.

At the same time, from NW side, the Rel-16/Rel-17 defined RRM relaxation criteria, and the link condition can be used to determine how to do the MG activation/deactivation.

Observation 3: From NW side, the Rel-16/Rel-17 defined RRM relaxation criteria

  - lowMobilityEvaluation or not-at-cell-edge criteria defined in 5G NR Rel-16

  - stationary or not-at-cell-edge criteria defined in 5G NR Rel-17

and the link condition can be used to determine how to do the MG activation/deactivation.

To demonstrate the achievable capacity gain of Proposal 1, we performed further SLS simulation by assuming NW can dynamically change the MG period using some link condition criteria, and we have the following results in Figure 4:

Figure 4: Capacity – max user number with (90% User @ “99% frame Tx done < 10ms”) with MG enhancement (Case 1 & Case 2)

where case1 and case2 are described below and mobility simulation is based on 30km/h UE speed and FR1 with 200m ISD:

- Case 1: The MG period is adapted to be 2 times when PCell RSRP is 2dB better than the best neighbor cell and 4 times when PCell RSRP is 4dB better than the best neighbour cell

  - Performance impact (handover fail rate): 2.08%  2.81%

- Case 2: The MG period is adapted to be 4 times when PCell RSRP is 2dB better than the best neighbor cell and 8 times when PCell RSRP is 4dB better than the best neighbour cell

  - Performance impact (handover fail rate): 2.08%  4.37%

Observation 4: Assuming NW can dynamically change the MG period using some link condition criteria (Case 1 and Case 2 described below), SLS results (mobility simulation is based on 30km/h UE speed and FR1 with 200m ISD) in Figure 4 show that

- The XR capacity increases from 2 to 6 under Case 1

- The XR capacity increases from 2 to 8 under Case 2

where

- Case 1: The MG period is adapted to be 2 times when PCell RSRP is 2dB better than the best neighbor cell and 4 times when PCell RSRP is 4dB better than the best neighbour cell

  - Performance impact (handover fail rate): 2.08%  2.81%

- Case 2: The MG period is adapted to be 4 times when PCell RSRP is 2dB better than the best neighbor cell and 8 times when PCell RSRP is 4dB better than the best neighbour cell

  - Performance impact (handover fail rate): 2.08%  4.37%

Enhancement of Cross Carrier HARQ Retransmission

For outdoor wide area deployment, TDD patterns favoring UL-heavy or DL-heavy traffic are very common and widely deployed, which leads to extra PDCCH alignment delay when retransmission happens.

Observation 5: For outdoor wide area deployment, TDD patterns favoring UL-heavy or DL-heavy traffic are very common and widely deployed, which leads to extra PDCCH alignment delay when retransmission happens.

One potential latency enhancement for cross-carriers scheduling is HARQ retransmissions across CCs, which will lead to latency enhancement in CA with TDD carriers. Hence, the initial HARQ transmission is sent on one particular CC and the associated HARQ retransmissions can be scheduled and sent on another CC. The cross-carriers scheduling proposals are also applicable for: FR1 vs FR2, Unlicensed vs licenced, FDD vs TDD.

One solution to remove this restriction is to design common HARQ processes pool per cell group or in addition to the HARQ process pools defined per CC.

Another alternative is to exchange information between two HARQ process pools.  Hence, establishing a mapping between the HARQ process for the initial transmission on a specific CC and the HARQ process for the retransmission on another CC.

Proposal 2: Under CA with different TDD patterns, data retransmission can take place on the carrier different from its initial transmission. This can be realized by either

- Designing common HARQ processes pool per cell group or in addition to the HARQ process pools defined per CC, or

- Establishing a mapping between the HARQ process for the initial transmission on a specific CC and the HARQ process for the retransmission on another CC

Previous discussion of soft-ACK reporting using delta-CQI/MCS in Rel-16/17

In the FL summary of XR capacity [3] in RAN1 #110, some companies proposed soft-ACK reporting enhancement using delta-CQI/MCS. This kind of enhancement was also discussed in Rel-16/17, and some concerns were brought up during the discussion [4], including

- How to decide the soft value?

- UE power consumption due to the unneeded 99% of delta-CQI/MCS reports computation

- Impact to the latency and reliability due to the increase in the HARQ codebook size

Observation 6: The soft-ACK reporting enhancement using delta-CQI/MCS was discussed in Rel-16/17 URLLC. Some concerns were brought up including

- How to decide the soft value?

- UE power consumption due to the unneeded 99% of delta-CQI/MCS reports computation

- Impact to the latency and reliability due to the increase in the HARQ codebook size

Proposal 3: The concerns brought up in Rel-16/17 URLLC should be addressed when discussing potential Rel-18 XR capacity enhancement based on soft-ACK reporting enhancement using delta-CQI/MCS.

On enhancement for "one DCI to schedule multiple-slot PDSCH/PUSCH"

In the FL summary of XR capacity [3] in RAN1 #110, some companies proposed enhancements for "one DCI to schedule multiple-slot PDSCH/PUSCH". Since control overhead is not the bottle neck for XR capacity according to R17 study, we suggest to deprioritize enhancement for "one DCI to schedule multiple-slot PDSCH/PUSCH" with similar reason as SPS enhancement.

Proposal 4: Since control overhead is not the bottle neck for XR capacity according to R17 study, we suggest to deprioritize enhancement for "one DCI to schedule multiple-slot PDSCH/PUSCH" with similar reason as SPS enhancement.

Conclusion

In this contribution, we have made the following observations and proposals:

Observation 1: In 5G NR system, measurement gaps (MG) are configured to allow UE to do inter-frequency neighbour cell measurement and the corresponding RF tuning for RRM purposes (e.g. mobility, load balancing, CA set-up). MG is a feature specific to mobile network, which is different from WiFi. In measurement gap, NW cannot schedule UE to transmit/receive data.

- System level simulation in Figure 3 shows that

  - XR DL Capacity falls from 10 (no MG) to less than 2 (MGRP=80, MGL=6) and less than 1 (MGRP=40, MGL=6), if all UEs are configured with MG

  - XR DL Capacity falls from 10 (no MG) to 6 (MGRP=80, MGL=6) and less than 2 (MGRP=40, MGL=6), if only the 20% cell-edge UEs are configured with MG

    - The capacity loss is much more than 20%

Observation 2: It should be exploited to enhance measurement gap for XR with orchestrated gNB/UE coordination, say more dynamic MG activation/deactivation or MG setting (including duration and period) change

- current spec only allows RRC reconfiguration to change the MG settings or enable/disable MG.

Proposal 1: Support a more dynamic DCI/MAC-CE based MG activation/deactivation or MG setting (including duration and period) change for XR capacity enhancement.

Observation 3: From NW side, the Rel-16/Rel-17 defined RRM relaxation criteria

  - lowMobilityEvaluation or not-at-cell-edge criteria defined in 5G NR Rel-16

  - stationary or not-at-cell-edge criteria defined in 5G NR Rel-17

and the link condition can be used to determine how to do the MG activation/deactivation.

Observation 4: Assuming NW can dynamically change the MG period using some link condition criteria (Case 1 and Case 2 described below), SLS results (mobility simulation is based on 30km/h UE speed and FR1 with 200m ISD) in Figure 4 show that

- The XR capacity increases from 2 to 6 under Case 1

- The XR capacity increases from 2 to 8 under Case 2

where

- Case 1: The MG period is adapted to be 2 times when PCell RSRP is 2dB better than the best neighbor cell and 4 times when PCell RSRP is 4dB better than the best neighbour cell

  - Performance impact (handover fail rate): 2.08%  2.81%

- Case 2: The MG period is adapted to be 4 times when PCell RSRP is 2dB better than the best neighbor cell and 8 times when PCell RSRP is 4dB better than the best neighbour cell

  - Performance impact (handover fail rate): 2.08%  4.37%

Observation 5: For outdoor wide area deployment, TDD patterns favoring UL-heavy or DL-heavy traffic are very common and widely deployed, which leads to extra PDCCH alignment delay when retransmission happens.

Proposal 2: Under CA with different TDD patterns, data retransmission can take place on the carrier different from its initial transmission. This can be realized by either

- Designing common HARQ processes pool per cell group or in addition to the HARQ process pools defined per CC, or

- Establishing a mapping between the HARQ process for the initial transmission on a specific CC and the HARQ process for the retransmission on another CC

Observation 6: The soft-ACK reporting enhancement using delta-CQI/MCS was discussed in Rel-16/17 URLLC. Some concerns were brought up including

- How to decide the soft value?

- UE power consumption due to the unneeded 99% of delta-CQI/MCS reports computation

- Impact to the latency and reliability due to the increase in the HARQ codebook size

Proposal 3: The concerns brought up in Rel-16/17 URLLC should be addressed when discussing potential Rel-18 XR capacity enhancement based on soft-ACK reporting enhancement using delta-CQI/MCS.

Proposal 4: Since control overhead is not the bottle neck for XR capacity according to R17 study, we suggest to deprioritize enhancement for "one DCI to schedule multiple-slot PDSCH/PUSCH" with similar reason as SPS enhancement.

References

- Chairman’s Notes, RAN1 #109-e (9.11.2 XR-specific capacity enhancements techniques)

- Chairman’s Notes, RAN1 #110 (9.11.2 XR-specific capacity enhancements techniques)

- R1-2207822, “Moderator Summary#3 – Study on XR Specific Capacity Improvements”, Moderator (Ericsson), RAN1 #110

- RP-211462, “Discussion on CSI feedback enhancements for URLLC/IIoT”, MediaTek Inc., RAN #92-e