3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2209565

# e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.213 | CR | xxxx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on priority value of PSFCH transmission with conflict information for condition 2-A-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For condition 2-A-1 of inter-UE coordination scheme 2, where other UE’s reserved resource(s) identified by UE-A are fully/partially overlapping with resource(s) indicated by UE-B’s SCI in time-and-frequency, the priority value of PSFCH transmission with conflict information is equal to the smallest priority value of the conflicting TBs. For condition 2-A-2 of inter-UE coordination scheme 2, where UE-A, as an intended receiver of UE-B, does not expect to perform sidelink reception from UE-B due to half duplex operation on the resource(s) indicated by UE-B’s SCI, the priority value of PSFCH transmission with conflict information is undefined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For condition 2-A-2 of inter-UE coordination scheme 2, the priority value of PSFCH transmission with conflict information is equal to the priority value of UE-B’s SCI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is unclear for the priority value of PSFCH transmission with conflict information for condition 2-A-2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: No inter-operatability issue is identified. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=================================== Start of Text Proposal =================================

#### 16.2.4.2	Simultaneous PSFCH transmission/reception

For a PSFCH transmission or reception with HARQ-ACK information, a priority value for the PSFCH is equal to the priority value indicated by an SCI format 1-A associated with the PSFCH.

For PSFCH transmission with conflict information, a priority value for the PSFCH is equal to the smallest priority value determined by the corresponding SCI formats 1-A for the conflicting resources, or is equal to the priority value determined by the corresponding SCI format 1-A associated with the resource(s) in slot(s) in which a UE does not expect to perform sidelink reception due to half duplex operation, if the UE is a destination UE of the TB associated with the SCI format 1-A.

For PSFCH reception with conflict information, a priority value for the PSFCH is equal to the priority value determined by the corresponding SCI format 1-A for the conflicting resource.

< Unchanged parts are omitted >

=================================== End of Text Proposal ==================================