3GPP TSG RAN WG1 #110bis-e	                                         	           R1-2209574

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.1.4.2

Source:	Apple Inc.

Title:	Views on SRI/TPMI enhancement for enabling 8 TX UL transmission

Document for:	Discussion/Decision

# Introduction

In Rel-18, a work item was approved for further MIMO enhancements (WID in RP-213598 [1]), and the study and possible specification of 8 Tx UL operation is included as one of the objectives.

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

We made agreements to support 8Tx PUSCH in RAN1#110. In this contribution, we focus on the SRI and TPMI enhancements for enabling 8 Tx UL operation.

# Discussions

## Codebook based transmission

UE antenna configurations and reporting

In RAN1#109-e and RAN1#110, we have the following agreements on antenna configurations:

Agreement

For 8TX UE, consider the following UE antenna layouts for codebook design,

- For non-coherent UEs, consider linear array (1D/2D) of cross-polarized or single-polarized antenna configuration

- For fully/partial-coherent UEs, consider linear array (1D/2D)

  - Where the array is either cross-polarized antenna configuration or single polarized antenna configuration

  - Ng>=1 antenna groups can be considered where each group comprises coherent antennas, and across groups, antennas can be non-coherent/coherent depending on device types

    - An example of an antenna group is a panel

  - Within an antenna group, antenna elements are uniformly spaced. Across different antenna groups, companies to provide details.

Additional information for definition of antenna layout

- Based on the number of coherent groups, following exemplary cases can be considered where, within each group, antenna elements are spaced by 0.5λ, and then dG-H, dG-V represent the horizontal and vertical spacings between the centers of adjacent antenna groups, respectively

  - Further down-selection can be done in the next meeting, if needed

  - The shown exemplary placing of antenna groups can be used for evaluation purpose, but the codebook design is not restricted to shown cases.

  - Other antenna layouts for other use cases are not precluded.

  - To start companies may report their results according to their preferred layout.

| Case | Ng | (M, N, P) per group | Antenna Layout | Antenna Pattern/Antenna Element Gain |
| --- | --- | --- | --- | --- |
| 1 | 1 | (2, 2, 2), (1, 4, 2) |  | Isotropic (Indoor/Outdoor FWA & Industrial) 8 dBi, 65° HPBW(Outdoor FWA) |
| 2 | 2 | (1, 2, 2) |  | Isotropic (Indoor/Outdoor FWA & Industrial) 8 dBi, 65° HPBW(Outdoor FWA) |
| 3 | 4 | (1, 1, 2) |  | Isotropic (Indoor/Outdoor FWA & Industrial) 4 dBi, 110° HPBW(Indoor FWA & Industrial |

Agreement

For 8TX PUSCH, at least support

- Ng=1, 2, 4

Note: The above does not restrict the Ng for the non-coherent case

The targeted antenna configurations are essential for codebook design, and it is critical that the gNB knows what kind of antenna configurations the UE has, so that the proper codebook can be configured for the UE. Therefore, UE capability reporting of full coherency, partial coherency, or non-coherency in antenna configuration is necessary, similar to the legacy systems.

In addition, for full coherency or partial coherency, there can be multiple possible antenna layouts, which also affects the codebook. This should also be reported by the UE.

Proposal 1: For the support of 8 Tx UL with codebook based transmission scheme, UE reports:

- Whether it supports full coherency, partial coherency, or non-coherency in antenna configuration.

- For a full-coherent or partial coherent UE, it further reports the antenna layout.

  - For a full-coherent UE, it reports whether it supports (2, 2, 2) or (4, 1, 2) layout.

    - Note that whether the layout is considered as (4, 1, 2) or (1, 4, 2) is not critical for the UE, because the UE may rotate the direction.

  - For a partial-coherent UE, it reports whether it supports 2 or 4 antenna groups.

At the same time, the gNB may configure which codebook set to use, depending on UE capability. The details can be discussed after the codebook design is done.

SRS Configuration

For SRS enhancements to support 8Tx UL, there are proposals to either extend 1 SRS resource to support 8 ports, or to use 2 SRS resources or resource sets, each with 4 ports. In our view, the main motivation of this objective is to support the simultaneous transmission from all the antenna ports with up to 8 layers. Therefore, the most straightforward extension is to support 1 SRS resource with up to 8 ports. This also provides sufficient differentiation from the simultaneous multi-panel transmission in AI 9.1.4.1 and can largely avoid the overlap of discussions under two AIs.

Proposal 2: For codebook based transmission scheme with 8Tx UL, support 1 SRS resource with up to 8 ports.

For codebook based transmission, the following was agreed in RAN1#109-e/110 for the codebook design:

Agreement

For 8TX UE codebook-based uplink transmission, down-select one of

- Alt1-a:

  - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of the codebook for non-coherent UEs

  - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully/partially-coherent UEs

- Alt1-b:

  - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of the codebook for partially/non-coherent UEs

  - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully-coherent UEs

- Alt2-a:

  - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of codebook for fully/partially/non-coherent UEs

- Alt2-b:

  - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) in combination with those based on NR Rel-15 DL Type I codebooks as the starting point for design of codebook for fully/partially/non-coherent UEs

- Alt3:

  - Study NR Rel-15 DL Type I codebook as the starting point for design of codebook for fully/partially/non-coherent UEs

- Transmission using one or multiple precoders corresponding to one or multiple SRS resources can be studied as part of the above alternatives.

Agreement

RAN1 further studies Alt1b and Alt2a for down-selection of one of the two in RAN1 meeting #110b-e.

- Transmission using one or multiple precoders corresponding to one or multiple SRS resources can be studied as part of the above alternatives.

Full coherent antenna configurations

For full coherent antenna configurations 1-a and 1-b in the agreements, two types of design are being considered: (1) use NR Rel-15 DL Type I codebook as the starting point; (2) use NR Rel-15 UL 2Tx/4Tx codebooks as the starting point. Either way can support up to 8-layer transmission, but the underlying assumptions on the antenna configurations may not be the same.

For example, for 1x4 linear array antenna layout with x-pol (configuration 1-a), Rel-15 DL Type I codebook uses length-4 DFT vectors as the orthogonal basis, which matches the linear array layout with 4 antennas equally spaced. However, if we use the Kronecker product of Rel-15 UL 2Tx and 4Tx codebooks, the orthogonal basis does not match the 4 equally spaced antennas any more.

Therefore, we prefer to use the DL Type I codebook design as the starting point.

If we directly reuse the DL Type I codebook, the DCI overhead can be quite significant. It requires 10 bits (for M=1 and N=4) or 12 bits (for M=N=2) for RI/PMI alone for 8-layer transmission. Certain simplification can be adopted, e.g., by using smaller oversampling factor with O=2 for UL instead of O=4 for DL. In fact, the Rel-15 UL 2Tx/4Tx codebooks are already using O=2 to reduce the DCI overhead, and it makes sense that we follow the same approach for 8Tx codebook design.

Partial coherent antenna configurations

For partial coherent antenna configurations with multiple groups of coherent antennas, the default assumption should be that the UE can transmit simultaneously on the multiple antenna groups (same as in Rel-15). As discussed in RAN1#109-e, it is possible that the multiple antenna groups may be located in different places on the UE and facing different directions, which makes it necessary to have separate precoder indication for the multiple groups.

For the codebook design with partial coherent antenna configurations, since each antenna group has <= 4 Tx, the legacy codebook for 4 Tx UL can be reused for each group. How the layers are split among the groups can be further discussed.

Non-coherent antenna configurations

For non-coherent antenna configurations, it is essentially antenna port selection, and the number of antenna ports selected correspond to the number of layers for transmission. A mechanism that supports an exhaustive list of combinations (e.g. a 8-bit bitmap, or the same mechanism as used for SRI for non-codebook based transmission) can provide full flexibility in antenna port selection to support up to 8 layers. This can be considered as the starting point of discussion, which requires 8 bits in the worst case. If the overhead is considered too high, overhead reduction mechanisms can be further investigated. For example, a reduced set of precoders can be supported for certain ranks in order to reduce the overhead.

In summary, we propose:

Proposal 3: For codebook based transmission scheme with 8Tx, support Alt 1b.

- For full coherent antenna configuration, reuse the Rel-15 DL Type I codebook design for 8 Tx with small oversampling factor (O=2).

- For partial coherent antenna configuration, use the Rel-15 UL 2Tx/4Tx codebooks for the per-antenna-group precoding.

  - FFS how the layers are split among the groups

- For non-coherent antenna configuration, a mechanism that provides full flexibility for the antenna port selection is considered as the starting point for the design.

  - FFS overhead reduction

Full power transmission

It has been agreed to study full power transmission, and the details are to be discussed after we have more clarify on the codebook design. As a high-level principle, we think it is reasonable to assume that UE has all full-rated PAs because 8 UL Tx requires advanced UE and may not have strict requirements on power consumption. This allows us to significantly simplify the design for the full power mode.

Proposal 4: For full power transmission, advanced UEs similar to a UE that supports ul-FullPwrMode-r16 in Rel-16 (with all full-rated PAs) should be assumed to simplify the design.

## Non-codebook based transmission

SRS Configuration

The following was agreed in RAN1#110 regarding the SRS configuration for 8Tx non-codebook based transmission:

Agreement

For SRS configuration for non-codebook UL transmission for an 8TX UE, down-select from

- Alt1: A single SRS resource set configured with up to 8 single-port SRS resources

- Alt2: Up to two SRS resource sets, each configured with up to 4 single-port SRS resources

- Alt3: Support both alternatives.

Agreement

For SRS resource set(s) with usage ‘nonCodebook’ support 8 1-port SRS resources in one or multiple OFDM symbols.

- Note: The maximum number of simultaneous SRS resources is determined via UE-capability signalling.

We think it is straightforward and sufficient to extend 1 SRS resource set to support up to 8 SRS resources, each with a single antenna port.

Proposal 5: For non-codebook based transmission scheme with 8Tx UL, support Alt 1: a single SRS resource set configured with up to 8 single-port SRS resources.

SRI indication

If we would like to provide the full flexibility for the gNB to choose any number of SRS ports (corresponding to the number of layers) out of 8 SRS ports, up to 8 bits are needed for SRI (SRS resource indicator). If the overhead is considered too high, overhead reduction mechanisms can be further investigated. For example, a reduced set of combinations can be supported for certain ranks in order to reduce the overhead.

Proposal 6: For non-codebook based transmission scheme with 8Tx UL, a mechanism that provides full flexibility for the SRI indication is considered as the starting point for the design.

- FFS overhead reduction

## Number of layers

In terms of how many layers to support, the following was agreed in RAN1#110:

Agreement

Support up to X layers for codebook and non-codebook UL transmission for 8TX UE where X=4, 8 is determined based on separate UE capability

- For uplink transmission with rank<=4, single CW is supported

- For uplink transmission with rank>4, whether single or dual CW is used will be decided in RAN1 meeting #110b-e

The above applies only with regards to the work scope of this agenda item.

For the case with rank>4, supporting dual CW may have some performance advantage compared to single CW due to the imbalance among the layers. On the other hand, it may also bring additional complexity at UE side to support larger number of CWs.

Proposal 7: For uplink transmission with rank > 4, single CW is considered as the baseline. Two CWs can be considered only if significant gain is identified.

# Conclusion

In contribution, we have discussed the SRI/TPMI enhancements for 8 Tx UL, and have the following proposals:

Proposal 1: For the support of 8 Tx UL with codebook based transmission scheme, UE reports:

- Whether it supports full coherency, partial coherency, or non-coherency in antenna configuration.

- For a full-coherent or partial coherent UE, it further reports the antenna layout.

  - For a full-coherent UE, it reports whether it supports (2, 2, 2) or (4, 1, 2) layout.

    - Note that whether the layout is considered as (4, 1, 2) or (1, 4, 2) is not critical for the UE, because the UE may rotate the direction.

  - For a partial-coherent UE, it reports whether it supports 2 or 4 antenna groups.

Proposal 2: For codebook based transmission scheme with 8Tx UL, support 1 SRS resource with up to 8 ports.

Proposal 3: For codebook based transmission scheme with 8Tx, support Alt 1b.

- For full coherent antenna configuration, reuse the Rel-15 DL Type I codebook design for 8 Tx with small oversampling factor (O=2).

- For partial coherent antenna configuration, use the Rel-15 UL 2Tx/4Tx codebooks for the per-antenna-group precoding.

  - FFS how the layers are split among the groups

- For non-coherent antenna configuration, a mechanism that provides full flexibility for the antenna port selection is considered as the starting point for the design.

  - FFS overhead reduction

Proposal 4: For full power transmission, advanced UEs similar to a UE that supports ul-FullPwrMode-r16 in Rel-16 (with all full-rated PAs) should be assumed to simplify the design.

Proposal 5: For non-codebook based transmission scheme with 8Tx UL, support Alt 1: a single SRS resource set configured with up to 8 single-port SRS resources.

Proposal 6: For non-codebook based transmission scheme with 8Tx UL, a mechanism that provides full flexibility for the SRI indication is considered as the starting point for the design.

- FFS overhead reduction

Proposal 7: For uplink transmission with rank > 4, single CW is considered as the baseline. Two CWs can be considered only if significant gain is identified.

# Reference

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021