3GPP TSG- RAN WG1 Meeting #110bis-e                                               R1- 2209590

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.5.3

Source:	Apple Inc.

Title:	Discussions on Positioning for RedCap UEs

Document for:	Discussion/Decision

# Introduction

In RAN #94 [1], a SID defining objectives on a Study on expanded and improved NR positioning was agreed on. In the SID, the following justification for sidelink positioning was identified:

- RedCap UEs could support NR positioning functionality but there is a gap in that the core and performance requirements have not been specified for the positioning related measurements performed by RedCap UEs, and no evaluation was performed to see how the reduced capabilities of RedCap UEs might impact eventual position accuracy. This gap is to be investigated by the present SI.

and the following objective was taken:

- Positioning support for RedCap UEs, considering the following:

  - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs[RAN1]

  - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with for RedCap UEs [RAN1, RAN2]

In this contribution, we evaluate the positioning performance of RedCap UEs using DL-TDOA positioning based on an evaluation methodology agreed to in RAN1 #109-e [5] and RAN1 #110 [6], compare the positioning performance to that of a Rel-16 baseline UE and suggest enhancements to RedCap positioning to close the gap in performance between the two UE types.

# Discussion

## RedCap UE overview

Release-17 has studied  and specified support for RedCap UEs with reduced bandwidth support and reduced complexity including reduced number of receive chains [2]. Table 1 below summarizes the device capability  differences between Rel-16 baseline devices and Redcap devices and highlights the differences that will affect the RedCap UE positioning performance for FR1.

Table 1: Rel-16 baseline and Redcap device Capabilities

|  | Rel-16 Baseline Devices | Redcap Devices | Affects positioning performance |
| --- | --- | --- | --- |
| Maximum Bandwidth | 100 MHz | 20 MHz | Yes |
| Number of Rx branches | 2 | 1 or 2 | Yes |
| Maximum Number of DL MIMO layers | 4 | 1 or 2 | No |
| Maximum DL Modulation Order | 256 QAM | 64 QAM 256 QAM (optional) | No |
| Duplex Operations | FD-FDD, TDD | HD-FDD FD-FDD TDD | No |

From the table we see that only the maximum BW and the number of Rx antenna branches will affect the evaluation. In RAN1 #109-e [5] and RAN1 #110 [6], an evaluation methodology was agreed to in which the bandwidth and number of Rx branches for the RedCap UEs were set to the values in the table.

Observation 1: Of the device capability differences between Rel-16 baseline UEs and RedCap UEs, only the maximum bandwidth and the number of Rx antenna branches affect the positioning performance.

## Evaluation Methodology and Parameters

In RAN1 #109-e, the following agreements were made [5]:

From May 20th GTW session Agreement The following scenarios are evaluated for positioning performance of Redcap - Baseline: (Case 1): Umi street canyon, as described in Table 6.1-1-4 of 38.855 - Optional outdoor: - (Case 2): Uma, as described in Table 6.1-1-6 of 38.855 - (Case 3): Rma (FFS details of the scenario) - Baseline: (Case 4): InF-SH as described in Table 6.1-1 of 38.857 - Optional indoor: (Case 5) Indoor Open Office, as described in Table 6.1-1-3 of 38.855 - Optional indoor: (Case 6) InF-DH as described in Table 6.1-1 of 38.857 Agreement The evaluation methodology for RedCap UEs positioning performance uses DL PRS and/or UL SRS for positioning. - The methodology does not define any baseline reference signal configuration. Sources should detail the chosen configuration of reference signal(s) when presenting performance evaluations.

In the following evaluation, we use the commercial use case highlighted in TR 38.857 [4] and evaluate the relative performance for the UMi street canyon scenario (case 1 above) at FR1 with ISD 200 m. For the Rel-16 baseline, we set the maximum BW to 50 MHz and use a PRS configuration of Comb-2 with 6 symbols per base station. The evaluation scenarios and  parameters are shown in Table 2 below and  detailed in the Appendix based on further agreements in RAN1 #109-e [5] and RAN1 #110 [6].

Table 2: evaluation scenarios and parameters

| Parameter | Case 1 (FR1) |
| --- | --- |
| Scenario (baseline, otherwise state any modifications) | Umi |
| Carrier frequency | 3.5 GHz |
| Subcarrier spacing | 30 kHz |
| Reference Signal Transmission Bandwidth | 50 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-2 with 6 symbols per base station |
| Reference signal (type of sequence, number of ports, …) | DL-PRS, single port |
| Number of sites | 57 |
| Number of symbols used per occasion | 6 |
| number of occasions used per positioning estimate | 1 |
| Power-boosting level | N/A |
| Uplink power control (applied/not applied) | N/A |
| interference modelling (ideal muting, or other) | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | N/A |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor’s series |
| Network synchronization assumptions | Ideal |
| UE/gNB RX and TX timing error | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Ideal beam selection |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Ideal beamforming |
| UE antenna configuration | (M, N, P, Mg, Ng) = (1, 1, 1, 1, 1) |
| UE noise figure | 9 dB |
| UE antenna height | 1.5 m |
| gNB antenna height | 10m |
| Additional notes, if any | N/A |

The simulation metric is the horizontal positioning accuracy, defined as the difference between the calculated horizontal position and the actual horizontal position of a UE.

## Evaluation Results

The following figure shows the difference in horizontal positioning accuracy between a RedCap UE and a baseline Rel-16 UE.

Figure 1: Horizontal Accuracy for RedCap UE vs Rel-16 baseline UE

The horizontal accuracy at the 90th percentile for RedCap UEs is as follows:

Table 3: Horizontal Accuracy at 90th percentile

|  | RedCap UE | Rel-16 baseline UE |
| --- | --- | --- |
| Horizontal Accuracy (90%) | 16 m | 8.5m |

As seen, there is a positioning performance loss for this scenario and enhancements are needed to close the gap.

Observation 2: Based on the evaluation assumptions agreed to in RAN1 #109-e, the horizontal accuracy at the 90th percentile for RedCap UEs is worse than that of the Rel-16 baseline UE as follows:

Table 4: Horizontal Accuracy at 90th percentile

|  | RedCap UE | Rel-16 baseline UE |
| --- | --- | --- |
| Horizontal Accuracy (90%) | 16 m | 8.5m |

## Positioning Enhancements for RedCap UEs

### PRS and SRS bandwidth hopping

In RAN1 #110, the following agreement was made [6]:

Agreement The potential benefits and performance gains of frequency hopping of the DL PRS and UL SRS can be investigated in release 18, which may take into account at least the following: - The impact of Doppler, phase offset, timing offset, power imbalance among hops - RedCap UE capability and complexity considerations - Impact of RF retuning during frequency hopping - Details of frequency hopping (including Tx hopping and/or Rx hopping, BWP switching) for the study are FFS

In this enhancement, the effective bandwidth of the PRS/SRS transmission is increased over the physical bandwidth of the RedCap UE by enabling PRS and SRS bandwidth hopping over the larger effective bandwidth.

#### PRS and SRS bandwidth hopping with tone overlap

To limit phase discontinuity in the case the bandwidths are disjoint, the PRS/SRS configuration should allow for overlapping tones. For the downlink, this could be by configuring the PRS over only the bandwidth to be measured by the RedCap UE (as shown in Figure 2 (a))  or over the entire effective bandwidth and the allowing the UE to measure over its actual bandwidth (as shown in Figure 2(b)). Note that updates to the PRS/SRS configuration (including frequency mapping and repetition, measurement gaps and muting patterns) will be needed to accommodate this enhancement.

#### Capability and Complexity Considerations based including BWP switching and retuning

For the UE DL PRS processing capability, the UE reports one combination of (N, T) values per band where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE. The following sets of values for N, T and B are supported

- Values for N = {0.125, 0.25, 0.5, 1, 2, 4, 8, 12, 16, 20, 25, 30, 35, 40, 45, 50} ms

- Values for T = {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms

- Values for maximum DL PRS BW in MHz, reported by UE:

  - For FR1: {5, 10, 20, 40, 50, 80, 100} MHz

  - For FR2: {50, 100, 200, 400} MHz

Note: that the reporting of (N, T) values for maximum BW in MHz is not dependent on SCS.

Given need for BWP switching and/or RF retuning of the PRS/P-SRS as it spans the positioning bandwidth, the existing sets of values may need to be adjusted as the Maximum # of DL PRS resources that UE can process in a slot assumes no BWP switching (DL) or RF retuning. This may need some feedback from RAN4.

Figure 2: PRS Bandwidth Hopping with Tone Overlap for both RedCap and non-RedCap UEs.

Proposal 1: RAN1 should update the  PRS/SRS configuration (including frequency mapping, repetition, measurement gaps and muting patterns) to accommodate PRS Bandwidth Hopping with Tone Overlap.

Proposal 2: RAN1 should update the existing sets of values for the UE DL PRS processing capability as the maximum # of DL PRS resources that UE can process in a slot assumes no BWP switching (DL) or RF retuning. This may need some feedback from RAN4.

### Additional Enhancements

To address the loss in positioning performance, the following enhancements can also be additionally investigated:

Reduced accuracy requirement indication: In this enhancement, the RedCap UE may indicate that it may not require high accuracy positioning. This could be as a UE capability, in which case the LMF may configure the UE to feed back information based on other reference signals to estimate the UE position. Alternatively, it could be based on a maintenance mode (for example the UE is stationary for a certain time duration), in which case the LMF may send an indication for the UE to modify or stop measurement, transmission and feedback of positioning signals and feedback for a desired duration as shown in Figure 3.

Figure 3: Reduced Accuracy Indication due to no movement of the RedCap UE

Group based positioning schemes: In this enhancement, RedCap UEs that are co-located (or in close proximity) to other UEs (e.g. with better positioning capability), may form a group to estimate the UE positions. This could be by jointly estimating the UE position or by delegating the estimation of the position to a “delegate UE” in the group (see Figure 4). Note that there should be an authorization by the UEs to ensure the privacy of the UEs in the group.

Figure 4: Group Based Positioning

Energy Aware Positioning: For some RedCap UEs, energy consumption is extremely important. In this enhancement, to limit energy loss due to positioning, the positioning procedure may account for the DRX cycle so as to (a) ensure that the RS configurations match the DRX ON duration  in the short and long DRX cycle as shown in Figure 5 (b)  the RS measurements occur only during a DRX active time and/or (c) for a DRX inactive time, the RS configurations match the paging cycle.

Figure 5: Energy Aware positioning

Proposal 3: RAN1 should investigate additionally investigate the following enhancements for RedCap UE positioning:

- Reduced accuracy requirement indication: the RedCap UE may indicate that it may not require high accuracy positioning or it is stationary and does not require any position update

- Group based positioning schemes: RedCap UEs that are co-located (or in close proximity) to other UEs (e.g. with better positioning capability), may form a group to estimate the UE positions.

- Energy Aware Positioning: the positioning procedure may account for the DRX cycle so as to (a) ensure that the RS configurations match the DRX ON duration  in the short and long DRX cycle (b)  the RS measurements occur only during a DRX active time and/or (c) for a DRX inactive time, the RS configurations match the paging cycle

# Conclusion

In this contribution, we have evaluated the positioning performance of RedCap UEs using DL-TDOA positioning and have the following observations and proposals:

Observation 1: Of the device capability differences between Rel-16 baseline UEs and RedCap UEs, only the maximum bandwidth and the number of Rx antenna branches affect the positioning performance.

Observation 2: Based on the evaluation assumptions agreed to in RAN1 #109-e, the horizontal accuracy at the 90th percentile for RedCap UEs is worse than that of the Rel-16 baseline UE as follows:

Table 5: Horizontal Accuracy at 90th percentile

|  | RedCap UE | Rel-16 baseline UE |
| --- | --- | --- |
| Horizontal Accuracy (90%) | 16 m | 8.5m |

Proposal 1: RAN1 should update the  PRS/SRS configuration (including frequency mapping, repetition, measurement gaps and muting patterns) to accommodate PRS Bandwidth Hopping with Tone Overlap.

Proposal 2: RAN1 should update the existing sets of values for the UE DL PRS processing capability as the maximum # of DL PRS resources that UE can process in a slot assumes no BWP switching (DL) or RF retuning. This may need some feedback from RAN4.

Proposal 3: RAN1 should investigate additionally investigate the following enhancements for RedCap UE positioning:

- Reduced accuracy requirement indication: the RedCap UE may indicate that it may not require high accuracy positioning or it is stationary and does not require any position update

- Group based positioning schemes: RedCap UEs that are co-located (or in close proximity) to other UEs (e.g. with better positioning capability), may form a group to estimate the UE positions.

- Energy Aware Positioning: the positioning procedure may account for the DRX cycle so as to (a) ensure that the RS configurations match the DRX ON duration  in the short and long DRX cycle (b)  the RS measurements occur only during a DRX active time and/or (c) for a DRX inactive time, the RS configurations match the paging cycle

# Reference

- RP-213588, Revised SID on Study on expanded and improved NR positioning

- TR 38.875, Study on support of reduced capability NR devices, (Release 17)

- 38.855, Study on NR positioning support, (Release 16)

- 38.857, Study on NR Positioning Enhancements; (Release 17)

- Chairman’s Notes, RAN1 #109-e, May 2022.

- Chairman’s Notes, RAN1 #110, August 2022

# Appendix

Table 6-1: Common scenario parameters applicable for all scenarios for Redcap UEs evaluations

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 3.5GHz, 700MHz (optional) Note 1 | 28GHz Note 1 |
| Bandwidth, MHz | TBD | TBD |
| Subcarrier spacing, kHz | 30KHz, 15KHz (for 700MHz carriers) | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns (Optional) |  |
| UE/gNB RX and TX timing error | (Optional) The UE/gNB RX and TX timing error, in FR1/FR2, can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1: - T1: X ns for gNB and Y ns for UE - X and Y are up to sources - Note: RX and TX timing errors are generated per panel independently Apply the timing errors as follows: - For each UE drop, - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*Y,2*Y] and another random sample for the Rx error according to the same [-2*Y,2*Y] distribution. - For each gNB - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*X,2*X] and another random sample for the Rx error according to the same [-2*X,2*X] distribution. - Any additional Time varying aspects of the timing errors, if simulated, can be left up to each company to report. - For UE evaluation assumptions in FR2, it is assumed that the UE can receive or transmit at most from one panel at a time with a panel activation delay of 0ms. |  |
| Note 1: According to TR 38.802 Note 2: According to TR 38.901 |  |  |

Agreement

For the evaluation of RedCap positioning, the following bandwidth can be evaluated:

- FR1: 20MHz baseline, 5MHz optional

- FR2: 100MHz

Agreement (further updated on May 21st as shown in red)

- Adopt the following table for the UE model parameters

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| UE model parameters |  |  |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, for 1Rx UEs: (M, N, P, Mg, Ng) = (1, 1, 1, 1, 1) for 2Rx UEs: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1) | • (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) as minimum antenna configuration (baseline) • (M, N, P, Mg, Ng) = (2, 2, 2, 1, 1) as optional configuration. |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| Number of UE branches | Baseline: 1Rx 1Tx Optional: 2Rx 1 Tx | TBD |
| Note 1: According to 3GPP TR 38.802 |  |  |