3GPP TSG RAN WG1 Meeting #110bis-e	R1-2209647

e-Meeting, October 10th – 19th, 2022

Source: 	National Spectrum Consortium

Title: 	Physical Channel Design Considerations

Agenda item:	9.4.1.2

Document for:	Discussion and Decision

# Background

The WI on NR sidelink evolution [1] includes an objective to study and specify support for sidelink operation in the unlicensed bands with focus on the FR1 unlicensed bands, n46 and n96/n102. The agreements related to SL-U physical channel design at the 3GPP TSG RAN WG1 #110 meeting are listed below [2].

Agreement

For PSCCH and PSSCH in SL-U:

- Both R16/R17 NR SL contiguous RB-based and interlace RB-based transmissions similar to R16 NR-U are supported

Agreement

For PSCCH and PSSCH in SL-U:

- For interlace RB-based transmission

  - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission

  - 1 sub-channel equals K interlace

    - FFS: whether K is fixed as 1 or (pre-)configured

  - Discuss whether one or both of the following alternatives are supported

    - Alt 1: 1 sub-channel is confined within 1 RB set

    - Alt 2: 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool

Agreement

To meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS, FFS details.

Agreement

If RAN1 decides that LBT is performed for S-SSB transmission, in addition to the S-SSB occasions in R16/R17 NR SL design, support additional candidate S-SSB occasions

- FFS the number and locations of additional candidate S-SSB occasions

- FFS when a UE transmits S-SSB on such additional candidate S-SSB occasions, and the related Rx UE’s behavior

Agreement

Regarding PSFCH transmission, at least the followings alternatives can be further studied

- Alt 1: each PSFCH transmission occupies a common interlace and zero or one or more dedicated PRB(s)

- Alt 2: each PSFCH transmission occupies an interlace, and may or may not further apply code domain enhancement (e.g., OCC, PRB-level cyclic shifts)

- Alt 3: each PSFCH transmission occupies some dedicated PRBs and some common PRBs

- FFS details of above alternatives

Agreement

If RAN1 decides that LBT is performed for PSFCH transmission, for the time and frequency domain locations of PSFCH resources, at least the followings alternatives can be further studied

- Alt 1: PSFCH resources are (pre-)configured

- Alt 2: PSFCH resources are dynamically indicated

- Combination of above alternatives are not precluded

- FFS details of above alternatives

Agreement

For S-SSB and synchronization in SL-U:

- No changes on R16 NR SL S-PSS/S-SSS sequence generation

- Continue studying the 4 options from the previous agreement and whether/how temporary exemption of OCB requirement is applicable for S-SSB transmission, e.g., how to meet the minimum of 2 MHz requirement under 15 kHz SCS

Agreement

For PSCCH and PSSCH resource indication in time/frequency domain:

- For time domain: R16 NR SL TRIV is reused as baseline

- For frequency domain:

  - further study sub-channel indexing and resource indication

- FFS: whether any enhancement needed on R16 NR SL TRIV/FRIV if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission

In this contribution we present our views on open and unaddressed issues on this topic.

# Channel Design

## PSSCH, PSCCH, PSFCH

We first consider interlace design for SL-U PSSCH and PSCCH. Legacy frequency-domain resource allocation for PSSCH is in terms of sub-channels comprising contiguous PRBs; the sub-channel assignment for sidelink transmission is determined using the "Frequency resource assignment" field in the associated SCI. The lowest sub-channel for sidelink transmission is the sub-channel on which the lowest PRB of the associated PSCCH is transmitted. The maximum number of Rel-16 sub-channels configurable per resource pool is 27, while each sub-channel is configured to comprise 10, 12, 15, 20, 25, 50, 75, or 100 PRBs. On the other hand, in NR-U/SL-U there are either 10 or 5 interlaces with 15 kHz and 30 kHz SCS, respectively, and one interlace includes 10/11 PRBs per 20 MHz.

The current agreement is that 1 sub-channel equals K interlaces, where it remains to decide if K is one or greater than one. If K is fixed to one, then one sub-channel spans 10/11 PRBs, which covers the minimum PSCCH allocation from Rel-16. Coding rate issues are not anticipated with K = 1 since a UE PSSCH can be assigned multiple sub-channels (equivalent to interlaces) from two to ten (all 106 or 51 PRBs). Next, one sub-channel need not be restricted to one RB set since NR-U interlaces can span adjacent RB sets during wideband transmission. A remaining FFS point for interlace-based transmission from RAN1#109-e is whether/how to address IBE (In Band Emission) impact, this requires inputs from RAN4.

Proposal 1: For PSCCH and PSSCH in SL-U:

- For interlace RB-based transmission, K is fixed to one.

  - Support Alt 2: 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool.

Next, we consider PSFCH resource allocation if LBT is performed for PSFCH transmission. Our first preference is for PSFCH to always be transmitted without LBT, especially in COT-sharing situations. In the event that LBT is needed, RAN1 should first design the simpler case of preconfigured resources for PSFCH – the design objective should be fast HARQ feedback in response to a PSSCH transmission. Introducing a second option for dynamically-indicated PSFCH resource is an unnecessary optimization given the scale of the overall WI.

Proposal 2: If RAN1 decides that LBT is performed for PSFCH transmission, for the time and frequency domain locations of PSFCH resources, support only Alt 1: PSFCH resources are (pre-)configured.

## Slot Structure

It has been agreed that at least R16/R17 NR SL slot-based PSCCH/PSSCH transmission is supported and FFS: whether/how to support additional starting symbol(s) within a slot for the PSCCH/PSSCH transmission. Supporting additional starting symbols is essential to bring parity between SL-U and NR-U in terms of channel access agility. While NR-U can exploit multiple non-slot durations to initiate a COT, as a compromise SL-U can be restricted to two or three starting points for simplicity.

Proposal 3: There are two starting symbols within a slot for the PSCCH/PSSCH transmission

- 1st starting symbol index is 0, and 2nd starting symbol index is (pre-)configured

- When a UE starts transmission from either the 1st or 2nd starting symbol, the transmission occupies all the remaining symbols available for PSSCH in that slot.

## S-SSB

The support of additional candidate S-SSB occasions was agreed in RAN1 #110 if LBT is performed for S-SSB. The legacy S-SSB is 13 symbols in duration for normal CP and 12 symbols for ECP. Within a S-SSB period of 16 frames, 1, 2, 4, 8, 16, 32, or 64 S-SSB transmissions can currently be indicated to the UE. Thus, there is ample white space within a S-SSB period to introduce additional S-SSB occasions. We suggest allowing the transmission of a S-SSB that was blocked due to LBT failure to occur in the following slot, i.e., two transmission attempts per S-SSB and up to a maximum of 128 S-SSB candidate locations within a S-SSB period. At the same time, further analysis of shorter 4-symbol S-SSBs should continue.

Proposal 4: For additional S-SSB candidate positions under LBT, allow the transmission of a legacy-length S-SSB that was blocked due to LBT failure to occur in the following slot, i.e., two transmission attempts per S-SSB and up to a maximum of 128 S-SSB candidate locations within a S-SSB period.

Next, we examine frequency-domain allocation. Legacy S-SSB spans 132 subcarriers, which equates to 1.98 MHz and 3.96 MHz for 15 kHz and 30 kHz SCS, respectively. From RAN1#109-e we have:

- Down-selection at least one of the following solutions to meet OCB and PSD requirement for S-SSB transmission

  - Option 1: Using interlaced RB transmission

  - Option 2: S-SSB multiplexing with other SL transmissions in the same slot

  - Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain

  - Option 4: S-PSS/S-SSS/PSBCH with wider bandwidth

Option 2 is a high-priority approach that is similar to the DRS multiplexing rules defined for NR-U. Option 1 is not preferred due to the need for link-level performance analysis, and there is no precedent for interlaced SSB in NR-U. Options 3 and 4 were also considered for NR-U but ultimately not supported with a tacit understanding that the temporary OBW relaxation allowance and multiplexing by scheduling are sufficient.

Proposal 5: To meet OCB and PSD requirement for S-SSB transmission, support only Option 2: S-SSB multiplexing with other SL transmissions in the same slot.

# Summary of Proposals

This contribution puts forth the following proposals for SL-U physical channel design.

Proposal 1: For PSCCH and PSSCH in SL-U:

- For interlace RB-based transmission, K is fixed to one.

  - Support Alt 2: 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool.

Proposal 2: If RAN1 decides that LBT is performed for PSFCH transmission, for the time and frequency domain locations of PSFCH resources, support only Alt 1: PSFCH resources are (pre-)configured.

Proposal 3: There are two starting symbols within a slot for the PSCCH/PSSCH transmission

- 1st starting symbol index is 0, and 2nd starting symbol index is (pre-)configured

- When a UE starts transmission from either the 1st or 2nd starting symbol, the transmission occupies all the remaining symbols available for PSSCH in that slot.

Proposal 4: For additional S-SSB candidate positions under LBT, allow the transmission of a legacy-length S-SSB that was blocked due to LBT failure to occur in the following slot, i.e., two transmission attempts per S-SSB and up to a maximum of 128 S-SSB candidate locations within a S-SSB period.

Proposal 5: To meet OCB and PSD requirement for S-SSB transmission, support only Option 2: S-SSB multiplexing with other SL transmissions in the same slot.

# References

- 3GPP RP-221938: "WID revision: NR sidelink evolution ".

- Chairman’s Notes, RAN WG1 #110