3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2209656

E-meeting, October 10th – 19th, 2022

Agenda Item:	9.11.1

Source:	Ericsson

Title:	On coverage enhancements for NR NTN

Document for:	Discussion

# 1	Introduction

A work item (WI) for enhancements of NR NTN was approved at RAN#94-e. The latest WID can be found in [1]. One objective of the WI is NTN-specific coverage enhancements. As a first step, a 6-month study phase has been pursued, focusing on evaluating the coverage performance and identifying the candidate physical radio channels that have coverage issues specific to NTN. Based on the outcome, RAN#97 agreed to study and specify support for some UL enhancements. In this contribution, we evaluate the performance of PUCCH repetitions for Msg.4 HARQ-ACK and segmented DMRS bundling for PUSCH.

# 2	Discussion

## 2.1	Summary of RAN#97-e

At RAN#97-e, the NTN enhancements WID [1] was updated with the following detailed objectives for UL coverage enhancements:

- To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g. repetition) [RAN1, RAN4]

- To study DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) and, if necessary, specify enhancements to the Rel-17 procedures [RAN1]

Further, a reference scenario was agreed for evaluation of UL coverage enhancements:

The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarisation loss (per antenna port).

Note: It is understood that the enhancements defined for LEO can also apply to GEO and MEO scenarios as appropriate. No additional work is expected for MEO/GEO.

The targeted services are VoIP using AMR 4.75 kbps and data transmission services with Low data rate of 3 kbps.

RAN#97-e also decided to postpone the discussion of PRACH enhancements, DL channel enhancements and High power UE for NTN FDD FR1 bands until RAN#99:

conclusion:

To be discussed at RAN#99,

- whether the Rel-18 PRACH enhancements (defined as part of Rel-18 NR coverage enhancements WI) are sufficient to address the coverage gaps with respect to NTN specifics (i.e. agreed at RAN1#109-e/RAN1#110, under NR_NTN_enh WI) and whether additional PRACH enhancements need to be specified;

- DL channel enhancements for NTN coverage (including the need) ;

- Decision to specify High power UE (e.g. 26 dBm Tx power) for NTN FDD FR1 band(s), i.e. Rx/Tx requirements (This includes the study of relevant coexistence scenarios) [RAN4].

Regarding the PFD limitation issue, it was stated in the WID that no work on this topic will take place in the WGs before RAN#99:

The following sentence will be revisited in RAN#99 as part of the DL enhancements discussion:

“The evaluation should also take into account any related regulatory requirements, e.g., ITU limitation of power flux density.” No work on this topic will take place in RAN WGs before the discussion on DL enhancements in RAN#99.

## 2.2	Uplink

### 2.2.1	Link budget calculations

A link budget analysis for the reference scenario agreed by RAN#97-e is shown in Table 1. The assumptions are the same as in [6] except that the UE antenna gain is assumed to be -5.5 dB, following RAN4 recommendations [7].

Table 1: Link budget analysis for LEO1200 uplink with Set-1 parameters and 30° elevation angle.

| TX EIRP [dBm] |  | 17.5 |
| --- | --- | --- |
| RX G/T [dB/K] |  | 1.1 |
| Free space path loss [dB] |  | 164.5 |
| Atmospheric loss |  | 0.066 |
| Shadow fading margin [dB] |  | 3 |
| Scintillation loss [dB] |  | 2.2 |
| Polarization loss [dB] |  | 3 |
| Additional losses [dB] |  | 0 |
| Target SNR [dB] | 1 PRB | -8.1 |
|  | 2 PRBs | -11.1 |

### 2.2.2	PUCCH for Msg.4 HARQ-ACK

The main reason why the performance of PUCCH for Msg.4 HARQ-ACK is insufficient is that PUCCH repetitions are not supported for this case in the current specification. The most straightforward enhancement is therefore to add support for repetitions.

Simulations of PUCCH for Msg.4 HARQ-ACK repetitions have been performed. Simulation assumptions are according to agreements from RAN1#109-e. Specific simulation assumptions are listed in Table 2.

Table 2: Assumptions for PUCCH simulations.

| Parameter | Value |
| --- | --- |
| Antenna configuration | 1T2R |
| Modulation | BPSK |
| Channel model | NTN-TDL-C Rural |
| Elevation Angle | 30° |
| Frequency hopping / FH offset | Yes / 50 PRBs |
| Channel bandwidth | 1 PRB |
| PUCCH format | format 1 |
| Payload | 1 bit |
| PUCCH duration | 14 OS |
| Repetitions | No/2/4/8 repetitions |
| Performance metric | 1% ACK missed and 0.1% NACK to ACK at 1% DTX to ACK rate |

Simulation results are summarized in Table 3. Detailed results can be found in Appendix C. It can be seen that already with two repetitions, the performance target is met.

Table 3: Summary of performance for PUCCH for Msg.4 HARQ-ACK.

|  | LLS results ACK missed [dB] |  |  |  |
| --- | --- | --- | --- | --- |
| Target CNR [dB] | 8 repetitions | 4 repetitions | 2 repetitions | no repetitions |
| -8.1 | -12.1 | -10.3 | -8.3 | -6.7 |

The CNR target of PUCCH for Msg.4 HARQ-ACK can be reached with 2 repetitions.

Specify support for repetitions of PUCCH for Msg.4 HARQ-ACK.

### 2.2.3	PUSCH for VoIP

RAN#97-e agreed that DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) should be studied. When DMRS bundling is used, the UE needs to transmit coherently across bundled UL slots. This means that uncoordinated updates of the TA should be avoided between the slots. However, TA updates due to open-loop pre-compensation are necessary to maintain accurate timing of the received signal at gNB.

A potential solution is to use segmented DMRS bundling, i.e., define segments of the transmission during which the TA is not updated, similar to the "segmented UL transmissions" for IoT NTN. Only between the segments, the UE may update its TA. This enables DMRS bundling within each segment while slots from different segments can use regular non-bundled repetitions.

If the UE is synchronized to the DL signal, Doppler shifts in both DL and UL will contribute to timing drift in the signal gNB receives. The maximum Doppler shift due to satellite movement is 21 ppm for a LEO1200 satellite [5]. This applies for both the service link and the feeder link and in both UL and DL. Therefore, the total timing drift can be up to 84 ppm.

In case of LEO1200, the timing drift can be up to 84 ppm.

The timing accuracy requirement for NTN UE is 29Ts (assuming 15 kHz SCS) (see clause 7.1c.2 of TS 38.133 [2]). Not all of this timing error budget can be used for managing the timing drift due to paused updating of the open-loop TA, since there are also other sources of timing error (e.g., UE position inaccuracy, satellite ephemeris/propagation inaccuracy, TAC granularity, TA adjustment inaccuracy, DL synchronization inaccuracy and other UE-internal sources of error). In other words, there is likely a timing error already at the start of a transmission of DMRS bundled slots. However, for VoIP in LEO1200, the regular UL transmissions and the moderate RTT would likely make closed-loop TAC efficient. Therefore, it can be assumed that at least part of the timing error can be cancelled by TAC, so that the timing error at the start of a transmission of bundled slots is mainly due to TAC granularity and UE TA adjustment inaccuracy. For 15 kHz SCS, the granularity of TAC is 8Ts (see clause 4.3 of TS 38.213 [3]), which means that the timing error could ideally be cancelled to within ±4 Ts. The tolerance of TA updates is 4Ts (see clause 7.3C.2.2 of TS 38.133 [2]). Therefore, (29-4-4)Ts = 0.684 µs can be assumed to be available to handle the timing drift. With a timing drift of 84 ppm, the timing error will exceed 0.684 µs in 0.684/84 s = 8.1 ms. This means that DMRS bundling can be performed only within segments of 8.1 ms, or 8 slots in case of 15 kHz SCS. Considering suboptimal closed-loop TA control, shorter DMRS bundling segments may be needed.

With a timing drift of 84 ppm and an optimistic assumption on the efficiency of closed-loop TA control, the UE has to update its autonomous TA pre-compensation every 8 slots to avoid excessive timing offset. With a less optimistic assumption on the efficiency of closed-loop TA control, more frequent TA updates are needed.

Simulations of segmented DMRS bundling have been performed. Simulation assumptions are according to agreements from RAN1#109-e. Specific simulation assumptions are listed in Table 4. It has been assumed that the segment length of DMRS bundling should be a divisor of the number of repetitions, i.e., 20.

Table 4: Assumptions for PUSCH simulations of VoIP.

| Parameter | Value |
| --- | --- |
| Antenna configuration | 1T2R |
| Modulation order | QPSK |
| Channel Model | NTN-TDL-C Rural |
| Elevation Angle | 30° |
| Frequency hopping | No |
| Channel bandwidth | 1 PRB, 2 PRBs |
| DMRS configuration | Type I, 1 or 2 DMRS symbols, no multiplexing with data. |
| PUSCH duration | 14 OS |
| Repetitions | 20 slots w/ type A repetition |
| DMRS bundling segments | 1x20 (no DMRS bundling) 2x10 4x5 5x4 7+7+6 8+8+4 10x2 20x1 (full DMRS bundling) |
| HARQ configuration | No |
| TBS/MCS | MCSs from Table 6.1.4.1-1 of 38.214 [4] are used. The MCS depends on the number of PRBs and DMRS symbols: 1 PRB, 1 DMRS symbol: MCS=9 (TBS=208) 1 PRB, 2 DMRS symbols: MCS=9 (TBS=184) 2 PRBs, 1 DMRS symbol: MCS-4 (TBS=184) 2 PRBs, 2 DMRS symbols: MCS=5 (TBS=208) |
| Performance metric | 2% BLER |

Simulation results are summarized in Table 5. Detailed results can be found in Appendix C. It can be seen that it is possible to meet the target CNR when the segment length for DMRS bundling is limited to 7 slots.

Table 5: PUSCH BLER performance for VoIP (AMR 4.75) with segmented DMRS bundling.

| PRBs | DMRS | Target CNR [dB] | Simulated SNR at 2% BLER for different DMRS bundling configurations |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 20x1 (full bundling) | 10x2 | 8+8+4 | 7+7+6 | 5x4 | 4x5 | 2x10 |
| 1 | 1 | -8.1 | -7.4 | -7.0 | -6.6 | -6.7 | -6.3 | -6.1 | -4.3 |
| 1 | 2 | -8.1 | -9.1 | -8.5 | -8.1 | -8.1 | -7.8 | -7.6 | -6.6 |
| 2 | 1 | -11.1 | -10.4 | -9.8 | -9.4 | -9.4 | -9.2 | -8.9 | -7.8 |
| 2 | 2 | -11.1 | -11.7 | -10.8 | -10.5 | -10.4 | -10.2 | -9.9 | -9.1 |

It is possible to meet the target CNR for PUSCH VoIP with segmented DMRS bundling when the segment length is limited to 7 slots.

Specify support for segmented DMRS bundling for PUSCH.

## 2.3	Downlink

Since RAN#97-e concluded that no work on PFD limitation will take place in the WGs before RAN#99, and the simulation results from RAN1#110 show that DL performance is sufficient without PFD limitation, there is no need to discuss DL enhancements in RAN1 until after RAN#99.

Postpone RAN1 discussions of DL enhancements until after RAN#99.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The CNR target of PUCCH for Msg.4 HARQ-ACK can be reached with 2 repetitions.

Observation 2	In case of LEO1200, the timing drift can be up to 84 ppm.

Observation 3	With a timing drift of 84 ppm and an optimistic assumption on the efficiency of closed-loop TA control, the UE has to update its autonomous TA pre-compensation every 8 slots to avoid excessive timing offset. With a less optimistic assumption on the efficiency of closed-loop TA control, more frequent TA updates are needed.

Observation 4	It is possible to meet the target CNR for PUSCH VoIP with segmented DMRS bundling when the segment length is limited to 7 slots.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Specify support for repetitions of PUCCH for Msg.4 HARQ-ACK.

Proposal 2	Specify support for segmented DMRS bundling for PUSCH.

Proposal 3	Postpone RAN1 discussions of DL enhancements until after RAN#99.

# References

RP-222654, "NR NTN (Non-Terrestrial Networks) enhancements", Thales, RAN#97, September 2022.

3GPP TS 38.133, "NR; Requirements for support of radio resource management ", v17.6.0, June 2022

3GPP TS 38.213, "NR; Physical layer procedures for control ", v17.3.0, September 2022

3GPP TS 38.214, "NR; Physical layer procedures for data", v17.3.0, September 2022.

3GPP TR 38.821, "Solutions for NR to support Non-Terrestrial Networks (NTN)", v16.1.0 June 2021.

R1-2207762, "On coverage enhancements for NR NTN ", Ericsson, RAN1#110, August 2022.

R4-2214968, "LS Reply on UE antenna gain for NR NTN coverage enhancement", RAN WG4, RAN4#104-e, August 2022

# Appendix A	Agreements from RAN1#109-e

Agreement

For NR NTN coverage enhancement, evaluate only handset terminals as UE type.

- i.e., VSAT is not considered.

Agreement

Coverage performance in NR NTN is evaluated according to the following steps.

- Step 1: CNR is calculated as defined in 6.1.3.1 of TR38.821

  - For polarization loss,

    - 3 dB polarization loss is assumed as baseline, and companies are encouraged to report the value and corresponding justification if other value is used

- Step 2: Required SNR of target service is evaluated by LLS

- Step 3: The CNR and the required SNR are compared

Agreement

Coverage performance in NR NTN is evaluated for GEO/LEO-1200/LEO-600 scenarios.

- Note: Service type for each scenario is discussed separately

- Note: Parameter set (Set-1/2) is discussed separately

- Note: MEO can be evaluated optionally

Agreement

For evaluation of coverage performance in NR NTN,

- It is assumed that carrier bandwidth is sufficiently large to transmit each channel.

- Companies are encouraged to report BWP bandwidth, when necessary (e.g. for frequency hopping).

- Note: each channel bandwidth is discussed separately.

Agreement

For VoIP, AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) with 20 ms data arriving interval is used in the evaluations.

- Each packet is transmitted within 20 ms, if packet combining is not used.

  - Companies are encouraged to evaluate at least packet transmission without combining

  - Companies are encouraged to report how to apply packet combining, if used.

  - Note: in packet combining, two packets can be combined into a single packet at TX side

    - Companies should report the impact on E2E latency

- VoIP is evaluated only in LEO scenario.

- Note 1: PRB/MCS/TBS determinations are discussed separately

- Note 2: companies should report if HARQ is used in the evaluations, and if evaluations depart from the assumption that each packet is transmitted within 20 ms

Agreement

Reuse Set-1/2 satellite parameters as in table 6.1.1.1-1/2 of TR38.821 for GEO/LEO-1200/LEO-600 and S-band, and as in table 6.1.1.1-1/2 of RP-220590 for MEO and S-band.

- In addition, evaluations assuming relevant ITU regulatory limitations on power flux density can be reported in the study phase.

  - Companies should report which value of EIRP density is used and corresponding justification.

Agreement

For link budget calculation, parameters in the following table is assumed.

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2 GHz for DL and UL (S-band) |
| Channel bandwidth | FFS |
| Satellite altitude | 600 km, 1200 km, 10000 km, 35786 km |
| Target elevation angle | [30° (LEO), 12.5° (GEO-Set 1) , 20° (GEO –Set 2), 30° (MEO)] |
| Atmospheric loss | Equation (6.6-8) in [2] |
| Shadowing margin | 3 dB |
| Scintillation loss | Section 6.6.6 in [2] Ionospheric loss: = 2.2 dB (note 1) Tropospheric loss: Table 6.6.6.2.1-1 of [2] |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Satellite antenna polarization | Circular polarization |
| Terminal type | [S band: (M, N, P) = (1,1,2)] |
| Free space path loss | Equation (6.6-2) in [2] |
| Terminal RF parameters | FFS |
| Satellite RF parameters | FFS |
| Polarization loss | As agreed separately |
| Outcome | CNR |
| - NOTE 1: Based on P3 curve for 1% of time from Figure 6.6.6.1.4-1 of [2] after frequency scaling. - NOTE 2: [2] in this table is 3GPP TR 38.811 v15.2.0: "Study on New Radio (NR) to support non-terrestrial networks (Release 15)" |  |

Agreement

If corresponding channel (including SCS) is agreed as evaluation target channel, the following features introduced in Rel-17 Coverage enhancement WI can be applied in coverage evaluation of NR NTN.

- For VoIP, max 20 PUSCH repetitions if SCS = 15 kHz and packet combining/HARQ are not applied; otherwise, max 32 PUSCH repetitions with consideration of the impact on E2E latency

- For low-data rate service, max 32 PUSCH repetitions

- TBoMS

- Joint channel estimation (DMRS bundling)

  - Companies are encouraged to report how to apply

- Max 16 Msg.3 PUSCH repetitions

Agreement

For low-data rate service, the following target data rate is assumed.

- For DL, 3 kbps if satellite EIRP density lower than values in table 6.1.1.1-1/2 of TR38.821 for GEO/LEO-1200/LEO-600 and S-band, or values in table 6.1.1.1-1/2 of RP-220590 for MEO and S-band due to ITU regulatory limitations on power flux density is considered; otherwise, 1 Mbps

- For UL, 3 kbps and 100 kbps

  - FFS: which data rate applies for GEO/MEO/LEO

Agreement

For NR NTN coverage enhancement, the following channels/signals can be evaluated.

- PUSCH for VoIP

- PUSCH for low data rate service

- PUCCH format 1 with 2 bits

- PUCCH format 3 with 11 bits

- PRACH format 0

- PRACH format 2

- PRACH format B4

- PUSCH Msg.3

- PUCCH for Msg.4 HARQ-ACK

- SSB

- PDSCH for VoIP

- PDSCH for low data rate service

- PDSCH Msg.2

- PDSCH Msg.4

- PDCCH

- Broadcast PDCCH (PDCCH of Msg.2)

Agreement

Evaluate coverage performance for the following UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization, Tx/Rx antenna gain, and antenna type and configuration.

| Characteristics | Handheld |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | 1 TX, 2TX (optional) / 2 RX with omni-directional antenna element Note: companies should provide their assumption on polarization |
| Polarisation | Linear |
| Rx Antenna gain | [X] dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | [X] dBi per element |

- X = -5 as working assumption

  - Send an LS to RAN4 to ask whether above antenna gain is valid and if invalid, appropriate value.

Agreement

For coverage performance evaluation, the following elevation angle is assumed.

- 30 deg for LEO, 12.5 deg for GEO-Set 1, 20 deg for GEO-Set 2, as in in Table 6.1.3.2-1 of TR38.821

  - Note: For GEO-Set 1, channel parameters for 10 deg is used in LLS.

- 30 deg for MEO

- Other elevation angles can be evaluated as optional

- Note: these values are elevation angles at the edge of the edge beam.

Agreement

For NR NTN coverage enhancement, evaluate the following cases.

| Case | Satellite orbit | Satellite parameter set | Elevation angle (deg) | Terminal | Frequency band | Service type |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | GEO | 1 | 12.5 | Handset | S-band | Low-data rate service |
| 2 | GEO | 2 | 20 | Handset | S-band | Low-data rate service |
| 3 (Optional) | LEO-1200 | 1 | 30 | Handset | S-band | VoIP |
| 4 | LEO-1200 | 2 | 30 | Handset | S-band | VoIP |
| 5 | LEO-1200 | 2 | 30 | Handset | S-band | Low-data rate service |
| 6 (Optional) | LEO-600 | 1 | 30 | Handset | S-band | VoIP |
| 7 | LEO-600 | 2 | 30 | Handset | S-band | VoIP |
| 8 (Optional) | LEO-600 | 2 | 30 | Handset | S-band | Low-data rate service |
| 9 (Optional, with higher priority than case 10) | MEO | 1 | 30 | Handset | S-band | Low-data rate service |
| 10 (Optional) | MEO | 2 | 30 | Handset | S-band | Low-data rate service |

Agreement

For coverage performance evaluation, the following are assumed for all channels/signals

- Channel model/Delay spread

  - Channel model as in Table 6.1.2-4 of TR38.821, assuming NTN-TDL-A (NLOS) and NTN-TDL-C (LOS)

- Evaluation scenario

  - Rural (LOS/NLOS)

  - Sub-urban (LOS/NLOS) (optional)

- Channel estimation: Realistic estimation

  - Companies are encouraged to report channel estimation method.

- SCS

  - 15 kHz only

- UE speed: 3 km/h

- Frequency drift: Not assumed

- Frequency offset: 0.1 ppm

Agreement

For coverage evaluation of PUSCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains | 1, 2 (optional) |
| DMRS configuration | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| Waveform | DFT-s-OFDM, CP-OFDM (optional) |
| PUSCH duration | 14 OS |
| Repetitions | w/ type A repetition, optional for type B repetition. The actual number of repetitions is reported by companies. |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| PRBs/TBS/MCS for low data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK, pi/2 BPSK (optional) |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PUCCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 1, 2bits UCI. Format 3, 11 bits UCI |
| Frequency hopping | w/ frequency hopping |
| BLER | - For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. - For PUCCH format 3: BLER for Ack/Nack, SR: 1% BLER for CSI: 1%, optional for 10%. |
| Number of UE transmit chains | 1 |
| DMRS configuration | Number of DMRS symbols for PUCCH Format 3: Reported by companies |
| Repetitions | w/ repetition. The maximum number of repetitions is 8. |
| PUCCH duration | 14 OS |
| Number of PRBs | 1 PRB |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PRACH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Format | Format 0, Format B4, Format 2 |
| SCS | Reported by companies. |
| Performance metric | 1% missed detection at 0.1% false alarm probability 10% missed detection: reported by companies if this value is used |
| Number of UE transmit chains | 1, 2 (optional) |
| Other parameters | Reported by companies. |

Agreement

For coverage evaluation of PUSCH Msg.3 in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| Number of UE transmit chains | 1, 2 (optional) |
| Number of DMRS symbol | w/o frequency hopping: 3, w/ frequency hopping: 2 for each hop |
| Waveform | DFT-s-OFDM |
| HARQ configuration | Whether/How is adopted is reported by companies. |
| PUSCH duration | 14 OS |
| Number of PRBs | 2 |
| TBS | 56 bits |
| Other parameters | Reported by companies. |

Agreement

For coverage evaluation of SSB in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Periodicity | 20ms |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |
| Other parameters | Reported by companies. |

Agreement

For coverage evaluation of PDSCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Waveform | CP-OFDM |
| Number of UE receive chains | 2 for 2GHz |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| DMRS configuration | 3 DMRS symbols is used for PDSCH of Msg.2. For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| PRBs/TBS/MCS for low data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK |
| PDSCH duration | 12 OS |
| Payload size for PDSCH of Msg.4 | 1040 bits |
| Other parameters | Reported by companies. |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PDCCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | Reported by companies |
| BLER | 1% BLER optional for 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

# Appendix B	Agreements from RAN1#110

Agreement

For NR-NTN coverage enhancement, RAN1 concludes that coverage enhancements specifically for GEO and MEO are de-prioritized in Rel-18.

- Potential enhancements for LEO can also apply to GEO and MEO

Agreement

For NR-NTN coverage enhancement in Rel-18, link budget of parameter set-1 for LEO-1200 operating at LOS is considered as the target to evaluate whether each channel/signal with the existing specification needs to be enhanced or not. The targeted performances are used to evaluate the following services:

- VoIP using AMR 4.75 kbps.

- Low data rate of 3 kbps.

- Potential enhancements for deployments with parameter set-1 can also apply for deployments for parameter set-2

Observation

For PUCCH format 1 with parameter set-1 for LEO-1200 operating at LOS,

- Five sources observed that the existing specification can meet the performance requirement.

Conclusion

RAN1 concluded that enhancement is unnecessary for PUCCH format 1 with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PUCCH format 3 with parameter set-1 for LEO-1200 operating at LOS,

- Six sources observed that the existing specification can meet the performance requirement.

- One source observed that the existing specification cannot meet the performance requirement with at least 0.6 dB gap.

Conclusion

RAN1 concluded that enhancement is unnecessary for PUCCH format 3 with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PUCCH for Msg4 HARQ-ACK with parameter set-1 for LEO-1200 operating at LOS,

- One source observed that the existing specification can meet the performance requirement.

- Three sources observed that the existing specification cannot meet the performance requirement with a gap of 1.8 to 6 dB.

Conclusion

RAN1 concluded that PUCCH for Msg4 HARQ-ACK should be enhanced to meet the coverage requirements for parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PUSCH for low data rate of 3 kbps with parameter set-1 for LEO-1200 operating at LOS,

- Eight sources observed that the existing specification can meet the performance requirement.

Conclusion

RAN1 concluded that enhancement is unnecessary for PUSCH for low data rate of 3 kbps with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PRACH format 0 with parameter set-1 for LEO-1200 operating at LOS,

- One source observed that the existing specification can meet the performance requirement

- Eight sources observed that the existing specification cannot meet the performance requirement with a gap of 0.3 to 5.3 dB

For PRACH format 2 with parameter set-1 for LEO-1200 operating at LOS,

- Ten sources observed that the existing specification can meet the performance requirement

- Two sources observed that the existing specification cannot meet the performance requirement with a gap of 1.9 to 8.8 dB

For PRACH format B4 with parameter set-1 for LEO-1200 operating at LOS,

- Ten sources observed that the existing specification cannot meet the performance requirement with a gap of 1.2 to 11.9 dB

Note: for the observations above, some sources used 1 Rx antenna and some sources used 2 Rx antennas at the satellite.

Observation

For PUSCH for VoIP with parameter set-1 for LEO-1200 operating at LOS,

- Six sources observed that the existing specification can meet the performance requirement with a margin of 0 to 1.7 dB

  - One company simulated by using 20 repetitions without DMRS bundling

  - Four companies simulated by using 20 repetitions with DMRS bundling

  - One company simulated by using 32 repetitions with DMRS bundling

    - Note: this is the only result using frame combining by application layer

- Nine sources observed that the existing specification cannot meet the performance requirement with a gap of 0.3 to 8.6 dB

  - Eight companies simulated by using 20 repetitions without DMRS bundling

  - Seven companies simulated without frequency hopping

  - One company simulated by using 16 repetitions with DMRS bundling

Note: for the observations above, some sources used 1 Rx antenna and some sources used 2 Rx antennas at the satellite.

Observation

RAN1 concluded that enhancement for PUSCH for VoIP may be needed to meet the coverage requirements for parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain, when DMRS bundling is not applied.

Observation

For Msg3 PUSCH with parameter set-1 for LEO-1200 operating at LOS,

- Eight sources observed that the existing specification can meet the performance requirement

- One source observed that the existing specification cannot meet the performance requirement with a gap of 1.5 dB.

Conclusion

RAN1 concluded that enhancement is unnecessary for Msg3 PUSCH with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

# Appendix C	Detailed simulation results

## C.1	PUCCH for Msg.4 HARQ-ACK

Figure 1: PUCCH Msg.4 HARQ-ACK performance.

## C.2	PUSCH for VoIP

Figure 2: PUSCH BLER with 1 PRB and 1 DMRS symbol.

Figure 3: PUSCH BLER with 1 PRB and 2 DMRS symbols.

Figure 4: PUSCH BLER with 2 PRBs and 1 DMRS symbol.

Figure 5: PUSCH BLER with 2 PRBs and 2 DMRS symbols.