3GPP TSG RAN WG1 #110-bis-e    	                                              R1-2209703

e-Meeting, October 10th – 19th, 2022

Agenda Item:		8.5

Source:				Samsung

Title:					Discussion on SRS collision timeline check in inactive state

Document for:	Discussion

Introduction

In the R17 WI for positioning enhancement [1], one important direction is to improve positioning latency and efficiency of the Rel-16 NR positioning methods. This contribution discusses the maintenance on latency and efficiency improvement related enhancement.

Positioning in Inactive state

In RAN1 #107-e meeting, the following agreement was made for latency reduction:

Agreement - The following options are supported for SRS for positioning transmission by RRC_INACTIVE UEs: - Option 1: - Subject to UE capability (which is a prerequisite for option 2), a UE may be configured with an SRS for Positioning associated with the initial UL BWP and transmitted, during the RRC_INACTIVE state, inside the initial UL BWP with the same CP and SCS as configured for initial UL BWP. - Option 2: - Subject to UE capability, a UE may be configured with an SRS for Positioning where the following parameters are additionally configured for the transmission of the SRS for Positioning during the RRC_INACTIVE state: frequency location and bandwidth, SCS, CP length. The UE shall not transmit the SRS for Positioning when it is expected to perform UL transmissions in the initial UL BWP in RRC_INACTIVE state. Working assumption For Option 2 of SRS for positioning transmission in RRC_INACTIVE, a UE capability for switching time between SRS Tx and other Tx in initial UL BWP or Rx in initial DL BWP is introduced - The capability is reported per band - The capability applies at least to TDD - FFS: FDD - The switching time value(s) are left up to RAN4 discussion - If the transmission of SRS for positioning with the switching time overlaps/collides in time domain with other DL reception or UL transmission at least for TDD, the SRS for positioning transmission is dropped in the symbol(s) where the overlap/collision occurs - Note: Transmission of SRS for positioning with the switching time covers the following example TDD cases: - “switching after SRS” (i.e., transmission of SRS + switching time) - “switching before SRS” (i.e., switching time + transmission of SRS)

Agreement from RAN1#109e

The PRS collision detection timeline for the case when PRS is lower priority than the DL signals/channels is define as following.

- For an activated type 1A and type 1B PRS processing window

  - If UE determines the presence of other DL signals/channels except SSB of higher priority than PRS in the PPW no later than [N symbol/T ms] before the start of the PPW, UE expects to receive the DL signals/channels and drop the all DL PRS in the PPW.

- For an activated type 2 PRS processing window

  - If UE determines the presence of other DL signals/channels except SSB of higher priority than PRS on a PRS symbol no later than [N symbol/T ms] before the PRS symbol, UE expects to receive the DL signals/channels and drop the PRS symbol.

Agreement from RAN1#109e

The PRS collision detection timeline for the case when PRS is lower priority than the DL signals/channels is define as following.

- For a type 1A and type 1B PRS processing window

  - If UE determines the presence of other DL signals/channels except SSB of higher priority than PRS in the PPW later than [N symbol/T ms] before the start of the PPW, UE is not required to receive the other DL signals/channels except SSB of higher priority and may receive the DL PRS in the PPW.

- For a type 2 PRS processing window considered active

  - If UE determines the presence of other DL signals/channels except SSB of higher priority than PRS on a PRS symbol later than [N symbol/T ms] before the PRS symbol, UE is not required to receive the other DL signals/channels except SSB of higher priority and may receive the PRS symbol.

- Note 1: This implies that if the scheduling of other DL signals/channels of higher priority arrives too late, UE may consider the PRS as higher priority than the other DL signals/channels.

Note 2: If the scheduling of other DL signals/channels of higher priority arrives too late, it is up to UE implementation whether to receive the other DL signals/channels.

Since there could be SDT procedure in inactive state, which means the regular scheduled DL reception or UL transmission could happen, as well as non-scheduling transmission, e.g, RACH. So the collision handling between SRS and these signals should be considered as well.

SRS vs scheduled UL/DL signals

Following the discussion like PRS timeline collision with DL/UL signals, the SRS tx also needs preparation time, and there is also timeline for detecting the DL Rx and/or UL Tx, so the SRS timeline check with other UL/DL signals is also needed.

Thus if there is a DL signal expected to be received (or a UL signal expected to be transmitted) in the time span of SRS transmission determined by UE at a time earlier than N time unit (could include the switching time gap) before the start of SRS transmission, UE expects to receive DL signals (or transmit the UL signals) and drop SRS; Otherwise, UE expects to receive the DL (or transmit the UL signals) and may transmit SRS for positioning

Proposal 1: When the UE determines the presence of other DL signals and channels(or the transmit of UL signals and channels) of higher priority than the SRS no later than [N symbol(s)/T ms] before the first symbol of the SRS transmission, the UE is expected to receive the other DL signals and channels (or transmit the UL signals and channels) and drop SRS; Otherwise, the UE is not required to receive the other DL signals and channels (or the transmit of UL signals and channels) and may transmit SRS.

The detailed the CR for proposal 2 is referred to our companion draft CR [2].

Conclusion

This contribution discusses the latency reduction for measurement/report part for the positioning methods. Observations and proposals are summarized as follows:

Proposal 1: When the UE determines the presence of other DL signals and channels(or the transmit of UL signals and channels) of higher priority than the SRS no later than N2 symbols, defined in clause 6.4 for the subcarrier spacing μ of the DL PRS, before the first symbol of the SRS transmission, the UE is expected to receive the other DL signals and channels (or transmit the UL signals and channels) and drop SRS; Otherwise, the UE is not required to receive the other DL signals and channels (or the transmit of UL signals and channels) and may transmit SRS.

- References

- RP-210903, New WID on NR Positioning Enhancements, RAN#91-e, Mar, 2021.

- R1-2209704, “Draft CR for SRS collision timeline check in inactive state”, RAN1#110, Samsung, 2022 Aug.