3GPP TSG RAN WG1 #110-bis-e			R1-2209722

e-Meeting, October 10th– October 19th, 2022

Agenda item:	9.2.2.1

Source:	Samsung

Title:	Views on Evaluation of AI/ML for CSI feedback enhancement

Document for:	Discussion and Decision

# Introduction

In RAN#94-e, Rel-18 new study item on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” is endorsed. One of the objective of the study item [1] is the following:

*** text omitted*** Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels *** text omitted*** For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

In this contribution, we will provide our views on the evaluation of AI/ML for CSI feedback enhancement.

# Evaluation on AI/ML for CSI feedback enhancement.

Based on the part of SID [1], captured above, the evaluation methodologies (EVMs) and KPIs for the use cases under consideration will be studied. In this regard, this contribution discusses EVMs and KPIs for one of the use cases, namely, CSI feedback enhancement. Moreover, we consider four sub-use cases under CSI feedback enhancement. A brief description of the sub-use cases, which are described in details in [2], is given below.

- CSI prediction:  This sub-use case considers the prediction of CSI in time domain. An AI/ML solution located at either the UE or gNB performs CSI prediction based on a set of inputs, e.g. past CSI measurements/reports.

- Frequency-domain CSI extrapolation: This sub-use case considers extrapolation of CSI in the frequency domain.  An AI/ML solution located at either the UE or gNB performs CSI prediction in one frequency band based on a set of inputs, e.g. CSI measurements/reports for another frequency band.

- Spatial-frequency domain CSI compression: This sub-use case envisions compression of the CSI feedback in spatial and frequency domains based on a two-sided model. One side of the model located at the UE encodes the CSI to its compressed representation. Another side of the model located at the gNB decodes and reconstructs the received compressed CSI feedback. The pair of encoder and decoder, hence, is referred to as an auto-encoder (AE).

- Joint CSI prediction and compression: This sub-use case considers the compression of CSI feedback in spatial, frequency, and time domains. As this sub-use case considers the three aforementioned compression domains, it can be loosely considered as a combination of the above two sub-use cases.

## General Aspects of Evaluation Methodologies

In RAN1#109-e, EVMs based on SLS as a baseline and LLS as optional are agreed for evaluation of AI/ML-based CSI feedback enhancement. The tables for the agreed evaluation parameters are included in the Appendix section. Moreover, as shown below, it was agreed that companies can consider performing intermediate evaluation on AI/ML model to derive the intermediate KPIs. In our view, the intermediate evaluation with properly selected intermediate KPIs can be used to compare the performance advantages and complexity requirements of potential sub use cases. This greatly simplifies the evaluation efforts and allows the study to consider a diverse set of sub use cases. Eventual evaluation with system level performance metrics will assist in drawing accurate conclusions on the benefits of AI/ML for CSI feedback enhancement and for possible recommendations of normative projects. With this in mind, in the below, we provide our view on some general aspects of the SLS and LLS-based evaluation methodologies.

Agreement For the performance evaluation of the AI/ML based CSI feedback enhancement, system level simulation approach is adopted as baseline - Link level simulation is optionally adopted Agreement For the evaluation of the AI/ML based CSI feedback enhancement, companies can consider performing intermediate evaluation on AI/ML model performance to derive the intermediate KPI(s) (e.g., accuracy of AI/ML output CSI) for the purpose of AI/ML solution comparison.

### 2.1.1.Baseline SLS-based EVM

One of the remaining issues in the agreed SLS-based EVM shown in Table 4.1 is the issue of channel estimation. For the system level evaluation, realistic channel estimation is agreed to be used as a baseline assumption and whether ideal channel estimation can be considered is suggested for future study. Moreover, as shown below, ideal DL channel estimation is optionally taken into the baseline of EVM for the purpose of calibration and/or comparing intermediate results. Moreover, in RAN1#110, it was agree that it is up to companies to choose the error modelling method for realistic channel estimation which can be reported by willingness. We note that realistic channel estimation is advantageous to have a realistic performance evaluation and clearly identifying the benefits of AI/ML-based schemes. However, optionally considering ideal channel estimation simplifies the evaluation work and provides error modelling-independent performance comparison between AI/ML and traditional schemes.

Agreement RAN1#109-e For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, ideal DL channel estimation is optionally taken into the baseline of EVM for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.) - Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation. - FFS: the ideal channel estimation is applied for dataset construction, or performance evaluation/inference. - FFS: How to model the realistic channel estimation - FFS: Whether ideal channel is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation Conclusion RAN1#110 For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, it is up to companies to choose the error modeling method for realistic channel estimation and report by willingness. - Note: It is not precluded that companies use ideal channel to calibrate

Proposal #1: For the SLS-based evaluation of the AI/ML based CSI feedback enhancement, additional to realistic channel estimation, consider ideal DL channel estimation for system performance evaluation.

- Whether the ideal or realistic channel estimation is applied for dataset construction is up to companies

- Ideal channel is used as a target CSI for intermediate results calculation for AI/ML output CSI from ideal channel estimation.

- Realistic channel estimation is used as target CSI for intermediate results calculation for AI/ML output CSI from realistic channel estimation.

Another remaining issue is whether to consider ‘full buffer’ traffic model or FTP models.  It can be argued that FTP traffic models closely approximate reality. However, considering full-buffer traffic allows to disjointedly identify the full potential of AI/ML schemes as compared to traditional schemes by reducing the effect of scheduling. Moreover, for intermediate evaluation on AI/ML models as the evaluation is not dependent on user scheduling and resource utilization, hence, it can be considered as full-buffer traffic. In order to align, the intermediate and eventual system performance evaluation, considering full buffer traffic is beneficial. With this, we support the consideration of both FTP Model 1 and full buffer traffic as a baseline and optional traffic models, respectively.

Proposal #2: For the evaluation of the AI/ML based CSI feedback enhancement consider the following traffic models

- FTP model 1 as baseline

- Full buffer as optional

Additionally, it is beneficiary to align the other remaining parameters with the agreed EVM table for Rel-18 Doppler  CSI enhancement considering that the two items have considerable overlap. In this regard, we propose the following:

Proposal #3: For SLS-based evaluation of the AI/ML based CSI feedback enhancement consider the following values for the remaining parameters

- Simulation bandwidth: 10MHz for 15KHz SCs as a baseline

- MIMO scheme: SU/MU-MIMO with rank adaptation as a baseline

- Traffic load (resource utilization): 50/70 % for SU/MU-MIMO with rank adaptation, 20% for SU-MIMO with rank adaptation, companies are encouraged to report the MU-MIMO utilization.

### 2.1.2. LLS-based EVM

Moreover, in RAN1#109-e, the LLS-based EVM with parameters listed in Table 4.2. is agreed. For the same reasoning as the one mention for SLS-based evaluation, we propose for ideal channel estimation to be adopted as optional consideration.

Proposal #4: For the LLS-based evaluation of the AI/ML based CSI feedback enhancement, additional to realistic channel estimation consider ideal DL channel estimation for system performance evaluation.

- Whether the ideal or realistic channel estimation is applied for dataset construction is up to companies

- Ideal channel is used as a target CSI for intermediate results calculation with AI/ML output CSI from ideal channel estimation.

- Realistic channel estimation is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation.

## Generalization Performance

When the performance of a model is kept within the required range across multiple scenarios and configurations, it implies a single model can be applied to the multiple scenarios/ configurations for which the model generalizes to. This has impact on the overhead associated to the storage and the model life-cycle management of AI/ML models. Moreover, this is particularly useful to UE-side AI/ML models and two-sided AI/ML models as UE may frequently move across multiple deployment scenario (UMa, UMi, InH, etc.), face different configurations (arrangement of gNB antennas), etc. However, some of the considerations on scenarios may apply to both UE and gNB , e.g., relative velocity of UE and gNB, SINR range, etc. Therefore, it is essential to identify which mixture of configurations and scenarios an AI/ML model can generalize and which configurations/scenarios require separate AI/ML models.

Regarding the above, the below agreements were made in RAN1#110. Therein, three cases are agreed to be considered to evaluate the generalization performance of an AI/ML model with respect to scenarios and configurations. An initial set of scenarios and configurations to be considered were also agreed.

Agreement RAN1#110 The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point: - Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Note: Companies to report the ratio for dataset mixing - Note: number of the multiple scenarios/configurations can be larger than two - FFS the detailed set of scenarios/configurations - FFS other cases for generalization verification, e.g., - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B. Agreement RAN1#110 For CSI enhancement evaluations, to verify the generalization performance of an AI/ML model over various scenarios, the set of scenarios are considered focusing on one or more of the following aspects as a starting point: - Various deployment scenarios (e.g., UMa, UMi, InH) - Various outdoor/indoor UE distributions for UMa/UMi (e.g., 10:0, 8:2, 5:5, 2:8, 0:10) - Various carrier frequencies (e.g., 2GHz, 3.5GHz) - Other aspects of scenarios are not precluded, e.g., various antenna spacing, various antenna virtualization (TxRU mapping), various ISDs, various UE speeds, etc. - Companies to report the selected scenarios for generalization verification Agreement For CSI enhancement evaluations, to verify the generalization/scalability performance of an AI/ML model over various configurations (e.g., which may potentially lead to different dimensions of model input/output), the set of configurations are considered focusing on one or more of the following aspects as a starting point: - Various bandwidths (e.g., 10MHz, 20MHz) and/or frequency granularities, (e.g., size of subband) - Various sizes of CSI feedback payloads, FFS candidate payload number - Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports) - Other aspects of configurations are not precluded, e.g., various numerologies, various rank numbers/layers, etc. - Companies to report the selected configurations for generalization verification - Companies are encouraged to report the method to achieve generalization over various configurations to achieve scalability of the AI/ML input/output, including pre-processing, post-processing, etc.

As indicated as one FFS point in the above agreement, one consideration is Case 2A wherein a model trained based on one Scenario#A/Configuration#A is updated based on another scenario/configuration different from Scenario#A/Configuration#A. The intention herein is to evaluate how much an AI model is suitable for transfer learning from one scenario/configuration to another scenario/configuration. While the intention is clear, however, this training case can be classified as part of Cases 1-3 depending other considerations.

For example, A model can be trained based on a  Scenario#A and updated by Scenario#B then tested on Scenario#B. One can see this case as Case 1 as from the testing perspective the model is last trained/finetuned by the same scenario as the scenario the test is conducted. Besides, RAN1 didn’t demarcate the clear boundary to differentiate training and fine-tuning dataset. If the dataset employed for model fine-tune is considerably large or if the learning rate is significantly high, the effect of the initial training diminishes.

One can also view Case2A as part of Case 3. In the above example, the fine-tuning training can basically mix the training dataset. The mixed training dataset in Case 3 can be applied in both interleaved manner (dataset samples from multiple scenarios/configurations interleaved in the training) or ordered manner (dataset samples from one scenario/configuration applied one after the other).

Observation #1: For generalization performance evaluation of AI-based CSI feedback enhancement, Case2A can be viewed as part of Case 1-3 depending on factors such as:  the relative size of fine-tuning dataset, learning rate, or the order of dataset samples for training or fine-tuning.

Proposal 5: For generalization performance evaluation of AI-based CSI feedback enhancement, focus on the agreed three cases, i.e., Case 1, Case 2 and Case 3, to simplify the evaluation. The case described as Case 2A and other possible sub-cases can be studied later with lower priority.

## Evaluation Metrics

Agreement RAN1#109-e For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and/or NMSE as part of the ‘Evaluation Metric’ to evaluate the accuracy of the AI/ML output CSI - For GCS/SGCS, - FFS: how to calculate GCS/SGCS for rank>1 - FFS: whether GCS or SGCS is adopted - FFS other metrics, e.g., equivalent MSE, received SNR, or numerical spectral efficiency gap. Agreement RAN1#110-e For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, between GCS and SGCS, SGCS is adopted

Another remaining issue is details on intermediate KPIs. For example on how to calculate SGCS for rank>1, for methods were discussed

- Method 1: Average over all ranks

- Method 2: Weighted average over all ranks

- Method 3: SGCS is separately calculated for each rank (e.g., for K ranks, K SGCS values are derived respectively, and comparison is performed for per rank)

In our view Method 2 is reasonable as it closely emulates the expected system level throughput. In order to forge this alignment between intermediate KPIs and the eventual system performance metrics, the weights can be set as the singular values of the channel’s covariance matrix.

Proposal #6: For the evaluation of the AI/ML based CSI feedback enhancement, calculate SGCS for rank>1 as weighted average over all ranks

SGCS=E1N×Ki=1NjK(λij)2j=1K(λij)2||wijHwij ||||wij||  ||wij||2

where λij is an eigenvalue of the channel covariance matrix corresponding to wij.

Note: wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over multiple samples.

Furthermore, some discussion were made in the last RAN1 meeting whether to consider additional intermediate metrics. This includes Realized relative SNR (RRSNR), Chordal distance, Numerical spectral efficiency gap, Normalized Expected Directional Gain (NEDG). The main motivation is to emulate the DL throughput better. As an example RR SINR and NEDG capture the residual interlayer interference. In particular, consider the below formulation for RR SINR

RRSNRK≔ E1NK i=1Nj=1K(λij)2i=1Nj=1KHiwij2wij2

where λij is an eigenvalue of the channel covariance matrix corresponding to wij.

Note: Hi is the channel matrix at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over.

Consider the channel matrix Hi=UiΣiWiH  where Ui=[ui1ui2…]  and W=[wi1wi2…] are the right and left-side eignvector matrices of Hi while Σi=diag(λi1λi2…). Then, Hiwij=k=1Kuik(λik)2(wik)Hwij . Here, it can be noted that RRSNR captures the signal power from the j-th layer projected on the subspace spanned by the other eignevectors too, i.e.,  (wik)Hwij  for k≠j. However, if the receiver (UE) applies the left-side eigenvectors to receive the transmission for different layers, i.e., (uij)HHiwij=(λik)2(wij)Hwij to receive the j-th layer, this leaking energy is suppressed (nullified). Besides, for MU-MIMO, the gNB may apply inter-layer interference nulling, e.g., by applying zero-forcing, after reconstructing the reporting precoder. Thus, in MU-MIMO scenario, when the gNB do not directly apply the reconstructed precoder, RRSNR does not give practical advantage in terms of emulating the DL MU-MIM as compared to SGCS.

Observation #2: RRSNR as intermediate KPI may be advantageous on providing better emulation of the MU-MIMO DL throughput performance than GCS in some receiver assumptions. If the UE performs inter-layer interference nulling, this advantage is lost.  This observation holds to other proposed intermediate KPIs such as numerical spectral efficiency gap and NEDG.

Observation #3: In MU-MIMO scenario, when the gNB do not directly apply the reconstructed precoder, RRSNR does not give a practical advantage in terms of emulating the DL MU-MIMO throughput performance than SGCS. This observation holds to other proposed intermediate KPIs such as numerical spectral efficiency gap and NEDG.

Proposal #7: For the evaluation of the AI/ML based CSI feedback enhancement, deprioritize discussion on additional intermediate KPIs.

# Preliminary Evaluation Results

# Preliminary Evaluation Results

## CSI prediction

Recent research results from both academia and industry indicate that AI-based CSI prediction strategies can significantly reduce prediction error beyond that achieved by the sample-and-hold strategy that is supported by Rel. 15-17.  Many of these results utilize deep learning techniques to learn the temporal channel correlations (and, in some instances, spatial-frequency channel correlations).

In our previous contribution [7], we showed that a 3D-CNN that had been augmented with a ResNet yielded a prediction error (i.e. average NMSE) of -10.5 dB, compared to the prediction error of 4.789 dB for a baseline sample-and-hold predictor.  In this contribution, we utilize this 3D-CNN+ResNet for our evaluations.

Our evaluation results were generated using the following parameters:

- UE speed of {10, 20, 30} km/h

- 3GPP UMa channel model

- Carrier frequency of 4 GHz

- Channel bandwidth of 10 MHz

- Inter-BS distance of 200 m

- 100% outdoor UE

- gNB has Nt = 32 transmit antennas and Nr = 4 receive antennas

- K = 52 resource blocks

- CSI-RS periodicity of 5 ms.

The 3D-CNN+ResNet that we used, along with its inputs, is shown in Fig. 1-1.  In particular, the objective is to predict the next 3-D CSI sample H, which has dimensions of K x Nt x Nr.

Fig. 1-1: AI-based CSI predictor that utilizes residual neural network

We define the following parameters:

- B is the batch size

- L is the number of past channel observations that are input to the 3D-CNN+ResNet

- X = 2*Nr*floor(Nt/2)*floor(K/4), where floor() is the floor function.

The parameters of the 3D-CNN+ResNet are shown in Table 1-1.

Table 1-1: Parameters of 3D-CNN+ResNet

| Module | Parameter | Value |
| --- | --- | --- |
| Conv Block 1 | Input and output dimensions | (B, 2*L, Nr, Nt, K) and (B, 4*L, Nr, Nt, K) |
|  | (Kernel, padding, stride) | (3, 7, 5) and (1, 3, 2) and (1, 1, 1) |
| MaxPool 1 | (Kernel, padding, stride) | (3, 3, 3) and (1, 1, 1) and (1, 1, 1) |
| Conv Res Block (1st Conv Block) | Input and output dimensions | (B, 4*L, Nr, Nt, K) and (B, 8*L, Nr, Nt, K) |
|  | (Kernel, padding, stride) | (3, 7, 5) and (1, 3, 2) and (1, 1, 1) |
| Conv Res Block (2nd Conv Block) | Input and output dimensions | (B, 8*L, Nr, Nt, K) and (B, 16*L, Nr, Nt, K) |
|  | (Kernel, padding, stride) | (3, 7, 5) and (1, 3, 2) and (1, 1, 1) |
| Conv Res Block (Conv Layer) | Input and output dimensions | (B, 16*L, Nr, Nt, K) and (B, 4*L, Nr, Nt, K) |
|  | (Kernel, padding, stride) | (3, 7, 5) and (1, 3, 2) and (1, 1, 1) |
| Conv Block 2 | Input and output dimensions | (B, 4*L, Nr, Nt, K) and (B, 2, Nr, Nt, K) |
|  | (Kernel, padding, stride) | (3, 7, 7) and (1, 3, 3) and (1, 1, 1) |
| MaxPool 2 | (Kernel, padding, stride) | (1, 2, 4) and (0, 0, 0) and (1, 2, 4) |
| FC Block | Input and output dimensions | (B, X) and (B, 2*Nr*Nt*K) |

The hyper-parameters that we used for model training are shown in Table 2.

Table 1-2: Hyper-parameters for model training

| Parameter | Value |
| --- | --- |
| Batch size | 256 |
| Number of epochs | 100 |
| Optimizer | Adam |
| Initial learning rate | 0.001 |
| Learning rate schedule | 0.1 at (100, 200, 250) epochs |
| Training/validation split | 90% / 10% |

We also utilized a different baseline predictor than sample-and-hold in this contribution.  In particular, to predict the next CSI sample Hn+1, we computed a linear combination of the current and previous CSI samples Hn and Hn-1, respectively.  Thus, we obtained a prediction Hpred,n+1 = αHn+βHn-1.  For each UE speed, we used a numerical search to determine the weights α and β.

The prediction error (i.e. average NMSE) that we obtained for our baseline predictor and our 3DCNN+ResNet (with Case 1 for verifying generalization performance) are shown in Table 3.

Table 1-3: Prediction error (dB) as function of UE speed

|  | 10 km/h | 20 km/h | 30 km/h |
| --- | --- | --- | --- |
| Linear combination | -4.14 | 2.14 | 2.34 |
| 3DCNN+ResNet | -20.8 | -19.3 | -18.5 |

Figure 1-2 shows the CDF of the SGCS for our baseline predictor and our 3DCNN+ResNet for a testing dataset where the UE speed was set to 10 km/h.  The green trace in this plot corresponds to Case 1 for verifying generalization performance, as the UE speed was also set to 10 km/h for the training dataset.  The black trace in this plot corresponds to Case 3 for verifying generalization performance, as the training dataset was equally split between data where the UE speed was set to 1) 10 km/h and 2) 20 km/h.  Here, we determined (α, β) = (1.6, -0.6).

Figure 1-2: CDF of SGCS for 10 km/h

Figure 1-3 shows the CDF of the SGCS for our baseline predictor and our 3DCNN+ResNet for a testing dataset where the UE speed was set to 20 km/h.  The green trace in this plot corresponds to Case 1 for verifying generalization performance, as the UE speed was also set to 20 km/h for the training dataset.  The black trace in this plot corresponds to Case 3 for verifying generalization performance, as the training dataset was equally split between data where the UE speed was set to 1) 20 km/h and 2) 30 km/h.  Here, we determined (α, β) = (0.7, 0.3).

Figure 1-3: CDF of SGCS for 20 km/h

Figure 1-4 shows the CDF of the SGCS for our baseline predictor and our 3DCNN+ResNet for a testing dataset where the UE speed was set to 30 km/h.  The green trace in this plot corresponds to Case 1 for verifying generalization performance, as the UE speed was also set to 30 km/h for the training dataset.  The black trace in this plot corresponds to Case 3 for verifying generalization performance, as the training dataset was equally split between data where the UE speed was set to 1) 20 km/h and 2) 30 km/h.  Here, we determined (α, β) = (0.7, 0.3).

Figure 1-4: CDF of SGCS for 30 km/h

Observation 1-1: AI-based CSI prediction can yield performance benefits over a range of UE speeds, compared to a conventional CSI predictor that computes a linear combination of the current and previous CSI samples.

## Frequency-domain CSI Extrapolation:

The benefits of AI-based CSI prediction can be applied to extrapolation in other domains (e.g. frequency, space; we use “extrapolation” to refer to those domains).  For example, a gNB can configure a UE to send it CSI reports for an inactive bandwidth part (BWP).  The UE can use received DL CSI-RS on an active BWP and then perform AI-based CSI extrapolation to infer CSI on the inactive BWP.  The gNB can then decide whether to configure the UE to switch to the inactive BWP, depending on the CSI reports for the active and inactive BWPs.

The problem that we are considering, along with the neural network architecture that we developed to address it, is shown in Fig. 2-1.  In particular, the objective is to extrapolate 72+N CSI-RS observations to CSI on the 288 SCs in the region in yellow.  In this case, if N > 0, then some interpolation is performed to infer the 288 SCs in the region in yellow (hence, we refer to our approach as “extra(inter)-polation”).

Fig. 2-1: AI-based CSI frequency extra(inter)-polator

Our evaluation results are shown in Fig. 3, where we compare the performance of our AI-based CSI frequency extra(inter)-polator (which we denote “TestNet”) with an ideal 2-D Wiener filter and an AI-based channel estimator (which we denote “EDSR”) [8].  For both of the latter approaches, we placed CSI-RS on every 4th subcarrier in the region in yellow in Fig. 2.  These results were generated using the following parameters:

- UE speed of 5 km/h

- Delay spread of 100 ns

- TDL-C (NLoS) channel model

- Carrier frequency of 3.5 GHz

- gNB has Nt = 1 transmit antenna and Nr = 1 receive antenna

- K = 48 resource blocks.

Fig. 2-2: AI-based CSI frequency extra(inter)-polation error

The parameters of the neural network architecture that we developed in this case are shown in Figures 4 and 5.

Fig. 2-3: Parameters of AI-based CSI frequency extra(inter)-polator

Fig. 2-4: Parameters of “Basic Block” in AI-based CSI frequency extra(inter)-polator

The hyper-parameters that we used for model training are shown in Table 3.

Table 2-1: Hyper-parameters for model training

| Parameter | Value |
| --- | --- |
| Initial batch size | 16 |
| Batch size schedule | 2 at (60, 120, 180, 240, 300) epochs |
| Number of epochs | 300 |
| Optimizer | Adam |
| Initial learning rate | 0.001 |
| Learning rate schedule | 0.1 at (100, 200, 250) epochs |
| Training/testing split | 80% / 20% |

Observation 2-1: AI-based CSI frequency extrapolation can be enhanced by utilizing additional CSI-RS observations in the band to be extrapolated, which amounts to AI-based CSI frequency extra(inter)-polation.

## Spatial-frequency-domain CSI Compression

The training dataset and inference dataset is obtained from the SLS following the Rel-16 SLS assumptions (see Table 3-1).

Table 3-1. Assumption on dataset generation

| Parameter | Value |
| --- | --- |
| Scenario | UMa and UMi (7 macrocell sites and 3 sectors per site), InH |
| Carrier Frequency | 2GHz |
| Inter-BS distance | 500m for UMa and 200m for UMi |
| Simulation bandwidth | 10MHz (52 RB, 13 subbands) |
| UE distribution | 80% indoor, 20% outdoor |

Figure 3-1. Block diagram of ViT-AE based CSI compression

For the compression and reconstruction of CSI, the autoencoder (AE) consisting of the encoder for the compression task and the decoder for the reconstruction task is used (see Figure 1). As an AI model in the encoder and decoder, vision transformer (ViT) [3], a useful model to exploit the self-attention mechanism on 2D input data, is used with some modifications. In our evaluations, uniform quantization is used for the quantizer/de-quantizer and GCS is used as a loss function. The system-level channel data for training is generated from 512 drops and 420 UEs per simulation drop, and thus 215,040 training samples are obtained. For the inference dataset, 8400 samples are generated from 20 drops. Further, the ViT-based AE compresses and reconstructs the rank 1 eigenvector for 13 subbands. As an intermediate AI performance metric, the generalized cosine similarity (GCS) is used. The detailed hyper-parameters are shown in Table 3-2.

Table 3-2. Hyperparameters on CSI compression training

| Parameter | Value |
| --- | --- |
| Batch size | 256 |
| Number of epochs | 1000 with early stopping |
| Optimizer | Adam |
| Initial learning rate | 0.001 |
| Learning rate schedule | 0.96 at every 10 epochs |
| Training/validation split | 80% / 20% |

In Section 3.2.2, we evaluate the AI performance for three cases as agreed in RAN1#109-e:

- Case 1: (co-scenario evaluation) the training dataset is constructed by training inputs from a single configuration#A/scenario#A, and the testing/inference is performed for the same configuration#A/scenario#A.

- Case 2: (cross-scenario evaluation) the training dataset is constructed by training inputs from a single configuration#A/scenario#A, and the testing/inference is performed for a different single configuration#B/scenario#B.

- Case 3: (mixed-scenario evaluation) the training dataset is constructed by mixing training inputs from multiple configurations/scenarios, and the testing/inference is performed for a single configuration/scenario.

Note that we can verify the generalization capability through the cross-scenario evaluation in case 2 and mixed-scenario evaluation in case 3.

3.2.2 Evaluation results for co-scenario, cross-scenario, and mixed-scenario training/inference

In this section, we provide the initial simulation results for AI-based spatial-frequency domain CSI compression for co-scenario, cross-scenario, and mixed-scenario.

3.2.2.1 Performance-related KPI

[Case 1] In the co-scenario evaluation, the training dataset and the inference dataset are constructed from a single configuration/scenario. In these evaluations, the number of feedback bits is 338 in AI approaches while eType II CB uses 338 bits for UMa and 272 bits for UMi and InH. The GCS performances for different three scenarios are provided in Table 3-3.

Table 3-3. GCS performances for UMa, UMi, and InH scenarios (GCS).

|  | UMa | UMi | InH |
| --- | --- | --- | --- |
| ViT-based AE | .9376 | .9580 | .9530 |
| CNN-based AE | .8992 | .9101 | .9336 |
| eType II CB for ParamCombination=6 | .9348 | .9418 | .9399 |

Observed from Table 3-3, the performance of AI-based CSI compression varies with the channel scenario. For example, the GCS performance of ViT-based AE changes from 0.9376 to 0.9580. Whereas, the performance variation of eType II CB is relatively small. Note that, since the training and inference datasets are drawn from the same configuration/scenario, the AI-based results (i.e., ViT-based AE and CNN-based AE) could be used as an approximate upper bound in each scenario. These GCSs are used to measure the generalization capability in the subsequent cases.

[Case 2] In the cross-scenario evaluation, configuration/scenario of training dataset and that of inference dataset is different. Thus, through these evaluations, it can be investigated whether the AI model trained with a certain configuration/scenario can generalize the unseen configuration/scenario or not. Note that the eType II CB is free from the performance degradation caused by the cross-scenario since it is not a data-driven approach. In order to measure the generalization, we need to determine a performance metric. For example, in our evaluations, a degree of the performance degradation over the co-scenario performance (in a unit of percentage) can be used. If the degree of the performance degradation is marginal, this implies that the trained AI model generalizes two configurations/scenarios well. The evaluation results of cross-scenario evaluation are in Table 3.

Table 3-4. Cross-scenario performance for UMa, UMi, and InH (GCS).

| Inference Training | UMa (0.9348 for eType II) | UMi (0.9418 for eType II) | InH (0.9399 for eType II) |
| --- | --- | --- | --- |
| UMa | .9376 | .9563 (-0.2%) | .9008 (-5.4%) |
| UMi | .9314 (-0.6%) | .9580 | .9085 (-2.7%) |
| InH | .8858 (-5.5%) | .9064 (-5.4%) | .9530 |

In Table 3-4, we observe that the degree of GCS degradation in UMa and UMi scenarios is much smaller than that in InH scenario. For example, when the AI model is trained with the UMa dataset, the GCS for UMi and InH is degraded by -0.2 % and -5.4 %, respectively (see the second row in Table 5). Compared to eType II, the performance degradation between UMa and UMi is tolerable, implying that the AI model trained with UMa or UMi can be used for the cross-scenario. It is worth noting that, from these evaluations, the generalization capability among various configuration(s)/scenario(s) can be identified. Then, the identified generalization capability can be used to determine the way of dataset construction in the training. For example, compared to the generalization from UMa to UMi, it is difficult to generalize from UMa to InH. Hence, when we construct the mixed-scenario dataset (see Case 3 below), it would be better to include the InH dataset rather than UMi dataset.

Observation 3-1: The following observation were made for generalization performance across deployment scenario

- AI model generalizes well from UMa to UMi and vice versa.

- It is relatively difficult to generalize from UMa or UMi to InH.

[Case 3] For the mixed-scenario evaluation, the UMa dataset and the InH dataset are mixed when constructing the training dataset. The mixing ratio of UMa data samples and InH data samples is 50% and 50%, respectively while maintaining the size of dataset same as the co-scenario/cross-scenario evaluations. From these evaluations, we can verify how much the performance degradation in cross-scenario evaluation can be mitigated. The evaluation results are provided in Table 6.

Table 3-5. Mixed-scenario performance for UMa and InH (GCS).

| Inference Training | UMa | InH |
| --- | --- | --- |
| UMa | .9376 | .9008 (-5.4%) |
| InH | .8858 (-5.5%) | .9530 |
| 50% UMa + 50% InH | .9324 (-0.6%) | .9337 (-2%) |

From Table 3-5, we observe that GCS degradation in case 2 is mitigated significantly. For example, when the AI model is trained with the UMa dataset and is tested with the InH dataset, the GCS degradation is around -5.4%. In case of the mixed-scenario training, the GCS degradation is around -2%. For the generalization, mixing various scenarios (or channel parameters, configurations) might be a useful option to alleviate the degradation of the generalization performance.

Observation 3-2: The following observations were made regarding approaches to improve generalization performance

- GCS degradation in cross-deployment-scenario, i.e., Casse 2, can be alleviated by mixing various datasets.

- For the generalization, mixing various configuration(s)/scenario(s) is a viable option to alleviate the degradation of the generalization performance.

In this part, the generalization performance for rank>1 configurations is provided. Note that for the generalization evaluation of multiple layers, the dimension of inputs and outputs is fixed. In our evaluation, the compression ratio is 1/8, which makes the dimension of encoded codeword 104. Hence, the total number of feedback bits is 208 when the 2-bit scalar quantization is used. In the training, we use 100,800 channel data generated under UMa scenario with 240 drops. As an AI performance metric, the squared generalized cosine similarity (SGCS) is used. For the generalization evaluation, two options are considered as followings:

• Option 1 (layer-specific): Separated AI/ML models are trained per each MIMO layer and applied for corresponding layers to perform individual inference.

• Option 2 (layer-common): A unified AI/ML model is trained and applied across multiple layers to perform inference.

- Option 2-A: A unified AI/ML model is trained with the dataset for layer 1, i.e., the dominant eigenvector. In the inference, the trained model is applied to an inference data for layer 1.

- Option 2-B: A unified AI/ML model is trained with the mixed dataset for layer 1 and layer 2, i.e., the first and second dominant eigenvectors, respectively. In the inference, the trained model is applied to an inference data for layer 1 and layer 2.

The evaluation results of rank generalization for layer 1 and layer 2 are provided in Table 3-6.

Table 3-6. Evaluation results for layer generalization (SGCS)

| Inference Training | Layer 1 | Layer 2 |
| --- | --- | --- |
| Layer 1 | 0.8466 | 0.7580 |
| Layer 2 | 0.8327 | 0.7573 |
| Mixed for Layer 1 and Layer 2 | 0.8161 | 0.7267 |

Observed from Table 3-6, the generalization capability between layer=1 and layer=2 cases would be high. For example, when the AI model trained with layer 1 (dominant eigenvector) dataset is applied to layer 2 dataset, SGCS is 0.7580. This SGCS value is comparable to the SGCS=0.7573 obtained from the case that the AI model trained by layer 2 dataset is applied to layer 2 dataset. On the other hand, when a unified model trained with the mixed dataset for layer 1 and layer 2 is applied to layer 1 dataset and layer 2 dataset, the SGCS performance is degraded. For example, in case of layer 1, SGCS is degraded by 3.6% (from 0.8466 to 0.8161).

Observation 3-3: For AI/ML based CSI compression, the following observations were made for generalization performance across layer 1 and layer 2

- AI model generalizes well from layer 1 to layer 2 thus a layer-common model can be used.

- Training an AI model with the mixed dataset consisting of layer 1 and layer 2 doesn’t help it rather degrades the SGCS performance as compared to training an AI model with a single dataset (layer 1 or layer 2).

In order to verify the generalization performance between indoor and outdoor, two AI models trained with outdoor channel dataset and indoor channel dataset are used. The evaluation results for indoor/outdoor generalization are provided in Table 3-7.

Table 3-7. Evaluation results for indoor/outdoor generalization (SGCS)

| Inference Training | Outdoor channel | Indoor channel |
| --- | --- | --- |
| Outdoor channel | 0.8331 | 0.7952 (-1.9%) |
| Indoor channel | 0.8307 (-0.3%) | 0.8105 |

From Table 3-7, the AI model generalizes well from indoor channel to outdoor channel. For example, the performance degradation is around -0.3% when the AI model trained with indoor channel dataset is applied to the outdoor channel dataset. In case of the generalization from outdoor channel to indoor channel, the performance degradation is around -1.9%.

Observation 3-4: For AI/ML based CSI compression, the AI model generalizes well from indoor channel to outdoor channel.

In order to verify the generalization performance between line-of-sight (LOS) and non-LOS (NLOS), two AI models trained with LOS channel dataset and NLOS channel dataset are used. The evaluation results for LOS/NLOS generalization are provide in Table 3-8.

Table 3-8. Evaluation results for LOS/NLOS generalization (SGCS)

| Inference Training | LOS channel | NLOS channel |
| --- | --- | --- |
| LOS channel | 0.8980 | 0.8796 |
| NLOS channel | 0.8978 | 0.8796 |

From Table 3-8, the AI model generalizes well from LOS channel to NLOS channel and vice versa. For example, there is no performance degradation from LOS channel to NLOS channel (i.e., SGCS=0.8796).

Observation 3-5: For AI/ML based CSI compression, the AI model generalizes well from LOS channel to NLOS channel and vice versa.

3.2.2.2 Capability-related KPI

As a capability-related KPI, the number of model parameters (e.g., weights and biases) and the number of FLOPs are considered. In Table 7, we provide the capability-related KPI for ViT-based AE and eType II codebook-based PMI feedback.

Table 3-6 Capability-related KPI for ViT-based AE and eType II codebook-based PMI feedback

|  | The number of model parameters |  | The number of FLOPs |  |
| --- | --- | --- | --- | --- |
|  | UE | gNB | UE | gNB |
| ViT | 183,283 | 185,208 | 4.2×106 | 4.2×106 |
| CNN | 142,273 | 142,940 | 8.5×105 | 8.5×105 |
| eType II | - |  | 1.6×105 | - |

As shown in Table 7, the AI approach only requires the model parameters to run the AI model. Further, since the decoder at gNB reconstructs the CSI in the AI approach, the additional FLOPs are required. We observe that the number of FLOPs to perform the AE operations is much larger than eType II codebook-based PMI feedback. For example, at the UE side, the number of FLOPs for ViT and CNN are around 26 times and 5 times as large as that for eType II codebook, respectively. Even, including the FLOPs for the AE decoder operation, the number of FLOPs for ViT is around 53 times as large as that for eType II (10 times for CNN-based AE). In this respect, the effectiveness of the AI approach at the cost of these large amount of computational complexities can be investigated. Note that the number of FLOPs for ViT is around 5 times as large as that for CNN while the performance of ViT has around 6% gain over the CNN. When using the AI approach, the model selection can be performed using the trade-off between the performance and complexity.

Observation 3-3: The number of FLOPs to perform the AE operations is much larger than eType II.

## Joint CSI Perdition & Compression (Temporal-spatial-frequency-domain CSI compression)

In the following the performance of temporal-spatial-frequency domain compression is provided. As it is detailed in [9.2.2.2] and depicted in Fig. 3.4.1 below, two approaches are considered, namely, Approach 1 gNB-side prediction and UE-side prediction. Thus, this sub use case can be considered as joint CSI prediction and compression.

Fig. 4-1.  Approach 1 gNB-side prediction vs. Approach 2  UE-side prediction

The performance is evaluated according to the agreed LLS-based evaluation [9] with the parameters selected as shown in Table 4.1.

Table 4-1 Parameters for LLS-based evaluation

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD , OFDM |
| Carrier frequency | 2GHz |
| Bandwidth | 10MHz |
| Subcarrier spacing | 15kHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C |
| UE speed | 3kmhr, 10km/h, |
| Delay spread | 300ns |
| Channel estimation | ideal |
| Rank per UE | Rank 1 |
| CSI-RS resources | Periodic with 5ms |
| CSI report | 4ms delay between measurement and report |

First, let us evaluate the compressibility of time-correlated CSI. For this the setups shown in Fig. 4-2 are considered. Two setups are depicted in the figure, i.e., suitable for (a) aperiodic burst CSI-RS resources and (b) periodic and semi-persistent CSI-RS resources.

Fig. 4-2. Set up to test time-domain compressibility of CSI

For aperiodic CSI-RS resources, the UE compresses CSI from  measurements after receiving a burst of CSI-RS resources at different time instants. In particular, per each layer, M eigenvectors matrices, denoted as  V1,V2,⋯VM,  are considered.  for  corresponds to the  m-th measurement, m=1,2,⋯,M, and consists of eigenvectors for the subbands, i.e., Vm=vm,1vm,2⋯vm,Ns. The autoencoder in Fig. 4-2. (a) compresses these measurements to generate the codeword   which potential can be partitioned to M sub-codewords C=[c1c2⋯cM. The CSI for the M measurements can then be reported in either a single CSI report or in multiple CSI reports.

For, periodic and semi-persistent CSI-RS resources, as shown in Fig. 3.4.2 (b), a CSI corresponding to a single measurement can be reported at a time. Then the decoder considers the codewords reported for the past  measurements, i.e., cm-M+1,⋯,cm-1and concatenate them with current codeword to reconstruct the CSI. This way the time-domain correlation can still be exploited for spatial-frequency-time domain CSI compression even if the UE reports a CSI for a single measurement at a time.

The detailed AI/ML model and training configuration for the autoencoder is depicted in Fig. 3.4.3. A Bi-LSTM based neural network is employed at both the encoder and decoder. At the encoder, the input layer dimension is Ns×2Nt due to concatenation of real and imaginary precoder parts of Ns eigenvectors. Further, the first and second hidden layers are of dimension Ns×8Nt. The third Bi-LSTM layer generates a P dimensional codeword per subband such that the total output dimension is Ns×P. This is converted in to a 1D sequence of length NsP using a reshape layer. Finally, a 1-bit Quantizer converts them into a discrete sequence in the set {-1,1}.

The decoder input layer concatenates the current and past M-1 measurement reports. The following reshaping layer converts the input in to Ns×PM dimension. This is further processed by two Bi-LSTM layers of dimension Ns×16Nt. The eigenvectors are then reconstructed by a third Bi-LSTM layer by generating a Ns×2Nt output sequence.

| AE Training configuration |  |
| --- | --- |
| Dataset size | 300000 |
| Number of epochs | 150 |
| Loss function | Cosine similarity |
| Optimizer | Nadam |
| Learning rate | 0.001 |
| Train test split | 80%-20% |

Fig. 4-3. Set up to test time-domain compressibility of CSI

Fig. 4-4. shows the performance of spatial-frequency-time-domain compression in terms of GCS with respect to various payload sizes. The performance of the baseline scheme, i.e., Rel-16 eType II codebook, with parameter combination 1-8 as specified TR 38.214 is also provided. The payload size is determined by the payload of a single layer PMI for the 8 parameter combinations for Rel-16 eType II codebook. The measurements are 5ms apart and parameters listed in Table 3.4.1 are considered. One observation is that adding time-domain in the compression domain achieves a significant advantage in terms of overhead reduction. As an example when M=5 measurements are considered, to achieve GCS=0.85, over 185% payload size reduction is achieved by the autoencoder over Rel-16 CB. On the other hand, when the payload size is fixed to 157 bits per layer over 7% gain is observed in terms of GCS. This is significant as compared to the 57% payload overhead reduction achieved by an AI-based spatial-frequency-domain CSI compression.

Fig. 4-4. Performance of spatial-frequency-time-domain compression with variable number of measurements (M=1, 3, 5)

Observation 4-1: Adding the time-domain in the CSI compression domains, i.e., CSI compression in spatial-frequency-time domains, achieves a higher compression, i.e., further reduction in CSI report overhead, as compared to spatial-frequency domain CSI compression.

Table 4.1 Complexity in terms of mega-FLOPs and model size

| Index | Payload | Rel-16 CB complexity # of FLOPs (Mega-FLOPs) For paramCombination-r16=1-8. | Bi-LSTM model complexity (# of FLOPs (Mega-FLOPs) |  |  |  | Model Size (# of parameters) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | UE side | Encoder | Decoder |  |  | Encoder | Decoder |
|  |  |  |  | M=1 | M=3 | M=5 |  |  |
| 1 | 55 | 0.032 | 44.5 | 171.7 | 172.5 | 173.4 | 17,16,404 | 66,15,616 |
| 2 | 84 | 0.032 | 44.6 | 171.9 | 173.2 | 174.4 | 17,20,698 | 66,23,808 |
| 3 | 100 | 0.087 | 44.7 | 172 | 173.5 | 175 | 17,22,872 | 66,27,904 |
| 4 | 157 | 0.087 | 45 | 172.5 | 175.1 | 177.6 | 17,34,012 | 66,48,384 |
| 5 | 214 | 0.087 | 45.2 | 172.9 | 176.4 | 179.8 | 17,43,248 | 66,64,768 |
| 6 | 272 | 0.087 | 45.5 | 173.4 | 177.6 | 181.9 | 17,52,772 | 66,81,152 |
| 7 | 230 | 0.163 | 45.3 | 173.1 | 176.7 | 180.3 | 17,45,602 | 66,68,864 |
| 8 | 564 | 0.163 | 47 | 175.8 | 185 | 194.1 | 18,13,124 | 67,75,360 |

Table 3.4.1 provides the complexity comparison of the aforementioned autoencoder-based CSI compression with respect to Rel-16 CB in terms of mega-FLOPs. Two observations can be made from the table. There is a multiple order of increase on computational complexity is incurred by AI-based CSI compression as compared to PMI derivation based on Rel-16 CB. However, in order to have the full picture of the requirements for AI-based CSI compression, the impact of such increase in computational complexity on inference latency shall be studied.  This is particularly important as the level of parallelization allowed for the tasks in AI-based CSI compression and in legacy codebooks based CSI computation might be different. Additional observation is that the increase in computational complexity is invariant with respect to the number of time-domain measurements as the complexity per measurement remains the same at the encoder.

Observation 4-2: AI-based CSI compression incurs a multiple order of increase in the computational complexity (measured in terms of number of FLOPs) as compared to CSI computation based on Rel-16 eType II codebook.

- The increase in FLOPs is invariant with respect to the number of time-domain CSI measurements considered in the compression, i.e., same for spatial-frequency-domain and spatial-frequency-time-domain compression.

- The model size remains in the same range for spatial-frequency-domain and temporal-spatial-frequency -domain compression

In the following we provide the performance for joint prediction and compression. In this regard, as shown in Fig. 4-5., we considered a UE-side prediction based on the (a) eigenvectors and (b) full-channel matrix.

Fig. 4-4. UE-side prediction: eigenvector-based and channel matrix-based prediction

The performance of the two approaches (Approach 2-1 and Approach 2-2) is provided in Fig. 4-5 in terms of GCS vs. prediction delay. The performance is evaluated based on the parameters list in the Table 3.4.1 with UE speed set to 10km/hr. As a baseline, a Rel-16 CB based reporting with no prediction (sample-and-hold) is considered. Moreover, the two payload sizes per layer are considered, i.e., 272 bits and 564 bits. The GCS is computed between the recovered (predicted and compressed) CSI and the ground truth value (genie-aided CSI). For the baseline method, the GCS is simply computed between the reported CSI and the ground truth at a certain prediction delay value. The first and obvious observation is that the GCS performance degrades as the prediction delay increases. However, this degradation is severe (higher slop) if no prediction is applied. In this regard, joint compression and prediction out performs the baseline by 32% at prediction delay of 20ms. Another observation is that most of the gain is attributed to the CSI prediction aspect compared to the compression aspect as the gain from increasing the payload size from 272 to 564 is less significant ( 2%) as opposed to the 32% gain from compression and prediction.

Fig. 4-5. Performance for UE-side prediction: eigenvector-based and channel matrix-based prediction

Observation 4-3: Significant gain is observed for UE-side joint CSI prediction and compression as compared to Rel-16 CB reporting without prediction.

- Most of the performance gain is attributed to CSI prediction as compared to compression.

- The prediction based on full channel matrices outperforms prediction based on eigenvectors.

In the following the performance of  gNB-side prediction (Approach 1) and UE-side prediction (Approach 2) are presented. It can be noticed from Fig. 4-5 that the eigenvectors-based prediction does not work well as compared to full-channel matrix based  prediction. One challenge for Approach 1 is that the overhead to make the full-channel matrix H available at the gNB. Note here that the full-channel matrix can be decomposed as  H=UΣVH for the right eigenvectors in V where SVDHHH=VΣ2VH and left eigenvectors U given as  SVDHHH=UHΣ2U. Conventionally, the right eigenvectors in V are sent to gNB via CSI feedback. Even if sending the full-channel matrix H which is Nr×Nt per reporting subband as CSI feedback the reporting overhead is hindering. However, the gNB is usually interested in channel information corresponding to few dominant layers (low rank). In this case, the UE may share channel matrix corresponding to the few dominant vectors, e.g.,  Hl=uvlH for layers, l=1,2...   Note here that H=lλlHl .

- gNB-side prediction with availability of u and V vectors at the gNB

- Payload 564 bits

- Payload =272 bits

Fig. 4-6.  Performance for gNB-side prediction (with the availability of left and right eigenvectors U and V. i.e., Hl=uvlH) paylaoad size set to 274,  time-domain compression for M=3 past and future time instants for UE-side and gNB-side, respectively, are compressed and reported).

In Figure 4-6 (b) and (c), it can be noticed that gNB-side prediction performs well with comparable performance as UE-side prediction, if the UE-side eigenvectors are available at the gNB side. Note hear the same payload size is used to feedback the {v}  and {u+v} vectors, in the UE-side prediction and gNB-side prediction, respectively. In Fig. 4-6 (b), the relatively high payload size, i.e., 564 bits, ensured the accurate delivery of the left-side and right-side eigenvectors; therefore, performance is dictated by the prediction performance rather than by feedback reconstruction performance. Additionally,  Fig. 4-6 (c), provides the result for moderate payload size, i.e., 272bits. Herein, even if the performance of UE-side prediction and gNB-side prediction degraded from the one based on uncompressed full-channel matrix, the degradation is not substantial. Thus, both UE-side and gNB-side prediction asymptotically achieved the prediction without (ideal) compression.

Observation 4-4: gNB-side prediction performs well as UE-side prediction if the UE-side eigenvectors are available at the gNB side.

- The UE-side eigenvectors can be compressed and reported with overhead that scales up with the reported rank.

# Conclusion

In this contribution the following observations are made:

Observation #1: For generalization performance evaluation of AI-based CSI feedback enhancement, Case2A can be viewed as part of Case 1-3 depending on factors such as:  the relative size of fine-tuning dataset, learning rate, or the order of dataset samples for training or fine-tuning.

Observation #2: RRSNR as intermediate KPI may be advantageous on providing better emulation of the MU-MIMO DL throughput performance than SGCS in some receiver assumptions. If the UE performs inter-layer interference nulling, this advantage is lost.  This observation holds to other proposed intermediate metrics such as numerical spectral efficiency gap and NEDG.

Observation #3: In MU-MIMO scenario, when the gNB do not directly apply the reconstructed precoder, RRSNR does not give a practical advantage in terms of emulating the DL MU-MIMO throughput performance than SGCS. This observation holds to other proposed intermediate metrics such as numerical spectral efficiency gap and NEDG.

Observation 1-1: AI-based CSI prediction can yield performance benefits over a range of UE speeds, compared to a conventional CSI predictor that computes a linear combination of the current and previous CSI samples.

Observation 2-1: AI-based CSI frequency extrapolation can be enhanced by utilizing additional CSI-RS observations in the band to be extrapolated, which amounts to AI-based CSI frequency extra(inter)-polation.

Observation 3-3: The number of FLOPs to perform the AE operations is much larger than eType II.

Observation 4-1: Adding the time-domain in the CSI compression domains, i.e., CSI compression in spatial-frequency-time domains, achieves a higher compression, i.e., further reduction in CSI report overhead, as compared to spatial-frequency domain CSI compression.

Observation 4-2: AI-based CSI compression incurs a multiple order of increase in the computational complexity (measured in terms of number of FLOPs) as compared to CSI computation based on Rel-16 eType II codebook.

- The increase in FLOPs is invariant with respect to the number of time-domain CSI measurements considered in the compression, i.e., same for spatial-frequency-domain and spatial-frequency-time-domain compression.

- The model size remains in the same range for spatial-frequency-domain and temporal-spatial-frequency -domain compression

Observation 4-3: Significant gain is observed for UE-side joint CSI prediction and compression as compared to Rel-16 CB reporting without prediction.

- Most of the performance gain is attributed to CSI prediction as compared to compression.

- The prediction based on full channel matrices outperforms prediction based on eigenvectors.

Observation 4-4: gNB-side prediction performs well as UE-side prediction, if the UE-side eigenvectors are available at the gNB side.

- The UE-side eigenvectors can be compressed and reported with overhead that scales up with the reported rank.

And the following proposals are provided:

Proposal #1: For the SLS-based evaluation of the AI/ML based CSI feedback enhancement, additional to realistic channel estimation, consider ideal DL channel estimation for system performance evaluation.

- Whether the ideal or realistic channel estimation is applied for dataset construction is up to companies

- Ideal channel is used as a target CSI for intermediate results calculation for AI/ML output CSI from ideal channel estimation.

- Realistic channel estimation is used as target CSI for intermediate results calculation for AI/ML output CSI from realistic channel estimation.

Proposal #2: For the evaluation of the AI/ML based CSI feedback enhancement consider the following traffic models

- FTP model 1 as baseline

- Full buffer as optional

Proposal #3: For SLS-based evaluation of the AI/ML based CSI feedback enhancement consider the following values for the remaining parameters

- Simulation bandwidth: 10MHz for 15KHz SCs as a baseline

- MIMO scheme: SU/MU-MIMO with rank adaptation as a baseline

- Traffic load (resource utilization): 50/70 % for SU/MU-MIMO with rank adaptation, 20% for SU-MIMO with rank adaptation, companies are encouraged to report the MU-MIMO utilization.

Proposal #4: For the LLS-based evaluation of the AI/ML based CSI feedback enhancement, additional to realistic channel estimation consider ideal DL channel estimation for system performance evaluation.

- Whether the ideal or realistic channel estimation is applied for dataset construction is up to companies

- Ideal channel is used as a target CSI for intermediate results calculation with AI/ML output CSI from ideal channel estimation.

- Realistic channel estimation is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation.

Proposal 5: For generalization performance evaluation of AI-based CSI feedback enhancement, focus on the agreed three cases, i.e., Case 1, Case 2 and Case 3, to simplify the evaluation. The case described as Case 2A and other possible sub-cases can be studied later with lower priority.

Proposal #6: For the evaluation of the AI/ML based CSI feedback enhancement, calculate SGCS for rank>1 as weighted average over all ranks

SGCS=E1N×Ki=1NjK(λij)2j=1K(λij)2||wijHwij ||||wij||  ||wij||2

where λij is an eigenvalue of the channel covariance matrix corresponding to wij.

Note: wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over multiple samples.

Proposal #7: For the evaluation of the AI/ML based CSI feedback enhancement, deprioritize discussion on additional intermediate KPIs.

# Appendix

Table 5-1 SLS-based EVM

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, FFS 2GHz or 4GHz as a baseline |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth |  | FFS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | FFS |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | FFS |
| Traffic load (Resource utilization) |  | FFS |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) FFS whether/what other indoor/outdoor distribution and/or UE speeds for outdoor UEs needed |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline FFS ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | FFS |

Table 5-2 LLS-based EVM

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD (TDD is not precluded), OFDM |
| Carrier frequency | 2GHz as baseline, optional for 4GHz |
| Bandwidth | 10MHz or 20MHz |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C as baseline, CDL-A as optional |
| UE speed | 3kmhr, 10km/h, 20km/h or 30km/h to be reported by companies |
| Delay spread | 30ns or 300ns |
| Channel estimation | Realistic channel estimation algorithms (e.g. LS or MMSE) as a baseline, FFS ideal channel estimation |
| Rank per UE | Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied |

# References

[1] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Dec, 2021.

[2] R1-2203898, Samsung, “Representative sub use cases for CSI feedback enhancement” 3GPP TSG RAN1#109-e

[3] A. Dosovitskiy, L. Beyer, A. Kolesnikov, D. Weissenborn, X. Zhai, T. Unterthiner, M. Dehghani, M. Minderer, G. Heigold, S. Gelly, J. Uszkoreit, N. Houlsby “An Image is Worth 16x16 Words: Transformers for Image Recognition at Scale,” Proc. Int. Conf. Learning Representations (ICLR), May 2021.

[3] R1-2203896, Samsung, “General aspects of AI ML framework and evaluation methodology” 3GPP TSG RAN1#109-e

[4] TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, V17.0.0 (2022-03)

[5] R1-2007151, Samsung, “Moderator summary for multi-beam enhancement: EVM”, 3GPP TSG RAN1 #102-e

[6] R1-2007151, Fraunhofer IIS, Fraunhofer HHI, “MIMO Enhancements for Rel.-18”, 3GPP TSG RAN Rel.-18Workshop

[7] R1-2203897, Samsung, “Views on evaluation of AI/ML for CSI feedback enhancement” 3GPP TSG RAN1#109-e

[8] L. Li, H. Chen, H.H. Chang, and L. Liu, “Deep residual learning meets OFDM channel estimation”, IEEE Wireless Communications Letters, vol. 9, no. 5, pp. 615-618.

[9] Chair's notes RAN1#109-e

[9] Chair's notes RAN1#110