3GPP TSG RAN WG1 #110bis-e	        		              R1-2209749

e-meeting, October 10th – 19th, 2022

Agenda item:		9.10.2

Source:			Samsung

Title:			Considerations on Capacity Improvements for XR

Document for:		Discussion and decision

# Introduction

This contribution considers capacity enhancements for XR.

# Discussion

SA2 in TR 23.700 defined a “PDU set” that includes a number of PDUs with information generated at the application level (e.g. a frame/video slice). Individual IP packets of the PDU set are inter-dependent and need to be received within the PDB for the PDU set. However, while the PDU set framework provides XR-awareness to RAN, whether or not IP packets/PDUs can be discarded after some PDUs are lost, or cannot be delivered within the PDB, is implementation-dependent and that is not captured by video-coded or 3GPP specifications. In some implementations, the entire PDU set can be discarded if at least one PDU cannot be delivered within the PDB (i.e. all PDUs are needed by the application layer) while in other implementations error concealment can apply for up to a certain number of lost PDUs but the order of PDUs is then also relevant. Therefore, whether or not it makes sense to discard PDUs if PDB is not met is implementation-dependent.

RAN1 evaluations also considered different frames types where I-frames are more important than P-frames and B-frames are practically inconsequential. However, based on an LS response from SA4 to SA2 in [2], “The concept of I, P and B pictures dates back to MPEG-2 video and broadcast distribution and is not generally applicable to new codecs and in particular for low-latency applications. In modern video codecs, complex prediction structures are used that take into account application constraints, encoding complexity, latency and dynamic decisions in the encoding.”. Therefore, using I/P/B-frames cannot be universal, there cannot be “common” traffic characteristics, and such characteristics depend on the application, the video codec, the encoding operation and correlation among PDU sets, etc.

Observation 1: Designs relying on existence of I/P/B frames or on discarding PDUs may not be applicable for XR.

In RAN1#110, a large number of L1 designs were proposed for XR and they were well captured in [1]. Almost all of those L1 designs could be generally applicable to eMBB (and several were previously considered for non-XR applications). However, eMBB has been heavily optimized until Rel-17 and further CA/MIMO/scheduling enhancements are pursued in Rel-18. XR-specific L1 enhancements for capacity, if at all possible, need to be based on XR-specific characteristics. Also, application awareness at RAN, for both UL and DL, can improve XR capacity as scheduling for IP packets of a PDU set can be adjusted according to the size and the PDB for the PDU set. However, RAN2 is the appropriate WG for handling such aspects.

Among those L1 designs, several modifications were proposed for SPS/CG operation in order to address XR traffic attributes such as existence of multiple traffic flows, variable TB sizes per traffic flow, and jitter. It is noted that for a PDU set, the jitter exists both within a set for the start of the first PDU and the end of the last PDU, and among successive PDU sets. Also, XR traffic is dynamic and an application server may provide content depending on the pose/control information from the UE (e.g. provide content when the user looks at a first location and not provide content when the user looks at a second location). RAN1 has assumed that pose/control information is provided every 4 msec but SA4 in TR 23.748 seems to consider a rate as fast as 1 msec. In any case, data rate requirements can have large variations in time and there is no traffic predictability for packet sizes, or periodicity, or time of arrival. Therefore, relying on SPS/CG modifications to support XR is fundamentally irrelevant and inapplicable. Also, link adaptation cannot be provided by SPS/CG while a DCI overhead of ~60-80 bits is negligible for capacity compared to the required TB sizes where, even for the smaller XR data rates such as for cloud gaming, a 30 Mbps data rate having a mean packet size of 62500 bytes (500,000 bits) is required.

The conclusion from RAN1#110 identified absence of RAN1 consensus on possible benefits from introducing modifications to SPS PDSCH operation for the purpose of XR capacity enhancements. The same applies for modifications to CG PUSCH operation. Although CG PUSCH is attractive in reducing latency, it also requires predictable traffic, with fixed TB sizes, that can be served with a small bandwidth and does not require link adaptation. None of those conditions is fulfilled by XR traffic. Capacity would significantly decrease when a large bandwidth is reserved but there is no transmission (e.g. due to jitter) or when there is transmission requiring a smaller bandwidth (e.g. due to traffic variations), the majority of bandwidth cannot be reserved semi-statically for a single UE. Also, link adaptation is required for spectral efficiency. Any DCI-based signaling to adapt parameters of CG-PUSCH configurations is also meaningless as CRC is a large fixed overhead (and there may be additional padding to the DCI format size to preserve the “3+1” DCI format size budget). It is noted that for pose/control information (having a fixed and somewhat small TB size without jitter), Rel-17 CG PUSCH operation can apply. Further, there are no UE power savings gains from not receiving PDCCHs for scheduling PDSCHs but instead receiving multiple SPS PDSCHs over large bandwidth for high data rates and for multiple configurations when only one of those configurations may be valid at a time, or receiving PDCCH for adapting SPS PDSCH configurations, or operating XR without link adaptation and requiring longer times for each PDU set.

Observation 2: In addition to SPS PDSCH, there is no need to further consider CG PUSCH modifications for XR.

Another issue that remains pending from RAN1#110 is the support of multi-slot PXSCH scheduling in FR1/FR2. Although the reasons for introducing multi-slot PXSCH scheduling in FR2-2 (PDCCH limitations and short slot duration) do not apply in FR1/FR2, PDCCH capacity/overhead is not an issue for XR, and scheduling restrictions associated with multi-slot PXSCH scheduling are undesirable for XR, it may be argued that multi-slot PXSCH scheduling can also apply to FR1/FR2 subject to UE capability. Suggested modifications to the Rel-17 multi-slot PXSCH scheduling framework include having independent MCS, and/or FDRA, and possibly TPC commands for PUSCH, per PXSCH for multi-slot PXSCH, based on an objective for having independent BLER and/or TBS per PXSCH for different flows, while saving overhead from fields that can exist only once (such as the HARQ-ACK timing indicator or the PUCCH resource indicator and, primarily, the CRC). Then, the design issues become similar to the ones considered in the Rel-18 WI for multi-cell scheduling by a single DCI format.

However, unlike multi-cell scheduling where an overhead increase due to DCI padding/size-matching in order to maintain the “3+1” DCI format size budget may be avoided or limited to one scheduled cell, that is not possible for multi-slot PXSCH scheduling as the “3+1” size budget is per scheduled cell. DCI size matching would offset DCI overhead reduction. Also, as the DCI format would need to be dimensioned for a fixed/configured number of slots, DCI overhead reduction would also be offset when scheduling is over a smaller number of slots. Further, CBG-based retransmissions can provide meaningful benefit to XR capacity and allow for a larger BLER for initial TB transmissions, but CBG-based HARQ-ACK is not supported for multi-slot PDSCH scheduling and would be rather complex to introduce (e.g. as studied in Rel-17 for beyond 52.6 GHz). Also, multi-slot PXSCH scheduling does not result to any UE power savings as it is detrimental and practically not possible for a UE to not monitor PDCCH when e.g. the UE receives multi-slot PDSCH. Also, UE power consumption is dominated by the PDSCH receptions, and there is no possibility even for micro-sleep by a UE.

In general, given that PDCCH capacity/overhead are not issues for XR and any DCI design optimization would not make a material impact on XR capacity (for the 1-2 XR UEs scheduled per slot with DCI being a miniscule amount of the total number of DCI + data bits), it is preferable to avoid efforts and time that would be required to fine-tune a design for a multi-slot PXSCH scheduling DCI format with independent link adaptation per PXSCH.

Observation 3: If multi-slot PXSCH scheduling is to be considered for XR, changes to DCI formats and impact to DCI format size matching, support of CBG-based retransmissions, and approximate XR capacity gains should be first concluded.

Another issue that remains pending from RAN1#110 is about CSI reporting using “soft-ACK” or “delta-MCS” as considered in Rel-17 URLLC where discussion focused on “delta-MCS” without conclusion. The determination of a “delta-MCS” value is based on UE implementation (e.g. using estimated SINR, LLR values, LDPC decoder iterations, etc.). Therefore, due to the UE proprietary determination, testing of the feature is difficult. Main additional shortcomings are that delta-MCS is primarily useful for TB retransmissions, thereby failing to offer a benefit ~90%-99% of the time, and that the channel, the interference, the bandwidth, and the target BLER for a TB retransmission need to be identical as for the previous transmission of the TB in order for a “delta-MCS” report to be meaningful. Such requirements are not generally realistic. Also, unlike sporadic URLLC traffic, XR traffic is largely predictable within a time period (including the jitter), and performing link adaptation based on CSI reports is robust and compatible with existing UE/network implementations.

Observation 4: There is no need to further consider “soft-ACK” or “delta-MCS” reporting for XR.

C-DRX operation is considered for XR under the scope of UE power savings. It is noted that C-DRX is detrimental for XR as it reduces the probability of satisfying the PDB. C-DRX is also detrimental for XR capacity as it hinders link adaptation since a UE does not provide CSI reports when the UE is not in Active Time and, when the UE enters Active Time, a burst for XR scheduling may need to occur in order to meet PDB before the UE provides a CSI report and exits Active Time. Also, the UE does not perform beam management when not in Active Time and the PDB will not be met if the UE enters Active Time using a low SINR beam. For eMBB, as PDB is not an issue, the above are not critical problems since there can be delay tolerance for CSI reporting and beam management, when needed, and scheduling may be delayed.

One solution, similar to handling jitter, is to extend Active Time in order to enable link adaptation prior to scheduling. Extending Active Time to capture jitter and enable link adaptation procedures would often cancel use of C-DRX considering typical XR traffic periodicities, jitter values, and CSI reporting time requirements. Use of C-DRX is also questionable if transmissions of pose-control information by a UE need to be supported every 4 msec (or even more frequently, such as every 1 msec, as suggested in the SA4 TR 23.748). In any case, operation without C-DRX should be supported and UE power savings can be based on PDCCH skipping and other enhancements as discussed in [3]. If C-DRX is supported, for example for the larger XR traffic periodicities, it is beneficial for XR capacity to support CSI-RS triggering for CSI measurements and beam management and corresponding CSI reports as part of a “wake-up” signaling design.

Proposal 1: If C-DRX is supported for XR, support CSI-RS triggering and CSI reporting mechanisms as part of “wake-up” signaling prior to scheduling.

RAN1 considered that a UE provides pose/control information every 4 msec and SA4 seems to consider even faster updates every 1 msec. The above are not possible for typical TDD UL-DL configurations at 15 kHz and may also not be possible at 30 kHz for the faster rates. In Rel-17, latency reduction for HARQ-ACK information is supported through PUCCH cell switching for UEs supporting (and configured with) inter-band UL CA. This can also reduce latency for a UE to provide a positive SR and get scheduled. Although considered several times in the past, a similar mechanism is not supported for the PUSCH mainly because a MAC entity for a given HARQ process will need to be shared by multiple cells in order to support retransmissions on any of the multiple cells. However, pose/control information is UCI similar to CSI report providing information for subsequent scheduling, and there is no need for HARQ retransmissions of outdated pose/control information and Rel-17 PUCCH cell switching (semi-static) is directly applicable.

Proposal 2: Support CG-PUSCH cell switching for pose/control information (no HARQ retransmissions).

In addition to pose/control information, the PDB for UL-video cannot be met for typical TDD UL-DL configurations. If UL CA is used with same UL-DL configurations on each cell, the problem remains as, unlike the DL, data rates will be limited by the UE available power as the UE will need to simultaneously transmit over a large bandwidth and with large spectral efficiency on each cell. Different UL-DL configurations on each cell that are possible in inter-band CA would avoid UE power limitation and provide more opportunities for transmissions in time. To maximize the probability of satisfying PDB and utilize the existence of different UL-DL configurations on different cells, HARQ retransmission of a TB can be on a different cell than the initial transmission. To avoid changes to the MAC layer implementation that are associated with a common pool of HARQ processes among cells, the UE can be indicated by a field in the DCI format the cell of the initial TB transmission in order to avoid confusion of the TB associated with the indicated HARQ process. Soft combining of the initial TB transmission and the TB retransmission can be left to gNB implementation – without soft combining, there is some SINR loss (e.g. ~3 dB) but time and cell/frequency transmission diversity are achieved.

Proposal 3: Consider mechanisms to satisfy the PDB in the sub-6 GHz NR (TDD) bands such as enabling HARQ retransmission of a TB on a different cell than the cell of the initial TB transmission.

# Conclusions

This contribution considered mechanisms for XR capacity enhancements and proposes the following.

Proposal 1: If C-DRX is supported for XR, support CSI-RS triggering and CSI reporting mechanisms as part of “wake-up” signaling prior to scheduling.

Proposal 2: Support CG-PUSCH cell switching for pose/control information (no HARQ retransmissions).

Proposal 3: Consider mechanisms to satisfy the PDB in the sub-6 GHz NR (TDD) bands such as enabling HARQ retransmission of a TB on a different cell than the cell of the initial TB transmission.

In addition, the following observations are made.

Observation 1: Designs relying on existence of I/P/B frames or on discarding PDUs may not be applicable for XR.

Observation 2: In addition to SPS PDSCH, there is no need to further consider CG PUSCH modifications for XR.

Observation 3: If multi-slot PXSCH scheduling is to be considered for XR, changes to DCI formats and impact to DCI format size matching, support of CBG-based retransmissions, and approximate XR capacity gains should be first concluded.

Observation 4: There is no need to further consider “soft-ACK” or “delta-MCS” reporting for XR.

# References

- R1-2207820, “Moderator Summary#1 – Study on XR Specific Capacity Improvements”, Moderator (Ericsson)

- S4-220505,	“Reply to: LS on QoS support with PDU Set granularity”

- R1-2209748, “Considerations on Power Savings for XR”, Samsung