3GPP TSG RAN WG1 #110bis-e	R1-2209812

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.8.2

Source: 	LG Electronics

Title: 	Discussion on other aspects for NCR

Document for:	Discussion and decision

Introduction

In RAN#97-e meeting, a work item on NR network-controlled repeaters is approved and the corresponding description is provided in [1]. According to the WID, objectives of the work item are as follows in RAN1 perspective.

Specify the signalling and behavior of the following side control information for controlling the NCR-Fwd [RAN1, RAN2] - Beamforming - UL-DL TDD operation - ON-OFF information Note: Power control aspect will be checked in RAN#98e. Specify control plane signalling and procedures [RAN2, RAN1] - The configuration of signalling for side control information indication - NOTE: Down-selection of solutions in section 7.2 of TR 38.867 is needed

In this contribution, we discuss other aspects for NCR including control plane signalling and procedures relevant to RAN1

Discussion

According to TR 38.867, NCT-MT can obtain the necessary configurations to receive L1/L2 signaling of the side control information [2].

For the configuration of signalling, the NCR-MT can obtain the necessary configuration for receiving the L1/L2 signaling of the side control information. - Option 1: The necessary configuration is from RRC. - Option 2: The necessary configuration is from OAM or hard-coded. - Option 3: The necessary configuration is partially configured by RRC and partially configured by OAM or hard-coded. The necessary configurations from RRC and/or OAM (or hard-coded) contain: - The configurations of PHY channels to carry the L1/L2 signaling including - The configurations for receiving PDCCH and PDSCH. - The configurations for transmitting PUCCH, if needed. - The configurations for transmitting PUSCH, if needed. - The configurations of L1/L2 signaling including - The configurations for DCI. - The configurations for UCI, if needed. - The configurations for MAC CE, if needed. For the parameters in the necessary configurations for L1/L2 signaling, the existing parameters for PDCCH, PDSCH, PUCCH, PUSCH, DCI, UCI and MAC CE in Rel-17 are the baseline for further discussion.

For this, it will be discussed how these configurations will be given to NCR (i.e., RRC or OAM/hard-coded) during work item phase. However, from the RAN1 perspective, how the configuration for the operation of the NCR-MT is provided is not an issue. In any case, NCR-MT may operate according to a given configuration. Instead, RAN1 needs to discuss the L1 feature to support for NCR-MT operation.

In our view, the basic L1 feature required for the operation of NCR-MT may not much different from that of IAB-MT. The discussion on the L1 mandatory features to be supported for IAB-MT was conducted in Rel-16 IAB WI, and Table 4.2.15.1-1 of TS38.306 shows the determined L1 mandatory features of the IAB-MT. Table 1 summarizes the feature groups of the L1 mandatory features for IAB-MT.

In order to discuss the mandatory features of NCR-MT, rather than repeating the discussion similar to that in Rel-16 IAB, we propose to consider these L1 mandatory features of IAB-MT as mandatory features in NCR-MT as a baseline.

Table 1. Layer-1 mandatory features for IAB-MT

| - Features | Index | Feature group |
| --- | --- | --- |
| 0. Waveform, modulation, subcarrier spacings, and CP | 0-1 | CP-OFDM waveform for DL and UL |
|  | 0-3 | DL modulation scheme |
|  | 0-4 | UL modulation scheme |
| 1. Initial access and mobility | 1-1 | Basic initial access channels and procedures |
|  | 1-3 | SS block based RLM |
| 2. MIMO | 2-1 | Basic PDSCH reception |
|  | 2-5 | Basic downlink DMRS for scheduling type A |
|  | 2-6 | Basic downlink DMRS for scheduling type B |
|  | 2-12 | Basic PUSCH transmission |
|  | 2-16 | Basic uplink DMRS (uplink) for scheduling type A |
|  | 2-16a | Basic uplink DMRS for scheduling type B |
|  | 2-22 | Aperiodic beam report |
|  | 2-32 | Basic CSI feedback |
|  | 2-50 | Basic TRS |
|  | 2-52 | Basic SRS |
| 3. DL control channel and procedure | 3-1 | Basic DL control channel |
| 4. UL control channel and procedure | 4-1 | Basic UL control channel |
|  | 4-10 | Dynamic HARQ-ACK codebook |
| 5. Scheduling/HARQ operation | 5-1 | Basic scheduling/HARQ operation |
| 6. CA/DC, BWP, SUL | 6-1 | Basic BWP operation with restriction |
| 7. Channel coding | 7-1 | Channel coding |
| 8. UL TPC | 8-3 | Basic power control operation |

In addition, it is necessary to discuss the additional features supported by NCR-MT among the remaining L1 features of Rel-15/16/17. Through this, it will be necessary to decide which features to be additionally supported for NCR-MT and whether support of each feature is mandatory or optional. For example, it can be discussed whether TCI-related features including unified TCI introduced in Rel-17 FeMIMO need to be supported for beam indication of NCR-MT.

Proposal 1: For L1 features supported for NCR-MT,

- As baseline, consider L1 mandatory features for IAB-MT in Table 4.2.15.1-1 of TS38.306 as mandatory features for IAB-MT.

- Additionally, discuss L1 features supported for NCR-MT among the L1 features of Rel-15/16/17.

Summary

In this contribution, we discuss other aspects for NCR and obtained following proposal.

Proposal 1: For L1 features supported for NCR-MT,

- As baseline, consider L1 mandatory features for IAB-MT in Table 4.2.15.1-1 of TS38.306 as mandatory features for IAB-MT.

- Additionally, discuss L1 features supported for NCR-MT among the L1 features of Rel-15/16/17.

Reference

- ZTE Corporation, “New WID on NR network-controlled repeaters,” RP-222673, e-Meeting, September 12 – 16, 2022

- 3GPP TR 38.867 V 18.0.0