3GPP TSG-RAN WG1 Meeting #110b-e	R1-2209830

e-Meeting, October 10-19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | XX | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on description of valid PSFCH occasion for scheme 2 in TS 38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to RAN1#108-e agreement below, in order to be UE-B, the PSFCH occasions for resource conflict information of each pair of UEs should not be passed. In current TS 38.213, “valid” is used to describe this condition, but there is no meaning given to the word ‘‘valid’’, and interpretations may differ when implementing the specification. Therefore, “not passed” should be captured as in agreement. Agreement: Confirm the following working assumption with modification in RED. Note that the terminology of “indicationUEB flag” means the indication of whether UE scheduling a conflict TB can be UE-B or not. - Working Assumption: - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, - for each pair of UEs scheduling the conflicting TBs whose PSFCH occasions for resource conflict indication are not yet passed and indicationUEB flag is set to 1 if the higher parameter of indicationUEBScheme2 is (pre)configured to ‘Enabled’, a UE with the higher priority value is UE-B. When the UEs in the pair have the same priority value, UE-A determines one of the UEs to be UE-B by its implementation. - UE-A considers the SCIs received earlier than or equal to sl-MinTimeGapPSFCH before the PSFCH occasion for conflict indication when determining UE-B. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct description on PSFCH occasion by replacing “valid” with “not passed”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No definition given of when a PSFCH occasion is considered “valid”, risking differing interpretations among UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

16.3.0	UE procedure for transmitting PSFCH with control information

< Unchanged parts are omitted >

A first UE determines a UE for providing the conflict information to in a PSFCH as follows

-	if, for a resource pool, typeAUEScheme2 is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception,

-	detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE,

-	detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and

-	determines that the first and second resources overlap in time and frequency

-	the PSFCH occasions for resource conflict information of the second UE and the third UE are not passed

-	the indicationUEB flag in SCI Format 1-A from the second UE and the third UE is set to 1, if indicationUEBScheme2 = 'enabled'

-	determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information

-	determines to transmit to the second UE the PSFCH with the conflict information

-	determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1

< Unchanged parts are omitted >