3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2209835

# e-meeting, October 10 – 19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.213 | CR | xxxx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on TDD configuration for IAB-MT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eIAB_enh-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In Rel-16, the slot formats with the sequence order UL-Flexible-DL for IAB node MT were introduced, and the following agreement on the dedicated RRC signalling (referred as TDD-UL-DL-ConfigDedicated-IAB-MT) was achieved in RAN1#99: Agreements: If the IAB-MT is additionally provided TDD-UL-DL-ConfigDedicated-IAB-MT, the parameter TDD-UL-DL-ConfigDedicated-IAB-MT overrides all symbols (with a limitation that effectively only flexible symbols can be overwritten in Rel-16) per slot over the number of slots as provided by TDD-UL-DL-ConfigurationCommon. According to the agreement, tdd-UL-DL-ConfigDedicated-IAB-MT can override all symbols per slot over the number of slots as provided by tdd-UL-DL-ConfigurationCommon. However, this was not captured in current specification, i.e., only flexible symbols can be overridden by tdd-UL-DL-ConfigDedicated-IAB-MT in Rel-16. To increase the opportunity of simultaneous operation, the limitation of the overriding rule in Rel-16 should be removed in Rel-17. - In Rel-16, when tdd-UL-DL-ConfigDedicated-IAB-MT is provided, IAB-MT follows the UE behaviors by replacing tdd-UL-DL-ConfigurationDedicated with tdd-UL-DL-ConfigDedicated-IAB-MT. However, if tdd-UL-DL-ConfigurationDedicated-IAB-MT can override all symbols in Rel-17, the behavior of IAB-MT with tdd-UL-DL-ConfigurationDedicated-IAB-MT should be revisited. Retaining the constraint for tdd-UL-DL-ConfigurationDedicated-IAB-MT can dramatically reduce the opportunities for simultaneous operations. To solve this issue, the collision between cell-specific signals/channels and tdd-UL-DL-ConfigurationDedicated-IAB-MT should be allowed. In the slots with the collision, the IAB node should ignore tdd-UL-DL-ConfigurationDedicated-IAB-MT and it should be able to transmit/receive the cell-specific signals/channels. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the functional description of 38.213 on TDD-UL-DL-ConfigDedicated-IAB-MT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IAB node can only operate with simultaneous Tx/Rx operations on a limit set of resources, i.e. flexible symbols configured by TDD-UL-DL-ConfigCommon with sequence order UL-Flexible-DL. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated Impact Analysis: No inter-operatability issue is identified. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 14	Integrated access-backhaul operation

< Unchanged parts are omitted >

For each serving cell, an IAB-MT can be provided an indication for a slot format over a number of slots by tdd-UL-DL-ConfigurationDedicated-IAB-MT. If the IAB-MT is provided tdd-UL-DL-ConfigurationDedicated-IAB-MT, the statements in clause 11.1 that include "tdd-UL-DL-ConfigurationDedicated" apply to the IAB-MT of an IAB node by replacing "tdd-UL-DL-ConfigurationDedicated" with "tdd-UL-DL-ConfigurationDedicated-IAB-MT" for the IAB-MT, except that the tdd-UL-DL-ConfigurationDedicated-IAB-MT provides

-	a set of slot configurations by slotSpecificConfigurationsToAddModList-IAB-MT

-	for each slot configuration from the set of slot configurations

-	a slot index for a slot provided by slotIndex

-	a set of symbols for a slot by symbols-IAB-MT where

-	if symbols-IAB-MT = allDownlink, all symbols in the slot are downlink

-	if symbols-IAB-MT = allUplink, all symbols in the slot are uplink

-	if symbols-IAB-MT = explicit, nrofDownlinkSymbols provides a number of downlink first symbols in the slot and nrofUplinkSymbols provides a number of uplink last symbols in the slot. If nrofDownlinkSymbols is not provided, there are no downlink first symbols in the slot and if nrofUplinkSymbols is not provided, there are no uplink last symbols in the slot. The remaining symbols in the slot are flexible.

-	if symbols-IAB-MT = explicit-IAB-MT, nrofUplinkSymbols provides a number of uplink first symbols in the slot and nrofDownlinkSymbols provides a number of downlink last symbols in the slot. If nrofUplinkSymbols is not provided, there are no uplink first symbols in the slot and if nrofDownlinkSymbols is not provided, there are no downlink last symbols in the slot. The remaining symbols in the slot are flexible.

-	the parameter tdd-UL-DL-ConfigurationDedicated-IAB-MT overrides all symbols per slot over the number of slots as provided by tdd-UL-DL-ConfigurationCommon.

-	the parameter tdd-UL-DL-ConfigurationDedicated-IAB-MT does not apply

-	for a set of symbols of a slot that are indicated to an IAB-MT for reception of SS/PBCH blocks, or

-	for a set of symbols of a slot that are indicated to an IAB-MT for a CORESET for Type0-PDCCH CSS set, the IAB node should ignore tdd-UL-DL-ConfigurationDedicated-IAB-MT, or

-	for operation on a single carrier in unpaired spectrum, for a set of symbols of a slot corresponding to a PRACH occasion and  Ngap symbols before the valid PRACH occasion

< Unchanged parts are omitted >