3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2209855

# e-meeting, Oct 10-19 ,2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 38.213 on mixed Rel-16 and 17 SSSGS combinations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The spec seems to allow possibility of configuring Rel-17 SSSG switching timer for legacy DCI format 2_0 based SSSG switching. For a UE configured with Rel-17 SSSG switching, the following behavior should not apply as it leads to incorrect SSSG switching. - “Rel-16 SSSG switching behaviour when UE is not provided SearchSpaceSwitchTrigger” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that Rel-16 SSSG switching timer is used for legacy DCI format 2_0 based SSSG switching. Clarify that the Rel-16 behaviour when UE is not provided SearchSpaceSwitchTrigger applies “when UE is provided searchSpaceGroupIdList-r16”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 10.4	Search space set group switching and skipping of PDCCH monitoring

A UE can be provided a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList for PDCCH monitoring on a serving cell. If the UE is not provided searchSpaceGroupIdList for a search space set, the following procedures are not applicable for PDCCH monitoring according to the search space set.

If a UE is provided cellGroupsForSwitchList, indicating one or more groups of serving cells, the following procedures apply to all serving cells within each group; otherwise, the following procedures apply only to a serving cell for which the UE is provided searchSpaceGroupIdList.

When a UE is provided searchSpaceGroupIdList, the UE resets PDCCH monitoring according to search space sets with group index 0, if provided by searchSpaceGroupIdList.

A UE can be provided by searchSpaceSwitchDelay a number of symbols Pswitch where a minimum value of Pswitch is provided in Table 10.4-1 for UE processing capability 1 and UE processing capability 2 and SCS configuration μ. UE processing capability 1 for SCS configuration μ applies unless the UE indicates support for UE processing capability 2.

Table 10.4-1: Minimum value of Pswitch [symbols]

| μ | Minimum Pswitch value for UE processing capability 1 [symbols] | Minimum Pswitch value for UE processing capability 2 [symbols] |
| --- | --- | --- |
| 0 | 25 | 10 |
| 1 | 25 | 12 |
| 2 | 25 | 22 |
| 3 | 40 | - |
| 5 | 160 | - |
| 6 | 320 | - |

A UE can be provided, by searchSpaceSwitchTimer, a timer value for a serving cell that the UE is provided searchSpaceGroupIdList or, if provided, for a set of serving cells provided by cellGroupsForSwitchList. The UE decrements the timer value by one after each slot based on a reference SCS configuration that is the smallest SCS configuration μ among all configured DL BWPs in the serving cell, or in the set of serving cells. The UE maintains the reference SCS configuration during the timer decrement procedure.

If a UE is provided by SearchSpaceSwitchTrigger a location of a search space set group switching flag field in a DCI format 2_0, as described in clause 11.1.1, for a serving cell where the UE has active DL BWP with SCS configuration μ

-	if the UE detects a DCI format 2_0 and a value of the search space set group switching flag field in the DCI format 2_0 is 0, the UE starts monitoring PDCCH according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, for the serving cell

-	at the beginning of the first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0 when μ∈0, 1, 2, 3

-	at the beginning of the first slot, of a group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0 when μ∈5, 6

-	if the UE detects a DCI format 2_0 and a value of the search space set group switching flag field in the DCI format 2_0 is 1, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, for the serving cell

-	at the beginning of the first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0, when μ∈0, 1, 2, 3

-	at the beginning of the first slot, of a group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0 when μ∈5, 6

and the UE sets the timer value to the value provided by searchSpaceSwitchTimer-r16

-	if the UE monitors PDCCH for a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH for the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, for the serving cell

-	at the beginning of the first slot that is at least Pswitch symbols after a slot where the timer expires or after a last symbol of a remaining channel occupancy duration for the serving cell if indicated by DCI format 2_0 when μ∈0, 1, 2, 3

-	at the beginning of the first slot, of a group of Xs slots, that is at least Pswitch symbols after a slot where the timer expires or after a last symbol of a remaining channel occupancy duration for the serving cell if indicated by DCI format 2_0 when μ∈5, 6

If a UE is provided searchSpaceGroupIdList-r16 and is not provided SearchSpaceSwitchTrigger for a serving cell,

-	if the UE detects a DCI format by monitoring PDCCH according to a search space set with group index 0, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, for the serving cell

-	at the beginning of the first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format when μ∈0, 1, 2, 3,

-	at the beginning of the first slot, of a group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format when μ∈5, 6

the UE sets the timer value to the value provided by searchSpaceSwitchTimer if the UE detects a DCI format by monitoring PDCCH in any search space set

-	if the UE monitors PDCCH for a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH for the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, for the serving cell

-	at the beginning of the first slot that is at least Pswitch symbols after a slot where the timer expires or, if the UE is provided a search space set to monitor PDCCH for detecting a DCI format 2_0, after a last symbol of a remaining channel occupancy duration for the serving cell if indicated by DCI format 2_0 when μ∈0, 1, 2, 3

-	at the beginning of the first slot, of a group of Xs slots, that is at least Pswitch symbols after a slot where the timer expires or, if the UE is provided a search space set to monitor PDCCH for detecting a DCI format 2_0, after a last symbol of a remaining channel occupancy duration for the serving cell if indicated by DCI format 2_0 when μ∈5, 6

<omit unchanged text>