3GPP TSG RAN WG1 #110bis-e			     R1-2209873

e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on condition to be UE-A for SL IUC scheme 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current specification, a UE with higher priority value is set as UE-B when the two UEs scheduling the conflicting TBs support UE-B behavior of scheme 2 (and with further conditions). However, an important case is missed: only either of the two UEs supports UE-B behavior of scheme 2 and the other UE does not support. Rather, this situation is the motivation to introduce ‘Conflict information receiver flag’ field; thus, the correction is an essential feature. That is, when only either of the two UEs indicating overlapping resource(s) supports scheme 2, UE-A sets the capable UE as UE-B. For whole procedure, UE-A should firstly apply the above capability-based UE-B determination and then the priority-based UE-B determination; otherwise, i.e. in the opposite order, none of UE might be set as UE-B. There was an argument that collision indication might be sent to UE transmitting a TB with lower priority, which leads to worse resource efficiency. However, this argument is invalid definitely. Firstly, when the two UEs do not detect each other, which is referred as hidden-node issue and is one of the main situations to be solved in this agenda, there is no impact on resource efficiency and better reliability/latency can be obtained. Secondly, this agenda focuses on better reliability and latency. It is quite strange if we aim to better efficiency at a cost of reliability/latency. Of course, both UEs might perform reselection due to conflict indicator and pre-emption, but this should be OK for better reliability/latency. Thirdly, appropriate (pre-)configuration of collision detection, e.g. SIR-based conflict detection, can handle such a non-preferred situation from resource efficiency perspective. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Condition 2-A-1 in Scheme 2, when sl-TypeUE-A is enabled or when sl-TypeUE-A is disabled and the destination UE of the conflicting TBs is UE-A, if there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and ‘Conflict information receiver flag’ field is set to 1 if sl-IndicationUE-B is (pre)configured to ‘Enabled’, UE-A sets the UE as UE-B. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Resource conflict in much less situations can be detected and as the result inter-UE coordination scheme 2 achieves little gain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 16.3.0	UE procedure for transmitting PSFCH with control information

<Unchanged parts omitted>

A first UE determines a UE for providing the conflict information to in a PSFCH as follows

-	if, for a resource pool, typeAUEScheme2 is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception,

-	detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE,

-	detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and

-	determines that the first and second resources overlap in time and frequency

-	the PSFCH occasions for resource conflict information of the second UE and the third UE are valid

-	determines that one of the following conditions are met

-	Conflict information receiver flag field in SCI Format 1-A from the second UE is set to 1, Conflict information receiver flag field in SCI Format 1-A from the third UE is set to 0, and if sl-IndicationUE-B = 'enabled'

-	the indicationUEB flag Conflict information receiver flag field in SCI Format 1-A from the second UE and the third UE is set to 1, p2<p1, and if indicationUEBScheme2 sl-IndicationUE-B = 'enabled'

-	p2<p1 if sl-IndicationUE-B = 'disabled'

-	determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information

-	determines to transmit to the second UE the PSFCH with the conflict information

-	determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1

<Unchanged parts omitted>