3GPP TSG RAN WG1 #110bis-e			R1-2209897

e-Meeting, October 10th – 19th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on AI/ML for CSI feedback enhancement

Agenda Item:	9.2.2.2

Document for: 	Discussion and Decision

- Introduction

At the RAN#94-e meeting, a new SID [1] on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” was approved. This SID captures the objective of SI in terms of potential specification impacts as following.

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference),  and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

  - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2

    - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable

    - Consider the need and implications for AI/ML processing capabilities definition

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference),  and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

  - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2

    - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable

    - Consider the need and implications for AI/ML processing capabilities definition

In this contribution, we discuss sub use-cases and potential specification impacts on AI/ML for CSI feedback enhancements.

- Discussion on sub use-cases and potential specification impacts on AI/ML for CSI feedback enhancements

  - Representative sub use-cases

At the RAN1#109-e meeting, spatial-frequency domain CSI compression using two-sided AI models, as known as auto-encoder of CSI feedback was agreed to be a representative sub use case as following [2].

Agreement

Spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case.

- Note: Study of other sub use cases is not precluded.

- Note: All pre-processing/post-processing, quantization/de-quantization are within the scope of the sub use case.

Agreement

Spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case.

- Note: Study of other sub use cases is not precluded.

- Note: All pre-processing/post-processing, quantization/de-quantization are within the scope of the sub use case.

Fig. 1 illustrates the framework of CSI compression with two-sided models. As shown in Fig. 1, UE is equipped with an AI/ML encoder to compress CSI into encoded bits, while the corresponding AI/ML decoder is deployed on gNB to reconstruct CSI from encoded bits. In CSI compression with two-sided models, UE calculates downlink CSI, such as channel matrix or precoding matrix, and feeds the CSI into the encoder for compression. After the AI/ML encoder extracts essential features and outputs the encoded bits, UE reports the encoded bits to gNB where CSI can be reconstructed from encoded bits with the AI/ML decoder. In this contribution, the inputs of encoder and outputs of decoder are assumed to be the same.

Figure 1. The framework of spatial-frequency domain CSI compression with two-sided models.

With this AI/ML-based compression technique, accuracy improvements under a certain overhead of CSI reports and overhead reduction for CSI reports achieving a certain performance are observed [3]. In addition to spatial-frequency domain CSI compression, other sub use cases were proposed with the expected gain at the RAN1#110 meeting [2]. However, some negative views were observed in all sub use cases other than spatial-frequency domain CSI compression at the RAN1#110 meeting. Hence, given that time is limited for finalizing sub use cases, we would rather prioritize spatial-frequency domain CSI compression in this agenda than other sub use cases.

Proposal 1: Prioritize the discussion of spatial-frequency domain CSI compression from other sub-use case in 9.2.2.

In the subsequent sections, we discuss the spatial-frequency domain CSI compression.

  - Training procedure for spatial-frequency domain CSI compression

Agreement

In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied:

- Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided.

- Type 2: Joint training of the two-sided model at network side and UE side, repectively.

- Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively.

- Note: Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation. Joint training could be done both at single node or across multiple nodes (e.g., through gradient exchange between nodes).

- Note: Separate training includes sequential training starting with UE side training, or sequential training starting with NW side training [, or parallel training] at UE and NW

- Other collaboration types are not excluded.

Agreement

In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied:

- Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided.

- Type 2: Joint training of the two-sided model at network side and UE side, repectively.

- Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively.

- Note: Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation. Joint training could be done both at single node or across multiple nodes (e.g., through gradient exchange between nodes).

- Note: Separate training includes sequential training starting with UE side training, or sequential training starting with NW side training [, or parallel training] at UE and NW

- Other collaboration types are not excluded.

At the RAN1#110 meeting, the following potential training procedures were captured to deploy an encoder and a decoder at UE and NW, respectively [2].

・Type 1: Joint training of two-sided model at a single side/entity

・Type 2: Joint training of the two-sided model at network side and UE side, respectively.

・Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively.

In this contribution, we assume that separate training is sequential training, because the only sequential training is observed as feasible separate training procedure with simulation results as far as we know.

The above three types of training procedures affect the signaling overhead and the performance of AI/ML two-sided models. In sequential training, the model on one side is trained based on the dataset collected from the trained model on the other side. In other words, the model on one side is trained to mimic the model jointly trained on the other side. Thus, joint training provides better performance than separate training, because the performance of joint training is the upper bound of separate training.

Observation 1: Joint training provides better performance than separate training.

Also, the required overhead is different according to the training procedure. The required signaling overhead for each training procedure is evaluated in our companion contribution [3]. As Type2 and Type3 require dataset transfer from one side to the other, the signaling overhead is expected to be large.

Observation 2: Type 2 and Type 3 training procedure requires large signalling overhead due to the dataset transfer from one side to the other.

From performance and overhead perspective, Type 1 training procedure is the most reasonable one in our view. The next discussion point could be if the Type 1 training procedure has specification impact or not. If the model delivery is specified in 3GPP, a lot of specification impacts are expected in 3GPP. In addition, proprietary and feasibility concerns against the 3GPP signalling model delivery were brought up. Hence, it is reasonable to consider model delivery without 3GPP signalling as the first step in Rel-18 AI/ML. Specifically, it can be assumed that the offline agreements are made between multiple UE and NW vendors outside 3GPP before the model delivery. In that case, the proprietary issues can be solved without specification enhancement efforts. Therefore, we prefer to consider Type 1 training procedure with model delivery outside 3GPP in Rel-18 AI/ML as baseline, where offline agreements are made between multiple vendors before the model delivery.

Proposal 2: Consider Type 1 training procedure with model delivery outside 3GPP in Rel-18 AI/ML as baseline, where offline agreements are made between multiple vendors before the model delivery.

  - Inputs of encoder / Outputs of decoder

In this contribution, the inputs of encoder and outputs of decoder are assumed to be the same. At the RAN1#109-e meeting, two CSI types were proposed as the input of encoders and the output of model inference by decoders as follows: channel matrix and eigenvector(s) [2][4].

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the details of their models, including:

- The structure of the AI/ML model, e.g., type (CNN, RNN, Transformer, Inception, …), the number of layers, branches, real valued or complex valued parameters, etc.

- The input CSI type, e.g., raw channel matrix estimated by UE, eigenvector(s) of the raw channel matrix estimated by UE, etc.

  - FFS: the input CSI is obtained from the channel with or without analog BF

- The output CSI type, e.g., channel matrix, eigenvector(s), etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

Agreement

In CSI compression using two-sided model use case, further study potential specification impact on CSI report, including at least

- CSI generation model output and/or CSI reconstruction model input, including configuration(size/format) and/or potential post/pre-processing of CSI generation model output/CSI reconstruction model input.

- CQI determination

- RI determination

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the details of their models, including:

- The structure of the AI/ML model, e.g., type (CNN, RNN, Transformer, Inception, …), the number of layers, branches, real valued or complex valued parameters, etc.

- The input CSI type, e.g., raw channel matrix estimated by UE, eigenvector(s) of the raw channel matrix estimated by UE, etc.

  - FFS: the input CSI is obtained from the channel with or without analog BF

- The output CSI type, e.g., channel matrix, eigenvector(s), etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

Agreement

In CSI compression using two-sided model use case, further study potential specification impact on CSI report, including at least

- CSI generation model output and/or CSI reconstruction model input, including configuration(size/format) and/or potential post/pre-processing of CSI generation model output/CSI reconstruction model input.

- CQI determination

- RI determination

The information related to channel matrix or eigenvector(s) has been already specified as PMI in the specification. However, PMI is not qualified to be the inputs and outputs of CSI compression, because PMI was designed to reduce the overhead of CSI report and some channel information is lost to represent PMI by the quantization. If PMI is replaced by encoded bits based on channel matrix or eigenvector(s), some specification enhancements are necessary. Also, these potential specification impacts are different according to CSI type for inputs/outputs. As compatibility with the existing 5G NR is important, these expected specification impacts should be considered in CSI type discussion as well as the simulation results. Hence, potential specification impacts brought per each CSI type for input/output should be studied.

Proposal 3: Study the potential specification impacts according to each CSI type for input/output.

Table 1 summarizes the pros and cons of CSI types for input/output. Since the current CSI reporting only covers information related to precoding matrix, there would be some parts that existing frameworks can be reused when eigenvector(s) are CSI type inputs/outputs. On the other hand, channel matrix as CSI type for input/output requires a new mechanism such as normalization of a channel matrix. Therefore, eigenvector(s) seems more reasonable as CSI type for input/output for spatial-frequency domain CSI compression in terms of the specification impacts. On the other hand, if DL raw channel matrix is available at NW, this information might be useful for other usages in addition to precoding matrix calculation.

Observation 3: The existing framework can be reused to some extent if CSI type for input/output is eigenvector(s) for spatial-frequency domain CSI compression.

Observation 4: If CSI type for input/output is raw channel matrix for spatial-frequency domain CSI compression, DL CSI might be useful for other usages in addition to precoding matrix calculation.

Table 1.  Summary of input/output CSI type for spatial-frequency domain CSI compression.

| reconstructed CSI | Pros | Cons |
| --- | --- | --- |
| Eigenvector | Similar to the existing CSI feedback framework (e.g., can reuse mechanism of CQI/RI) |  |
| Channel matrix | Might be beneficial for other usages in addition to precoding matrix calculation | Require new mechanism (e.g., normalization) |

Even though the most of the existing CQI frameworks can be reused for CSI when the reconstructed CSI type is eigenvector, some enhancements are necessary so that CQI is applicable to the CSI compression. For example, the CQI is calculated at UE assuming that the precoding matrix reported by PMI is applied. If encoded bits information are replaced with PMI, what precoding matrix is assumed in CQI calculation needs to be specified for CSI compression scenarios. Also, unless the decoder is compiled at UE in addition to the encoder, the reconstructed CSI is not available to UE. In that case, UE does not fully understand the precoding matrix reconstructed at gNB. Likewise, CQI enhancements might be necessary so that CQI calculation is applicable to the CSI compression.

Observation 5: CQI enhancements might be necessary so that CQI calculation is applicable to the CSI compression.

It is also under the discussion regarding whether input of encoder, output of decoder and pre/post processing should be specified in the specification or not. As discussed in the previous section (Sec.2.2), we think the offline multi-vendor agreements outside 3GPP should be assumed for the two-sided model delivery. In that scenario, the input of encoder, output of decoder and pre/post processing can be known between UE side and NW side outside 3GPP. Moreover, the pre/post processing can be viewed as one of model inference process, which is implementation specific as AI algorithm and model. If model information should be proprietary assets, pre/post processing should also be treated as proprietary assets because this information is closely related to the model structure. Therefore, it is not necessary to specify the input of encoder, output of decoder and pre/post processing in the specification, unless some technical issues are observed.

Observation 6: If the offline multi-vendor agreement is assumed for the model delivery of two-sided models, the input of encoder, output of decoder, and pre/post-processing can be known between UE side and NW side outside 3gpp

Observation 7: Pre-/post-processing can be views as one of model inference process, which is implementation specific as AI algorithm and model.

Proposal 4: It is not necessary to specify input of encoder, output of decoder and pre/post processing in the specification, unless some technical issues are observed.

  - Performance monitoring for spatial-frequency domain CSI compression

As discussed in our companion contribution in AI 9.2.1, (near) real time model performance should be available at NW for reliable NW operation with AI models, and model monitoring can be categorized into two types: NW-based model monitoring and UE-based model monitoring [5].

One example of NW-based model monitoring is illustrated in Fig. 2. As shown in this example, UE reports results of model inference, e.g., encoded bits, and the target values of model inference, e.g., input CSI, so that NW can monitor how accurate AI model is based on comparison between reconstructed CSI and the target CSI in NW-based model monitoring. The disadvantage of NW-based model monitoring could be large overhead of non-AI-based CSI feedback and quantization error caused by non-AI-based CSI feedback.

Figure 2. One example of NW-based model monitoring for spatial-frequency domain CSI compression.

Fig.3 shows one example of UE-based model monitoring. In UE-based model monitoring, UE monitors the (near) real time performance and reports the monitored performance to NW. In the example of Fig.3, UE has the paired decoder to output the reconstructed CSI. In this case, UE can calculate the model accuracy by comparing the target CSI with reconstructed CSI at real time. However, it requires UE to compile the paired decoder in addition to the encoder, which could be the burden in terms of storage perspective. Likewise, there are multiple aspects to be considered in model monitoring specific to spatial-frequency domain CSI compression. For further analysis, NW-based model monitoring and UE-based model monitoring should be studied, respectively.

Figure 3. One example of UE-based model monitoring for spatial-frequency domain CSI compression.

Proposal 5: Study NW-based model monitoring and UE-based model monitoring specific to spatial-frequency domain CSI compression.

  - Encoded bit reporting

The mechanisms to report encoded bits as UCI need to be specified in the specification. In the current specification, some CSI is reported only on PUSCH according to the codebook type and band granularity. The similar discussion is necessary for reporting encoded bits as well. On top of it, when UE compiles multiple AI/ML models achieving different compression ratios, static/semi-static/dynamic ML model activation according to channel quality can bring the gain. Furthermore, it could be beneficial to switch applying AI/ML approach or conventional CSI reports according to the performance of AI/ML models. In that case, the determination mechanisms on what UCI bits UE shall report, as depicted in Fig.4, need to be defined too.

Figure 4. Determination mechanisms on what UCI bits UE shall report.

Observation 8: The mechanisms in CSI reports, such as determination mechanisms on reported UCI bits and how to report encoded bits, could be specification impacts.

- Conclusion

In this contribution, we discussed the sub use-cases and potential specification impacts on AI/ML for CSI feedback enhancements. Based on the discussion we made the following observations and proposals.

Observation 1: Joint training provides better performance than separate training.

Observation 2: Type 2 and Type 3 training procedure requires large signalling overhead due to the dataset transfer from one side to the other.

Observation 3: The existing framework can be reused to some extent if CSI type for input/output is eigenvector(s) for spatial-frequency domain CSI compression.

Observation 4: If CSI type for input/output is raw channel matrix for spatial-frequency domain CSI compression, DL CSI might be useful for other usages in addition to precoding matrix calculation.

Observation 5: CQI enhancements might be necessary so that CQI calculation is applicable to the CSI compression.

Observation 6: If the offline multi-vendor agreement is assumed for the model delivery of two-sided models, the input of encoder, output of decoder, and pre/post-processing can be known between UE side and NW side outside 3gpp

Observation 7: Pre-/post-processing can be views as one of model inference process, which is implementation specific as AI algorithm and model.

Observation 8: The mechanisms in CSI reports, such as determination mechanisms on reported UCI bits and how to report encoded bits, could be specification impacts.

Proposal 1: Prioritize the discussion of spatial-frequency domain CSI compression from other sub-use case in 9.2.2.

Proposal 2: Consider Type 1 training procedure with model delivery outside 3GPP in Rel-18 AI/ML as baseline, where offline agreements are made between multiple vendors before the model delivery.

Proposal 3: Study the potential specification impacts according to each CSI type for input/output.

Proposal 4: It is not necessary to specify input of encoder, output of decoder and pre/post processing in the specification, unless some technical issues are observed.

Proposal 5: Study NW-based model monitoring and UE-based model monitoring specific to spatial-frequency domain CSI compression.

References

- Moderator (Qualcomm), RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Dec. 2021.

- Moderator (Apple), R1-2208077 “Summary #4 on other aspects of AI/ML for CSI enhancement”, Aug. 2022.

- NTT DOCOMO, INC., R1-2209896, “Discussion on evaluation on AI/ML for CSI feedback enhancements”, Oct. 2022.

- NTT DOCOMO, INC., R1-2207401, “Discussion on evaluation on AI/ML for CSI feedback enhancements”, Aug. 2022.

- NTT DOCOMO, INC., R1-2209895, “Discussion on general aspects of AI/ML framework”, Oct. 2022.