3GPP TSG RAN WG1 Meeting #110-Bis-e																R1-2209963

October 10th – 19th, 2022

Agenda item:	8.16.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on Rel-17 UE features topic 1

Document for:	Discussion/Decision

- Introduction

Updated RAN1 UE features list for Rel-17 NR after RAN1 #110 is captured in R1-2207923. In this contribution, we discuss our views on various remaining details of UE features on eIIoT & URLLC, RedCap, UE power saving, coverage enhancement, NB-IoT & eMTC, sidelink, MBS, 5G terrestrial broadcast, UL TX switching, and SDT.

- Coverage Enhancements

We suggest the following changes:

Proposal 2-1: Add the following constraint to 30-4h (non-back-to-back case):

“Note: This capability is only applicable when UE is configured with single uplink carrier.”

Proposal 2-2: Add the following constraints to UE capability FGs 30-4a, 4b, 4c, and 4d (the four back-to-back cases):

“Note: This capability is applicable in the following multi-carrier scenarios:

- FR1+FR2 CA/DC with one band on FR1 and another band on FR2. DMRS bundling configuration is limited to one uplink NR carrier in total on all FRs at a time.

- SUL

- FR1+FR1 Inter-band CA/DC involving two or more FR1 bands where UE does not expect concurrent uplink transmissions on any two carriers

All carriers must belong to a single TAG”

Proposal 2-3: UE capability FGs 30-4a, 4b, 4c, and 4d are signaled at per band and per band combination granularity.

# MBS

## FG33-1 and 33-1-x

We suggest minor changes on FG 33-1 as:

| 33. NR_MBS | 33-1 | Broadcast | 1. Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. 2. Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI(s) for MTCH. 3. Support of CFR configuration for broadcast. 4. Support of CORESET and common search space for broadcast. 5. Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. 6. Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. 7. Support MCCH change notification indication via DCI. 8. support of higher layer configured slot-level repetition up to 8 for MTCH 9. support of FDMed MCCH and PBCH 10. support of up to 64QAM for FR1/FR2 |  | Up to RAN2 |  |  | Up to RAN2 | Up to RAN2 | Up to RAN2 |  | It is up to RAN2 whether/how to introduce the capability for support of N > 1 G-RNTIs for broadcast for a UE | Up to RAN2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1-1 | DCI indicated slot-level repetition up to 16 for broadcast MTCH | Support up to 16 times dynamic slot-level repetition for broadcast MTCH. | 33-1 | Up to RAN2 |  |  | Up to RAN2 | Up to RAN2 | Up to RAN2 |  |  | Up to RAN2 |
| 33. NR_MBS | 33-1-2 | FDM-ed unicast PDSCH and group-common PDSCH for broadcast | 1. Support FDM between one unicast PDSCH and one group-common PDSCH for broadcast in RRC CONNECTED mode in a slot. | 33-1 | Yes |  |  | FFS | FFS | FFS |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-1-3 | Support of 256QAM for broadcast PDSCH in FR1 | 1. For FR1, up to 256QAM is supported | 33-1 | Yes |  |  | Per band | N/A | N/A |  |  | Optional with capability signalling |

## FG33-2 and FG33-2x

We suggest the following changes for the remaining FFSs on FG 33-2 and FG33-2x as:

| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast for PCell | 1. Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for PCell. 2. Support of CFR configuration for multicast. 3. Support of CORESET and common search space configuration for multicast. 4. Support of DCI format 4_1 with CRC scrambled with G-RNTI for multicast. 5. Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. 6. Support {2, 4, 8} times semi-static slot-level repetition for group-common PDSCH for multicast |  | Yes |  |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-2a | Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling 2) Support of PTM retransmission for multicast 3) support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only 4) Support of shared PUCCH resource configurations with unicast | 33-2 | Yes |  |  | Per BC | N/A | N/A |  | Note: A UE is not expected to be scheduled with unicast and multicast HARQ-ACK feedback with same priority in the same PUCCH if UE does not support FG33-3-3a or 33-3-3b. | Optional with capability signalling |
| 33. NR_MBS | 33-2b | DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling via DCI format 4_2 | 33-2a, 33-2f | Yes |  |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2d | PTP retransmission for multicast dynamic scheduling | Support of PTP retransmission for multicast on the same cell as multicast initial transmission | 33-2a | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2e | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for multicast | 33-2 | Yes |  |  | Per UE | [Yes] | Yes |  | Reporting type of FG 33-2e is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2 | Optional with capability signalling |
| 33. NR_MBS | 33-2f | Dynamic multicast with DCI format 4_2 | Support of DCI format 4_2 with CRC scrambled with G-RNTI for multicast | 33-2 | Yes |  |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2g | MIMO layers for multicast PDSCH | Supported maximal number of MIMO layers for multicast PDSCH | 33-2 | Yes |  | UE supports 1 MIMO layer only for multicast PDSCH | Per FSPC | N/A | N/A |  | Candidate values: {2,4,8} Note: If UE supports up to 8 layers, the UE supports TB2 | Optional with capability signalling |
| 33. NR_MBS | 33-2h | Dynamic scheduling for multicast for SCell | Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for SCell. | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  | Note: A UE is not expected to be configured simultaneously with more than one component carrier for multicast reception | Optional with capability signalling |
| 33. NR_MBS | 33-2i | Supported maximal modulation order for multicast PDSCH | 1. For FR1, up to 1024QAM is supported, candidate values {256QAM, 1024QAM} 2. For FR2, up to 256QAM is supported, candidate values {64QAM, 256QAM} | 33-2 | Yes |  | The UE supports the same modulation order as unicast | Per band | N/A | N/A |  | Note: A UE shall support the corresponding mandatory maximum modulation for unicast. | Optional with capability signalling |
| 33. NR_MBS | 33-2j | Supported maximum modulation order used for maximum data rate calculation for multicast PDSCH | 1. For FR1, up to 1024QAM is supported as maximum modulation order used for maximum data rate calculation for multicast PDSCH, candidate values {256QAM, 1024QAM} 2. For FR2, up to 256QAM is supported as maximum modulation order used for maximum data rate calculation for multicast PDSCH, candidate values {64QAM, 256QAM} | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |

## FG33-3-3-x

We suggest the following changes for the remaining FFSs of FG 33-3-x and add FG33-3-2a for additional scaling factor for max data rate and TBS LBRM to support FDMed unicast PDSCH and multicast PDSCH, where details of the additional scaling factor are discussed in [2].

| 33. NR_MBS | 33-3-1 | Dynamic Slot-level repetition for group-common PDSCH | 1. Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast. | 33-2 | Yes |  |  | Per UE | Yes | Yes |  | Candidate values for X is: {8, 16} This FG is reported for TN and licensed. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-1a | Dynamic Slot-level repetition for group-common PDSCH for NTN and unlicensed | 1. Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast for NTN and unlicensed | 33-2 | Yes |  |  | Per band | N/A | N/A |  | Candidate values for X is: {8, 16} This FG is reported for NTN and unlicensed | Optional with capability signalling |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH for multicast | 1. Support FDM between one unicast PDSCH and one group-common PDSCH for multicast in RRC CONNECTED mode in a slot. | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-2a | Scalng factor for maximum data rate and TBS LBRM of FDMed unicast PDSCH and group-common PDSCH | Scaling factor for max data rate and TBS LBRM to support FDMed unicast PDSCH and group-common PDSCH per CC. | 33-3-2, 33-1-2 | Yes |  | If not reported, same as the scaling factor for max data rate of unciast PDSCH | Per FSPC | N/A | N/A |  | value of scaling factor: {1.75, 1.5, 1, and 0.75} | Optional with capability signalling |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | 1. Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. 2. Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC 3. Support TDM among N (N>1) group-common PDSCHs in a slot per CC 4. Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC 5. The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). | 33-1, 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  | value of M+1: {2, 4, 7} value of N: {2, 4, 7} value of K+L: {2, 4, 7} Note: up to one broadcast PDSCH in a slot. | Optional with capability signalling |
| 33. NR_MBS | 33-3-3a | FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | 1. Support of FDM-ed Type-1 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast 2. Support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast with max number X of G-RNTIs | 33-2a, 33-3-2 | Yes |  |  | Per BC | N/A | N/A |  | Note1: FDM-ed Type-1 HARQ-ACK codebook is generated by concatenating the Type-1 sub-codebook for unicast and the Type-1 sub-codebook for multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. Candidate values of X is {2, 3, 4} with X no lareger than max number of G-RNTIs of FG33-2e | Optional with capability signalling |
| 33. NR_MBS | 33-3-3b | Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | 1. Support of Mode 2 TDM-ed Type-1 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast 2. Support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast with max number X of G-RNTIs | 33-2a | Yes |  |  | Per BC | N/A | N/A |  | Note1: Mode 2 TDM-ed Type-1 HARQ-ACK codebook is generated based on the union TDRA tables from unicast and multicast and the union of k1 sets from unicast and multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. Candidate values of X is {2, 3, 4} with X no lareger than max number of G-RNTIs of FG33-2e | Optional with capability signalling |
| 33. NR_MBS | 33-3-4 | Mode 1 for type1 codebook generation | Supports type1-Codebook-Generation-Mode configured as mode 1 | 33-3-3b | Yes |  |  | Per BC | N/A | N/A |  | This FG is for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast on PUCCH or PUSCH | Optional with capability signalling |
| 33. NR_MBS | 33-3-5 | Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook type | Support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot | 33-2a | Yes |  |  | per BC | N/A | N/A |  |  | Optional with capability signalling |

## FG33-4 and FG33-4-1

We suggest the following changes for the remaining FFSs of FG 33-4 and 33-4-x:

| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast with ACK/NACK transforming | 1. Support NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast, including: a) A single TB with NACK-only feedback transmitted in PUCCH b) One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits | 33-2a | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-4a | NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission | 1. Support NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast, including: a) Multiple TB with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 2. Support of separate PUCCH resource configurations from unicast 3. Extended PDSCH processing time by adding d3=N2 for different PRBs of PUCCHs configured for more than one NACK-only feedback | 33-4 | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-4-1 | DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling via DCI format 4_2 | 33-4, 33-2f | Yes |  |  | Per band | N/A | N/A |  |  | Optional with capability signalling |

## FG33-5-1 and FG33-5-1x

We suggest the following changes for the remaining FFSs of FG33-5-1, FG33-5-1x and FG33-5-2 as

| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | 1. Support one SPS group-common PDSCH configuration for multicast 2. Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH 3. Support of group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast 4. Support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast 5. ACK/NABK-based HARQ-ACK feedback for SPS group-common PDCCH activation and SPS release associated with G-CS-RNTI | 33-2 | Yes |  |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | 1. Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, SPS group-common PDSCH activation, and SPS release PDCCH 2. Support of PTM retransmission for SPS multicast associated with G-CS-RNTI 3. Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4. Support of shared SPS-PUCCH-AN-List with unicast | 33-5-1 | Yes |  |  | Per BC | N/A | N/A |  | Note: UE is not expected to be scheduled with unicast and multicast HARQ-ACK feedback with same priority in the same SPS-PUCCH-AN-List if UE does not support 33-5-1 | Optional with capability signalling |
| 33. NR_MBS | 33-5-1b | DCI-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling via DCI format 4_2 | 33-5-1a, 33-5-1i | Yes |  |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1d | PTP retransmission for SPS group-common PDSCH for multicast | Support of PTP retransmission associated with CS-RNTI for SPS multicast on the cell same as multicast initial transmission | 33-5-1a | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1e | Dynamic Slot-level repetition for SPS group-common PDSCH for multicast | Support up to X times dynamic slot-level repetition for SPS group-common PDSCH for multicast. | 33-5-1 | Yes |  |  | Per BC | N/A | N/A |  | Candidate values for X is: {8, 16} | Optional with capability signalling |
| 33. NR_MBS | 33-5-1f | NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling a) A single TB with NACK-only feedback transmitted in PUCCH b) One or multiple TBs with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits | 33-5-1a | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1g | DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling via DCI format 4_2 | 33-5-1f, 33-5-1i | Yes |  |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1h | Multiple G-CS-RNTIs for SPS group-common PDSCHs | Max number of G-CS-RNTIs for SPS multicast | 33-5-1 | Yes |  |  | Per UE | [Yes] | Yes |  | Reporting type of FGs 33-5-1h is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2 | Optional with capability signalling |
| 33. NR_MBS | 33-5-1i | Multicast SPS scheduling with DCI format 4_2 | Support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling | 33-5-1 | Yes |  |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1j | NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission for SPS group-commmon PDSCH for multicast | 1. Support NACK-only based HARQ-ACK feedback for SPS PDSCH for multicast, including: a) Multiple TBs with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 2. Support of separate SPS-PUCCH-AN-List from unicast 3. Extended PDSCH processing time by adding d3=N2 for different PRBs of PUCCHs configured for more than one NACK-only feedback | 33-5-1f | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | 1. Support up to 8 SPS group-common PDSCH configuration per CFR for multicast 2. Support M>=1 activated SPS group-common PDSCH configurations per CFR for multicast 3. The total number of SPS configurations for both multicast and unicast is no larger than 8 [per cell], and activated SPS group-common PDSCH configurations is no larger than M. | 33-5-1 | Yes |  |  | Per FS | N/A | N/A |  | Candidate value set for M is {1, 2, …, 8} | Optional with capability signalling |

## FG33-6-x

We suggest the following changes for the remaining FFSs of FG33-6-x as

| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 4_2 with CRC scrambled with G-RNTI for multicast. 2. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast and multicast at a UE | 33-2a, 33-2f | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-6-1a | DL priority configuration for SPS multicast | Support of priority indicator field configured in DCI formats 4_2 for multicast HARQ-ACK feedback of SPS multicast | 33-5-1a, 33-5-1i | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for unicast and multicast at a UE | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for unicast and multicast at a UE. | 33-6-1 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |

## FG33-8

We suggest the changes for the remaining FFSs of FG 33-8 as:

| 33. NR_MBS | 33-8-1 | PUCCH resource configuration for multicast feedback for dynamically scheduled multicast | Support of a PUCCH-Config for multicast HARQ-ACK feedback, separate from that of unicast configurations | 33-2a | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-8-2 | Up to 2 PUCCH resources configuration for multicast feedback for dynamically scheduled multicast | Support of a PUCCH-ConfigurationList for multicast HARQ-ACK feedback, separate from that of unicast configurations | 33-8-1, 33-6-1 | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-8-3 | PUCCH resource configuration for multicast feedback for SPS GC-PDSCH | Support of a SPS-PUCCH-AN-List for multicast HARQ-ACK feedback of all multicast SPS configuration(s), separate from that of SPS unicast configurations | 33-5-1a | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |

## FG33-9

We suggest the changes for the remaining FFSs of FG 33-9 as

| 33. NR_MBS | 33-9 | Supporting unicast PDCCH to release SPS group-common PDSCH | Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH | 33-5-1 | Yes |  |  | Per BC | N/A[No] | N/A[No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# URLLC

For FG 25-16, 12-1 should not be prerequisite for this FG. FG 12-1 is Rel-16 intra-UE prioritization. FG 25-16 is Rel-17 intra-UE multiplexing. A UE can choose to implement FG 25-16 direclty, without implementing FG 12-1. FGs 11-3 and 11-4a are for sub-slot PUCCH transmissions. They are not prerequisite for FG 25-16. The only prerequisite sounds reasonable is 11-4.

Proposal 4-2: Only 11-4 is set as prerequisite for FG 25-16. Remove “[11-3, 12-1]” on prerequisite for FG 25-16.

For FG 25-18, there is no strong need to set prerequisite for it.

Proposal 4-2: No prerequisite is set for FG 25-18. Remove the FFS on prerequisite for FG 25-18.

# References

- R1-2207923, Updated RAN1 UE features list for Rel-17 NR after RAN1 #110 Thursday, Moderators (AT&T, NTT DOCOMO, INC.).