3GPP TSG RAN WG1 Meeting #110-Bis-e																R1-2209964

October 10th – 19th, 2022

Agenda item:	8.16.2

Source: 	Qualcomm Incorporated

Title: 	Discussion on Rel-17 UE features topic 2

Document for:	Discussion/Decision

- Introduction

Updated RAN1 UE features list for Rel-17 NR after RAN1 #110 is captured in R1-2207923. In this contribution, we discuss our views on various remaining details of UE features on FeMIMO, NR from 52.6GHz to 71 GHz, NR-NTN, positioning, eIAB, DSS, IoT over NTN, 1024QAM.

- Multi-Beam Operation

Proposal 2-1: For FG 23-1-1 family, support to specify UE capabilities for separate DL/UL TCI states

- Separate FG for separate DL/UL TCI + intra-cell beam management

- Separate FG for separate DL/UL TCI + inter-cell beam management

- As starting point, the contents of above new FGs can be similar to FG 23-1-1, FG 23-1-1a, and FG 23-1-1b

- HST-SFN

In RAN1 meeting #106bis-e, there is an existing RAN1 agreement for an optional UE feature for the support DCI format 1_0 for scheduling PDSCH with either one TCI or two TCI states when scheduling offset is larger than the threshold. UE applies either both TCI states of the scheduling CORESET (i.e SFN PDSCH) or the apply the single TCI state of the CORESET (i.e., single TCI PDSCH).  This FG is missing in UE FG group for HST (FG 23-6) and should be reflected.

Agreement For PDSCH reception scheduled by DCI format 1_0, [1_1 and 1_2], if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL - Support configuration when there is no TCI field in the DCI scheduling PDSCH - UE applies the state(s) of the scheduling CORESET when receiving the PDSCH - if there are two active TCI states for the CORESET, UE applies the both QCL assumption of the CORESET that schedules the PDSCH when receiving the PDSCH - otherwise, UE applies the one active TCI state of the CORESET when receiving the PDSCH - FFS if the time offset between the reception of the DL DCI and the corresponding PDSCH is smaller than the threshold timeDurationForQCL This is a UE optional feature.

Proposal 3-1: Add FG 23-6-7 for the HST-SFN FGs to support UE determining a single or two TCI states of the PDSCH, scheduled by DCI format 1_0, based the scheduling CORESET whether the  CORESET is activated with one or two TCI states  when the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL

| 23. NR_FeMIMO | 23-6-7 | Support DCI format 1_0 scheduling PDSCH with single or two TCI states based on the scheduling CORESET when time offset is larger than the threshold | Support determining single TCI state or two TCI states for PDSCH scheduled by DCI format 1_0 based on the scheduling CORESET when time offset is larger than the threshold |  | Yes | N/A |  | Per band | n/a | n/a |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- References

- R1-2207923, Updated RAN1 UE features list for Rel-17 NR after RAN1 #110 Thursday, Moderators (AT&T, NTT DOCOMO, INC.).