3GPP TSG RAN WG1 Meeting #110b-e                                                               	     R1-2210020

e-Meeting, Oct 10th – 19th, 2022

Agenda Item:	8.14

Source:	Lenovo

Title:	Maintenance for IoT NTN

Document for:	Discussion and decision

- Introduction

In RAN1 #110 meeting, issues on dropping symbols/slots due to TA adjustment for uplink segmented transmission was discussed. In this contribution, we provide our views on the remaining issue for completion of Rel.17 IoT NTN.

- Discussion

In RAN1 #109-e meeting, the following agreement was achieved for UE symbols/slots dropping due to TA adjustment for uplink segmented transmission.

Agreement

- The single UE capability that governs UE behavior w.r.t gaps between segments for PUSCH, PUCCH and NPUSCH, when the UE performs segmented pre-compensation, is as follows:

- When a single capability is signalled: UE drops one or more of the following durations of uplink transmission between segments (indicated by the capability):

  - 1 slot (applicable to eMTC)

  - 1 subframe (applicable to eMTC)

  - 1 slot (applicable to NB-IoT)

  - 2 slots (applicable to NB-IoT)

  - 1 symbol (applicable to both eMTC and NB-IoT)

  - UE follows legacy behaviour at slot boundaries due to TA adjustment

- When capability is NOT signalled: UE follows legacy behaviour at slot boundaries due to TA adjustment

For the UE symbol/slots dropping due to TA adjustment for NPUSCH segmented transmission, it can follow the legacy UE behavior as TS36.213 Section 16.1.2.

TS36.213 16.1.2

When the UE's uplink NPUSCH transmissions in NB-IoT uplink slot n and NB-IoT uplink slot n+1 are overlapped due to the timing adjustment, the UE shall complete transmission of NB-IoT uplink slot n and not transmit the overlapped part of NB-IoT uplink slot n+1.

However, for the UE symbols/slots for NPRACH segmented transmission, the corresponding UE behavior has not been specified in current specification.  The segment of NPACH transmission duration for IoT NTN is defined with the unit of duration of one preamble transmission including guard period (TCP+TSEQ+TGP), and the dropping duration can be configured up to subframe/repetition unit, which can’t be absorbed by preamble CP or guard period.

TS36.331

PRACH-TxDuration-r17::=				SEQUENCE {

prach-TxDuration-r17		ENUMERATED {n1, n2, n4, n8, n16, n32, n64, n128}

}

Agreement

For NB-IoT, UE pre-compensation per segment of NPRACH is applied from one segment to the next segment by using one or more of the following methods if supported by UE implementation

- UE may drop / Insert samples

- UE may blank subframe / repetition unit where UE drops a subframe / repetition unit

The total transmission time is not changed

FFS Details of method(s) to drop / insert samples / blank subframe / repetition unit

FFS Specification impact

TP#1

TS36.213

16.1.2	Timing synchronization

Upon reception of a timing advance command, the UE shall adjust uplink transmission timing for NPUSCH, and SR if configured with higher layer parameter sr-WithoutHARQ-ACK-Config, based on the received timing advance command.

The timing advance command indicates the change of the uplink timing relative to the current uplink timing as multiples of 16. The start timing of the random access preamble is specified in [3].

In case of random access response, an 11-bit timing advance command [8], TA, indicates NTA values by index values of TA = 0, 1, 2, ..., 1536, where an amount of the time alignment is given by NTA = TA 16. NTA is defined in [3].

In other cases, a 6-bit timing advance command [8] or the Timing advance adjustment field in DCI format N0 if present [4], TA, indicates adjustment of the current NTA value, NTA,old, to the new NTA value, NTA,new, by index values of TA = 0, 1, 2,..., 63, where NTA,new = NTA,old + (TA 31)16. Here, adjustment of NTA value by a positive or a negative amount indicates advancing or delaying the uplink transmission timing by a given amount respectively.

For a timing advance command reception ending in DL subframe n, the corresponding adjustment of the uplink transmission timing shall apply from the first available NB-IoT uplink slot following the end of n+12 DL subframe and the first available NB-IoT uplink slot is the first slot of a NPUSCH transmission. When the UE's uplink NPUSCH transmissions in NB-IoT uplink slot n and NB-IoT uplink slot n+1 are overlapped due to the timing adjustment, the UE shall complete transmission of NB-IoT uplink slot n and not transmit the overlapped part of NB-IoT uplink slot n+1. When the UE's uplink NPRACH transmission in nth preamble transmission and (n+1)th preamble transmission are overlapped due to the timing adjustment, the UE shall complete transmission of nth preamble transmission and not transmit the overlapped part of (n+1)th preamble transmission.

If the received downlink timing changes and is not compensated or is only partly compensated by the uplink timing adjustment without timing advance command as specified in [10], the UE changes NTA accordingly.

Based on the above analysis, the following CR reference (e.g., TP#1) is proposed to clarify the UE dropping mechanism for NPRACH segmented transmission when transmission collision.

Proposal 1: Adopt TP#1 to TS36.213 to clarify the UE dropping rule for NPRACH segmented transmission when transmission collision.

Regarding the UE dropping rule for eMTC PUCCH and PUSCH segmented transmission when transmission collision, the legacy UE behavior can be reused. The legacy text in TS36.213 for UE behavior includes the dropping among different channels (e.g., between channels of PUSCH and PUCCH) and same channel (e.g., channel of PUSCH and PUSCH with repetitions). Note the corresponding TA update restriction of not adjusting uplink transmission timing during an ongoing repetition period other than at initial transmission in TS36.133 should be updated accordingly.

TS36.213

4.2.3	Transmission timing adjustments

For a BL/CE UE, for a timing advance command received on subframe n, the corresponding adjustment of the uplink transmission timing shall apply for the uplink PUCCH/PUSCH/SRS transmissions in subframe n+6+Koffset. When the BL/CE UE's uplink PUCCH/PUSCH/SRS transmissions in subframe n and subframe n+1 are on the same narrowband and are overlapped due to the timing adjustment, the UE shall complete transmission of subframe n and is not required to transmit in subframe n+1 until the first available symbol that has no overlapping portion with subframe n. When the BL/CE UE's uplink PUCCH/PUSCH/SRS transmissions in subframe n and subframe n+1 are on different narrowbands, and the timing adjustment occurs in the guard period for narrowband retuning, the UE is not required to transmit in subframe n+1 until the first available symbol that has no overlapping portion with subframe n and which does not reduce the guard period.

Regarding the UE dropping rule for eMTC PRACH segmented transmission when transmission collision, the potential dropping duration is OFDM sample level as the agreement. The dropping duration can be absorbed in the guard period of preamble, so no dropping mechanism is needed for PRACH segmented transmission.

Observation 1: No CR is needed for specifying the UE dropping rule for all uplink segmented transmission for eMTC.

- Conclusions

In this contribution, considerations of dropping symbols/slots due to TA adjustment for uplink segmented transmission are provided. The following proposals are given.

Proposal 1: Adopt TP#1 to TS36.213 to clarify the UE dropping rule for NPRACH segmented transmission when transmission collision.

Observation 1: No CR is needed for specifying the UE dropping rule for all uplink segmented transmission for eMTC.

- References

3GPP TR 36.763 V1.0.0 (2021-06)

R1-2208074 Summary #2 of AI 8.14Maintenance on NB-IoT/eMTC to support NTN: time and frequency synchronization	Moderator (MediaTek Inc.)