3GPP TSG RAN WG1 Meeting #110bis-e	R1-2210021

e-Meeting, October 10 – 19, 2022

Agenda item:	9.7.1

Source: 	Lenovo

Title: 	Performance evaluation for network energy saving

Document for:	Discussion

Introduction

During RAN1#110 meeting, significant progress has been achieved for the NES evaluation [1]. However, there are still some remaining issues. In this contribution, we discuss and provide our views on the evaluation for network energy saving, especially for the energy consumption model and evaluation assumptions.

Energy consumption model for BS

In [Post-110-R18- NW_ES2] email discussion [2], the agreements for energy consumption model have been achieved as shown below.

Agreement - The total transition time for set 2 and set 3 is the same as that for set 1. - Companies are encouraged to check the input and values provided in section 2.1.4.2 of R1-2208312 for further determination.

The relative power for reference configuration set 2 and set 3 is still a remaining issue. From R1-2208312, some companies provide their preference relative power and there is a proposed working assumption from FL shown in Table 1. Although the absolute powers for Set 1 and Set 2 should be different due to different configurations of TxRU number, bandwidth, and transmission power, it is feasible to assume the same relative power model for set 2 and set 1 for evaluation purpose. For the proposed relative power values, which are the average from preference values of companies and are almost the same as that of Set 1, we can accept proposal to move forward the following evaluations.

Table 1. The relative power values for reference configuration set 2 and set 3 proposed by FL

| Power state | Relative Power P for Category 1 |  | Relative Power P for Category 2 |  |
| --- | --- | --- | --- | --- |
|  | Set 2 | Set 3 | Set 2 | Set 3 |
| Deep sleep | 1 | 1 | 1 | 1 |
| Light sleep | 23 | 20 | 2.6 | 1.8 |
| Micro sleep | 50 | 38 | 5 | 3 |
| Active DL | 240 | 152 | 40 | 8.4 |
| Active UL | 90 | 80 | 5.8 | 4.2 |

Table 2. Reference configuration for evaluation and BS energy consumption modeling purpose, for single CC case

|  | Set 1 FR1 | Set 2 FR1 | Set 3 FR2 |
| --- | --- | --- | --- |
| Duplex | TDD | FDD | TDD |
| System BW | 100 MHz | 20 MHz | 100 MHz |
| SCS | 30 kHz | 15 kHz | 120 kHz |
| Number of TRP | 1 | 1 | 1 |
| Total number of DL TX RUs | 64 | 32 | 2 |
| Total DL power level | 55dBm | 49dBm | 33dBm EIRP limited to 63dBm |
| Total number of UL Rx RUs | 64 | 32 | 2 |

Proposal 1: Consider to take the relative power values for reference configuration set 2 and set 3 proposed by FL as Table 1 for evaluation purpose.

Simulation assumption

In [Post-110-R18- NW_ES2] email discussion, the agreements for the simulation assumption have been achieved as shown below.

Agreement - For FR1, adopt the Reference SLS configurations in Annex-A in R1-2208312 as baseline SLS assumptions. - Other carrier frequencies can be optionally considered. - FFS For FR2 adopt the Reference SLS configuration used in Dense Urban Config.B in Table2 of RP-180524 for IMT-2020 with the following clarification/update as initial SLS assumption. - BS antenna configurations - 2 TxRU (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1) - (dH, dV) = (0.5λ, 0.8λ) (dg,H, dg,V) = (4.0λ, 3.6λ) - Traffic model & UE density - Follow previous agreements with adjusted UE density - Total transmit power per TRxP - Value scaled from that in set 3 reference configuration considering BW - Further adjustment/clarification can be discussed in the next meeting.

In our point of view, we agree that the Reference SLS configuration used in Dense Urban Config.B in Table2 of RP-180524[3] for IMT-2020 can be used as initial SLS assumption for FR2. The final reference configurations for evaluation and BS energy consumption modeling purpose are summarised in table 2. A whole picture of parameter in RP-180524 is shown in Appendix A. However, some necessary clarification/updates should be made for the reference SLS configuration to adapt to the previous agreements or the scenarios.

The original number of TXRU setting is 8TXRU, (Mp,Np,P,Mg,Ng) =(1,1,2,2,2) in RP-180524. However, our agreed reference for TXRU number is set to 2, which is the reference configuration for set 3 FR2 in RAN1#109-e meeting. Considering this, we can update the TXRU per TRxP to 2TXRU, (Mp,Np,P,Mg,Ng) =(1,1,2,2,2).

For the total transmit power, the original assumption is 37 dBm for 40 MHz bandwidth in RP-180524. While the value in set 3 reference configuration is agreed as 33 dBm for 100MHz. If considering the bandwidth updates to 100 MHz for evaluation, the total transmit power can be set to 33dBm. If companies use other bandwidth other than 100MHz (e.g., 40 MHz) for evaluation, the total transmit power can be scaled considering bandwidth.

For the traffic model, because packets for these scenarios may be burst and the SID is agreed to focus on idle/empty and low/medium load scenario, we think the full buffer is not feasible. We have already achieved three traffic models (FTP, IM and VoIP) in previous discussion. The traffic model can be adapted to these three models. Furthermore, there can be different UE density defined corresponding to low/light/medium load cases.

The updated baseline parameter is shown in table 3 with updated parameters highlighted in yellow.

Table 3. Updated parameters based on Dense Urban Config.B

| Dense Urban - eMBB | Config. B |
| --- | --- |
| Carrier frequency for evaluation | 1 layer (Macro) with 30 GHz |
| BS antenna height | 25 m |
| Total transmit power per TRxP | 33 dBm for 100 MHz bandwidth |
| UE power class | 23 dBm |
| Number of antenna elements per TRxP | 256Tx/Rx, (M,N,P,Mg,Ng) = (4,8,2,2,2), (dH,dV) = (0.5, 0.5)λ. (dg,H,dg,V) = (4.0, 2.0)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 2TXRU, (Mp,Np,P,Mg,Ng) =(1,1,2,2,2) |
| Number of UE antenna elements | 32Tx/Rx, (M,N,P,Mg,Ng) = (2,4,2,1,2), (dH,dV) = (0.5, 0.5)λ (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; 0°,90° polarization |
| Traffic model | FTP, IM or VoIP |
| Simulation bandwidth | 100 MHz |
| UE density | adjusted for different load cases |

Proposal 2: Adopting the Reference SLS configuration used in Dense Urban Config.B in Table2 of RP-180524 for IMT-2020 with necessary updates as initial SLS assumption for FR2.

Conclusion

In summary, we provide the following observations and proposals for performance evaluation for network energy savings in Rel-18 NR:

Proposal 1: Consider to take the relative power values for reference configuration set 2 and set 3 proposed by FL as Table 1 for evaluation purpose.

Proposal 2: Adopting the Reference SLS configuration used in Dense Urban Config.B in Table2 of RP-180524 for IMT-2020 with necessary updates as initial SLS assumption for FR2.

References

- RAN1 Chair’s Notes of 3GPP TSG RAN WG1 #110.

- R1-2208312,	“FL summary for Post-110-R18- NW_ES2”, Moderator (Huawei).

- RP-180524, “Summary of calibration results for IMT-2020 self evaluation”, Huawei

Appendix A

Table 2 Baseline parameter for Dense Urban – eMBB

| Dense Urban - eMBB | Config. A | Config. B |
| --- | --- | --- |
| Carrier frequency for evaluation | 1 layer (Macro) with 4 GHz | 1 layer (Macro) with 30 GHz |
| BS antenna height | 25 m | 25 m |
| Total transmit power per TRxP | 41 dBm for 10 MHz bandwidth | 37 dBm for 40 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 20% high loss, 80% low loss (applies to Channel model B) | 20% high loss, 80% low loss |
| Inter-site distance | 200 m | 200 m |
| Number of antenna elements per TRxP | 128Tx/Rx, (M,N,P,Mg,Ng) = (8,8,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 256Tx/Rx, (M,N,P,Mg,Ng) = (4,8,2,2,2), (dH,dV) = (0.5, 0.5)λ. (dg,H,dg,V) = (4.0, 2.0)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 4TXRU, (Mp,Np,P,Mg,Ng) = (2,1,2,1,1) | 8TXRU, (Mp,Np,P,Mg,Ng) =(1,1,2,2,2) |
| Number of UE antenna elements | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization | 32Tx/Rx, (M,N,P,Mg,Ng) = (2,4,2,1,2), (dH,dV) = (0.5, 0.5)λ (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; 0°,90° polarization |
| Number of TXRU per UE | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) | 4TXRU, (Mp,Np,P,Mg,Ng)=(1,1,2,1,2) |
| Device deployment | 80% indoor, 20% outdoor (in car) Randomly and uniformly distributed over the area under Macro layer | 80% indoor, 20% outdoor (in car) Randomly and uniformly distributed over the area under Macro layer |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction |
| UE speeds of interest | Indoor users: 3km/h Outdoor users (in-car): 30 km/h | Indoor users: 3km/h Outdoor users (in-car): 30 km/h |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 7 dB |
| UE noise figure | 7 dB | 10 dB |
| BS antenna element gain | 8 dBi | 8 dBi |
| BS antenna element pattern | See Table 6 in Section 4.6 | See Table 6 in Section 4.6 |
| UE antenna element gain | 0 dBi | 5 dBi |
| UE antenna element pattern | Omni-directional | See Table 8 in Section 4.6 |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | Full buffer | Full buffer |
| Simulation bandwidth | 10 MHz | 40 MHz |
| UE density | 10 UEs per TRxP | 10 UEs per TRxP |
| UE antenna height | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |
| Channel model variant | Alt. 1: Channel model A Alt. 2: Channel model B | (Channel model A or B is the same) |
| TRxP number per site | 3 | 3 |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | (According to Zenith angle in "Beam set at TRxP") | (According to Zenith angle in "Beam set at TRxP") |
| Handover margin (dB) | 0 (i.e., the strongest cell is selected) | 0 (i.e., the strongest cell is selected) |
| TRxP boresight | 30 / 150 / 270 degrees | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula as shown in Appendix 3) from port 0 The UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. |
| Wrapping around method | Geographical distance based wrapping | Geographical distance based wrapping |
| Minimum distance of TRxP and UE | d2D_min=10m | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 | Model-2 in TR36.873 |
| Beam set at TRxP (Constraints for the range of selective analog beams per TRxP) | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = [-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16] Zenith angle θj = [5*pi/8, 7*pi/8] NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = [-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16] Zenith angle θj = [5*pi/8, 7*pi/8] NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) |
| Beam set at UE (Constraints for the range of selective analog beams for UE) | - | For direction of UE analog beam steering (in LCS): Azimuth angle φi = [-3*pi/8, -pi/8, pi/8, 3*pi/8]; Zenith angle θj = [pi/4, 3*pi/4]; NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) |
| Criteria for selection for serving TRxP | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered |
| Criteria for analog beam selection for serving TRxP | Select the best beam pair among the limited set of DFT analog beams, based on the criteria of maximizing receive power after beamforming. | Select the best beam pair among the limited set of DFT analog beams, based on the criteria of maximizing receive power after beamforming. |
| Criteria for analog beam selection for interfering TRxP | Random selecting the random beams for non-serving TRxP | Random selecting the random beams for non-serving TRxP |