3GPP TSG-RAN WG1 Meeting #110bis-e	2210056

# eMeeting, October 10-19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR 38.213 Rel-17 CORESET Configured with CSS and Follow Unified TCI State |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current specification is not clear on the TCI state for CORESET B (CSS only) when DCI indicates unifiedTCIstate associated with cell with different PCI than serving cell and CORESET is configured with CSS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Dot not apply the followUnifiedTCIstate for a CSS part of the CORESET or CORESET with CSS only when indicated unifiedTCIstate indicates RS associated with the PCI different that of a serving cell PCI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | unclear TCI state for CSS reception in the serving cell/the TCI state cannot be updated for the CSS portion of the CORESET in the serving cell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 UE procedure for determining physical downlink control channel assignment |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version of the CR |  |  |  |  |  |  |  |  |

==================== Start of Text proposal ============================

## 10.1	UE procedure for determining physical downlink control channel assignment

<Unchanged text is omitted>

If a UE is provided followUnifiedTCIstate for a CORESET, other than a CORESET with index 0, associated at least with CSS sets other than Type3-PDCCH CSS sets, and if the RS indicated by the indicated DLorJoint-TCIState is associated with a physical cell identity other than the one provided by physCellId in ServingCellConfigCommon, the CORESET, the assumes that the followUnifiedTCIstate does not apply for the PDCCH monitoring on the CSS sets.

<Unchanged text is omitted>

=================== End of Text proposal ============================