3GPP TSG RAN WG1 Meeting #110b-e		R1-2210067

e-meeting, October 10 – 19, 2022

Agenda item:       9.1.4.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	UL enhancements for enabling 8Tx UL transmission

Document for:     Discussion and Decision

# 1	Introduction

In RAN#94-e two new objectives were agreed related for UL MIMO enhancements in Rel-18 NR [1], to address SRI/TPMI enhancements for enabling 8Tx UL transmission:

5. Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In the RAN1 #110 meeting, good progress on uplink 8Tx was achieved with agreements on antenna groups, some details on 8Tx CB design, and support of higher layer transmission. In this contribution, we are continuing the discussions and provide our views on 8Tx UL operations, covering 8Tx codebook design, considerations of 8Tx coherent levels and full-power transmissions, and multi-layer support for 8Tx transmission.

# 2	Codebook based transmission to support UL 8Tx

## 2.1	New NR 8Tx codebook

Based on the WID, “8Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices” is under the Rel-18 study, and if justified, for specifying. Since there is no 8Tx UL codebook in NR, we shall consider introducing 8Tx codebook for NR uplink.

RAN1 had this agreement in the last meeting:

Agreement For evaluation purpose of codebook alternatives when a precoder based on Rel-15 DL Type I is used, following oversampling ratios are assumed - (O1, O2) = (1,1), (2,1), (2,2) - Note: Other values may be used and reported by companies - Note: When deciding the supported O1, O2 combination, the signalling overhead, performance, UE complexity, etc should be considered Agreement RAN1 further studies Alt1b and Alt2a for down-selection of one of the two in RAN1 meeting #110b-e. - Transmission using one or multiple precoders corresponding to one or multiple SRS resources can be studied as part of the above alternatives.

As background information, there are two alternatives under discussion in the last meeting:

- Alt1-b:

  - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of the codebook for partially/non-coherent Ues

  - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully-coherent UEs

- Alt2-a:

  - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of codebook for fully/partially/non-coherent UEs

There are two key issues related to the two alternatives, i) 8Tx CB design for full-coherent UEs, and ii) 8Tx CB design for partial/non-coherent UEs. Various Tx coherence levels shall have significant impact on the codebooks. It would be straightforward to separate full coherence from partial coherence for 8Tx CB design. Besides, non-coherent 8Tx CB design is in general trivial. We shall follow Rel-15 UL approach to define non-coherent 8Tx codebooks.

For full coherent UEs, two alternatives of 8Tx codebook design are based on either Rel-15 UL codebooks or Rel-15 DL type-I codebooks. There is no coherent 8Tx codebook in Rel-15 uplink, and we may refer to Rel-15 DL Type-I codebook for 8Tx. Therefore, we shall have the CB design based on Type-I 8Tx codebook for full-coherent 8Tx codebook; for partial coherent cases we may reuse the design principle of Rel-15 UL codebooks. The alternative approach of using Rel-15 uplink approach to design 8Tx full coherent codebook is not necessary.

Proposal 1: 	Support Alt1-b: study Rel-15 DL Type I CB design for full-coherent 8Tx, and study Rel-15 UL CB design for partial-coherent 8Tx.

DL 8Tx type-I codebook is a 2-stage codebook, different from 2Tx and 4Tx NR uplink codebooks. Let’s review the Type-I codebook design for possible 8Tx support.

Given the precoding is defined with

y=Wx,

where precoded signal y is a P×1 vector with P antenna ports, x is a rank-v data symbol vector with size of v×1, and the matrix W of size P×v is the precoder related with P antenna ports and rank v. For Rel-15 NR, uplink codebooks are single-stage codebooks which define codewords W. With the two-stage codebook design in Type-I codebook, the precoder W has the form

W=W1W2

where W1 contains the wideband component of the precoder and W2 usually contains the sub-band specific components of the precoder. For wideband only support, we may pick single W2 for all sub-bands. For type-I codebook, there are some general assumptions:

- The codebook design is based on 2-D DFT beam grid with oversampling.

- Each 2-D DFT beam in the type-I codebook has two components, related with azimuth and elevation direction of the 2-D antenna array.

- Cross-polarization antennas are assumed.

Although the Type-I codebook is a good reference for 8Tx codebook design, some of the DL codebook design assumptions are not necessarily true for UE’s implementation. Because of restricted form factor at the UE, even for CPE/FWA/vehicle/Industrial devices, half-lambda antenna spacing may be hard to maintain for UE implementation, if not impossible, especially for FR1 with lower carrier frequency where radio wavelength is larger. We may need to address the potential impact of almost arbitrarily spacing of UE’s antenna elements.

RAN1 #109-e meeting reached this agreement to address this concern:

Agreement For 8TX UE, consider the following UE antenna layouts for codebook design, - For non-coherent UEs, consider linear array (1D/2D) of cross-polarized or single-polarized antenna configuration - For fully/partial-coherent UEs, consider linear array (1D/2D) - Where the array is either cross-polarized antenna configuration or single polarized antenna configuration - Ng>=1 antenna groups can be considered where each group comprises coherent antennas, and across groups, antennas can be non-coherent/coherent depending on device types - An example of an antenna group is a panel - Within an antenna group, antenna elements are uniformly spaced. Across different antenna groups, companies to provide details. Additional information for definition of antenna layout - Based on the number of coherent groups, following exemplary cases can be considered where, within each group, antenna elements are spaced by 0.5λ, and then dG-H, dG-V represent the horizontal and vertical spacings between the centers of adjacent antenna groups, respectively - Further down-selection can be done in the next meeting, if needed - The shown exemplary placing of antenna groups can be used for evaluation purpose, but the codebook design is not restricted to shown cases. - Other antenna layouts for other use cases are not precluded. - To start companies may report their results according to their preferred layout. Case; Ng; (M, N, P) per group; Antenna Layout; Antenna Pattern/Antenna Element Gain. 1; 1; (2, 2, 2), (1, 4, 2); Isotropic (Indoor/Outdoor FWA & Industrial) 8 dBi, 65° HPBW(Outdoor FWA). 2; 2; (1, 2, 2); Isotropic (Indoor/Outdoor FWA & Industrial) 8 dBi, 65° HPBW(Outdoor FWA). 3; 4; (1, 1, 2); Isotropic (Indoor/Outdoor FWA & Industrial) 4 dBi, 110° HPBW(Indoor FWA & Industrial. o Other UE antenna assumption for the purpose of evaluation Outdoor FWA; Indoor FWA; Industrial. UE antenna height; 6, 3 m (To start); According to 36.873; According to 38.901.

In the listed three cases of UE antenna layouts, Case-2 and Case-3 addressed the case where the antenna spacing might be arbitrary, which is typical for spacing of non-coherent UE antennas. UE antenna layout Case-1 refers the full coherent 8Tx code (with Ng=1), we shall apply the DL 8Tx codebook to support the full coherent 8Tx codebook design.

Following 2-D DFT beam grid, Type-I single beam codebook with rank 1 has its choice for W1 and W2 as

W1(1)i11,i12=vi11,i1200vi11,i12

W2(1)i2=1P1φ1(1)i2

where φ1(1)∈1,j,-1,-j, and vi11,i12 is defined as a DFT matrix. Note that a single beam is selected at wideband, and co-phasing across polarizations is performed per subband.

One 2-D DFT beam has an azimuth component and an elevation component, as

tl=1, ej2πlO1N1, ⋯, ej2πlN1-1O1N1T

um=1, ej2πmO2N2, ⋯, ej2πmN2-1O2N2T

Each 2-D beam is defined as

vl,m=tl⊗um

The antenna port configuration per polarization is given with (N1, N2), and oversampling factors O1 and O2 for two components, respectively. For Type-I codebook, these configurations are used:

Table 1    Type-I codebook configuration: 4Tx and 8Tx

| # of CSI-RS Ports | (N1,N2) | (O1,O2) |
| --- | --- | --- |
| 4 | (2, 1) | (4, 1) |
| 8 | (2, 2) | (4, 4) |
|  | (4, 1) | (4, 1) |

There are two (N1,N2) configurations for Type-I 8Tx codebook, as (2, 2) and (4, 1). The configuration of (2, 2) provides better elevation capability. These two configurations are considered in the Case-1 of UE antenna layouts.

NR downlink also supports Type-I multi-panel codebook. The Type-I multi-panel 8Tx codebook has this antenna configuration as shown in Table 2.

Table 2    Type-I multi-panel codebook configuration: 8Tx

| # of CSI-RS Ports | (Ng, N1,N2) | (O1,O2) |
| --- | --- | --- |
| 8 | (2, 2, 1) | (4, 1) |

This codebook is based on the Type-I 4Tx codebook with co-phasing across panels (2 panels). The antenna placement with two panels can be illustrated as

Note that this layout is similar to Case-2 UE antenna layout.

From previous RAN1 agreements, Case-1 UE antenna layout implies N1,N2=(2, 2) and (4, 1), for layout 1-a, and 1-b, respectively. The oversampling ratios are assumed to be (O1, O2)=(1, 1), (2, 1), and (2, 2). With Type-I single-beam codebook, the full codebook is thus defined.

## 2.2	System simulation evaluation

RAN1 #109-e meeting has this agreement on EVM for system-level simulation for 8Tx codebook:

Agreement Adopt the following Table as the reference EVM for SLS evaluation.- - Companies may provide additional evaluation results per their case of interest. Parameter; Value. Frequency range; 3.5 GHz. Multiple access; OFDMA. Numerology; 14 OFDM symbol slot SCS , 30 KHz. Scenario; Outdoor FWA (38.901): UMa (ISD = 500 m), 100% Outdoor, 3Km/h. Indoor FWA (38.901): UMi (ISD = 200 m), 100% Indoor, 3Km/h. Industrial (38.901): Indoor Office (Inh ), 3Km/h. Channel model; 38.901. System bandwidth; 20 MHz, 100 MHz. gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝); Outdoor FWA : (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Indoor FWA : (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Industrial: (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ. gNB antenna radiation pattern parameters; Outdoor/Indoor FWA : 38.901 Table 7.3-1, 8 dBi , 65° HPBW Industrial: IMT.2412 Table 10,5 dBi , 90° HPBW. gNB receiver noise figure; 5dB. gNB receiver; MMSE-IRC. gNB scheduler; Single user with proportional fair. Modulation; - Up to 64 QAM - Up to 256QAM. MIMO scheme; SU-MIMO with rank adaptation. UE speed; 3 Km/h. UE TX antenna configuration; To be defined according to outcome of Proposal 2.1. Traffic model; - FTP model 1: Packet size 500KB, RU= 50% and suggested low/high RU of values of 20% and 70% - Full buffer (optional). Suggested benchmarking; R15 UL 4-Tx codebook , Eigen-based, companies report PRG assumption. Precoder granularity; Wideband. Power control; Open loop, - alpha = 0.8 - P0 = -50, -80 dBm to be selected according to the deployment scenario. UE power rating; 23 dBm (UE, 38.101) 32 dBm (FWA, 38.101). Metric; UL mean-user throughput, 5%-ile and 95%-ile UPT.

For the study phase of this objective, there are several key questions to be answered:

- What gains can be achieved from supporting 8TX on the UL while retaining the current limitation of a maximum of 4 transmission layers?

- What gains can be achieved if Rel-18 supports a maximum of 8 transmission layers on the UL in addition to supporting 8TX?

To answer these questions, an extensive set of system simulation results were generated according to the parameters listed in the above table as part of the agreed evaluation methodology.  These simulation results were generated in both the outdoor and indoor FWA scenarios with configuration 1B for 8TX with both omni-directional UE antennas as well as with the directional antennas agreed to in the EVM.  Coherent UE configurations were assumed with the wideband TPMI selected according to the downlink Type I CSI (L=1) for both 8TX and 4TX.  Given the potential performance impact of the maximum supported modulation on the UL, system results are also provided for two maximum modulation orders: 64 QAM and 256QAM.

As shown in the appendix, the gains from supporting 8TX and the additional gains from supporting up to 8 transmission layers are highly variable and depend on the scenario, the traffic characteristics, and the maximum modulation order.  However, the gains from supporting 8TX and up to 8 transmission layers are quite substantial across both the Outdoor and Indoor FWA scenarios, both types of UE antenna patterns (omni and directional), both maximum modulation orders (64QAM and 256QAM) and across the traffic types specified in the EVM for this objective (bursty and full buffer).

Observation 1: The gains from supporting 8Tx and up to 8 Tx layers are quite substantial across both the Outdoor and Indoor FWA scenarios, both types of UE antenna patterns, both maximum modulation orders, and across traffic types.

Given this observation from system level simulations, there is significant gain for support 8Tx over 4Tx, and also signification gain for maximum 8 transmission layers over maximum 4 transmission layers, for uplink transmission scenarios specified in the EVM.

# 3	Considerations with 8Tx UE coherence levels and full-power modes

## 3.1	Support of Coherence levels

Due to UE’s different RF chain implementation, coherence level cross multiple antenna ports could be different. Between two uplink Tx ports, UE may transmit coherently or non-coherently, depending on whether the signal phase between the two ports are calibrated. Rel-15 2Tx codebook has two coherence levels, to support full coherent and non-coherent transmission. For 4Tx, there are three levels of transmission coherence level: full coherent, partial coherent, and non-coherent.

With the introduction of 8Tx, there will be various combination of coherence levels. For example, there will have 2x4 where 2-coherent Tx ports as a pair, and non-coherent among the 4 pairs of ports. Alternatively, 4x2 where 4-coherent Tx ports as a group, and non-coherent between the two groups. The legacy approach of using partial coherence to indicate this case is no longer valid.

Observation 2:  The legacy approach of using partial coherence to indicate 8Tx coherence levels is no longer sufficient.

One possible approach is to group coherent antenna ports together.

Table 5    Coherence Group

| Tx ports | Rel-15 indications | Ng: Num of coherence groups | Number of ports per coherence group |
| --- | --- | --- | --- |
| 2Tx | Coherent | 1 | 2 |
|  | Non-coherent | 2 | 1 |
| 4Tx | coherent | 1 | 4 |
|  | Partial-coherent | 2 | 2 |
|  | Non-coherent | 4 | 1 |
| 8Tx | Coherent | 1 | 8 |
|  |  | 2 | 4 |
|  |  | 4 | 2 |
|  | Non-coherent | 8 | 1 |

RAN1 #109-e meeting and RAN1 #110 meeting have these related agreements:

Agreement For 8TX UE, consider the following UE antenna layouts for codebook design, - For non-coherent UEs, consider linear array (1D/2D) of cross-polarized or single-polarized antenna configuration - For fully/partial-coherent UEs, consider linear array (1D/2D) - Where the array is either cross-polarized antenna configuration or single polarized antenna configuration - Ng>=1 antenna groups can be considered where each group comprises coherent antennas, and across groups, antennas can be non-coherent/coherent depending on device types - An example of an antenna group is a panel - Within an antenna group, antenna elements are uniformly spaced. Across different antenna groups, companies to provide details. … Agreement For 8TX PUSCH, at least support - Ng=1, 2, 4 Note: The above does not restrict the Ng for the non-coherent case

In the agreement, with the introduction of antenna groups, each antenna group comprises coherent antennas. Thus a antenna group can be considered as a coherence group where antennas within the group are coherent in transmission. For 8Tx non-coherent Tx ports, the number of antenna groups will be Ng=8. Its related codebook should be based on antenna-switching precoders, similar to 4Tx defined in Rel-15 UL codebooks.

In RAN1 #110 meeting, there were discussions on the number of antenna groups for a full coherent UE. The question is whether Ng>1 is allowed when all antennas are coherent. The antenna group, introduced as a group that comprises coherent antennas, indicates a coherence group where antennas within the groups are coherent. Although there is no clear RAN1 definition on whether it is coherent between two antenna groups, there is no clear benefit to allow coherence among antenna groups. Doing so will certainly introduce unnecessary complicated indications of coherence levels. Unless there is other benefit, RAN1 shall clarify that Ng=1 for full coherent case.

Proposal 2: 	For 8Tx PUSCH, for full coherent case, the antenna group number Ng=1.

### 3.1.1	Antenna group Ng=2

When the antenna group Ng=2, each antenna group shall have 4 coherent Tx ports. We may consider Rel-15 4Tx UL codebook. The Rel-15 4Tx codebook is based on ULA such as this placement

This placement with cross-polarzation is not supported in the Rel-15 4Tx codebook.

Consider that the (M, N, P) = (2, 2, 2) and (1, 4, 2) are considered for 8Tx design, we shall consider x-pol support, based on DL Type-I codebooks.

Alternatively, we may reuse certain design principle from Rel-15 uplink 2Tx codebook. For example, with full rank support up to 8, the 8Tx precoders can be

W=W100W2

where W1 and W2 are 4Tx coherent precoders. This design provides rank support: 2, 3, 4, 5, 6, 7, 8.

If half rank (up to rank-4), the 8Tx precoders can be

W10, 0W2

where W1 and W2 are 4Tx coherent precoders, with these rank support: 1, 2, 3, 4

### 3.1.2	Antenna group Ng=4

When Ng=4, the number of coherent groups is 4, each of which has two coherent antenna ports (x-pol antenna ports in a typical case). If we also reuse design principle of Rel-15 uplink codebook to reuse Rel-15 UL 2Tx codebook precoders W1,W2,W3,W4, the 8Tx precoder can have this form

W10000W20000W30000W4

with rank support of 4, 5, 6, 7, 8.

Alternatively, we may have these 8Tx precoders

| W100W20000 | W10000W300 | W1000000W4 | 00W200W300 |
| --- | --- | --- | --- |
| 00W20000W4 | 0000W300W4 |  |  |

to support rank of 2, 3, 4.

Proposal 3: 	Consider reusing Rel-15 uplink codebook design principle for 8Tx partial coherent codebooks with Ng=2 and Ng=4.

## 3.2	Full-power modes for 8Tx

NR Rel-16 introduced the full Tx power modes for uplink transmission. This feature can be extended to 8Tx. For coherent 8Tx transmission, the UE shall deliver full Tx power depending on UE’s power class. The full-power modes are only applied for “partial-” or “non-” coherent cases, similar to the Rel-16 case.

Rel-16 mode-1 can be extended to 8Tx where a subset of the 8Tx codebook shall support full power. For non-coherent case with Ng=8, one example precoder for rank-1 would be

W=12211111111

to support full Tx power transmission for 8Tx ports. The antenna group is coherent group, as defined in the agreement of last RAN1 meeting. For other than rank-1, similar approach can be applied to limit TPMI in the 8Tx codebook. For other Ng cases, the TPMI limitation can be applied for the 8Tx partial coherent codebooks.

The Rel-16 full power mode-2 can also be applied with virtualized SRS ports. One simple approach is to virtualize 8Tx ports to a single virtual SRS port.

Proposal 4: 	Extend Rel-16 full power mode 1 and mode 2 support to 8Tx.

# 4	Multiple Layers and Codewords

Based on WID, 8Tx operation is targeted for CPE/FWA/Industrial applications. As demonstrated in system-level simulation results shown in Appendix and discussions in Section 2.2, with maximum rank of 8 for 8Tx UE, there is clear indication that full rank operation will provide UE throughput gains. Besides, for CPE/FWA/Industrial applications, high peak Tx rate transmission in uplink could be necessary for certain use cases.

RAN1 #110 reached this agreement to support rank X=4,8 transmission:

Agreement Support up to X layers for codebook and non-codebook UL transmission for 8TX UE where X=4, 8 is determined based on separate UE capability - For uplink transmission with rank<=4, single CW is supported - For uplink transmission with rank>4, whether single or dual CW is used will be decided in RAN1 meeting #110b-e The above applies only with regards to the work scope of this agenda item.

One open issue related to 8Tx transmission is about the support of one or two codewords for high-rank transmission, particularly when rank-8 operation is supported. Do we need single codeword, or two codewords?

In the context of this contribution, we assume single TRP operation for 8Tx transmission. Even though single CW can support 8-layer transmission, two CWs can also be supported as well. If 2CW is supported for 8-layer transmission, it would be a natural choice to limit maximum rank per CW to 4. For up to 8 layer transmission, 2CWs may have various rank-combinations. Note that NR DL support 2CW for high layer transmission.

Proposal5:	For uplink transmission with rank>4, dual CW shall be used.

It is not necessary to support all possible rank combinations for 2CW support. Following DL two CW design, we can consider these rank combinations for 2CW to reach greater than 4-layer transmissions: rank combinations 2+3, 3+3, 3+4, and 4+4 for two CW. These rank combinations provide support of 5, 6, 7, and 8 layer transmission, respectively.

Proposal 6:	If two codewords are supported for uplink Tx, consider supporting rank combinations of 2+3, 3+3, 3+4, and 4+4.

# 5	Conclusions

In this contribution, we are addressing the Objective 5 of the Rel-WID [1] on the uplink enhancements to enable 8Tx UL transmission. Here is the summary of our proposals and observations:

Proposal 1: 	Support Alt1-b: study Rel-15 DL Type I CB design for full-coherent 8Tx, and study Rel-15 UL CB design for partial-coherent 8Tx.

Observation 1: 	The gains from supporting 8Tx and up to 8 Tx layers are quite substantial across both the Outdoor and Indoor FWA scenarios, both types of UE antenna patterns, both maximum modulation orders, and across traffic types.

Observation 2:  	The legacy approach of using partial coherence to indicate 8Tx coherence levels is no longer sufficient.

Proposal 2: 	For 8Tx PUSCH, for full coherent case, the antenna group number Ng=1.

Proposal 3: 	Consider reusing Rel-15 uplink codebook design principle for 8Tx partial coherent codebooks with Ng=2 and Ng=4.

Proposal 4: 	Extend Rel-16 full power mode 1 and mode 2 support to 8Tx.

Proposal5:	For uplink transmission with rank>4, dual CW shall be used.

Proposal 6:	If two codewords are supported for uplink Tx, consider supporting rank combinations of 2+3, 3+3, 3+4, and 4+4.

# References

- RP-213598, “New WID: MIMO Evolution for Downlink and Uplink,” RAN#94e.

- RAN1 109-e meeting Chair notes, RAN1 109-e meeting, May 2022

- RAN1 110 meeting Chair Notes, RAN1 #110 meeting, Toulouse, August 2022

# Appendix – Detailed Simulation Results

## Gain of 8TX with max 4 layers over 4TX with max 4 layers (UE config 1B)

These results show the gains that can be achieved in Rel-18 simply by enabling 8TX while keeping the current maximum of 4 transmission layers.  Small gains with full buffer, but much larger gains with bursty traffic.

Figure 1:  Outdoor FWA scenario with UE config 1B:  gain of 8TX with max 4 layers over 4TX with max 4 layers: Left plot is the gain in the Mean UE SE, and the right plot is the gain in the Edge UE SE.  Four traffic scenarios: FTP model 1 with Target RU = 0.2, 0.5, 0.7 and Full Buffer.  Four configurations: Max 64QAM vs Max 256QAM and omni UE antennas vs directional UE antennas.

Figure 2:  Indoor FWA scenario with UE config 1B:  gain of 8TX with max 4 layers over 4TX with max 4 layers: Left plot is the gain in the Mean UE SE, and the right plot is the gain in the Edge UE SE.  Four traffic scenarios: FTP model 1 with Target RU = 0.2, 0.5, 0.7 and Full Buffer.

## Gain of 8TX with max 8 layers over 8TX with max 4 layers (UE config 1B)

These results show the additional gains that can be obtained by enabling the 8TX UL to support up to 8 layers rather than 4 layers.  Gains are variable, but quite substantial.

Figure 3:  Outdoor FWA scenario with UE config 1B:  gain of 8TX with max 8 layers over 8TX with max 4 layers: Left plot is the gain in the Mean UE SE, and the right plot is the gain in the Edge UE SE.  Four traffic scenarios: FTP model 1 with Target RU = 0.2, 0.5, 0.7 and Full Buffer.

Figure 4:  Indoor FWA scenario with UE config 1B:  gain of 8TX with max 8 layers over 8TX with max 4 layers: Left plot is the gain in the Mean UE SE, and the right plot is the gain in the Edge UE SE.  Four traffic scenarios: FTP model 1 with Target RU = 0.2, 0.5, 0.7 and Full Buffer.

## Gain of 8TX with max 8 layers over 4TX with max 4 layers (UE config 1B)

These results show the total possible gain that can be achieved with Rel-18 compared to Rel-17 if Rel-18 enables both 8TX and up to 8 transmission layers.    Most of these gains are rather large no matter what the combination of maximum modulation or whether the UE uses omni or directional antennas, etc.

Figure 5:  Outdoor FWA scenario with UE config 1B:  gain of 8TX with max 8 layers over 4TX with max 4 layers: Left plot is the gain in the Mean UE SE, and the right plot is the gain in the Edge UE SE.  Four traffic scenarios: FTP model 1 with Target RU = 0.2, 0.5, 0.7 and Full Buffer.

Figure 6:  Indoor FWA scenario with UE config 1B:  gain of 8TX with max 8 layers over 4TX with max 4 layers: Left plot is the gain in the Mean UE SE, and the right plot is the gain in the Edge UE SE.  Four traffic scenarios: FTP model 1 with Target RU = 0.2, 0.5, 0.7 and Full Buffer.

# RAN1 Previous Agreements

## RAN1 #109-e meeting Agreements

Agreement

Study fully-coherent, partially-coherent and non-coherent UEs for uplink transmission with 8TX UEs .

Agreement

Study full power transmission for 8TX UEs .

- Details are FFS upon completion of codebook design

Agreement

Adopt the following Table as the reference EVM for LLS evaluation

- Companies may provide additional evaluation results per their case of interest

- LLS is optionally used for 8Tx UL evaluation, if needed

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| Waveform | CP-OFDM |
| SCS | 30 KHz |
| System bandwidth | 20 MHz, 100 MHz |
| Scheduled PRBs | 5, 25, 50, 260 PRBs |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ |
| UE TX antenna configuration | To be defined according to outcome of Proposal 2.1 |
| UE speed | 3 Km/h |
| Number of Layers | Adaptive, Fixed (reported by company) |
| AMC | Adaptive, Fixed (reported by company) |
| DMRS configuration | Type 1; 1 front loaded + 1 additional symbol |
| Channel estimation | Real |
| Channel Model | CDL-A (30ns), CDL-B (100ns), CDL-C (300ns) |

Agreement

For 8TX UE uplink transmission, study codebook- and non-codebook-based transmission with maximal layer number of both 4 and 8 layers.

R1-2205221	FL Summary on SRI/TPMI Enhancements; Second Round	Moderator (InterDigital Inc.)

Agreement

Adopt the following Table as the reference EVM for SLS evaluation.-

- Companies may provide additional evaluation results per their case of interest.

| Parameter | Value |
| --- | --- |
| Frequency range | 3.5 GHz |
| Multiple access | OFDMA |
| Numerology | 14 OFDM symbol slot SCS , 30 KHz |
| Scenario | Outdoor FWA (38.901): UMa (ISD = 500 m), 100% Outdoor, 3Km/h |
|  | Indoor FWA (38.901): UMi (ISD = 200 m), 100% Indoor, 3Km/h |
|  | Industrial (38.901): Indoor Office (Inh ), 3Km/h |
| Channel model | 38.901 |
| System bandwidth | 20 MHz, 100 MHz |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | Outdoor FWA : (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Indoor FWA : (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Industrial: (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ |
| gNB antenna radiation pattern parameters | Outdoor/Indoor FWA : 38.901 Table 7.3-1, 8 dBi , 65° HPBW Industrial: IMT.2412 Table 10,5 dBi , 90° HPBW |
| gNB receiver noise figure | 5dB |
| gNB receiver | MMSE-IRC |
| gNB scheduler | Single user with proportional fair |
| Modulation | - Up to 64 QAM - Up to 256QAM |
| MIMO scheme | SU-MIMO with rank adaptation |
| UE speed | 3 Km/h |
| UE TX antenna configuration | To be defined according to outcome of Proposal 2.1 |
| Traffic model | - FTP model 1: Packet size 500KB, RU= 50% and suggested low/high RU of values of 20% and 70% - Full buffer (optional) |
| Suggested benchmarking | R15 UL 4-Tx codebook , Eigen-based, companies report PRG assumption |
| Precoder granularity | Wideband |
| Power control | Open loop, - alpha = 0.8 - P0 = -50, -80 dBm to be selected according to the deployment scenario |
| UE power rating | 23 dBm (UE, 38.101) 32 dBm (FWA, 38.101) |
| Metric | UL mean-user throughput, 5%-ile and 95%-ile UPT |

R1-2205497	FL Summary on SRI/TPMI Enhancements; Third Round	Moderator (InterDigital Inc.)

Agreement

For 8TX UE, consider the following UE antenna layouts for codebook design,

- For non-coherent UEs, consider linear array (1D/2D) of cross-polarized or single-polarized antenna configuration

- For fully/partial-coherent UEs, consider linear array (1D/2D)

  - Where the array is either cross-polarized antenna configuration or single polarized antenna configuration

  - Ng>=1 antenna groups can be considered where each group comprises coherent antennas, and across groups, antennas can be non-coherent/coherent depending on device types

    - An example of an antenna group is a panel

  - Within an antenna group, antenna elements are uniformly spaced. Across different antenna groups, companies to provide details.

Additional information for definition of antenna layout

- Based on the number of coherent groups, following exemplary cases can be considered where, within each group, antenna elements are spaced by 0.5λ, and then dG-H, dG-V represent the horizontal and vertical spacings between the centers of adjacent antenna groups, respectively

  - Further down-selection can be done in the next meeting, if needed

  - The shown exemplary placing of antenna groups can be used for evaluation purpose, but the codebook design is not restricted to shown cases.

  - Other antenna layouts for other use cases are not precluded.

  - To start companies may report their results according to their preferred layout.

| Case | Ng | (M, N, P) per group | Antenna Layout | Antenna Pattern/Antenna Element Gain |
| --- | --- | --- | --- | --- |
| 1 | 1 | (2, 2, 2), (1, 4, 2) |  | Isotropic (Indoor/Outdoor FWA & Industrial) 8 dBi, 65° HPBW(Outdoor FWA) |
| 2 | 2 | (1, 2, 2) |  | Isotropic (Indoor/Outdoor FWA & Industrial) 8 dBi, 65° HPBW(Outdoor FWA) |
| 3 | 4 | (1, 1, 2) |  | Isotropic (Indoor/Outdoor FWA & Industrial) 4 dBi, 110° HPBW(Indoor FWA & Industrial |

o   Other UE antenna assumption for the purpose of evaluation

|  | Outdoor FWA | Indoor FWA | Industrial |
| --- | --- | --- | --- |
| UE antenna height | 6, 3 m (To start) | According to 36.873 | According to 38.901 |

Agreement

For 8TX UE codebook-based uplink transmission, down-select one of

- Alt1-a:

  - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of the codebook for non-coherent UEs

  - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully/partially-coherent UEs

- Alt1-b:

  - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of the codebook for partially/non-coherent UEs

  - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully-coherent UEs

- Alt2-a:

  - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of codebook for fully/partially/non-coherent UEs

- Alt2-b:

  - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) in combination with those based on NR Rel-15 DL Type I codebooks as the starting point for design of codebook for fully/partially/non-coherent UEs

- Alt3:

  - Study NR Rel-15 DL Type I codebook as the starting point for design of codebook for fully/partially/non-coherent UEs

- Transmission using one or multiple precoders corresponding to one or multiple SRS resources can be studied as part of the above alternatives.

## RAN1 #110 meeting Agreements

Agreement

8TX PUSCH is supported in Rel-18

Agreement

For 8TX PUSCH, at least support

- Ng=1, 2, 4

Note: The above does not restrict the Ng for the non-coherent case

Agreement

For evaluation purpose of codebook alternatives when a precoder based on Rel-15 DL Type I is used, following oversampling ratios are assumed

- (O1, O2) = (1,1), (2,1), (2,2)

- Note: Other values may be used and reported by companies

- Note: When deciding the supported O1, O2 combination, the signalling overhead, performance, UE complexity, etc should be considered

Agreement

RAN1 further studies Alt1b and Alt2a for down-selection of one of the two in RAN1 meeting #110b-e.

- Transmission using one or multiple precoders corresponding to one or multiple SRS resources can be studied as part of the above alternatives.

Agreement

Support up to X layers for codebook and non-codebook UL transmission for 8TX UE where X=4, 8 is determined based on separate UE capability

- For uplink transmission with rank<=4, single CW is supported

- For uplink transmission with rank>4, whether single or dual CW is used will be decided in RAN1 meeting #110b-e

The above applies only with regards to the work scope of this agenda item.

Agreement

For SRS configuration for non-codebook UL transmission for an 8TX UE, down-select from

- Alt1: A single SRS resource set configured with up to 8 single-port SRS resources

- Alt2: Up to two SRS resource sets, each configured with up to 4 single-port SRS resources

- Alt3: Support both alternatives.

Agreement

Study low overhead solutions for SRI and/or transmitter precoder matrix indication for codebook-based, and SRI indication for non-codebook-based UL transmission by an 8TX UE,

- FFS using single or separate (exiting or new) fields for the indication, other solutions are not precluded.

- Note: Low overhead schemes for study include those using Rel-15 SRI/TPMI indication mechanisms