3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2210087

e-Meeting, October 10th – 19th, 2022

Agenda Item:	8.16.2

Source:	Ericsson

Title:	UE features topics 2

Document for:	Discussion

# 1	Introduction

In this contribution we provide our views about remaining technical aspects on UE features for the following Rel-17 Work Items:

- NR-MIMO (section 2)

# 2	Rel-17 UE features for NR-MIMO

The following was included in the LS to RAN2 after RAN1#110 [1]:

| 23. NR_FeMIMO | 23-1-1a | Unified TCI with joint DL/UL TCI update for inter-cell beam management | 1. Support of unified TCI with joint DL/UL TCI update for inter-cell beam management 2. Support K additional MAC-CE indicated joint TCI states per CC in a band 3. Support K additional MAC-CE activated joint TCI states across all CC(s) in a band |
| --- | --- | --- | --- |

Note that columns 5-14 have been deleted. This was subsequently added to 38.306 [2]:

| unifiedJointTCI-InterCell-r17 Indicates the support of Unified TCI with joint DL/UL TCI update for inter-cell beam management including following parameters: - additionalMAC-CE-PerCC-r17 indicates the number of K additional MAC-CEs to indicate joint TCI states per CC in a band. - additionalMAC-CE-AcrossCC-r17 indicates the number of K additional MAC-CE activated joint TCI states across all CC(s) in a band. NOTE: A UE that supports unifiedJointTCI-InterCell-r17 supports K additional MAC-CE activated joint TCI states across all CC(s) in a band in addition to the maximum number of MAC-CE activated joint TCI states across all CC(s) in a band signalled in unifiedJointTCI-r17. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

However, there was a typo in the LS from RAN1 for component 2: the correct formulation is provided below:

| 23. NR_FeMIMO | 23-1-1a | Unified TCI with joint DL/UL TCI update for inter-cell beam management | 1. Support of unified TCI with joint DL/UL TCI update for inter-cell beam management 2. Support K additional MAC-CE indicated activated joint TCI states per CC in a band 3. Support K additional MAC-CE activated joint TCI states across all CC(s) in a band |
| --- | --- | --- | --- |

With this update, the same formulation is used for component 2 and 3.

Update the description of component 2 of FG 23-1-1a by replacing “indicated” with “activated”.

# References

R1-2207923,	“Updated RAN1 UE features list for Rel-17 NR after RAN1 #110 Thursday”, Moderators” (AT&T, NTT DOCOMO, INC.), RAN1 #110, Toulouse, France, August 22nd – August 26th, 2022

3GPP TS 38.306 V17.1.0, NR; User Equipment (UE) radio access capabilities, RAN#96, June 2022

# Annex 1:

| - Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |