3GPP TSG-RAN WG1 Meeting # 110bis-e	R1-2210103

# e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | Draft | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.214 on aperiodic CSI reporting on PUSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Clause 5.2.3, it is specified that when DCI format 0_1 schedules multiple PUSCH allocations, the aperiodic CSI report is carried on either the second PUSCH (if two PUSCHs are scheduled) or the penultimate PUSCH (if more than two PUSCHs are scheduled). However, in Clause 6.1.2, when AP-CSI-MultiplexingMode is enabled, a different behaviour is specified for multiplexing aperiodic CSI multilplexing on PUSCH as copied below: For PUSCH repetition Type A, when a DCI format 0_1 and DCI format 0_2 indicate codepoint "10" or "11" for the SRS resource set indicator and schedule aperiodic CSI report(s) on PUSCH with transport block by a 'CSI request' field on a DCI, the CSI report(s) multiplexing is determined as follows - if higher layer parameter AP-CSI-MultiplexingMode in CSI-AssociatedReportConfigInfo is enabled and UCI other than CSI report(s) are not multiplexed on PUSCH, the CSI report(s) is transmitted separately only on the first transmission occasion associated with the first SRS resource set and the first transmission occasion associated with the second SRS resource set. - otherwise, the CSI report(s) is transmitted only on the first transmission occasion. Even though our understanding is that the clauses in 5.2.3 and 6.1.2 are targetting different features, there may be a potential ambiguity on how to multilpex Aperiodic CSI on PUSCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To remove the potential ambiguity, clarificaiton is made to the text in clause 5.2.3, where the section in 5.2.3 applies only when two SRS resource sets are not configured for codebook based or non-codebook based PUSCH, or when the SRS resource set indicator field doesn’t indicates two SRI field. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous behaviour specified in two different clauses of the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<UNCHANGED PARTS OMITTED>

### 5.2.3	CSI reporting using PUSCH

A UE shall perform aperiodic CSI reporting using PUSCH on serving cell c upon successful decoding of a DCI format 0_1 or DCI format 0_2 which triggers an aperiodic CSI trigger state.

When a DCI format 0_1 schedules two PUSCH allocations, and when DCI format 0_1 does not include an SRS resource set indicator field or when DCI includes an SRS resource set indicator field indicating codepoint “00” or “01”, the aperiodic CSI report is carried on the second scheduled PUSCH. When a DCI format 0_1 schedules more than two PUSCH allocations, and when DCI format 0_1 does not include an SRS resource set indicator field or when DCI includes an SRS resource set indicator field indicating codepoint “00” or “01”, the aperiodic CSI report is carried on the penultimate scheduled PUSCH.

An aperiodic CSI report carried on the PUSCH supports wideband, and sub-band frequency granularities. An aperiodic CSI report carried on the PUSCH supports Type I, Type II, Enhanced Type II and Further Enhanced Type II Port Selection CSI.

<UNCHANGED PARTS OMITTED>