3GPP TSG RAN WG1 Meeting # 110bis-e	 R1-2210145

# E-meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Draft CR] Correction on PUCCH cell switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current specifications do not clearly define on how to apply the PDSCH-to-HARQ slot offset for semi-static and dynamic PUCCH cell switching. For semi-static PUCCH cell switching, the slot offset is to be applied on PCell and not the cell for PUCCH transmission, whereas for dynamic PUCCH cell switching and no PUCCH switching, the slot offset is to be applied on the cell for PUCCH transmission. The following earlier RAN1 agreements are available: Agreement from RAN1#106bis-e For semi-static PUCCH cell switching, PCell / PSCell / PUCCH-SCell is reference cell: - The time domain pattern configurations are based on the numerology of the reference cell. - The PDSCH to HARQ-ACK offset k1 is interpreted based on the numerology and PUCCH configuration of a reference cell to be able to apply the time-domain PUCCH cell switching pattern. - Note: There may not be a need to define a ‘reference cell’ in the specification. This terminology is used for further clarifications of the procedure. Agreement from RAN1#105-e: For PUCCH carrier switching based on dynamic indication in DCI scheduling a PUCCH (i.e. Alt. 1), the PDSCH to HARQ-ACK offset k1 is interpreted based on the numerology of the dynamically indicated target PUCCH cell.. |
| Summary of change: |  | Correct the applicable PDSCH-to-HARQ slot offset for semi-static PUCCH cell switching. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect timing for HARQ-ACK reporting based on PDSCH-to-HARQ slot offset on PUCCH-sSCell for semi-static PUCCH cell switching. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

### 9.2.3	UE procedure for reporting HARQ-ACK

< Unchanged parts are omitted >

If the UE is provided subslotLengthForPUCCH, n is the last UL slot of the PCell if the UE is provided pucch-sSCellPattern, or the last UL slot of the serving cell for PUCCH transmission if the UE is not provided pucch-sSCellPattern, that overlaps with a PDSCH reception or with a PDCCH reception providing a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception; otherwise, n is the last UL slot of the PCell if the UE is provided pucch-sSCellPattern, or the last UL slot of the serving cell for PUCCH transmission if the UE is not provided pucch-sSCellPattern, that overlaps with the DL slot nD for the PDSCH reception or with the DL slot nD for the PDCCH reception in case of a DCI format that triggers a HARQ-ACK information report and does not schedule a PDSCH reception.

For a SPS PDSCH reception ending in DL slot nD, the UE transmits the PUCCH in UL slot n+k of the PCell where k is provided by the PDSCH-to-HARQ_feedback timing indicator field, if present, in a DCI format activating the SPS PDSCH reception.

If the UE detects a DCI format that does not include a PDSCH-to-HARQ_feedback timing indicator field and schedules a PDSCH reception or activates a SPS PDSCH reception ending in DL slot nD, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot n+k of the PCell if the UE is provided pucch-sSCellPattern, or UL slot n+k of the serving cell for PUCCH transmission if the UE is not provided pucch-sSCellPattern, where k is provided by dl-DataToUL-ACK, or dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-DCI-1-2-r17, or dl-DataToUL-ACK-MulticastDciFormat4_1.

If the UE detects a DCI format scheduling a number of PDSCH receptions ending in DL slot nD or if the UE detects a DCI format generating a HARQ-ACK information bit and does not schedule a PDSCH reception through a PDCCH reception ending in DL slot nD, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot n+k of the PCell if the UE is provided pucch-sSCellPattern, or UL slot n+k of the serving cell for PUCCH transmission if the UE is not provided pucch-sSCellPattern, where k is a number of slots and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-DCI-1-2-r17, or dl-DataToUL-ACK-MulticastDciFormat4_1.

< Unchanged parts are omitted >