3GPP TSG RAN WG1 #110bis-e R1-2210158 e-Meeting, October 10th – 19th, 2022 CR-Form-v12.0. CHANGE REQUEST. 38.213; CR; rev; -; Current version:; 17.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on simultaneous configuration of Type-1 HARQ-ACK codebook and dci-enabler for multicast service. Source to WG:; Lenovo. Source to TSG:; R1. Work item code:; NR_MBS-Core; Date:; 2022-09-30. Category:; F; Release:; Rel-17. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; UE behaviour is unclear when UE is simultaneously configured with both Type-1 HARQ-ACK codebook and dci-enabler. For Type-1 HARQ-ACK codebook determination, each candidate PDSCH reception occasion is mapped to one HARQ-ACK information bit. However, if high layer signaling configures dci-enabler with the presence of 1-bit Enabling/disabling HARQ-ACK feedback indication in DCI format 4_2, if the DCI format 4_2 is detected, for a UE, whether to generate 1 bit ACK/NACK according to 1 bit Enabling/disabling HARQ-ACK feedback indication or always generate 1 bit ACK/NACK by ignoring dci-enabler is not clear for the Type-1 HARQ-ACK codebook. To make it clear, it is necessary to restrict dci-enabler only applicable to Type-2 HARQ-ACK codebook and ignored to Type-1 HARQ-ACK codebook.. -. Summary of change:; Add descriptions that dci-enabler is applicable if the UE is provided pdsch-HARQ-ACK-Codebook = dynamic, and ignored if the UE is provided pdsch-HARQ-ACK-Codebook = semi-static.. Consequences if not approved:; Type-1 HARQ-ACK codebook is ambiguous when UE is simultaneously configured with both Type-1 HARQ-ACK codebook and dci-enabler.. Clauses affected:; 18. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:; This is the first version for this CR.. # 18 Multicast Broadcast Services This clause is applicable only for PDCCH receptions, PDSCH receptions, and PUCCH transmissions for MBS on a serving cell. DCI formats with CRC scrambled by G-RNTI or G-CS-RNTI scheduling PDSCH receptions are referred to as multicast DCI formats and the PDSCH receptions are referred to as multicast PDSCH receptions. DCI formats with CRC scrambled by MCCH-RNTI or G-RNTI for MTCH scheduling PDSCH receptions are referred to as broadcast DCI formats and the PDSCH receptions are referred to as broadcast PDSCH receptions. HARQ-ACK information associated with multicast DCI formats or multicast PDSCH receptions is referred to as multicast HARQ-ACK information. <Unchanged part omitted> A PDSCH reception providing an initial transmission of a transport block is scheduled only by a multicast DCI format. For the first HARQ-ACK reporting mode, a PDSCH reception providing a retransmission of the transport block can be scheduled either by a multicast DCI format using a same G-RNTI as the G-RNTI of the initial transmission of the transport block, or by a unicast DCI format using a C-RNTI [6, TS 38.214]. An activation for SPS PDSCH receptions using a G-CS-RNTI for a corresponding SPS PDSCH configuration is provided only by a multicast DCI format as described in clause 10.2 by replacing CS-RNTI with the G-CS-RNTI. A release for SPS PDSCH receptions using a G-CS-RNTI for a corresponding SPS PDSCH configuration is provided by a multicast DCI format as described in clause 10.2 by replacing CS-RNTI with the G-CS-RNTI, or by a DCI format with CRC scrambled by CS-RNTI. For the first HARQ-ACK reporting mode and for a transport block that a UE received in a SPS PDSCH, a PDSCH reception providing a retransmission of the transport block can be scheduled either by a unicast DCI format using a CS-RNTI or by a multicast DCI format using a same G-CS-RNTI as the G-CS-RNTI of the initial transmission of the transport block [6, TS 38.214]. A UE can be configured per G-RNTI or per G-CS-RNTI, by harq-FeedbackEnablerMulticast with value set to 'enabled', to provide HARQ-ACK information for PDSCH receptions. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI or G-CS-RNTI, the UE does not provide HARQ-ACK information for respective PDSCH receptions. If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI, the UE determines whether or not to provide the HARQ-ACK information for PDSCH receptions based on an indication by the multicast DCI format associated with the G-RNTI or the G-CS-RNTI [4, TS 38.212] if the UE is provided pdsch-HARQ-ACK-Codebook = dynamic, or provides the HARQ-ACK information for PDSCH receptions and ignores an indication in the multicast DCI format associated with the G-RNTI or the G-CS-RNTI [4, TS 38.212] if the UE is provided pdsch-HARQ-ACK-Codebook = semi-static. If a UE would multiplex multicast HARQ-ACK information according to the second HARQ-ACK reporting mode with multicast HARQ-ACK information according to the first HARQ-ACK reporting mode, or unicast HARQ-ACK information, or CSI reports in a first PUCCH or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH when the UE is provided moreThanOneNackOnlyMode, the UE considers that the UE would transmit the second PUCCH using any PUCCH resource from the PUCCH resources associated with the second HARQ-ACK reporting mode when all values of the HARQ-ACK information are 'ACK'. If a UE is provided multiple G-RNTIs or G-CS-RNTIs, a configuration for a HARQ-ACK codebook type applies to all G-RNTIs or G-CS-RNTIs. If a UE is provided pdsch-HARQ-ACK-Codebook = semi-static for multicast HARQ-ACK information, the UE generates a Type-1 HARQ-ACK codebook as described in clauses 9.1.2, 9.1.2.1, and 9.1.2.2. If a UE is provided pdsch-HARQ-ACK-Codebook = dynamic for multicast HARQ-ACK information, the UE generates a Type-2 HARQ-ACK codebook as described in clause 9.1.3.1. If a UE would report unicast HARQ-ACK information and multicast HARQ-ACK information with same priority index in a slot, the UE multiplexes the unicast HARQ-ACK information and the multicast HARQ-ACK information following the procedures in this clause and in clauses 9.1.2 and 9.1.3. <Unchanged part omitted>