3GPP TSG-RAN WG1 #110bis-e	R1-2210174

e-meeting, October 10th –19th, 2022

Agenda Item:	9.5.1.1

Source:	Ericsson

Title:	Evaluation of NR SL positioning and ranging

Document for:	Discussion, Decision

# Introduction

In this contribution we provide our simulation results for positioning and ranging. The simulations focus on the V2X Highway deployment. In the appendix the simulation results are tabulated.

# Simulation results

## Positioning

A number of use cases are agreed to be evaluated. In this contribution we are presenting results for the V2X highway scenario. In the previous meeting the following agreement was made related to V2X positioning.

In order to develop an understanding of the achievable performance and impact of certain design choices, simulations for the agreed highway scenario have been performed. The deployment is shown in Figure 1, showing the hexagonal three-sector site macro deployment, as well as the UE-RSUs and UEs. Additionally, the lanes of the highway are shown in blue, and the orientation of the different nodes are shown with line segments. It can also be observed that UEs are dropped at the centre of the deployment, essentially within the centre most macro cell.

Figure 1: Highway deployment with macro gNBs (red circle), UE RSUs (blue cross) and vehicular UEs (green dot), including lines indicating node orientations.

Simulations are following the agreed simulation assumptions, with details found in Table 1 and Table 2.

Table 1: Simulation parameters

| Parameter | Value |
| --- | --- |
| Carrier frequency, GHz | 4GHz (Uu), 6GHz (SL) |
| Bandwidth for all links, MHz | [10 20 100] MHz |
| Subcarrier spacing, kHz | 30kHz |
| gNB channel model | TR38901 5G RURAL MACRO |
| UE-RSU channel model | TR37885 5G V2V HIGHWAY |

In Figure 2, horizontal positioning performance is shown for round trip time (RTT) based and time difference of arrival (TDOA) based positioning performed between RSUs and vehicle UEs. For comparison, the performance of Uu based DL-TDOA performed between macro gNBs and vehicle UEs is shown. The simulations have been performed with three different SRS/PRS bandwidths, 10, 20 and 100 MHz.

From Figure 2 it is clear that SL RTT and SL-TDOA has almost the same performance, under ideal assumptions. It should be noted that the RSU are perfectly synchronized in these simulations. If synchronization errors would be modelled, RTT is expected to outperform TDOA due to its inherent robustness to such errors. Moreover, since Multi-RTT positioning requires one less reference point than TDOA, one can expect that in a more sparse deployment of RSUs, Multi-RTT performs better than TDOA.

Figure 2: Simulation results for the Highway deployment, with RSU based SL RTT and TDOA positioning evaluated for different SRS bandwidths. For comparison, Uu based DL-TDOA using macro gNBs is shown.

For this Highway scenario, the results indicated that large signal bandwidth is needed to meet the Set A (1.5m@90%) and Set B (0.5m@90%) horizontal positioning requirement, at least with timing based methods only. Here,  only the Set A requirement are met with 100 MHz of bandwidth with an accuracy of 0.7m@90% for SL RTT/TDOA.

Looking at the performance of Uu based positioning, it is interesting to observe that both Set A and Set B requirements can be met with 100 MHz of bandwidth. This highlights the relevance of Uu based positioning for vehicular positioning, and additionally it points towards the potential of hybrid positioning with Uu and SL measurements.

SL positioning using RTT and TDOA are observed to have the same performance under the assumption of perfect RSU synchronization. RTT is expected to outperform TDOA under realistic sync conditions.

Uu based positioning (DL-TDOA) is observed to outperform SL based positioning for the given deployments. This due to larger output powers and a better deployment for positioning.

Figure 3: Longitudinal (along the road) and lateral (across road) positioning performance for SL RSU based RTT/TDOA and, for comparison, macro gNB based DL-TDOA.

For many vehicle safety applications, the information of the lateral placement on the road is essential. In Figure 3, the lateral and longitudinal positioning performance is shown for both RSU bases SL positioning and macro gNB based Uu positioning for the case of 20 MHz bandwidth. As can be seen, SL positioning using roadside RSU deployment can deliver accurate longitudinal performance (along the road), but fail to deliver sufficiently accurate positioning to place the vehicle in the correct lane with a lateral error of 3.5m@90%. Comparing with the performance of Uu based DL-TDOA, it is observed that the lateral positioning performance is significantly better with a lateral error of 1.2m@90%. Again, this highlights the relevance of Uu based positioning for vehicular positioning, and points towards the potential of hybrid positioning with Uu and SL measurements.

Capture the following observation in the TR: The RSU deployment for highway, with nodes deployed on each side of the road, is not optimal for positioning. Lateral accuracy is improved when the RSU deployment is complemented with gNBs deployment.

## Ranging

In the previous meeting the following agreement was made related to ranging.

In the evaluations, we consiter two different timing based ranging techniques, i.e., RTT and timing advance (TA) assisted ranging. The former rely on time of arrival measurmetn in both directions, where we assume perfect RxTx time measurements. The latter is described in our previous contribution [2], where we propose a way to exploit measurements and procedures over the UU interface to achieve a scalable and resource efficient one-way ranging solution. The method exploits that aggregated timing advance (TA) information can be used to resolve the unknown timing between two UEs. The method makes use of the aggregate TA for two users along with time-of-arrival (TOA) measurements on a single SL PRS transmission performed with UE specific Uu link timing. The estimated time of flight (TOF) between two UEs can then be estimated as:

TOF=TOA-TA2-TA1

where TA1 and TA2 is the aggregated TA for the two UEs at the gNB. The TOF error can then be expressed as the TOF error, Etof, can be modelled as:

Etof=Etoa-Eta,2-Eta,1

In the simulation results provided in this contribution we model the TA errors (Eta,n) as the sum of the TOA errors of an UL SRS and DL PRS transmitter over UU, and the TOA error (Etoa) as the TOA error of a SL PRS transmission between devices.

The simulation results for V2X is presented in the next section.

### V2X ranging

To evaluate the ranging performance for V2X, we consider the Highway deployment. Since we only consider vehicle-to-vehicle ranging we remove the RSUs from the deployment, but keep the macro gNBs as to be able to evaluate the TA assisted ranging performance. The deployemnt is illustrated in Figure 4, where a set of assisting UEs perform ranging with other UEs, and additionally perform TA related measurements with the gNBs.

Figure 4: V2X Highway deployment, with assisting UEs, regular UEs and gNBs.

In Figure 5 the ranging performance is shown for the proposed methods, evaluated for different bandwidths. Note that the same bandwidth is used for all links (Uu and SL) during a simulation. As can be seen RTT based ranging outperform TA assisted one-way-ranging ranging, but it should be noted that this comes at a cost of larger resource utilization. Specifically in scenarios and use cases where ranging needs to be performed between one UE and a number of N other UEs. There, RTT resource utilization scales as 2*N, while it’s equal to 1 for TA assisted one-way-ranging independent of number of other UEs (assuming the TA information is anyway available). The two methods can therefore complement each other.

Accurate SL RTT ranging and TA assisted (hybrid) one-way-ranging can be complementary solutions, trading accuracy for resource efficiency.

Coming back to Figure 5, RTT can reach 2.3m@90% with 100MHz bandwidth, and TA assisted (hybrid) one-way-ranging achieving 8.0m@90% with the same bandwidth.

Figure 5: Ranging performance with RTT and TA assisted (hybrid) ranging, with different bandwidths evaluated for the involved links.

To further gain insights in the ranging performance at different distances, the estimated RTT range is plotted against the distance between involved UEs is shown in Figure 6. As can be seen, the error does not grow with distance, as may be expected. On the contrary, the variance of the range error seems to be larger at close distance. As of now, we cannot provide a compelling answer to way, but it’s likely a consequence of the properties of the used channel model (TR37885 5G V2V HIGHWAY).

Figure 6: True range plotted against estimated RTT based range for Highway scenario.

# Conclusions

In the previous sections we made the following observations:

Observation 1	SL positioning using RTT and TDOA are observed to have the same performance under the assumption of perfect RSU synchronization. RTT is expected to outperform TDOA under realistic sync conditions.

Observation 2	Uu based positioning (DL-TDOA) is observed to outperform SL based positioning for the given deployments. This due to larger output powers and a better deployment for positioning.

Observation 3	Accurate SL RTT ranging and TA assisted (hybrid) one-way-ranging can be complementary solutions, trading accuracy for resource efficiency.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Capture the following observation in the TR: The RSU deployment for highway, with nodes deployed on each side of the road, is not optimal for positioning. Lateral accuracy is improved when the RSU deployment is complemented with gNBs deployment.

# References

RP-213588, Study on expanded and improved NR positioning,  RAN#94e, 2021

R1-2210175, On potential solutions for SL positioning, Ericsson, RAN1#110b-e, 2022

# Appendix: Simulation result summary

Table 2: Common assumption for all scenarios

| Parameter |  |
| --- | --- |
| Carrier frequency | 4GHz (Uu), 6GHz (SL) |
| Subcarrier spacing | 30 kHz |
| Reference Signal Transmission Bandwidth | [10 20 100] MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | - |
| Reference signal including PRS, SRS and SL-PRS (type of sequence, number of ports, …) | SRS (SL,UL): 2 symb, 4 comb PRS (DL): comb 12 |
| Number of symbols used per occasion | (see above) |
| number of occasions used per positioning estimate | 1 |
| Power-boosting level | - |
| Uplink power control (applied/not applied) | - |
| interference modelling (ideal muting, or other) | No interference modelled |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Robust least square |
| Synchronization assumptions | Perfect sync |
| UE/gNB RX and TX timing error assumption | Perfect timing |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Assume single dual polarized antenna element |
| Additional notes, if any |  |

##### Highway scenario for V2X use case simulation results

Table h-2 Simulation results for highway for absolute positioning - horizontal accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A (1.5m) | Whether meet the requirement of set B (0.5m) |
| --- | --- | --- | --- | --- | --- | --- |
| BW#10M, FR#1, #TDOA | 1.4 | 2.1 | 3.1 | 4.5 | No (54%) | No (12%) |
| BW#20M, FR#1, #TDOA | 1.0 | 1.6 | 2.3 | 3.5 | No (64%) | No (18%) |
| BW#100M, FR#1, #TDOA | 0.2 | 0.3 | 0.5 | 0.7 | Yes | No (81%) |
| BW#10M, FR#1, #Multi-RTT | 1.4 | 2.1 | 3.1 | 4.5 | No (54%) | No (12%) |
| BW#20M, FR#1, #Multi-RTT | 1.0 | 1.6 | 2.3 | 3.5 | No (64%) | No (18%) |
| BW#100M, FR#1, #Multi-RTT | 0.2 | 0.3 | 0.5 | 0.7 | Yes | No (81%) |

Table h-6 Simulation results for highway for ranging - distance accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A (1.5m) | Whether meet the requirement of set B (0.5 m) |
| --- | --- | --- | --- | --- | --- | --- |
| BW#10M, FR#1, #RTT | 1.1 | 1.8 | 2.7 | 4.2 | No (59%) | No (25%) |
| BW#20M, FR#1, #RTT | 0.9 | 1.5 | 2.2 | 3.5 | No (68%) | No (31%) |
| BW#100M, FR#1, #RTT | 0.4 | 0.7 | 1.2 | 2.5 | No (83%) | No (57%) |
| BW#10M, FR#1, #TA-OWR | 4.2 | 6.7 | 9.8 | 14.8 | No (19%) | No (6%) |
| BW#20M, FR#1, #TA-OWR | 3.2 | 5.1 | 8.0 | 13.1 | No (27%) | No (9%) |
| BW#100M, FR#1, #TA-OWR | 1.7 | 2.7 | 4.3 | 7.9 | No (46%) | No (19%) |