3GPP TSG-RAN WG1 Meeting #110b-e	R1-2210203

e-Meeting, October 10-19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | XX | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE-A is the destination UE of a TB transmitted by UE-B in TS 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per RAN1#106-e agreement below, a destination UE of a TB transmitted by UE-B can be UE-A for explicit request-based scheme 1. Agreement: - In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by an explicit request in Mode 2: - A UE that sends an explicit request for inter-UE coordination information can be UE-B - A UE that received an explicit request from UE-B and sends inter-UE coordination information to the UE-B can be UE-A - Working assumption At least a destination UE of a TB transmitted by UE-B can be UE A - The above feature can be enabled or disabled or controlled by (pre-)configuration - FFS: Details on how to support this, including (pre-)configuration signaling granularity - FFS: Additional details and conditions on UE-A and UE-B For preferred resource set case, MAC spec has already captured this in section 5.22.1.1 of TS 38.321 as below (see underlined part below) 5.22.1.1 SL Grant reception and SCI transmission <other parts are omitted> 3> if configured by RRC, interUECoordinationScheme1Explicit or interUECoordinationScheme1Condition enabling reception of preferred resource set and non-preferred resource set and when the UE does not have own sensing result as specified in clause 8.1.4 of TS 38.214 [7] and if a preferred resource set is received from a UE: 4> randomly select the time and frequency resources for one transmission opportunity from the resources belonging to the received preferred resource set for a MAC PDU to be transmitted to the UE providing the preferred resource set, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. <other parts are omitted> However, the case of non-preferred resource is not captured yet. Given that non-preferred resource is handled in PHY during sensing and resource exclusion procedure, it is proposed to capture this case in PHY. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When inter-UE coordination information is triggered by an explicit request from the UE-B and UE-A determines non-preferred resource for UE-B, clarify that UE-A is the destination UE of a TB transmitted by UE-B. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN1 agreement is not fully captured, i.e., it’s missing in specification that UE-A needs to be the destination UE of a TB transmitted by UE-B for Scheme 1 “explicit request-based, non-preferred resource” case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.1.4A	UE procedure for determining a set of preferred or non-preferred resources for another UE's transmission

< Unchanged parts are omitted >

When this procedure is triggered by another UE's explicit request, the fields in the request are interpreted as follows:

-	The field 'Resource selection window location' is the concatenation of the starting time location and the ending time location of the resource selection window. The starting and ending time locations of the resource selection window are each encoded in the same way as the reference slot as described in clause 8.1.5A.

-	The field 'Resource reservation period' is encoded in the same way as the field of the same name in SCI format 1-A.

A UE shall not determine a non-preferred resource set unless the UE is a destination UE of a TB to be transmitted by the UE who transmitted the explicit request.

When determining a preferred resource set, the UE applies the procedure described in clause 8.1.4 with the above parameters and the following modifications:

-	Step 6a) The UE excludes candidate single-slot resource(s) belonging to slot(s) where the UE does not expect to perform SL reception of a TB due to half-duplex operation, if all the following conditions are met:

-	the UE is a destination UE of the TB for whose transmission the preferred resource set is being determined;

-	the higher layer parameter condition1A2Scheme1Disabled is not set to 'Disabled'.

When determining a non-preferred resource set, the UE considers any resource(s) within the resource selection window, if indicated by a received explicit request, and satisfying at least one of the following conditions as non-preferred resource(s):

-	resource(s) indicated by a received [SCI format 1-A], satisfying at least one of the following criteria:

-	the RSRP measurement performed, according to clause 8.4.2.1, for the received [SCI format 1-A], is higher than ThprioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The internal parameter Th(pi) is set to the corresponding value of RSRP threshold indicated by the k-th field in thresholdRSRPCondition1B1Option1Scheme1, where k=pi.

-	the UE is a destination UE of a TB associated with the received [SCI format 1-A] and the RSRP measurement performed, according to clause 8.4.2.1 for the received [SCI format 1-A], is lower than Th'prioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The internal parameter Th'(pi) is set to the corresponding value of RSRP threshold indicated by the k-th field in thresholdRSRPCondition1B1Option2Scheme1, where k=pi.

-	resources(s) in slot(s) in which the UE does not expect to perform SL reception due to half duplex operation, if the UE is a destination UE of a TB for whose transmission the non-preferred resource set is being determined.

< Unchanged parts are omitted >