3GPP TSG RAN WG1 Meeting #110bis-e	R1- 2210236

Electronic Meeting, Oct. 10th – 19th, 2022

Title:	[Draft] Reply to RAN4 LS LS on Pemax,c of S-SSB transmission when multiple resource pool is configured in a carrier

Response to:	R4-2214421

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	Qualcomm [RAN1]

To:	RAN2

Cc:

Contact person:

Name:	Gabi Sarkis

E-mail Address:	gsarkis@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# Overall Description

RAN1 thanks RAN4 for their LS on Pemax,c of S-SSB transmission and would like to provide a reply to the included questions.

Question 1: RAN4 agreed not to define PEMAX,c for S-SSB configured Tx Power in TS38.101-1 as highlighted yellow below and would like to know if this can have any impact to RAN1/2, for example, Pcompensation defined in idle mode procedure in TS 38.304. 6.2E.4 Configured transmitted power for V2X 6.2E.4.1 General The NR V2X UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds: PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c with PCMAX_L,f, c = MIN {PEMAX,c, PPowerClass, V2X – MAX(MAX(MPRc , A-MPRc) + DTIB,c , P-MPRc), PRegulatory,c } PCMAX_H,f, c = MIN {PEMAX,c, PPowerClass, V2X, PRegulatory,c } where - PCMAX,f,c is configured for PSSCH\PSCCH, S-SSB and PSFCH, respectively; - For the total transmitted power PCMAX,PSSCH/PSCCH, PEMAX,c is the value given by IE sl-maxTransPower, defined by TS 38.331. - For the total transmitted power PCMAX,S-SSB, the PCMAX_L,f,c and PCMAX_H,f,c are defined as follows: PCMAX_L,f,c = MIN {PPowerClass, V2X – MAX(MAX(MPRc , A-MPRc) + DTIB,c , P-MPRc), PRegulatory,c} PCMAX_H,f,c = MIN {PPowerClass, V2X, PRegulatory,c} RAN4 would like to know if there is an impact on RAN1 and RAN2 about the agreement RAN4 made on S-SSB.

The below transmit power equation in TS 38.213, uses PCMAX from TS 38.101-1 as an input and hence no impact to on RAN1 specifications is expected:

### 16.2.0 S-SS/PSBCH blocks A UE determines a power PS-SSB(i) for an S-SS/PSBCH block transmission occasion in slot i on active SL BWP b of carrier f as PS-SSB(i)=minPCMAX,PO,S-SSB+10log102μ∙MRBS-SSB+αS-SSB⋅PL [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PO,S-SSB is a value of dl-P0-PSBCH if provided; else, PS-SSB(i)=PCMAX - αS-SSB is a value of dl-Alpha-PSBCH, if provided; else, αS-SSB=1 - PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - MRBS-SSB=11 is a number of resource blocks for a S-SS/PSBCH block transmission with SCS configuration μ

# Actions

To RAN4:

Action: RAN1 kindly asks RAN4 to take the above reply into account for further work.

# Dates of Next TSG-RAN1 Meetings:

RAN1 #111			14 November – 18 November 2022				             Toulouse