3GPP TSG RAN WG1 #110bis-e			           R1-220XXXX

e-Meeting, Oct. 10th– 19th, 2022

Agenda Item: 8.7

Source: Moderator (MediaTek)

Title: Summary#1 for maintenance on UE power saving enhancements

Document for: Discussion and Decision

# Introduction

This summary is for maintenance of R17 UE power saving enhancements based on reports of RAN1#110 [1]-[2] and companies’ submissions to RAN1#110bis-e [3]-[25]. The following are to be discussed and decided:

- Section 2: Maintenance on DCI-Based Power Saving Adaptation

- Section 3: Maintenance on Idle/Inactive UE power saving enhancements

After 1st_round of email discussion, session chair has decided the issues/questions for further discussion. Accordingly, proposals labelled with “2nd_round” are provided further. In summary section, there suggest simple alignment CRs for GTW2 on Wed.

# Maintenance on DCI-Based Power Saving Adaptation

In this section, companies’ inputs related to DCI based power saving adaptation are summarized and categorized. Potential issues are then identified for companies to check and provide views.

## PDCCH monitoring adaptation and active BWP change

Table 1: Companies’ views related to PDCCH monitoring adaptation and active BWP change from [3] - [25]

| Company | Views |
| --- | --- |
| vivo | CR in R1-2208607 ## TS 38.213 ## 10.4 Search space set group switching and skipping of PDCCH monitoring *** Unchanged text is omitted *** If the UE changes to a new active DL BWP of the serving cell by the expiration of bwp-InactivityTimer in a PDCCH skipping duration, the UE - resumes PDCCH monitoring according to the search space sets on the new active BWP of the serving cell, if the UE is in a PDCCH skipping duration and not provided searchSpaceGroupIdList-r17 on the new active DL BWP - otherwise, monitors PDCCH according to search space sets with group index 0 on the new active BWP of the serving cell.. |
| OPPO | CR in R1-2208839 TP to Clause 10.4, TS 38.213 <Unchanged part omitted> When the PDCCH monitoring adaptation field indicates to a UE to start PDCCH monitoring according to search space sets with a first group index and stop PDCCH monitoring according to search space sets with a second group index, the UE applies the indication - at the beginning of a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field when μ∈0, 1, 2, 3, if the DCI does not indicate an active DL BWP change for the serving cell, - at the beginning of a first slot, of a slot group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field when μ∈5, 6, if the DCI does not indicate an active DL BWP change for the serving cell, - at the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI, if the DCI also indicates an active DL BWP change for the serving cell <Unchanged part omitted>. |
| xiaomi | CR in R1- 2209252 If PDCCH monitoring adaptation and BWP switching are indicated in the same DCI, the PDCCH monitoring adaptation will be applied after BWP switching on the new BWP. |
| Nokia | CR in R1-2209377 Observation 2: When PDCCH monitoring adaptation is triggered in same DCI as BWP switch, it would seem simplest to ignore the PDCCH monitoring adaptation indication, and fall back to default operation on target BWP. Proposal 2: After BWP switch, UE falls back to default PDCCH monitoring of the target BWP. Proposal 3: Adopt following TP to TS38.213 Section 10.4: TP to Clause 10.4, TS 38.213 <Unchanged part omitted> If the UE changes to a new active DL BWP of the serving cell by the expiration of bwp-InactivityTimer or when DCI indicates change of active DL BWP change when PDCCH monitoring adaptation is appliedin a PDCCH skipping duration, the UE - resumes PDCCH monitoring according to the search space sets on the new active BWP of the serving cell, if the UE is not provided searchSpaceGroupIdList-r17 on the new active DL BWP - otherwise, monitors PDCCH according to search space sets with group index 0 on the new active BWP of the serving cell. <Unchanged part omitted>. |
| Huawei, HiSilicon | CR in R1-2209817 ## TS 38.213 ## 10.4 Search space set group switching and skipping of PDCCH monitoring *** Unchanged text is omitted *** When the PDCCH monitoring adaptation field indicates to a UE to start PDCCH monitoring according to search space sets with a first group index and stop PDCCH monitoring according to search space sets with a second group index on an active DL BWP of a serving cell, the UE applies the indication - at the beginning of a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field when μ∈0, 1, 2, 3, if the DCI does not indicate an active DL BWP change for the serving cell - at the beginning of a first slot, of a slot group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field when μ∈5, 6, if the DCI does not indicate an active DL BWP change for the serving cell - at the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI, if the DCI also indicates an active DL BWP change for the serving cell When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field if the DCI does not indicate an active DL BWP change for the serving cell. Otherwise, the UE starts skipping of PDCCH monitoring at the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI. If the UE transmits a PUCCH providing a positive SR after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring starting at the beginning of a first slot that is after a last symbol of the PUCCH transmission. During the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running, the UE shall not skip PDCCH monitoring on SpCell. If the DRX group of the serving cell is configured and enters outside Active Time, the UE terminates PDCCH skipping for the serving cell. *** Unchanged text is omitted ***. |
| Ericsson | - Proposal 3 A UE ignores the PDCCH monitoring adaptation field in a DCI indicating a BWP switch. Endorse draft CR in R1-2209856. - “A UE ignores the PDCCH monitoring adaptation field in a DCI indicating an active BWP switch.” |

Moderator would like to thank companies’ valuable inputs. From the above table, there are two issues identified, and, correspondingly, we have the following two questions for companies’ check:

Question 2.1-1 (1st_round): Whether/how to discuss and decide UE behavior on PDCCH monitoring adaptation when receiving a scheduling DCI that indicates active BWP change? If you agree to discuss the question, what is your preference/view on the following alternatives:

- Alt1: Interpretation on the PDCCH monitoring adaptation field follows clause 12 of TS 38.213, but there is spec impact to clarify the application timing of SSSG switching or PDCCH skipping (e.g. after active BWP change).

- Alt2: Skip interpretation on the PDCCH monitoring adaptation field, but there is still spec impact, e.g., for reusing the same UE behavior as the case where active BWP change is triggered by the expiration of bwp-InactivityTimer

- Alt 3: If neither of the above two alternatives is preferred, do you have any other suggestion?

Table 2 (1st_round): Companies’ answers to Question 2.1-1

| Company name | Support to discuss this issue (Y/N)? | If you support to discuss the issue, what will be your preference/view in Alt 1, Alt 2 or Alt 3 in the above? |
| --- | --- | --- |
| Xiaomi | Y | Prefer Alt 1. Alt 2 is not power saving since it will need another DCI to indicate PDCCH monitoring adaptation after switching to the new BWP. |
| Nordic | N | Current status is Alt1, with exception that UE applies SSSG switching and skipping during the BWP switching. |
| Nokia1 | Y | Prefer Alt2 to clarify the expected UE behaviour. |
| OPPO | Y | We prefer Alt1 but we also fine with Alt2. The ambiguity issue should be fixed. |
| Spreadtrum | Y | Alt 1. BWP switching and PDCCH adaptation are decoupled, although BWP switching and cross-slot scheduling are coupled in R16 discussion. They are different. PDCCH adaptation does not relate to PDSCH reception. |
| Huawei, HiSilicon | Y | Alt.1 is already specified in TS 38.213 as following. There needs technical benefit to justify any UE behaviour change compared with that specified from Rel-15. <skipped text> If a bandwidth part indicator field is configured in a DCI format and indicates an UL BWP or a DL BWP different from the active UL BWP or DL BWP, respectively, the UE shall - for each information field in the DCI format - if the size of the information field is smaller than the one required for the DCI format interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, the UE prepends zeros to the information field until its size is the one required for the interpretation of the information field for the UL BWP or DL BWP prior to interpreting the DCI format information fields, respectively - if the size of the information field is larger than the one required for the DCI format interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, the UE uses a number of least significant bits of the DCI format equal to the one required for the UL BWP or DL BWP indicated by bandwidth part indicator prior to interpreting the DCI format information fields, respectively - set the active UL BWP or DL BWP to the UL BWP or DL BWP indicated by the bandwidth part indicator in the DCI format. Anyway, the issue needs discussion and clarification. |
| vivo | N | - In case of a scheduling DCI indicating PDCCH monitoring adaptation and BWP switching simultaneously, it is in principle to apply the SSSG or PDCCH skipping to the target BWP. And with this clarification, no additional specification change is needed. That is, - For PDCCH skipping, the duration is counted at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field - For SSSG Switching, the timer is counter after the P_switch Even without these specification impact to clarify the application delay of SSSG switching or PDCCH skipping, it can still work, since most time the BWP switching delay is larger than the application delay. |
| LGE | N | We don’t think there is need to discuss. UE does not expect to receive a DCI indicating PDCCH skipping/SSSG switching after active BWP change. We don’t think this behavior is realistic and likely to happen. |
| Samsung | N | There is no correction that needs to be made. Contents of a DCI indicating active BWP change are applicable to the indicated BWP (and the DCI takes effect at the slot indicated by the TDRA field – i.e. no need for any discussion or new specifications. |
| Qualcomm | Y | Alt 1. We share a similar view as vivo and believe that the specification will not completely break even without any changes. However, as the FL commented, we may need some clarification on the application time, especially when the numerologies are different across BWPs. |
| Ericsson1 | Y | Prefer Alt 2 and OK to discuss if there is any additional spec impact. |
| MediaTek | Y | From UE perspective, the principle to accommodate active BWP change is not to processing anything before finishing the active BWP change. Existing specification, however, requires UE to take into account the time during active BWP change so as to decide the precise UE behavior after the active BWP change. This somehow conflicts with the principle. In this regard, clarification is needed to ensure aligned understanding between UE and gNB. Regarding Alt 1 or Alt 2, we have the preference on Alt 1 since exiting interpretation rule on DCI can be reused. Alt 2 requires additional handling, which does not simplify UE implementation. |
| CATT | N | PDCCH monitoring adaptation is per BWP. Current specification is clear that UE would still perform PDCCH monitoring adaptation (skipping or SSSG switching) at the current BWP and prepare the BWP switching to the new BWP. Thus, UE would not perform PDCCH monitoring during BWP switching. Once UE completes the BWP switching to new BWP, UE starts the PDCCH monitoring adaptation based on the configuration of the new BWP. No further discussion is needed. |
| ZTE, Sanechips | Y | Alt1 is preferred. The application delay of SSSG switch and PDCCH skipping should be clarified when it is indicated by BWP switching DCI. |

Question 2.1-1 (1st_round):

- Y (9 companies):

  - Alt 1 (7): Xiaomi,  OPPO, Spreadtrum, HW, QC, MTK, ZTE

  - Alt 2 (3): Nokia, OPPO, Ericsson

- N (5 companies): Nordic, vivo, LGE, Samsung, CATT

- Support rate for further discussion: 64%

  - Within the supporting companies, Alt 1: 78%, Aspect 2: 33%

Guidance from session chair Xiaodong (CMCC):

No further discussion in this meeting

Question 2.1-2 (1st_round): Whether to discuss and decide removing the condition “in a PDCCH

skipping duration” in the quoted text of clause 10.4 of TS 38.213?

- According to R1-2208607 (vivo), the UE behaviors in the subbullets of the quoted text are required, irrespective of the condition “in a PDCCH skipping duration”

- Note: There may require revision to the sub-bullet(s), e.g., “resumes PDCCH monitoring” may not be accurate if UE already monitors PDCCH before the active BWP change.

If the UE changes to a new active DL BWP of the serving cell by the expiration of bwp-InactivityTimer in a PDCCH skipping duration, the UE - resumes PDCCH monitoring according to the search space sets on the new active BWP of the serving cell, if the UE is not provided searchSpaceGroupIdList-r17 on the new active DL BWP - otherwise, monitors PDCCH according to search space sets with group index 0 on the new active BWP of the serving cell.

Table 3 (1st_round): Companies’ answers to Question 2.1-2

| Company name | Support to discuss this issue (Y/N)? | If you support to discuss the issue, what will be your preference/view in removing the condition (“in a PDCCH skipping duration”) and any suggested revision to the subbullet(s)? |
| --- | --- | --- |
| Xiaomi | N | Currently we don’t see the reason to associate SSSG configuration and PDCCH skipping resuming on the new BWP. |
| Nordic | Y | change “resumes“ to “monitors” |
| Nokia1 | Y | Support the removing in principle, detailed wording (and changes in sub-bullet) can be further discussed based on Q 2.1-1 outcome. |
| OPPO | Y(with some clarification) | It seems the proposal is to make the behaviour clear and general. The removing “In a PDCCH skipping duration” seems not enough. We should also remove “by the expiration of bwp-InactivityTimer” In the specification, there is no definition about when a BWP activated, what SSSG will be used. We would like to have solution in thorough way. |
| Spreadtrum | N | Coupling BWP switching and PDCCH adaptation is not good for spec evolution. For “outside any PDCCH skipping duration”, the current spec can also work. |
| Huawei, HiSilicon | N | We don’t see any problem to keep the “in a PDCCH skipping duration”. Also, the first sub-bullet mentions “resume PDCCH monitoring”. It is reasonable to keep “in a PDCCH skipping duration” in the main sentence. |
| vivo | Y | In fact, there is no need to limited search space sets group switching to group index 0 by the condition “in a PDCCH skipping duration”, if the UE changes to a new active DL BWP of the serving cell by the expiration of bwp-InactivityTimer. |
| LGE | Y | We are okay to remove the condition because UE’s resuming PDCCH monitoring behavior in the new active BWP is common regardless of whether the UE’s changing BWP is within in a PDCCH skipping duration or not. |
| Samsung | Y | Agree with Nordic – current text can be confusing and is basically incorrect |
| Qualcomm | Y | We are okay to discuss this issue. Also, we think it can be further generalized. Similar to the Rel-16 K0_min/K2_min adaptation, the behavior when the BWP switching is triggered by a DCI without a PDCCH monitoring adaptation field can be specified (i.e., when the source BWP is not configured with a PDCCH monitoring adaptation, while the target BWP is). |
| Ericsson1 | Y | OK to discuss - there should be no impact to Rel-16 SSSG switching. |
| MediaTek | Y | While current correction is still valid, the question motivates us to extend the UE behavior to cover the case without “in a PDCCH skipping duration”. In this regard, we are supportive of the suggested revision in R1-2208607. |
| CATT | N | We believe that the current spec is clear enough without any essential correction. |
| ZTE, Sanechips | Y | Okay to make the spec clearer. |

Question 2.1-2 (1st_round)

- Y (10 companies): Nordic, Nokia, OPPO, vivo, LGE, Samsung, QC, Ericsson, MTK, ZTE

- N (4 companies): Xiaomi, Spreadtrum, HW, CATT

- Support rate for further discussion: 71%

Guidance from session chair Xiaodong (CMCC):

no further discussion in this meeting, since no broken issues are identifed but to extend the applicability. if it is not aligned with existing agreements, let's do it in next meeting

## Corrections Related to RAN2 LS

Table 4: Companies’ views for other topics of DCI-based PDCCH monitoring adaptation from [4]-[45]

| Company | Views |
| --- | --- |
| Nokia | CR in R1-2209376 Observation 1: RAN1 specification is not aligned with RAN2 specification in respect of PDCCH skipping behaviour in all serving cells when SR is pending. Proposal 1: Adopt following TP to TS38.213 Section 10.4: TP to Clause 10.4, TS 38.213 <Unchanged part omitted> When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. If the UE transmits a PUCCH providing a positive SR after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring starting at the beginning of a first slot that is after a last symbol of the PUCCH transmission in all serving cells of the corresponding Cell Group. During the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running, the UE shall not skip PDCCH monitoring on SpCell. If UE transmits a RACH due to positive SR, the UE shall not skip PDCCH monitoring on any serving cell of the corresponding Cell Group during the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running. If the DRX group of the serving cell is configured and enters outside Active Time, the UE terminates PDCCH skipping for the serving cell. <Unchanged part omitted>. |
| Huawei, HiSilicon | Figure 1 illustration of PDCCH skipping is indicated after a SR transmission Proposal 1: If the UE receives a DCI indicating PDCCH skipping for a duration after the UE transmits a SR, the UE ignores PDCCH skipping before the pending SR is cancelled. CR in R1-2209816 ## TS 38.213 ## 10.4 Search space set group switching and skipping of PDCCH monitoring *** Unchanged text is omitted *** When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. If the UE transmits a PUCCH providing a positive SR after the slot where a DCI format providing the PDCCH monitoring adaptation field is indicated to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring starting at the beginning of a first slot that is after a last symbol of the PUCCH transmission. If a DCI format providing the PDCCH monitoring adaptation field is indicated to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell after the UE transmits a PUCCH providing a positive SR, the UE ignores the indication of skipping PDCCH monitoring before the pending SR is cancelled [11, TS 38.321]. During the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running, the UE shall not skip PDCCH monitoring on SpCell. If the DRX group of the serving cell is configured and enters outside Active Time, the UE terminates PDCCH skipping for the serving cell. *** Unchanged text is omitted ***. |

Moderator would like to thank companies’ careful check on whether RAN2 agreements are precisely captured in RAN1 specifications. From the above table, there identified two missing aspects, which then leads to the following question for companies to check and feedback:

Question 2.2-1 (1st_round): Whether to discuss and decide inclusion of the following missing aspects regarding RAN2 agreements on PDCCH skipping:

- Aspect 1: Ensuring no PDCCH skipping on all serving cells of the corresponding Cell Group while SR is pending, according to RAN2 agreements in RAN2#119-e meeting (as quoted below):

- RAN2 #119-e (May, 2022) UE ignores PDCCH skipping on all serving cells of the corresponding Cell Group while SR is pending. Further enhancements on restricting the serving cells where UE skip PDCCH while SR is pending are not pursued.

- Aspect 2: UE ignores PDCCH skipping indication to ensure no PDCCH skipping when SR is pending (Ref: R1-2209815 and R1-2209816 by Huawei)

Table 5 (1st_round): Companies’ answers to Question 2.2-1

| Company name | Support to discuss this issue (Y/N)? | If you support to discuss the issue, what will be your views in the above Aspects 1 and 2? |
| --- | --- | --- |
| Xiaomi | Y | Agree with Aspect 1. For Aspect 2, in fact we have already discussed it in R1#109 e-meeting. and at that time FL gave an explanation that “ UE can send SR and then UE receive a UL Grant DCI with PDCCH skipping -> For this case, UE will go to PDCCH skipping, since it is naturally UE will always follows the latest command. I see no additional spec is needed for this case. UE can send SR, but gNB fails to detect it, and then gNB transmit a DL grant with PDCCH skipping. -> For this case, UE will send SR after prohibit timer, after another SR being transmitted, the PDCCH skipping will be terminated. Agreeing the P2-1a(v7) will allow us to do so.” People can check the email discussion in 20th ,May(sent by Xiaodong Shen) We are open to this issue if companies think it should be discussed again. |
| Nordic | Y | Clarify Aspect1 |
| Nokia1 | Y | In general we should aim to align with RAN2. For Aspect 2, in principle fine, but we send a LS to RAN2 in last meeting to clarify the intent of ‘ignore’ so we could wait till RAN2 reply. (Proponent of Aspect 1, so no further comment.) |
| OPPO | Y | Agree at least the Aspect1 should be addressed for all the serving cell in the CG. However, the Aspect2 may not be specially treated as we already agree that in the RAN1 we just terminate skipping after SR. |
| Spreadtrum | Y |  |
| Huawei, HiSilicon | Y | We think these two aspects should be discussed. For Aspect 2, first of all, RAN2 reached the agreement that ‘UE ignores PDCCH skipping while the SR is pending’, and the agreement is expected to be captured in RAN1 spec since RAN2 spec does not touch PDCCH skipping. The condition ‘SR is pending’ includes two scenarios: 1) PDCCH skipping is indicated and then SR is transmitted; 2) SR is transmitted and then gNB indicates PDCCH skipping. The second scenario exists when gNB indicates PDCCH skipping for a duration before gNB finishes the processing of the already transmitted SR. For the text quoted by Xiaomi, we don’t think we had further discussion on it due to there was more critical issues need to be discussed in that meeting, especially for the case 2) mentioned above. We think the issue should be clarified. We don’t think the UE should follow the PDCCH skipping indication after the SR is transmitted, which does not follow RAN2’s agreements. |
| vivo | Partial yes | Only clarification on aspect 1 is needed. And the descriptions of the UE behaviours should be consistent to single cell, i.e., not written in the way like ‘ignore…’. |
| LGE | Y | We prefer Aspect 1. |
| Samsung | Y | OK to clarify Aspect 1. |
| Qualcomm | Y | Aspect 1can be discussed. For Aspect 2, as Xiaomi quoted above, it was discussed in RAN1 #109-e, and as we commented before, it would be left up to gNB’s implementation. |
| Ericsson1 | Y | OK to discuss. |
| MediaTek | Y | For Aspect 1, current RAN1 specification does not comply with the RAN2 agreement and should be further updated. For Aspect 2, we see it is possible gNB sends skipping indication before finishing processing UE’s SR (not the case of missing UE’s SR). For such case, even UE terminates skipping after sending SR, the UE will be indicated to start a new round of PDCCH skipping, which causes additional latency for gNB to response UE’s SR. In this regard, we are supportive of discussing whether/how to handle such case. |
| CATT | Y | OK to discuss mainly for Aspect 1. |
| ZTE, Sanechips | Y | Okay to discuss Aspect 1. And according to RAN2 agreements, with pending SR, UE shall not skip PDCCH monitoring in all serving cell. Minor changes on the specification is sufficient. |

Question 2.2-1 (1st_round)

- Y (14 companies):

  - Aspect 1 (14): Xiaomi, Nordic, Nokia, OPPO, Spreadtrum, HW, vivo, LGE, Samsung, QC, Ericsson, MTK, CATT, ZTE

  - Aspect 2 (4): Xiaomi, HW, Ericsson, MTK

- Support rate for further discussion: 100%

  - Within the supporting companies, Aspect 1: 100%, Aspect 2: 28%

Guidance from session chair Xiaodong (CMCC):

Continue the discussion in this meeting on Aspect 1 from Question 2.1-1, and focus on clarification.

Based on the guidance, proposed CR in R1-2209376 (Nokia), and companies’ feedbacks, the following proposal is suggested for companies’ check

Proposal 2.2-1 (2nd_round)

- To clarify no PDCCH skipping on all serving cells of the corresponding Cell Group while SR is pending, according to RAN2 agreements in RAN2#119-e meeting, the following TP to clause 10.4 of TS 38.213, as provided in R1-2209376, is endorsed.

<Unchanged part omitted> When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. If the UE transmits a PUCCH providing a positive SR after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring starting at the beginning of a first slot that is after a last symbol of the PUCCH transmission in all serving cells of the corresponding Cell Group. During the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running, the UE shall not skip PDCCH monitoring on SpCell. If UE transmits a RACH due to positive SR, the UE shall not skip PDCCH monitoring on any serving cell of the corresponding Cell Group during the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running. If the DRX group of the serving cell is configured and enters outside Active Time, the UE terminates PDCCH skipping for the serving cell. <Unchanged part omitted>

Table 6 (2nd_round): Companies’ views on Proposal 2.2-1

| Company name | Company view(s) |
| --- | --- |

## Clarification for Mixed Rel-16 and Rel-17 Configurations

| Company | Views |
| --- | --- |
| vivo | CR in R1-2208606 TS 38.213 ## 10.4 Search space set group switching and skipping of PDCCH monitoring A UE can be provided - a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList for PDCCH monitoring on a serving cell, - a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell, - a set of durations by PDCCHSkippingDurationList for Type3-PDCCH CSS sets or USS sets for PDCCH monitoring on an active DL BWP of a serving cell. If the UE is not provided any of the above informationsearchSpaceGroupIdList for a search space set, the following procedures are not applicable for PDCCH monitoring according to the search space set(s). < Unchanged parts are omitted >. |
| ZTE, Sanechips | - CR in R1-2209191 - Proposal 2: The following change should be adopted. TS 38.213 <Unchanged parts are omitted> 10.4 Search space set group switching and skipping of PDCCH monitoring A UE can be provided - a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList for PDCCH monitoring on a serving cell, - a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell, - a set of durations by PDCCHSkippingDurationList for Type3-PDCCH CSS set or USS set for PDCCH monitoring on an active DL BWP of a serving cell If the UE is not provided searchSpaceGroupIdList for a search space set, any of the above information, the following procedures are not applicable for PDCCH monitoring according to the search space set(s). <Unchanged parts are omitted>. - Proposal 3: The following change should be adopted. TS 38.213 10.4 Search space set group switching and skipping of PDCCH monitoring <Unchanged parts are omitted> If a UE is provided a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r16, and is not provided SearchSpaceSwitchTrigger for a serving cell, - if the UE detects a DCI format by monitoring PDCCH according to a search space set with group index 0, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, for the serving cell - at the beginning of the first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format when μ∈0, 1, 2, 3, - at the beginning of the first slot, of a group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format when μ∈5, 6 the UE sets the timer value to the value provided by searchSpaceSwitchTimer if the UE detects a DCI format by monitoring PDCCH in any search space set. |
| Ericsson | - Proposal 1: Endorse draft CR in R1-2209854 to correctly capture the UE behavior with respect to PDCCH skipping behavior for Type 3 CSS and USS sets. - Clarify that if the UE is not provided searchSpaceGroupIdList for a search space set, the procedures “that are based on search space set group switching” are not applicable for PDCCH monitoring according to the search space set. ## TS 38.213 ## 10.4 Search space set group switching and skipping of PDCCH monitoring *** Unchanged text is omitted *** A UE can be provided a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList for PDCCH monitoring on a serving cell. If the UE is not provided searchSpaceGroupIdList for a search space set, the following procedures that are based on search space set group switching are not applicable for PDCCH monitoring according to the search space set. If a UE is provided cellGroupsForSwitchList, indicating one or more groups of serving cells, the following procedures apply to all serving cells within each group; otherwise, the following procedures apply only to a serving cell for which the UE is provided searchSpaceGroupIdList. When a UE is provided searchSpaceGroupIdList, the UE resets PDCCH monitoring according to search space sets with group index 0, if provided by searchSpaceGroupIdList. - *** Unchanged text is omitted ***. - Proposal 2: Endorse draft CR in R1-2209855 for corrections on SSSG switching with respect to mixed Rel-16 and 17 SSSGS combinations. - Clarify that Rel-16 SSSG switching timer is used for legacy DCI format 2_0 based SSSG switching. - Clarify that the Rel-16 behaviour when UE is not provided SearchSpaceSwitchTrigger applies “when UE is provided searchSpaceGroupIdList-r16”. |

Moderator would like to thank companies’ careful check. From the above table, there are two issues identified, and, correspondingly, the following question is provided for companies’ further check:

Question 2.3-1 (1st_round): Whether to discuss and decide updating the following paragraphs, taking into account possibly mixed Rel-16 and Rel-17 configurations?

- Paragraph 1 (the first paragraph of clause 10.4 of TS 38.213 and to be updated to avoid precluding Rel-17 PDCCH monitoring adaptation procedures when Rel-16 configuration is not provided):

A UE can be provided a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList for PDCCH monitoring on a serving cell. If the UE is not provided searchSpaceGroupIdList for a search space set, the following procedures are not applicable for PDCCH monitoring according to the search space set. …

- Paragraph 2 (to be updated to clarify that the Rel-16 behaviour when UE is not provided SearchSpaceSwitchTrigger applies “when UE is provided searchSpaceGroupIdList-r16”):

If a UE is not provided SearchSpaceSwitchTrigger for a serving cell, - if the UE detects a DCI format by monitoring PDCCH according to a search space set with group index 0, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, for the serving cell - at the beginning of the first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format when μ∈0, 1, 2, 3, - at the beginning of the first slot, of a group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format when μ∈5, 6 the UE sets the timer value to the value provided by searchSpaceSwitchTimer if the UE detects a DCI format by monitoring PDCCH in any search space set

Table 7 (1st_round): Companies’ answers to Question 2.3-1

| Company name | Support to discuss this issue (Y/N)? | If you support to discuss the issue, what will be your views updating the above Paragraph 1 and Paragraph 2? |
| --- | --- | --- |
| Nordic | Y | It is enough to fix Paragraph 1, RRC parameter alignment can be left to Spec editor |
| Nokia | Y | Fine with updates suggested by Paragraph 1 and 2. |
| Spreadtrum | Y |  |
| Huawei, HiSilicon | Y for paragraph 1 | We feel paragraph 1 may be OK. For paragraph 2, the current specification is clear based on the context. |
| vivo | Y | For Paragraph 1, the first paragraph of clause 10.4 of TS 38.213 is correct. Hence, it needs to be updated to address the condition whether the specification text is applicable or not for R17 PDCCH monitoring adaptation. The following is proposed, CR in R1-2208606 TS 38.213 ## 10.4 Search space set group switching and skipping of PDCCH monitoring A UE can be provided - a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList for PDCCH monitoring on a serving cell, - a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell, - a set of durations by PDCCHSkippingDurationList for Type3-PDCCH CSS sets or USS sets for PDCCH monitoring on an active DL BWP of a serving cell. If the UE is not provided any of the above informationsearchSpaceGroupIdList for a search space set, the following procedures are not applicable for PDCCH monitoring according to the search space set(s). < Unchanged parts are omitted >. |
| LGE | Y | For Paragraph 1, it needs to be updated because only ‘searchSpaceGroupIdList’ is stated in the current spec. The proposals of vivo’s or ZTE’s are fine to us. For Paragraph 2, it needs the update of “when UE is provided searchSpaceGroupIdList-r16”. |
| Samsung | Y | OK to discuss and, in principle, OK with the proposals |
| Qualcomm | Y | We are fine with the updates for Paragraphs 1 and 2. |
| Ericsson1 | Y, with updates | Apart from these two paragraphs, there was another change proposed in our contribution in 8955 (shown below, adding suffix ‘-r16’) that should also be discussed. |
| MediaTek | Y | Correction will be needed for both Paragraph 1 and Paragraph 2; otherwise, UE behavior will not be correct when there is mixed Rel-16 and Rel-17 configurations to the UE. |
| CATT | Y | We prefer vivo’s text proposal. |
| ZTE, Sanechips | Y | Both corrections are needed. Meanwhile, other corrections on the mixed paragraph proposed in our contribution are also needed, which are being discussed in section 2.4, otherwise, UE behaviors are not aligned with the intention of introducing Rel-17 PDCCH monitoring reduction. |

Question 2.3-1 (1st_round)

- Y (12 companies):

  - Paragraph 1 (12): Nordic, Nokia, Spreadtrum, HW, vivo, LGE, Samsung, QC, Ericsson, MTK, CATT, ZTE

  - Paragraph 2 (8): Nokia, Spreadtrum, LGE, Samsung, QC, Ericsson, MTK, ZTE

- Support rate for further discussion: 100%

  - Within the supporting companies, Paragraph 1: 100%, Paragraph 2: 67%

Guidance from session chair Xiaodong (CMCC):

Continue the discussion in this meeting on Paragraph 1 and Paragraph 2 from Question 2.3-1

For the TP to Paragraph 1, the proposed CRs in R1-2209191(ZTE, Sanechips), R1-2208606 (vivo), and R1-2209854 (Ericsson) are incorporated, and the following proposal is suggested for companies’ check

Proposal 2.3-1 (2nd_round)

- To avoid precluding Rel-17 PDCCH monitoring adaptation procedures when Rel-16 configuration is not provided, the following TP to clause 10.4 of TS 38.213, based on R1-2208606, R1-2209191 and R1-2209854, is endorsed.

10.4 Search space set group switching and skipping of PDCCH monitoring A UE can be provided - a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList for PDCCH monitoring on a serving cell, - a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell, - a set of durations by PDCCHSkippingDurationList for Type3-PDCCH CSS set or USS set for PDCCH monitoring on an active DL BWP of a serving cell If the UE is not provided searchSpaceGroupIdList for a search space set, any of the above information, the following procedures are not applicable for PDCCH monitoring according to the search space set(s). <Unchanged parts are omitted>

Table 8 (2nd_round): Companies’ views on Proposal 2.3-1

| Company name | Company view(s) |
| --- | --- |

For the TP to Paragraph 2, the proposed CRs in R1-2209191(ZTE, Sanechips) and R1-2209855 (Ericsson) are incorporated, and the following proposal is suggested for companies’ check

Proposal 2.3-2 (2nd_round)

- To clarify the condition for the Rel-16 behaviour when UE is not provided SearchSpaceSwitchTrigger, the following TP to clause 10.4 of TS 38.213, based on R1-2209191 and R1-2209855, is endorsed.

10.4 Search space set group switching and skipping of PDCCH monitoring <Unchanged parts are omitted> If a UE is provided searchSpaceGroupIdList-r16 and is not provided SearchSpaceSwitchTrigger for a serving cell, - if the UE detects a DCI format by monitoring PDCCH according to a search space set with group index 0, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, for the serving cell - at the beginning of the first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format when μ∈0, 1, 2, 3, - at the beginning of the first slot, of a group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format when μ∈5, 6 the UE sets the timer value to the value provided by searchSpaceSwitchTimer if the UE detects a DCI format by monitoring PDCCH in any search space set <Unchanged parts are omitted>

Table 9 (2nd_round): Companies’ views on Proposal 2.3-2

| Company name | Company view(s) |
| --- | --- |

## Others

| Company | Views |
| --- | --- |
| ZTE, Sanechips | - CR in R1-2209191 - Proposal 1: The following change should be adopted. <Unchanged parts are omitted> When a UE receives - a first PDCCH in a first slot that provides a DCI format with a PDCCH monitoring adaptation field having a first value indicating skipping PDCCH monitoring, or indicating start of PDCCH monitoring according to a search space sets with a first group index and stop of PDCCH monitoring according to search space sets with a second group index, for an active DL BWP and - a second PDCCH that provides a DCI format with a PDCCH monitoring adaptation field having a second value indicating skipping PDCCH monitoring, or indicating no skipping in PDCCH monitoring, or indicating start of PDCCH monitoring according to search space sets with a first group index and stop of PDCCH monitoring according to search space sets with a second group index different than the first group index, for the active DL BWP where the second PDCCH is received - in the first slot if the first value indicates skipping PDCCH monitoring - before a slot that is at least Pswitch symbols after the first slot if the first value indicates start of PDCCH monitoring according to search space sets with a first group index the UE does not expect the second value to be different than the first value. <Unchanged parts are omitted>. - Proposal 4: The following change should be adopted. TS 38.213 10.4 Search space set group switching and skipping of PDCCH monitoring <Unchanged parts are omitted> A UE can be provided, by searchSpaceSwitchTimer, a timer value for a serving cell that the UE is provided searchSpaceGroupIdList-r16 or, if provided, for a set of serving cells provided by cellGroupsForSwitchList. The UE decrements the timer value by one after each slot based on a reference SCS configuration that is the smallest SCS configuration μ among all configured DL BWPs in the serving cell, or in the set of serving cells. The UE maintains the reference SCS configuration during the timer decrement procedure. <Unchanged parts are omitted> A UE determines a slot and a symbol in the slot to start or stop PDCCH monitoring according to search space sets for a serving cell that the UE is provided searchSpaceGroupIdList-r16 or, if cellGroupsForSwitchList is provided, for a set of serving cells, based on the smallest SCS configuration μ among all configured DL BWPs in the serving cell or in the set of serving cells and, if any, in the serving cell where the UE receives a PDCCH and detects a corresponding DCI format 2_0 triggering the start or stop of PDCCH monitoring according to search space sets.. Agreement RAN1#106bis-e - If the UE monitors PDCCH according to SSSG#1 and the timer expires, the UE starts monitoring PDCCH according to Beh 2. - If the UE monitors PDCCH according to SSSG#2 and the timer expires, - Alt 1: the UE monitoring PDCCH according to Beh 2 - Other alternatives are not precluded -Timer can be optionally configured.. - Proposal 5: The following change should be adopted. TS 38.213 10.4 Search space set group switching and skipping of PDCCH monitoring <Unchanged parts are omitted> A UE can be provided group indexes for a Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell and, if the UE is not provided PDCCHSkippingDurationList for the active DL BWP of the serving cell, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmissions and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 1 bit or of 2 bits for the serving cell. If the field has 1 bit and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell - a '0' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with other group indexes, if any - a '1' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with other group indexes, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided If the field has 2 bits and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell - a '00' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with other group indexes, if any - a '01' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with other group indexes, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided - a '10' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 2 and stop of PDCCH monitoring according to search space sets with other group indexes, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided - a '11' value is reserved A UE can be provided a set of durations by PDCCHSkippingDurationList and group indexes for a Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell and, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmissions, and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 2 bits. If the set of durations includes one value and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell - a '00' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with group index 1, if any - a '01' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with group index 0, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided - a '10' value for the bits indicates skipping PDCCH monitoring for a duration provided by the value in the set of durations - a '11' value is reserved If the set of durations includes two values and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on active DL BWP of the the serving cell - a '00' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with group index 1, if any - a '01' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with group index 0, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided - a '10' value for the bits indicates skipping PDCCH monitoring for a duration provided by the first value in the set of durations - a '11' value for the bits indicates skipping PDCCH monitoring for a duration provided by the second value in the set of durations. |
| Nokia, Nokia Shanghai Bell | Proposal 4: As the work item determines the scope to be DRX Active Time, it should be confirmed that Rel-17 PDCCH monitoring adaptation is configured only together with C-DRX. |

Moderator would like to thank ZTE and Nokia for additional proposals. Accordingly, the following questions are for companies to check the above proposals and provide their views:

Question 2.4-1 (1st_round): Whether to discuss and decide Proposals 1, 4 and 5 in R1-2209191 (as quoted in the above table)?

Table 10 (1st_round): Companies’ answers to Question 2.4-1

| Company name | Support to discuss this issue (Y/N)? | If you support to discuss the issue, what are your views for Proposals 1, 4, and 5 in R1-2209191? |
| --- | --- | --- |
| Xiaomi | Y | For Proposals 1, we agree with it For Proposals 4, maybe it should be discussed together with section 2.3(Clarification for Mixed Rel-16 and Rel-17 Configurations)? For Proposals 5, OK with the update, but want to confirm, if searchSpaceSwitchTimer-r17 is not provided, what will be the case? Do we need to define a default timer value? |
| Nordic | N |  |
| Nokia1 | Y | On Proposal 1 and 4, fine in principle. Also on Proposal 5, seems to be aligned with other similar cases in 38.213. |
| OPPO | N |  |
| Huawei, HiSilicon |  | We are fine to discuss proposal 1 and 4. Others seems not essential change. Additionally, in CR in R1-2209816, we update the name of higher layer parameters for pdcch-SkippingDurationList to align with higher layer specification, as following. It should be discussed together. 10.4 Search space set group switching and skipping of PDCCH monitoring *** Unchanged text is omitted *** A UE can be provided a set of durations by pdcch-SkippingDurationList for PDCCH monitoring on an active DL BWP of a serving cell and, if the UE is not provided searchSpaceGroupIdList-r17 on the active DL BWP of the serving cell, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmission, and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 1 bit or of 2 bits. *** Unchanged text is omitted *** A UE can be provided group indexes for a Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell and, if the UE is not provided pdcch-SkippingDurationList for the active DL BWP of the serving cell, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmissions and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 1 bit or of 2 bits for the serving cell. *** Unchanged text is omitted *** A UE can be provided a set of durations by pdcch-SkippingDurationList and group indexes for a Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell and, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmissions, and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 2 bits. *** Unchanged text is omitted ***. |
| Vivo | Partial Y | Proposal 4, generally the RRC can be fixed across the whole section if needed. Proposal 5, in principle it is right. |
| LGE | Y | For Proposal 1, we don’t think the change is necessary. UE receiving DCI format with a PDCCH monitoring adaptation field does not expect to receive another DCI format with a PDCCH monitoring adaptation indication field indicating change to another SSSG or skipping for the same active BWP, thus there is no need to include ‘indicating no skipping in PDCCH monitoring’. We are oaky with the Proposal 4 and 5. |
| Samsung | N | Proposal 1: “Indicating skipping” includes that the indicated skipping is “zero/no skipping” Proposal 4: It would be improper to add ‘-r16’ – the suffix is not added in the first release where the parameter is introduced, only in later releases for the same parameter. Proposal 5: The ‘if provided’ is always implied for optional configurations (visible in 38.331) – OK to add but also no need to add. |
| Ericsson1 | OK to discuss P4 | P4 : We do not think addition of only ‘-r16’ is enough - that paragraph (at least some parts) also applies for rel-17. |
| MediaTek | Y | Support to discuss the editorial CRs, including the suggested alignment on pdcch-SkippingDurationList in Huawei’s response |
| CATT | Y | We are OK for discussion of Proposals 4 and 5. Proposal 1 is not essential correction. |
| ZTE, Sanechips | Y | We think Proposal 1, Proposal 4, and Proposal 5 need discussion. For proposal 1, we think it is clear in the current spec that indication of “skipping” and “no skipping ” are different. And if UE is firstly indicated with PDCCH skipping, it doesn’t expect to receive a DCI indicating PDCCH monitoring change, which includes “no skipping”. However, the current specification of the corresponding paragraph only considers “a DCI format with a PDCCH monitoring adaptation field having a second value indicating skipping PDCCH monitoring” . The case that DCI indicating “no skipping ” is missed. For proposal 2, we agree that the suffix “-rel16” is not added in Rel-16 RRC parameters. However, since the Rel-16 and Rel-17 PDCCH monitoring adaptation are mixed in the same sub-clause. Without the “-rel16”, UE’s behavior of PDCCH monitoring will be wrongly interpreted. We are okay to discuss proposal 4 with the proposals in section 2.3. For proposal 5, without “if provided”, it seems that earchSpaceSwitchTimer-r17 has to be always configured, which is not aligned with RRC parameter defined in TS38.331. |

Question 2.4-1 (1st_round)

      - Y (9 companies):

        - Proposal 1 (5): Xiaomi, Nokia, HW, MTK, ZTE

        - Proposal 4 (7): Nokia, HW, LGE, Ericsson, MTK, CATT, ZTE

        - Proposal 5 (6): Xiaomi, vivo, LGE, MTK, CATT, ZTE

      - N (3 companies): Nordic, OPPO, Samsung

      - Support rate for further discussion: 75%

        - Within the supporting companies, Proposal 1: 56%, Proposal 2: 78%, Proposal 3: 67%

Guidance from session chair Xiaodong (CMCC):

Continue the discussion in this meeting on Proposal 4 and Proposal 5 from Question 2.4-1

Since Proposal 4 is in fact an alignment CR, which is different from Proposal 5, the following separated proposals for therefore suggested:

Proposal 2.4-1 (2nd_round)

For alignment TS 38.213 CR, the following corrections provided in R1-2209191 are endorsed for the editorial corrections:

- ‘searchSpaceGroupIdList’ is revised to ‘searchSpaceGroupIdList-r16’

- Note: ‘searchSpaceGroupIdList-r17’ is not changed

Table 11 (2nd_round): Companies’ views on Proposal 2.4-1

| Company name | Company view(s) |
| --- | --- |

Proposal 2.4-2 (2nd_round)

To match RAN1#106bis-e agreements, where there states that SSSG timer can be optionally configured, the following TP to clause 10.4 of TS 38.213, as provided in R1-2209191, is endorsed.

10.4 Search space set group switching and skipping of PDCCH monitoring <Unchanged parts are omitted> A UE can be provided group indexes for a Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell and, if the UE is not provided PDCCHSkippingDurationList for the active DL BWP of the serving cell, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmissions and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 1 bit or of 2 bits for the serving cell. If the field has 1 bit and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell - a '0' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with other group indexes, if any - a '1' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with other group indexes, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided If the field has 2 bits and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell - a '00' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with other group indexes, if any - a '01' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with other group indexes, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided - a '10' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 2 and stop of PDCCH monitoring according to search space sets with other group indexes, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided - a '11' value is reserved A UE can be provided a set of durations by PDCCHSkippingDurationList and group indexes for a Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell and, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmissions, and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 2 bits. If the set of durations includes one value and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell - a '00' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with group index 1, if any - a '01' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with group index 0, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided - a '10' value for the bits indicates skipping PDCCH monitoring for a duration provided by the value in the set of durations - a '11' value is reserved If the set of durations includes two values and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on active DL BWP of the the serving cell - a '00' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with group index 1, if any - a '01' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with group index 0, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided - a '10' value for the bits indicates skipping PDCCH monitoring for a duration provided by the first value in the set of durations - a '11' value for the bits indicates skipping PDCCH monitoring for a duration provided by the second value in the set of durations #### <Unchanged parts are omitted>

Table 12  (2nd_round): Companies’ views on Proposal 2.4-2

| Company name | Company view(s) |
| --- | --- |

Question 2.4-2 (1st_round): Whether to discuss and decide clarification on that Rel-17 PDCCH monitoring adaptation is configured only together with C-DRX?

Table 13 (1st_round): Companies’ answers to Question 2.4-2

| Company name | Support to discuss this issue (Y/N)? | If you support to discuss the issue, what are your views on the joint configuration? |
| --- | --- | --- |
| Xiaomi | N | We don’t think it is good to artificially restrict the use case of PDCCH monitoring adaptation to only together with C-DRX without any benefit, since technically, there is no any issue to apply PDCCH monitoring adaptation without C-DRX. We understand the reason is, in the WI scope restricts to the case when C-DRX is configured, so we should not develop solutions outside of the scope. but in fact, we have done similar things before. For example, for NR-U, the WI is certainly restricted to unlicensed spectrum, but the Type 3 HARQ-ACK codebook mechanism developed in NR-U can also be applied to licensed spectrum. |
| Nordic | N | We do not see need to restrict, but can accept majority view. |
| Nokia1 | Y | (Proponent; to align with the WID) |
| OPPO | N |  |
| Huawei, HiSilicon | N | We have never agreed that PDCCH monitoring adaptation is only used when C-DRX is configured. We should not have this kind of restriction of the feature. |
| vivo | N | No need to have this artificially restriction. RAN2 has discussed this issue last meeting and does not agree to restrict it. Therefore, the discussion on it can be closed. |
| LGE |  | We don’t think PDCCH monitoring adaptation is not limited to be configured only together with C-DRX. Although this AI started from DCI-based power saving adaptation during DRX Active Time, PDCCH monitoring adaptation for a UE not configured with C-DRX also be very helpful for power saving. Power saving can be introduced by adapting monitoring adaptation with C-DRX into monitoring adaptation without C-DRX. |
| Samsung | N | No need to discuss, there is nothing to correct, and not only there is no need for such artificial restriction but it will be detrimental. |
| Qualcomm | N | We agree with Nokia’s view: when we discuss on Rel-17 UE power saving WI scope 3 years ago, it was the common view. However, at this stage, we don’t think limit the scope because it can also impact Rel-18 XR power saving discussion. |
| MediaTek | N | While the study of the scheme started with C-DRX configuration in order to ensure meaningful power saving gain beyond C-DRX, the mechanism of PDCCH monitoring adaptation can actually works without C-DRX. In this regard, it is not technically true to put C-DRX restriction on Rel-17 PDCCH monitoring adaptation schemes. |
| CATT | N | There is no agreement in limiting the PDCCH monitoring adaptation only when DRX is configured The work item with the objective of investigating PDCCH monitoring adaptation when DRX is configured does not set the limitation of PDCCH monitoring adaptation mechanism could only apply when DRX is configured. |
| ZTE, Sanechips | N | We don’t think this coupling is needed. |

Question 2.4-2 (1st_round)

      - Y (1 company): Nokia

      - N (11 companies): Xiaomi, Nordic, OPPO, HW, vivo, LGE, Samsung, QC, MTK, CATT, ZTE

      - Support rate for further discussion: 8%

Guidance from session chair Xiaodong (CMCC):

No further discussion in this meeting

# Maintenance on Idle/Inactive UE Power Saving Enhancements

In this section, companies’ inputs related to Idle/Inactive UE Power Saving Enhancements are first summarized, and potential issues are then identified for companies to check and provide views.

Table 14: Companies’ views related to Idle/Inactive UE power saving enhancements from [3] - [25]

| Company | Views |
| --- | --- |
| Nokia, Nokia Shanghai Bell | CR in R1-2209375 #### TS 38.211 #### < Unchanged parts are omitted > #### 7.3.1.5 Mapping to virtual resource blocks The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS according to clause 7.4.1.5 if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource; - unless the non-zero power CSI-RS are configured by TRS-ResorceSet IE, in which case the resource elements are considered available for PDSCH - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. The mapping to resource elements (k',l)p,μ allocated for PDSCH according to [6, TS 38.214] and not reserved for other purposes shall be in increasing order of first the index k' over the assigned virtual resource blocks, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index l. < Unchanged parts are omitted >. |
| Huawei, HiSilicon | CR in R1-2208478 - Updates on the parameter names to align with TS 38.331： - Update the parameter name of “numberOfresources” to “nrofResources”; - Update “scramblingID” to “ScramblingID”; - Update “TRS-ResourceSetConfig” to “trs-ResourceSetConfig”. #### TS 38.214 #### 5.1.6.1.1 CSI-RS for tracking #### < Unchanged parts are omitted > A UE in RRC_IDLE or RRC_INACTIVE can receive a higher layer configuration of TRS occasions via a trs-ResourceSetConfig. - For frequency range 1, the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a TRS-ResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. If no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon, then the UE may be configured with one or more NZP CSI-RS set(s), where a TRS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot. - For frequency range 2 the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a TRS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot or by a TRS-ResourceSet of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. Each NZP CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter TRS-ResourceSet with the following restrictions for a UE in RRC_IDLE or RRC_INACTIVE: - the number of periodic NZP CSI-RS resources configured by a TRS-ResourceSet is given by nrofResources - the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), is one of - , , or for frequency range 1 and frequency range 2, - , , , , , or for frequency range 2. - where the first symbol location in a slot is indicated by firstOFDMSymbolInTimeDomain in the TRS-ResourceSet and the second symbol location in a slot is firstOFDMSymbolInTimeDomain + 4 - a single port CSI-RS resource with density given by Table 7.4.1.5.3-1 from [4, TS 38.211]. - the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter nrofRBs, startingRB and frequencyDomainAllocation in a TRS-ResourceSet and applies to all resources in a TRS-ResourceSet. Bandwidth, nrofRBs, and the initial CRB index, startingRB, of the NZP CSI-RS resource configured by TRS-ResourceSet are not restricted by initial DL BWP. - UE is not required to receive TRS occasions outside the initial DL BWP. - the periodicity for periodic NZP CSI-RS resources, is given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet, is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211], applies to all resources in a TRS-ResourceSet. The slot offset given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet provides the location of the first slot containing the periodic NZP CSI-RS resources configured by by a TRS-ResourceSet. - the UE does not expect the TRS-ResourceSet to be configured with the periodicity of slots if the bandwidth of NZP CSI-RS resource is larger than 52 resource blocks. - the UE may assume the sub-carrier spacing of the NZP CSI-RS resources configured by TRS-ResourceSet to be same as the sub-carrier spacing of the initial DL BWP. - powerControlOffsetSS given by a TRS-ResourceSet applies to all resources in a TRS-ResourceSet. - the QCL information for periodic NZP CSI-RS resources, is given by the higher layer parameter ssb-Index configured by a TRS-ResourceSet, is a SS/PBCH block, applies to all resources in a TRS-ResourceSet. - One or more ScramblingIDs where if a single ScramblingID is configured, it applies to all NZP-CSI-RS resources in the resource set, otherwise, each NZP-CSI-RS resource is provided with a ScramblingID. - the UE may assume the following quasi co-location type(s): - 'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block. For each TRS-ResourceSet the index of the associated bit in TRS availability indication field [5, TS 38.212], is given by the higher layer parameter indBitID. < Unchanged parts are omitted >. CR in R1-2208479 - Correct the description of availability indication of TRS resources. - Update “iS” to “i_s”, which is the correct one used in TS 38.304. ## TS 38.213 *** Unchanged text is omitted *** ## 10.4A PDCCH monitoring for early indication of paging A UE can be provided the following for detection of a DCI format 2_7 in RRC_IDLE state or in RRC_INACTIVE state [12, TS 38.331] - a search space set, by pei-SearchSpace, to monitor PDCCH for detection of DCI format 2_7 according to a Type2A-PDCCH CSS set as described in clause 10.1 - a number of frames, by pei-FrameOffset, from the start of a frame to the start of a first paging frame of paging frames associated with a number of PDCCH monitoring occasions for DCI format 2_7 [17, TS 38.304] - a number of symbols, by firstPDCCH-MonitoringOccasionOfPEI-O, from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 - a size, by payloadSizeDCI-2-7 - a number of subgroups per paging occasion, NSGPO, by subgroupsNumPerPO - a number of paging occasions associated with the number of PDCCH monitoring occasions for DCI format 2_7, NPOPEI, by po-NumPerPEI A paging indication field of DCI format 2_7 includes NPOPEI segments of K bits, where K=NSGPO. For a subgroup index iSG, 0≤iSG<K, a UE determines a value for the iPO⋅K+iSG bit in the paging indication field, where iPO=UE_IDmodN⋅NS+i_siSmodNPOPEI is a paging occasion index, and UE_ID, N, NS, iSG, and i_siS are defined in [17, TS 38.304]. When the value is '1', the UE monitors a paging occasion determined according to [17, TS 38.304]; otherwise, the UE is not required to monitor the paging occasion. If NPOPEI<NS, the number of symbols from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 that is associated with paging occasion index iPO is the i_siSNPOPEI+1-th value from the NSNPOPEI values provided by firstPDCCH-MonitoringOccasionOfPEI-O. ## 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7, if peiSearchSpace is provided, and a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. A value of '1' for a bit of the bitmap indicates presence of a TRS belonging to the associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the presence of a TRS belonging to the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap indicates no change to a current assumption for the availability, or unavailability if the UE is not in a validity duration, of the TRS belonging to the associated TRS resource sets. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames.. |
| Ericsson | CR in R1-2209857 - Clarify the RRC parameter names and description to align with 38.331. #### TS 38.214 #### 5.1.6.1.1 CSI-RS for tracking #### < Unchanged parts are omitted > A UE in RRC_IDLE or RRC_INACTIVE can receive a higher layer configuration of TRS occasions via a trs-ResourceSetConfig. - For frequency range 1, the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a TRS-ResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. If no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon, then the UE may be configured with one or more TRS resource set(s), where a TRS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot. - For frequency range 2 the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a TRS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot or by a TRS-ResourceSet of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. Each NZP CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter TRS-ResourceSet with the following restrictions for a UE in RRC_IDLE or RRC_INACTIVE: - the number of periodic NZP CSI-RS resources configured by a TRS-ResourceSet is given by nrofResources - the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), is one of - , , or for frequency range 1 and frequency range 2, - , , , , , or for frequency range 2. - where the first symbol location in a slot is indicated by firstOFDMSymbolInTimeDomain in the TRS-ResourceSet and the second symbol location in a slot is firstOFDMSymbolInTimeDomain + 4 - a single port CSI-RS resource with density given by Table 7.4.1.5.3-1 from [4, TS 38.211]. - the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter nrofRBs, startingRB and frequencyDomainAllocation in a TRS-ResourceSet and applies to all resources in a TRS-ResourceSet. Bandwidth, nrofRBs, and the initial CRB index, startingRB, of the NZP CSI-RS resource configured by TRS-ResourceSet are not restricted by initial DL BWP. - UE is not required to receive TRS occasions outside the initial DL BWP. - the periodicity for periodic NZP CSI-RS resources, is given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet, is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211], applies to all resources in a TRS-ResourceSet. The slot offset given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet provides the location of the first slot containing the periodic NZP CSI-RS resources configured by by a TRS-ResourceSet. - the UE does not expect the TRS-ResourceSet to be configured with the periodicity of slots if the bandwidth of NZP CSI-RS resource is larger than 52 resource blocks. - the UE may assume the sub-carrier spacing of the NZP CSI-RS resources configured by TRS-ResourceSet to be same as the sub-carrier spacing of the initial DL BWP. - powerControlOffsetSS given by a TRS-ResourceSet applies to all resources in a TRS-ResourceSet. - the QCL information for periodic NZP CSI-RS resources, is given by the higher layer parameter ssb-Index configured by a TRS-ResourceSet, is a SS/PBCH block, applies to all resources in a TRS-ResourceSet. - One or more scrambling IDs according to scramblingID-Info where if a single scramblingIDforCommon is configured, it applies to all NZP-CSI-RS resources in the resource set, otherwise, each NZP-CSI-RS resource is provided with a scrambling ID according to scramblingIDperResourceListWith2 or scramblingIDperResourceListWith4. - the UE may assume the following quasi co-location type(s): - 'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block. For each TRS-ResourceSet the index of the associated bit in TRS availability indication field [5, TS 38.212], is given by the higher layer parameter indBitID. < Unchanged parts are omitted >. |

Moderator would like to thank companies’ careful check. From the above table, the following 4 questions (3-1 to 3-4) are also identified for companies’ further check and answers:

Question 3-1 (1st_round): Whether to discuss and decide revising the previous correction to clause 7.3.1.5 of TS 38.211, as quoted below, because of the ambiguity as identified in R1-2209373?

#### < Unchanged parts are omitted > #### 7.3.1.5 Mapping to virtual resource blocks The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with to resource elements in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and not configured by TRS-ResourceSet IE, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource; - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. The mapping to resource elements allocated for PDSCH according to [6, TS 38.214] and not reserved for other purposes shall be in increasing order of first the index over the assigned virtual resource blocks, where is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index . #### < Unchanged parts are omitted >

Table 15 (1st_round): Companies’ answers to Question 3-1

| Company name | Support to discuss this issue (Y/N)? | If you support to discuss the issue, what will be your view in the proposed CR in R1-2209375? |
| --- | --- | --- |
| Xiaomi | N, partially | We think Nokia’s version is clear, but we also think the previous version is clear too, no ambiguity at least from our side. Open to go with majority. |
| Nordic | Y | Could be coded the same way as e.g. mobility CSI-RS i.e. “except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter TRS-ResourceSet” |
| Sharp | N | We think Nokia’s wording may be ambiguous for the case of the same TRS resources provided by SIB and dedicated configuration. In the case, the REs occupied by TRS should be considered not available even though the non-zero power CSI-RS are configured by TRS-ResorceSet IE. |
| Nokia1 | Y | (Proponent, but) There is some risk of ambiguity, thus some clarification would be good. |
| OPPO | Y | Agreed to discuss it to remove ambiguity. |
| Spreadtrum | Y | Open to discuss |
| Huawei, HiSilicon | N | We don’t think there is ambiguity. Current spec is clear enough. In current spec, ‘which is according to clause 7.4.1.5 and not configured by TRS-ResourceSet IE’ is a whole sentence separated with other sentences with commas. So it is clear the first understanding in Nokia’s contribution is the common understanding. |
| vivo |  | Similar view as Xiaomi. |
| LGE | Y | Fine to discuss the issue. |
| Samsung | N | We do not identify any ambiguity – the specification text is clear. |
| Ericsson1 | Y | OK to discuss. |
| Qualcomm | N, partially | We agreed with Nokia that the spec text looks ambiguous. But Nokia’s proposal may not be completely accurate either. If a RE is in a non-zero power CSI-RS that is configured by TRS-ResorceSet IE, it does not mean the RE is available for PDSCH. It is possible that another non-zero power CSI-RS that is NOT configured by TRS-ResorceSet IE contains the same RE. Then the RE is still not available to the UE’s PDSCH. A minor update to current wording like the follows may work: - not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and if the non-zero-power CSI-RS is not configured by TRS-ResourceSet IE, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource; |
| CATT | N | We don’t see any ambiguity of current spec. |
| ZTE, Sanechips | N | We think the current specification is clear, no ambiguity is observed. |

Question 3-1 (1st_round)

- Y (7 companies): Nordic, Nokia, OPPO, Spreadtrum, LGE, Ericsson, QC (CR with revision)

- N (7 companies): Xiaomi, Sharp, HW, vivo, Samsung, CATT, ZTE

- Support rate for further discussion: 50%

Guidance from session chair Xiaodong (CMCC):

No further discussion in this meeting

Question 3-2 (1st_round): Whether to discuss and decide revising clause 10.4B of TS 38.213 for the following identified issues from R1-2208479?

- Issue 1: Based on RAN1#105-e agreements made for TRS/CSI-RS availability, the L1 signalling is for the availability indication of the RS on the configured RS occasions. However, current specification describes as the configured resource is presence, which is different from the agreement. The TRS resource is configured by SIB signalling and is always presence as a configured resource set. The availability should refer to whether an instantaneous TRS/CSI-RS is presence or not on the configured resource/occasion.

- Issue 2: The description of value ‘0’ of availability indication field in current specification only specifies the case where UE assumes the TRS on the associated TRS resource is available. However, there is not any description regarding how UE obtain the “current assumption of unavailability”.

- For potential solutions to the above issues, companies can check the CR in R1-2208479 (also quoted in the above table)

Table 16 (1st_round): Companies’ answers to Question 3-2

| Company name | Support to discuss this issue (Y/N)? | If you support to discuss the issue, what will be your views on issue 1 and issue 2? |
| --- | --- | --- |
| Xiaomi | Y | OK with the proposed CR. |
| Nordic | N |  |
| Sharp | Y | We are OK with proposed CR |
| Nokia1 | Y | Issue 1: As described above, there indeed it would better ensure correct interpretation of the text. Issue 2: Also fine. |
| Huawei, HiSilicon | Y | Support the proposed CR |
| vivo | Y |  |
| LGE | Y | Issue 1: Fine with discussion. Meanwhile, if I understand correctly, a bit of the bitmap can be associated with multiple TRSs. So I would like to ask whether “of a TRS belonging to” is correct wording, or is should be modified to “of TRSs belonging to”. Issue 2: Current spec seems fine for us, but open to discuss this issue. |
| Samsung | Y | OK in principle for alignment CR – the “if the UE is not in a validity duration” is not needed (clear from the previous paragraph in 10.4B) |
| Ericsson1 | Y, partially | First change: OK. Second change: Wording “if UE is not in a validity duration” is not clear. |
| MediaTek | Y | Fine to discuss both Issue 1 and Issue 2. |
| Qualcomm | Y, partially | Issue 1: Y Issue 2: N, the current spec is clear. If the bit is 0, the TRS is still available if it was available and the TRS is still unavailable if it was unavailable. |
| CATT | N | We don’t see the wording “of a TRS belonging to” making it clear. The proposed change does not make it clear. |
| ZTE, Sanechips | Y | Issue 1 okay to discuss. Issue 2: we don’t think the change “if UE is not in a validity duration” is clear. |

Question 3-2 (1st_round)

- Y (11 companies):

  - Issue 1 (11): Xiaomi, Sharp, Nokia, HW, vivo, LGE, Samsung, Ericsson, MTK, QC, ZTE

  - Issue 2 (7): Xiaomi, Sharp, Nokia, HW, vivo, LGE, MTK

- N (2 companies): Nordic, CATT

- Support rate for further discussion: 85%

  - Within the supporting companies, Issue 1: 100%, Issue 2: 64%

Guidance from session chair Xiaodong (CMCC):

Continue the discussion in this meeting on Issue 1 and Issue 2 from Question 3-2. For issue 2, it looks a bit controversial, the reason that some companies say no is not fine with the change itself in some sense rather than the change request. it mayb worth a shot

Based on the guidance, moderator suggests to consider separate proposals for Issue 1 and Issue 2, respectively.

Proposal 3.2-1 (Issue 1; 2nd_round)

To clarify that the availability indication refers to whether an instantaneous TRS/CSI-RS is presence or not on the configured resource/occasion, the following TP to clause 10.4B of TS 38.213, as provided in R1-2208479, is endorsed.

#### < Unchanged parts are omitted > A value of '1' for a bit of the bitmap indicates presence of a TRS belonging to the associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the presence of a TRS belonging to the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap indicates no change to a current assumption for the availability or unavailability of the TRS belonging to the associated TRS resource sets. < Unchanged parts are omitted >

Table 17 (2nd_round): Companies’ views on Proposal 3.2-1

| Company name | Company view(s) |
| --- | --- |

Proposal 3.2-2 (Issue 2; 2nd_round)

To clarify how UE obtain the “current assumption of unavailability”, the following TP to clause 10.4B of TS 38.213, as provided in R1-2208479, is endorsed.

#### < Unchanged parts are omitted > A value of '0' for a bit of the bitmap indicates no change to a current assumption for the availability, or unavailability if the UE is not in a validity duration, of associated TRS resource sets. < Unchanged parts are omitted >

Table 18 (2nd_round): Companies’ views on Proposal 3.2-2

| Company name | Company view(s) |
| --- | --- |

Question 3-3 (1st_round): Whether to discuss and decide revising clause 5.1.6.1.1	(CSI-RS for tracking) of TS 38.214 for alignment with the scrambling ID description in TS 38.331 (quoted below), as suggested in R1-2209857?

scramblingID-Info One or more scrambling IDs are configured for a TRS resource set. If a common scrambling ID is configured, it applies to all the TRS resources within the TRS resource set. Otherwise, each TRS resource within the TRS resource set is provided with a scrambling ID. If the number of TRS resources for the TRS resource set is 2, scramblingIDperResourceListWith2-r17 is configured, while scramblingIDperResourceListWith4-r17 is configured for the case that the number of TRS resources for the TRS resource set is 4.

Table 19 (1st_round): Companies’ answers to Question 3-3

| Company name | Support to discuss this issue (Y/N)? | If you support to discuss the issue, what will be your views on the suggested CR in R1-2209857? |
| --- | --- | --- |
| Xiaomi | Y | OK with the proposed CR. |
| Nordic | N | Can be left up to spec editor |
| Sharp |  | Can be left up to spec editor |
| Nokia1 | Y | Fine to discuss. One alternative would be to shorten the text to “One or more scrambling IDs according to scramblingID-Info” and leave the detailed description to filed description in TS38.331. |
| OPPO | Y |  |
| Huawei, HiSilicon | Y | We have different proposals on the change. We are are fine to discuss the detailed change in the next round discussion. |
| vivo |  | Can be left up to spec editor |
| LGE | Y | OK |
| Samsung | Y | OK to include in alignment CR |
| Ericsson1 | Y | Support the CR (proponent). |
| MediaTek | Y | The proposed alignment is reasonable. |
| Qualcomm | Y | The following TRS resource set(s) should be TRS-ResourceSet(s) A UE in RRC_IDLE or RRC_INACTIVE can receive a higher layer configuration of TRS occasions via a trs-ResourceSetConfig. - For frequency range 1, the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a TRS-ResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. If no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon, then the UE may be configured with one or more TRS resource set(s), where a TRS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot. |
| CATT | Y | OK with the change |
| ZTE, Sanechips | Y | OK with the change, it can be left up to spec editor |

Question 3-3 (1st_round)

- Y (11 companies): Xiaomi, Nokia, OPPO, HW (different CR), LGE, Samsung, Ericsson, MTK, QC, CATT, ZTE

- N (3 companies): Nordic, Sharp, vivo

- Support rate for further discussion: 79%

Guidance from session chair Xiaodong (CMCC):

Alignment CR, will be noted in the session notes for spec editor's reference

Based on the guidance, the following proposal is suggested:

Proposal 3.3 (2nd_round)

For alignment TS 38.214 CR, the following corrections provided in R1-2209857 are endorsed for the editorial corrections:

One or more scrambling IDs according to scramblingID-Info where if a single scramblingIDforCommon is configured, it applies to all NZP-CSI-RS resources in the resource set, otherwise, each NZP-CSI-RS resource is provided with a scrambling ID according to scramblingIDperResourceListWith2 or scramblingIDperResourceListWith4.

Table 20 (2nd_round): Companies’ views on Proposal 3-3

| Company name | Company view(s) |
| --- | --- |

Question 3-4 (1st_round): Whether to discuss and decide the following alignment corrections (Category D) for idle/inactive UE power saving enhancements?

- Editorials on clause 10.4A of TS 38.213 for alignment with TS 38.304:

  - Update “iS” to “i_s”, which is the correct one used in TS 38.304.

- Editorials on clause 5.1.6.1.1 of TS 38.214 for alignment with TS 38.331:

  - Update the parameter name of “numberOfresources” to “nrofResources”;

  - Update “TRS-ResourceSetConfig” to “trs-ResourceSetConfig”.

Table 21 (1st_round): Companies’ answers to Question 3-4

| Company name | Support to discuss this issue (Y/N)? | If you support to discuss the issue, what will be your views on the editorial corrections for TS 38.213 and TS 38.214 (as the sub-bullets under Question 3-4)? |
| --- | --- | --- |
| Nordic | N | Can be left up to spec editor. |
| Sharp | Y | We are OK with proposed CR |
| Nokia1 | Y | Seem editorial. |
| OPPO | Y |  |
| Spreadtrum | Y | Both CR and Editor revision are fine for us |
| Huawei, HiSilicon | Y | We support the proposed CR. |
| LGE | Y | OK |
| Samsung | Y | OK to include in alignment CR |
| Ericsson1 | Y | OK with the proposed corrections. |
| MediaTek | Y | Reasonable alignments with RAN2 specifications |
| Qualcomm | Y | Fine with the editorial changes |
| CATT | Y | OK with the changes |
| ZTE, Sanechips | Y | OK with the changes, it can be left up to spec editor |

Question 3-4 (1st_round)

- Y (12 companies): Sharp, Nokia, OPPO, Spreadtrum, HW, LGE, Samsung, Ericsson, MTK, QC, CATT, ZTE

- N (1 company): Nordic

- Support rate for further discussion: 92%

Guidance from session chair Xiaodong (CMCC):

Alignment CR, will be noted in the session notes for spec editor's reference

Based on the guidance, the following proposals are suggested:

Proposal 3.4-1 (2nd_round)

For alignment TS 38.213 CR, the following corrections provided in R1-2208479 are endorsed for the editorial corrections:

  - Update “iS” to “i_s”, which is the correct one used in TS 38.304.

Proposal 3.4-2 (2nd_round)

For alignment TS 38.214 CR, the following corrections, as provided in R1-2208478 and R1-2209857, are endorsed for the editorial corrections:

  - Update “TRS-ResourceSetConfig” to “trs-ResourceSetConfig”.

  - Update the parameter name of “numberOfresources” to “nrofResources”;

Table 22 (2nd_round): Companies’ views on Proposal 3.4-1 and Proposal 3.4-2

| Company name | Company view(s) |
| --- | --- |

# Remaining Critical Issue(s) and The Supporting Reason(s)

This section is for companies to provide inputs in any remaining critical issue(s) and the supporting reason(s). The intention is to assist companies’ judgment for maintenance of critical and essential issues in next RAN1 meeting.

Table 23: Companies’ views on the remaining critical issue(s) and the supporting reason(s), if available

| Company name | Company view(s) |
| --- | --- |

# Summary

Proposal 3.4-1

For alignment TS 38.213 CR, the following corrections provided in R1-2208479 are endorsed for the editorial corrections:

  - Update “iS” to “i_s”, which is the correct one used in TS 38.304.

Proposal 3.4-2

For alignment TS 38.214 CR, the following corrections, as provided in R1-2208478 and R1-2209857, are endorsed for the editorial corrections:

  - Update “TRS-ResourceSetConfig” to “trs-ResourceSetConfig”.

  - Update the parameter name of “numberOfresources” to “nrofResources”;

Proposal 3.3

For alignment TS 38.214 CR, the following corrections provided in R1-2209857 are endorsed for the editorial corrections:

One or more scrambling IDs according to scramblingID-Info scramblingIDs where if a single scramblingIDforCommon is configured, it applies to all NZP-CSI-RS resources in the resource set, otherwise, each NZP-CSI-RS resource is provided with a scrambling ID according to scramblingIDperResourceListWith2 or scramblingIDperResourceListWith4 scramblingID.

Proposal 2.4-1

For alignment TS 38.213 CR, the following corrections provided in R1-2209191 are endorsed for the editorial corrections:

- ‘searchSpaceGroupIdList’ is revised to ‘searchSpaceGroupIdList-r16’

- Note: ‘searchSpaceGroupIdList-r17’ is not changed

# Reference

Report of 3GPP TSG RAN WG1 #110 v1.0.0, MCC Support; online available @ https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_110/Report/Final_Minutes_report_RAN1%23110_v100.zip

R1-2207970, “Summary#2 of maintenance on idle/inactive UE power saving enhancements”,	Moderator (MediaTek)

R1-2208838, “Remaining issues for DCI-based power saving adaptation”, OPPO

R1-2209190, “Discussion on remaining issues of PDCCH monitoring adaptation”, ZTE, Sanechips

R1-2209251, “Remaining issues of PDCCH monitoring adaptation for power saving”, xiaomi

R1-2209373, “Open issues on Idle/Inactive mode power saving”, Nokia, Nokia Shanghai Bell

R1-2209374, “Open issues on PDCCH monitoring adaptation for UE power saving”, Nokia, Nokia Shanghai Bell

R1-2209815, “Discussion on UE behavior when PDCCH skipping received after a SR transmission”, Huawei, HiSilicon

R1-2209853, “Discussion on corrections for PDCCH monitoring adaptation”, Ericsson

R1-2208478	Correction on TRS occasions for IDLE mode UE	Huawei, HiSilicon

R1-2208479	Correction on power saving for IDLE mode UE	Huawei, HiSilicon

R1-2208606	Correction on search space set group switching and PDCCH skipping	vivo

R1-2208607	Correction on PDCCH monitoring adaptation and BWP switching	vivo

R1-2208839	Draft CR for or DCI-based power saving adaptation	OPPO

R1-2209191	Correction on PDCCH monitoring adaptation	ZTE, Sanechips

R1-2209252	UE hehavior for PDCCH monitoring adaptation and BWP switching indicated in the same DCI	xiaomi

R1-2209375	Clarification of virtual resource block mapping in respect of TRS-ResourceSet IE	Nokia, Nokia Shanghai Bell

R1-2209376	Alignment of PDCCH skipping behaviour with SR	Nokia, Nokia Shanghai Bell

R1-2209377	Completion of PDCCH monitoring adaptation behaviour and BWP switching	Nokia, Nokia Shanghai Bell

R1-2209816	Correction on UE behavior when PDCCH skipping received after a SR transmission	Huawei, HiSilicon

R1-2209817	Correction on PDCCH monitoring adaptation and BWP switching	Huawei, HiSilicon

R1-2209854	Correction for 38.213 on PDCCH skipping based monitoring adaptation	Ericsson

R1-2209855	Correction for 38.213 on mixed Rel-16 and 17 SSSGS combinations	Ericsson

R1-2209856	Correction for 38.213 on PDCCH monitoring adaptation field in a DCI indicating BWP change	Ericsson

R1-2209857	Corrections for 38.214 on TRS occasions	Ericsson