3GPP TSG RAN WG1 #110bis-e							    R1-2210286

Electronics Meeting, 10 – 19 October, 2022

Source:	Moderator (OPPO)

Title:	FL summary #2 for AI 8.11: R17 eSL power saving RA maintenance

Agenda item:	8.11

Document for:	Discussion and Decision

Introduction

According to the outcome of the preparation phase discussion declared by the session chair, the following R17 eSL power saving RA issues are identified as high priority and editorial for maintenance to be treated in this meeting.

[110bis-e-R17-Sidelink-03] Email discussion for maintenance on resource allocation for power saving for the following issues in R1-2210333 – Kevin (OPPO)

- Issues 1-6, 1-7, 1-9

- Editorial issues for providing to spec editors: issues 1-15, 1-16, 1-17

- Check points: October 14, October 19

| Issue# | Issue | References |
| --- | --- | --- |
| 1-6 | Clarification on the min number of Y and Y’ slots In the higher layer parameter section (before Step 1) - Optionally, minimum number of Y slots as Ymin (sl-MinNumCandidateSlotsPeriodic), which indicates the minimum number of Y slots that are included in the candidate resources corresponding to periodic-based partial sensing operation for periodic transmissions. [5] - Optionally, minimum number of Y slots as Ymin (sl-MinNumCandidateSlotsPeriodic), which indicates the minimum number of Y slots that are included in the candidate resources corresponding to periodic-based partial sensing for resource (re)selection triggered by periodic transmission. [7] - Optionally, minimum number of Y slots as Ymin (sl-MinNumCandidateSlotsPeriodic), which indicates the minimum number of Y slots that are included in the candidate resources if Prsvp_TX≠0. [9] - Optionally, minimum number of Y slots as Ymin (minNumCandidateSlotsPeriodic), which indicates the minimum number of Y slots that are included in the resources corresponding to periodictransmission. [27] - Optionally, minimum number of Y' slots as Y'min (sl-MinNumCandidateSlotsAperiodic), which indicates the minimum number of Y' slots that are included in the candidate resources corresponding to contiguous partial sensing operation for aperiodic transmissions. [5] - Optionally, minimum number of Y' slots as Y'min (sl-MinNumCandidateSlotsAperiodic), which indicates the minimum number of Y' slots that are included in the candidate resources corresponding to periodic-based partial sensing and/or contiguous partial sensing for resource (re)selection triggered by aperiodic transmission. [7] - Optionally, minimum number of Y' slots as Y'min (sl-MinNumCandidateSlotsAperiodic), which indicates the minimum number of Y' slots that are included in the candidate resources if Prsvp_TX=0. [9] - Optionally, minimum number of Y' slots as Y'min (sl-MinNumCandidateSlotsAperiodic), which indicates the minimum number of Y' slots that are included in the candidate resources corresponding to periodic-based partial sensing and/or contiguous partial sensing operation. [23] - Optionally, minimum number of Y' slots as Y'min (minNumCandidateSlotsAperiodic), which indicates the minimum number of Y' slots that are included in the resources corresponding to aperiodic transmission. [27] | [5] [7] [9] [23] [27] |
| 1-7 | Step 1), clarification on Y and Y’ candidate slots based on partial sensing and/or Prsvp_TX In Step 1) - The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 for UE performing periodic-based partial sensing if Prsvp_TX≠0, correspond to one candidate single-slot resource, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] for UE performing at least contiguous partial sensing if Prsvp_TX=0, correspond to one candidate single-slot resource, where … [7] - The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 if Prsvp_TX≠0, correspond to one candidate single-slot resource, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] if Prsvp_TX=0, correspond to one candidate single-slot resource, where … [9] - The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 for UE performing periodic-based partial sensing correspond to one candidate single-slot resource if Prsvp_TX≠0, or … [14] [5] - The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 for UE performing periodic-based partial sensing correspond to one candidate single-slot resource for a resource (re)selection triggered by periodic transmission, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] for UE performing contiguous partial sensing if Prsvp_TX=0, correspond to one candidate single-slot resource for a resource (re)selection triggered by aperiodic transmission, where … [23] - The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 if Prsvp_TX≠0correspond to one candidate single-slot resource, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] if Prsvp_TX=0, correspond to one candidate single-slot resource, where … [27] - Y' is selected by UE where Y'≥Ymin'. When the UE performs at least contiguous partial sensing and if Prsvp_TX=0, … [7] - Y' is selected by UE where Y'≥Ymin'. If Prsvp_TX=0, … [9] [27] | [5] [7] [9] [14] [23] [27] |
| 1-9 | Step 2), add CPS for the case of sl-MultiReserveResource is enabled and Prsvp_TX=0, remove a redundant sentence In Step 2) - From [2]: - For the case of sl-MultiReserveResource is enabled and Prsvp_TX=0, CPS description is additionally included for Y’ candidate slots and the parameter sl-CPS-WindowAperiodic is used for the M value. - For the case of sl-MultiReserveResource is disabled, it is clarified that t'y0SL is the first slot of the selected Y’ candidate slots. Also form [4] [7] - From [4]: - For the case of sl-MultiReserveResource is enabled and Prsvp_TX=0, reuse existing CPS behavior from the case of sl-MultiReserveResource is disabled and Prsvp_TX=0, since the behavior is the same in RAN1’s agreement for these two cases. Similar change from [7], [9] and [23] but they both use different modification methods. - Move the condition “if Prsvp_TX≠0” to the beginning of the section in the description when UE performs both PBPS and CPS, since the whole section is for the case when Prsvp_TX≠0. Also form [7] [9] - Remove the following redundant sentence since it is not applicable for the case when Prsvp_TX≠0. “Prsvp_TX=0” Also form [7] | [2] [4] [7] [9] [23] |
| 1-15 | Editorial corrections in Step 2) In Step 2) - Whether the UE is required to performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by higher layer parameter sl-PartialSensingInactiveTime. - For cases of Prsvp_TX=0 and Prsvp_TX≠0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB]. | [7] |
| 1-16 | Editorial corrections in Step 6) In Step 6) - Replace (t0'SL, t1'SL,⋯, tTmax'-1'SL) with t'0SL,t'1SL,...,t'T'max-1SL in two places | [7] |
| 1-17 | Editorial corrections in re-evaluation and pre-emption checking In re-evaluation and pre-emption checking for periodic transmission (Prsvp_TX≠0) - By default, M is 31 unless (pre-)configured with another value, by sl-CPS-WindowPeriodic. In re-evaluation and pre-emption checking for aperiodic transmission (Prsvp_TX=0) - For minimum size M of the contiguous partial sensing window [n+TA, n+TB] | [2] [7] |

High priority issues for discussion

## [CLOSED] Issue #1-6: Clarification on the min number of Y and Y’ slots

Background:

In the higher layer parameter section (before Step 1)

-	Optionally, minimum number of Y slots as Ymin (sl-MinNumCandidateSlotsPeriodic), which indicates the minimum number of Y slots that are included in the candidate resources corresponding to periodic-based partial sensing operation for periodic transmissions. [5]

-	Optionally, minimum number of Y slots as Ymin (sl-MinNumCandidateSlotsPeriodic), which indicates the minimum number of Y slots that are included in the candidate resources corresponding to periodic-based partial sensing for resource (re)selection triggered by periodic transmission. [7]

-	Optionally, minimum number of Y slots as Ymin (sl-MinNumCandidateSlotsPeriodic), which indicates the minimum number of Y slots that are included in the candidate resources if Prsvp_TX≠0. [9]

-	Optionally, minimum number of Y slots as Ymin (minNumCandidateSlotsPeriodic), which indicates the minimum number of Y slots that are included in the resources corresponding to periodictransmission. [27]

-	Optionally, minimum number of Y' slots as Y'min   (sl-MinNumCandidateSlotsAperiodic), which indicates the minimum number of Y' slots that are included in the candidate resources corresponding to contiguous partial sensing operation for aperiodic transmissions. [5]

-	Optionally, minimum number of Y' slots as Y'min   (sl-MinNumCandidateSlotsAperiodic), which indicates the minimum number of Y' slots that are included in the candidate resources corresponding to periodic-based partial sensing and/or contiguous partial sensing for resource (re)selection triggered by aperiodic transmission. [7]

-	Optionally, minimum number of Y' slots as Y'min   (sl-MinNumCandidateSlotsAperiodic), which indicates the minimum number of Y' slots that are included in the candidate resources if Prsvp_TX=0. [9]

-	Optionally, minimum number of Y' slots as Y'min   (sl-MinNumCandidateSlotsAperiodic), which indicates the minimum number of Y' slots that are included in the candidate resources corresponding to periodic-based partial sensing and/or contiguous partial sensing operation. [23]

-	Optionally, minimum number of Y' slots as Y'min   (minNumCandidateSlotsAperiodic), which indicates the minimum number of Y' slots that are included in the resources corresponding to aperiodic transmission. [27]

Agreement from RAN1#104-e: In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window, where - FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE - The resource selection window is [n+T1, n+T2] - As a baseline, T1 and T2 are defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4] - Further discuss whether or not to introduce a threshold to re-define T1 and T2 such that - T1 ≥ 0 (subject to processing time constraint Tproc, 1), and T2 ≤ remaining PDB - T2-T1 ≤ (pre-)configured threshold - A minimum value for Y is (pre-)configured from a range of values, FFS details - FFS any restriction to determine Y candidate slots (including its relationship with SL-DRX) - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX - Note: The terminology “periodic-based partial sensing” is based on the “partial sensing” used in LTE-V and it is intended to be used for the design and discussion of partial sensing in Rel-17. Agreement from RAN1#106-e When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, - For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n - A set of candidate resource (SA) is initialized to the set of selected Y candidate slots of PBPS - UE performs contiguous partial sensing in [n+TA, n+TB] for resource exclusion from the initialized candidate resource set (SA) - FFS details of TA and TB based on the agreement(s) from previous RAN1 meetings - Note, re-evaluation and pre-emption checking based on periodic-based and contiguous partial sensing schemes is considered separately FFS: The condition under which UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled Agreement from RAN1#107-e When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, - The UE selects a set of Y’ candidate slots with corresponding PBPS and/or CPS results (if available) within the RSW. - If the total number of Y’ candidate slots is less than a (pre-)configured threshold Y’min, - How UE includes other candidate slots is up to UE implementation - Candidate resource set (SA) is initialized to the set of all single-slot candidate resources in the selected Y’ candidate slots. - For the CPS monitoring window [n+TA, n+TB]: - TA and TB are both selected such that UE has sensing results starting at M consecutive logical slots before ty0 and ending at Tproc,0 + Tproc,1 slots earlier than ty0. - FFS: By default, M is 31 unless (pre-)configured with another value, or where M is (pre-)configured based on transmission priority - FFS: The range of (pre-)configured M from a TBD lowest value up to 30 - When the minimum M slots for CPS cannot be guaranteed, support both - Option A, the UE ensures the Y’min criterion is fulfilled - Option B: UE performs random resource selection - When the UE performs Option A or Option B is up to UE implementation

FL assessment/comments/recommendation:

- When Y and Ymin slots were first discussed during the WI, these were agreed for periodic-based partial sensing and for periodic transmissions. Therefore, I can see that all of the above TPs are correct.

- On the other hand, Y' and Y'min slots were introduced for aperiodic transmissions. The selection of Y’ candidate slots may correspond to PBPS and/or CPS results if available within the selection window.

- Therefore, FL recommends the following changes in Section 2.2.1.

### FL Proposal for Round 1

Text Proposal 1-6 (I):

------------------------------------------------ Start of Text Proposal for TS 38.214 ------------------------------------------------

8.1.4   UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

-	Optionally, minimum number of Y slots as Ymin (sl-MinNumCandidateSlotsPeriodic), which indicates the minimum number of Y slots that are included in the candidate resources corresponding to periodic-based partial sensing for resource (re)selection triggered by periodic transmission (Prsvp_TX≠0).

-	Optionally, minimum number of Y' slots as Y'min   (sl-MinNumCandidateSlotsAperiodic), which indicates the minimum number of Y' slots that are included in the candidate resources corresponding to periodic-based partial sensing and/or contiguous partial sensing results (if available) for resource (re)selection triggered by aperiodic transmission (Prsvp_TX=0).

--------------------------------------------------------End of Text Proposal ------------------------------------------------------------

| Company | Comments |
| --- | --- |
| Sharp | We wonder whether PBPS and CPS are necessary in these 2 paragraphs, as they are almost the same as the change in Issue 1-7. In our views, we could simply change as: - Optionally, minimum number of Y slots as Ymin (sl-MinNumCandidateSlotsPeriodic), which indicates the minimum number of Y slots that are included in the candidate resources corresponding to periodic-based partial sensing for resource (re)selection triggered by periodic transmission (Prsvp_TX≠0). - Optionally, minimum number of Y' slots as Y'min (sl-MinNumCandidateSlotsAperiodic), which indicates the minimum number of Y' slots that are included in the candidate resources corresponding to periodic-based partial sensing and/or contiguous partial sensing results (if available) for resource (re)selection triggered by aperiodic transmission (Prsvp_TX=0). FL response: When implementing the TP for issues #1-6 and #1-7, the intention is to match their descriptions closely. That is, if the wordings for PBPS and CPS are used in one place, it is better to mention PBPS and CPS in other places too for Y and Y’ slots in Step 1) to be consistent. Given that there is a concern from CATT/GH on the proposed deletion and others seem OK with the original proposal, my suggestion is to keep the original one. |
| Qualcomm | We are ok with the proposal |
| vivo | We are ok with the proposal |
| DCM | OK |
| CATT/GH | We support the proposal. For Sharp’s change, deleting partial sensing related description may lead to some misunderstanding for the usage of Ymin or Y’min. Align the description of Ymin and Y’min with the corresponding RRC parameter as the TP seems more reasonable. |
| ZTE, Sanechips | We are ok with the proposal. |
| Lenovo | We are supportive of the proposal. |
| Xiaomi | We support the proposal |
| OPPO | Support |

### Outcome of Week 1 Wednesday GTW session

Agreement

Adopt the following TP for TS 38.214

------------------------------------------------ Start of Text Proposal for TS 38.214 ------------------------------------------------

8.1.4   UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

-	Optionally, minimum number of Y slots as Ymin (sl-MinNumCandidateSlotsPeriodic), which indicates the minimum number of Y slots that are included in the candidate resources corresponding to periodic-based partial sensing and contiguous partial sensingfor resource (re)selection triggered by periodic transmission (Prsvp_TX≠0).

-	Optionally, minimum number of Y' slots as Y'min   (sl-MinNumCandidateSlotsAperiodic), which indicates the minimum number of Y' slots that are included in the candidate resources corresponding to periodic-based partial sensing and/or contiguous partial sensing results (if available) for resource (re)selection triggered by aperiodic transmission (Prsvp_TX=0).

--------------------------------------------------------End of Text Proposal ------------------------------------------------------------

## [CLOSED] Issue #1-7: Step 1), clarification on Y and Y’ candidate slots based on partial sensing and/or Prsvp_TX

Background:

In Step 1)

- The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 for UE performing periodic-based partial sensing if Prsvp_TX≠0, correspond to one candidate single-slot resource, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] for UE performing at least contiguous partial sensing if Prsvp_TX=0, correspond to one candidate single-slot resource, where … [7]

- The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 if Prsvp_TX≠0, correspond to one candidate single-slot resource, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] if Prsvp_TX=0, correspond to one candidate single-slot resource, where … [9]

- The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 for UE performing periodic-based partial sensing correspond to one candidate single-slot resource if Prsvp_TX≠0, or … [14] [5]

- The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 for UE performing periodic-based partial sensing correspond to one candidate single-slot resource for a resource (re)selection triggered by periodic transmission, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] for UE performing contiguous partial sensing if Prsvp_TX=0, correspond to one candidate single-slot resource for a resource (re)selection triggered by aperiodic transmission, where … [23]

- The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 if Prsvp_TX≠0correspond to one candidate single-slot resource, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] if Prsvp_TX=0, correspond to one candidate single-slot resource, where … [27]

- Y' is selected by UE where Y'≥Ymin'. When the UE performs at least contiguous partial sensing and if Prsvp_TX=0, … [7]

- Y' is selected by UE where Y'≥Ymin'. If Prsvp_TX=0, … [9] [27]

FL assessment/comments/recommendations:

- According to the past agreements cited in Section 2.2.1 for Issue #1-6 (not copied here again to save space) and taking into account of all the proposed TPs in this meeting, the following TP 1-7 (I) in Section 2.3.1 is recommended from the FL. This is intended to align with the recommended TP 1-6 (I) in the previous Issue #1-6.

### FL Proposal for Round 1

Text Proposal 1-7 (I):

------------------------------------------------ Start of Text Proposal for TS 38.214 ------------------------------------------------

8.1.4   UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

1)	A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 correspond to one candidate single-slot resource for UE performing periodic-based partial sensing and resource (re)selection triggered by periodic transmission (Prsvp_TX≠0), or in a set of Y' candidate slots within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing at least contiguous partial sensing and resource (re)selection triggered by aperiodic transmission (Prsvp_TX=0), where

-	selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL  , where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP;

-	if  T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots).

-	Y is selected by UE where Y≥Ymin.

-	Y' is selected by UE where Y'≥Ymin'. When the UE performs at least contiguous partial sensing and if Prsvp_TX=0, the UE selects a set of Y' candidate slots with corresponding PBPS and/or CPS results (if available). If the number of candidate single-slot resources Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots.

The total number of candidate single-slot resources is denoted by Mtotal.

--------------------------------------------------------End of Text Proposal ------------------------------------------------------------

| Company | Comments |
| --- | --- |
| Sharp | We are fine with the changes. |
| Qualcomm | We are ok with the proposal |
| vivo | We are ok with the proposal |
| DCM | We propose a bit update. Y slots are included in the candidate resources corresponding to PBPS, this is the text in issue 1-6. But this does not mean Y slots are for UE performing PBPS. Both CPS/PBPS are performed for the Y slots, and how to decide Y slots are based on PBPS. Then now the current update ‘for UE performing periodic-based partial sensing’ is misleading like only PBPS is performed. To avoid this, we suggest the following update. in a set of Y candidate slots within the time interval n+T1,n+T2 correspond to one candidate single-slot resource corresponding to periodic-based partial sensing and for UE performing resource (re)selection triggered by periodic transmission (Prsvp_TX≠0), FL response: According to the PBPS agreement made in RAN1#104-e (cited above) and in my understanding, if the Y slots are selected and correspond to PBPS, then it means the UE performs PBPS. For the suggested modification, it does not change the meaning technically but now the sentence reads a bit strange. Since other companies are OK with the original proposal and no supports for this modified version, the original version seems stable to the majority. |
| CATT/GH | We support the proposal. |
| ZTE, Sanechips | We are ok with the proposal. |
| Lenovo | OK. |
| Xiaomi | OK |
| OPPO | Support |

### Outcome of Week 1 Wednesday GTW session

Agreement

Adopt the TP below for TS38.214

------------------------------------------------ Start of Text Proposal for TS 38.214 ------------------------------------------------

8.1.4   UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

1)	A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 correspond to one candidate single-slot resource for UE performing periodic-based partial sensing together with contiguous partial sensing and resource (re)selection triggered by periodic transmission (Prsvp_TX≠0), or in a set of Y' candidate slots within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing at least contiguous partial sensing and resource (re)selection triggered by aperiodic transmission (Prsvp_TX=0), where

-	selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL  , where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP;

-	if  T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots).

-	Y is selected by UE where Y≥Ymin.

-	Y' is selected by UE where Y'≥Ymin'. When the UE performs at least contiguous partial sensing and if Prsvp_TX=0, the UE selects a set of Y' candidate slots with corresponding PBPS and/or CPS results (if available). If the number of candidate single-slot resources Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots.

The total number of candidate single-slot resources is denoted by Mtotal.

--------------------------------------------------------End of Text Proposal ------------------------------------------------------------

## [ACTIVE] Issue #1-9: Step 2), add CPS for the case of sl-MultiReserveResource is enabled and Prsvp_TX=0, remove a redundant sentence

Background:

In Step 2)

- From [2]:

  - For the case of sl-MultiReserveResource is enabled and Prsvp_TX=0, CPS description is additionally included for Y’ candidate slots and the parameter sl-CPS-WindowAperiodic is used for the M value.

  - For the case of sl-MultiReserveResource is disabled, it is clarified that t'y0SL is the first slot of the selected Y’ candidate slots. Also form [4] [7]

- From [4]:

  - For the case of sl-MultiReserveResource is enabled and Prsvp_TX=0, reuse existing CPS behavior from the case of sl-MultiReserveResource is disabled and Prsvp_TX=0, since the behavior is the same in RAN1’s agreement for these two cases. Similar change from [7], [9] and [23] but they both use different modification methods.

  - Move the condition “if Prsvp_TX≠0” to the beginning of the section in the description when UE performs both PBPS and CPS, since the whole section is for the case when Prsvp_TX≠0. Also form [7] [9]

  - Remove the following redundant sentence since it is not applicable for the case when Prsvp_TX≠0. “Prsvp_TX=0” Also form [7]

FL assessment/comments/recommendation:

- Observing from all the proposed TPs for the missing case “when sl-MultiReserveResource is enabled and Prsvp_TX=0”, instead of introducing an additional description in the spec as proposed in [2], it is simpler to adopt the approach proposed in [4] [7] [9] of making use of the existing description for the case “when sl-MultiReserveResource is disabled and Prsvp_TX=0”. At the same time, I have also tried to make the change even simpler in the recommended TP 1-9 (I) in Section 2.4.1.

- For the case of Prsvp_TX≠0, simply followed the majority proposal and removed the redundant sentence at the end of the paragraph.

### FL Proposal for Round 1

Text Proposal 1-9 (I):

------------------------------------------------ Start of Text Proposal for TS 38.214 ------------------------------------------------

8.1.4   UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled and Prsvp_TX≠0, the sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot t'y0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. Prsvp_TX≠0  The value of M is (pre-)configured with the sl-CPS-WindowPeriodic. If sl-CPS-WindowPeriodic is not (pre-)configured, M equals to 31.Prsvp_TX=0

When the UE performs at least contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) either disabled or enabled and if Prsvp_TX=0, the sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at least M consecutive logical slots before t'y0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y'candidate slots. The value of M is (pre-)configured with the sl-CPS-WindowAperiodic. If sl-CPS-WindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

--------------------------------------------------------End of Text Proposal ------------------------------------------------------------

| Company | Comments |
| --- | --- |
| Sharp | We are fine with the proposed changes. |
| Qualcomm | We are largely ok with the proposal but propose the following modification for brevity: When the UE performs at least contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) either disabled or enabled and if Prsvp_TX=0, the sensing window |
| vivo | We are ok with the proposal |
| DCM | OK |
| CATT/GH | We are generally fine with the proposal. For the second paragraph of the TP, we support the more concise modification proposed by Qualcomm. |
| ZTE, Sanechips | We are ok with the proposal, also agree with Qualcomm’s update. |
| Lenovo | OK. |
| Xiaomi | OK. We are also fine with QC version |
| OPPO | Support |

### Text Proposal for Week 1 Wednesday GTW session and continuing discussion in Round 1

Text Proposal 1-9 (II):

------------------------------------------------ Start of Text Proposal for TS 38.214 ------------------------------------------------

8.1.4   UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled and Prsvp_TX≠0, the sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot t'y0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. Prsvp_TX≠0  The value of M is (pre-)configured with the sl-CPS-WindowPeriodic. If sl-CPS-WindowPeriodic is not (pre-)configured, M equals to 31.Prsvp_TX=0

When the UE performs at least contiguous partial sensing and if Prsvp_TX=0, the sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at least M consecutive logical slots before t'y0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y'candidate slots. The value of M is (pre-)configured with the sl-CPS-WindowAperiodic. If sl-CPS-WindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

--------------------------------------------------------End of Text Proposal ------------------------------------------------------------

| Company | Comments |
| --- | --- |

Editorial issues for discussion

## [ACTIVE] Issue #1-15, #1-16, #1-17: Editorial corrections in Step 2), Step 6) and re-evaluation and pre-emption checking

### Text Proposal for Round 1

FL comment:

- While implementing the editorial TP below, I have also identified other editorial/format issues in the same sections that could be fixed at the same time to align with other parts of the spec. It is proposed to make these changes now to save discussion in the future.

Text Proposal 1-15/16/17 (I):

------------------------------------------------ Start of Text Proposal for TS 38.214 ------------------------------------------------

8.1.4   UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot t'y0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. If Prsvp_TX≠0  the value of M is (pre-)configured with the sl-CPS-WindowPeriodic. If sl-CPS-WindowPeriodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

When the UE performs contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) disabled and if Prsvp_TX=0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at least M consecutive logical slots before t'y0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than t'y0SL. The value of M is (pre-)configured with the sl-CPS-WindowAperiodic. If sl-CPS-WindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

Whether the UE is required to performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by higher layer parameter sl-PartialSensingInactiveTime. When it is enabled, if UE performs periodic-based partial sensing on the slots in SL DRX inactive time for a given periodicity corresponding to Preserve, UE monitors only the default periodic sensing occasions (most recent sensing occasion) from the slots; if UE performs contiguous partial sensing on the slots in SL DRX inactive time, UE monitors a minimum of M slots from the slots.

<Unchanged parts omitted>

c)	the SCI format received in slot t'mSL or the same SCI format which, if and only if the 'Resource reservation period' field is present in the received SCI format 1-A, is assumed to be received in slot(s) t'm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where if the UE is configured with full sensing by its higher layer, t'n'SL = n if slot n belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise slot t'n'SL is the first slot after slot n belonging to the set t'0SL,t'1SL,...,t'T'max-1SL; If UE is configured with partial sensing by its higher layer, t'n'SL=t'yiSL-Tproc,1SL if slot t'yiSL-Tproc,1SL belongs to the set (t0'SL, t1'SL,⋯, tTmax'-1'SL)t'0SL,t'1SL,...,t'T'max-1SL, otherwise, slot t'n'SL is the first slot after slot t'yiSL-Tproc,1SL belonging to the set (t0'SL, t1'SL,⋯, tTmax'-1'SL)t'0SL,t'1SL,...,t'T'max-1SL. Otherwise Q=1. If the UE is configured with full sensing by its higher layer, Tscal is set to selection window size T2 converted to units of msec. If UE is configured with partial sensing by its higher layer, Tscal=t'yLSL-(t'yiSL-Tproc,1SL) shall be converted to milliseconds, where slot t'yLSL is the last slot of the Y or Y' candidate slots. The slot t'yiSL is the first slot of the selected/remaining set of Y or Y' candidate slots.

<Unchanged parts omitted>

When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX=0,

-	During the qth reservation period (q=0,1,2,…, Cresel-1), candidate resource set (SA) is initialized to the remaining Y candidate slots starting from slot t'yiSL and ending at the last slot of the Y candidate slots, where the slot indices of the remaining Y candidate slots are equal to t'y+q×Prsvp_TX'SL, where t'ySL is a slot index of Y candidate slots used in the initial resource (re)selection.

-	t'yiSL is the first candidate slot after slot n+T3.

-	The UE performs PBPS for the remaining Y candidate slots according to ty'-k×Preserve''SL, wheret'y'SL is a slot belonging to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than t'yiSL-(Tproc,0SL+Tproc,1SL ) if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if sl-Additional-PBPS-Occasion is (pre-)configured.

-	The UE performs CPS starting from M logical slots earlier than t'yiSL to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL.

-	By default, M is 31 unless (pre-)configured with another value by sl-CPS-WindowPeriodic.

When the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX≠0 ,

-	Candidate resource set (SA) is initialized to the remaining Y' candidate slots starting from slot t'yiSL and ending at the last slot of the Y' candidate slots, where t'yiSL is the first candidate slot after slot n+T3.

-	It is up to UE implementation that UE may perform PBPS for periodic sensing occasions after the resource (re)selection when higher layer parameter sl-MultiReserveResource is enabled

-	UE performs CPS starting from at least M consecutive logical slots earlier than t'yiSL to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL

-	For minimum size M of the contiguous partial sensing window [n+TA, n+TB], by default, M is 31 unless (pre-)configured with another value, by sl-CPS-WindowAperiodic.

When the minimum M slots for CPS cannot be guaranteed, UE senses in all available slots starting from the resource (re)selection trigger slot of the same TB to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL. The UE re-evaluation and pre-emption checking is based on all available sensing results after n –T0.

--------------------------------------------------------End of Text Proposal ------------------------------------------------------------

| Company | Comments |
| --- | --- |
| Sharp | Regarding the first two changes with the addition of “contiguous partial”, we don’t think it is essential. In this case, UE is configured by higher layer to perform partial sensing, not full sensing, so there is no ambiguity. If majority companies support the change, we are fine. For pre-emption/re-evaluation, it seems FL made a typo for the editorial changes. The first should be for Prsvp_TX≠0 and the latter is for Prsvp_TX=0. |
| Qualcomm | We have the same comment as Sharp about Prsvp_TX≠0 and Prsvp_TX=0. |
| vivo | Regarding the addition of “contiguous partial”, we think it is needed for clarity. We have the same comment as Sharp about Prsvp_TX≠0 and Prsvp_TX=0. |
| DCM | Same view with QC. We are fine with adding ‘contiguous partial’. |
| CATT/GH | We also think the addition of “contiguous partial” in the first two paragraphs is essential for clarification and also for alignment with the re-evaluation and pre-emption part. We share the same view with other companies that the typo for Prsvp_TX≠0 and Prsvp_TX=0 should be modified. |
| ZTE, Sanechips | Same comments about Prsvp_TX≠0 and Prsvp_TX=0. |
| Lenovo | We hold the same view with Sharp regarding Prsvp_TX≠0 and Prsvp_TX=0. |
| OPPO | Same comments about Prsvp_TX≠0 and Prsvp_TX=0. |

### Text Proposal for Week 1 Wednesday GTW session and continuing discussion in Round 1

Text Proposal 1-15/16/17 (II):

------------------------------------------------ Start of Text Proposal for TS 38.214 ------------------------------------------------

8.1.4   UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot t'y0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. If Prsvp_TX≠0  the value of M is (pre-)configured with the sl-CPS-WindowPeriodic. If sl-CPS-WindowPeriodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

When the UE performs contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) disabled and if Prsvp_TX=0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at least M consecutive logical slots before t'y0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than t'y0SL. The value of M is (pre-)configured with the sl-CPS-WindowAperiodic. If sl-CPS-WindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

Whether the UE is required to performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by higher layer parameter sl-PartialSensingInactiveTime. When it is enabled, if UE performs periodic-based partial sensing on the slots in SL DRX inactive time for a given periodicity corresponding to Preserve, UE monitors only the default periodic sensing occasions (most recent sensing occasion) from the slots; if UE performs contiguous partial sensing on the slots in SL DRX inactive time, UE monitors a minimum of M slots from the slots.

<Unchanged parts omitted>

c)	the SCI format received in slot t'mSL or the same SCI format which, if and only if the 'Resource reservation period' field is present in the received SCI format 1-A, is assumed to be received in slot(s) t'm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where if the UE is configured with full sensing by its higher layer, t'n'SL = n if slot n belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise slot t'n'SL is the first slot after slot n belonging to the set t'0SL,t'1SL,...,t'T'max-1SL; If UE is configured with partial sensing by its higher layer, t'n'SL=t'yiSL-Tproc,1SL if slot t'yiSL-Tproc,1SL belongs to the set (t0'SL, t1'SL,⋯, tTmax'-1'SL)t'0SL,t'1SL,...,t'T'max-1SL, otherwise, slot t'n'SL is the first slot after slot t'yiSL-Tproc,1SL belonging to the set (t0'SL, t1'SL,⋯, tTmax'-1'SL)t'0SL,t'1SL,...,t'T'max-1SL. Otherwise Q=1. If the UE is configured with full sensing by its higher layer, Tscal is set to selection window size T2 converted to units of msec. If UE is configured with partial sensing by its higher layer, Tscal=t'yLSL-(t'yiSL-Tproc,1SL) shall be converted to milliseconds, where slot t'yLSL is the last slot of the Y or Y' candidate slots. The slot t'yiSL is the first slot of the selected/remaining set of Y or Y' candidate slots.

<Unchanged parts omitted>

When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX≠0,

-	During the qth reservation period (q=0,1,2,…, Cresel-1), candidate resource set (SA) is initialized to the remaining Y candidate slots starting from slot t'yiSL and ending at the last slot of the Y candidate slots, where the slot indices of the remaining Y candidate slots are equal to t'y+q×Prsvp_TX'SL, where t'ySL is a slot index of Y candidate slots used in the initial resource (re)selection.

-	t'yiSL is the first candidate slot after slot n+T3.

-	The UE performs PBPS for the remaining Y candidate slots according to ty'-k×Preserve''SL, wheret'y'SL is a slot belonging to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than t'yiSL-(Tproc,0SL+Tproc,1SL ) if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if sl-Additional-PBPS-Occasion is (pre-)configured.

-	The UE performs CPS starting from M logical slots earlier than t'yiSL to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL.

-	By default, M is 31 unless (pre-)configured with another value by sl-CPS-WindowPeriodic.

When the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX=0 ,

-	Candidate resource set (SA) is initialized to the remaining Y' candidate slots starting from slot t'yiSL and ending at the last slot of the Y' candidate slots, where t'yiSL is the first candidate slot after slot n+T3.

-	It is up to UE implementation that UE may perform PBPS for periodic sensing occasions after the resource (re)selection when higher layer parameter sl-MultiReserveResource is enabled

-	UE performs CPS starting from at least M consecutive logical slots earlier than t'yiSL to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL

-	For minimum size M of the contiguous partial sensing window [n+TA, n+TB], by default, M is 31 unless (pre-)configured with another value, by sl-CPS-WindowAperiodic.

When the minimum M slots for CPS cannot be guaranteed, UE senses in all available slots starting from the resource (re)selection trigger slot of the same TB to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL. The UE re-evaluation and pre-emption checking is based on all available sensing results after n –T0.

--------------------------------------------------------End of Text Proposal ------------------------------------------------------------

| Company | Comments |
| --- | --- |

- References

- R1-2208386	Discussion on resolving ambiguous text in 38.214	FUTUREWEI

- R1-2208610	Corrections for partial sensing resource selection	vivo

- R1-2208816	Draft CR on Q formula in step 6c for periodic-based partial sensing	OPPO

- R1-2208817	Draft CR on CPS sensing window	OPPO

- R1-2208818	Draft CR on the description of candidate slots for partial sensing	OPPO

- R1-2208819	Draft CR on starting slot and pre-condition in re-evaluation and pre-emption checking for partial sensing	OPPO

- R1-2208919	Correction on the operations of partial sensing	CATT, GOHIGH

- R1-2208922	Discussion on remaining issues for R17 eSL power saving RA maintenance	CATT, GOHIGH

- R1-2209309	Corrections on the selection of Y or Y’ candidate slots for partial sensing	CMCC

- R1-2209562	Correction on Q formula for the second most recent periodic sensing occasion	Apple

- R1-2209563	Correction on CPS monitoring length during sidelink DRX inactive time	Apple

- R1-2209676	Clarification on pre-emption and re-evaluation for periodic transmission in partial sensing	Sharp

- R1-2209677	Clarification on monitoring slots for pre-emption check due to half-duplex constraint in partial sensing	Sharp

- R1-2209678	Correction on candidate slots selection for partial sensing	Sharp

- R1-2209680	Clarification on Preserve for periodic based partial sensing	Sharp

- R1-2209681	Clarification on candidate slots for aperiodic transmission in partial sensing	Sharp

- R1-2209683	Remaining issues on NR sidelink enhancement	Sharp

- R1-2209827	Correction on Q formula in step 6 of sensing and resource exclusion procedure in TS 38.214	Huawei, HiSilicon

- R1-2209828	Correction on description of random resource selection in TS 38.214	Huawei, HiSilicon

- R1-2209874	Draft CR on half-duplex consideration for SL re-evaluation/pre-emption check	NTT DOCOMO, INC.

- R1-2209875	Draft CR on insufficient candidate resources for SL re-evaluation/pre-emption check	NTT DOCOMO, INC.

- R1-2209876	Draft CR on slot n+T3 excluded from SL re-evaluation/pre-emption check	NTT DOCOMO, INC.

- R1-2209877	Draft CR on Y/Y’ candidate slots for SL partial sensing	NTT DOCOMO, INC.

- R1-2210125	[Draft] Consideration of associated processing times for contiguous partial sensing	Ericsson

- R1-2210126	[Draft] Correction to allowed resource selection mechanisms in a resource pool in mode 2	Ericsson

- R1-2210127	[Draft] Correction to contiguous partial sensing window	Ericsson

- R1-2210154	Draft CR on corrections for the description of candidate slots in TS38.214	Lenovo

- R1-2210333	Moderator summary for AI 8.11: Maintenance on NR sidelink enhancement	Moderator (LG Electronics)