3GPP TSG RAN WG1 #110bis	                                  	R1-2210300

eMeeting, October 10th– 14th, 2022

Agenda Item: 8.14

Source:	Moderator (Sony)

Title:	FL summary 2 of AI 8.14: Maintenance on Timing Relationships for IoT-NTN

Document for:	Discussion

Table of Contents

1	Introduction	1

1.1	Stable Proposals	2

2	Detailed Analysis of Retained Issues	4

2.1	Issues# 1-6: Koffset name alignment to higher layer specs	4

2.1.1	FIRST ROUND Discussion Issues# 1-6: Koffset name alignment to higher layer specs	4

2.1.2	SECOND ROUND Discussion Issues# 1-6: Koffset name alignment to higher layer specs	5

2.2	Issues# 1-7: Redundant definition of Kmac in Clause 16.6 of TS36.213	5

2.2.1	FIRST ROUND Discussion Issues# 1-7: Redundant definition of Kmac in Clause 16.6 of TS36.213	6

2.2.2	SECOND ROUND Discussion Issues# 1-7: Redundant definition of Kmac in Clause 16.6 of TS36.213	6

2.3	Issues# 1-8: Missing Koffset in Clause 7.3.1 of TS36.213	6

2.3.1	FIRST ROUND Discussion Issues# 1-8: Missing Koffset in Clause 7.3.1 of TS36.213	7

2.3.2	SECOND ROUND Discussion Issues# 1-8: Missing Koffset in Clause 7.3.1 of TS36.213	9

3	Referenced Documents	10

# Introduction

This is the feature lead (FL) summary of contributions to the maintenance of timing relationships in the following discussion on Rel17 IoT NTN:

[110bis-e-R17-IoT-NTN-03] Email discussion for maintenance on timing relationship enhancements for issues 1-6, 1-7 and 1-8 to be handed as recommendation for editor’s alignment CR in R1-22XXXXX – Sam (Sony)

- Check points: October 14

Following the preparatory phase discussions on what issues to discuss at RAN1#110bis [R1-2210434], the issues related to timing relationships in IoT NTN for retained for discussion are summarised in the table below. For continuity, we will reuse the issue numbering from [R1-2210434].

| Issue# | Issue | References | Nature of Issue |
| --- | --- | --- | --- |
| 1-6 | Align Koffset parameter names in TS36.213 with names used in higher layer specs TS 36.331 and TS 36.321 | R1-2208689 R1-2210219 | Editorial |
| 1-7 | Remove redundant definition of Kmac (clause 16.6) | R1-2208689 | Editorial |
| 1-8 | Koffset parameter is missing in the procedures described in Clause 7.3.1 of TS 36.213 v17.2.0 | R1-2209650 R1-2210070 | Functionality |

## Stable Proposals

FL Proposal 1-6-2: Recommend TP1 to spec editor

TP1 related to Clauses 4.2.3, 5.1.1.1, 6.1.1, 7.2.3, 7.3, 8, 10, 16, 16.1.2 and 16.6 of TS36.213.

Reason for change:

- Align parameter names to those from higher layer specs

Summary of change:

- Change the parameter names in definition of Koffset to the names for the relevant parameters as are used in the higher layer specs.

Consequence if not approved:

- Confusion on higher layer parameter names

---------------------------------------Text Start---------------------------------------------

-	if the UE is configured with the higher layer parameter CellSpecificKoffset k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter CellSpecificKoffset k-Offset provided by higher layers, and

KUE_offset is the parameter UESpecificKoffset Differential Koffset provided by higher layers, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

-----------------------------------------------End of TP---------------------------------------

FL Proposal 1-7-2: Adopt TP2 for Clause 16.6 of  TS36.213.

TP2 related to Clause 16.6 of TS36.213.

Reason for change:

- Editorial

Summary of change:

- Remove duplicate definition of Kmac.

Consequence if not approved:

- Redundant text in specification

---------------------------------------Text Start---------------------------------------------

If the UE has initiated a NPUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the NPDCCH UE-specific search space in a search space window starting in subframe n+4+Kmac with duration given by higher layer parameter pur-SS-window-duration where Kmac is provided by higher layer parameter K-mac, otherwise Kmac=0. Upon detection of a NPDCCH with DCI format N0 with CRC scrambled by PUR-RNTI intended for the UE within the search space window and the value of "modulation and coding scheme" field () in the corresponding DCI is set to '14', the UE is not required to monitor the NPDCCH UE-specific search space for the remaining search space window duration			-----------------------------------------------End of TP---------------------------------------

FL Proposal 1-8-1: Adopt TP3 for Clause 7.3.1 of  TS36.213.

TP2 related to Clause 7.3.1 of TS36.213.

Reason for change:

- Specify the correct function

Summary of change:

- Take timing relationship enhancement in index of PDSCH subframe.

Consequence if not approved:

- Wrong specification

-------------------------------------------------- Text Starts --------------------------------------------------------

7.3.1	FDD HARQ-ACK reporting procedure

For FDD with PUCCH format 1a/1b transmission, when both HARQ-ACK and SR are transmitted in the same sub-frame/slot, a UE shall transmit the HARQ-ACK on its assigned HARQ-ACK PUCCH format 1a/1b resource for a negative SR transmission and transmit the HARQ-ACK on its assigned SR PUCCH resource for a positive SR transmission.

---------------------------------------------------Text Omitted -----------------------------------------------------

For a BL/CE UE with higher layer parameter ce-PDSCH-14HARQ-Config configured, for PDSCH transmission in subframe n - k - K_offset Koffset, if the UE is in half-duplex FDD operation and is configured with CEModeA, and 'PDSCH scheduling delay and HARQ-ACK delay for 14 HARQ' field is present in the corresponding DCI,

-	if the HARQ-ACK delay value as defined in [4], in the corresponding DCI indicates value k, the UE shall determine the subframe n as the HARQ-ACK transmission subframe.

For a BL/CE UE in half-duplex FDD operation, if the UE is configured with CEModeA, and if the UE is configured with higher layer parameter ce-HARQ-AckBundling and the 'HARQ-ACK bundling flag' in the corresponding DCI is set to 1,

-	for HARQ-ACK transmission in subframe n, the UE shall generate one HARQ-ACK bit by performing a logical AND operation of HARQ-ACKs across all  BL/CE DL subframes for which subframe n is the 'HARQ-ACK transmission subframe'.

-	if subframe n - k1 - Koffset is the most recent subframe for which subframe n is the 'HARQ-ACK transmission subframe', and if the 'Transport blocks in a bundle' field in the corresponding DCI for PDSCH transmission in subframe n - k1 - Koffset indicates a number of transport blocks in a bundle other than , the UE shall generate a NACK for HARQ-ACK transmission in subframe n.

-	if the UE has received W PDSCH transmissions before subframe n, and if the UE is expected to transmit HARQ-ACK for the W PDSCH transmissions in subframes , the UE is not expected to receive a new PDSCH transmission in subframe n, where W=10 if higher layer parameter ce-pdsch-tenProcesses-config is set to 'On', W=12 if higher layer parameter ce-PDSCH-14HARQ-Config is configured, and W=8 otherwise.

-	if the UE is expected to transmit HARQ-ACK for the PDSCH transmissions received before subframe n in subframes , the UE is not expected to receive a new PDSCH transmission in subframe n for which the HARQ-ACK is to be transmitted in subframe

-------------------------------------------------- Text Ends ---------------------------------------------------------

# Detailed Analysis of Retained Issues

From company contributions to RAN1#110bis and the preparatory phase discussions, three issues have been retained as outlined in the table above. In this section, FL will summarise company contributions on each of the retained issues and make proposals.

## Issues# 1-6: Koffset name alignment to higher layer specs

ZTE and Huawei point out this issue in numerous clauses of TS 36.213 where the specification tries to define Koffset using parameter names CellSpecificKoffset and UESpecificKoffset. These names are not used by higher layer specifications such as TS 36.331 and TS 36.321. These specifications use instead the names k-Offset and Differential Koffset for the parameters in question. In R1-2208689, ZTE lists the affected clauses as: 4.2.3, 5.1.1.1, 6.1.1, 7.2.3, 7.3, 8, 10, 16, 16.1.2 and 16.6.

TPs are proposed to change the parameter names wherever they occur in TS36.213 such that the value of Koffset is defined by using the same names for the relevant parameters as are used in the higher layer specs:

==============Unchanged Text Omitted ==============================

-	if the UE is configured with the higher layer parameter CellSpecificKoffset k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter CellSpecificKoffset k-Offset provided by higher layers, and

KUE_offset is the parameter UESpecificKoffset Differential Koffset provided by higher layers, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

==============End of TP==============================

### FIRST ROUND Discussion Issues# 1-6: Koffset name alignment to higher layer specs

FL thinks the TP is well construed and makes the following proposal.

Companies are respectfully asked to consider FL Proposal 1-6-1 on this issue and make their views known in the table.

FL Proposal 1-6-1: Adopt the proposed change of Koffset definition in clauses 4.2.3, 5.1.1.1, 6.1.1, 7.2.3, 7.3, 8, 10, 16, 16.1.2 and 16.6 of TS36.213.

| Company | Support/No support Proposal 1-6-1 | Comments and Proposal |
| --- | --- | --- |
| Lenovo | Support |  |
| MediaTek | Support |  |
| ZTE | Support |  |
| Nokia, NSB | Support |  |
| SONY | Support |  |
| Sequans | Support |  |
| Ericsson | Support |  |

### SECOND ROUND Discussion Issues# 1-6: Koffset name alignment to higher layer specs

All 7 responding companies support the TP. FL will propose for email endorsement at the checkpoint.

FL Proposal 1-6-2: Recommend TP1 to spec editor

TP1 related to Clauses 4.2.3, 5.1.1.1, 6.1.1, 7.2.3, 7.3, 8, 10, 16, 16.1.2 and 16.6 of TS36.213.

Reason for change:

- Align parameter names to those from higher layer specs

Summary of change:

- Change the parameter names in definition of Koffset to the names for the relevant parameters as are used in the higher layer specs.

Consequence if not approved:

- Confusion on higher layer parameter names

---------------------------------------Text Start---------------------------------------------

-	if the UE is configured with the higher layer parameter CellSpecificKoffset k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter CellSpecificKoffset k-Offset provided by higher layers, and

KUE_offset is the parameter UESpecificKoffset Differential Koffset provided by higher layers, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

-----------------------------------------------End of TP---------------------------------------

## Issues# 1-7: Redundant definition of Kmac in Clause 16.6 of TS36.213

Clause 16.6 of TS36.213 commences with the general statement:

“Throughout this clause, if a NB-IoT UE is configured with higher layer parameter k-Mac, Kmac = k-Mac otherwise, Kmac = 0.”

But later in the clause, there is a further definition of Kmac within the text. In the following TP, ZTE rightly proposes that this later definition be suppressed.

<Unchanged parts are omitted>

If the UE has initiated a NPUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the NPDCCH UE-specific search space in a search space window starting in subframe n+4+Kmac with duration given by higher layer parameter pur-SS-window-duration where Kmac is provided by higher layer parameter K-mac, otherwise Kmac=0. Upon detection of a NPDCCH with DCI format N0 with CRC scrambled by PUR-RNTI intended for the UE within the search space window and the value of "modulation and coding scheme" field () in the corresponding DCI is set to '14', the UE is not required to monitor the NPDCCH UE-specific search space for the remaining search space window duration.

<End of TP>

### FIRST ROUND Discussion Issues# 1-7: Redundant definition of Kmac in Clause 16.6 of TS36.213

Firstly, the designation “K-mac” should be “k-Mac” as defined in TS36.331. Secondly, there is clearly no need to have yet another definition of Kmac within the text. FL therefore makes FL Proposal 1-7-1. Companies are respectfully asked to consider FL Proposal 1-7-1 on this issue and make their views known in the table.

FL Proposal 1-7-1: Adopt the proposed TP for Clause 16.6 of  TS36.213.

| Company | Support/No support Proposal 1-7-1: | Comments and Proposal |
| --- | --- | --- |
| Lenovo | Support |  |
| MediaTek | Support |  |
| ZTE | Support |  |
| Nokia, NSB | Support |  |
| SONY | Support |  |
| Sequans | Support |  |
| Ericsson | Support |  |

### SECOND ROUND Discussion Issues# 1-7: Redundant definition of Kmac in Clause 16.6 of TS36.213

All 7 responding companies support the TP. FL will propose for email endorsement at the checkpoint.

FL Proposal 1-7-2: Adopt TP2 for Clause 16.6 of  TS36.213.

TP2 related to Clause 16.6 of TS36.213.

Reason for change:

- Editorial

Summary of change:

- Remove duplicate definition of Kmac.

Consequence if not approved:

- Redundant text in specification

---------------------------------------Text Start---------------------------------------------

If the UE has initiated a NPUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the NPDCCH UE-specific search space in a search space window starting in subframe n+4+Kmac with duration given by higher layer parameter pur-SS-window-duration where Kmac is provided by higher layer parameter K-mac, otherwise Kmac=0. Upon detection of a NPDCCH with DCI format N0 with CRC scrambled by PUR-RNTI intended for the UE within the search space window and the value of "modulation and coding scheme" field () in the corresponding DCI is set to '14', the UE is not required to monitor the NPDCCH UE-specific search space for the remaining search space window duration			-----------------------------------------------End of TP---------------------------------------

## Issues# 1-8: Missing Koffset in Clause 7.3.1 of TS36.213

For Clause 7.3.1 of TS36.213, Ericsson makes the following TP:

--------------------------------------------3GPP TS 36.213 --------------------------------------------

-------------------------------------------------- Text Starts --------------------------------------------------------

7.3.1	FDD HARQ-ACK reporting procedure

For FDD with PUCCH format 1a/1b transmission, when both HARQ-ACK and SR are transmitted in the same sub-frame/slot, a UE shall transmit the HARQ-ACK on its assigned HARQ-ACK PUCCH format 1a/1b resource for a negative SR transmission and transmit the HARQ-ACK on its assigned SR PUCCH resource for a positive SR transmission.

---------------------------------------------------Text Omitted -----------------------------------------------------

For a BL/CE UE with higher layer parameter ce-PDSCH-14HARQ-Config configured, for PDSCH transmission in subframe n - k - K_offset Koffset, if the UE is in half-duplex FDD operation and is configured with CEModeA, and 'PDSCH scheduling delay and HARQ-ACK delay for 14 HARQ' field is present in the corresponding DCI,

-	if the HARQ-ACK delay value as defined in [4], in the corresponding DCI indicates value k, the UE shall determine the subframe n as the HARQ-ACK transmission subframe.

For a BL/CE UE in half-duplex FDD operation, if the UE is configured with CEModeA, and if the UE is configured with higher layer parameter ce-HARQ-AckBundling and the 'HARQ-ACK bundling flag' in the corresponding DCI is set to 1,

-	for HARQ-ACK transmission in subframe n, the UE shall generate one HARQ-ACK bit by performing a logical AND operation of HARQ-ACKs across all  BL/CE DL subframes for which subframe n is the 'HARQ-ACK transmission subframe'.

-	if subframe n - k1 - Koffset is the most recent subframe for which subframe n is the 'HARQ-ACK transmission subframe', and if the 'Transport blocks in a bundle' field in the corresponding DCI for PDSCH transmission in subframe n - k1 - Koffset indicates a number of transport blocks in a bundle other than , the UE shall generate a NACK for HARQ-ACK transmission in subframe n.

-	if the UE has received W PDSCH transmissions before subframe n, and if the UE is expected to transmit HARQ-ACK for the W PDSCH transmissions in subframes , the UE is not expected to receive a new PDSCH transmission in subframe n, where W=10 if higher layer parameter ce-pdsch-tenProcesses-config is set to 'On', W=12 if higher layer parameter ce-PDSCH-14HARQ-Config is configured, and W=8 otherwise.

-	if the UE is expected to transmit HARQ-ACK for the PDSCH transmissions received before subframe n in subframes , the UE is not expected to receive a new PDSCH transmission in subframe n for which the HARQ-ACK is to be transmitted in subframe

-------------------------------------------------- Text Ends ---------------------------------------------------------

--------------------------------------------3GPP TS 36.213 --------------------------------------------

### FIRST ROUND Discussion Issues# 1-8: Missing Koffset in Clause 7.3.1 of TS36.213

In the view of FL, the TP highlights omissions and so is sound. Companies are respectfully asked to consider FL Proposal 1-8-1 on this issue and make their views known.

FL Proposal1-8-1: Adopt the TP for Clause 7.3.1 of TS36.213

| Company | Support/No support Proposal 1-8-1: | Comments and Proposal |
| --- | --- | --- |
| Lenovo | No | The latest version (TS36.213 v17.3.0) has already correctly capture the Koffset. Please have a double check. Regarding the following update, I don’t think it is necessary. Subframe n-k_1 is the most recent subframe with n. if subframe n - k1 - Koffset is the most recent subframe for which subframe n is the 'HARQ-ACK transmission subframe', |
| ZTE | Support |  |
| Nokia, NSB | Support |  |
| SONY | Support | Referring to Lenovo’s comment about v17.3.0, there are two places that refer to ce-PDSCH-14HARQ-Config: For a BL/CE UE with higher layer parameter ce-PDSCH-14HARQ-Config not configured, for PDSCH transmission in subframe n-k-Koffset, if the UE is in half-duplex FDD operation and is configured with CEModeA and higher layer parameter ce-HARQ-AckBundling and the 'HARQ-ACK bundling flag' in the corresponding DCI is set to 1, or if the UE is configured with higher layer parameter ce-SchedulingEnhancement, - if the 'HARQ-ACK delay' field in the corresponding DCI indicates value k, the UE shall determine the subframe n as the HARQ-ACK transmission subframe. - the HARQ-ACK delay value k is determined from the corresponding DCI based on the higher layer parameters according to Table 7.3.1-2. For a BL/CE UE with higher layer parameter ce-PDSCH-14HARQ-Config configured, for PDSCH transmission in subframe n-k-K_offset, if the UE is in half-duplex FDD operation and is configured with CEModeA, and 'PDSCH scheduling delay and HARQ-ACK delay for 14 HARQ' field is present in the corresponding DCI, - if the HARQ-ACK delay value as defined in [4], in the corresponding DCI indicates value k, the UE shall determine the subframe n as the HARQ-ACK transmission subframe. The blue text is the “not configured” case. We agree that “Koffset” is correctly captured. The green case is the “configured” case and we have the incorrect “K_offset” (cyan). We understand this issue refers to the “configured” case and hence a correction is needed. |
| Sequans | Support |  |
| Ericsson | Support |  |

### SECOND ROUND Discussion Issues# 1-8: Missing Koffset in Clause 7.3.1 of TS36.213

Of the 7 responding companies, 6 support the proposal and one (Lenovo) does not.

FL has checked the latest version of the spec (TS36.213 v17.3.0) as suggested by Lenovo and can confirm as Sony also did that there is indeed one case that still uses the incorrect ‘K_offset’. Regarding the second point in Lenovo’s objection, RAN1 previously agreed that the timing relationship between the reception of a PDSCH and the transmission of its HARQ-ACK shall be enhanced by delaying the HARQ-ACK transmission by Koffset subframes.

This means that if a PDSCH is received in subframe n, then its HARQ-ACK is transmitted in subframe n+k+Koffset where k is the HARQ-ACK delay value indicated in the DCI that scheduled the PDSCH. It therefore follows that any timing relationship enhanced HARQ-ACK that is being transmitted in subframe n, must be for a PDSCH that was received in subframe n-k-Koffset. This is what the TP is about.

FL hopes this is an adequate explanation for Lenovo. Accordingly, FL will propose this TP for endorsement at the checkpoint.

FL Proposal 1-8-1: Adopt TP3 for Clause 7.3.1 of  TS 36.213.

TP2 related to Clause 7.3.1 of TS36.213.

Reason for change:

- Specify the correct function

Summary of change:

- Take timing relationship enhancement in index of PDSCH subframe.

Consequence if not approved:

- Wrong specification

-------------------------------------------------- Text Starts --------------------------------------------------------

7.3.1	FDD HARQ-ACK reporting procedure

For FDD with PUCCH format 1a/1b transmission, when both HARQ-ACK and SR are transmitted in the same sub-frame/slot, a UE shall transmit the HARQ-ACK on its assigned HARQ-ACK PUCCH format 1a/1b resource for a negative SR transmission and transmit the HARQ-ACK on its assigned SR PUCCH resource for a positive SR transmission.

---------------------------------------------------Text Omitted -----------------------------------------------------

For a BL/CE UE with higher layer parameter ce-PDSCH-14HARQ-Config configured, for PDSCH transmission in subframe n - k - K_offset Koffset, if the UE is in half-duplex FDD operation and is configured with CEModeA, and 'PDSCH scheduling delay and HARQ-ACK delay for 14 HARQ' field is present in the corresponding DCI,

-	if the HARQ-ACK delay value as defined in [4], in the corresponding DCI indicates value k, the UE shall determine the subframe n as the HARQ-ACK transmission subframe.

For a BL/CE UE in half-duplex FDD operation, if the UE is configured with CEModeA, and if the UE is configured with higher layer parameter ce-HARQ-AckBundling and the 'HARQ-ACK bundling flag' in the corresponding DCI is set to 1,

-	for HARQ-ACK transmission in subframe n, the UE shall generate one HARQ-ACK bit by performing a logical AND operation of HARQ-ACKs across all  BL/CE DL subframes for which subframe n is the 'HARQ-ACK transmission subframe'.

-	if subframe n - k1 - Koffset is the most recent subframe for which subframe n is the 'HARQ-ACK transmission subframe', and if the 'Transport blocks in a bundle' field in the corresponding DCI for PDSCH transmission in subframe n - k1 - Koffset indicates a number of transport blocks in a bundle other than , the UE shall generate a NACK for HARQ-ACK transmission in subframe n.

-	if the UE has received W PDSCH transmissions before subframe n, and if the UE is expected to transmit HARQ-ACK for the W PDSCH transmissions in subframes , the UE is not expected to receive a new PDSCH transmission in subframe n, where W=10 if higher layer parameter ce-pdsch-tenProcesses-config is set to 'On', W=12 if higher layer parameter ce-PDSCH-14HARQ-Config is configured, and W=8 otherwise.

-	if the UE is expected to transmit HARQ-ACK for the PDSCH transmissions received before subframe n in subframes , the UE is not expected to receive a new PDSCH transmission in subframe n for which the HARQ-ACK is to be transmitted in subframe

-------------------------------------------------- Text Ends ---------------------------------------------------------

# Referenced Documents

R1-2208689	Corrections on timing relationship for IoT-NTN, 	ZTE

R1-2209650	On SIB accumulation and Timing relationship enhancements in IoT NTN	Ericsson Limited

R1-2210070	DRAFT CR Missing Koffset in FDD HARQ-ACK reporting procedure	Ericsson

R1-2210219	Corrections on timing relationship parameter for IoT NTN	Huawei, HiSilicon

R1-2210201	Corrections on NPDCCH monitoring restriction for IoT NTN	Huawei, HiSilicon

R1-2210434	Summary of [110bis-e-R17-IoT-NTN-01] Email discussion to determine maintenance issues to be handled in RAN1#110bis-e, 				MediaTek