3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2210328

e-Meeting, 10th – 19th October, 2022

Agenda Item:	8.2

Source:	Moderator (Ericsson)

Title:	FL Summary for AI 8.2 – Enhancements for PUCCH Formats 0/1/4

Document for:	Discussion, Decision

# 1	Introduction

This document summarizes contributions under AI 8.2 related to Enhancements for PUCCH formats 0/1/4 in the Rel-17 work item "Extending current NR operation to 71 GHz."

| Issue# | Issue | References | FL initial assessment |
| --- | --- | --- | --- |
| 8.2.3-1 | RRC parameter name alignment | [1] | Handle in editor alignment CR |

# 2	RRC parameter name alignment for cell-specific PUCCH

A draft CR is provided in [1] to correct an RRC parameter misalignment between 38.213 and 38.331. The draft CR is copied into Appendix A of this summary for convenience. The rationale provided in [1] is as follows:

| Reason for change: | The RAN1 specifies multi-PRB PUCCH for format 0/1/4, the RRC parameter nrofPRBs provided in PUCCH-ConfigCommon in TS 38.331 indicates the number of PRBs for common PUCCH, and the RRC parameter pucch-ResourceCommon indicates an index to a row of Table 9.2.1-1 in TS 38.213. However, in the current spec 38.213, the parameter pucch-ResourceCommon is used to indicate the PRB number of common PUCCH which is the incorrect. |
| --- | --- |
| Summary of change: | Change the parameter used to indicate the number of PRBs of common PUCCH, from “pucch-ResourceCommon” to “nrofPRBs provided in PUCCH-ConfigCommon”. |
| Consequences if not approved: | Incorrect parameter to indicate the number of PRBs of common PUCCH, UE cannot acquire the correct common PUCCH resource. |

It is the moderator's view that this correction should be handled in the editor alignment CR for 38.213.

# References

R1-2208596	Correction on the parameter indicating  the number of PRBs for common PUCCH	vivo

# Appendix A – Draft CR from [1]

*** Unchanged text omitted ***

### 9.2.1	PUCCH Resource Sets

If a UE does not have dedicated PUCCH resource configuration, provided by PUCCH-ResourceSet in PUCCH-Config, a PUCCH resource set is provided by pucch-ResourceCommon through an index to a row of Table 9.2.1-1 for transmission of HARQ-ACK information on PUCCH in an initial UL BWP of NBWPsize PRBs. For operation in FR2-2, nrofPRBs provided in PUCCH-ConfigCommon can also provide a number of NRB RBs for the PUCCH resource set; otherwise NRB=1.

*** Unchanged text omitted ***