3GPP TSG-RAN WG1 #110bis-e	  R1-2210373

e-Meeting, October 10 – 19, 2022

Agenda Item:	8.12

Source:	Moderator (Huawei)

Title:	FLS#2 on the HARQ-ACK related issues for Rel-17 NR MBS

Document for:	Discussion and Decision

# Introduction

The FLS in R1-2210371 documents the discussion in the preparation phase to determine maintenance issues to be handled in RAN1#110bis-e, where the conclusion of the discussion are summarized in section 3.

This FLS will discuss the HARQ-ACK related issues for Rel-17 NR MBS based on the following assignment:

[110bis-e-R17-MBS-02] Email discussion for maintenance on mechanisms to improve reliability for RRC_CONNECTED UEs for the following issues in R1-2210371 – Jinhuan (Huawei)

- Issues 1-1, 1-2, 1-3, 1-4, 1-5, 1-6, 1-7 (including whether case 2 and case 3 with NACK-only mode2 is supported in Rel-17), 1-8, 1-9, 1-10, 1-11, 1-12, 1-21

- Editorial/alignment issues for providing to spec editors: 1-14, 1-15

- Discuss for clarification of the issue (potentially discuss CR at RAN1#111, or conclude at RAN1#110bis-e that the issue is not essential): 1-16, 1-17, 1-18, 1-19, 1-20

- Discuss a potential conclusion (without CR) for issue 1-22

- Check points: October 14, October 19.

# Issues to be discussed

For convenience, the issues to be discussed in this meeting are summarized in Table 1 in this section for reference.

Table 1: Summary of HARQ-ACK related issues

| Issue# | Issue | References |
| --- | --- | --- |
| 1-1 | PRI for NACK-only mode2 FL Note: sources have different views on whether PRI is interpreted. Need to first clarify whether moreThanOneNackOnlyMode is only applicable to the case when more than one TB is scheduled. | vivo-CR-x08618, Nokia-TP-x08701, CMCC-Dis-x09310, ZTE-CR-x09475, Apple-Dis-x09566, Huawei-CR-x08468, Huawei-Dis-x09822 Samsung-Dis-x09708, Ericsson-Dis-x10173 |
| 1-2 | Codebook type for NACK-only FL Note: Discuss whether type1 CB can be configured if UE is configured with NACK-only. | Samsung -Dis-x09708, Qualcomm-CR-x09955, Huawei-CR-x08467, Huawei-Dis-x09822, Ericsson-Dis-x10173 |
| 1-3 | HARQ-ACK feedback for 4_1 when configured “dci-enabler” FL Note: clarify UE behavior whether generate HARQ-ACK for DCI 4_1 when UE is configured with “dci-enabler”. | vivo-CR-x08617, CATT-CR-x08928, Lenovo-CR-x10156 |
| 1-4 | Timeline for NACK-only FL Note: Tdocs from UE vendors point to a serious implementation issue. | ZTE-Dis-x09470, MediaTek-CR-x09526, Samsung-Dis-x09708, Qualcomm-CR-x09959, Huawei-CR-x08466, Huawei-CR-x09832, |
| 1-5 | PUCCH resources for NACK-only multicast SPS FL Note: FFS from previous meeting and spec change is needed. | Nokia-TP-x08701, ZTE-Dis-x09470, Samsung-TP-x09708, Huawei-Dis-x09822 |
| 1-6 | Type1 CB and “dci-enabler” not configured simultaneously FL Note: This conclusion should be the common understanding but whether spec change is needed can be discussed to collect views. | Qualcomm-CR-x09954, Lenovo-CR-x10158 |
| 1-7 | NACK-only mode2 for case2 and case3 | Nokia-TP-x08701, NEC-Dis-x09137, CMCC-Dis-x09310, ZTE-Dis-x09470, MediaTek-Dis-x09527, Apple-Dis-x09566, Samsung-TP-x09708, DOCOMO-CR-x09884, Lenovo-Dis-x10159, Ericsson-Dis-x10173 |
| 1-8 | Whether DAI should count the PDCCH with “dci-enabler” indicating value 0— if not counted, the spec for Type2 CB needs update for the DAI description. | Langbo-CR-x08995, Lenovo-CR-x10157 |
| 1-9 | Type2 CB for multicast SPS—38.213 states UE separately applies the procedures in this clause per G-RNTI or per G-CS-RNTI, but the clause also includes SPS PDSCHs. How to generate Type2 CB for multicast SPS PDSCH is unclear FL Note: FL agrees that it may cause confusion. The discussion in this meeting can focus on whether change is needed. | vivo-CR-x08619 |
| 1-10 | the current description only allows to configure one HARQ-ACK codebook for multicast-- pdsch-HARQ-ACK-Codebook is replaced by the relevant entry in pdsch-HARQ-ACK-CodebookListMulticast-r17 FL Note: aligning with TS38331 should be easily agreeable for this meeting | CMCC-CR-x09312 |
| 1-11 | Change “UE is provided moreThanOneNackOnlyMode” to “UE is not provided moreThanOneNackOnlyMode” FL Note: aligning with TS38331 should be easily agreeable for this meeting | vivo-CR-x08618, Huawei-CR-x08468 |
| 1-12 | Multiplexing one unicast SPS and DG multicast-- gNB wouldn’t be able to schedule multicast PDSCHs to transmit HARQ-ACK in the same slot with unicast SPS HARQ-ACK if UE does not support more than one SPS configuration | vivo-CR-x08887 |
| 1-13 [POSTPONED] | PTP retrans for NACK-only when applicable—when NACK-only is converted into ACK/NACK, PTP retrans can be applied. NACK-only taking PTM retrans is too restrictive. | Huawei-CR-x10207 |
| 1-14 | Delete the redundant descriptions of “if pdsch-HARQ-ACK-Codebook-Multicast = semiStatic is configured” to 38212. FL Note: avoid unnecessary checking whether other meaning is implied by the redundant description and easily agreeable for this meeting | Lenovo-CR-x10155 |
| 1-15 | Change “pdsch-HARQ-ACK-Codebook-Multicast” to “pdsch-HARQ-ACK-Codebook”. FL Note: editorial but easily agreeable for this meeting | CMCC-CR-x09312 |
| 1-16 | pdsch-HARQ-ACK-retx for multicast- pdsch-HARQ-ACK-retx can include the HARQ-ACK feedback for multicast | CATT-CR-x08925, |
| 1-17 | spsHARQdeferral for multicast SPS-- whether RRC parameter IE spsHARQdeferral can be used for multicast SPS configuration | CATT-CR-x08926, |
| 1-18 | Type3 for NACK-only mode-- For generating a Type-3 HARQ-ACK codebook with 〖NDI〗_HARQ=1, the UE considers the HARQ-ACK information corresponding to a PDSCH reception has been reported if a PUCCH was not transmitted due to all values of HARQ-ACK information including the one for the PDSCH reception being ‘ACK’ according to the second HARQ-ACK reporting mode. | Langbo-CR-x08996 |
| 1-19 | - NACK-only mode2 multiplexed with other UCI/PUSCH is supported FL Note: supported but the current spec suffices? - SR and ack/nack based multiplexing with 1bit for unicast and 1 bit for multicast. FL Note: how to change spec? - HARQ-ACK multiplexing for u-cast and m-cast is per priority and then applying the intra-UE multiplexing FL Note: how to change spec? | LGE-Dis-x09449 |
| 1-20 | UE behavior for disabled HARQ for NTN multicast needs to clarified. FL Note: for UE supporting both NTN and multicast. | Qualcomm-CR-x09960 |
| 1-21 | UE will not transmit NACK-only PUCCH when the HARQ-ACK information is ‘ACK’. This is missing for NACK-only mode1?? | ZTE-CR-x09476 |
| 1-22 | Multiplexing NACK-only and SR FL Note: open issue has been discussed for several times. As clarified, if no consensus for the solution, it means not support the multiplexing and it is up to UE so UE behavior is not needed. | Nokia-TP-x08701, NEC-Dis-x09137, ZTE-Dis-x09470, MediaTek-Dis-x09527, DOCOMO-CR-x09885 |

# Discussions for the issues

## (1-1)PRI for NACK-only mode2

| vivo-CR-x08618, | Proposal: When UE is configured with the second HARQ-ACK reporting mode, for the case with 1 TB with NACK-only, regardless that UE is configured with NACK-only mode 1 or mode 2, the UE provides the HARQ-ACK information according to the second HARQ-ACK reporting mode. - Regardless that UE is configured with NACK-only mode 1 or mode 2, PRI indicates the PUCCH for the HARQ-ACK feedback including the case of one TB in scheduled. |
| --- | --- |
| Nokia-TP-x08701, | When NACK-only based feedback is configured with mode 2 and HARQ-ACK feedback is to be provided for only one TB, UE uses the PRI and CCE-based legacy PUCCH resource selection mechanism. |
| CMCC-Dis-x09310, | Proposal 2. For PRI included in DCI format 4_1/4_2: - if the G-RNTI is configured with NACK-only mode1, PRI indicates the PUCCH for the HARQ-ACK feedback; - if the G-RNTI is configured with NACK-only mode2, PRI is ignored. |
| ZTE-CR-x09475, | The PUCCH resource is indicated by PUCCH resource indicator field in DCI format 4-1/4-2 from the configured PUCCH set 0 with up to 32 PUCCH resources for the second HARQ-ACK reporting mode. |
| Apple-Dis-x09566, | Proposal: If the G-RNTI is configured with NACK-only mode2, PRI is ignored. |
| Huawei-CR-x08468, Huawei-Dis-x09822 | Proposal 1: For PRI included in DCI format 4_1/4_2, if the G-RNTI is configured with NACK-only mode2, PRI will be ignored by UEs. |
| Samsung-Dis-x09708, | There is no practical benefit from using the PRI in DCI format 4_1/4_2 for NACK-only mode 2 and current specifications suffice. |
| Ericsson-Dis-x10173 | For NACK-only mode2, the PRI in the DCI is interpreted by UEs in NACK-only mode to select one of the PUCCH resource pools configurations for NACK-only. |

### (CLOSED)Round-1(Pro)

FL’s analysis:

The following was agreed:

Agreement

For PRI included in DCI format 4_1/4_2,

- if the G-RNTI is configured with NACK-only mode1, PRI indicates the PUCCH for the HARQ-ACK feedback including the case of one TB in scheduled.

- If the G-RNTI is configured with NACK-only mode2,

  - FFS on whether/how to interpret the PRI.

Some companies (vivo, Nokia) view that unified solution for one TB is preferred for both NACK-only mode1 and mode2, for both of which one TB can be possibly scheduled. Vivo also mentioned that the configuration of moreThanOneNackOnlyMode is not applicable to one TB. The RRC parameter name moreThanOneNackOnlyMode may be confusing, but it is FL’s understanding that whether one TB or more than one TB scheduled is dynamically changing per scheduling, which could happen despite moreThanOneNackOnlyMode configured or not. Moreover, when one UE receives only one TB, it does not mean only one TB is transmitted by gNB.

DCI missing could happen commonly to NACK-only mode1 and mode2. However, it should be noted that DCI missing is UE specific issue so the group-common DCI/PDSCH missing could happen to one but may not happen to the other UEs. From network perspective, if more than one TB is scheduled to feedback HARQ-ACK in the same PUCCH in NACK-only mode1, network can set a proper PRI in the group-common DCI which will be individually mapped to UE specific PRI and the associated PUCCH resource.

However, if it is NACK-only mode2, for other UEs not missing DCI, PUCCH resource selection will depend on UE decoding result. It is not predictable for network to set a proper PRI in the group-common DCI which may be missed by some UE because the indicated PUCCH is expected to be shared by all the UEs. In addition, the mapping table between HARQ-ACK values and the PUCCH resources is tolerable to DCI missing and the missed TB/PDSCH can be retrieved as long as not all UEs missed the same TB/PDSCH. Therefore, for NACK-only mode2, it is not practically easy to use PRI indicating the PUCCH resource for the case of one TB scheduled due to the DCI missing issue.

#### Proposal 3.1.1

For PRI included in DCI format 4_1/4_2, if the G-RNTI is configured with NACK-only mode2, PRI will be ignored by UEs.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Support |
| Qualcomm | Assuming case 2/3 are not supported, PRI can be ignored by UE. |
| vivo | We want to clarify whether the parameter moreThanOneNackOnlyMode is applicable for the one TB case first. Note that during pervious discussion, there is no issue for NACK-only with one TB in the beginning, i.e., UE transmits the PUCCH when it is NACK and does not transmit the PUCCH when it is ACK. Then some issues raised for the case with more than one NACK only, then we made discussion and the following agreements was made. In addition, the description of the latest TS 38.331 for moreThanOneNackOnlyMode and the description in TS 38.213-h10, it is clearly that moreThanOneNackOnlyMode is only for the case with more than one TB. My question is when we did we make the agreement to apply this parameter for the one TB case? If moreThanOneNackOnlyMode is applied for all TB cases, if UE is configured with NACK-only mode 1, for the case of one TB, does UE still need to transform NACK-only to ACK/NACK? I think there is no need for the UE do have such transform. Agreement (RAN1#108-e) For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements): - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. - FFS: how to determine PUCCH resource - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission. - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast. - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID. - FFS mapping details. - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook. - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs - Alt4 is not supported for more than 4 TBs - FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR - FFS: UE capability moreThanOneNackOnlyMode Indicates the mode of supporting more than one NACK-only feedback in the same PUCCH transmission. Mode 1 means UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. Mode 2 means UE transmitting a specific sequence or a PUCCH transmission corresponding to the combination of more than one NACK-only HARQ feedback. If multicast CFR is not configured, this field is not included. Otherwise, if the field is absent, UE uses mode 1 for multicast CFR. TS 38.213-h10 For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is 2, 3, or 4, the UE can be indicated by moreThanOneNackOnly-Mode to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode or by selecting a resource from a set of resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1. |
| NTT DOCOMO | Support |
| ZTE | For Case 1, the network does not need to indicate the PUCCH resource since the PUCCH resource is determined by the PDSCH decoding. We prefer to limit this proposal to Case 1. For Case 2 and Case 3, the PRI can be discussed separately. |
| MTK | The initial purpose of introducing the NACK only mode 2 is trying to resolve the more than one NACK only bits in one PUCCH slot, so, from this perspective, we have the similar view with vivo. Another question needs to be clarified that whether the normal one TB scheduling for NACK only and more than one TB scheduling for NACK only will be scheduling in TDMed manner as illustrated in the following figure? If the answer is yes, which PUCCH determination mechanism will be used for more than one NACK only case if UE only the one TB and miss other three TBs when two different PUCCH determination mechanism are defined? If the answer is no, we think the current spec can work even if some IE naming cause a little confusing. |
| CATT | Support. |
| ZTE2 | For clarification, our CR-x09475 is actually to capture the previous agreement in the spec for NACK-only mode 1 when only one PDSCH is scheduled. For NACK-only mode 1, it was agreed that the NACK-only PUCCH resource is indicated by the PRI in case one PDSCH is scheduled. This should be captured in the specification so that the NACK-only PUCCH resource determination is clear in this case. Agreement(RAN1#107bis-e) For the separate PUCCH-Config/ PUCCH-ConfigurationList configured to UE for NACK-only based feedback, - 1 PUCCH resource set in each PUCCH-Config. - up to 32 PUCCH resources in PUCCH resource set - Note: the separate PUCCH-Config/PUCCH-ConfigurationList applies to all configured G-RNTIs configured with NACK-only based feedback. Agreement(RAN1#110) For PRI included in DCI format 4_1/4_2, - if the G-RNTI is configured with NACK-only mode1, PRI indicates the PUCCH for the HARQ-ACK feedback including the case of one TB in scheduled. - If the G-RNTI is configured with NACK-only mode2, - FFS on whether/how to interpret the PRI.. The following CR is proposed. For the second HARQ-ACK reporting mode, if moreThanOneNackOnlyMode is not provided, when a number of HARQ-ACK information bits is one, a UE transmits a PUCCH only when the HARQ-ACK information bit has NACK value. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. The PUCCH resource is indicated by PUCCH resource indicator field in DCI format 4-1/4-2 from the configured PUCCH set 0 with up to 32 PUCCH resources for the second HARQ-ACK reporting mode.. |
| Nokia, NSB | We are ok to compromise at this late stage. Although, in our view, there is no technical issue of missing the DCI more than any other use cases of PRI, as anyway DAI is used for this purpose in the DCI. Therefore, the usage of PRI in NACK-only mode 2 for one TB would bring more flexibility on PUCCH resource selection. To clarify vivo, for NACK-only mode 1, if the UE receives only one TB, it shall use the PRI and not convert NACK-only to ACK/NACK feedback. The conversion only applies if the feedback is scheduled in the same PUCCH for more than 1 TBs. Otherwise, this would be ACK/NACK feedback, rather than NACK-only. |

### Round-2(CR)

Agreement

For PRI included in DCI format 4_1/4_2, if the G-RNTI is configured with NACK-only mode2, PRI will be ignored by UEs.

This round discusses the moderator draft CR. To FL’s understanding, even though we agreed that PRI will be ignored for NACK-only mode2, the current spec suffices (i.e., when UE is provided moreThanOneNackOnlyMode, PUCCH is selected based on Table18-1) and the part missing is using PRI as described in clause 9.2 for NACK-only mode1, i.e., when UE is not provided moreThanOneNackOnlyMode.

#### Draft CR 3.1.2

The draft CR in Moderator draft CR on issue 1-1_v001_Mod (to be replaced by the link to the draft CR in the inbox) is endorsed.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | We understand the intention to be that, for the case of 1 NACK-only bit, the UE uses the PUCCH resource from the table. However, the CR conditions that on moreThanOneNackOnlyMode being provided which is not necessary and can lead to unintended UE behavior. Suggest to: - Separate the statements about moreThanOneNackOnlyMode and about resource selection for “NACK-only” into separate paragraphs to avoid possible confusion as the issues are different (was the editor’s mistake). Also, either clarify that moreThanOneNackOnlyMode is applicable for >1 bit or (preferably to minimize changes) add a reference to 38.331. - Capture that Table 18-1 applies when a UE would transmit a PUCCH with “NACK-only”. “A UE that is indicated the second HARQ-ACK reporting mode for only one G-RNTI can be indicated by moreThanOneNackOnlyMode to provide associated HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode or according to the second HARQ-ACK reporting mode [12, TS 38.331]. When a UE would transmit a PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode, the UE by selectsing a PUCCH resource from a set of PUCCH resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1. |
| Qualcomm | This is same discussion as in 1-11. To align with 38.331, moreThanOneNackOnlyMode Indicates the mode of supporting more than one NACK-only feedback in the same PUCCH transmission. Mode 1 means UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. Mode 2 means UE transmitting a specific sequence or a PUCCH transmission corresponding to the combination of more than one NACK-only HARQ feedback. If multicast CFR is not configured, this field is not included. Otherwise, if the field is absent, UE uses mode 1 for multicast CFR. When moreThanOneNackOnlyMode is not configured, it is NACK-only mode1 for second HARQ reporting mode. So, we suggest the following TP for 38.213 (TS 38.212 is a typo in the draft CR): "A UE that is indicated the second HARQ-ACK reporting mode for only one G-RNTI can be indicated by moreThanOneNackOnlyMode to provide associated HARQ-ACK information bits in a PUCCH by selecting a PUCCH resource from a set of PUCCH resources for the PUCCH transmission based on the values of the HARQ-ACK information bits is configured as described in Table 18-1. A UE that is indicated the second HARQ-ACK reporting mode but not configured with moreThanOneNackOnlyMode is to provide associated HARQ-ACK information bits in a PUCCH or PUSCH according to the first HARQ-ACK reporting mode if colliding with other PUCCH/PUSCH" To clarify that the G-RNTI/G-CS-RNTI configured with second HARQ-ACK reporting mode can have mode1 or mode2. It may be misleading that only mode2 is second HARQ-ACK reporting mode. Our understanding is mode1 is to transform the second reporting mode into the first HARQ-ACK reporting mode. |
| vivo | We think Samsung suggestion is fine. It is preferred to clarify that moreThanOneNackOnlyMode is applicable for >1 bit, it may be not a good choice to change 331. |
| Nokia, NSB | Agree with Qualcomm’s view. In our understanding, if the UE is configured with the second HARQ-ACK reporting mode (NACK-only HARQ-ACK feedback) with mode 1 (converting NACK-only to ACK/NACK for more than 1 NACK-only feedback), the UE still uses the PRI field if there is only 1 NACK-only feedback to be transmitted (and uses the PRI also for the case of multiple NACK-only feedback converted into ACK/NACK, but then selects a resource from unicast PUCCH-config). |
| MTK | From our understanding, for the NACK only mode 1, the UE is to determine the PUCCH resource based on the PRI field, and for the NACK only mode 2, the UE will determine the PUCCH resource based on the agreed table 18-1 in TS 38.213 h30, which also include the one TB case, e.g., the UE only receive one TB even if the gNB schedule more than one TBs, it can also use the table 18-1 to choose the PUCCH resource to feedback the HARQ-ACK information. |
| ZTE | With the agreement, we prefer the understanding that the parameter moreThanOneNackOnlyMode is also applied to 1 bit because the UE behavior is different in the two cases, i.e., the PRI indicates the PUCCH resource for NACK-only in NACK-only mode 1 and is ignored by the UE in NACK-only mode 2. |
| NTT DOCOMO | If moreThanOneNackOnlyMode is applied even when the HARQ-ACK is 1 bit, then mode 1 would be the same method as ACK/NACK based feedback, and it would make no sense to use mode 1. |

## (1-2)CB type for NACK-only

| Samsung-Dis-x09708, | Proposal 1: Consider simultaneous configuration of Type-1 HARQ-ACK codebook and NACK-only mode 2 as a misconfiguration; if not agreeable, conclude on Alt1 from Proposal 2.3.2 of [1] and Alt2 from RAN1#108e agreement. |
| --- | --- |
| Qualcomm-CR-x09955, | the UE is not expected to be configured with Type-1 HARQ-ACK codebook for multicast HARQ-ACK information if a G-RNTI/G-CS-RNTI is indicated by using second HARQ-ACK reporting mode |
| Huawei-CR-x08467, Huawei-Dis-x09822 | Proposal 3: When UE is configured with NACK-only mode2, UE is not expected to be configured with Type-1 codebook for multicast and then configuration of fdmed-ReceptionMulticast, or type1-Codebook-GenerationMode is not applicable accordingly. |
| Ericsson-Dis-x10173 | When pdsch-HARQ-ACK-CodebooklistMulticast indicates Type-1 CB, fdmed-ReceptionMulticast or type1-Codebook-GenerationMode is configured, for NACK-only in mode2, - Alt1: the configured parameters are used for multiplexing NACK-only mode2 with other UCI or PUSCH according to the corresponding agreements |

### (CLOSED)Round-1(Pro)

FL’s analysis:

ACK/NACK based or NACK-only based HARQ-ACK feedback is configured per G-RNTI. For NACK-only based feedback, mode1 (converting into ACK/NACK) or mode2 (PUCCH channel selection) is configured per CFR. The pdsch-HARQ-ACK-CodebooklistMulticast, fdmed-ReceptionMulticast, or type1-Codebook-GenerationMode is configured per UE for multicast.

When UE is configured with a G-RNTI with NACK-only mode2, the question is whether pdsch-HARQ-ACK-CodebooklistMulticast, fdmed-ReceptionMulticast, or type1-Codebook-GenerationMode is configured or when configured how to interpret the configurations.

For NACK-only mode2, one of 15 PUCCH resources will be chosen based on the decoding result for the HARQ-ACK transmission and counting/ordering the HARQ-ACK bits is based on C-DAI. In addition, when NACK-only feedback collides with other UCI or PUSCH, the NACK-only is converted into ACK/NACK feedback for multiplexing. If NACK-only mode2 only supports Type-2 codebook, when UE multiplexes with other UCI or PUSCH, the codebook for NACK-only mode2 should still be based on Type-2. Otherwise, from UE implementation perspective, dropping Type-2 codebook and regenerating codebook based on Type-1 to multiplex HARQ-ACK for unicast or PUSCH is more complex and may require additional time.

Since multiplexing different codebook types for unicast and multicast is supported, despite Type-1 or Type-2 codebook for unicast, it is supported to multiplex HARQ-ACK feedback for unicast and NACK-only mode that is always based on Type-2 codebook. If UE is configured with multiple G-RNTIs, given the pdsch-HARQ-ACK-CodebooklistMulticast configuration is applied to all configured G-RNTIs, UE is expected to be configured with Type-2 codebook as long as one G-RNTI is configured with NACK-only mode2. Otherwise, it may happen that NACK-only mode2 alone based on Type-2 will be dropped and a Type-1 codebook will be regenerated instead for multiplexing Type-1 codebook for the other G-RNTI(s).

It seems agreeable to take simultaneous configuration of Type-1 HARQ-ACK codebook and NACK-only mode 2 as a misconfiguration and Qualcomm also views that Type-1 HARQ-ACK codebook should not be configured together with NACK-only mode1 either.

#### Proposal 3.2.1

Which one is agreeable from the following:

- Alt1: UE is not expected to be configured with Type1 CB for NACK-only.

- Alt2: UE is not expected to be configured with Type1 CB for NACK-only mode2.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Alt1 – it is simpler which is preferable at this stage and for that issue. |
| Qualcomm | Alt1 |
| vivo | Alt 1 |
| NTT DOCOMO | We prefer Alt1. |
| ZTE | We don’t support this proposal. In our understanding, the configured HARQ-ACK codebook type is only applied to the G-RNTI configured with ACK/NACK even though these parameters are configured per CFR or per UE because more than one G-RNTIs can be configured with ACK/NACK feedback. When NACK-only is transferred to ACK/NACK, type-2 codebook is generated. According to the current spec, the type-2 codebook transferred from NACK-only is concatenated with Type-1 codebook. We don’t see any issue for this case. If type-1 CB and NACK-only can be enabled, this can bring more flexibility for the network. It should not be excluded. |
| MTK | Alt 1 |
| CATT | Alt 2 Actually, we don’t see any issue that configured Type-1 CB for NACK-only mode1. Thus, we support Alt.2. |
| Nokia, NSB | We do not support and agree with ZTE’s understanding. The issue has already been clarified with previous discussions. No dropping occurs, rather NACK-only Type-2 codebook is concatenated with Type-1 codebook. Also, the wording of the proposal is not good. The UE does not get configure Type-1 codebook for NACK-only at all. |

### Round-2(Pro)

FL’s analysis:

Based on the discussion in the 1st GTW session on Oct.12, companies are encouraged to further check the background and why there is an issue.

Firstly, the following agreement was reached in RAN1#108-e:

Agreement

For multiplexing NACK-only feedback for the first G-RNTI with ACK/NACK based feedback for the second G-RNTI or for unicast, down-select from:

- Alt1: the converted NACK-only bits are concatenated to the ACK/NACK feedback.

- Alt2: the codebook construction/concatenation is the same as when the feedback mode is ‘ACK/NACK’ for both G-RNTIs.

Based on the above agreement, when UE is configured with Type-1 HARQ-ACK codebook (note that pdsch-HARQ-ACK-CodebooklistMulticast is configured per UE for multicast despite G-RNTI(s) and this codebook type is applied to both PUCCH and PUSCH), UE should convert NACK-only into ACK/NACK based on Type-1 HARQ-ACK codebook.

Secondly, the issue now comes from:

- When NACK-only is transmitted on PUCCH without multiplexing with other UCI or PUSCH, the HARQ-ACK is based on Type-2 codebook.

- If UE is configured with Type-1 codebook for multicast, when multiplexing the HARQ-ACK onto PUSCH, UE would drop the generated Type-2 CB and regenerate the HARQ-ACK based on Type-1 CB for the multiplexing. Which is challenge for UE implementation within the current timeline and causes unnecessary complexity to UE.

For solving this problem, the proposed solution is for UE is configured NACK-only feedback, UE does not expect to be configured with Type-1 codebook.

Technically, NACK-only mode1 does not have such issue because Type-1 CB is applicable to NACK-only mode1. However, NACK-only mode1 or mode2 is configured per G-RNTI, but codebook type for multicast is per UE. Therefore, as long as G-RNTI is configured with NACK-only mode2, CB type is not expected to be Type-1, so the simpler solution is not configuring Type1 CB and NACK-only in general simultaneously. Otherwise, the issue still exists when to multiplex G-RNTI1 NACK-only mode 1 Type1 CB and G-RNTI2 NACK-only mode 2 Type2 following the above agreement.

Considering the wording suggestion from Round-1, the proposal is updated as follows:

#### Proposal 3.2.2

For UE provided NACK-only mode2 HARQ-ACK feedback for G-RNTI(s)/G-CS-RNTI(s), UE does not expect to be provided Type-1 HARQ-ACK codebook for multicast.

Company views:

| Company | Comments |
| --- | --- |
| MTK | From our understanding, if UE is configured with NACK only mode 1, the UE will transform the NACK-only into ACK/NACK bits no matter whether the NACK only based HARQ-ACK bit multiplex with other UCIs. So, we can live with the proposal even if our first preference is previous proposal. |
| Samsung | OK |
| Nokia, NSB | We can compromise if the FL can clarify why the following scenarios are a challenge for the current UE implementation: (1) If the UE is scheduled to provide NACK-only feedback with mode 2 and a PUSCH transmission, even if Type-1 codebook is configured for ACK/NACK-based feedback, the UE can create a Type-2 codebook and multiplex it in PUSCH (sent with data only, i.e. no ack/nack feedback) . (2) if the UE is scheduled to provide NACK-only feedback with mode 2 and another HARQ transmission with Type-1 codebook with ACK/NACK, the UE can generate the single combined Type-1 codebook (as in case for two multiplexed ACK/NACK-based feedback for 2 G-RNTIs configured with Type-1 codebook). |
| Qualcomm | Not support, it seems majority companies prefer this based on the previous discussion. #### Proposal 3.2.2 For UE provided NACK-only mode2 HARQ-ACK feedback for G-RNTI(s)/G-CS-RNTI(s), UE does not expect to be provided Type-1 HARQ-ACK codebook for multicast. If moreThanOneNackOnlyMode is not configure, UE only transform NACK-only into A/N when colliding with other PUCCH/PUSCH; otherwise, UE does not transmit ACK. But if UE is configured with Type-1 CB, does UE need to always feedback ACK/NACK to fill the bit(s) in Type-1 CB in this case? It seems meaningless to configure NACK-only feedback to the G-RNTI/G-CS-RNTI and we need more specification to clarify Type-1/Type-2 CB separately. For Nokia’s question: All the G-RNTI/G-CS-RNTI share one CB type. So if one G-RNTI is configured with NACK-only, we assume only Type2 CB is allowed and all the multicast feedback is using Type2 CB. If UE needs to generate different CBs for G-RNTI with mode2 and other G-RNTI without mode2, it complicates the UE implementations. |
| NTT DOCOMO | OK |

## (1-3)HARQ-ACK feedback for DCI 4_1 when configured “dci-enabler”

| vivo-CR-x08617, | If a UE is provided harq-FeedbackEnablerMulticast with value set to ‘dci-enabler’ for a G-RNTI or a G-CS-RNTI, the UE provides HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_1 associated with the G-RNTI or the G-CS-RNTI and determines whether or not to provide the HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_2 based on an indication by the multicast DCI format 4_2 associated with the G-RNTI or the G-CS-RNTI. |
| --- | --- |
| CATT-CR-x08928, | If a UE is provided harq-FeedbackEnablerMulticast with value set to ‘dci-enabler’ for a G-RNTI or a G-CS-RNTI, the UE determines whether or not to provide the HARQ-ACK information for PDSCH receptions based on an indication by the DCI format 4_2 associated with the G-RNTI or the G-CS-RNTI [4, TS 38.212], and provides the HARQ-ACK information for PDSCH receptions scheduled by the DCI format 4_1 associated with G-RNTI or the G-CS-RNTI. |
| Lenovo-CR-x10156 | If a UE is provided harq-FeedbackEnablerMulticast with value set to ‘dci-enabler’ for a G-RNTI or a G-CS-RNTI, the UE provides the HARQ-ACK information for PDSCH receptions scheduled by the multicast DCI format 4-1 associated with the G-RNTI or the G-CS-RNTI, and determines whether or not to provide the HARQ-ACK information for PDSCH receptions based on an indication by the multicast DCI format 4_2 associated with the G-RNTI or the G-CS-RNTI [4, TS 38.212]. |

### (CLOSED)Round-1(CR)

FL’s analysis:

The submitted CRs basically are proposing the similar changes. The harmonized CR is provided by moderator draft CR on this issue.

#### Draft CR 3.3.1

The draft CR in Moderator Draft CR on issue 1-3 (to be replaced by the link to the draft CR in the inbox) is endorsed.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Support. We suggest that all agreed CRs are combined into a single CR as was the case in some other Wis in RAN1#110 for easier tracking. |
| Qualcomm | We have concern on the proposed CRs. For multicast feedback enabling/disabling, the default UE behavior is not to transmit the HARQ-ACK feedback. If a G-RNTI/G-CS-RNTI is configured with dci-enabler, we think the following two alternatives can be considered and we slightly prefer Alt1 for sake of simplicity. Alt1: the UE is not expected to be scheduled by DCI format 4_1 for this G-RNTI/G-CS-RNTI Alt2: the UE is not transmitting the multicast feedback (i.e., fall back to default UE behavior) when scheduled by DCI format 4_1 for this G-RNTI/G-CS-RNTI. |
| Vivo | Support. We think alt1 proposed by Qualcomm is too restricted for gNB’s scheduling. Alt 2 can be considered. But considering UE already supports dynamic indication of enabling/disabling HARQ-ACK, UE supports HARQ-ACK feedback for multicast, defining transmitting the multicast HARQ-ACK can be beneficial for the performance, the behavior defined in the draft CR is preferred. |
| NTT DOCOMO | Support |
| ZTE | We are fine with this proposal that HARQ-ACK is always enabled for the PDSCH scheduled by DCI format 4_1. |
| MTK | Support the draft CR since it clarifies the issue clearer. Regarding QC’s comments, we are also fine with the Alt 1if the gNB can ensure it. |
| CATT | Support. |
| Nokia, NSB | Support the draft. We can also be ok with Alt.2 of Qualcomm, but Alt.1 would bring unnecessary limitations. |

### Round-2(Pro)

FL’s analysis:

Although the moderator draft in Round-1 seems agreeable to the majority, one company commented that for multicast feedback enabling/disabling, the default UE behavior is not to transmit the HARQ-ACK feedback, so UE either is not expected to be scheduled by DCI format 4_1 for this G-RNTI/G-CS-RNTI or the UE is not transmitting the multicast feedback (i.e., fall back to default UE behavior) when scheduled by DCI format 4_1 for this G-RNTI/G-CS-RNTI and the former is preferred.

Before pursuing the CR, it would be more sufficient to align people in the group what UE behavior is expected for the case due to different views in Round-1.

#### Proposal 3.3.2

If a G-RNTI/G-CS-RNTI is configured with ‘dci-enabler’, which alternatives is agreeable from the following and why?

- Alt1: the UE is not expected to be scheduled by DCI format 4_1 for this G-RNTI/G-CS-RNTI

- Alt2: the UE is not transmitting the multicast feedback when scheduled by DCI format 4_1 for this G-RNTI/G-CS-RNTI.

- Alt3: the UE is transmitting the multicast feedback when scheduled by DCI format 4_1 for this G-RNTI/G-CS-RNTI.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Alt3. We think Alt3 is supported by current agreements and another agreement is not necessary. No need for any scheduling restrictions (Alt1) and no need for the UE to not provide HARQ-ACK (Alt2) – if the gNB did not want HARQ-ACK, it could have used DCI format 4_2 and disable it. |
| Qualcomm | We don’t think that Alt3 is supported by current agreements. As the name indicated, ‘dci-enabler’ is used to switch on the feedback in the DCI. Also, if harq-FeedbackEnablerMulticast is not configured, there will be no ‘dci-enabler’ even in DCI format 4_2 and UE does not transmit the feedback as the default behavior. Harq-FeedbackEnablerMulticast-r17 ENUMERATED {dci-enabler,enabled} OPTIONAL, -- Need S harq-FeedbackEnablerMulticast Indicates whether the UE shall provide HARQ feedback for MBS multicast. Value dci-enabler means that whether the UE shall provide HARQ feedback for MBS multicast is indicated by DCI. Value enabled means the UE shall always provide HARQ feedback for MBS multicast. When the field is absent, the value “disabled” is used as defined in TS 38.213 [3]. To align the same default UE behavior, UE does not transmit the feedback for DCI format 4_1. We can compromise to Alt2, although we think Alt1 is simpler. |
| Vivo | Alt 3 is preferred. We don’t think the Alt3 is supported by current agreements. UE behavior for PDSCH scheduled by DCI 4_1 is not clear in this case. Regarding the spec cited by Qualcomm, it says when the field is absent, the value “disabled” is used as defined. However, now the field is not absent, it is configured as “dci-enabler”, but there is no such indication field in DCI 4_1, so default UE behavior needs to be defined. |
| Nokia, NSB | We do not think that Alt1 is needed, as this would bring unnecessary restrictions to scheduling that would impact also other UEs receiving the same multicast service. We can compromise to accept Alt2, however, ideally Alt3 would be preferable as it provides more information to the network to improve link adaptation. |
| MTK | Prefer Alt 3. |
| Samsung2 | We previously incorrectly thought that when RRC indication for DCI enabling/disabling is not provided, the default is ‘enabled’. However, it is the opposite, and we agree that Alt.3 is not currently supported. The RRC parameter structure is unfortunate (same parameter for RRC and DCI enabling/disabling) and does not follow the RAN1 agreement – the specifications now support Alt.2 for DCI format 4_1 (HARQ-ACK enabling by DCI indication is not possible, HARQ-ACK report is not enabled by RRC). We continue to support Alt.3 for functionality but it will have spec impact. |
| ZTE | Alt 3 is preferred. |
| Qualcomm2 | If “the specifications now support Alt.2 for DCI format 4_1 (HARQ-ACK enabling by DCI indication is not possible, HARQ-ACK report is not enabled by RRC)”, why need to change the spec for Alt3? |
| NTT DOCOMO | We prefer either Alt1 or Alt3. |

## (1-4)Timeline for NACK-only

| ZTE-Dis-x09470, | the same transmission power can be obtained as long as the network configures the PUCCH resource for NACK-only with the same RB allocations. Based on the above analysis, for NACK-only PUCCH with PF0 and PF1, it still needs the decoding result of MBS PDSCH to determine the transmitted information in NACK-only PUCCH. We found no need to redefine processing time for NACK-only feedback. |
| --- | --- |
| MediaTek-CR-x09526, | adding d3 to T_proc,1 in TS38214 with value N1 |
| Samsung-Dis-x09708, | There is no need to consider any change to TProc,1. Transmission power determination does not require extending a timeline of PUCCH transmission for NACK-only mode 2. |
| Qaulcomm-CR-x09959 | adding d3 to T_proc,1 in TS38214 with value N2 |
| Huawei-CR-x08466, Huawei-CR-x09832, | adding d3 to T_proc,1 in TS38214 with value N2 |

### Round-1(Pro)

FL’s analysis:

Three UE vendors proposed the similar resolution. ZTE and Samsung basically view the current T_proc,1 is not relevant and should not be extended since the timeline for decoding PDSCH as specified in TS38.214 clause 5.3 is not affected due to delay PUCCH resource determination after decoding PDSCH. Note that I also noticed that the email thread [110bis-e-NR-R15-08] is discussing the time needed for UE adjusting power for PUSCH/SRS, from which clearly vast majority see the issue of adjusting power by extremely short time.

Similarly, UE vendors point it is a serious implementation issue. The following proposal which was also discussed in the last meeting appears the good middle ground for both UE and network. Where the change is made can be further discussed if companies really concerned to change the current timeline in TS38.214 clause 5.3.

#### Proposal 3.4.1

- For UE configured with NACK-only mode2, UE is expected to have additional d3 to provide valid HARQ-ACK feedback on top of  TProc,1 as follows: TProc,1=N1+d1,1+d2+d32048+144∙ƙ2-μ∙TC+Text, where d3=N2  if the PUCCH resources for NACK-only mode2 is not allocated in the same PRB. Otherwise, d3=0.

- FFS the change is made to TS38.214 clause 5.3 or made in TS38.213 clause 18 in this meeting.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Do not support. There is no need, and it is actually detrimental, to introduce another TProc,1 – justification was provided in x9708. |
| Qualcomm | Considering the RF power adjustment time, we support to add d3=N2 when UE transmits the NACK-only feedback on one of the PUCCH resources with different PRBs configured for NACK-only mode2. Just need to mention that if NACK-only mode2 feedback is to be multiplexed with other PUCCH/PUSCH, the other PUCCH/PUSCH will used (known after DCI detection) and d3=0. We support the proposal with the following changes and prefer the changes to be in 38.214. #### Proposal 3.4.1 - For UE configured with NACK-only mode2, UE is expected to have additional d3 to provide valid HARQ-ACK feedback on top of TProc,1 as follows: TProc,1=N1+d1,1+d2+d32048+144∙ƙ2-μ∙TC+Text, where d3=N2 if the UE transmits the NACK-only mode2 feedback on one of the PUCCH resources for NACK-only mode2 not allocated in the same PRB. Otherwise, d3=0. - FFS the change is made to TS38.214 clause 5.3 or made in TS38.213 clause 18 in this meeting. |
| ZTE | At least for single PDSCH scheduling, there is no need to prolong the timeline as discussed previously. |
| MTK | Support. Since the UE only can determine the PUCCH RB position after decoding all the related PDSCHs for the NACK only mode 2, it does not have sufficient time to process the TPC procedure, the extending for the processing time is needed for UE better support this MBS feature, which is similar with the discussion in email thread [110bis-e-NR-R15-08]. |
| Nokia, NSB | Ok to target UE vendors’ concerns for NACK-only mode 2. |

## (1-5)PUCCH resources for NACK-only multicast SPS

| Nokia-TP-x08701, | If a UE multiplexes in PUCCH HARQ-ACK information associated only with the second HARQ-ACK reporting mode and associated with multicast SPS PDSCH receptions, when the UE is provided moreThanOneNackOnlyMode and the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode, the UE determines a PUCCH resource from SPS-PUCCH-AN-List for unicast SPS PDSCH receptions as described in clause 9.2.1. |
| --- | --- |
| ZTE-Dis-x09470, | Proposal 5: PUCCH resource configured by PUCCH-Config/sps-PUCCH-AN-List can be used for HARQ-ack information for SPS PDSCH for MBS transferred from NACK-only when PUCCH-Config/PUCCH-ConfigurationList is not configured for MBS. |
| Ssmsung-TP-x09708, | If a UE multiplexes in a PUCCH only first HARQ-ACK information associated with multicast SPS PDSCH receptions and second HARQ-ACK information associated with multicast DCI formats and having same priority value as the first HARQ-ACK information, and the first and second HARQ-ACK information are according to different HARQ-ACK reporting modes, the UE determines the PUCCH resource based on: - sps-PUCCH-AN-ListMulticast, or sps-PUCCH-AN-List if sps-PUCCH-AN-ListMulticast is not provided, if the first HARQ-ACK information is according to the first HARQ-ACK reporting mode or - the last multicast DCI format, as described in clause 9.2.3, if the second HARQ-ACK information is according to the first HARQ-ACK reporting mode. |
| Huawei-Dis-x09822 | Proposal 5: For the case of NACK-only converted into ACK/NACK HARQ-ACK bits, the PUCCH resource used for transmitting the multiplexed HARQ-ACK bits is determined from SPS-PUCCH-AN-List configured for multicast with NACK-only mode1. If SPS-PUCCH-AN-List configured for multicast with NACK-only mode1 is not configured, the SPS-PUCCH-AN-List configured for unicast is used. |

### (CLOSED)Round-1(CR1+Pro2)

FL’s analysis:

The discussion is to resolve the FFS from the following agreement:

Agreement

- For the case when the PUCCH transmission for the NACK-only based feedback for one G-RNTI collides with the PUCCH transmission for ACK/NACK feedback for another G-RNTI with the same priority and then UE multiplexes the NACK-only based feedback with the ACK/NACK feedback onto the same PUCCH by transforming NACK-only into the ACK/NACK based HARQ-ACK bit, the PUCCH resources for transmitting the multiplexed HARQ-ACK bits is from the PUCCH-Config/PUCCH-ConfigurationList configured for multicast with ACK/NACK based feedback.

- For a UE configured with G-RNTI(s) with NACK-only HARQ-ACK feedback, when NACK-only HARQ-ACK bits are transformed into ACK/NACK HARQ-ACK bits, the PUCCH resource used for transmitting the multiplexed HARQ-ACK bits is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast with ACK/NACK based feedback based on the k1 and the PRI indication in the last DCI scheduling multicast. If PUCCH-Config/PUCCH-ConfigurationList configured for multicast with ACK/NACK based feedback is not configured, the PUCCH-Config/PUCCH-ConfigurationList configured for unicast is used.

- FFS: the case NACK-only is for multicast SPS PDSCH only.

Note that the above agreement did not consider that UE can be configured with UE specific PUCCH resources for NACK-only mode1. Therefore, when NACK-only HARQ-ACK bits are transformed into ACK/NACK HARQ-ACK bits, the PUCCH resource used for transmitting the multiplexed HARQ-ACK bits does not have to use the resources configured for ACK/NACK based feedback for another G-RNTI since UE may only support a single G-RNTI, or have to use the resources configured for unicast when PUCCH-Config/PUCCH-ConfigurationList configured for multicast with ACK/NACK based feedback is not configured.

Based on the submitted resolutions, at least the CR for the missing part “the resource determination for a PUCCH with HARQ-ACK for multicast DCI-based PDSCHs and HARQ-ACK for multicast SPS PDSCHs when corresponding HARQ-ACK reporting modes are different” is needed as proposed by SS. In addition, for the case NACK-only is for multicast SPS PDSCH only, how to determine the resources for the converted ACK/NACK, maybe using the resources configured for unicast is more agreeable.

#### Draft CR 3.5.1

The draft CR in Moderator Draft CR on issue 1-5 (to be replaced by the link to the draft CR in the inbox) is endorsed

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Support |
| Qualcomm | Ok in principle |
| NTT DOCOMO | Support |
| ZTE | For the first bullet, additional condition is that the second HARQ-ACK information is also associated with multicast SPS PDSCH receptions, which should also be captured. |
| CATT | OK |
| Nokia, NSB | Ok. |

#### Proposal 3.5.1

For a UE configured with G-CS-RNTI(s) with NACK-only HARQ-ACK feedback, when NACK-only HARQ-ACK bits are transformed into ACK/NACK HARQ-ACK bits, the PUCCH resource used for transmitting the multiplexed HARQ-ACK bits is determined from the sps-PUCCH-AN-List configured for unicast, if sps-PUCCH-AN-ListMulticast is not configured.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Support |
| NTT DOCOMO | Support |
| ZTE | We are fine with this proposal. |
| CATT | Support |
| Nokia, NSB | In our view, sps-PUCCH-AN-ListMulticast would be configured to provide NACK-only feedback in such a scenario (group-common resources). Therefore, the UE shall use UE-specific unicast resources. Does the group have a view that the orthogonal resources would also be configured in SPS-PUCCH-AN-ListMulticast? |

### Round-2 (CR1+CR2)

#### Draft CR 3.5.2-1 (same as Draft CR 3.5.1)

The draft CR in Moderator Draft CR on issue 1-5 (to be replaced by the link to the draft CR in the inbox) is endorsed.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | OK |
| ZTE | We are fine with this draft CR. |

Regarding Proposal 3.5.1

Based on the agreement made in 1st GTW session on Oct. 12:

Agreement

For a UE configured with G-CS-RNTI(s) with NACK-only HARQ-ACK feedback, when NACK-only HARQ-ACK bits are transformed into ACK/NACK HARQ-ACK bits, the PUCCH resource used for transmitting the multiplexed HARQ-ACK bits is determined from the sps-PUCCH-AN-List configured for unicast, if sps-PUCCH-AN-ListMulticast is not configured.

This round discusses the moderator draft CR.

#### Draft CR 3.5.2-2

The draft CR in Moderator draft CR on issue 1-5-2_v000_Mod (to be replaced by the link to the draft CR in the inbox) is endorsed.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | OK. Suggest the following update for somewhat better reading. If a UE multiplexes in a PUCCH only HARQ-ACK information associated with multicast SPS PDSCHs receptions of same priority that is according to the second HARQ-ACK reporting mode and is not provided sps-PUCCH-AN-ListMulticast, the UE determines the PUCCH resource from the sps-PUCCH-AN-List configured for unicast as described in clause 9.2.1. |

## (1-6)Type1 CB and ‘dci-enabler’ not configured simultaneously

| Qulacomm-CR-x09954, | A UE can be configured per G-RNTI or per G-CS-RNTI, by harq-FeedbackEnablerMulticast with value set to ‘enabled’, to provide HARQ-ACK information for PDSCH receptions. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI or G-CS-RNTI, the UE does not provide HARQ-ACK information for respective PDSCH receptions. If a UE is provided harq-FeedbackEnablerMulticast with value set to ‘dci-enabler’ for a G-RNTI or a G-CS-RNTI and pdsch-HARQ-ACK-Codebook = dynamic for multicast HARQ-ACK information, the UE determines whether or not to provide the HARQ-ACK information for PDSCH receptions based on an indication by the multicast DCI format associated with the G-RNTI or the G-CS-RNTI [4, TS 38.212]. If a UE is configured with pdsch-HARQ-ACK-Codebook = semi-static for multicast HARQ-ACK information, the UE is not expected to be provided harq-FeedbackEnablerMulticast with value set to ‘dci-enabler’ for a G-RNTI or a G-CS-RNTI. |
| --- | --- |
| Lenovo-CR-x10158 | A UE can be configured per G-RNTI or per G-CS-RNTI, by harq-FeedbackEnablerMulticast with value set to ‘enabled’, to provide HARQ-ACK information for PDSCH receptions. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI or G-CS-RNTI, the UE does not provide HARQ-ACK information for respective PDSCH receptions. If a UE is provided harq-FeedbackEnablerMulticast with value set to ‘dci-enabler’ for a G-RNTI or a G-CS-RNTI, the UE determines whether or not to provide the HARQ-ACK information for PDSCH receptions based on an indication by the multicast DCI format associated with the G-RNTI or the G-CS-RNTI [4, TS 38.212] if the UE is provided pdsch-HARQ-ACK-Codebook = dynamic, or provides the HARQ-ACK information for PDSCH receptions and ignores an indication in the multicast DCI format associated with the G-RNTI or the G-CS-RNTI [4, TS 38.212] if the UE is provided pdsch-HARQ-ACK-Codebook = semi-static. |

### (CLOSED)Round-1(CR)

FL’s analysis:

The difference between the two submitted draft CRs is whether UE is not expected to be configured with ‘dci-enabler’ or whether UE ignores the indication of ‘dci-enabler’. Since the codebook type is configured per UE for multicast, FL assesses the former makes more sense. In addition, to harmonize the draft CR to the issue in section 3.2, FL suggests taking x09954 as the baseline for the moderator draft CR.

#### Draft CR 3.6.1

The draft CR in Moderator Draft CR on issue 1-6 (to be replaced by the link to the draft CR in the inbox) is endorsed

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Support. Suggest the following rewording (e.g. by whom is the “UE is not expected”? – “UE does not expect” is clearer). If a UE is configured with provided pdsch-HARQ-ACK-Codebook = semi-static for multicast HARQ-ACK information, the UE is does not expected to be provided harq-FeedbackEnablerMulticast with value set to ‘dci-enabler’ for a G-RNTI or a G-CS-RNTI. |
| Qualcomm | Support Also fine with Editor’s suggested wording. |
| NTT DOCOMO | Support |
| ZTE | We are fine with this draft CR and also Samsung’s suggestion. |
| MTK | Ok |
| CATT | Support |
| Nokia, NSB | Ok. |

### Round-2(CR)

The draft CR seems agreeable in general and is updated based on Samsung’s suggestion. Please check whether the update is agreeable for email endorsement.

#### Draft CR 3.6.2

The draft CR in Moderator Draft CR on issue 1-6_V001_Mod (to be replaced by the link to the draft CR in the inbox) is endorsed

Company views:

| Company | Comments |
| --- | --- |
| Samsung | OK |
| vivo | The following change seems not needed. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI or G-CS-RNTI, no matter it is type 1 or type 2 codebook, UE would not provide HARQ-ACK information for respective PDSCH receptions. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI or G-CS-RNTI and pdsch-HARQ-ACK-Codebook = dynamic for multicast HARQ-ACK information, the UE does not provide HARQ-ACK information for respective PDSCH receptions. |
| ZTE | OK. |
| Qualcomm | Support |

## (1-7)NACK-only mode2 for case2 and case3

| Nokia-TP-x08701, | Clarify how the UE counts and orders the HARQ-ACK bits for multiple NACK-only feedback when configured with Alt-4. For Case 2, the UE counts the total number of HARQ-ACK bits based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI. For Case 3, the UE does not support Alt4. Delete “ for only one G-RNTI”. |
| --- | --- |
| NEC-Dis-x09137, | Proposal 1: The HARQ-ACK-to-PUCCH mapping table is applicable to both case2 and case3 for supporting NACK-only mode2. |
| CMCC-Dis-x09310, | Proposal 1. For addressing how to count and order the HARQ-ACK bits for NACK-only for Alt4, Opt 2-1-1 and Opt 3-1-1 are supported. |
| ZTE-Dis-x09470, | Proposal 3: For multicast, the following options are adopted for addressing to count and order the HARQ-ACK bits for NACK-only for NACK-only mode2: - Case 2: for the case of all UEs configured with the same set of G-RNTIs - Opt2-1-2 is supported for NACK-only mode2 - Opt2-1-2: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs - Case 3: for the case of different UEs configured with different G-RNTIs - Opt3-1-2 is supported for NACK-only mode2 - Opt3-1-2: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs. |
| MediaTek-Dis-x09527, | Proposal 1: For the NACK-only case, the mapping table between HARQ-ACK values and the PUCCH resources can be applied to multiple configured G-RNTIs. Proposal 2: For the NACK-only HARQ codebook generation, the DAI is counted per all G-RNTIs. |
| Apple-Dis-x09566, | Proposal 2: For case 2 and case3, adopt the above HARQ-ACK bit mapping to PUCCH resource table for NACK-only feedback mode Alt4 to support two G-RNTIs. |
| Samsung-TP-x09708, | From a UE perspective, there is no difference between Case 2 and Case 3. Case 2 is rather straightforward. From a network perspective, Case 3 can be supported with a joint DAI across G-RNTIs – e.g., if a UE is not configured a G-RNTI for a given DCI format, the UE behavior will be as if the UE missed the DCI format and the UE will then generate a NACK for that DCI format since the DAI value will be incremented in the next DCI format with a G-RNTI that is configured to the UE (C-DAI is assumed to run jointly over all scheduled G-RNTIs). Although that would be unnecessary additional HARQ-ACK for a UE configured with fewer G-RNTIs than the scheduled ones for a HARQ-ACK codebook, it is up to the NW to deploy Case 3 and a UE does not care. By removing the current “UE configured with only one G-RNTI” restriction, a network is given all flexibility to deploy the NACK-only mode 2 and there is practically no impact to UE implementation or to the specifications. If Case 3 is to be supported, a joint C-DAI across G-RNTIs for NACK only mode 2 should apply and the following text in Clause 9.1.3.1 of TS 38.213 v17.3.0 will need to be updated (joint C-DAI will also apply for Case 2 as a UE cannot differentiate – that would also be preferable in terms of performance/reliability as the “missed last DCI” problem is then generally suppressed). If Case 3 is not to be supported, or a reliability to the Type-2 HARQ-ACK CB for NACK-only mode 2 is to not be improved, the update for Clause 9.1.3.1 of TS 38.213 v.17.3.0 is not needed. As the specification impact from supporting Case 3 and improving Type-2 CB reliability for NACK-only mode 2 is rather trivial, it is preferable to operate the Type-2 CB assuming a joint C-DAI across G-RNTIs. |
| DOCOMO-CR-x09884, | Delete “ for only one G-RNTI”. |
| Lenovo-Dis-x10159, | Proposal 4: For multiplexing multiple NACK-only based feedback into one HARQ-ACK codebook, the number of HARQ-ACK information bits and ordering is based cDAI* accumulating the scheduled TBs across different G-RNTIs (support Option 2-1-2 and Option 3-1-2). |
| Ericsson-Dis-x10173 | cDAI* (former Opt2-1-2) is supported for Case 2. Using cDAI* for Case 3 (former Opt3-1-2) offers a working solution when all PDCCHs of the target G-RNTIs are received correctly. - Support an additional 2-bit field for cDAI* in Type I and Type II CB DCI formats. |

### Round-1(Que)

FL’s analysis:

The following is the agreement made in RAN1#109e:

Agreement

For multicast, for addressing how to count and order the HARQ-ACK bits for NACK-only for Alt4, further down-selection on the following cases/options in the next meeting:

- Case 2: for the case of all UEs configured with the same set of G-RNTIs

- Opt2-1: support Alt4 for this case

  - Opt2-1-1: based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI for counting the total number of HARQ-ACK bits. The ordering of HARQ-ACK bits is per C-DAI from each G-RNTI and in the ascending order of G-RNTI values.

  - Opt2-1-2: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs

- Opt2-2: does not support Alt4 for this case.

- Case 3: for the case of different UEs configured with different G-RNTIs

- Opt3-1: support Alt4 for this case

  - Opt3-1-1: based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI for counting the total number of HARQ-ACK bits. The ordering of HARQ-ACK bits is per C-DAI from each G-RNTI and in the ascending order of G-RNTI values.

  - Opt3-1-2: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs.

- Opt3-2: does not support Alt4 for this case.

As mentioned in the preparation phase, introducing any RRC parameters should be refrained at this stage and it is doubtful to reach a solution with consensus to either support case 2/3 (Opt2-1-2, Opt3-1-2) or not support. To support case 2 (and case 3), the spec needs to be updated as mentioned by some company. The current baseline is that the mapping table for NACK-only is for only one G-RNTI that implies case 2 and case 3 is NOT supported.

Now the question is that, if case 2 and case 3 are NOT supported, whether additional spec change is needed to clarify UE behavior for this case, e.g., UE is not expected to provide HARQ-ACK for NACK-only mode2 corresponding to PDSCHs with more than one G-RNTIs in the same PUCCH, or if case 2/3 happen UE only provide HARQ-ACK for NACK-only mode2 corresponding to PDSCHs with the lowest G-RNTI value in the same PUCCH?

Maybe we can also collect views whether only deleting “for only one G-RNTI” is agreeable, which means case 2/3 can be up to the NW to deploy and there is no impact to UE implementation because UE behaviors just follows the legacy, i.e., generating HARQ-ACK per C-DAI.

#### Question 3.7.1-1

If case 2 and case 3 are NOT supported with NACK-only mode2, whether additional spec change is needed to clarify UE behavior for this case? e.g.,

- UE is not expected to provide HARQ-ACK for NACK-only mode2 corresponding to PDSCHs with more than one G-RNTIs in the same PUCCH, or

- if case 2 or case 3 happen UE only provides HARQ-ACK for NACK-only mode2 corresponding to PDSCHs with the lowest G-RNTI value in the same PUCCH for example.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Given the current specifications for the above case, a non-consistent configuration can be viewed as gNB misconfiguration without any spec impact – UE behavior is undefined regardless of a “UE does not expect …” (e.g. similar to the UE indicating a capability for X G-RNTIs and being configured with X+1 G-RNTIs – nothing needs to be said). |
| Qualcomm | To make it clear that case 2/3 are error cases, we prefer to limit only one G-RNTI with second HARQ-ACK feedback mode if moreThanOneNackOnlyMode is configured, which corresponds to this in the following spec of 38.213: “A UE that is indicated the second HARQ-ACK reporting mode for only one G-RNTI can be indicated by moreThanOneNackOnlyMode to provide associated HARQ-ACK information bits in a PUCCH by selecting a PUCCH resource from a set of PUCCH resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1.” |
| vivo | We think the current spec is clear that only for the case that UE is configured with one G-RNTI, UE can be indicated with moreThanOneNackOnlyMode. Otherwise, moreThanOneNackOnlyMode can’t be provided. Then the default mode is mode 1.if company wants minor change to make it clearer, we can also accept. |
| ZTE | We think at least Case 2 should be supported. We don’t think a big spec impact is needed because the UE can select the PUCCH resource by following the DAI order for ACK/NACK mode. In addition, even if we exclude Case 2 and Case 3, the spec impact may still be needed. There is no new RRC parameters needed. For Case 3, we think it may be difficult to resolve, it can be excluded. |
| MTK | For the one G-RNTI case, we think the current spec description is clear. If case 2 and case 3 is not supported for NACK only mode 2, the corresponding clarification is needed, and the 1st sub-bullet is preferred. |
| CATT | It should clarify that only one G-RNTI of UE can be configured with both of NACK-only mode2 and moreThanOneNackOnlyMode. |
| Nokia, NSB | We agree with ZTE’s view. |

#### Question 3.7.1-2

Whether only deleting “for only one G-RNTI” from applying the table 18-1 in TS38.213 is agreeable,

- which means case 2 and case 3 can be up to the NW to deploy and there is no impact to UE implementation because UE just follows the legacy behaviour, i.e., generating HARQ-ACK per C-DAI.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | We support deleting the “for only one G-RNTI” restriction as it places an unnecessary restriction on deployments, it practically has no spec impact, and no other supporting spec impact is necessary. |
| Qualcomm | No |
| vivo | No. |
| NTT DOCOMO | We support deleting the statement “for only one G-RNTI” |
| ZTE | No |
| MTK | No |
| CATT | Don’t support. |
| Nokia, NSB | We support the proposal. Note that the current text also limits the operation of “NACK-only mode-1” to only 1 G-RNTI, therefore, this part needs to be reformed. |

## (1-8)DAI vs. ‘dci-enabler’

| Langbo-CR-x08995, | 38.213 9.1.3.1 A value of the counter downlink assignment indicator (DAI) field in DCI formats denotes the accumulative number of {serving cell, PDCCH monitoring occasion}-pairs in which PDSCH receptions, excluding PDSCH receptions that provide only transport blocks for HARQ processes associated with disabled HARQ-ACK information if donwlinkHARQ-FeedbackDisabled is provided or PDSCH receptions scheduled by DCI formats indicating not to provide HARQ-ACK information if harq-FeedbackEnablerMulticast is provided with value set to 'dci-enabler', or HARQ-ACK information bits that are not in response for PDSCH receptions, associated with the DCI formats, excluding the SPS activation DCI, is present up to the current serving cell and current PDCCH monitoring occasion, // The value of the total DAI, when present [5, TS 38.212], in a DCI format denotes the total number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s), excluding PDSCH receptions that provide only transport blocks for HARQ processes associated with disabled HARQ-ACK information if donwlinkHARQ-FeedbackDisabled is provided or PDSCH receptions scheduled by DCI formats indicating not to provide HARQ-ACK information if harq-FeedbackEnablerMulticast is provided with value set to 'dci-enabler', or HARQ-ACK information that does not correspond to PDSCH receptions, associated with DCI formats, excluding the SPS activation DCI, is present, up to the current PDCCH monitoring occasion m and is updated from PDCCH monitoring occasion to PDCCH monitoring occasion. // else if there is a PDSCH providing a transport block for a HARQ process with enabled HARQ-ACK information or a PDSCH scheduled by a DCI format indicating to provide HARQ-ACK information on serving cell c associated with PDCCH in PDCCH monitoring occasion m, or there is a PDCCH providing a DCI format associated with HARQ-ACK information without scheduling PDSCH reception on serving cell c |
| --- | --- |
| Lenovo-CR-x10157 | 38.213 9.1.3.1 A value of the counter downlink assignment indicator (DAI) field in DCI formats denotes the accumulative number of {serving cell, PDCCH monitoring occasion}-pairs in which PDSCH receptions, excluding PDSCH receptions that provide only transport blocks for HARQ processes associated with disabled HARQ-ACK information if donwlinkHARQ-FeedbackDisabled is provided and PDSCH receptions for multicast if the corresponding HARQ-ACK feedback is disabled by Enabling/disabling HARQ-ACK feedback indication in associated DCI format 4-2 if higher layer parameter harq-FeedbackEnabler-Multicast indicates dci-enabler or if the high layer parameter harq-FeedbackEnabler-Multicast is not provided, or HARQ-ACK information bits that are not in response for PDSCH receptions, associated with the DCI formats, excluding the SPS activation DCI, is present up to the current serving cell and current PDCCH monitoring occasion, // The value of the total DAI, when present [5, TS 38.212], in a DCI format denotes the total number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s), excluding PDSCH receptions that provide only transport blocks for HARQ processes associated with disabled HARQ-ACK information if donwlinkHARQ-FeedbackDisabled is provided and PDSCH receptions for multicast if the corresponding HARQ-ACK feedback is disabled by Enabling/disabling HARQ-ACK feedback indication in associated DCI format 4-2 if higher layer parameter harq-FeedbackEnabler-Multicast indicates dci-enabler or if the high layer parameter harq-FeedbackEnabler-Multicast is not provided, or HARQ-ACK information that does not correspond to PDSCH receptions, associated with DCI formats, excluding the SPS activation DCI, is present, up to the current PDCCH monitoring occasion m and is updated from PDCCH monitoring occasion to PDCCH monitoring occasion. |

### (CLOSED)Round-1(CR)

FL’s analysis:

The two submitted draft CRs are basically proposing the same thing, C-DAI and T-DAI should not count the PDCCH with ‘dci-enabler’ in the DCI indicating value 0 (i.e., disabling HARQ-ACK feedback) and x08995 also makes the change to the pseudo-code for Type2 HARQ-ACK generation.

Moreover, there is no T-DAI in DCI format 4_2, so the change made to T-DAI is not needed. Mentioning format 4_2 is not so necessary for conciseness because it is clear from TS38.212 and the draft CR for the issue in section 3.3 has already clarified it also.

The moderator draft CR is provided based the above.

#### Draft CR 3.8.1

The draft CR in Moderator Draft CR on issue 1-8 (to be replaced by the link to the draft CR in the inbox) is endorsed.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | OK in principle – an editorial revision is suggested below. or PDSCH receptions scheduled by DCI formats indicating absence of corresponding HARQ-ACK information if harq-FeedbackEnablerMulticast is provided with value set to 'dci-enabler' |
| Qualcomm | Ok |
| vivo | Generally fine. |
| NTT DOCOMO | OK |
| Langbo | OK, also fine with Samsung’s revision. |
| ZTE | We are fine with this draft CR and also fine with Samsung’s updates. |
| Ok | Ok |
| CATT | OK |
| Nokia, NSB | Ok. |

### Round-2(CR)

The draft CR seems agreeable in general and is updated based on Samsung’s suggestion. Please check whether the update is agreeable for email endorsement.

#### Draft CR 3.8.2

The draft CR in Moderator Draft CR on issue 1-8_v001_Mod (to be replaced by the link to the draft CR in the inbox) is endorsed

Company views:

| Company | Comments |
| --- | --- |
| Samsung | OK |
| vivo | Ok |
| ZTE | OK |
| Qualcomm | Shall we clarify DCI format 4_2 here? or PDSCH receptions scheduled by DCI formats 4_2 indicating absence of corresponding HARQ-ACK information if harq-FeedbackEnablerMulticast is provided with value set to 'dci-enabler' |

## (1-9)Type2 CB for multicast SPS

| vivo-CR-x08619 | If a UE is configured to monitor PDCCH for multicast DCI formats with CRC scrambled by one or more G-RNTIs or G-CS-RNTIs that the UE generates a Type-2 HARQ-ACK codebook, the UE separately applies the procedures in this clause except the procedures for SPS PDSCHs per G-RNTI or per G-CS-RNTI and determines the Type-2 HARQ-ACK codebook by concatenating the Type-2 HARQ-ACK codebook for unicast DCI formats, followed by the HARQ-ACK codebooks for the multicast DCI formats in ascending order of the corresponding G-RNTI values, followed by the HARQ-ACK codebooks for the multicast DCI formats in ascending order of the corresponding G-CS-RNTI values, followed by the HARQ-ACK codebook for SPS PDSCHs. |
| --- | --- |

### Round-1(CR)

FL’s analysis:

The reason for change from x-8619 is: for type 2 codebook construction for unicast HARQ-ACK and multicast HARQ-ACK, the following agreements were made in previous meeting, based on which it can be concluded that the HARQ-ACK sub-codebook for both unicast SPS PDSCHs and multicast SPS PDSCHs are jointly constructed and concatenated with HARQ-ACK sub-codebook for unicast DG PDSCHs and sub-codebook for multicast DG PDSCHs and is placed after HARQ-ACK sub-codebook for multicast DG PDSCHs. However, the current specification in 9.1.3.1, it captures that the UE separately applies the procedures in this clause per G-RNTI or per G-CS-RNTI, which may lead HARQ-ACK sub-codebook for SPS PDSCHs would be generated in multiple times, since the procedure to generate HARQ-ACK sub-codebook for unicast SPS PDSCHs and multicast SPS PDSCHs is also included in “the procedures in this clause”.

Agreement: (RAN1#104)

For ACK/NACK based feedback if supported for multicast, for Type-2 HARQ-ACK feedback construction for PTM scheme 1,

- DAI for unicast and DAI for multicast are separately counted.

- Concatenation of Type-2 HARQ-ACK codebook for unicast and multicast is supported.

  - FFS details on concatenating the codebooks.

- FFS whether to support concatenating more than one Type-2 HARQ-ACK codebook for multicast.

Agreement: (RAN1#104bis)

For Type-2 HARQ-ACK codebook concatenation to be multiplexed in the same PUCCH resource,

- The first Type-2 HARQ-ACK sub-codebook for unicast precedes the second Type-2 HARQ-ACK sub-codebook for multicast.

- FFS: The number of Type-2 HARQ-ACK sub-codebooks for multicast.

- Note: The case of SPS PDSCH will be discussed separately.

Agreement: (RAN1#107bis)

When UE is configured with unicast SPS and multicast SPS with ACK/NACK based feedback for multiplexing on the same PUCCH for the same priority case, the HARQ-ACK codebook is constructed as for multiple SPS PDSCHs regardless of unicast SPS PDSCH or multicast SPS PDSCH.

The current specification may cause confusion as mentioned by the source and the CR will be useful for the clarification. The moderator draft CR is provided based on x08619.

#### Draft CR 3.9.1

The draft CR in Moderator Draft CR on issue 1-9 (to be replaced by the link to the draft CR in the inbox) is endorsed.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | OK although a correction is not necessary as the pseudocode applies to DCI formats and the text at the end of the pseudocode provides the necessary description for SPS PDSCH. |
| Qualcomm | A question for clarification: Is the proposed concatenating order of unicast DG, multicast DG, multicast SPS, unicast SPS still aligned with the pseudocode equation? In current spec, the pseudocode equation seems bound unicast DG and unicast SPS together followed by multicast DG and multicast SPS. |
| vivo | Support. The proposed concatenating order is unicast DG + multicast DG+ multicast SPS and unicast SPS, and HARQ-ACK for multicast SPS and unicast SPS is jointly generated. This is aligned with our previous agreements. For Qualcomm question, I am not sure what exactly does “the pseudocode equation” mean? I think in the current spec, the pseudocode for SPS HARQ-ACK is for both unicast and multicast SPS. And the CR is aligned with it. |
| NTT DOCOMO | Support |
| ZTE | In our understanding, when the UE generates codebook for unicast, the SPS configuration for multicast is not considered if multicast is configured. In other words, the a SPS is skipped if it is for multicast transmission. So, the sub-codebook for SPS PDSCH is generated only once. No duplication for such sub-codebook. We don’t think the draft CR is needed. |
| Nokia, NSB | Ok. |

## (1-10)number of HARQ-ACK codebook for multicast

| CMCC-CR-x09312 | If a UE is configured with pdsch-HARQ-ACK-CodebookListMulticast-r17, pdsch-HARQ-ACK-Codebook is replaced by the relevant entry in pdsch-HARQ-ACK-CodebookListMulticast-r17 in clauses 7.3.1.1.2, 7.3.1.1.3 and 7.3.1.5.3 for multicast HARQ-ACK codebook. |
| --- | --- |

### (STABLE CR)Round-1(CR)

FL’s analysis:

Based on the submitted CR, the current description of the DCI formats scheduling multicast in TS38.212 does not support configuring more than one HARQ-ACK codebook for multicast, which is not aligned with the agreement. The moderator draft CR is provided based on input x09312.

#### Draft CR 3.10.1

The draft CR in Moderator Draft CR on issue 1-10 (to be replaced by the link to the draft CR in the inbox) is endorsed.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | OK (suggest to put “in this clause” at the beginning of the sentence). |
| Qualcomm | ok |
| vivo | Ok |
| NTT DOCOMO | OK |
| ZTE | We are fine with the draft CR. |
| MTK | Ok |
| CATT | OK |
| Nokia, NSB | Ok. |

## (1-11)config. of moreThanOneNackOnlyMode

| vivo-CR-x08618, | A UE that is indicated the second HARQ-ACK reporting mode for only one G-RNTI can be indicated by moreThanOneNackOnlyMode to provide associated HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode or by selecting a PUCCH resource from a set of PUCCH resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1 when a number of HARQ-ACK information bits is more than one. The UE generates HARQ-ACK information bits for the second HARQ-ACK reporting mode according to a Type-2 HARQ-ACK codebook as described in clause 9.1.3.1. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. // If a UE is provided pucch-ConfigurationListMulticast1 or pucch-ConfigurationListMulticast2 for PUCCH transmissions with a priority value, the UE transmits a PUCCH with the priority value according to pucch-ConfigurationListMulticast1 or pucch-ConfigurationListMulticast2 for each G-RNTI or G-CS-RNTI that the UE provides associated HARQ-ACK information according to the first HARQ-ACK reporting mode or the second HARQ-ACK reporting mode, respectively. For HARQ-ACK information associated only with the second HARQ-ACK reporting mode, when the UE is not provided moreThanOneNackOnlyMode and the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode, the UE determines a PUCCH resource from pucch-ConfigurationListMulticast1, if provided; otherwise, the UE determines a PUCCH resource from pucch-Config/pucch-ConfigurationList. |
| --- | --- |
| Huawei-CR-x08468 | A UE that is indicated the second HARQ-ACK reporting mode for only one G-RNTI can be indicated to provide associated HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode when moreThanOneNackOnlyMode is not configured and the UE determines a PUCCH or a PUSCH to provide the HARQ-ACK information as described in clause 9.2 or by selecting a PUCCH resource from a set of PUCCH resources for the PUCCH transmission based on the values of the HARQ-ACK information bits when moreThanOneNackOnlyMode is configured as described in Table 18-1. The UE generates HARQ-ACK information bits for the second HARQ-ACK reporting mode according to a Type-2 HARQ-ACK codebook as described in clause 9.1.3.1. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. Table 18-1: Mapping of values of HARQ-ACK information bits to PUCCH resources for the second HARQ-ACK reporting mode when moreThanOneNackOnlyMode is configured |

### Round-1(CR)

FL’s analysis:

The commonality of the two submitted CRs is correcting providing moreThanOneNackOnlyMode actually meaning NACK-only mode2 and not providing moreThanOneNackOnlyMode meaning NACK-only mode1. Each submitted CRs also address other different corrections individually, e.g., PRI indication, which could be discussed separately.

The moderator draft CR is provided based on the two submitted CRs.

#### Draft CR 3.11.1

The draft CR in Moderator Draft CR on issue 1-11 (to be replaced by the link to the draft CR in the inbox) is endorsed.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Support |
| Qualcomm | The issue is clear, but the CR is not correct, because NACK-only mode1 is only applied when NACK-only feedback collides with other PUCCH/PUSCH. Otherwise, 1TB NACK-only does not transform to ACK/NACK. So, we suggest the following TP for 38.213 (TS 38.212 is a typo in the draft CR): "A UE that is indicated the second HARQ-ACK reporting mode for only one G-RNTI can be indicated by moreThanOneNackOnlyMode to provide associated HARQ-ACK information bits in a PUCCH by selecting a PUCCH resource from a set of PUCCH resources for the PUCCH transmission based on the values of the HARQ-ACK information bits is configured as described in Table 18-1. A UE that is indicated the second HARQ-ACK reporting mode but not configured with moreThanOneNackOnlyMode is to provide associated HARQ-ACK information bits in a PUCCH or PUSCH according to the first HARQ-ACK reporting mode if colliding with other PUCCH/PUSCH" |
| vivo | Support Regarding Qualcomm’s comment that NACK-only mode1 is not applied to the case of 1 TB and no collision with other channels. We agree with this. But we think this issue can be discussed in section 3.1. |
| NTT DOCOMO | Support |
| ZTE | We are fine with this draft CR. |
| MTK | Ok |
| Nokia, NSB | Ok with Qualcomm’s intention, however, we can also leave this to a correction to be made for Section 3.1. |

## (1-12)Multiplexing one unicast SPS and DG multicast

| vivo-CR-x08887 | Proposal: When HARQ-ACK for unicast SPS PDSCHs only and HARQ-ACK for multicast dynamic grant PDSCHs with ACK/NACK based feedback with the same priority are indicated to be transmitted in the same PUCCH slot, - If SPS-PUCCH-AN-List for unicast is not configured, HARQ-ACK for multicast dynamic grant PDSCHs with ACK/NACK based feedback is dropped. |
| --- | --- |

### (CLOSED)Round-1(Pro)

FL’s analysis:

For the multiplexing of HARQ-ACK for unicast SPS PDSCHs and multicast dynamic grant PDSCHs with ACK/NACK based feedback with the same priority, it was agreed to multiplex these HARQ-ACK on the PUCCH configured by SPS-PUCCH-AN-List configured for unicast in previous meetings.

Agreement (RAN1#107bis)

When HARQ-ACK for unicast SPS PDSCHs and multicast dynamic grant PDSCHs with ACK/NACK based feedback are multiplexed on the same PUCCH for the same priority case, down-select from:

- Option 1: the PUCCH carrying the multiplexed HARQ-ACK is determined from the SPS-PUCCH-AN-List configured for unicast.

- Option 2: the PUCCH carrying the multiplexed HARQ-ACK is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast.

Agreement (RAN1#108)

When HARQ-ACK for unicast SPS PDSCHs and multicast dynamic grant PDSCHs with ACK/NACK based feedback are multiplexed on the same PUCCH for the same priority case, the following option 1 (from the previous agreement) is adopted:

- Option 1: the PUCCH carrying the multiplexed HARQ-ACK is determined from the SPS-PUCCH-AN-List configured for unicast.

- Option 2: the PUCCH carrying the multiplexed HARQ-ACK is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast

However, SPS-PUCCH-AN-List configured for unicast is based on the capability of supporting multiple of unicast SPS PDSCHs, i.e., FG12-2, which should be independent from R17 multiplexing of unicast HARQ-ACK and multicast HARQ-ACK with the same priority. It seems unreasonable to always bound these two features. Considering at least R17 UEs without capability of SPS-PUCCH-AN-List for unicast, the relevant UE behavior needs to be specified. Otherwise, gNB wouldn’t be able to schedule multicast PDSCHs to transmit HARQ-ACK in the same slot with unicast SPS HARQ-ACK.

When SPS-PUCCH-AN-List configured for unicast is not configured, one way is to multiplex HARQ-ACK for unicast SPS PDSCHs and multicast dynamic grant PDSCHs with ACK/NACK based feedback on one of the PUCCH resources configured for multicast. i.e., adopting option 2 in the above agreements in this case. Another simpler way is to drop one of these two kinds of HARQ-ACK. Either dropping unicast SPS HARQ-ACK, or dropping multicast DG HARQ-ACK is workable. Considering gNB may get HARQ-ACK information from other UEs in the same MBS group and use PTM for retransmission of multicast PDSCHs, it may make more sense to drop HARQ-ACK for multicast in this case.

#### Proposal 3.12.1

For UEs not supporting more than one SPS configuration, when UE would multiplex HARQ-ACK for unicast SPS PDSCHs and multicast dynamic grant PDSCHs with ACK/NACK based feedback,

  - Alt1: the PUCCH carrying the multiplexed HARQ-ACK is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast.

  - Alt2: drop HARQ-ACK for multicast in this case.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Alt1 is preferable |
| Qualcomm | We prefer gNB to avoid the collision in this case, i.e., If SPS-PUCCH-AN-List for unicast is not configured, the UE does not expect that the HARQ-ACK for multicast dynamic grant PDSCHs with ACK/NACK based feedback and unicast SPS feedback collide in the same PUCCH. |
| vivo | Similar issue was discussed in URLLC intra-UE in the last meeting where is t decouple the features of R17 intra-UE multiplexing of different priorities and multiple SPS configuration. Then it was agreed to drop low priority HARQ-ACK if SPS-PUCCH-AN-List for high priority is not provided. The situation is similar and similar solution i.e., Alt 2 can be supported. Regarding Qualcomm’s suggestion, it means gNB wouldn’t be able to schedule multicast PDSCHs to transmit HARQ-ACK in the same slot with unicast SPS HARQ-ACK, this is great restrictions on gNB’s scheduling, especially when unicast SPS PDSCH is configured with short periodicity. |
| ZTE | We prefer Alt 1. In addition, we also think it can be up to implementation to resolve this issue. Dropping the HACK-ACK for multicast is not good since this may affect the performance. |
| Nokia, NSB | Ok with Alt1. |

### Round-2(CR)

FL’s analysis:

Agreement

For UEs not provided with SPS-PUCCH-AN-List, when UE would multiplex HARQ-ACK for unicast SPS PDSCHs and multicast dynamic grant PDSCHs, the PUCCH carrying the multiplexed HARQ-ACK is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast.

Based on the agreement made in 1st GTW session on Oct. 12, this round discusses the moderator draft CR.

#### Draft CR 3.12.2

The draft CR in Moderator draft CR on issue 1-12_v000_Mod (to be replaced by the link to the draft CR in the inbox) is endorsed.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | OK in principle. For consistency with the other text and completeness of ‘multicast’, we suggest the following change. … the UE determines the PUCCH resource from SPS-PUCCH-AN-List for unicast SPS PDSCH receptions if provided as described in clause 9.2.1; otherwise, the UE determines the PUCCH resource from PUCCH-Config/PUCCH-ConfigurationList for multicast PDSCH receptions. |
| ZTE | Just for clarification, since PUCCH-Config/PUCCH-ConfigurationList configured for multicast may not be configured, and the PUCCH resource configured for unicast is used in this case. Therefore, the CR means that the PUCCH resource configured for multicast is always configured. Maybe we can make the following change on top of Samsung’s version. When to use resource for multicast and when to use resource for unicast have been described in the spec. There is no ambiguity. PUCCH-Config/PUCCH-ConfigurationList configured for multicast PDSCH receptions or unicast PDSCH receptions. |

## (1-13)[POSTPONED]PTP retx for NACK-only when applicable

| Huawei-CR-x10207 | A PDSCH reception providing an initial transmission of a transport block is scheduled only by a multicast DCI format. A PDSCH reception providing a retransmission of the transport block can be scheduled either by a multicast DCI format using a same G-RNTI as the G-RNTI of the initial transmission of the transport block, or by a unicast DCI format using a C-RNTI when applicable [6, TS 38.214]. An activation for SPS PDSCH receptions using a G-CS-RNTI for a corresponding SPS PDSCH configuration is provided only by a multicast DCI format as described in clause 10.2 by replacing CS-RNTI with the G-CS-RNTI. A release for SPS PDSCH receptions using a G-CS-RNTI for a corresponding SPS PDSCH configuration is provided by a multicast DCI format as described in clause 10.2 by replacing CS-RNTI with the G-CS-RNTI, or by a DCI format with CRC scrambled by CS-RNTI. For a transport block that a UE received in a SPS PDSCH, a PDSCH reception providing a retransmission of the transport block can be scheduled either by a unicast DCI format using a CS-RNTI when applicable or by a multicast DCI format using a same G-CS-RNTI as the G-CS-RNTI of the initial transmission of the transport block [6, TS 38.214]. |
| --- | --- |

The preparation phase concludes this issue is postponed to the next meeting.

## (1-14)deleting redundant descriptions

| Lenovo-CR-x10155 | Delete “if pdsch-HARQ-ACK-Codebook-Multicast = semiStatic is configured” and “if the higher layer parameter pdsch-HARQ-ACK-Codebook-Multicast = dynamic is configured” for 1st DAI and 3rd DAI in DCI format 0_1 and DCI format 0_2. |
| --- | --- |

### (STABLE CR)Round-1(CR)

FL’s analysis:

As the initial assessment states this submitted draft CR is editorial refinement but should be easily agreeable.

The moderator draft CR is provided based on x10155.

#### Draft CR 3.14.1

The draft CR in Moderator Draft CR on issue 1-14 (to be replaced by the link to the draft CR in the inbox) is endorsed.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Support |
| Qualcomm | ok |
| vivo | Not necessary but ok |
| NTT DOCOMO | OK |
| ZTE | OK |
| CATT | OK |
| Nokia, NSB | Ok. |

## (1-15)aligning RRC parameter name for HARQ-ACK CB

| CMCC-CR-x09312 | Change “pdsch-HARQ-ACK-Codebook-Multicast” to “pdsch-HARQ-ACK-Codebook”. |
| --- | --- |

### (STABLE CR for alignment)Round-1(CR)

FL’s analysis:

As the initial assessment states this submitted draft CR is RRC parameters alignment and should be easily agreeable.

The moderator draft CR is provided based on x09312.

#### Draft CR 3.15.1

The draft CR in Moderator Draft CR on issue 1-15 (to be replaced by the link to the draft CR in the inbox) is endorsed.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | OK – should go to the Rel-17 alignment CR. |
| Qualcomm | ok |
| vivo | Ok – should go to the Rel-17 alignment CR. |
| NTT DOCOMO | OK |
| ZTE | Agree that it should be alignment CR |
| Nokia, NSB | Ok. |

## (1-16)pdsch-HARQ-ACK-retx for multicast

| CATT-CR-x08925 | Summary of change: If pdsch-HARQ-ACK-retx is configured by RRC, the retransmission for multicast HARQ-ACK codebook can be triggered by DCI format 1_1 or format 1_2 when HARQ-ACK retransmission indicator is indicated as 1，the retransmission procedure is as described in clause 9.1.5. |
| --- | --- |

### Round-1(Que)

FL’s analysis:

As discussed in the preparation phase, the discussion at RAN1#110bis-e is only to clarify the issue and whether it is essential (to control the workload).

Reason for change from the submitted CR:

Current specification has supported HARQ-ACK codebook retransmission when the codebook is dropped due to multiplexing procedure between low and high priority HARQ-ACK codebook in a same PUCCH slot, but it is not clear whether the re-transmission codebook can include multicast HARQ-ACK information or not. To improve multicast transmission efficiency, it is suggested that retransmission HARQ-ACK codebook trigged by DCI format 1_1/1_2 can include multicast feedback.

#### Question 3.16.1

- Whether the submitted CR is essential or not?

- If the submitted CR is not agreeable, any additional spec change is needed for clarification regarding the discussed issue?

Company views:

| Company | Comments |
| --- | --- |
| Samsung | The CR is not needed. The corresponding specifications do not differentiate/consider ‘unicast’ or ‘multicast’ HARQ-ACK CB. |
| Qualcomm | Even if the UE supports DCI 1_1/1_2 to trigger CB retransmission, it seems no spec impact that unicast DCI can trigger retransmission of the CB, which includes all the feedbacks for all HARQ processes, including unicast and multicast CB feedback. |
| vivo | The CR may not be needed. Type 3/enhanced type 3 CB can include HARQ-ACK for all HARQ processes, including unicast and multicast CB feedback. Conclusion may be enough if company wants to make it clearer. |
| ZTE | We share the same view with other companies that CR is not needed. |
| MTK | The CR is not needed and it does not have any additional spec change. |
| CATT | If it is a common understanding that HARQ-ACK CB retransmission can be applied to multicast CB as well, a CR or conclusion is required to make it clear since HARQ-ACK CB retransmission has not been discussed in MBS. |
| Nokia, NSB | No need to have additional spec changes at this point for a new Rel-17 feature. |

## (1-17)spsHARQdeferral for multicast SPS

| CATT-CR-x08926 | Summary of change: The HARQ-ACK information for SPS PDSCH can be deferred if spsHARQdeferral is configured in a multicast SPS configuration by RRC for first HARQ-ACK reporting mode, the detail is as described in clause 9.2.5.4. |
| --- | --- |

### Round-1(Que)

FL’s analysis:

As discussed in the preparation phase, the discussion at RAN1#110bis-e is only to clarify the issue and whether it is essential (to control the workload).

Reason for change from the submitted CR:

Current specification has supported deferring HARQ-ACK for SPS PDSCH when the codebook would drop due to PUCCH resource has overlapping with downlink symbols.  But it is not clear whether RRC parameter IE spsHARQdeferral can be used for multicast SPS configuration. To improve multicast transmission efficiency, it is suggested that MBS also supports deferring HARQ-ACK for SPS PDSCH if first HARQ-ACK reporting mode is configured.

#### Question 3.17.1

- Whether the submitted CR is essential or not?

- If the submitted CR is not agreeable, any additional spec change is needed for clarification regarding the discussed issue?

Company views:

| Company | Comments |
| --- | --- |
| Samsung | The CR is not needed. The corresponding specifications do not differentiate/consider ‘unicast’ or ‘multicast’ SPS HARQ-ACK. |
| Qualcomm | May need further clarification on If sps-HARQ-Deferral can be applied to multicast feedback, why it only applies to first HARQ-ACK feedback mode for SPS multicast? |
| vivo | Same question as Qualcomm. |
| ZTE | Same question as Qualcomm |
| MTK | The CR is not needed and it does not have any additional spec change. |
| CATT | We think it is a common understanding that sps-HARQ-Deferral can be applied to multicast CB, a CR or conclusion is required to make it clear. If sps-HARQ-Deferral applies to second HARQ-ACK feedback, what we concern is that will trigger additional discussions. For example, if UE generates all ‘ACK’ for SPS PDSCH, the UE doesn’t require HARQ CB feedback. How to handle this case for UE? But if most of companies think sps-HARQ-Deferral can be applied to second HARQ-ACK feedback as well, we are also ok with it. |
| Nokia, NSB | No need to introduce spec changes at this stage for a new Rel-17 feature. |

## (1-18)Type3 for NACK-only mode

| Langbo-CR-x08996 | Summary of change: For generating a Type-3 HARQ-ACK codebook with NDIHARQ=1, the UE considers the HARQ-ACK information corresponding to a PDSCH reception has been reported if a PUCCH was not transmitted due to all values of HARQ-ACK information including the one for the PDSCH reception being ‘ACK’ according to the second HARQ-ACK reporting mode. |
| --- | --- |

### Round-1(Que)

FL’s analysis:

As discussed in the preparation phase, the discussion at RAN1#110bis-e is only to clarify the issue and whether it is essential (to control the workload).

Reason for change from the submitted CR:

For a type-3 HARQ-ACK codebook, if NDI values are not included, a UE determines the value of a HARQ-ACK information bit according to whether HARQ-ACK information for a PDSCH reception of corresponding HARQ process number has been reported or not. For NACK-only reporting mode, it is unclear whether HARQ-ACK information corresponding to a PDSCH reception should be considered reported or not reported if the PUCCH was not transmitted due to only ACK values were generated. In order to avoid misalignment between UE and gNB, it is reasonable for the UE to consider in this case that the HARQ-ACK information for the PDSCH reception has been reported and the corresponding HARQ-ACK bits in the type-3 HARQ-ACK codebook is set to the default value of NACK.

#### Question 3.18.1

- Whether the submitted CR is essential or not?

- If the submitted CR is not agreeable, any additional spec change is needed for clarification regarding the discussed issue?

Company views:

| Company | Comments |
| --- | --- |
| Samsung | The CR is not needed. Type-3 operates on HARQ processes (for corresponding ACK/NACK values). |
| Qualcomm | Probably we can assume NACK-only mode1 feedback for this case, since the NACK-only feedback is to be multiplexed with other feedback. |
| vivo | We think it is reasonable for the UE to consider in this case that the HARQ-ACK information for the PDSCH reception has been reported and the corresponding HARQ-ACK bits in the type-3 HARQ-ACK codebook is set to the default value of NACK. |
| Langbo | An example is shown below to have a better picture of this issue: - a PUCCH for HARQ process #0 was triggered but not transmitted due to ‘ACK’ in NACK-only mode; - a DCI triggers a Type-3 HARQ-ACK codebook operating on all HARQ processes without NDI bits ; - a HARQ-ACK bit value for HARQ process #0 is determined based on whether HARQ-ACK information corresponding to the PUCCH has been reported; - Is the ‘ACK’ considered as reported or not reported? We think this issue is essential. It will lead to ambiguous understandings on type-3 HARQ-Ack codebook construction. We are open to solutions to address this issue. |
| ZTE | For the NACK-only mode, it should be assumed that UE has reported the ACK/NACK after the PUCCH resource even though it is ACK because the network will assume ACK for the PDSCH if nothing is received on the PUCCH resource. We don’t think CR is needed. |
| Nokia, NSB | Agree with vivo that UE shall consider already HARQ-ACK has been reported and put the default “NACK” value. Perhaps a clarification on this in the spec is needed. |

## (1-19)three issues from x09449

| LGE-Dis-x09449 | For issue A: NACK-only mode2 multiplexed with other UCI/PUSCH is supported Proposal 1: If the resultant NACK-only based PUCCH selected for multiplexing multiple NACK-only HARQ-ACK bits in Alt4 is overlapped with other PUCCH or PUSCH, the resultant NACK-only based HARQ-ACKs in Alt4 are transformed to ACK/NACK based HARQ-ACK for multiplexing the NACK-only HARQ-ACK bits in the PUSCH. For issue B: SR and ack/nack based multiplexing with 1bit for unicast and 1 bit for multicast. Proposal 2: For multiplexing a positive SR and two HARQ-ACK bits for unicast and multicast respectively of a same priority, - For PUCCH format 0 for unicast HARQ-ACK, - Three bits are multiplexed on the PUCCH resource with PUCCH format 0 allocated for unicast HARQ-ACK according to the existing rules. - For PUCCH format 1 for unicast HARQ-ACK and SR, - Two HARQ-ACK bits are multiplexed on the SR PUCCH resource according to the existing rules - For PUCCH format 1 for unicast HARQ-ACK and PUCCH format 0 for SR - SR is dropped and Two HARQ-ACK bits are multiplexed on the PUCCH resource with PUCCH format 1 allocated for unicast HARQ-ACK according to the existing rules. For issue C: HARQ-ACK multiplexing for u-cast and m-cast is per priority and then applying the intra-UE multiplexing Proposal 3: For multiplexing HARQ-ACK information for unicast and multicast respectively and other UCIs of both LP and HP, - HARQ-ACKs for unicast and HARQ-ACKs for multicast are multiplexed per each priority before applying R17 intra-UE multiplexing rules. - Afterwards, R17 intra-UE multiplexing rules are applied to multiplexing PUCCH for HP and PUCCH for LP, assuming that the multiplexed unicast/multicast HARQ-ACKs of different priorities are considered HP/LP HARQ-ACKs respectively in the rules. |
| --- | --- |

### Round-1(Que)

FL’s analysis:

As discussed in the preparation phase, the discussion at RAN1#110bis-e is only to clarify the issue and whether it is essential (to control the workload).

The submitted tdoc discusses three issues without TP or draft CR.

For issue A: NACK-only mode2 multiplexed with other UCI/PUSCH is supported

Issue description: A resultant PUCCH resource selected for multiplexing four NACK-only based HARQ-ACK bits according to Alt4 could be overlapped with other PUCCH or PUSCH in time. In this case, it is not clear whether the multiplexed NACK-only based HARQ-ACK can be transformed to ACK/NACK based HARQ-ACK for multiplexing NACK-only based PUCCH and other PUCCH/PUSCH in a same slot, even when Alt4 is configured for multiplexing NACK-only based HARQ-ACKs.

For issue B: SR and ack/nack based multiplexing with 1bit for unicast and 1 bit for multicast.

Issues description and solutions: It seems not discussed yet how to multiplex positive SR, 1 HARQ-ACK bit for unicast and 1 HARQ-ACK bit for multicast for PUCCH format 0/1. Considering the existing multiplexing rules, for a same priority, we propose to multiplex a positive SR and two HARQ-ACK bits for unicast and multicast respectively.

For issue C: HARQ-ACK multiplexing for u-cast and m-cast is per priority and then applying the intra-UE multiplexing

Issues description: For Rel-17 intra-UE multiplexing, RAN1 agreed that for multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH or a PUSCH in R17, separate coding for the two HARQ-ACKs are supported. Considering this agreement, we propose that HARQ-ACKs for unicast and HARQ-ACKs for multicast are multiplexed per each priority before applying R17 intra-UE multiplexing rules. Afterwards, R17 intra-UE multiplexing rules can be applied to multiplexing PUCCH for HP and PUCCH for LP, assuming that the multiplexed unicast/multicast HARQ-ACKs of different priorities are considered HP/LP HARQ-ACKs respectively in the rules.

#### Question 3.19.1

- Whether the discussed issues are essential or not?

- If the discussed issues are not essential, any additional spec change is needed for clarification regarding the discussed issues?

Company views:

| Company | Comments |
| --- | --- |
| Samsung | For all above issues, the specifications are clear and no further update is needed. If the proponent thinks otherwise, we can discuss draft CRs next time. |
| ZTE | For the first issue, we think the spec is clear that NACK-only should be transferred to ACK/NACK when it overlaps with other PUCCH or PUSCH. It is applied to both mode 1 and mode 2. For the second issue, we think the current mechanism for unicast is reused. For the third issue, it is the same as the unicast HARQ-ACK with different priorities. |

## (1-20)NTN multicast

| Qualcomm-CR-x09960 | Summary of change: If a UE is configured with harq-FeedbackEnablerMulticast, the UE is not expected to receive a transport block reception associated with a G-RNTI or a G-CS-RNTI for a HARQ process with disabled HARQ-ACK information as indicated by HARQ-feedbackEnabling-disablingperHARQprocess, if provided. |
| --- | --- |

### Round-1(Que)

FL’s analysis:

As discussed in the preparation phase, the discussion at RAN1#110bis-e is only to clarify the issue and whether it is essential (to control the workload).

Reason for change from the submitted CR is:

UE may support both NTN and multicast features. However, for HARQ-ACK feedback enabling/disabling, there are two different schemes for the two features.

- For NTN unicast, the enabling/disabling is configured per HARQ process ID by RRC signaling. The UE will report the NACK in the Type-1 CB for the HARQ process with disabled feedback.

- For TN multicast, the enabling/disabling is configured per G-RNTI/G-CS-RNTI by RRC or indicated in the DCI with the G-RNTI/G-CS-RNTI. It is up to UE to report NACK or ACK/NACK in the Type-1 CB for G-RNTI/G-CS-RNTI with disabled feedback.

When UE is configured to receive NTN multicast with enabled/disabled HARQ-ACK feedback, the UE behavior needs to be clarified whether the UE can be configured to receive multicast PDSCH with a HARQ process configured with disabled HARQ-ACK as indicated by HARQ-feedbackEnabling-disablingperHARQprocess.

#### Question 3.20.1

- Whether the submitted CR is essential or not?

- If the submitted CR is not agreeable, any additional spec change is needed for clarification regarding the discussed issue?

Company views:

| Company | Comments |
| --- | --- |
| Samsung | The CR is not needed. No need to mix MBS and NTN. Also, there is no scenario where the UE will be connected to a NTN for unicast and to TN for MBS – several aspects will then be problematic, including PDCCH monitoring, MBS HARQ-ACK reporting (when multiplexed with unicast), power control, …. |
| Qualcomm | support the CR. The key thing is that the spec is not clear whether/when a HARQ process with disabled feedback can be scheduled for multicast or not. It is a scenario that UE can support NTN for both unicast and multicast, not the scenario of NTN for unicast and TN for MBS. |
| ZTE | We don’t think this CR is needed. The network can avoid this issue. For example, the network does not use the HARQ process with disabling for scheduling multicast. |
| MTK | The CR is not needed. If we want to discuss the CR, one issue is needed to be clarified firstly whether the NTN UE can receive the MBS multicast service since there is not any conclusion for this. However, for the NTN and MBS, there are two separate features defined in Rel-17, we do not suggest discussing the two FGs together in this late stage and make the discussion complexity. |
| Nokia, NSB | Agree with MTK’s view. |
| Qualcomm2 | The UE can support NTN feature and MBS feature. We allow UE report MBS in NTN band, e.g., FG 33-3-1a with capability to Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast for NTN and unlicensed If as ZTE said “the network does not use the HARQ process with disabling for scheduling multicast”, it is necessary to clarify in the spec that UE is not expected that HARQ process with disabled feedback can be scheduled for multicast. |

## (1-21)missing statement for mode1

| ZTE-CR-x09476 | Summary of change: If a UE would multiplex multicast HARQ-ACK information according to the second HARQ-ACK reporting mode with multicast HARQ-ACK information according to the first HARQ-ACK reporting mode, or unicast HARQ-ACK information, or CSI reports in a first PUCCH or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH when the UE is provided moreThanOneNackOnlyMode, the UE considers that the UE would transmit the second PUCCH using any PUCCH resource from the PUCCH resources associated with the second HARQ-ACK reporting mode when all values of the HARQ-ACK information are 'ACK'; otherwise, the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are 'ACK'. |
| --- | --- |

### (STABLE CR)Round-1(CR)

Reason for change from the submitted CR is:

For multiplexing between NACK-only PUCCH and the other PUCCH or PUSCH, it is assumed that the UE would transmit the NACK-only PUCCH regardless the PDSCH decoding results according to the agreement. The reason is that the UE will not transmit NACK-only PUCCH when the HARQ-ACK information is ‘ACK’. This has been reflected in the specification when the UE is configured with NACK-only mode 2. However, the description for UE configured with NACK-only mode 1 is still missing since the HARQ-ACK feedback is still NACK-only mode when there is only one PDSCH for MBS. It should be added in the specification.

FL’s analysis:

After further clarification from proponent during the preparation phase, the intent of the CR is better understood now. Basically,

- For both NACK-only mode1 and mode2, when UE would multiplex multicast HARQ-ACK with others, the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are 'ACK.

- Later when reflect the agreement of all PUCCH resources being configured with the same starting symbol and duration, the above bullet is restricted to NACK-only mode2.

- Therefore, the CR needed for now is to correct “the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are 'ACK” should still apply to NACK-only mode1.

However, the submitted CR may cause confusion by using ‘otherwise’. The moderator CR is provided based on the reason for change from the submitted CR in x09476.

#### Draft CR 3.21.1

The draft CR in Moderator Draft CR on issue 1-21 (to be replaced by the link to the draft CR in the inbox) is endorsed.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Support |
| Qualcomm | Ok in general |
| vivo | Ok |
| NTT DOCOMO | OK |
| ZTE | We support this draft CR. |
| Nokia, NSB | Ok. |

## (1-22)NACK-only multiplexing SR

| Nokia-TP-x08701, NEC-Dis-x09137, ZTE-Dis-x09470, MediaTek-Dis-x09527, DOCOMO-CR-x09885 |  |
| --- | --- |

### (CLOSED)Round-1(Pro)

FL’s analysis:

This open issue has been discussed for several times. As clarified, if no consensus for the solution, it means not support the multiplexing and it is up to UE so UE behavior is not needed.

The preparation phase discussion concluded CRs are not pursued but a potential conclusion is to be discussed since some company commented that whether we can have a chair’s note to conclude that this is left to UE implementation on which one to drop – i.e., SR or NACK-only.

#### Suggested conclusion 3.22.1

It is up to UE implementation on which one to drop (i.e., SR or NACK-only) when NACK-only collides with SR.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | OK with the conclusion. |
| Qualcomm | We prefer to specify the error case as The UE does not expect to be scheduled with NACK-only and SR in the same PUCCH. |
| Vivo | Same view as Qualcomm |
| NTT DOCOMO | OK |
| ZTE | We still prefer to have a solution to resolve this issue. If no consensus, we can accept this proposal. Defining as error case is not acceptable since it may bring restriction for the network configuration. |
| MTK | QC’s suggestion is fine |
| Nokia, NSB | Support the FL’s proposal. We can add the following in the specification for clarification: A UE is not required to multiplex in a PUCCH multicast HARQ-ACK information according to second HARQ-ACK reporting mode and an SR. |

### Round-2(CR)

FL’s analysis:

Agreement

It is up to UE implementation on which one to drop between SR or NACK-only when NACK-only collides with SR when UE would transmit PUCCH with HARQ-ACK according to the second reporting mode.

Based on the agreement made in 1st GTW session on Oct. 12, this round discusses the moderator draft CR.

#### Draft CR 3.22.2

The draft CR in Moderator draft CR on issue 1-22_v000_Mod (to be replaced by the link to the draft CR in the inbox) is endorsed.

Company views:

| Company | Comments |
| --- | --- |
| Samsung | Text updates are needed as the language of the agreement cannot be a copy-paste in the specs (e.g. there is no ‘NACK-only’ defined, dropping is for a channel transmission not for ‘SR’ or ‘NACK-only’, the SR needs to be ‘positive’, …). Suggest the following If a UE would simultaneously transmit a first PUCCH with positive SR and a second PUCCH with HARQ-ACK according to the second HARQ-ACK reporting mode, it is up to UE implementation for the UE to transmit either the first PUCCH or the second PUCCH. |
| ZTE | We suggest the following updates on top of Samsung’s version since it is applied for the SR and NACK-only with the same priority. If a UE would simultaneously transmit a first PUCCH with positive SR and a second PUCCH with HARQ-ACK with the same priority index according to the second HARQ-ACK reporting mode, it is up to UE implementation for the UE to transmit either the first PUCCH or the second PUCCH. |

# Proposals for GTW on Oct. 12

#### Proposal 3.12.1

For UEs not supporting more than one SPS configuration, when UE would multiplex HARQ-ACK for unicast SPS PDSCHs and multicast dynamic grant PDSCHs with ACK/NACK based feedback,  support the following Alt1:

  - Alt1: the PUCCH carrying the multiplexed HARQ-ACK is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast.

  - Alt2: drop HARQ-ACK for multicast in this case.

#### Proposal 3.1.1

For PRI included in DCI format 4_1/4_2, if the G-RNTI is configured with NACK-only mode2, PRI will be ignored by UEs.

#### Proposal 3.2.1

UE is not expected to be configured with Type1 CB for NACK-only.

Which one is agreeable from the following:

- Alt1: UE is not expected to be configured with Type1 CB for NACK-only.

- Alt2: UE is not expected to be configured with Type1 CB for NACK-only mode2.

#### Proposal 3.5.1

For a UE configured with G-CS-RNTI(s) with NACK-only HARQ-ACK feedback, when NACK-only HARQ-ACK bits are transformed into ACK/NACK HARQ-ACK bits, the PUCCH resource used for transmitting the multiplexed HARQ-ACK bits is determined from the sps-PUCCH-AN-List configured for unicast, if sps-PUCCH-AN-ListMulticast is not configured.

#### Suggested conclusion 3.22.1

It is up to UE implementation on which one to drop (i.e., SR or NACK-only) when NACK-only collides with SR.

#### Agreement from the GTW:

Agreement

For UEs not provided with SPS-PUCCH-AN-List, when UE would multiplex HARQ-ACK for unicast SPS PDSCHs and multicast dynamic grant PDSCHs, the PUCCH carrying the multiplexed HARQ-ACK is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast.

Agreement

For PRI included in DCI format 4_1/4_2, if the G-RNTI is configured with NACK-only mode2, PRI will be ignored by UEs.

Agreement

For a UE configured with G-CS-RNTI(s) with NACK-only HARQ-ACK feedback, when NACK-only HARQ-ACK bits are transformed into ACK/NACK HARQ-ACK bits, the PUCCH resource used for transmitting the multiplexed HARQ-ACK bits is determined from the sps-PUCCH-AN-List configured for unicast, if sps-PUCCH-AN-ListMulticast is not configured.

Agreement

It is up to UE implementation on which one to drop between SR or NACK-only when NACK-only collides with SR when UE would transmit PUCCH with HARQ-ACK according to the second reporting mode.

# Proposals for GTW on Oct. 14

#### Proposal 3.2.2

For UE provided NACK-only mode2 HARQ-ACK feedback for G-RNTI(s)/G-CS-RNTI(s), UE does not expect to be provided Type-1 HARQ-ACK codebook for multicast.

#### Proposal 3.3.2

If a G-RNTI/G-CS-RNTI is configured with ‘dci-enabler’, the UE is transmitting the multicast feedback when scheduled by DCI format 4_1 for this G-RNTI/G-CS-RNTI.

which alternatives is agreeable from the following and why?

- Alt1: the UE is not expected to be scheduled by DCI format 4_1 for this G-RNTI/G-CS-RNTI

- Alt2: the UE is not transmitting the multicast feedback when scheduled by DCI format 4_1 for this G-RNTI/G-CS-RNTI.

- Alt3: the UE is transmitting the multicast feedback when scheduled by DCI format 4_1 for this G-RNTI/G-CS-RNTI

- FeedbackEnablerMulticast is not configured, there will be no ‘dci-enabler’ even in DCI format 4_2 and UE does not transmit the feedback as the default behavior. - Harq-FeedbackEnablerMulticast-r17 ENUMERATED {dci-enabler,enabled} OPTIONAL, -- Need S - harq-FeedbackEnablerMulticast - Indicates whether the UE shall provide HARQ feedback for MBS multicast. Value dci-enabler means that whether the UE shall provide HARQ feedback for MBS multicast is indicated by DCI. Value enabled means the UE shall always provide HARQ feedback for MBS multicast. When the field is absent, the value “disabled” is used as defined in TS 38.213 [3].

#### Proposal for issue1-7 NACK-only mode2 for case2 and case3

Endorse the following text proposal:

------------------------------Start of text proposal to TS38.213 V17.3.0 clause 18-------------------------------

A UE that is indicated the second HARQ-ACK reporting mode for only one G-RNTI can be indicated by moreThanOneNackOnlyMode to provide associated HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode or by selecting a PUCCH resource from a set of PUCCH resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1. The UE generates HARQ-ACK information bits for the second HARQ-ACK reporting mode according to a Type-2 HARQ-ACK codebook as described in clause 9.1.3.1. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0.

For a UE that is indicated the second HARQ-ACK reporting mode and moreThanOneNackOnlyMode, all PUCCH resources associated with the second HARQ-ACK reporting mode have same starting symbol and same number of symbols.

- Table 18-1: Mapping of values of HARQ-ACK information bits to PUCCH resources for the second HARQ-ACK reporting mode

| Value of HARQ-ACK information bits |  |  |  | PUCCH resource |
| --- | --- | --- | --- | --- |
| {0} | {0,0} | {0,0,0} | {0,0,0,0} | 1st PUCCH resource from resourceList |
|  | {1,0} | {1,0,0} | {1,0,0,0} | 2nd PUCCH resource from resourceList |
|  | {0,1} | {0,1,0} | {0,1,0,0} | 3rd PUCCH resource from resourceList |
|  |  | {1,1,0} | {1,1,0,0} | 4th PUCCH resource from resourceList |
|  |  | {0,0,1} | {0,0,1,0} | 5th PUCCH resource from resourceList |
|  |  | {1,0,1} | {1,0,1,0} | 6th PUCCH resource from resourceList |
|  |  | {0,1,1} | {0,1,1,0} | 7th PUCCH resource from resourceList |
|  |  |  | {1,1,1,0} | 8th PUCCH resource from resourceList |
|  |  |  | {0,0,0,1} | 9th PUCCH resource from resourceList |
|  |  |  | {1,0,0,1} | 10th PUCCH resource from resourceList |
|  |  |  | {0,1,0,1} | 11th PUCCH resource from resourceList |
|  |  |  | {1,1,0,1} | 12th PUCCH resource from resourceList |
|  |  |  | {0,0,1,1} | 13th PUCCH resource from resourceList |
|  |  |  | {1,0,1,1} | 14th PUCCH resource from resourceList |
|  |  |  | {0,1,1,1} | 15th PUCCH resource from resourceList |

------------------------------End of text proposal to TS38.213 V17.3.0 clause 18-------------------------------

# References

R1-2208466	Correction on processing timeline for NACK-only mode2 to TS38.214	Huawei, HiSilicon, CBN

R1-2208467	Correction on codebook type for NACK-only HARQ-ACK feedback to TS38.213	Huawei, HiSilicon, CBN

R1-2208468	Correction on PRI for NACK-only HARQ-ACK feedback to TS38.213	Huawei, HiSilicon, CBN

R1-2208469	Correction on UE behaviors of PDCCH monitoring for configured RM patterns to TS38.213	Huawei, HiSilicon, CBN

R1-2208470	Correction on SS0 availability for scheduling MBS to TS38.213	Huawei, HiSilicon, CBN

R1-2208617	Draft CR on HARQ-ACK feedback for PDSCH scheduled by DCI format 4-1	vivo

R1-2208618	Draft CR on PUCCH determination for UE configured with NACK-only feedback mode	vivo

R1-2208619	Draft CR on type 2 codebook determination with DG PDSCHs and SPS PDSCHs	vivo

R1-2208620	Discussion on SPS PDSCH overlapping handling in FDM case	vivo

R1-2208701	Remaining Issues for RRC_CONNECTED UEs supporting MBS	Nokia, Nokia Shanghai Bell

R1-2208887	Draft CR on HARQ-ACK multiplexing of unicast SPS PDSCHs and multicast DG PDSCHs	vivo

R1-2208923	Discussion on MBS supporting HARQ-ACK codebook retransmission	CATT

R1-2208924	Discussion on MBS supporting  deferring HARQ-ACK for SPS PDSCH	CATT

R1-2208925	Draft CR on MBS supporting HARQ-ACK codebook retransmission	CATT

R1-2208926	Draft CR on MBS supporting  deferring HARQ-ACK for SPS PDSCH	CATT

R1-2208927	Draft CRs for NR Multicast and Broadcast Service	CATT

R1-2208928	Corrections on multicast DCI format to enable/disable HARQ-ACK	CATT

R1-2208929	Discussion on  multicast DCI format to enable/disable HARQ-ACK	CATT

R1-2208995	Correction on Type-2 HARQ-ACK codebook for MBS	Langbo

R1-2208996	Correction on Type-3 HARQ-ACK codebook for MBS	Langbo

R1-2209137	Remaining Issues on NR MBS	NEC

R1-2209310	Remaining issues on HARQ-ACK feedback for multicast	CMCC

R1-2209311	Discussion on specs alignment of PDSCH simultaneous reception for MBS	CMCC

R1-2209312	Draft CR on multicast HARQ-ACK codebook type configuration in DCI formats	CMCC

R1-2209313	Draft CR on multicast rate-matching pattern configuration	CMCC

R1-2209314	Draft CR on SPS and dynamic scheduling PDSCH(s) collision for MBS	CMCC

R1-2209315	Draft CR on RRC parameters correction in TS 38.211	CMCC

R1-2209316	Draft CR on RRC parameters correction in TS 38.212	CMCC

R1-2209317	Draft CR on RRC parameters correction in TS 38.213	CMCC

R1-2209318	Draft CR on RRC parameters correction in TS 38.214	CMCC

R1-2209449	Maintenance on NR Multicast and Broadcast Services	LG Electronics

R1-2209470	Maintenance of broadcast and multicast for MBS	ZTE

R1-2209471	Draft CR on CFR configuration for multicast	ZTE

R1-2209472	Draft CR on terms of G-RNTI used for MTCH	ZTE

R1-2209473	Draft CR on restrictions of simultaneous reception	ZTE

R1-2209474	Draft CR on SPS collision handling	ZTE

R1-2209475	Draft CR on 1 bit NACK-only feedback	ZTE

R1-2209476	Draft CR on determining NACK-only PUCCH in NACK-only mode1	ZTE

R1-2209524	Corrections on the MBS reception type combinations in TS 38.202	MediaTek Inc.

R1-2209525	Corrections on the MBS in TS 38.213	MediaTek Inc.

R1-2209526	Corrections on the MBS in TS 38.214	MediaTek Inc.

R1-2209527	Remaining issues on NR MBS	MediaTek Inc.

R1-2209566	Remaining issues on NR Multicast and Broadcast Services	Apple

R1-2209708	Maintenance on multicast-broadcast services	Samsung

R1-2209822	Remaining issues for Rel-17 MBS	Huawei, HiSilicon, CBN

R1-2209832	Correction on processing timeline for NACK-only mode2 to TS38.213	Huawei, HiSilicon, CBN

R1-2209833	Correction on the max data rate for multiplexing MBS and unicast to TS38.214	Huawei, HiSilicon, CBN

R1-2209882	Draft CR on DAI field in DCI format 4_2	NTT DOCOMO, INC.

R1-2209883	Draft CR on HARQ-ACK feedback for SPS GC-PDSCH	NTT DOCOMO, INC.

R1-2209884	Draft CR on NACK-only based feedback for multicast	NTT DOCOMO, INC.

R1-2209885	Draft CR on multiplexing NACK-only based feedback with SR	NTT DOCOMO, INC.

R1-2209954	Draft CR on DCI-indicated enabling/disabling multicast feedback for Type-1 CB	Qualcomm Incorporated

R1-2209955	Draft CR on Type-2 CB for NACK-only multicast feedback	Qualcomm Incorporated

R1-2209956	Draft CR on max data rate per CC in case of FDMed unicast and MBS PDSCHs	Qualcomm Incorporated

R1-2209957	Scaling factor for FDMed unicast and MBS PDSCHs	Qualcomm Incorporated

R1-2209958	Draft CR on upper bound of TBS LBRM in case of FDMed unicast and MBS PDSCHs	Qualcomm Incorporated

R1-2209959	Draft CR on PDSCH processing time required to select PUCCH for NACK-only mode2 based multicast feedback	Qualcomm Incorporated

R1-2209960	Draft CR on multicast PDSCH with a HARQ process with disabled HARQ-ACK feedback	Qualcomm Incorporated

R1-2209961	Draft CR on PDCCH monitoring when overlapping with rate matching patterns	Qualcomm Incorporated

R1-2210075	Correction on MBS SPS	ASUSTeK

R1-2210095	Correction on configurations of G-RNTI and G-CS-RNTI	ASUSTeK

R1-2210096	Correction on configuration of PDSCH aggregation factor for MBS	ASUSTeK

R1-2210155	Correction on HARQ-ACK codebook types in UL DCI formats for scheduling MBS	Lenovo

R1-2210156	Draft CR on HARQ-ACK feedback for PDSCH scheduled by DCI format 4_1	Lenovo

R1-2210157	Draft CR on DAI update for multicast DCI formats	Lenovo

R1-2210158	Draft CR on simultaneous configuration of Type-1 HARQ-ACK codebook and dci-enabler for multicast service	Lenovo

R1-2210159	Remaining issues on HARQ-ACK feedback for NR MBS	Lenovo

R1-2210173	Maintenance on NR Multicast and Broadcast Services	Ericsson

R1-2210207	Correction on retransmission schemes for MBS HARQ-ACK feedback to TS38.213	Huawei, HiSilicon, CBN

R1-2210208	Correction on the channel combinations for MBS UE handling to TS38.213	Huawei, HiSilicon, CBN

R1-2210209	Correction on the channel combinations for MBS UE handling to TS38.214	Huawei, HiSilicon, CBN

R1-2210210	Correction on the channel combinations for MBS UE handling to TS38.202	Huawei, HiSilicon, CBN