3GPP TSG RAN WG1#110bis-e		R1-2210458

e-Meeting, October 10th – 19th, 2022

Agenda Item:	8.2

Source:	Moderator (Lenovo)

Title:	FL summary #1 for email discussion [110bis-e-R17-FR2-2-04] on PDCCH monitoring enhancements

Document for:	Discussion, Decision

# Introduction

As stated by the chairman:

[110bis-e-R17-FR2-2-04] Email discussion for maintenance on PDCCH monitoring enhancements for FR2-2 for issues PDCCH-2, PDCCH-4 and PDCCH-1 (incl. whether this should be handled in UE features) in R1-2210392 – Alex (Lenovo)

Check points: October 14, October 19

# Discussion

## [Closed] Topic PDCCH-1: multi-slot PDCCH monitoring for Group (2) SSs

Ericsson [5] and ZTE, Sanechips [1] have provided draft CRs, with an additional discussion document provided by Ericsson [6]. The relevant parts of the two draft CRs for clause 10.1 of 38.213 are provided below side-by-side.

| ZTE, Sanechips [1] | Ericsson [5] |
| --- | --- |
| For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with S≤10 search space sets where, for each search space set from the S search space sets, the UE is provided the following by SearchSpace: - a search space set index s, 0<s<40 , by searchSpaceId - […] - a bitmap, by monitoringSlotsWithinSlotGroup, that applies per group of slots and provides a PDCCH monitoring pattern indicating slots in a group of slots for PDCCH monitoring - a size of the group of slots is same as a size of monitoringSlotsWithinSlotGroup - for a Type1-PDCCH CSS set provided by ra-SearchSpace in dedicated RRC signaling, or for a Type3-PDCCH CSS set, or for a USS set, the PDCCH monitoring pattern indicates only consecutive slots in the group of slots for PDCCH monitoring and, at least for one combination Xs,Ys indicated by the UE as a capability, a number of the consecutive slots is not larger than Ys - for a Type1-PDCCH CSS set provided by ra-SearchSpace in SIB1, the PDCCH monitoring pattern indicates only up to 1 slot in the group of slots for PDCCH monitoring - for a Type0-PDCCH CSS set or for a Type0A-PDCCH CSS set, or for a Type2-PDCCH CSS set, the PDCCH monitoring pattern indicates slots in the group of slots for PDCCH monitoring, and the slots are not restricted to be consecutive, a number of those slots is not larger than the size of monitoringSlotsWithinSlotGroup | For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with S≤10 search space sets where, for each search space set from the S search space sets, the UE is provided the following by SearchSpace: - a search space set index s, 0<s<40 , by searchSpaceId - […] - a bitmap of length L, by monitoringSlotsWithinSlotGroup, that applies per group of slots and provides a PDCCH monitoring pattern indicating slots in a group of slots for PDCCH monitoring - a size of the group of slots is same as a size of monitoringSlotsWithinSlotGroup - for a Type1-PDCCH CSS set provided by ra-SearchSpace in dedicated RRC signaling, or for a Type3-PDCCH CSS set, or for a USS set, the PDCCH monitoring pattern indicates only consecutive slots in the group of slots for PDCCH monitoring and, at least for one combination Xs,Ys indicated by the UE as a capability, a number of the consecutive slots is not larger than Ys - for a Type0/0A/2-PDCCH CSS set, the PDCCH monitoring pattern can indicate consecutive or non-consecutive slots in the group of slots for PDCCH monitoring, and the number of slots can be up to L - for a Type1-PDCCH CSS set provided by ra-SearchSpace in PDCCH-ConfigCommon, the PDCCH monitoring pattern can indicate one slot in the group of slots for PDCCH monitoring |

### First round discussion

Question PDCCH-1.1a: Do you agree to capture the "Group (2)" agreements in 38.213? Please provide reasons for Yes or No.

| Company | Comment |
| --- | --- |
| Ericsson (from preparation phase) | RAN2 agreed to a CR to 38.331 last meeting that says "The number of slots for multi-slot PDCCH monitoring is configured according to clause 10 in TS 38.213 [13]." Hence 331 now refers back to 38.213, and the number of slots is not described for Group (2) SSs in the current version of 38.213. LGE mentions that FG24-4 and 24-5 already cover this; however, the FG list in 38.822 is not normative. If companies prefer to capture this in 38.306 (normative spec), then we should send an LS to RAN2 to ask them to change the reference in 38.331 to 38.306 instead of 38.213. We think it is easier just to capture this in 38.213 and avoid such an LS. |
| ZTE, Sanechips (from preparation phase) | We agree with Ericsson’s comment. The easier and straightforward way is to capture Group (2) SSs in TS 38.213 since the description of monitoringSlotsWithinSlotGroup in TS 38.331 directly refers to RAN1’s specification, also, it seems incomplete and irrational to only capture the feature for Group (1) SS sets. Hence, TS 38.213 should be updated accordingly. |
| Moderator | Closed; an agreement was reached in GTW on Wed 12 October. Please continue with discussion of a CR/TP in Question PDCCH-1.1b below. |

Question PDCCH-1.1b: Assuming that the "Group (2)" agreements are to be captured in 38.213, please state any comments on the two draft CR text proposals, or a preference which one is more suitable.

| Company | Comment |
| --- | --- |
| Moderator | If a majority prefers Ericsson's TP, the usage of "can" should be avoided (e.g. the number of slots is at most equal to L.) |
| Ericsson | No strong view on which TP is used as a basis, but if the Ericsson TP is used, then suggest the following updates: - the number of slots can be up to L the number of slots is at most equal to L - provided by ra-SearchSpace in PDCCH-ConfigCommon provided by ra-SearchSpace in SIB1 |
| Huawei, HiSilicon | If Ericsson’s TP is adopted, we would suggest to change the length “L” to other letter. L is already used in the section for aggregation level. |
| ZTE, Sanechips | Either one of the two draft CRs is ok for us and we prefer the wording “Type1-PDCCH CSS set provided by ra-SearchSpace in SIB1” instead of “Type1-PDCCH CSS set provided by ra-SearchSpace in PDCCH-ConfigCommon” to align with the following sentence decribed in TS 38.213: |
| LG Electronics | Fine with either TP considering Ericsson's additional comments. |
| Intel | We are OK with Ericsson’s TP with update to avoid ‘can’ |

### First round discussion summary

There is no strong preference between the two TPs provided by Ericsson and ZTE. The existing spec text already uses "same as a size of monitoringSlotsWithinSlotGroup", so there seems to be little need to introduce a new symbol for this size and we can re-use the wording for the added description. Therefore Moderator suggests to move ahead with ZTE's TP.

FL Proposal PDCCH-1.1

- Endorse TP in R1-2208710.

- Trim down "Reason for change" (from R1-2208710) to the following:

  - "In RAN1#108-e and RAN1#109-e  agreements were achieved for search space set configuration of multi-slot PDCCH monitoring, where a new parameter monitoringSlotsWithinSlotGroup-r17 is introduced to indicate which slot(s) within a slot group are configured for multi-slot PDCCH monitoring. Different types of search space set correspond to different PDCCH monitoring patterns. However, only the monitoring pattern for some SSs has been specified in TS 38.213 so far, neither TS 38.213 nor TS 38.331 capture PDCCH monitoring pattern indicating slots in a group of slots for Type0/0A/2 CSS and Type CSS without dedicated RRC signaling."

## Topic PDCCH-2: multi-slot PDCCH monitoring in CA or NR-DC scenarios

CATT has provided two draft CRs [3] [4] with an additional discussion document [2]. Moderator thinks both draft CRs should be handled and discussed individually.

### [ACTIVE] Topic PDCCH-2.1: Correction on BD/CCE budget of  scheduling cell(s)  for the features extending NR operation to 71 GHz

The relevant parts of draft CR [3] for 38.213 clause 10.1 are given below for easy reference. Please refer to [3] for more context.

If a UE is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells for which the UE is not provided monitoringCapabilityConfig, or is provided monitoringCapabilityConfig = r15monitoringcapability and is not provided CORESETPoolIndex, with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ where μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ≤Ncellscap, the UE is not required to monitor, on the active DL BWPs of the scheduling cells, - more than MPDCCHtotal,slot,μ=MPDCCHmax,slot,μ PDCCH candidates or more than CPDCCHtotal,slot,μ=CPDCCHmax,slot,μ non-overlapped CCEs per slot for each scheduled cell when the scheduling cell is from the Ncells,0DL,μ downlink cells, or - more than MPDCCHtotal,slot,μ=γ∙MPDCCHmax,slot,μ PDCCH candidates or more than CPDCCHtotal,slot,μ=γ∙CPDCCHmax,slot,μ non-overlapped CCEs per slot for each scheduled cell when the scheduling cell is from the Ncells,1DL,μ downlink cells - more than MPDCCHmax,slot,μ PDCCH candidates or more than CPDCCHmax,slot,μ non-overlapped CCEs per slot for CORESETs with same coresetPoolIndex value for each scheduled cell when the scheduling cell is from the Ncells,1DL,μ downlink cells Ncellscap is replaced by Ncells,r15cap-r16, if a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability. Ncellscap is replaced by Ncells, r15/r17cap-r17, if a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r17monitoringcapability. Ncellscap is replaced by Ncells, r15/ {r16, r17} cap-r17, if a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability and monitoringCapabilityConfig = r17monitoringcapability. If a UE - is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells for which the UE is not provided monitoringCapabilityConfig, or is provided monitoringCapabilityConfig = r15monitoringcapability and is not provided coresetPoolIndex, - with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cell(s) using SCS configuration μ, where μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ>Ncellscap, and - a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,slot,μ=Ncellscap⋅MPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j PDCCH candidates or more than CPDCCHtotal,slot,μ=Ncellscap⋅CPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) from the Ncells,0DL,μ+Ncells,1DL,μ downlink cells. Ncellscap is replaced by Ncells, r15cap-r16 if a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability. Ncellscap is replaced by Ncells, r15/r17cap-r17, if a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r17monitoringcapability. Ncellscap is replaced by Ncells, r15/ {r16, r17} cap-r17.If a UE is configured with downlink cells for which the UE is provided monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability and monitoringCapabilityConfig = r17monitoringcapability. […] If a UE is configured with Ncells,r16DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, where μ=01Ncells,r16DL,μ≤Ncellscap-r16, the UE is not required to monitor, on the active DL BWP of the scheduling cell, more than MPDCCHtotal,(X,Y),μ=MPDCCHmax,(X,Y),μ PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=CPDCCHmax,(X,Y),μ non-overlapped CCEs per span for each scheduled cell when the scheduling cell is from the Ncells,r16DL,(X,Y),μ downlink cells. If a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability, Ncellscap-r16 is replaced by Ncells,r16cap-r16. If a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r16monitoringcapability and monitoringCapabilityConfig = r17monitoringcapability, Ncellscap is replaced by Ncells, r16/r17cap-r17. If a UE is configured with downlink cells for which the UE is provided monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability and monitoringCapabilityConfig = r17monitoringcapability, Ncellscap is replaced by Ncells, r16/ {r15, r17} cap-r17. If a UE is configured only with Ncells,r16DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs - per set of spans on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells within every X symbols, if the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells results to PDCCH monitoring according to the combination X,Y and any pair of spans in the set is within Y symbols, where first X symbols start at a first symbol with a PDCCH monitoring occasion and next X symbols start at a first symbol with a PDCCH monitoring occasion that is not included in the first X symbols - per set of spans across the active DL BWP(s) of all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells, with at most one span per scheduling cell for each set of spans, otherwise where Ncells,r16DL,j is a number of configured cells with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration j. If a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability, Ncellscap-r16 is replaced by Ncells,r16cap-r16. If a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r16monitoringcapability and monitoringCapabilityConfig = r17monitoringcapability, Ncellscap is replaced by Ncells, r16/r17cap-r17. If a UE is configured with downlink cells for which the UE is provided monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability and monitoringCapabilityConfig = r17monitoringcapability, Ncellscap is replaced by Ncells, r16/ {r15, r17} cap-r17.

#### First round discussion

Question PDCCH-2.1: Do you agree to the draft CR in [3]?

| Company | Comment |
| --- | --- |
| Ericsson (from preparation phase) | We think the CR is incorrect, and the cases in question are already described in other paragraphs in 38.213. Hence, no CR needed. |
| Ericsson 2 | To elaborate on our comment from the prep phase, the other paragraphs are as follows. The highlighted clauses already cover the cases in draft CR in [3], hence the CR is not needed. If a UE is configured with Ncells,r17,0DL,μ+Ncells,r17,1DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the active DL BWPs of the scheduling cells, and with Ncells,r17,0DL,Xs,μ+Ncells,r17,1DL,Xs,μ of the Ncells,r17,0DL,μ+Ncells,r17,1DL,μ downlink cells using any combination Xs,Ys for a group of Xs slots for PDCCH monitoring, where μ=56Ncells,r17,0DL,μ+γ∙Ncells,r17,1DL,μ≤Ncellscap-r17, the UE is not required to monitor, on the active DL BWP of the scheduling cell, - more than MPDCCHtotal,Xs,μ=MPDCCHmax,Xs,μ PDCCH candidates or more than CPDCCHtotal,Xs,μ=CPDCCHmax,Xs,μ non-overlapped CCEs per group of Xs slots for each scheduled cell when the scheduling cell is from the Ncells,r17,0DL,Xs,μ downlink cells, or - more than MPDCCHtotal,Xs,μ=γ∙MPDCCHmax,Xs,μ PDCCH candidates or more than CPDCCHtotal,Xs,μ=γ∙CPDCCHmax,Xs,μ non-overlapped CCEs per group of Xs slots for each scheduled cell when the scheduling cell is from the Ncells,r17,1DL,Xs,μ downlink cells, or - more than MPDCCHmax,Xs,μ PDCCH candidates or more than CPDCCHmax,Xs,μ non-overlapped CCEs per group of Xs slots for CORESETs with same coresetPoolIndex for each scheduled cell when the scheduling cell is from the Ncells,r17,1DL,Xs,μ downlink cells If the UE is configured with downlink cells for which the UE is provided monitoringCapabilityConfig = r15monitoringcapability and downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the active DL BWPs, Ncellscap-r17 is replaced by Ncells,r17/r15cap-r17. If the UE is configured with downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the active DL BWPs, Ncellscap-r17 is replaced by Ncells,r17/r16cap-r17. If the UE is configured with downlink cells for which the UE is provided monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability and downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the active DL BWPs, Ncellscap-r17 is replaced by Ncells,r17/{r15,r16}cap-r17. If, for one or more of the cells, the UE is provided with monitoringCapabilityConfig = r16monitoringcapability, γ=1. If a UE is configured Ncells,r17,0DL,μ+Ncells,r17,1DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the active DL BWPs of the scheduling cells, and with Ncells,r17,0DL,Xs,μ+Ncells,r17,1DL,Xs,μ of the Ncells,r17,0DL,μ+Ncells,r17,1DL,μ downlink cells using any combination Xs,Ys for a group of Xs slots for PDCCH monitoring, where μ=56Ncells,r17,0DL,μ+γ∙Ncells,r17,1DL,μ>Ncellscap-r17, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,Xs,μ=Ncellscap-r17⋅MPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=56Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,j PDCCH candidates, or more than CPDCCHtotal,Xs,μ=Ncellscap-r17⋅CPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=56Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,j non-overlapped CCEs, per group of Xs slots on the active DL BWP(s) of scheduling cell(s) from the Ncells,r17,0DL,Xs,μ+Ncells,r17,1DL,Xs,μ downlink cells where Ncells,r17,0DL,j+Ncells,r17,1DL,j is a number of configured cells with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration j. If the UE is configured Ncells,r17,0DL,j+Ncells,r17,1DL,j downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability or monitoringCapabilityConfig = r16monitoringcapability, and monitoringCapabilityConfig = r17monitoringcapability for the active DL BWP, Ncellscap-r17 is replaced by Ncells,r17/r15cap-r17, or by Ncells,r17/r16cap-r17, or by Ncells,r17/{r15,r16}cap-r17, respectively, and MPDCCHtotal,Xs,μ=Ncells, refcap-r17⋅MPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=06Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,j, CPDCCHtotal,Xs,μ=Ncells,refcap-r17⋅CPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=06Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,j, and Ncells,refcap-r17 is one of Ncells,r17/r15cap-r17, Ncells,r17/r16cap-r17, or Ncells,r17/{r15,r16}cap-r17, respectively. If, for one or more of the cells, the UE is provided with monitoringCapabilityConfig = r16monitoringcapability, γ=1. |
| vivo | We think the CR is needed and correct. The above paragraph referred by Ericsson only handles that cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability in hybrid case. For example, if the UE is configured with downlink cells for which the UE is provided monitoringCapabilityConfig = r15monitoringcapability and downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the active DL BWPs, there are two cell limit: Ncells,r17/r15cap-r15 and Ncells,r17/r15cap-r17. Ncells,r17/r15cap-r17 is already handled in the above paragraph. However, handling of Ncells,r17/r15cap-r15 is missing in current spec and the above CR is targeting to solve this. |
| Huawei, HiSilicon | We shared the view from vivo. The BD/CCE on the serving cell with r15 and r16 monitoring capability should also be updated with Ncells, r15/r17cap-r17, Ncells, r15/{r16,r17}cap-r17 and Ncells, r16/r17cap-r17 , Ncells, r16/{r15,r17}cap-r17 respectively. |
| ZTE, Sanechips | Agree with vivo and Huawei. |
| LG Electronics | Support the CR with a similar view from vivo |
| Intel | The CR from CATT helps to make the spec clear. We are OK to agree on the CR. |

#### First round discussion summary

Most replying companies support the TP, suggesting to update the BD/CCE on the serving cell with r15 and r16 monitoring capability by Ncells, r15/r17cap-r17, Ncells, r15/{r16,r17}cap-r17 and Ncells, r16/r17cap-r17 , Ncells, r16/{r15,r17}cap-r17 (respectively) as well. Ericsson considers the CR unnecessary, however has not replied to further vivo's explanation why the CR is needed. Moderator would like to see if Ericsson can agree given vivo's explanation.

FL Suggestion

- Continue discussion, additionally considering further updating the BD/CCE on the serving cell with r15 and r16 monitoring capability by Ncells, r15/r17cap-r17, Ncells, r15/{r16,r17}cap-r17 and Ncells, r16/r17cap-r17 , Ncells, r16/{r15,r17}cap-r17 (respectively).

#### Second round discussion

Question PDCCH-2.1a: Considering vivo's explanation in the first round, can the TP in [3] be agreed in principle? Can Huawei (or other proponents) provide a TP to update the BD/CCE on the serving cell with r15 and r16 monitoring capability by Ncells, r15/r17cap-r17, Ncells, r15/{r16,r17}cap-r17 and Ncells, r16/r17cap-r17 , Ncells, r16/{r15,r17}cap-r17 (respectively)?

| Company | Comment |
| --- | --- |

### [ACTIVE] Topic PDCCH-2.2: Correction on the total serving cell(s) number when r17monitoringcapability of monitoringCapabilityConfig is configured for the features extending NR operation to 71 GHz

The relevant parts of draft CR [4] for 38.213 clause 10.1 are given below for easy reference. Please refer to [4] for more context.

If a UE is configured Ncells,r17,0DL,μ+Ncells,r17,1DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the active DL BWPs of the scheduling cells, and with Ncells,r17,0DL,Xs,μ+Ncells,r17,1DL,Xs,μ of the Ncells,r17,0DL,μ+Ncells,r17,1DL,μ downlink cells using any combination Xs,Ys for a group of Xs slots for PDCCH monitoring, where μ=56Ncells,r17,0DL,μ+γ∙Ncells,r17,1DL,μ>Ncellscap-r17, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,Xs,μ=Ncellscap-r17⋅MPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=56Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,j PDCCH candidates, or more than CPDCCHtotal,Xs,μ=Ncellscap-r17⋅CPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=56Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,j non-overlapped CCEs, per group of Xs slots on the active DL BWP(s) of scheduling cell(s) from the Ncells,r17,0DL,Xs,μ+Ncells,r17,1DL,Xs,μ downlink cells where Ncells,r17,0DL,j+Ncells,r17,1DL,j is a number of configured cells with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration j. If the UE is configured Ncells,r17,0DL,j+Ncells,r17,1DL,j downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability or monitoringCapabilityConfig = r16monitoringcapability, and monitoringCapabilityConfig = r17monitoringcapability for the active DL BWP, Ncellscap-r17 is replaced by Ncells,r17/r15cap-r17, or by Ncells,r17/r16cap-r17, or by Ncells,r17/{r15,r16}cap-r17, respectively, and MPDCCHtotal,Xs,μ=Ncells, refcap-r17⋅MPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=56Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,j, CPDCCHtotal,Xs,μ=Ncells, refcap-r17⋅CPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=56Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,jMPDCCHtotal,Xs,μ=Ncells, refcap-r17⋅MPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=06Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,jCPDCCHtotal,Xs,μ=Ncells,refcap-r17⋅CPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=06Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,jand Ncells,refcap-r17 is one of Ncells,r17/r15cap-r17, Ncells,r17/r16cap-r17, or Ncells,r17/{r15,r16}cap-r17, respectively. If, for one or more of the cells, the UE is provided with monitoringCapabilityConfig = r16monitoringcapability, γ=1.

#### First round discussion

Question PDCCH-2.2: Do you agree to the draft CR in [4]?

| Company | Comment |
| --- | --- |
| Ericsson | The current spec is correct, hence this CR is neither needed nor correct. Indeed the summation in the formula should be from 0 .. 6 (and not 5 ..6) since the formula applies to the case when there is a mixture of cells with Rel-15/16/17 monitoring by virtue of the sentence preceding the formula: If the UE is configured Ncells,r17,0DL,j+Ncells,r17,1DL,j downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability or monitoringCapabilityConfig = r16monitoringcapability, and monitoringCapabilityConfig = r17monitoringcapability for the active DL BWP, … A cell with r15monitoringcapability and a cell with r16monitoringcapability can be configured with 15, 30, 60, or 120 kHz SCS. This is in contrast to cells with r17monitoringcapability which can be configured only with 480 or 960 kHz SCS. Hence it makes sense that the summation in the formula is from 0 .. 6. |
| vivo | We are fine with this CR. In a case with a mixture of cells with Rel-15/16/17 monitoring capability, the cells with different capability type are independent to follow a limit of BD/CCE. So the summation in the formula for cells with R17 monitoring capability is the number of cells configured with r17 monitoring capability, i.e. from SCS 5 to 6. This is the same as the following spec for r16 monitoring capability that can only be configured in FR1 SCS 0 and 1. If a UE is configured only with Ncells,r16DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs However, if there is no change, the spec still works since the number of cells configured with r17 monitoring capability for SCS 0 to 4 is 0. We slightly prefer to have this change to align with the spec as R16. |
| Huawei, HiSilicon | We share the similar view as vivo. The BD/CCE should be counted for the serving cell with PDCCH monitoring capability of the same release. Thus, the sum should only be performed for 480 and 960kHz SCS. Another issue in the current spec we observed is the condition “for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability or monitoringCapabilityConfig = r16monitoringcapability, and monitoringCapabilityConfig = r17monitoringcapability for the active DL BWP”. To my understanding, current text only include the case of r15+17 or r16+17. The case of r15+r16+r17 is not included. maybe native speaker can judge whether my reading is correct or not. Compared with the TP provided CATT, we would suggest a simpler change as following. It can keep consistent written style with other release. If a UE is configured Ncells,r17,0DL,μ+Ncells,r17,1DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the active DL BWPs of the scheduling cells, and with Ncells,r17,0DL,Xs,μ+Ncells,r17,1DL,Xs,μ of the Ncells,r17,0DL,μ+Ncells,r17,1DL,μ downlink cells using any combination Xs,Ys for a group of Xs slots for PDCCH monitoring, where μ=56Ncells,r17,0DL,μ+γ∙Ncells,r17,1DL,μ>Ncellscap-r17, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,Xs,μ=Ncellscap-r17⋅MPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=56Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,j PDCCH candidates, or more than CPDCCHtotal,Xs,μ=Ncellscap-r17⋅CPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=56Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,j non-overlapped CCEs, per group of Xs slots on the active DL BWP(s) of scheduling cell(s) from the Ncells,r17,0DL,Xs,μ+Ncells,r17,1DL,Xs,μ downlink cells where Ncells,r17,0DL,j+Ncells,r17,1DL,j is a number of configured cells with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration j. If the UE is configured Ncells,r17,0DL,j+Ncells,r17,1DL,j downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability or monitoringCapabilityConfig = r16monitoringcapability, and monitoringCapabilityConfig = r17monitoringcapability for the active DL BWP, Ncellscap-r17 is replaced by Ncells,r17/r15cap-r17, or by Ncells,r17/r16cap-r17, or by Ncells,r17/{r15,r16}cap-r17, respectivelyMPDCCHtotal,Xs,μ=Ncells, refcap-r17⋅MPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=06Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,jCPDCCHtotal,Xs,μ=Ncells,refcap-r17⋅CPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=06Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,jNcells,refcap-r17Ncells,r17/r15cap-r17Ncells,r17/r16cap-r17Ncells,r17/{r15,r16}cap-r17. If, for one or more of the cells, the UE is provided with monitoringCapabilityConfig = r16monitoringcapability, γ=1. |
| ZTE, Sanechips | Fine with this CR. |
| LG Electronics | We share the view with vivo that the spec still works even if it does not change. If the spec change is needed, we are fine with the CR. |
| Intel | Fine with this CR. |

#### First round discussion summary

Most replying companies support the TP, however it was pointed out that even without the change the spec and the system works. From that perspective, it seems to be rather a question of taste.

FL Suggestion

- Check with companies if they can accept the TP even if there seems to be no strong technical need

#### Second round discussion

Question PDCCH-2.2a: Even if the proposed change in [4] is not strictly necessary, are you fine to move ahead with the change?

| Company | Comment |
| --- | --- |

## [Closed] Topic PDCCH-4: SSSG switching with multiple cells and different Xs

LG has provided a draft CR [7] with an additional discussion document [8].

The TP of draft CR [7] for 38.213 clause 10.4 is copied here for easy reference. Please refer to [7] for more context.

A UE determines a slot and a symbol in the slot to start or stop PDCCH monitoring according to search space sets for a serving cell that the UE is provided searchSpaceGroupIdList or, if cellGroupsForSwitchList is provided, for a set of serving cells, based on the largest Xs if the SCS configuration μ among all configured DL BWPs in the set of serving cells equals to 6, otherwise, based on the smallest SCS configuration μ among all configured DL BWPs in the serving cell or in the set of serving cells and, if any, in the serving cell where the UE receives a PDCCH and detects a corresponding DCI format 2_0 triggering the start or stop of PDCCH monitoring according to search space sets.

### First round discussion

Question PDCCH-4.1: Do you agree to the draft CR in [7]?

| Company | Comment |
| --- | --- |
| Ericsson | Support the draft CR |
| Huawei, HiSilicon | We are wondering whether gNB should avoid such configuration considering same nuerology are already configured across multiple cell. If group based SSSG switching are configured across multipe serving cell. |
| ZTE, Sanechips | Support. |
| LG Electronics | Support @Huawei: We didn't understand your comment clearly. Could you explain it again in more detail? |
| Intel | We are OK with LG’s CR |

### First round discussion summary

Most replying companies support the proposed change. Huawei questioned whether the scenario should be avoided by gNB configuration, however has not explicitly voiced a rejection of the proposal. Moderator suggests to check by email if there are substantial concerns against the draft CR.

FL Proposal PDCCH-4.1

- Endorse TP in R1-2209438.

# References

[1]	R1-2208710, Draft CR on multi-slot PDCCH monitoring for TS 38.213, ZTE, Sanechips

[2]	R1-2208931, Discussion corrections for  BD/CCE budge of  scheduling cell(s)  for the features extending NR operation to 71 GHz, CATT

[3]	R1-2208932, Correction on BD/CCE budge of  scheduling cell(s)  for the features extending NR operation to 71 GHz, CATT

[4]	R1-2208933, Correction on the total serving cell(s) number when  r17monitoringcapability of monitoringCapabilityConfig is configured for the features extending NR operation to 71 GHz, CATT

[5]	R1-2209177, Draft CR on Group 2 search space configuration, Ericsson

[6]	R1-2209178, Discussion on Group 2 search space configuration, Ericsson

[7]	R1-2209438, Draft CR for SSSG switching with multiple cells in FR2-2, LG Electronics

[8]	R1-2209439, Discussion on SSSG switching with multiple cells in FR2-2, LG Electronics